- RECOMMENDED IDENTIFIED REQUIREMENTS

PART A – Standard identified requirements

THE DEVELOPMENT

The following identified requirements have been imposed to ensure that the development activity is carried out in accordance with the plans / documents and any amendments approved under Part 5 of the Environmental Planning & Assessment Act 1979.

1.	The development must be carried out substantially in accordance with the following plans / documents as modified below and by any of the undermentioned identified requirements:

| Title / Name: | Drawing No / Document Ref | Revision / Issue: | Date [dd.mm.yyyy]: | Prepared by: |
| --- | --- | --- | --- | --- |
| Architectural |  |  |  |  |
| Cover Sheet | Sheet 01 | Rev B | 24.02.2022 | DTA Architects |
| Block Analysis Plan | Sheet 02 | Rev B | 24.02.2022 | DTA Architects |
| Site Analysis Plan | Sheet 03 | Rev B | 24.02.2022 | DTA Architects |
| Demolition Plan | Sheet 04 | Rev B | 24.02.2022 | DTA Architects |
| Development Data | Sheet 05 | Rev B | 24.02.2022 | DTA Architects |
| Site Plan | Sheet 06 | Rev B | 24.02.2022 | DTA Architects |
| Ground Floor Plan | Sheet 07 | Rev B | 24.02.2022 | DTA Architects |
| First Floor Plan | Sheet 08 | Rev B | 24.02.2022 | DTA Architects |
| Roof Plan | Sheet 09 | Rev B | 24.02.2022 | DTA Architects |
| Elevations | Sheet 10 | Rev B | 24.02.2022 | DTA Architects |
| Elevations | Sheet 11 | Rev B | 24.02.2022 | DTA Architects |
| Elevations – Street view | Sheet 12 | Rev B | 24.02.2022 | DTA Architects |
| Sections | Sheet 13 | Rev B | 24.02.2022 | DTA Architects |
| Solar Access – View from Sun | Sheet 14 | Rev B | 24.02.2022 | DTA Architects |
| Solar Access – View from Sun | Sheet 15 | Rev B | 24.02.2022 | DTA Architects |
| Shadow Diagrams | Sheet 16 | Rev B | 24.02.2022 | DTA Architects |
| Perspectives | Sheet 17 | Rev B | 24.02.2022 | DTA Architects |
| Landscape |  |  |  |  |
| Landscape Plan | Sheet 1 | Rev P7 | 07.02.2022 | RFA Landscape Architects |
| Civil Design |  |  |  |  |
| Specifications and Site Drainage Plan | Sheet 1 of 3 | Rev B | 20.10.2021 | MSL Consulting Engineers |
| Stormwater Details and Sections | Sheet 2 of 3 | Rev C | 17.02.2022 | MSL Consulting Engineers |
| Sediment and Erosion Control Plan | Sheet 3 of 3 | Rev B | 20.10.2021 | MSL Consulting Engineers |
| Cut and Fill Plan | Sheet 1 of 1 | Rev C | 14.03.2022 | MSL Consulting Engineers |
| Hydraulic Plan |  |  |  |  |
| Hydraulic Services Plan Legend & Notes | Sheet 1 of 4 | Rev F | 02.2022 | DeAngelis Taylor and Associated Pty |
| Hydraulic Services Plan Site/Ground Floor Plan | Sheet 2 of 4 | Rev F | 02.2022 | DeAngelis Taylor and Associated Pty |
| Hydraulic Services Plan First Floor/Roof Plan | Sheet 3 of 4 | Rev F | 02.2022 | DeAngelis Taylor and Associated Pty |
| Hydraulic Services Plan Roof Plan | Sheet 4 of 4 | Rev F | 02.2022 | DeAngelis Taylor and Associated Pty |
| BASIX/NatHers |  |  |  |  |
| BASIX Certificate | No. 1218683M_02 | 02 | 03.03.2022 | LC Consulting Engineers |
| Specialist Reports |  |  |  |  |
| Arboricultural Impact Assessment Report | Ref 6791 |  | 29.10.2021 | Redgum Horticultural |
| Residential Site Investigation Report | Ref. 36151A_GR01 | A | 07.04.2021 | Barnson Pty Ltd |
| Report relating to Class 1A single storey dwelling | Ref 020-014009 |  | 01.11.2021 | Philip Chin Building Compliance |
| Report relating to Class 2 two storey building | Ref 020-014009 |  | 01.11.2021 | Philip Chin Building Compliance |
| Access Report | Ref. 21228-R2.1 |  | 18.11.2021 | Access-i |
| Traffic Impact Assessment Report |  |  | 26.10.2021 | Park Transit |
| Waste Management Plan |  |  | 29.10.2021 | DTA Architects |
| Heritage Impact Statement |  |  | 11.2021 | Ian Rufus |

- All building work is to be undertaken in accordance with the National Construction Code and referenced Australian Standards.

- All commitments listed in the BASIX certificate and stamped plans shall be complied with.

- All construction documentation and building work is to be certified in accordance with Section 6.28 of the Environmental Planning and Assessment Act 1979.

- The land the subject of this approval must be consolidated into a single lot.  The plan of consolidation must be lodged at the NSW Land Registry Services and be registered prior to occupation of the development.  A copy of the registered plan is to be provided to the Land & Housing Corporation.

OPERATIONAL MATTERS

These identified requirements pertain to the use of the site and have been imposed to ensure that the development and its operation do not interfere with the amenity of the surrounding area.

Stormwater Run-off

- Stormwater is to be collected within the site, and conveyed in a pipeline to the appropriate gutter or drain under the control of the council for the area, substantially in accordance with the approved concept stormwater drainage plans.

- Alterations to the natural surface contours or surface absorption characteristics must not impede, increase or divert natural surface water runoff, so as to cause a nuisance to adjoining property owners.

- All driveways are to be graded in such a manner as to provide continuous surface drainage flow paths to appropriate points of discharge.

- To prevent water from entering buildings surface waters shall be collected and diverted clear of the buildings by a sub-surface / surface drainage system.

Vehicular Access & Parking

- A concrete vehicular crossing(s) and layback(s) must be provided at the entrance(s) / exit(s) to the property. The crossing(s) and layback(s) must be constructed in accordance with the council for the area’s standard requirements for residential crossings. Council shall be provided with plans for the crossing(s) and layback(s) together with the payment of any council inspection fees.

- Particular care should be taken in the location of vehicular crossings and/or laybacks to avoid poles, pits etc.  The cost of any necessary adjustments to utility mains and services associated with the construction of the layback / driveway is to be borne by the Land & Housing Corporation.  Obsolete gutter laybacks are to be constructed as kerb in accordance with the respective council’s standards.

Note: It is recommended that discussion be held with the relevant authorities before construction works commence.

- Car parking spaces and driveways must be constructed of concrete or other approved hard surfaced materials.  The spaces must be clear of obstructions and columns, permanently line marked and provided with adequate maneuvering facilities.  The design of these spaces must comply with AS 2890.1.

Site Works

- All soil erosion and sediment control measures required to be put in place prior to the commencement of demolition / construction works are to be maintained during the entire period of the works until disturbed areas are restored by turfing, paving or revegetation. Soil erosion and sediment control measures shall be designed in accordance with the guidelines set-out in the Blue Book Managing Urban Stormwater: Soils and Construction (4th edition, Landcom, 2004).

- An appropriately qualified person shall design retaining walls or other methods necessary to prevent the movement of excavated or filled ground, including associated stormwater drainage measures.

Building Siting

- All buildings are to be sited well clear of any easements affecting the site. The builder shall ascertain if any easements do exist and if so obtain full details of such prior to construction commencing.

Smoke Detection System(s)

- Smoke detection systems shall be installed throughout the building(s) in accordance with requirements of Clause E1.7 of the Building Code of Australia.  Detectors shall comply with AS 3786 and must:

- be connected to a permanent 240V power supply; and

- be provided with a battery backup to activate the alarm unit in the event of failure of the permanent power supply.

Site Soil Contamination

- If the site is identified as being potentially affected by soil contamination, it is to be inspected by a suitably qualified person to identify any contaminated or hazardous material present.  A proposal for remediation is to be prepared, which may include the preparation of a Remedial Action Plan, and remediation is to be carried out in accordance with the plan.  A Validation Report, prepared in accordance with Environment Protection Authority requirements, is to be obtained from a qualified expert on completion of the remediation work to verify that the site is suitable for the intended residential use.  A copy of the Validation Report is to be provided to the Land & Housing Corporation on completion of the remediation works.

Landscaping

- Landscaping is to be carried out substantially in accordance with the approved landscape plan(s) and maintained for a period of 12 months by the building contractor.  The council for the area is to be consulted in relation to the planting of any street trees along the site frontage(s).

- All scheduled plant stock shall be pre-ordered, prior to commencement of construction or 3 months prior to the commencement of landscape construction works, whichever occurs sooner, for the supply to the site on time for installation.  The builder shall provide written confirmation of the order to Council and provide a copy to the Land & Housing Corporation.

Tree Removal

- Removal of trees within the boundaries of the site is to be carried out in accordance with the trees shown for removal on the approved landscape plan and Arborist report and no other trees are to be removed without further approval. Trees identified for retention on the approved landscape plan and Arborist report are to be retained.

Fencing

- All front fencing and gates are to be constructed wholly within the boundaries of the site.  All gates associated with the fencing are to swing inwards towards the site.

Provision of Letterbox Facilities

- Suitable letterbox facilities shall be provided in accordance with Australia Post specifications.

Public Liability Insurance

- A valid public liability insurance policy of at least $10M shall be maintained throughout the demolition / construction works by the contractor.

PRIOR TO ANY WORK COMMENCING ON THE SITE

The following identified requirements are to be complied with prior to any work commencing on the site.

Long Service Levy

- The long service levy shall be paid by the Land & Housing Corporation to the Long Service Payments Corporation in accordance with the Building & Construction Industry Long Service Payments Act 1986 prior to the commencement of any building and construction works.

Note:

Where approval has been granted for development by a Crown body other than a council, the amount payable is 0.35% of the contract inclusive of GST.

(Deleted)

Service Authority Clearances

- An application for a compliance certificate is to be made to the relevant water utility (e.g. the local council for the area or Hunter Water or Sydney Water) or other evidence of the utility’s non-objection to the commencement of work on the basis of service availability is to be obtained prior to work commencing.

Note:

Payment of water and/or sewer service charges and/or a notice of requirements for works to be carried out during construction / prior to occupation may be applicable prior to issue of the compliance certificate.

- A written clearance from an electricity supply authority stating that electrical services have been made available to the development or that arrangements have been entered into for the provision of services to the development is to be obtained prior to work commencing.

- A certificate from an approved telecommunications carrier to certify that satisfactory arrangements have been made to ensure the provision of underground telephone services to the site and to each dwelling in the development is to be obtained prior to work commencing.

- Where the site is to be connected to reticulated gas, a certificate from an approved gas carrier to certify that satisfactory arrangements have been made to ensure the provision of underground gas services to the site and to each dwelling in the development is to be obtained prior to work commencing.

Utilities Service Provider Notification

- The demolition / construction plans must be submitted to the appropriate water utility’s office (eg Sydney Water office) to determine whether the development will affect the utility’s sewer and water mains, stormwater drains and/or easements. If the development complies with water utility’s requirements, the plans will be stamped indicating that no further requirements are necessary.

Disconnection of Services

- All previously connected services that are required to be disconnected are to be appropriately disconnected and made safe prior to commencement of the demolition / construction works. The various service authorities shall be consulted regarding their requirements for the disconnection of services.

- All existing services within the boundary to remain live must be identified, pegged and made safe.

Demolition / Site Clearance

- The builder is to notify the occupants of premises on either side, opposite and at the rear of the development site a minimum of two days prior to demolition/site clearance.  Such notification is to be clearly written on A4 size paper giving the date demolition/site clearance will commence and be placed in the letterbox of every premise (including every unit of a multi-unit residential building or mixed use building if applicable).  The demolition/site clearance must not commence prior to the date which has been stated in the notice letter.

- Prior to the demolition/site clearance, a Work Plan shall be prepared by a competent person(s) and shall be submitted to the Land & Housing Corporation.  The Work Plan shall outline the identification of any hazardous materials, including surfaces coated with lead paint, method of demolition/site clearance, the precautions to be employed to minimise any dust nuisance and the disposal methods for hazardous materials.

Stormwater Disposal

- A detailed stormwater drainage plan(s), substantially in accordance with the approved concept stormwater drainage plan(s), is to be prepared and submitted to the Land & Housing Corporation.  Any on-site detention system shall be designed in accordance with any catchment authority requirements (eg the Upper Parramatta River Catchment Trust On-site Detention Handbook) and/or the council for the area’s drainage code.

- Proof of lodgement of any plan of a required drainage easement at the NSW Land Registry Services must be submitted to the Land & Housing Corporation prior to commencement of works.  Registration of the plan of easement must be completed prior to occupation of the development and a copy of the registered plan is to be provided to the Land & Housing Corporation.

Council Notification

- The council for the area shall be advised in writing, of the date it is intended to commence work, including demolition/site clearance.  A minimum period of 5 working days notification shall be given.

Landfill

- Where site filling is necessary, a minimum of 95% standard compacting must be achieved and certified by a NATA registered Soils Laboratory.

- Land fill materials must satisfy the following requirements:

- be Virgin Excavated Natural Matter (VENM);

- be free of slag, hazardous, contaminated, putrescible, toxic or radio-active matter; and

- be free of industrial waste and building debris.

Site Facilities

- The following facilities shall be installed on the site:

- Toilet facilities shall be provided at the rate of 1 toilet for every 20 persons, or part thereof, employed at the site.  Each toilet provided shall be a standard flushing toilet and shall be connected to a public sewer or if connection to a public sewer is not practicable, to an accredited sewerage management facility provided by the council for the area or if this is not practicable to some other council approved management facility.

- Adequate refuse disposal methods and builders storage facilities.  Builders' wastes, materials or sheds are not to be placed on any property other than that which this approval relates to.

- Access to the site is to be provided only via an all-weather driveway on the property and is not to be provided from any other site.

Site Safety

- A sign must be erected in a prominent position on any site on which demolition or building work is being carried out:

- showing the name, address and telephone number of the responsible Land & Housing Corporation officer for the work, and

- showing the name of the principal contractor (if any) and a telephone number on which that person may be contacted during and outside working hours, and

- stating that unauthorised entry to the work site is prohibited.

Any such sign is to be maintained while the work is being carried out, but must be removed when the work has been completed.

Note:

This requirement does not apply in relation to building work that is carried out inside an existing building that does not affect the external walls of the building.

- A minimum 1.8m high security fence or a  Class A (minimum 1.8m high security fence) or Class B (overhead) hoarding must be erected between the work site and any public place prior to demolition / construction.  Access to the site shall be restricted to authorised persons only and the site shall be secured against unauthorised entry when demolition / construction work is not in progress or the site is otherwise unoccupied.

Note:

Approval from the relevant roads authority will be required under Section 138 of the Roads Act 1993 where a Class A or B hoarding encroaches onto the footpath of / or a public thoroughfare within a classified road.

- Building materials are not to be stored on the council for the area’s footpath or roadway.

Waste Management

- A final Waste Management Plan is to be prepared and submitted to the Land & Housing Corporation prior to the commencement of demolition / construction.  The plan shall detail the amount of waste material and the destination of all materials, recyclable and non-recyclable, including provision for adequate receptacle(s) to store all waste generated by the development, pending disposal.

DURING DEMOLITION / SITE CLEARANCE / CONSTRUCTION

The following identified requirements are to be complied with whilst works are occurring on the site.

Heritage

- Historic and indigenous archaeological sites and relics are protected under the Heritage Act 1977 and National Parks and Wildlife Act 1974 respectively. Should any relics be uncovered during the course of the approved works, work must cease in the affected area.  Subsequently, in cases where historical items have been uncovered, the Department of Planning, Industry and Environment must be contacted or if indigenous items have been uncovered the Department of Environment, Energy and Science must be contacted.

- All workers / contractors must be informed of their obligations under the National Parks and Wildlife Act 1974, namely that it is illegal to disturb, damage or destroy a relic without the prior approval of the Coordinator-General of the Department of Environment, Energy and Science.

Demolition

- Any existing structures are to be demolished/removed prior to commencement of construction of the approved development.

- Demolition is to be carried out in accordance with the appropriate provisions of AS 2601.

- Where materials containing asbestos are to be removed, the removal is to be carried out by a licensed contractor(s) who has a current SafeWork NSW accreditation in asbestos removal.

- Removal of asbestos-based thermal or acoustic insulation, such as sprayed asbestos and asbestos-based lagging, including friable asbestos boards, shall be carried out in accordance with the National Occupational Health and Safety Commission’s Code of Practice for the Safe Removal of Asbestos, 2nd Edition [NOHSC:2002 (2005)].

- Hazardous or intractable wastes, including all asbestos laden waste, arising from the demolition/site clearance process shall be removed and disposed of in accordance with the requirements of SafeWork NSW and the Department of Environment, Energy and Science.

- Documentary evidence in the form of tip receipts from an approved Waste Management Facility shall be obtained by the demolition/site clearance contractor and submitted to the Land & Housing Corporation demonstrating the appropriate disposal of the asbestos waste.

- Demolition/site clearance procedures shall maximise the reuse and recycling of materials from the site in order to reduce the environmental impacts of waste disposal.

- During demolition/site clearance, the public footway and public road shall be kept clear at all times and shall not be obstructed by any demolished material or vehicles.  The public road and footway shall be swept (not hosed) clean of any material, including clay, soil and sand.

- All vehicles leaving the site with demolition/site clearance materials shall have their loads covered and vehicles shall not track soil and other material onto the public roads and footways and the footway shall be suitably protected against damage when plant and vehicles access the site.  All loading of vehicles with demolished materials shall occur on site.

- Care shall be taken during demolition/site clearance to ensure that existing services on the site (ie sewer, electricity, gas, and telecommunications) are not damaged.

Survey Reports

- Survey reports are to be submitted to the Land & Housing Corporation prior to the placement of the footings / slab and on completion of the dwellings to verify the correct position of the structures in relation to the boundary allotments in accordance with this determination.

Hours of Demolition / Construction / Civil Work

- Demolition / site clearance / construction / civil work is only permitted on the site between the hours of 7.00am to 5.00pm Monday to Saturday with no work permitted on Sundays or public holidays.

Excavation & Backfilling

- All excavations and backfilling associated with site clearance and the erection of a building must be executed safely and in accordance with appropriate professional standards. All such work is to be guarded and protected to prevent it from being dangerous to life and property.

Pollution Control

- Any noise generated during the construction of the development shall not exceed the limits specified in the July 2009 Interim Construction Noise Guidelines, published by the Department of Environment and Climate Change (now Department of Environment, Energy and Science).

- No fires are to be lit or waste materials burnt on the site.

- No washing of concrete forms or trucks is to occur on the site.

- Any contamination / spills on the site during construction works must be actively managed and reported immediately to appropriate regulatory authorities to minimise any potential damage to the environment.

- Dust generation during demolition / site clearance / construction shall be controlled using regular control measures such as on site watering or damp cloth fences.

- All vehicles transporting loose materials and travelling on public roads must be secured (ie closed tail gate and covered) to minimise dust generation.

- Spraying of paint and other materials with the potential to become air borne particulates must only be undertaken in light wind conditions.

- Non-recyclable waste and other waste containers must be regularly collected and disposed of at a licensed landfill or other disposal site in accordance with details set out in the final Waste Management Plan to be prepared and submitted to the Land & Housing Corporation prior to the commencement of demolition / construction.

Impact of Construction Works

- The Land & Housing Corporation shall bear the cost of any necessary adjustments to utility mains and services.

- Care shall be taken to prevent any damage to adjoining properties.  The building contractor may be liable to pay compensation to any adjoining owner if, due to construction works, damage is caused to such adjoining property.

PRIOR TO OCCUPATION OF THE DEVELOPMENT

The following identified requirements are to be complied with prior to the occupation of the development.

General

- The occupation of the development shall not commence until all the identified requirements of this determination have been complied with.

Termite Protection

- To protect buildings from subterranean termite, termite barriers installed in accordance with AS 3660.1 are required on the underside and penetrations of the concrete slab floor.

In addition, a durable notice must be permanently fixed inside the meter box indicating:

- The method of protection.

- The date of installation of the system.

- Where a chemical barrier is used, its life expectancy as listed on the National Registration Authority label.

- The need to maintain and inspect the system on a regular basis.

Road Damage

- The cost of repairing any damage caused to the council for the area’s assets in the vicinity of the subject site as a result of demolition / site clearance / construction works associated with the approved development is to be met in full by the building contractor.

Stormwater Drainage

- Prior to occupation of the development, a Work As Executed Plan must be prepared by the Building Contractor clearly showing all aspects of the constructed stormwater drainage system, including any on-site detention system.  The plan must demonstrate general compliance with the approved concept stormwater drainage plan(s) and must include:

  - Sufficient levels and dimensions to verify the constructed storage volume.

  - Location and surface levels of all pits.

  - Invert levels of the internal drainage line, orifice plates fitted and levels within the outlet control pit.

  - Finished floor levels of all structures.

  - Verification that any required trash screens have been installed.

  - Locations and levels of any overland flow paths.

  - Verification that any drainage lines are located wholly within easements.

The Work-As-Executed Plan information should be shown on a copy of the final civil works drawings.

A positive covenant and restriction-as-to-user must be placed over the onsite detention system (if any) in accordance with the council for the area’s on-site detention policy to ensure that system will be adequately maintained.  The positive covenant and restriction-as-to-user must be registered at NSW Land Registry Services prior to occupation of the development.  A copy of the registered restriction-as-to-user is to be provided to the Land & Housing Corporation and the council for the area.

PART B – Additional identified requirements

Site Specific Requirements

- Site Investigation Report

The recommendations contained in the Site Investigation Report dated 7.04.2021 prepared by Barnson must be implemented, including but not limited to Building Foundation and Construction. In particular, during foundation construction, should the subsurface conditions vary to those inferred in the Site Investigation Report, a suitably experienced geotechnical engineer should review the design and recommendations given in the Report to determine if any alterations are required.

Requirements resulting from consideration of the council for the area’s notification response in the REF

Nil requirements

Requirements as requested by public authorities other than councils

Nil requirements

Requirements resulting from consideration of adjoining occupier responses to notification in the REF / consideration of other public submissions received

Nil requirements

Specific service / utility agency requirements

Nil requirements

ADVISORY NOTES

- Approval of this development activity does not imply or infer compliance with Section 23 of the Disability Discrimination Act 1992.  Refer to AS 1428.1 and the Building Code of Australia for detailed guidance.

- Information regarding the location of underground services may be obtained from Dial Before You Dig at www.1100.com.au or by dialling 1100.