REVIEW OF ENVIRONMENTAL FACTORS

Proposed Seniors Housing Development

at

18-22 Mundamatta Street, Villawood NSW 2163

August 2023

Acknowledgement of Country

The Department of Planning and Environment acknowledges that it stands on Aboriginal land. We acknowledge the Traditional Custodians of the land and we show our respect for Elders past, present and emerging through thoughtful and collaborative approaches to our work, seeking to demonstrate our ongoing commitment to providing places in which Aboriginal people are included socially, culturally and economically.

Published by NSW Department of Planning and Environment

dpie.nsw.gov.au

REVIEW OF ENVIRONMENTAL EFFECTS

First published: August 2023

Department reference number: SUB23/136254

Department of Planning & Environment

Land & Housing Corporation

Postal address: Locked Bag 5022, Parramatta, NSW 2124

W www.dpie.nsw.gov.au

T (02) 9377 6000

ABN 24 960 729 253

This document may only be used for the purposes associated with the subject activity to which this Part 5 Review of Environmental Factors relates and to the extent authorised under the provisions of the Environmental Planning and Assessment Act 1979 and Environmental Planning and Assessment Regulation 2021, for the express purposes set out under that legislation, and may not otherwise be copied, reproduced, distributed or used without the written permission of the authors.

DOCUMENT CONTROL REGISTER

The following register documents the preparation and issue of this Part 5 Review of Environmental Factors (REF), prepared by Mecone Group for the New South Wales Land & Housing Corporation.

| No | Date | Version | Change since last version |
| --- | --- | --- | --- |
| 1 | 20 March 2023 | v1 (draft) | Not applicable – initial version |
| 2 | 17 May 2023 | v2 | Updates following LAHC review |
| 3 | 12 July 2023 | Final | LAHC finalisation |

DOCUMENT SIGN-OFF

| REF Prepared & Peer Reviewed by: |  |  |
| --- | --- | --- |
| Having prepared the Review of Environmental Factors: - I have declared any possible conflict of interests (real, potential or perceived) to the Acting Head of Policy and Innovation. - I do not consider I have any personal interests that would affect my professional judgement. - I will inform the Acting Head of Policy and Innovation as soon as I become aware of a possible conflict of interest. |  |  |
| Name: | Georgia Sedgmen |  |
| Designation: | Director |  |
| Signature: |  | Date: 07 June 2023 |
| Peer reviewed by: |  |  |
| Name: | Susanna Cheng |  |
| Designation: | Team Leader, NSW Land & Housing Corporation |  |
| Signature: | Susanna Cheng | Date: 13 June 2023 |

| REF Reviewed & Endorsed by: |  |
| --- | --- |
| I certify that I have reviewed this Review of Environmental Factors: Name: Carolyn Howell Designation: Manager, Portfolio Services, NSW Land & Housing Corporation |  |
| Signature: | Date: 12 July 2023 |

| REF Authorised for Issue by: |  |
| --- | --- |
| Having endorsed the Review of Environmental Factors: 1. I have declared any possible conflict of interests (real, potential or perceived) to the Acting Head of Policy and Innovation. 2. I do not consider I have any personal interests that would affect my professional judgement. 3. I will inform the Acting Head of Policy and Innovation as soon as I become aware of a possible conflict of interest. Name: Yolanda Gil Designation: Director, Portfolio Services, NSW Land & Housing Corporation |  |
| Signature: | Date: 10 August 2023 |

Contents

1	Executive Summary	7

2	Introduction	8

2.1	Summary of Proposed Activity	8

2.2	Demolition	11

2.3	Removal of Trees	11

2.4	Proposed Dwellings	12

3	Existing Site & Locality	16

3.1	Existing Site and Immediately Adjoining Development	16

3.2	Site Description	18

3.3	Neighbouring Development and Locality	18

4	Zoning and Permissibility	21

5	Planning and Design Framework	22

5.1	State Legislation	22

5.2	Local Planning Controls	42

6	Notification, Consultation and Consideration of Responses	45

6.1	Council Notification	45

6.2	Notification of Occupiers of Adjoining Land and Other Persons	49

6.3	Notification of Specified Public Authorities	49

7	Review of Environmental Factors	50

7.1	Neighbourhood Character	50

7.2	Bulk and Density	50

7.3	Streetscape	51

7.4	Visual Impact	51

7.5	Privacy	51

7.6	Solar Access	52

7.7	Overshadowing	52

7.8 	Traffic & Parking	52

7.8	Flora and Fauna	52

7.9	Heritage (European / Indigenous)	53

7.10	Soils / Contamination / Acid Sulfate Soils / Salinity	54

7.11	Drainage / Flood Prone Land / Hydrology/ Water Quality	54

7.12	Bushfire Prone Land	55

7.13	Noise and Vibration	55

7.14	Air Quality	56

7.15	Waste Minimisation	56

7.16	Resource Use & Availability	57

7.17	Community / Social Effects	57

7.18	Economic Impact	58

7.19	Cumulative Impact Assessment	58

8	Conclusion	59

8.1	Summary of Key Issues Raised in Assessment	59

9	Recommendation	59

List of Figures

Figure 1 Extract from Architectural Plans – Site Plan	13

Figure 2 Extract from Architectural Plans – General Arrangement Plan – Ground Level	14

Figure 3 Extract from Architectural Plans – General Arrangement Plan – First Level	15

Figure 4 Extract from Architectural Plans – Mundamatta Road Streetscape Perspective	15

Figure 5 Location Plan	16

Figure 6 Street Perspective of 18-20 Mundamatta Street, Villawood	17

Figure 7 Street Perspective of 20-22 Mundamatta Street, Villawood	17

Figure 8 Adjoining Development to the North – 24 Mundamatta Street	19

Figure 9 Adjoining Development to the South – 16 Mundamatta Street	19

Figure 10 Street view to 25 & 27 Mundamatta Street, Villawood	20

Figure 11 Street view to 8 & 8a Mundamatta Street, Villawood	20

Figure 12 Land Zoning Map	21

Figure 13 Map of Properties Notified of the Proposed Development	49

List of Tables

Table 1 Compliance with subsection 3 of section 5.5 of the EP&A Act 1979	22

Table 2 Compliance with section 171 of the EPA Regulations 2021	23

Table 3 Factors to be taken into account concerning the impact of an activity on the environment	23

Table 4 Factors to be taken into account concerning activities in regulated catchments	25

Table 5 Compliance with relevant provisions under sections Part 5, Division 8 of the SEPP for ‘seniors housing development without consent’ carried out by LAHC	28

Table 6 Seniors Living Urban Design Guidelines departures	30

Table 7 Good Design for Social Housing – Relevant Goals & Principles	31

Table 8 Response to Design Principles (Part 5, Division 8)	32

Table 9 Minimum site size, site frontage and building height standards [section 84]	35

Table 10 Accessibility and useability standards [Schedule 4]	35

Table 11 Internal design and fittings standards [Schedule 4]	37

Table 12 Non-discretionary standards for self-contained dwellings	40

Table 13 Compliance with other applicable State and Environmental Planning Policies	41

Table 14 Canterbury Bankstown Local Environmental Plan 2023	42

Table 15 Canterbury Bankstown Development Control Plan 2023	43

Table 16 Issues raised in Council submission	46

Executive Summary

The subject site is located at 18-22 Mundamatta Street, Villawood, and is legally described as Lots 814, 815 & 816 in Deposited Plan 36608.

The proposed seniors housing development is described as follows:

- Demolition of existing dwellings and structures, removal of trees, and the construction of a seniors housing development comprising of 12 independent living seniors housing units (7 x 1 bedroom and 5 x 2 bedroom units) with associated landscaping and fencing, surface parking for 6 cars, consolidation of 3 lots into a single lot and construction of a stormwater drainage pipeline and easement over Lot 807 DP 36608, No. 82 Lowana Street, Villawood.

The proposed activity is located in a prescribed zone, and seniors housing can therefore be carried out by the NSW Land and Housing Corporation (LAHC) without consent under the provisions of Part 5, Division 8 of the State Environmental Planning Policy (Housing) 2021 (Housing SEPP) as it does not result in more than 40 dwellings on the site and does not exceed 9.5 metres in height.

Demolition has been considered as part of the proposed activity. Demolition is permitted with consent under the provisions of the applicable local environmental planning instrument and is therefore permitted without consent under the provisions of the Housing SEPP.

The removal of trees on the site is covered by the definition of consent under section 6 of the Housing SEPP. It has therefore been incorporated in this review of environmental factors under Part 5 of the Environmental Planning and Assessment Act 1979 (EP&A Act) and Part 8 of the Environmental Planning and Assessment Regulation 2021 (EP&A Regulation).

The REF demonstrates the following:

- from an analysis of the potential environmental impacts associated with the proposed activity, it has been concluded that the preparation of an Environmental Impact Statement is not required;

- based on a review of the potential environmental impacts resulting from the proposed activity it has been determined that, subject to implementation of mitigation measures to be incorporated as identified requirements, the activity will not have any significant adverse impact on the environment;

- the proposed activity will not have any effect on matters of national significance and its approval under the Commonwealth Environment Protection and Biodiversity Conservation Act 1999 is not required;

- the design of the proposed activity has adequately taken into account design principles and better practices set out in the Seniors Living Policy: Urban Design Guidelines for Infill Development and taken into consideration Good Design for Social Housing and LAHC’s Design Requirements;

- the site planning and design of the proposed activity adequately address the applicable local environmental planning and development controls of Canterbury Bankstown Council;

- a BASIX certificate and NatHERS certificate and stamped plans have been submitted for the proposed activity demonstrating compliance with the State Government’s environmental sustainability targets;

- there are no separate approvals, authorisations or notifications required in relation to the proposed activity prior to determination under Part 5 of the EP&A Act or under any other Acts; and

- Canterbury Bankstown Council and occupiers of adjoining land were notified of the proposed activity under the provisions of Housing SEPP. A response was received from Council dated 16 December 2022. Comments on the response are provided in section 6.1 of this REF. No submissions were received from occupiers of adjoining land.

The proposed activity, when carried out in accordance with the environmental mitigation measures outlined in the REF, will not result in any significant and long-term negative impacts on the environment and can proceed subject to the implementation of the Identified Requirements of determination.

Introduction

This Review of Environmental Factors (REF) under Part 5 of the Environmental Planning and Assessment Act 1979 (EP&A Act) is for an activity involving the demolition of existing dwellings and structures, removal of trees, and the construction of a seniors housing development comprising of 12 independent living seniors housing units (7 x 1 bedroom and 5 x 2 bedroom units) with associated landscaping and fencing, surface parking for 6 cars, and consolidation of 3 lots into a single lot and construction of a stormwater drainage pipeline and easement over Lot 807 DP 36608, No. 82 Lowana Street, Villawood. The activity* will be carried out by, or on behalf of, NSW Land and Housing Corporation (LAHC) and is ‘development without consent’ under the Housing SEPP.

This REF has been prepared by Mecone Group Pty Limited on behalf of LAHC in satisfaction of the provisions of Part 5 of the Environmental Planning & Assessment Act 1979 (EP&A Act) and Part 8 of the Environmental Planning & Assessment Regulation (EP&A Regulations) 2021.

A Statement of Compliance accompanying this REF certifies that in accordance with the requirements of the EP&A Act, all matters affecting or likely to affect the environment by reason of the proposed activity have been taken into account to the fullest extent possible and the activity will not have a significant impact on the environment.

* Note: The proposed development is permitted without consent and is therefore subject to environmental impact assessment as an ‘activity’ under Part 5 of the Environmental Planning and Assessment Act 1979.

Summary of Proposed Activity

The proposed activity involves demolition of existing dwellings and structures, removal of trees, and the construction of a seniors housing development comprising of 12 independent living seniors housing units (7 x 1 bedroom and 5 x 2 bedroom units) with associated landscaping and fencing, surface parking for 6 cars, and consolidation of 3 lots into a single lot and construction of a stormwater drainage pipeline and easement over Lot 807 DP 36608, No. 82 Lowana Street, Villawood.

The proposal is detailed in the following plans, drawings and specialist reports and supporting information:

Plans, Drawings & Supporting Document Details

The development is outlined in the following plans and drawings:

| Title / Name: | Drawing No. / Document Ref | Revision / Issue: | Date [dd.mm.yyyy]: | Prepared by: |
| --- | --- | --- | --- | --- |
| Architectural – Appendix A |  |  |  |  |
| Coversheet | DA-0.00 | 8 | 14 February 2023 | Custance |
| Site Analysis | DA-0.11 | 8 | 17 November 2022 | Custance |
| Site Analysis | DA-0.12 | 7 | 17 November 2022 | Custance |
| Site Plan | DA-0.20 | 9 | 22 November 2022 | Custance |
| GA – Ground | DA-1.20 | 12 | 14 February 2023 | Custance |
| GA – Level 1 | DA-1.21 | 9 | 17 November 2022 | Custance |
| GA - Roof | DA-1.22 | 8 | 17 November 2022 | Custance |
| Shadow Diagrams | DA-1.65 | 7 | 17 November 2022 | Custance |
| Shadow Diagrams | DA-1.66 | 7 | 17 November 2022 | Custance |
| Eyes from the Sun | DA-1.68 | 4 | 17 November 2022 | Custance |
| Eyes from the Sun | DA-1.69 | 4 | 17 November 2022 | Custance |
| Eyes from the Sun | DA-1.70 | 4 | 17 November 2022 | Custance |
| External Elevations | DA-3.00 | 8 | 14 February 2023 | Custance |
| Building Sections | DA-4.00 | 7 | 17 November 2022 | Custance |
| 3D Images & Streetscape Elevation | DA-6.00 | 7 | 17 November 2022 | Custance |
| Area Calculations | DA-11.00 | 7 | 17 November 2022 | Custance |
| Landscape – Appendix B |  |  |  |  |
| Cover Sheet | LA00 | - | 26 August 2022 | Taylor Brammer Landscape Architects |
| Ground Floor Landscape Plan | LA01 | F | 25 November 2022 | Taylor Brammer Landscape Architects |
| Existing Tree Retained and Removed | LA02 | F | 25 November 2022 | Taylor Brammer Landscape Architects |
| Landscape Details | LD01 | A | 15 September 2022 | Taylor Brammer Landscape Architects |
| Landscape Details | LD02 | B | 21 October 2022 | Taylor Brammer Landscape Architects |
| Environmental Site Management – Appendix U |  |  |  |  |
| Notes & Legend | ESM1 | 4 | 14 November 2022 | JN Engineering Consultants |
| Typical Details | ESM2 | 4 | 14 November 2022 | JN Engineering Consultants |
| ESM Site Plan | ESM3 | 6 | 31 January 2023 | JN Engineering Consultants |
| Stormwater Services – Appendix C |  |  |  |  |
| Notes & Legend | C001 | 4 | 14 November 2022 | JN Engineering Consultants |
| Site Plan | C010 | 6 | 14 November 2022 | JN Engineering Consultants |
| Typical Details | C050 | 7 | 31 January 2023 | JN Engineering Consultants |
| OSD Details | C051 | 1 | 31 January 2023 | JN Engineering Consultants |
| Ground Stormwater Plan | C200 | 9 | 31 January 2023 | JN Engineering Consultants |
| Level 1 Stormwater Plan | C210 | 4 | 14 November 2022 | JN Engineering Consultants |
| Roof Stormwater Plan | C300 | 2 | 14 November 2022 | JN Engineering Consultants |
| Survey – Appendix D and T |  |  |  |  |
| Detail and Level Survey | S Sheet 1 / 11 |  | 04 December 2021 | Norton Survey Partners |
| Mechanical Spatial – Appendix A |  |  |  |  |
| Mechanical Concept 1 / Cover Page | DA-0.00 | 1 | 23 June 2022 | Knox Advanced Engineering |
| Mechanical Concept 1 / GA–Ground | DA-1.20 | 1 | 23 June 2022 | Knox Advanced Engineering |
| Mechanical Concept 1 / GA–Level 1 | DA-1.21 | 1 | 23 June 2022 | Knox Advanced Engineering |
| BASIX and NatHERS – Appendix K and N |  |  |  |  |
| BASIX Certificate | 1318442M | - 02 | 18 May 2023 | JN Engineering Consultants |
| NatHERS Certificate | 0008210200 | - | 18 May 2023 | Gradwell Consulting |
| Arboricultural Plans – Appendix J |  |  |  |  |
| Cover Sheet | T-00 | A | 14 February 2023 | Arterra Design |
| Tree Retention Value Plan | T-01 | B | 14 February 2023 | Arterra Design |
| Tree Protection and Removal Plan | T-02 | B | 14 February 2023 | Arterra Design |
| Tree Protection Specifications | T-03 | A | 14 November 2022 | Arterra Design |
| Specialist Reports |  |  |  |  |
| Arboricultural Impact Assessment – Appendix J | AIA-01 | B | 14 February 2023 | Arterra Design |
| Access Report – Appendix H | 21353-DA | 1.2 | 22 November 2022 | Access-I |
| BCA Design Compliance Assessment – Appendix L | P220195 | 0 | 14 November 2022 | BCA Vision |
| Noise Intrusion Assessment – Appendix V | 20221154.2/0911A/R0/ANP | 0 | 9 November 2022 | Acoustic Logic |
| Geotechnical Investigation Assessment – Appendix P | 21/3539 | 0 | December 2021 | STS Geotechnics |
| Traffic Impact Assessment – Appendix S | 21.640r01 | v02 | 11 November 2022 | TRAFFIX |
| Waste Management Plan – Demolition and Construction Waste – Appendix R | - | - | - | LAHC |
| Waste Management Plan – Occupancy Stage – Appendix R | - | - | - | LAHC |

Design Compliance and hecklists – Appendix M

Architect’s Certificate of Building Design Compliance – Custance dated 03 April 2023

Certificate of Landscape Documentation Compliance – Aaron Lakeman dated 16 November 2022

Certificate of Civil Documentation Compliance – JN Engineering Consultants dated 16 November 2022

Certificate of Design Intent – National Construction Code – JN Engineering Consultants dated 18 October 2022

Mechanical Documentation Compliance – Knox Advanced Engineering dated 26 July 2022

Seniors Living Urban Design Guidelines Checklist – Appendix O

Seniors Living Policy: Urban design guidelines for infill development – Checklist, prepared by Custance dated 26 October 2022

Section 10.7 Planning Certificates – Appendix G

Planning Certificate, Certificate No 20234379 82 Lowana Street, Villawood – Issued by Canterbury Bankstown Council dated 11 July 2023

Planning Certificate, Certificate No 20234358 18 Mundamatta Street, Villawood – Issued by Canterbury Bankstown Council dated 10 July 2023

Planning Certificate, Certificate No 20234359 20 Mundamatta Street, Villawood – Issued by Canterbury Bankstown Council dated 10 July 2023

Planning Certificate, Certificate No 20234366 22 Mundamatta Street, Villawood – Issued by Canterbury Bankstown Council dated 10 July 2023

Titles and Deposited Plans – Appendix Q

Title Search, Folio: 807/36608, Search date 29 June 2022, First Schedule: New South Wales Land and Housing Corporation

Title Search, Folio: 814/36608, Search date 29 June 2022, First Schedule: New South Wales Land and Housing Corporation

Title Search, Folio: 815/36608, Search date 29 June 2022, First Schedule: New South Wales Land and Housing Corporation

Title Search, Folio: 816/36608, Search date 29 June 2022, First Schedule: New South Wales Land and Housing Corporation

- Deposited Plan 36608, Printed 1 November 2018

AHIMS – Appendix I

AHIMS Search Result, 18 Mundamatta Street, Villawood – 1000m search buffer, date 18 January 2023

Demolition

The proposed activity includes demolition of 2 single storey detached dwelling houses, associated structures, and crossovers as identified in the Demolition Plan (refer to Appendix A).

Refer to section 5.1.6 of this REF for further assessment of the proposed demolition works.

Removal of Trees

The proposal includes the removal of 21 trees, including 14 trees within the site, 3 street trees and 4 trees in the area of the proposed drainage easement at 82 Lowana Street.

The 14 trees within the site, the 4 trees in the area of the easement (82 Lowana Street) and one of the street trees are required to be removed to accommodate the proposed development (as discussed in the Arboricultural Impact Assessment in Appendix J).

Council has also advised they support removing all 3 street trees along the site’s frontage, subject to the planting of 6 replacement trees. See Council correspondence in Appendix F.

All of the trees to be removed have low or medium retention value.

The only high retention value trees, located within the site (Spotted Gum) and in the adjoining property at 82 Lowana Street (Lemon Scented Gum), are being retained.

For further detail, refer to the Arboricultural Impact Assessment in Appendix J.

More appropriate tree plantings, including 33 trees capable of reaching mature heights of 6m or greater, plus 6 new street trees in accordance with Council’s recommendations, will be provided as part of the proposed landscaping plan to compensate for the loss of these trees (refer to submitted Landscape Plan in Appendix B).

Proposed Dwellings

The proposed dwellings are 12 independent living units for the purpose of seniors housing across 2 levels – that is, 7 units on the ground floor comprising 3 x 1 bedroom and 4 x 2 bedroom units, and 5 units on the first floor comprising 4 x 1 bedroom and 1 x 2 bedroom units. The ground floor units have been designed as liveable units which are capable of being adapted to suit persons with a disability should the need arise in the future.

The proposed independent living units represent a contemporary, high-quality design, which will be an improvement on the existing single storey dwellings. The proposal uses a combination of face brick and metal cladding for the external walls as well as metal roof sheeting. The choice of this material is consistent with the existing and developing character of the Villawood suburb while also providing a modern style. Eight of the 12 proposed units will address Mundamatta Street (Units 1, 2, 3 & 4 on the ground floor and Units 8, 9, 10 & 11 on the first floor) with habitable rooms, kitchen windows and an upper-level balcony the street for passive surveillance.

Minor cut and fill is proposed to provide a level building platform, with less than 500mm across majority of the site and up to 930mm around the areas of Units 2 and 3. All cut and fill provisions are shown on the Cut & Fill Plans (refer to Appendix C).

A variety of new landscape plantings are proposed to offset the proposed tree removal and enhance the appearance of the site. New plantings will consist of a mixture of new trees, shrubs and groundcovers, which will enhance landscaped setbacks and contribute to the streetscape.

Each unit will be provided with its own enclosed private open space (POS) area. All these spaces are directly accessible from the living areas. All first floor units (8, 9, 10, 11 & 12) have POS in the form of balconies.

A total of 6 at grade car parking spaces will be provided on the site, including 2 accessible spaces and 1 adaptable space.

Stormwater will be collected via a series of stormwater pits and pipes connected to a below ground on-site detention (OSD) tank connected to an outlet pipe that will drain through 82 Lowana Street to the north east through a 1.2 metre easement and connect to a new kerb inlet pipe on Lowana Street, then connecting to an existing kerb inlet pit in front of 80 Lowana Street.  The construction of the outlet pipe through 82 Lowana Street doe not impact on the existing dwelling on this site.  Four trees (of low retention value as per the Arboricultural Impact Assessment Report) and low scale landscaping at 82 Lowana Street are to be removed to enable trenching and connection for stormwater to Lowana Street.

Roof water will be collected from downpipes that will connect to the gutters that flow into the underground rainwater tank for recycling with overflow connected to the underground detention tank.

Colorbond fencing is proposed along the side and rear boundaries at a 1.8m in height. Metal palisade privacy fencing is proposed in the private open space patios of the ground floor units at a 1.8m height. An Identified Requirement (No. 80) has been applied requiring the palisades around the patios to be spaced and angled in a manner that provides for privacy to occupants.

Fencing along the Mundamatta Street frontage will be a combination of brick and metal palisade, with brick used around the bin storage areas.

- Figure 1 to Figure 4 show extracts from the architectural plans illustrating the proposed development.

Figure 1 Extract from Architectural Plans – Site Plan

(Source: Architectural Plans, Custance, dated 22/11/2022)

Figure 2 Extract from Architectural Plans – General Arrangement Plan – Ground Level

(Source: Architectural Plans, Custance, dated 22/11/2022)

Figure 3 Extract from Architectural Plans – General Arrangement Plan – First Level

(Source: Architectural Plans, Custance, dated 17/11/2022)

Figure 4 Extract from Architectural Plans – Mundamatta Road Streetscape Perspective

(Source: Architectural Plans, Custance, dated 14/02/2023)

Existing Site & Locality

Existing Site and Immediately Adjoining Development

The site is located in the Canterbury Bankstown local government area (LGA) and comprises of 3 residential allotments. A location plan is provided at Figure 5.

Figure 5 Location Plan

(Source: SIX Maps)

- The site is currently occupied by 2 single storey brick dwellings with tiled roofs on 18 and 22 Mundamatta Street, while 20 Mundamatta Street is vacant (refer to photographs at Figure 6 and Figure 7.

Figure 6 Street Perspective of 18-20 Mundamatta Street, Villawood

(Source: Google Street View)

Figure 7 Street Perspective of 20-22 Mundamatta Street, Villawood

(Source: Google Street View)

Site Description

Copies of the section 10.7(2) & (5) Planning Certificates (Nos 20234358, 20234359, 20234366 and 20234379) dated 10 July 2023 and 11 July 2023 are provided in Appendix G.

The site has a total area of 2,210.36m2, a frontage to Mundamatta Street of approximately 60m, side (southern) boundary of approximately 42m, side (northern) boundary of approximately 47m and a rear (eastern) boundary of 43m (refer to the submitted Detail and Level Survey Plan in Appendix D).

The site is relativity flat across all three lots with a gradual fall from the front (southern) boundary at Mundamatta Street (RL36.75) toward the rear (northern) boundary (RL 34.84) approximately 1.91m. An easement of 1.2m for stormwater drainage is required through lot 807 SP 36608 (82 Lowana Street, Villawood).

The site is not within a flood planning area and is not subject to flood related development controls.

Fifteen trees are located within the site. Additionally, there are 3 street trees along the site’s frontage and 4 trees in the adjoining property at 82 Lowana Street that are within the area of the proposed development.

Water, sewer, electricity, gas and telephone facilities are available to the site (refer to the submitted Contour and Detail Plan for the location of available services at Appendix D and W). Water, electricity, gas and telephone services are located along the road alignment of Mundamatta Street. Sewer is located at the rear boundary of the site.

There are no encumbrances on title, section 10.7 certificates or indicated on the Detail and Level Survey Plan.

Neighbouring Development and Locality

The site is located within an established residential area which is characterised by a variety of dwelling styles. These include predominately single and some 2-storey detached dwelling houses mostly of brick construction with tiled roofs.

The properties that immediately surround the subject site consist of predominately single-storey brick dwellings with tile roof. The property to the north (24 Mundamatta Street) is a two-storey brick dwelling (refer to Figure 8). The property adjoining to the south (16 Mundamatta Street) contains a two-storey dwelling with a brick ground floor and a weatherboard first floor with tiled roof (refer to Figure 9). The properties directly across the street (25 and 27 Mundamatta Street) consist of one-storey brick dwellings with tiled roofs (refer to Figure 10).

Figure 11 shows a newer, contemporary style attached dwelling development in the near vicinity.

Figure 8 Adjoining Development to the North – 24 Mundamatta Street

(Source: Google Street View)

Figure 9 Adjoining Development to the South – 16 Mundamatta Street

(Source: Google Street View)

Figure 10 Street view to 25 & 27 Mundamatta Street, Villawood

(Source: Google Street View)

Figure 11 Street view to 8 & 8a Mundamatta Street, Villawood

(Source: Google Street View)

Two bus stops are located within approximately 208m and 240m of the site. The bus stops are serviced by the 905 service that travels from Bankstown to Fairfield and the S4 service that travels from Chester Hill to Fairfield via Carramar and Villawood. These services provide access to the Chester Hill, Bass Hill and Fairfield town centres, including the Chester Square shopping centre, Bass Hill Plaza shopping centre and Fairfield Chase shopping centre.

The site is located approximately 1.5 kilometres southwest of Chester Hill Railway Station. Residents will be able to access the station via the S4 bus service. The station provides regular services for the T2- Parramatta or Leppington to City line and T3 – Liverpool or Lidcombe to City via Bankstown Line.

Approximately 750m to the northeast of the site and along the S4 bus route is a neighbourhood centre in Bass Hill along Miller Road which includes a convenience store, barber, bakery café and takeaway shops.

Zoning and Permissibility

The site is zoned R2 Low Density Residential under the Canterbury Bankstown Local Environmental Plan 2023 (CBLEP 2023) (refer to Figure 12). The proposed development is defined as ‘seniors housing’ under the provisions of CBLEP 2023 and is prohibited in the R2 zone.

Figure 12 Land Zoning Map

(Source: NSW ePlanning Spatial Viewer)

Seniors housing is permitted with consent under section 81 of the Housing SEPP as the R2 zone is a prescribed zone under section 79 of the Housing SEPP. Further, as detailed in section 108A(b) of the Housing SEPP, seniors housing may be undertaken by LAHC in a prescribed zone.

The relevant objectives of the R2 zone, as set out in CBLEP 2023 are:

- To provide for the housing needs of the community within a low density residential environment.

- To promote a high standard of urban design and local amenity.

- To ensure suitable landscaping in the low density residential environment.

The proposed development provides seniors housing that meets the identified needs of the community and includes a mix of 1 and 2 bedroom units. The proposal is not located near heritage items or conservation areas and the development has also been sympathetically designed to fit within its context and complement the surrounding area as well as provide prospect for future development within the area. The proposal is therefore consistent with the relevant objectives of the R2 zone.

- Section 108B of the Housing SEPP permits seniors housing development to be carried out by LAHC as ‘development without consent’ subject to the provisions set out under that clause. Table 5 in sub-section 5.1.6 of this REF demonstrates compliance with the relevant provisions of section 108B of the SEPP.

Planning and Design Framework

State Legislation

Environmental Planning and Assessment Act 1979

Duty to consider environmental impact [Section 5.5]

Section 5.5(1) states that, for the purpose of attaining the objects of the EP&A Act relating to the protection and enhancement of the environment, a determining authority in its consideration of an activity shall, notwithstanding any other provisions of the Act or the provisions of any other Act or of any instrument made under the EP&A Act or any other Act, examine and take into account to the fullest extent possible all matters affecting or likely to affect the environment by reason of that activity.

Table 1 below demonstrates the effect of the proposed development activity on the matters listed for consideration in sub-section 3 of section 5.5.

Table 1 Compliance with subsection 3 of section 5.5 of the EP&A Act 1979

| Matters for consideration under sub-section 3 of section 5.5 of the EP&A Act |  |
| --- | --- |
| Matter for consideration | Effect of Activity |
| Sub-section 3 Without limiting subsection (1), a determining authority shall consider the effect of an activity on any wilderness area (within the meaning of the Wilderness Act 1987) in the locality in which the activity is intended to be carried on. | No effect, as the site and surrounding areas are not within a wilderness area (within the meaning of the Wilderness Act 1987). |

Biodiversity Conservation Act 2016 (BC Act)

Part 7 of the BC Act sets out the test for determining whether a proposed development or activity is likely to significantly affect threatened species, ecological communities or their habitats. For the purposes of Part 5 of the EP&A Act, an activity is to be regarded as an activity likely to significantly affect the environment if it is likely to significantly affect threatened species.

Based on the criteria set out in section 7.3 of the BC Act, the proposed activity is unlikely to affect threatened species, ecological communities or their habitats and therefore no further assessment is necessary. This is because the land does not contain threatened species, endangered ecological communities or constitute habitat of threatened species or ecological communities. The proposed activity will neither be a key threatening process and the land is not part of or in the vicinity of any declared area of outstanding biodiversity value.

Other Acts

No other State and Commonwealth Acts are applicable to the proposed activity.

Environmental Planning and Assessment Regulation 2021

Factors that must be taken into account concerning the impact of an activity on the environment [Section 171]

For the purposes of Part 5 of the EP&A Act, the factors in Table 2 below have been taken into account in considering the likely impact of the proposed activity on the environment.  The table and comments made in this section of the REF are not mutually exclusive and are to be read in conjunction with the other sections of the REF dealing with the environmental impacts of the proposed development activity.

Table 2 Compliance with section 171 of the EPA Regulations 2021

| Consideration of Guidelines. | Comment |
| --- | --- |
| Is the activity of a kind for which specific guidelines are in force? If so the factors to be taken into account when considering the likely impact of the activity on the environment are those referred to in the guidelines. | No specific guidelines are in force. This does not include guidelines such as the Seniors Living Urban Design Guidelines that are in force under other legislation or instruments. |
| Is the activity of any other kind for which general guidelines are in force? If so, the factors to be taken into account when considering the likely impact of the activity on the environment are those referred to in those guidelines. | Yes - Department of Planning and Environment issued “Guidelines for Division 5.1 assessments” made under Section 170 of the EPA regulation 2021. The proposed activity is assessed against the factors of consideration in Table 3 below. |

- Table 3 Factors to be taken into account concerning the impact of an activity on the environment

| - Matters to consider in Department of Planning and Environment’s Guidelines for Division 5.1 assessments (June 2022) | Relevant? | - Impact |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Yes/NA | - Nil | - Temporary | - Minor | - Significant - [Note 1] |
| (a) environmental impact on the community | Yes |  |  | x |  |
| (b) transformation of a locality; | Yes |  |  | x |  |
| (c) environmental impact on the ecosystems of the locality; | Yes |  |  | x |  |
| (d) reduction of the aesthetic, recreational, scientific or other environmental quality or value of a locality; | Yes |  |  | x |  |
| (e) effect on a locality, place or building having aesthetic, anthropological, architectural, cultural, historical, scientific or social significance or other special value for present or future generations; | Yes |  |  | x |  |
| (f) impact on the habitat of protected animals (within the meaning of Biodiversity Conservation Act 2016); | N/A | x |  |  |  |
| (g) endangering of any species of animal, plant or other form of life, whether living on land, in water or in the air; | N/A | x |  |  |  |
| (h) long-term effects on the environment; | Yes |  |  | x |  |
| (i) degradation of the quality of the environment; | Yes |  | x | x |  |
| (j) risk to the safety of the environment; | N/A | x |  |  |  |
| (k) reduction in the range of beneficial uses of the environment; | N/A | x |  |  |  |
| (l) pollution of the environment; | Yes |  | x | x |  |
| (m) environmental problems associated with the disposal of waste; | Yes |  |  | x |  |
| (n) increased demands on resources (natural or otherwise) that are, or are likely to become, in short supply; | Yes |  |  | x |  |
| (o) cumulative environmental effect with other existing or likely future activities. | Yes |  |  | x |  |
| (p) impact on coastal processes and coastal hazards, including those under projected climate change conditions. [Note 2] | N/A | x |  |  |  |
| (q) applicable local strategic planning statements, regional strategic plans or district strategic plans made under the Act, Division 3.1, | Yes Discussed below in section 5.1.5 | x |  |  |  |
| (r) other relevant environmental factors. | Yes Discussed in section 7 | x |  |  |  |

- Note 1: A significant impact triggers the preparation of an Environmental Impact Statement.

- Note 2: The NSW Coastal Planning Guideline: Adapting to Sea Level Rise provides guidance on considering projected climate change conditions such as sea level rise.

- The proposed seniors housing development is not expected to generate any significant or long- term impacts on the environment. The short term impacts, during construction, will be offset by positive social outcomes in the long term social benefits of providing affordable housing that meets the needs of the community. The applicable Local Strategic Planning Statement (LSPS) and Community Strategic Plan are considered below at section 5.1.5.

171A Activities in catchments—the Act, s 5.10(a)

- The site is located within a regulated catchment, being the Georges River Catchment. When considering the likely impact of the environment of an activity proposed to be carried out in a regulated catchment, a determining authority must take into account the matters a consent authority must consider under sections 6.6(1), 6.7(1) and 6.9(1) of State Environmental Planning Policy (Biodiversity and Conservation) 2021 (BC SEPP) and the matters of which a consent authority must be satisfied under sections 6.6(2), 6.7(2) and 6.9(2) of the BC SEPP. These matters are addressed in Table 4.

Table 4 Factors to be taken into account concerning activities in regulated catchments

| Factors to be taken into account concerning the impact of an activity in a regulated catchment | Response |
| --- | --- |
| 6.6 Water quality and quantity This clause requires the consent authority to consider whether the development will: - have a neutral or beneficial effect on the quality of water entering a waterway; - have an adverse impact on water flow in a natural waterbody; - increase the amount of stormwater run-off from a site, and - Incorporate on-site stormwater retention, infiltration or reuse. The consent authority is also required to assess the impact of the development - on the level and quality of the water table; - the cumulative environmental impact of the development on the regulated catchment: - and whether the development makes adequate provision to protect the quality and quantity of ground water. Further this clause also requires that consent is not issued unless the consent authority is satisfied the development ensures: - the effect on the quality of water entering a natural waterbody will be as close as possible to neutral or beneficial, and - the impact on water flow in a natural waterbody will be minimised. | The proposed development has been assessed in accordance with Appendix 2 of the Neutral or Beneficial Effect on Water Quality (NorBE) Assessment Guideline (WaterNSW, 2022) and found to have a neutral effect on water quality, as follows: 1. Are there identifiable potential impacts on water quality? The residential use will not result in any major potential pollutants, however sediments may arise during demolition and construction. 2. For each pollutant list the safeguards needed to prevent or mitigate potential impacts on water quality? A concept stormwater plan has been prepared which provides for suitable management of stormwater, including an onsite stormwater detention tank and pollution control pit, and an easement connecting to Council’s drainage system. An environmental site management plan has been prepared for the demolition and construction phases of development. Erosion and sediment control measures include silt fencing, diversion banks, undisturbed buffer zones, filter tube to kerb inlet pits, filter fabric inlet, geotextile wrapped hay bales and sand bags. 3. Will the safeguards be adequate for the time required? The property will be maintained by contractors during occupation / use. Identified Requirements (Nos. 6 and 42) are recommended to ensure stormwater is appropriately collected and conveyed to Council’s drainage system, and implemented in accordance with the concept plan for the duration of the development. The head contractor will implement environmental site management measures outlined in the environmental site management plan for the duration of demolition / construction works. Identified requirement (No. 13) recommends that sediment control measures be implemented during demolition / construction in accordance with Council requirements and/or the guidelines contained in the Blue Book Managing Urban Stormwater: Soils and Construction (4th edition, Landcom, 2004). 4. Will impacts on water quality be effectively contained on the site by the identified safeguards (above) and not reach any watercourse, waterbody or drainage depression? Subject to the above, erosion, sediment and stormwater runoff will be effectively contained on the site and connected to Council’s drainage system with no impacts on any watercourse, waterbody or drainage depression. 5. Is it likely that a neutral or beneficial effect on water quality will occur? Justify. In light of the above, it is considered the proposed development will have a neutral effect on water quality. The proposal provides for adequate safeguards for avoiding impacts on water quality and quantity. |
| 6.7 Aquatic ecology This clause requires the consent authority to consider whether the development: - will have a direct, indirect or cumulative adverse impact on terrestrial, aquatic or migratory animals or vegetation, - involves the clearing of riparian vegetation and, if so, whether the development will require either a controlled activity approval under the Water Management Act 2000, or a permit under the Fisheries Management Act 1994, - will minimise or avoid the erosion of land abutting a natural waterbody, or the sedimentation of a natural waterbody, or will have an adverse impact on wetlands that are not in the coastal wetlands and littoral rainforests area, - includes adequate safeguards and rehabilitation measures to protect aquatic ecology, - if the site adjoins a natural waterbody—whether additional measures are required to ensure a neutral or beneficial effect on the water quality of the waterbody. Further this clause also requires that consent is not issued unless the consent authority is satisfied the development ensures: - that the direct, indirect or cumulative adverse impact on terrestrial, aquatic or migratory animals or vegetation will be kept to the minimum necessary for the carrying out of the development, - the development will not have a direct, indirect or cumulative adverse impact on aquatic reserves, - if a controlled activity approval under the Water Management Act 2000 or a permit under the Fisheries Management Act 1994 is required in relation to the clearing of riparian vegetation—the approval or permit has been obtained, - the erosion of land abutting a natural waterbody or the sedimentation of a natural waterbody will be minimised, - the adverse impact on wetlands that are not in the coastal wetlands and littoral rainforests area will be minimised. | The subject site is not located on riparian land and will not require a controlled activity permit, nor is the site located in the vicinity of any wetland or natural waterbody. Suitable erosion and sedimentation controls will be required during works to minimise erosion and maintain water quality – these are illustrated in the erosion and sediment control plan in Appendix U. |
| 6.8 Flooding - This clause relates to flood liable land. | The site is not identified as flood liable land. |
| 6.9 Recreation and public access - This clause relates to development on recreational land and public access to natural water bodies and foreshores. | The proposed activity will not impact recreational land uses or alter public access to recreational land or foreshores. |

Strategic Planning Framework

- Canterbury Bankstown Connective City 2036 Local Strategic Planning Statement

- The Canterbury Bankstown Local Strategic Planning Statement was endorsed by Canterbury Bankstown Council in 2016. It is a 20 year plan that identifies 10 Evolutions for the LGA, focused around infrastructure, collaboration, liveability, productivity and sustainability.

- Notably, Evolution 6 seeks to promote diverse and affordable housing options especially with a growing ageing population. The proposed development will be contributing 12 seniors living units to the affordable housing supply in the Canterbury Bankstown LGA. It would be diversifying the residential uses within Villawood by introducing a seniors living housing option to accommodate the ageing population in the locality and is well serviced by existing public transport options.

- The proposed development of 12 seniors living units is consistent with the objectives set out in the Canterbury Bankstown Local Strategic Planning Statement, in that it will increase the provision of affordable and seniors living units.

- Canterbury Bankstown 2036 Community Strategic Plan

- The Canterbury Bankstown 2036 Community Strategic Plan was adopted from the 2028 community strategic plan. It was originally a 10 year plan that has been extended and outlines seven broad inter-related themes that are derived from an extensive community engagement process, which identified priorities for the community’s future. Under the seven themes there are key goals and subsequent strategies for Council to facilitate in partnership with the community, government agencies and business. These seven key strategic destinations are supported by key goals that will guide the Canterbury Bankstown LGA until 2036. The themes are focused around; Safe & Strong, Clean & Green, Prosperous & Innovative, Moving & integrated, Healthy & Active, Liveable & Distinctive and Leading & Engaged. The proposed development for 12 seniors living social housing units would contribute to achieving the goals outlined in the Canterbury Bankstown 2036 Community Strategic Plan.

State Environmental Planning Policy (Housing) 2021

Development without Consent

The proposed activity includes demolition of existing structures on the site. Section 42(2)(a) of the Housing SEPP permits LAHC to undertake demolition as “development without consent” provided the land the structures are located on is “non-heritage land” and is not “identified in an environmental planning instrument as being within a heritage conservation area”. The subject site contains neither of these heritage notations and therefore demolition can be undertaken by LAHC as “development without consent”. A waste management plan has been prepared and several Identified Requirements have been recommended in the Activity Determination to appropriately manage site safety and environmental protection during the demolition and construction phase.

- Section 108B of the Housing SEPP permits seniors housing to be carried out by LAHC as ‘development without consent’ subject to the provisions set out within Part 5, Division 8. An assessment against the relevant provisions of that section is provided in Table 5 below.

Table 5 Compliance with relevant provisions under sections Part 5, Division 8 of the SEPP for ‘seniors housing development without consent’ carried out by LAHC

| - Provision | Compliance |
| --- | --- |
| 108A – Development to which Division applies - This Division applies to development for purposes of senior housing involving the erection of a building on land - |  |
| - (a) on which development for purposes of seniors housing is permitted with consent under another environmental planning instrument, or | Under the CBLEP 2023 seniors housing is prohibited within the R2 zone. |
| - (b) in a prescribed zone | The R2 zone is a prescribed zone under the SEPP. Part 5, Division 8 of the SEPP therefore applies. |
| 108B – Seniors housing permitted without development consent - (1) Development to which this Division applies may be carried out by or on behalf of the Land and Housing Corporation without development consent if - |  |
| - (a) the Land and Housing Corporation has considered the applicable development standards specified in sections 84(2)(c)(iii), 85, 88, 89 and 108, and | Consideration of the development standards is provided in Table 9, Table 10, Table 11 and Table 12 below. |
| - (b) will not result in a building with a height of more than 9.5 m, and | The maximum building height is 7.2m. |
| - (c) the development will result in more than 40 dwellings on the site. | The development is for 12 units on the site. |
| - (2) State Environmental Planning Policy (Transport and Infrastructure) 2021, sections 2.15 and 2.17 apply to the development and, in the application of the clauses— | Sections 2.15 and 2.17 of State Environmental Planning Policy (Transport and Infrastructure) 2021 are not applicable to the site or development. |
| - (a) a reference in section 2.15 to “this Chapter” is taken to be a reference to this section, and | Noted. |
| - (b) a reference in the sections to a public authority is taken to be a reference to the relevant authority. | Noted. |
| 108C – Requirements for carrying out seniors housing - (1) Before carrying out development to which this Division applies, a relevant authority must – |  |
| (a) request the council to nominate a person or persons who must, in the council’s opinion, be notified of the development, and | Advice was sought from Canterbury Bankstown Council regarding additional persons or properties that should be notified of the development via an email sent to Council on 12 July 2022. Council provided a response on 14 July 2022 advising that the notification map is acceptable. |
| (b) give written notice of the intention to carry out the development to— (i) the council, and (ii) the person or persons nominated by the council, and (iii) the occupiers of adjoining land, and | A letter notifying Council of the proposed development activity was sent by LAHC on 28 November 2022. Letters notifying occupiers of adjoining land of the proposed development activity were sent by LAHC on the 28 November 2022. Details of the notification to adjoining occupiers are provided in section 6.2 of this REF. |
| (c) take into account the responses to the notice that are received within 21 days after the notice is given, and | Council responded to LAHC’s notification by email dated 16 December 2022. Comments on the response are provided in section 6.1 of this REF. No submissions were received from adjoining occupiers. |
| (d) take into account the relevant provisions of the Seniors Living Policy: Urban Design Guidelines for Infill Development, published by the Department in March 2004, and | Refer to checklist in Appendix O which indicates that the development generally complies with all relevant development standards in the Seniors Living Policy: Urban Design Guidelines for Infill Development. Where there are departures, these are discussed in sub-section 5.1.7 of this REF. |
| (d1) if the relevant authority is the Aboriginal Housing Office—consider the relevant provisions of the Aboriginal Housing Design Guidelines, published by the Aboriginal Housing Office in January 2020, and | Not applicable. |
| (e) if the relevant authority is the Land and Housing Corporation—consider the relevant provisions of— (i) Good Design for Social Housing, published by the Land and Housing Corporation in September 2020, and (ii) Land and Housing Corporation Design Requirements, published by the Land and Housing Corporation in February 2023, and | Refer to section 5.1.8 and 5.1.9 and the Architect’s Statement and Certificate of Building Design Compliance in Appendix M which indicate that the design requirements have been considered. |
| (f) consider the design principles set out in Division 6 | Consideration of these principles is discussed in Table 8. |
| (2) In this section, a reference to the council is a reference to the council for the land on which the development is proposed to be located. | Noted. |
| 108D - Exempt development Development for purposes of landscaping and gardening is exempt development if it is carried out by or on behalf of the Land and Housing Corporation in relation to seniors housing | Noted |
| 108E - Subdivision of seniors housing not permitted Development consent must not be granted for subdivision of seniors housing. | No subdivision proposed. |

Seniors Living Policy: Urban Design Guidelines for Infill Development

An assessment of the design of the activity against the Seniors Living Policy: Urban Design Guidelines for Infill Development is provided at Appendix O. The design has followed the Guidelines, except in relation to the following justifiable departures outlined in Table 6.

Table 6 Seniors Living Urban Design Guidelines departures

| Guideline Requirement | Response |
| --- | --- |
| 3.06 Setback upper levels behind the front building façade? | Upper levels are not setback further than the front building façade to simplify construction. Instead, a mix of materials and finishes allow for a varied façade that works well within the streetscape. |
| 3.27 Vary the driveway surface material to break it up into a series of smaller spaces? (e.g. to delineate individual dwellings) | Permeable paving will be limited to areas of the car park in tree protection zones (as indicated in the tree protection plan in Appendix J) but not the driveway. The visual impact of the driveway will be instead mitigated by a bend or deviation and landscaped edges. Identified Requirement (No.12) provides for limited application of permeable paving as per the Arboricultural Impact Assessment (Tree protection specifications section ) report. |
| 3.28 Limit driveway widths on narrow sites to single carriage with passing points? | The driveway has been minimised to 3m wide to minimise visual impacts. A separate driveway is provided for Unit 4, at maximum distance from the communal driveway. The project traffic engineer has confirmed that a passing bay is not required. Refer to drawing SK.01 (Rev.A) dated 22/06/2022 prepared by Traffix in Appendix S. |
| 3.29 Provide gates at the head of driveways to minimise visual ‘pull’ of the driveway | Driveway gates to common parking areas create maintenance issues and are not specified in the LAHC Design Standards. The communal driveway is landscaped on either side and at its termination to reduce visual impacts. |
| 4.03 Set upper storeys back behind the side or rear building line | Upper floors have not been setback. Nonetheless, streetscape impacts are mitigated through strong façade articulation and varied setbacks along the street façade. Furthermore, the development also provides a front landscaped setting to soften the built form, providing for an overall positive contribution to the streetscape. |
| 4.05 Incorporate second stories within the roof space and provide dormer windows? | Second storeys within roof spaces and dormer windows are not a characteristic in the locality. The proposed second storey complies with height and setback requirements and does not give rise to any adverse streetscape or amenity impacts. |
| 4.14 Locate private open space in front setbacks where possible to minimise negative impacts on neighbours? | The POSs of units 4 & 11 are oriented to the street with the remainder of POSs at the rear, however the rear facing POSs will have no adverse impacts as they are oriented to the centre of the site and screened from neighbours by an existing tree. |
| 4.16 Design dwellings around internal courtyards? | Ten of the 14 units face the internal communal open space / existing large tree. Two dwellings, Units 4 & 11 are oriented to the street but they will nonetheless have good outdoor amenity as their POSs are behind a landscaped front setback. |
| 5.29 Provide private open space areas that use pervious pavers where private open space is predominantly hard surfaced to allow for water percolation and reduced run-off? | Non-pervious pavers are proposed for POSs to ensure accessibility and minimise maintenance. Runoff is instead reduced by deep soil zones and landscaped areas that exceed minimum requirements, and the provision of rain water and onsite detention tanks. |

Good Design for Social Housing

An assessment of the proposed development against the Good Design for Social Housing document, published in September 2020, indicates that the proposed development has adequately considered the goals and principles as outlined in Table 7 below. Refer to the Certificate of Compliance in Appendix M.

Table 7 Good Design for Social Housing – Relevant Goals & Principles

| Goals | Principles | Comment - Discussion on how the design responds to the principles: |
| --- | --- | --- |
| Wellbeing | Healthy environments Good for tenants Quality homes | Safe access is provided from the car park to entries of the building through walking paths. The units have been designed to ensure tenants have privacy and feel safe. The development achieves BASIX requirements (Appendix K). The majority of balconies overlook a large tree retained within a large landscaped area. The proposal includes high quality landscaping and outdoor areas to enhance the site amenity for residents and the streetscape. |
| Belonging | Mixed tenure Good shared and public spaces Contribute to local character | Front entrances are easily identified. Attractively designed landscaping is provided along the street frontage and throughout the site. The scale of the development provides for good internal and outdoor amenity. The building design, materials and landscaping integrate with the surrounding residential neighbourhood. |
| Value | Whole of lifecycle approach Sustainability and resilience Make every dollar count | Durable building materials and low maintenance landscape species are proposed. All ground floor units are adaptable. The orientation of each dwelling and private open space area has been designed to optimise natural light to these areas. An underground rainwater tank is provided for the development to assist with sustainability and on-site water retention. Development meets BASIX requirements for building sustainability. Collaboration and consultation with key stakeholders was undertaken at critical design milestones. |
| Collaboration | - A good partner - Place Making - Continuous improvement | The proposal is of a scale and character that assists with place making, by ensuring integration with the surrounding development. Collaboration with a number of stakeholders has been undertaken during the design and assessment process, with the development shaped by input from a wide range of consultants and stakeholders, including Canterbury Bankstown Council. |

Land and Housing Corporation Design Requirements

- The Land and Housing Corporation Design Requirements (LAHC Design Requirements) (February 2023) are used to inform the design and development of the LAHC social housing portfolio. These requirements apply to all new LAHC developments and are driven by tenant wellbeing, design quality, environmental performance and operational effectiveness within cost parameters.

- Clause 108B (e)(ii) of the Housing SEPP requires LAHC to consider the relevant provisions of the Land and Housing Corporation Design Requirements when assessing a proposed residential housing development under Chapter 3. Part 5, Division 8 of the Housing SEPP.

- The proposed development was designed in accordance with the Land and Housing Corporation Dwellings Requirements 2020 (refer Appendix M). These requirements were replaced by the Land and Housing Corporation Design Requirements in February 2023. The proposed development is generally in accordance with the provisions of the updated requirements, except for meeting the target of a 7 star NatHERS rating. The development achieves an average 6.8 star rating. While adopted in the current design requirements, the updated BASIX requirements do not come into effect until October 2023, and therefore the project is compliant with the current BASIX requirements.   Further detail will be incorporated in the construction documentation.

Housing SEPP Design Principles

The Housing SEPP articulates a range of design principles that the LAHC must consider in determining whether or not to proceed with a proposed seniors housing activity. Table 8 below demonstrates how the principles have been considered in the design of the proposal.

Table 8 Response to Design Principles (Part 5, Division 8)

| Design Principle | Response |
| --- | --- |
| Neighbourhood amenity and streetscape [section 99] Seniors housing should be designed to— (a) recognise the operational, functional and economic requirements of residential care facilities, which typically require a different building shape from other residential accommodation, and | The proposal is not a residential care facility. |
| (b) recognise the desirable elements of— (i) the location’s current character, or (ii) for precincts undergoing a transition—the future character of the location so new buildings contribute to the quality and identity of the area, and | The proposal is generally compatible in form, scale and character of the area, with an emerging trend of larger dwellings and dual occupancy contemporary residential development as illustrated in Figure 11. |
| (c) complement heritage conservation areas and heritage items in the area, and | The site is not located within a heritage conservation area nor are there any heritage items within proximity of the site. |
| (d) maintain reasonable neighbourhood amenity and appropriate residential character by— (i) providing building setbacks to reduce bulk and overshadowing, and (ii) using building form and siting that relates to the site’s land form, and (iii) adopting building heights at the street frontage that are compatible in scale with adjacent buildings, and (iv) considering, where buildings are located on the boundary, the impact of the boundary walls on neighbours, and | Building setbacks allow for substantial landscape plantings, are consistent with newer style developments in the area that comply with Canterbury Bankstown DCP 2023 objectives and controls. A suitable balance of cut and fill and a retaining wall adjacent and set back from the southern boundary is proposed to address the access requirements for seniors housing under the Housing SEPP. The street front buildings will be two storey with the bulk of the development broken up through the use of stepped setbacks, a variety of materials and finishes, extensive landscaping and open space areas. No buildings are located on the boundary with appropriate side and rear setbacks adopted. |
| (e) set back the front building on the site generally in line with the existing building line, and | The building line is forward of the existing building line on Mundammatta Street, however, is consistent with the setback of the existing dwelling within the site at 18 Mundamatta Street and 16 Mundamatta Street on a corner lot, and the proposed front setback complies with Canterbury Bankstown DCP. |
| (f) include plants reasonably similar to other plants in the street, and | The submitted Landscape Plan (Appendix B) indicates the retention of 6 trees including a Spotted Gum, Lemon Scented Gum and Water Gum. Thirty-three semi mature trees are also proposed including native species such as Silver Gum and Weeping Lilly Pilly, as well as native shrubs such as Bottlebrush that will complement existing plants in the area. |
| (g) retain, wherever reasonable, significant trees, and | Two trees within/adjacent to the site have been identified as having high retention value, and these trees will be retained. All trees proposed for removal have a medium, low or nil retention value and will be replaced by suitable species. |
| (h) prevent the construction of a building in a riparian zone. | The site is not located within or adjacent to a riparian zone. |
| Visual and acoustic privacy [section 100] Seniors housing should be designed to consider the visual and acoustic privacy of adjacent neighbours and residents by— (a) using appropriate site planning, including considering the location and design of windows and balconies, the use of screening devices and landscaping, and (b) ensuring acceptable noise levels in bedrooms of new dwellings by locating them away from driveways, parking areas and paths. | The proposed development has been designed to maintain visual and acoustic privacy to adjoining properties and within the development. Design solutions include appropriate building setbacks and heights, dwelling layouts with primary orientation within the site or to the street, placement and sizes of window openings, glazing and location and future mature height of landscaping, particularly along the side and rear boundary adjacent to existing developed allotments. Boundary fencing to a height of 1,800mm will mitigate visual and acoustic impacts associated with the internal hard stand car parking located at the rear of the site. Perimeter landscaping will also act as a visual and acoustic buffer between the car park and adjacent development. The driveway and car park are well separated from the dwellings to minimise impacts from vehicle movements. All bedrooms in the development have a primary orientation away from the street and are well separated from driveways, parking areas and paths. |
| Solar access and design for climate [section 101] The design of seniors housing should— (a) for development involving the erection of a new building—provide residents of the building with adequate daylight in a way that does not adversely impact the amount of daylight in neighbouring buildings, and (b) involve site planning, dwelling design and landscaping that reduces energy use and makes the best practicable use of natural ventilation, solar heating and lighting by locating the windows of living and dining areas in a northerly direction. | The proposed development will provide adequate daylight access to its living areas and private open spaces and the living areas and private open spaces of adjoining properties. The northern orientation of proposed living and open space areas will maximise solar access. 83% of the proposed units will receive a minimum of three hours sunlight between 9am and 3pm on June 21 to their living area and private open space. Midwinter shadows to neighbouring development to the south (No.16 Mundamatta Street) will be confined to the afternoon period, with more than 50% of the rear garden maintaining solar access between 9am and 1pm (4 hours) (refer to shadow diagrams at Appendix A). Private open space areas of properties to the southeast (No.14 Mundamatta Street) will retain solar access to the majority of the rear garden between 9am and 2pm (5 hours), which is facilitated by the development stepping down to single storey in the south eastern corner of the site. Landscaping will also assist in microclimate management. |
| Stormwater [section 102] The design of seniors housing should aim to— (a) control and minimise the disturbance and impacts of stormwater runoff on adjoining properties and receiving waters by, for example, finishing driveway surfaces with semi-pervious material, minimising the width of paths and minimising paved areas, and (b) include, where practical, on-site stormwater detention or re-use for second quality water uses. | Impervious surfaces have been minimised in the design to reduce the potential impact from stormwater. Rainwater from roofs and site stormwater will be drained into an underground rain water tank and an on-site detention tank as shown on the submitted stormwater drainage plans (refer to Appendix C). The BASIX Certificate (Appendix K) indicates the proposed rainwater tank would be 5kL in volume, to irrigate 1,125m2 of common landscaped area within the site. |
| Crime prevention [section 103] Seniors housing should— (a) be designed in accordance with environmental design principles relating to crime prevention, and (b) provide personal property security for residents and visitors, and (c) encourage crime prevention by— (i) site planning that allows observation of the approaches to a dwelling entry from inside each dwelling and general observation of public areas, driveways and streets from a dwelling that adjoins the area, driveway or street, and (ii) providing shared entries, if required, that serve a small number of dwellings and that are able to be locked, and (iii) providing dwellings designed to allow residents to see who approaches their dwellings without the need to open the front door. | Fencing will be constructed along the front, side and rear site boundaries and along the edges of the ground floor private open space to each unit to provide an appropriate level of safety and security to future residents. The proposed development will allow for general surveillance of the common landscaped area by ground floor Units 1, 2, 3 and 5 and from the balconies of Units 8, 9, 10 and 12. Units 4 and 11 face Mundamatta Street providing casual surveillance of the street. All paths, driveways and the car park have direct sightlines to the dwelling units or the street. Identified Requirement (No. 78) is recommended to ensure front doors of each dwelling include door viewers to enable residents to view approaches to their dwelling without having to open the door. |
| Accessibility [section 104] Seniors housing should— (a) have obvious and safe pedestrian links from the site that provide access to transport services or local facilities, and (b) provide attractive, yet safe, environments for pedestrians and motorists with convenient access and parking for residents and visitors. | The Access Report (Appendix H) indicates that there is a suitable access pathway to bus stops that provide access to local facilities, subject to construction of a new section of footpath on Lowana Street. The pedestrian paths from the proposed at grade carpark to the units are separated from traffic. |
| Waste management [section 105] Seniors housing should include waste facilities that maximise recycling by the provision of appropriate facilities. | Waste and recycling facilities will be provided in accordance with Council’s requirements. The proposed development provides shared waste and recycling collection facilities on the site. |

General Requirements and Development Standards Applying to Seniors Housing

The Housing SEPP is very specific in terms of the matters that LAHC must consider in determining whether or not to proceed with a seniors housing development that meets the thresholds for self-approval under the SEPP. There are locational and detailed design requirements that also need to be considered. These are discussed below.

The proposed activity is located in an established urban area and there are no known natural environmental considerations affecting the subject land (including known significant environmental values, resources or hazards). The existing and approved uses of land in the vicinity of the proposed development is for residential purposes.

A site analysis was undertaken as part of the design process. The impact of the bulk, scale and built form is considered to be compatible with the existing uses, approved uses and the likely future character of the area.

There are services, facilities and infrastructure that will be available to meet the demands arising from the proposed activity.

(i) General Development standards [Section 84, 85 and Schedule 4]

The SEPP contains general development standards relating to minimum site size, site frontage and building height (section 84), which must be taken into consideration for all developments, and development standards specifically for independent living units (section 85 & Schedule 4), which must also be considered. Consideration of section 84 of the SEPP is demonstrated in the Table 9 below:

Table 9 Minimum site size, site frontage and building height standards [section 84]

| Development Standard | Required | Comment |
| --- | --- | --- |
| Site size (s.84(2)) | At least 1,000m2 | 2,210.36m2, although not applicable to LAHC (cl.84(4)(a)) |
| Site frontage (s.84(2)) | At least 20m | 59m, although not applicable to LAHC (cl.84(4)(a)) |
| Height in zones where residential flat buildings are not permitted: | 9.5m or less (excluding servicing equipment on the roof of the building) if the development results in a building with more than 2 storeys—the additional storeys are set back within planes that project at an angle of 45 degrees inwards from all side and rear boundaries of the site | Max 7.2m Max 2 storeys in height |

Consideration of the development standards for accessibility set out in section 85 and Schedule 4 is demonstrated in Table 10 below.

It should be noted that pursuant to section 85(2), LAHC is exempt from the siting standards for wheelchair access and access to common area / facilities set out below in relation to a unit or part of a unit that is located above the ground floor in a multi-storey building.

Table 10 Accessibility and useability standards [Schedule 4]

| Development Standard (Sch 4) | Required | Comment |
| --- | --- | --- |
| Siting Standards: |  |  |
| Wheelchair access |  |  |
| ‘Non-sloping’ sites i.e. with gradients entirely <1:10 | 100% of ground floor dwellings to have wheelchair access by a continuous accessible path of travel as per AS 1428.1 to an adjoining public road | The ground floor units are capable of achieving accessible paths of travel to the front boundary in accordance with AS1428.1, subject to detailed design in the construction documentation stage – refer to Access Report in Appendix H. |
| ‘Sloping’ sites i.e. with gradients entirely or partially >1:10 | % of ground floor dwellings, equal to % of site with gradient <1:10 or minimum 50% (whichever is greater), to have wheelchair access by a continuous accessible path of travel as per AS1428.1 to driveway or public road that is accessible to all residents | Not applicable, as the site does not have a gradient of more than 1 in 10. |
| Common areas | All common areas and facilities to have wheelchair accessibility as per AS1428.1 | The gradient and crossfalls of the external pathways leading to the communal area and carpark appear capable of complying, subject to finer design details being reviewed at construction documentation stage. |
| Security: |  |  |
| Pathway lighting | Pathway lighting to be designed and located to avoid glare for pedestrians and adjacent dwellings and to provide min 20 lux at ground level | All lighting will be designed as per requirements at tender stage (subject to Identified Requirement No.75). |
| Letterboxes | To be lockable, located in central location adjacent to street entry or in one or more central locations on a hard standing area and have wheelchair access and circulation by a continuous path of travel (as per AS1428.1) | Letterboxes are suitably situated and readily capable of compliance with this clause subject to review of the finer design details at construction documentation stage. |
| Private car accommodation: |  |  |
| Disabled car parking | Car parking spaces must comply with the requirements for parking for persons with a disability set out in AS2890 and at least 5% of the total number of car parking spaces (or at least 1 space if there are fewer than 20 spaces) must be designed to enable the width of the spaces to be increased to 3.8m | An adaptable space is provided for unit 4 and 2 accessible car parking spaces are provided in the common car park, which are capable of complying with the requirements of AS2890, subject to detailed design in the construction documentation stage. |
| Garages | Must have power-operated door or there must be a power point and an area for motor or control rods to enable a power-operated door to be installed at a later date | Not applicable, as the proposal does not incorporate garages. |
| Accessible entry |  |  |
| Every entry (whether front entry or not) | Must comply with clauses 4.3.1 and 4.3.2 of AS4299 | The entry doors are suitably detailed as confirmed in the Access Report in Appendix H. |
| Garbage: | To be provided in accessible location | Two bin storage areas have been provided in the front setback. Access Report (Appendix H) indicates these are provided with access ways connecting to each ground floor dwelling. |

- Consideration of the specific development standards for useability set out in Schedule 4 of the SEPP, i.e. the standards for internal design and fittings, is outlined in Table 11 below. It should be noted that pursuant to section 85(2), LAHC is exempt from those development standards in relation to a unit or part of a unit that is located above the ground floor in a multi-storey building, these being sections 2, 7 to 13 & 15 to 20, including the requirement to provide lift access for units above the ground level in multi-storey buildings. The following standards therefore only apply to ground floor units.

Table 11 Internal design and fittings standards [Schedule 4]

| Development Standard | Required | Comment |
| --- | --- | --- |
| Interior: general | Internal doorways must have a minimum clear opening that complies with AS1428.1 Internal corridors must have a minimum unobstructed width of 1,000mm Circulation space at approaches to internal doorways must comply with AS1428.1 | Internal doorways and corridors comply are readily capable of compliance as confirmed in the Access Report in Appendix H. |
| Bedroom | At least one bedroom within each dwelling must have: (a) an area sufficient to accommodate a wardrobe and a bed sized as follows: (i) in the case of a dwelling in a hostel-a single - size bed, (ii) in the case of a independent living unit - a queen size bed, and (b) a clear area for the bed of at least: (i) 1,200mm wide at the foot of the bed, and (ii) 1,000mm wide beside the bed between it and the wall, wardrobe or any other obstruction, and (c) 2 double general power outlets on the wall where the head of the bed is likely to be, and (d) at least 1 general power outlet on the wall opposite the wall where the head of the bed is likely to be, and (e) a telephone outlet next to the bed on the side closest to the door and a general power outlet beside the telephone outlet, and (f) wiring to allow a potential illumination level of at least 300 lux | As confirmed in the Access Report in Appendix H, the bedroom layout in each ground floor dwelling is readily capable of compliance with this clause subject to a review of the finer design details at construction documentation stage. |
| Bathroom | At least 1 bathroom within a dwelling must be on the ground (or main) floor and have the following facilities arranged within an area that provides for circulation space for sanitary facilities in accordance with AS1428.1: (a) a slip-resistant floor surface, (b) a washbasin with plumbing that would allow, either immediately or in the future, clearances that comply with AS1428.1, (c) a shower that complies with AS1428.1, except that the following must be accommodated either immediately or in the future: (i) a grab rail, (ii) portable shower head, (iii) folding seat, (d) a wall cabinet that is sufficiently illuminated to be able to read the labels of items stored in it, (e) a double general power outlet beside the mirror The requirement under item (c) does not prevent the installation of a shower screen that can easily be removed to facilitate future accessibility | As confirmed in the Access Report in Appendix H the bathroom layout in each ground floor dwelling is readily capable of compliance with this clause subject to review of the finer design details at construction documentation stage. |
| Toilet | A dwelling must have at least 1 toilet on the ground (or main) floor and be a visitable toilet that complies with the requirements for sanitary facilities of AS4299 | As confirmed in the Access Report in Appendix H, the toilets are suitably detailed for compliance with this clause subject to a detailed review of the finer design details at construction documentation stage. |
| Surface finishes | Balconies and external paved areas must have slip-resistant surfaces | As confirmed in the Access Report in Appendix H slip resistance to balconies and external paved areas are readily capable of compliance with this clause subject to review of the finer design details at construction documentation stage. |
| Door hardware | Door handles and hardware for all doors (including entry doors and other external doors) must be provided in accordance with AS4299 | As confirmed in the Access Report in Appendix H the door hardware throughout the ground floor dwellings is readily capable of compliance with this clause subject to review of the finer design details at construction documentation stage. |
| Ancillary items | Switches and power points must be provided in accordance with AS4299 | As confirmed in the Access Report in Appendix H the switches and power points throughout the ground floor dwellings are readily capable of compliance with this clause subject to review of the finer design details at construction documentation stage. |
| Living room and dining room | A living room in a self -contained dwelling must have: (a) a circulation space in accordance with clause 4.7.1 of AS4299, and (b) a telephone adjacent to a general power outlet. A living room and dining room must have wiring to allow a potential illumination level of at least 300 lux | As confirmed in the Access Report in Appendix H the living room layout in each ground floor dwelling is readily capable of compliance with this clause subject to review of the finer design details at construction documentation stage. |
| Kitchen | A kitchen in a self-contained dwelling must have: (a) a circulation space in accordance with clause 4.5.2 of AS4299, and (b) a circulation space at door approaches that complies with AS 1428.1, and (c) the following fittings in accordance with the relevant subclauses of clause 4.5 of AS4299: (i) benches that include at least one work surface at least 800mm in length that comply with clause 4.5.5 (a), (ii) a tap set (see clause 4.5.6), (iii) cooktops (see clause 4.5.7), except that an isolating switch must be included, (iv) an oven (see clause 4.5.8), and (d) “D” pull cupboard handles that are located towards the top of below-bench cupboards and towards the bottom of overhead cupboards, and (e) general power outlets: (i) at least one of which is a double general power outlet within 300mm of the front of a work surface, and (ii) one of which is provided for a refrigerator in such a position as to be easily accessible after the refrigerator is installed | As confirmed in the Access Report in Appendix H the kitchen layout in each ground floor dwelling is readily capable of compliance with this clause subject to review of the finer design details at construction documentation stage. |
| Access to kitchen, main bedroom, bathroom and toilet | In a multi-storey self-contained dwelling, the kitchen, main bedroom, bathroom and toilet must be located on the entry level | Not applicable, as units are not multi-storey. |
| Laundry | A self-contained dwelling must have a laundry that has: (a) width at door approaches that complies with cl.7 of this Schedule, and (b) provision for the installation of an automatic washing machine and a clothes dryer, and (c) a clear space in front of appliances of at least 1,300mm, and (d) a slip-resistant floor surface, and (e) an accessible path of travel to any clothes line provided in relation to the dwelling | As confirmed in the Access Report in Appendix H the laundry layout in each ground floor dwelling is readily capable of compliance with this clause subject to review of the finer design details at construction documentation stage. |
| Storage for linen | A self-contained dwelling must be provided with a linen storage in accordance with Clause 4.11.5 of AS 4299 | As confirmed in the Access Report in Appendix H linen storage in each ground floor dwelling is readily capable of compliance with this clause subject to review of the finer design details at construction documentation stage. |
| Garbage | A garbage storage area must be provided in an accessible location. | As noted in the Access Report in Appendix H a garbage enclosure has been provided with an accessway connecting back to each ground floor dwelling. |

(ii) Site related requirements

The proposed development complies with section 93 of the SEPP which relates to the accessibility of the site to services and facilities such as shops and other retail/commercial services, community services recreation facilities and the practice of a general medical practitioner.

The site is not within 400m walking distance of the facilities and services listed in section 93(1) of the SEPP and therefore must be accessible to bus services that can take future residents to and from the required services.

There are two bus stops located on each side of Lowana Street which are located within 400m from the site that have accessible gradients along a suitable path of travel for seniors (see chainage survey sheets included in Appendix T). Bus stop ID 2163180 on the western side of Lowana Street and bus stop ID 2163113 on the eastern side of Lowana Street are approximately 208m and 240m from the site, respectively.

The bus stops are serviced by route 905 between Bankstown and Fairfield and route S4 between Chester Hill and Fairfield via Carramar and Villawood. These services meet the frequency requirements in section 93 of the SEPP and would link residents to the services and facilities in the Bankstown, Bass Hill, Chester Hill and Fairfield town centres.

The Access Report (Appendix H) indicates that the gradient of the paths of travel to the two bus stops on Lowana Street complies with the requirements of section 93. The report recommends extending the footpath at bus stop ID 2163113 on the eastern side of Lowana Street to allow for sealed access to the bus door. Extension of the paving to ensure a suitable access pathway is provided to the bus is proposed as part of this seniors housing development. Plans of longitudinal sections to bus stops are provided in Appendix T.

(iii) Non-discretionary Development Standards

Table 12 outlines the non- discretionary standards that LAHC must consider under section 108 of the HSEPP.

Table 12 Non-discretionary standards for self-contained dwellings

| Development Standard | Required | Comment |
| --- | --- | --- |
| Building Height | 9.5m or less | Complies Maximum height 7.2m |
| Density and Scale | Floor Space Ratio 0.5:1 or less | Complies 0.36:1 |
| Landscaped Area | Minimum 35m2 per dwelling (12 x 35m2 = 420m2) | Complies 950m2 |
| Deep Soil Zone | Minimum 15% of area of site (15% x 2,210.36m2 = 331.6m2) | Complies 583m2 or 26.3% |
|  | Minimum 65% (378.95m2) to be preferably located at rear of site | Complies 488m2 provided at rear of site |
|  | Minimum dimension 3m | Complies Greater than 3m |
| Solar Access | 70% of living areas & main private open space to receive minimum 2 hrs direct solar access between 9 am and 3 pm at mid-winter | Complies 83% of living areas and private open space (10 units) achieve 3 hours of direct solar access to private open space and living areas between 9am and 3pm mid-winter |
| Private Open Space | Ground level: Minimum 15m2 per dwelling | Complies - All ground level units achieve at least 15m2 of private open space. |
|  | One area minimum 3m x 3m, accessible from living area | Complies All POS spaces are above 3m x 3m |
|  | Upper level/s: 1 bedroom: Minimum 6m2 Minimum dimensions 2m 2 or more bedrooms: Minimum 10m2 Minimum dimensions 2m | Minor variation All 1 bedroom units comply, with a minimum area of 9.24m2 and minimum dimension of 2m. All 2 bedroom units comply with the balcony area requirements, except for Unit 11, which has a balcony area of 9.83m2. This is considered acceptable as the variation is minor at 0.17m2 (1.7%) and will not detract from the amenity of the space which provides for a 2.7m deep area of useable space net of the clothesline. |
| Car parking | 1 parking space for every 5 dwellings | Complies Six car parking spaces for 12 units are proposed, consisting of 3 standard spaces and 2 accessible parking spaces in the main at-grade car park and 1 separate adaptable space off the separate driveway at the southern end of the development. |

Other State Environmental Planning Policies

Table 13 below outlines applicability of, and compliance with, other State and Environmental Planning Policies (SEPPs).

Table 13 Compliance with other applicable State and Environmental Planning Policies

| State Environmental Planning Policy | Applicability |
| --- | --- |
| SEPP (Building Sustainability Index: BASIX) 2004 | A BASIX Certificate has been obtained for the development proposal, as required under the SEPP (refer to Appendix K). |
| SEPP (Transport and Infrastructure) 2021 | The site has frontage to Mundamatta Street, which is not located in close proximity to a State Classified Road, nor is the development defined as a major traffic generating development as per section 2.122 and Schedule 3 of this SEPP. |
| SEPP (Biodiversity and Conservation) 2021 | Clause 2.6(1) of this SEPP requires a permit from Council for clearing of vegetation required under the policy. Notwithstanding, Clause 6 of Housing SEPP specifies that development permitted without consent may be carried out without another consent or a licence, permission, approval or authorisation otherwise required under another environmental planning instrument. This means the proposed removal of trees within the site can be included within the REF scope and does not require a permit from Council. Please refer to Table 4 in section 5.1.4 of the REF for consideration of the proposal against the factors to be taken into account concerning activities in regulated catchments. |
| SEPP (Resilience and Hazards) 2021 | Section 4.6 of this SEPP requires the consent authority to consider whether land is contaminated prior to granting development consent. The site is located within an established residential area of Villawood, which has a history of residential uses. LAHC historical layout plan No. L1482 (Appendix Q) indicates that the area was being developed for residential purposes from as early as 1953. The s10.7 Planning Certificates have not identified the site as potentially contaminated (refer to Appendix G). A standard recommended Identified Requirement (Nos. 17 and 63) requires implementation of management measures in the event of contamination during construction works. |

Local Planning Controls

Canterbury Bankstown Local Environmental Plan 2023 (CBLEP 2023)

Compliance with the relevant provisions / development standards set out in the CBLEP 2023 is demonstrated in Table 14 below.

Table 14 Canterbury Bankstown Local Environmental Plan 2023

| Relevant Provisions / Development Standards for Seniors Housing |  |  |  |
| --- | --- | --- | --- |
| Clause | Provision / Development Standard | Required | Provided |
| 4.3 | Height of Buildings | (2) The height of a building on any land is not to exceed the maximum height shown for the land on the Height of Buildings Map (9 metres) | Maximum building height (measured in accordance with the LEP definition) is 7.2 metres. |
| 4.4 | Floor Space Ratio | (2) The maximum floor space ratio for a building on any land is not to exceed the floor space ratio shown for the land on the Floor Space Ratio Map. The site is mapped as containing a maximum floor space ratio on the Floor Space Ratio Map of 0.5:1. | Proposed FSR is 0.36:1. |

Canterbury Bankstown Development Control Plan 2023

Canterbury Bankstown Development Control Plan 2023 (CBDCP 2023) does not contain specific development controls for seniors housing. As such, the building setbacks for multi-dwelling housing have been used for comparative purposes to demonstrate the suitability of the site for the proposed scale of the development as shown in Table 15 below. The general controls for all development set out in CBDCP 2023 have generally been addressed in the various sections of this REF that address compliance with the provisions of the Housing SEPP.

Table 15 Canterbury Bankstown Development Control Plan 2023

| - Compliance with setback controls for multi-dwelling housing |  |  |
| --- | --- | --- |
| Clause | Requirement | Proposed |
| 7.1 Isolation of allotments | The consent authority must not grant consent to any development on land within Zone R4 High Density Residential if the proposed development will have the effect of isolating land with an area of less than 1,200m2 and a width of less than 20 metres at the front building line so as to preclude the reasonable development of that land. | - N/A - The site is zoned R2 Low Density. |
| - 7.2 Storey limit | The storey limit for multi dwelling housing is 3 storeys. Council does not allow attics. | - The proposal will not contain more than 2 storeys, and no attics are proposed. |
| - 7.3 Setbacks to the primary and secondary frontages | - The minimum setback for a building wall to the primary and secondary frontages is 5.5 metres. | - The proposal’s front building wall is setback 5.5 metres from the street frontage. A minor variation is proposed at the balcony of unit 11 with a setback of 4.9m. The variation is relatively minor at 600mm for a single balcony, and serves to provide useable open space for the unit and contribute to façade articulation. |
| - 7.4 Setbacks to the side boundaries | - The minimum setback for a building wall to the side boundary is 0.9 metres except for east–west orientated allotments of land where the minimum side setback to the southern boundary is 3 metres. The intended outcome of the 3 metre setback is to minimise overshadowing on adjoining properties. | - The proposal has a side set back on the southern boundary of 1.929m on the ground floor and 3.0m on the first floor. As per the shadow diagrams in the architectural drawings, the development allows for the adjoining properties to maintain 3 hours of midwinter solar access. |
| - 7.5 Private open space | - Multi dwelling housing must provide a minimum 50m2 of private open space per dwelling behind the front building line. This may be in the form of a single area or a sum of areas per dwelling provided the minimum width of each area is 5 metres throughout. | - Each ground level unit has a private open space with an area of at least 15m2 in accordance with Housing SEPP non-discretionary standards, and each upper level unit has a balcony with minimum area of 9.54m2, which only slightly varies from the Housing SEPP non-discretionary standard of 10m2. - Additionally, residents will be able to enjoy the central landscaped area, which will feature seating and open turf for activities. - This private open space and landscape outcome is considered appropriate for seniors housing. |
| - 7.6 Density | - The site area per dwelling (excluding the area of access handles or rights of way for access) must not be less than 175 square metres. | - The site area per dwelling is 184.1m2 (2,210.36m2 / 12 units). |
| - 7.7 Access to sunlight | - At least one living area of each dwelling must receive a minimum 3 hours of sunlight between 8.00am and 4.00pm at the mid–winter solstice. Council may allow light wells and skylights to supplement this access to sunlight provided these building elements are not the primary source of sunlight to the living areas. | - 10 or 83 % of the units have a living area that receives a minimum of 3 hours sunlight. This complies with the Housing SEPP, which requires at least 70% of the development to achieve 3 hours of midwinter sunlight. |
| - 7.8 Access to Sunlight | - At least one living area of a dwelling on an adjoining allotment must receive a minimum 3 hours of sunlight between 8.00am and 4.00pm at the mid–winter solstice. Where this requirement cannot be met, the development must not result with additional overshadowing on the affected living areas of the dwelling. | - The adjoining properties to the south will maintain at least 3 hours of midwinter solar access from 9am as illustrated in the shadow diagrams in Appendix A. |
| - 7.9 Access to Sunlight | - A minimum 50% of the private open space required for each dwelling and a minimum 50% of the private open space of a dwelling on an adjoining allotment must receive at least 3 hours of sunlight between 9.00am and 5.00pm at the equinox. Where this requirement cannot be met for a dwelling on an adjoining allotment, the development must not result with additional overshadowing on the affected private open space. | - Greater than 50% of each unit’s private open space area and the private open spaces areas of the adjoining lots to the south will receive least 3 hours of direct sunlight. |
| - 7.10 Visual Privacy | - Where development proposes a window that directly looks into the living area or bedroom window of an existing dwelling, the development must: - (a) offset the windows between dwellings to minimise overlooking; or - (b) provide the window with a minimum sill height of 1.5 metres above floor level; or - (c) ensure the window cannot open and has obscure glazing to a minimum height of 1.5 metres above floor level; or - (d) use another form of screening to the satisfaction of Council. | - The upper floor bedrooms on the south side of the development (units 11 & 12) exceed the minimum setback from the side boundary. Also, the windows to these rooms have a sill height of 1.6m above floor level. |
| - 7.11 Visual Privacy | - Where development proposes a window that directly looks into the private open space of an existing dwelling, the window does not require screening where: - (a) the window is to a bedroom, bathroom, toilet, laundry, storage room, or other non–habitable room; or - (b) the window has a minimum sill height of 1.5 metres above floor level; or - (c) the window has translucent glazing to a minimum height of 1.5 metres above floor level; or - (d) the window is designed to prevent overlooking of more than 50% of the private open space of a lower–level or adjoining dwelling. | - The windows that overlook the private open space of neighbours are windows to bedrooms and bathrooms, which are not required to be screened. |
| - 7.12 Visual Privacy | - Council does not allow multi dwelling housing to have roof–top balconies and the like. | - The proposal does not include roof-top balconies. |
| - 7.13 Building design (car parking) | - Multi dwelling housing must provide: - (a) a minimum 1 car space per dwelling with 2 or less bedrooms; and - (b) a minimum 2 car spaces per dwelling with 3 or more bedrooms; and - (c) must locate the car parking spaces behind the front building line. | - The parking provision complies with the requirements that are set out in the Housing SEPP. Parking spaces are behind the front building line. |
| - 7.14 Landscaping | Development must retain and protect any significant trees on the allotment and adjoining allotments. To achieve this clause, the development may require a design alteration or a reduction in the size of the multi dwelling housing. | - Two trees within/adjacent to the site have been identified as having high retention value, and these trees will be retained. Refer to the Arboricultural Impact Assessment at Appendix J for further detail. |
| - 7.15 Landscaping | - Development must landscape the following areas on the allotment by way of trees and shrubs with preference given to native vegetation endemic to the City of Bankstown (refer to Appendix 4 and Appendix 5 for a list of suitable species): - (a) a minimum 45% of the area between the multi dwelling housing and the primary frontage; and - (b) plant at least one 75 litre tree between the multi dwelling housing and the primary frontage (refer to Appendix 5 for a list of suitable trees in the City of Bankstown or Appendix 6 for allotments that adjoin the Hume Highway) | - The area between the front building line and the street frontage contains at least 45% of landscaped area. The proposal does not involve the planting of a 75L tree however there is the retention of a large tree (22m high) in the centre of the site. |

Notification, Consultation and Consideration of Responses

Copies of the notification letters sent to the local council and to the adjoining occupiers are provided in Appendix F, together with copies of all responses received and a record of any verbal responses.

Council Notification

In accordance with section 108C of the Housing SEPP, Canterbury Bankstown Council was notified of the development by letter dated 28 November 2022 (refer to Appendix F). The notification response period formally closed on 16 December 2022 and Council responded to the notification by email dated 16 December 2022 with matters raised outlined in Table 16 below.

The comments provided by council pre-date the CBLEP2023 and CBDCP2023 coming into force. It is noted that in relation to the matters discussed below there are no significant changes between the previous controls and the current controls. Further, the CBDCP2023 is still divided into former Bankstown and former Canterbury LGAs in relation to the controls.

Table 16 Issues raised in Council submission

| Issues raised | Response |
| --- | --- |
| - 1. Streetscape - While Council does not have any specific controls for Seniors Housing, develop of this type would be considered attached dwelling and be assessed under Part C3 in Canterbury DCP 2012. Part C3.4 in the Canterbury Development Control Plan 2012 provides building design guidance to developers and encourages new buildings to be consistent or complimentary to the streetscape. The existing streetscape is typified by “brick and tile” finishes whereas the proposal relies heavy on rendered or treated grey finishes. LAHC is advised to reduce the quantity of grey finishes and increase the quantity of brick (medium colour range) finishes so as to provide a balanced street presentation that better compliments the streetscape. | The development has been considered against the provisions of Bankstown DCP in Table 15 as Villawood is a suburb of the former Bankstown local government area. The design has been updated to increase brickwork to the street façade as requested. The brick flanking both entry lobbies has been raised to the underside of the level 1 windowsill. This has increased the brick quantity in the street elevation by 19.4m2 (10%). Proposed total street elevation percentage is now 67.1%. |
| - 2. Ausgrid - The application is unclear as to the need for Ausgrid facilities to be installed. You are invited the [sic] clarify if new Ausgrid facilities are required as part of, or as a consequence of, this development. | - LAHC has contacted Ausgrid regarding the development. Ausgrid has issued a connection letter offer. - The development will require multiple national metre identifiers, which will be applied for in due course. |
| - 3. Street tree - The proposal includes the removal of street tree(s). As these are Council property they cannot be removed via this process and LAHC will require consent from Councils Tree Management unit for their removal (and replacement). | - LAHC liaised with Council’s tree management unit regarding the removal of street trees, and Council confirmed that the 3 street trees along the site’s frontage can be removed and replaced with 6 trees equally spaced between driveways (refer to correspondence in Appendix F). Council recommended a condition, which has been incorporated as an Identified Requirement (No. 82). |
| - 4. Landscape - The landscape plans are silent on site maintenance. How will LAHC maintain the site’s landscaping, including the nature strip? | - Landscape will be maintained by the builder for 12 months after handover. As this is a seniors living development, LAHC Asset Division will engage a landscape contractor to maintain landscape thereafter. |
| - 5. Development Engineer - Comments received from Development Engineer 03.01.2023: - a. Additional stormwater pits are required in the rear yards to be directed to the OSD system. - b. OSD bypass areas shall not be more than 25% of the total site area. - c. As no calculations provided with the plans the OSD volume shall limit discharge to predevelopment flows for all storms up to the 1% AEP including bypass areas. - d. The interallotment drainage pipe shall be sized to 5% AEP. | - Updated stormwater plans (Appendix C) have been prepared in response to Council’s comments. - a. A new pit and pipe system across the rear yard is not achievable due to the large TPZ zones The updated stormwater plans provide for multiple grated drains, which will catch a large majority of the grassed area and only leave a small bypass at the back of site. - b. The updated stormwater plan has approximately 9% bypass which is well below Council’s allowable limit. - c. This was accounted for in the original design, and a DRAINS screenshot has been included in the civil drawings indicating 1% AEP pre to post OSD site discharge. - d. The consultant design civil engineer has confirmed (by email dated 17/05/2023 – see Appendix M) that the interallotment drainage pipe has been sized to the 5% AEP event. |
| - 6. Resource Recovery - This DA has been assessed in accordance with Section B9 (Waste Management), page 203 to 210, Canterbury DCP 2012. However, to achieve best practise and to cater for the future alignment of waste collection services, the requirements for this development may differ from the Canterbury DCP and aligned with the draft consolidated Canterbury Bankstown DCP and Waste Design Guide for New Developments (Guide B – Multi Dwelling Housing). Best practise will ensure that the waste management facilities benefit residents and achieve a high operational efficiency. - a) To assist during re-submission - use the template and guides provided for Demolition and Construction Waste Multi Dwelling Housing for New Developments - found here or URL:https://www.cbcity.nsw.gov.au/development/planning-control-policies/canterbury-bankstown-dcp-waste-management - b) A Waste Management Demolition/Construction Plan has not been submitted. Areas to address: - i. Confirmation if the development involves the removal of asbestos, quantities, the licence details of asbestos removalist and the designated disposal site licensed to accept asbestos-related waste; - ii. Details regarding the types of waste and likely quantities of waste to be produced; - iii. Designation of appropriately licensed facilities (recycling and landfill) to receive the demolition and construction waste; - iv. Confirmation that all waste going to landfill is not hazardous; and - v. Ensure over 80% of this material is recycled. If not, explain why. c) Waste Configuration: This development, containing 12 residential dwellings needs to increase the number of bins. Fortunately, these extra bins will fit in the proposed bin storage areas. Updated the WMP and architectural plans to reflect the correct number of bulk bins. - c) Waste Configuration: This development, containing 12 residential dwellings needs to increase the number of bins. Fortunately, these extra bins will fit in the proposed bin storage areas. Updated the WMP and architectural plans to reflect the correct number of bulk bins. - d) - e) First communal bin storage area - i. 2 x 660L bulk garbage bins (collected weekly) - ii. 2 x 660L bulk recycling bins (collected fortnightly) - iii. 1 x 240L garden organics bins (collected fortnightly) - f) Second communal bin storage area - i. 2 x 660L bulk garbage bins (collected weekly) - ii. 2 x 660L bulk recycling bins (collected fortnightly) - iii. 1 x 240L garden organics bins (collected fortnightly) - g) Garden Organics Bins: It is the responsibility of the property manager or residents to present any provided garden waste bins to the kerbside for collection by Council the evening before the designated collection day. An area of at least 2m (l) x 1m (d) needs to be provided for this purpose. - h) Bulky Waste Room - i. A bulky waste room is to be included in the building design. The room must be at least 4m2; - ii. This is to be a separate room and signed as such on the door with at least 2m wide door opening outwards; - iii. The area must be no more than 5m from kerbside collection point with no steps/excessive slopes/obstacles and must have a maximum grade of 1:30 (3%); and - iv. Servicing must not be within or outside the access way/driveway, as this is a conflict point for collection crews and vehicles. | The development has been designed to the requirements of Bankstown DCP as Villawood is a suburb of the former Bankstown local government area. a. Updated waste management plans have been prepared using Council’s templates (Appendix R). b. Identified Requirement (No.1) refers to preliminary waste management plans which utilise Council’s templates. Identified Requirement (No.66) requires non-recyclable waste containers to be collected and disposed of at a licensed landfill or other disposal site, which will be detailed in a final Waste Management Plan required in Identified Requirement (No.37). Identified Requirement (No.81) requires a Site Clearance Certificate for all three lots to confirm the site is free of asbestos. c. Bin numbers have been updated as per Council’s advice. No green waste bins have been provided. Landscaping green waste is to be taken offsite by the relevant contractor, as indicated in section 2 of the waste management plan (Occupancy Stage) in Appendix R. d-e. Council’s rates used in updated waste management plan (Appendix R). - g. No green bins are provided, as green or garden waste is managed and removed by LAHC contractor, as indicated in section 2 of the waste management plan (Occupancy Stage) in Appendix R. h. Residents will instead have access to local council arranged pickups for bulky goods. In line with Councils WMP guidelines, developments of between 6-50 dwellings have access to up to 4 collections per annum. Bookings to be made by building’s caretaker, building manager or strata manager on behalf of the whole building. |

Notification of Occupiers of Adjoining Land and Other Persons

Under section 108C(1)(a) of the Housing SEPP, the Council for the area was requested to nominate any other persons who should, in the Council’s opinion, be notified of the development.

Advice was sought from Council regarding additional persons or property that should be notified of the development via an email dated 12 July 2022. Council provided an email response on 14 July 2022 advising that the notification map is acceptable to Council. Figure 13 illustrates the properties in which the occupiers were notified of the development.

Figure 13 Map of Properties Notified of the Proposed Development

(Source: LAHC)

Under section 108C(1)(b) of Housing SEPP, occupiers of adjoining land, as identified in the above map, were notified of the proposed development activity by letter dated 28 November 2022. Copies of the notification letters are provided at Appendix F.

The notification response period formally closed on 16 December 2022, and no submissions were received.

Notification of Specified Public Authorities

The development is “seniors housing” under section 108A of the Housing SEPP. As required by section 108B(2) of the Housing SEPP, consideration has been given to the need to notify the “specified public authorities” identified in State Environmental Planning Policy (Transport and Infrastructure) 2021, sections 2.15 and 2.17. The development is not located in an area that triggers the requirement to notify public authorities other than Council.

Review of Environmental Factors

Environmental factors associated with the proposed activity in terms of location, character, bulk and density, privacy, solar access and overshadowing have been considered in accordance with the provisions of the Housing SEPP and discussed in section 5.1.10 of this REF. A review of other environmental factors associated with the proposed activity, and the measures required to mitigate any adverse impacts to the environment, are provided below.

Neighbourhood Character

- The site is located within an established low-density residential area characterised by a mix of older style single storey detached dwelling houses of brick construction and newer two storey brick dwelling houses. A 2-storey detached dwelling house adjoins the site to the north and to the south, whilst directly opposite to the west, the developments consist of single storey dwellings.

- The wider surrounding locality is generally comprised of single storey dwelling houses with some two-storey dwelling developments interspersed.

- The development is 2-storeys stepping down to single storey at the rear which is compatible with the neighbourhood character. The development is articulated by varied setbacks, roof pitches and materials, two building lobbies and a balcony. The development provides a front landscaped setting, a communal garden and perimeter plantings to soften the built form providing for an overall positive contribution to the streetscape.

- Mitigation Measures

- No mitigation measures are required.

Bulk and Density

The proposed development complies with Council’s LEP controls for height and FSR. Council’s LEP specifies a height control of 9m, whereas the height of the proposed development is 7.2m. A floor space ratio of 0.36:1 is proposed which is below the maximum floor space ratio of 0.5:1 specified in CBLEP 2023. The development also complies with the dwelling density requirement in the DCP of minimum 175m2 of site area per dwelling, with 184.1m2 site area per dwelling proposed. The density and scale of the development are considered appropriate in accordance with the controls set out within the SEPP, Seniors Living Policy: Urban design guidelines for infill development, as well as local planning controls.

The proposal has appropriately considered the context of the site and has been designed to complement its surroundings. Any impacts from the multi dwelling housing typology have been mitigated by the provision of a compliant building height and density, landscaped area, private open space and deep soil in excess of minimum requirements, which ensures the development makes a positive contribution to the streetscape and general locality.

On balance, the proposed development provides an acceptable bulk and scale that is compatible with the single and two-storey residential character of the locality.

- Mitigation Measures

- No mitigation measures are required.

Streetscape

- The proposed development activity will make a positive contribution to the streetscape of Mundamatta Street, by virtue of the following:

- The proposed development will infill a vacant lot and replace ageing housing stock that has reached the end of its economic life with a new contemporary, architecturally designed residential development.

- Living rooms and balconies address and provide an active frontage to Mundamatta Street.

- The front façade is modulated by projecting and recessed elements and further articulated through the use of different materials and textures.

- The car park is located toward the rear of the site, in large part screened by the building and landscaping.

- Significant landscaping is provided throughout the development, including within the front setback and plantings adjacent to the driveways.

- Mitigation Measures

- No mitigation measures are required.

Visual Impact

The proposed development will make a positive contribution to the residential streetscape through construction of new contemporary dwellings and improved landscaping that respond to the site context. The building will be articulated by a balance of solid and glazed and recessed and projecting elements, and varied roof pitches. The development will sit within a landscaped garden setting, with existing and new plantings softening the built form and adding to the long-term visual amenity of the neighbourhood.

- Mitigation Measures

- No mitigation measures are required.

Privacy

A reasonable level of privacy will be maintained by the proposed development, by virtue of the following:

- All balconies, except for unit 11, are oriented internally into the site and not across the rear boundary, and the balcony of unit 11 faces the street.

- The development steps down to single storey at the rear of the site.

- No living areas are oriented to the side boundary. Bedrooms and bathrooms are adjacent the side boundary, with DCP-compliant setbacks.

- Mitigation Measures

- No mitigation measures are required.

Solar Access

The proposed building has been designed to maximise direct sunlight access in midwinter. 10 of the 12 units (83%) will achieve at least 3 hours of solar access to their living areas and POS at midwinter, which complies with the minimum of 70% stipulated in section 108 of the Housing SEPP.

- Mitigation Measures

- No mitigation measures are required.

Overshadowing

The shadow diagrams in Appendix A indicate that the adjoining dwellings will continue to receive a minimum 3 hours of midwinter solar access to primary living and open space areas. The development complies with side setback requirements and steps down at the rear, to maintain at least 3 hours of midwinter solar access to the south-adjoining neighbours at 14 and 16 Mundamatta Street.

- Mitigation Measures

- No mitigation measures are required.

- 7.8 	Traffic & Parking

Six surface car parking spaces for residents, including 2 accessible spaces and 1 adaptable space, are proposed. The provision of on-site car parking meets the non-discretionary parking requirements set out in the Housing SEPP for developments carried out by LAHC. Unrestricted street parking is available on Mundamatta Street to accommodate any overflow parking demand generated by the proposed development.

The Traffic and Parking Assessment Report (Appendix S) indicates that the development will have a projected nett decrease of 2 vehicles per hour in the AM peak hour to 0 vehicle trips and a nett increase in 3 vehicles per hour in the PM peak hour to 5 vehicle trips. The projected nett change in traffic activity as a consequence of the development proposal is negligible and will not have any unacceptable implications in terms of road network capacity.

The Traffic and Parking Assessment Report examined the adequacy of the proposed internal driveways and parking arrangement. The assessment confirmed that the design is suitable based on an assessment of the project vehicle movements as it demonstrates a B99 design vehicle will be able to enter and exit the site whilst driving in a forward direction.

Mitigation measures

No mitigation measures are required.

Flora and Fauna

An Arboricultural Impact Assessment has been prepared for the site by Arterra Design (Appendix J). The report considers 25 trees, including 15 within the site, 4 street trees on Mundamatta Street and 6 within neighbouring properties.

The proposal includes the removal of 21 trees, including 14 trees within the site, 3 street trees and 4 trees in the area of the proposed drainage easement at 82 Lowana Street.

The 14 trees within the site, the 4 trees at 82 Lowana Street and one of the street trees are required to be removed to accommodate the proposed development (as discussed in the Arboricultural Impact Assessment in Appendix J). LAHC owns 82 Lowana Street and consents to the removal of the 4 trees within that property.

Council has advised they support the removal of all 3 street trees along the site’s frontage, subject to the planting of 6 replacement trees - see correspondence in Appendix F. The removal and replacement of the street trees has been endorsed by Council, and Council’s advice has been incorporated as an Identified Requirement (No. 82).

All of the trees to be removed have low or medium retention value. The high retention value tree within the site (mature Spotted Gum) is being retained.

Appropriate replacement planting is proposed as indicated on the landscape plan and details. The new plantings will provide replacement tree cover on the site and increase the variety of species, including flowering trees, which will provide additional habitat for fauna in the long-term.

There will be no significant adverse impact on native fauna or flora as a result of the proposed development given that compensatory planting is proposed.

Mitigation measures

Works are to be carried out in accordance with the tree-sensitive construction methodologies and procedures specified at sections 3.2 to 3.10 of the Arboricultural Impact Assessment Report (refer to Identified Requirement No.79).

Replacement street trees are to be provided in accordance with Council’s recommendations (refer to Identified Requirement No. 82).

Heritage (European / Indigenous)

No heritage items are identified in the City of Canterbury Bankstown Council’s section 10.7(2) & (5) Planning Certificates located on or in the vicinity of the site.

Aboriginal Heritage

- An Aboriginal Heritage Information Management System (AHIMS) search dated 18 January 2023 (Appendix I) did not find any record of Aboriginal Sites or Places on the site or in the surrounding locality and the site is considered to be disturbed land. Consideration of the Due Diligence Code of Practice for the Protection of Aboriginal Objects in New South Wales, determined that no additional investigation was warranted. Discovery of cultural material during development activities cannot be ruled out, however, an identified requirement has been applied should any Aboriginal relics be discovered on the site during excavation/construction.

Other Cultural Heritage

No cultural heritage items have been identified in Canterbury Bankstown Council’s section 10.7(2) & (5) Planning Certificates and the likelihood of any heritage relics being discovered during excavation / construction is considered to be minimal.

Mitigation Measures

Identified Requirements (Nos. 46 and 47) have been applied should any cultural heritage relics be discovered on the site during excavation / construction.

Soils / Contamination / Acid Sulfate Soils / Salinity

Geotechnical

A Geotechnical Site Investigation Report prepared by STS Geotechnics (Appendix P) indicates the following:

- The subsurface conditions generally consist of topsoil overlying silty clays and weathered shale. The topsoil is present from the surface to depths of 0.2 and 0.3 metres. Firm to stiff becoming very stiff with depth, underlie the topsoil to depths of 1.3 to 1.5 metres. In BH2 auger refusal occurred in this material at a depth of 0.6 metres. Weathered shale underlies the silty clays to the depth of auger refusal, 1.8 to 2.0 metres.

- Groundwater was not observed during drilling works.

Mitigation Measures

No mitigation measures required.

Contamination

According to Council’s section 10.7(2) & (5) Planning Certificates, Council is not aware of the land being affected by any matters prescribed by section 59(2) of the Contaminated Land Management Act 1997.

The site is located within a developed residential area of Villawood, which has a history of residential uses. LAHC historical layout plan No. L1482 (Appendix Q) indicates that the area was being developed for residential purposes from as early as 1953. LAHC records indicate 20 Mundamatta Street was demolished in accordance with a Complying Development Certificate approval of 2016/2017.

- Given the long-term continuous use of the land for residential purposes, the highly disturbed nature of the site, and the section 10.7 Certificates have not identified the site as being potentially contaminated, it is unlikely the subject land is affected by contamination.

Mitigation Measures

Identified Requirements (Nos. 17 and 63) are recommended to cover the possibility of discovering site contamination during demolition / construction works.

An Identified Requirement (No.81) is recommended to require the preparation of a final Site Clearance Certificate confirming the all three lots are clear of contaminants post-demolition.

Salinity

- The Geotechnical Site Investigation Report prepared by STS Geotechnics (Appendix P) indicates that the site is not affected by salinity. The report indicates the electrical conductivity (ECe) values of 0.63 and 1.8 dS/m for the site which are consistent with the presence of non-saline soils, with reference to Site Investigations for Urban Salinity (DLWC, 2002).

Mitigation Measures

No mitigation measures are required.

Drainage / Flood Prone Land / Hydrology/ Water Quality

Stormwater will be collected via a series of stormwater pits and pipes connected to a below ground on-site detention (OSD) tank connected to an outlet pipe that will drain through 82 Lowana Street to the north east through a 1.2 metre easement and connect to a new kerb inlet pipe on Lowana Street, and then connect to an existing kerb inlet pit in front of 80 Lowana Street.  The construction of the outlet pipe through 82 Lowana Street does not impact on the existing dwelling on this site.  Four trees (of low retention value as per the Arboricultural Impact Assessment Report) and low scale landscaping at 82 Lowana Street are to be removed to enable trenching and connection for stormwater to Lowana Street.

Roof water will be collected from downpipes that will connect to the gutters that flow into the underground rainwater tank for recycling with overflow connected to the underground detention tank.

The section 10.7(2) & (5) Planning Certificates issued by Council for the subject site indicate that the land is not subject to flood related development controls.

The proposed development has been designed to have no adverse impact on the hydrology or water quality within the local area.

Mitigation Measures

Identified Requirements (Nos. 6-9, 14, 42 & 72) are recommended to ensure that stormwater drainage is managed in accordance with legislative requirements.

An Identified Requirement (No. 43) is recommended to ensure the plan of drainage easement through a neighbouring lot (owned by LAHC) is lodged prior to commencement of works and registered prior to occupation.

An Identified Requirement (No. 13) recommends that sediment control measures be implemented during demolition/construction in accordance with Council requirements and/or the guidelines contained in the Blue Book Managing Urban Stormwater: Soils and Construction (4th edition, Landcom, 2004).

Bushfire Prone Land

The section 10.7(2) & (5) Planning Certificates issued by Canterbury Bankstown Council for the subject site advise that the land is not bushfire prone.

Mitigation Measures

No mitigation measures are required.

Noise and Vibration

During Demolition / Construction

During demolition/construction typical noise levels associated with demolition/ building works will be generated within the hours consistent with the requirements of complying development.

During Occupation

Noise generated when the proposed buildings are completed and occupied will be entirely in keeping with their residential surroundings. No major plant or equipment, which would generate unacceptable noise during occupation, will be installed in the proposed development.

Buildings will be constructed to comply with the deemed-to-comply provisions of the Building Code of Australia with respect to noise transmission. The building will be designed and constructed to achieve internal noise level design targets.

- The Noise Intrusion Assessment prepared by Acoustic Logic (Appendix V) indicates noise levels in the development, including from traffic noise, would comply with Council and state government guidelines subject to the implementation of recommendations in respect of external wall, ceiling and roof construction, glazing and the sealing of openings.

Mitigation Measures

Demolition/construction noise will be controlled to within acceptable limits by sound attenuation measures and undertaking construction activities within EPA/local council requirements.

To address road noise, it is recommended that the building envelope (including glazed elements, external roof, ceiling and wall construction) and ventilation strategies (including any openings) be designed such that an internal noise level of ≤ 40 dB LAeq,15hrs can be achieved within all spaces except for bedrooms where an internal noise level of ≤ 35 dB LAeq,9hrs is preferred.

The building will be constructed to comply with the deemed-to-comply provisions of the Building Code of Australia and EPA criteria with respect to noise transmission.

Identified requirements (Nos. 2 and 60) are recommended to ensure compliance with the above mitigation measures, including carrying the development substantially in accordance with the acoustic design report.

Air Quality

Temporary and localised air quality impacts including dust, smoke, grit, odours, and fumes might be generated during the clearing and excavation of the site and construction of the proposed development.

Mitigation Measures

Standard identified requirements (Nos. 61, 63, 64, & 65) are recommended to mitigate any potential or adverse impacts on air quality.

Waste Minimisation

The following waste minimisation and management measures have been identified and are to be considered in conjunction with the specific details, including the estimated quantities of waste, provided in the final waste management plan prepared by Custance (Appendix R).

During Demolition

Demolition materials will be stored wholly within the site prior to removal for recycling or disposal.  Demolition waste will be removed from the site to an approved waste management facility or will be recycled.

Specific intentions for recycling / re-use / disposal of demolition waste will be determined by the demolition contractor prior to commencement of demolition.

During Construction

Construction materials must be stored wholly within the site prior to removal for recycling or disposal.  Construction materials waste must be removed from the site to an approved waste management facility or shall be recycled generally as follows:

- bricks, tile and concrete to be transported to building recycling facility;

- concrete shall be crushed and reused for filling, levelling or temporary road base;

- tiles shall be crushed and reused for filling, levelling or temporary road base;

- timber shall be sent to second hand suppliers;

- plasterboard shall be sent to building recycling facility; and

- metal offcuts from gutter and downpipes, etc. shall be recycled wherever possible.

During Occupation

General and non-recyclable waste will be disposed of in Council’s standard waste storage bins located in the garbage storage enclosures and placed on the street kerb by LAHC contractor for collection by Council’s waste services. Four 660L household waste (red) bulk bins are required, two per communal bin storage area.

Paper/metal/glass will be disposed of in Council’s standard waste recycling bins to be located in the garbage storage enclosures and placed on the street kerb by LAHC contractor for collection by Council’s waste services. Four 660L recyclable waste (yellow) bulk bins are required, two per communal bin storage area.

- No green bins are provided, as green or garden waste is managed and removed by LAHC contractor.

Mitigation Measures

Identified Requirements (Nos. 34, 48-56 & 66) are recommended to ensure construction/demolition waste is appropriately managed and disposed of.

A Identified Requirement (No. 37) is recommended to require the preparation of a final waste management plan for the demolition, construction and occupation phases of the development.

Resource Use & Availability

The proposed activity will not result in any discernable depletion or degradation of natural resources. The proposal has been designed to meet water and energy efficiency targets as demonstrated by the BASIX certificate for the proposal (Appendix K).

The proposed development is for the replacement of existing housing that has reached the end of its economic lifespan. The proposed development will provide contemporary housing that will satisfy current State Government environmental sustainability requirements, particularly through improved energy and water efficiency. These factors will ensure reduced depletion and degradation of natural resources in the long term.

Mitigation Measures

No additional mitigation measures are required.

Community / Social Effects

The proposed development will generate a number of positive community and social effects, including:

- assist LAHC in meeting its significant, long-standing and continually-growing demand for social housing in the Canterbury Bankstown local government and surrounding area;

- assist LAHC in improving the amenity of accommodation for its tenants, by providing new, more appropriate housing aligning with demand for social housing;

- improve the environmental sustainability of housing on the site, particularly through improved energy and water efficiency; and

- provide more accessible housing on the site.

Mitigation Measures

No mitigation measures are required.

Economic Impact

The proposed development is likely to contribute to a range of economic benefits in the Canterbury Bankstown local government and surrounding areas through:

- more efficient use of land resources, existing infrastructure and existing services;

- local sourcing of construction materials, where possible;

- the local sourcing of tradesmen and other construction-related professionals, where possible;

- on-going consumption from new / additional households;

- the reduced maintenance costs of the newer housing; and

- savings associated with improved energy and water efficiency.

Mitigation Measures

No mitigation measures are required.

Cumulative Impact Assessment

The proposed activity is not likely to have singular or cumulative environmental impacts which would result in unacceptable adverse effects for the following reasons:

- The proposed activity will not result in any adverse cumulative impact when considered in conjunction with any other proposals or developments in the area;

- there will be no synergistic effects of individual project impacts from the proposed activity when considered in combination; and

- there are no known environmental stresses in the area of the proposed activity that would be increased.

Mitigation Measures

No mitigation measures are required.

Conclusion

Summary of Key Issues Raised in Assessment

The proposed activity, given its scale, location, and design, will be sympathetic with its residential environment. Following a review of the site constraints, it has been determined that the subject land does not contain any significant environmental hazards and that there are no key issues that have been identified that require further assessment.

The proposed activity has been considered in terms of the provisions of section 5.5 of the EP&A Act and section 171 of the EP&A Regulation. Following an analysis of the potential impacts associated with the proposed activity it was determined that an environmental impact statement is not required.

As demonstrated in this REF, the proposed activity is consistent with the relevant objectives and standards set out in the Housing SEPP, CBLEP 2023, and the design principles and better practices set out in the relevant guidelines.

In addition, and as demonstrated by the environmental impact analysis and assessment undertaken in this REF, the proposed activity will have environmental impacts that can be mitigated to an acceptable level in accordance with current applicable standards, will have a number of positive environmental effects in terms of the built environment and will deliver a range of social and economic benefits.

The proposed development will enable LAHC to meet the increasing demand for 1 and 2 bedroom seniors housing in the local area. Therefore, the proposed development is clearly in the public interest.

Recommendation

Given the above review of environmental factors, it is recommended that LAHC proceed with the proposed activity subject to the implementation of identified requirements.

APPENDIX A – ARCHITECTURAL PLANS

APPENDIX B – LANDSCAPE PLANS

APPENDIX C – CIVIL PLANS

APPENDIX D – SURVEY PLAN

APPENDIX E – NOTIFICATION PLAN

APPENDIX F – NOTIFICATION LETTERS AND RESPONSES

APPENDIX G – SECTION 10.7 PLANNING CERTIFICATES

APPENDIX H – ACCESS REPORT

APPENDIX I – AHIMS SEARCH

APPENDIX J – ARBORIST REPORT

APPENDIX K – BASIX CERTIFICATE

APPENDIX L – BCA REPORT

APPENDIX M – DESIGN COMPLIANCE CERTIFICATES

APPENDIX N – NatHERS CERTIFICATE

APPENDIX O – HOUSING FOR SENIORS CHECKLIST

APPENDIX P – GEOTECHNICAL INVESTIGATIONS

APPENDIX Q – TITLE SEARCH AND DP

APPENDIX R – WASTE MANAGEMENT PLANS

APPENDIX S – TRAFFIC REPORT

APPENDIX T – SURVEY FOOTPATH LONG SECTION

APPENDIX U – ENVIRONMENTAL SITE MANAGEMENT PLAN

APPENDIX V – NOISE IMPACT ASSESSMENT

APPENDIX W – SERVICES