| Task undertaken: | Container Stack Heights | Date: | 29 June 2021 |
| --- | --- | --- | --- |
| Location: | Tyne CS - MT Movements -Gate B54 Port Botany NSW |  |
| JSEA conducted by: |  |  |  |
| Assessment undertaken: - Risk Assessment or - Safe Work Method Statement |  |  |  |

| POTENTIAL HAZARDS |  |  |  |  |  |  |  |  |  |  |  | PERMITS/ CHECKLISTS |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Health and Safety |  |  |  |  |  |  |  | Environmental |  |  |  |  |  |
|  | Biological |  | Hot Work |  | Remote/isolated work |  | UV exposure |  | Air Emissions |  | Hazardous Chemicals |  | Confined Space |
|  | Confined Space |  | Ladders |  | Repetitive actions |  | Work at heights |  | Asbestos |  | Hazardous waste |  | Hot Work |
|  | Electrical hazards |  | Manual Tasks |  | Slippery surfaces |  | Working with/near crane |  | Breach of Licence |  | Leachate |  | Manual Tasks |
|  | Ergonomics |  | Noise |  | Suspended loads |  | Working with / near plant |  | Dust |  | Odour |  | Permit to Work |
|  | Excavation |  | Non skilled worker |  | Temperature extremes |  | High Wind |  | Explosion – dust/fines |  | Radioactive waste |  |  |
|  | Fatigue |  | Plant – moving parts |  | Traffic Management |  | Falling Containers |  | Gas |  | Release to storm water |  |  |
|  | Flammable materials |  | Reduced visibility |  | Trip Hazards |  |  |  | Groundwater Contamination |  |  |  |  |

|  |  |  |  |  |  |  |  |  |  | Other: (please specify) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| PLANT & EQUIPMENT USED DURING TASK | Current WI / MAN available | LIST OF CHEMICALS USED DURING TASK | Current SDS available |
| --- | --- | --- | --- |
| Empty Container Handler - Forklift |  | N/A |  |
| SOP 1, 4 and 5 |  |  |  |

| RISK IDENTIFICATION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Steps Task & Environment Assessed | Safety / Environmental Risk and Hazards | Inherent Risk Level | Control Measure’s implemented | Residual Risk Level | Person Responsible |
| Empty container stacking heights. | Hazard: - Containers falling if they are not stacked correctly. - High Wind Events. Existing Risk Control: - Wind monitor has been Installed. - Siren installed for high wind events. - Annual calibration on the wind monitor tower. - Wind monitored in real time. - When stacking containers on hard stand surfaces, ensure they are stacked no higher than as follows: - - 40ft Containers – maximum of six - (7) containers high - 20ft Containers – maximum of six - (7) containers high - 40ft High Cube Containers – maximum of six - (6) containers high - 20ft High Cube Containers – maximum of six - (6) containers high | High | - The current container stacking plan shows a maximum of 7 high, with pyramid stacking (from 2 high) on northern and southern boundaries. Stacks on the western boundary to be stacked at maximum of 5 high. - The SOPs 1, 4 and 5 are in place. - Ensure all employees are trained on SOP 1 Container Loading and Unloading, SOP 4 Empty Container Handler Operations and SOP 5 Weather Monitoring and Container Park Securement. - Continue to monitor the weather conditions and lower the containers as required based on the severity of risk. - Any stack “leans” to be restacked immediately. - Lower stacks in high winds as per Wind SOP #5. Single stacks must always be made safe asap. - Complete and record Last shift checks before closing and leaving the site. | Medium | Depot Manager. |
| Environmental Amenity | Container stack height impacts the environmental amenity for nearby commercial sites users and residents. | High | Containers not to be stacked higher than the maximum container stack heights allowable. | Medium | Depot Manager |
| Environmental Amenity | Traffic issues caused by site operations impact the amenity to nearby commercial site users and motorists. | High | Ensure all loading and unloading activities are undertaken wholly within the site boundary. Ensure that all vehicles entering or exiting the site do so in a forward direction. Actively manage the truck rank by physically directing the traffic on site. | Medium | Depot Manager |

# Risk Assessment Model

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short-term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| Low 1, 2 | Will require localised control measures. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on SWMS. |  |  |  |  |
| High 6, 9 | Stop immediately. Implement permanent Preventive controls. Will require detailed pre-planning. Actions will be recorded on a SWMS. |  |  |  |  |

TRAINING, QUALIFICATION & LICENCES REQUIRED TO COMPLETE WORK

National Licence to Perform High Risk Work Class: - LF - Forklift

ADDITIONAL COMMENTS & REQUIREMENTS

Induction to SOP#s 1, 4 and 5

| IDENTIFICATION OF TASK RISK MANAGEMENT AND REVIEW |  |  |
| --- | --- | --- |
|  | Non -Routine Task - this task is a one-off occurrence |  |
|  | Routine Task - this task is undertaken on regular basis | Task / Risk and hazards identified must be included in the Site Risk Register |

| REVIEW |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Date | Reviewed by | Relevant changes & modifications made to SWMS OR Risk Assessment |  |  |  |
| SIGN-OFF Manager or Authorised Representative |  |  |  |  |
| Reviewed by Manager or Authorised Company Representative: On review of the information supplied in this form the work is authorised to proceed Name: _______Rohen Drury_______________________________________________ Signature:__________________________________________ Date:________________________ Name: _______Aaron Powell ____________________________________________ Signature:__________________________________________ Date:________________________ |  |  |  |  |
| SIGN-OFF Worker |  |  |  |  |
| By Signing below you agree that you have READ and UNDERSTOOD this SWMS and the hazards and risks relevant to this task and AGREE TO COMPLY with all controls identified. |  |  |  |  |
| Full Name (Please PRINT name clearly) |  |  | Signature | Date |