Statement of Environmental Effects and Heritage impact

Property: 187 Milton Street Ashbury (Lot 2   DP  310344 )

The property has a frontage of 10.975 m with a depth of 33.53 m and site area of

367.9m2 by calculation.

The land is in an R2 zone and is on the western side of  Milton Street.

The proposed development involves the demolition of the rear section of the dwelling house and construction of an addition containing a laundry,bathroom, new kitchen , living room, rear deck and  WC, storeroom in lower level. Demolition of a fibro garage and construction of a brick garage

There is vehicular access to the rear of the property to the proposed garage.

The existing house has a gross floor area of  about  100.8 m2 and this will increased.

The existing FSR is  0.27 :1  =  100.82 m2 GFA and this will  be increase to 0.48:1. 177m2 GFA

The adjoining properties to the north and south are single storey brick houses.

Zone =R2 - The proposed development is permissible in the zone.

Conservation Area = Yes.- Ashbury Heritage Conservation Area

Heritage Item = No

Acid sulphate soils = class 5 - minimal excavation about 500mm into ground and no swimming pool proposed

Flood Affected = No

Air craft noise = No

Height - 8.5m max allowed height- proposed height is 6.0m.

| Standard | Proposal | % of non compliance | Compliances |
| --- | --- | --- | --- |
| Floor Space Ratio Required: [0.5:1] | 0.48:1(177m2 GFA) | N/A | Yes |
| At least 15% deep soil area (73.99m2) of the site will be landscaped area with 50% in rear yard | 25.4% of the site landscaped (93.5m2)with 61 m2 or 65% of landscaped area in rear yard | N/A | Yes |
| Height 8.5m to ridge and 7m wall height for dwelling | Wall height is 4.0m and ridge height is 6.0m | N/A | Yes |
| Wall height of garage 3.5m and 4.8m to ridge | 2.8m wall height and 3.7m ridge | N/A | Yes |
| Site coverage 60%(220.74m2) | 44.5% -164.05m2 | N/A | Yes |
| Maximum outbuilding area=30m2 | 20.5m2 | N/A | Yes |
| Deep soil Planting 15% =55.18m2 | 93.5m2=25.4% | N/A | Yes |
| Dwelling form 2 storey | Single storey when viewed from street | N/A | Yes |
| Rear setback 6m | Setback 8.8m m | N/A | Yes |
| Side setback dwelling 900mm | Setback 900mm | N/A | Yes |
| Side setback garage 450mm | Setback 450mm | N/A | Yes |
| Solar access to adjoining properties 3hours on 22 June | Greater than 3 hours | N/A | Yes |
| Basement -non habitable | Non habitable use | N/A | Yes |

Views.

The proposed development is of similar scale as the adjoining properties. A recent development application has been submitted to council for a similar size development on the adjoining property 185 Milton Street Ashbury.  There are no iconic,water or main views impacted by the development.

Privacy

Existing boundary fences maintain privacy. A1.8m louvred privacy screen on the southern side of the rear deck is proposed. The garage is to the rear of the property and screens views across 185 Milton Street Ashbury. The proposed development will not cause any further impact on privacy.

Solar Access

The proposed development will have minimal impact on solar access to the adjoining property to the south as there is a driveway to the south.

t is considered that the proposed development will comply with council's development control for solar access.

Streetscape.

The proposed development is using materials compatible with those used in the Ashbury Conservation Area and the new garage is at the rear of the property is sympathetic to the style and character of the dwelling house.

Heritage Conservation Area. The property is in the Ashbury Heritage Conservation Area. The development is low in scale and is considered to be sympathetic to the style and character of the existing Californian bungalow.   External materials are consistent with those used in the conservation area

Trees

There is a small tree in the rear yard which will remain. The development is clear of the tree.

Parking

The proposed garage will provide for one car parking space and space for bicycles.. The garage has a floor area of 20.5m2 The size of the garage is far less than the maximum size allowed for this type of structure under the Canterbury DCP The garage is replacing an asbestos clad garage. The garage is on the northern side of the property as such  does not affect the amenity of adjoining properties. The FSR and landscaping are compliant.

Open Space

The proposed open space will be reduced by the development however the proposed FSR and open space complies with councils controls.

The land is situated within a residential zone and the development is permissible with Council’s consent.

The proposed development is not against the public interest. There will be no adverse impact caused by the proposed development on the built and unbuilt environment.

The house on the property is more than 70years old and there is no known contamination of the land.

National Construction Code.

The proposed development (house addition) is to be setback 900mm from the closest side boundary and will comply with the requirements of the NCC. The garage is to be setback a minimum of 450mm from side boundaries.

Stormwater

Roof water overflow from a 2,500 litre rainwater tank to be connected to the existing storm water drains discharging to Milton Street. Stormwater discharges by gravity to the street. Roof water from the garage will be discharged into an absorption trench as is the existing garage.