# STATEMENT OF ENVIRONMENTAL EFFECTS

# FOR:

### Multi-dwelling residential development at

14 A Briggs Street, Young. NSW 2594

HILLTOPS COUNCIL (Young)

LOT 3  DP 25503

Land Zone R1

Site area is 1993 sqm.

### INTRODUCTION

This Development Application is for four single storey detached duplex dwellings.

Dwelling No. 1 consisting of an open plan kitchen, dining and living room, separate lounge room, 4 bedrooms, laundry and bathroom and a double garage.

Dwellings No. 2, 3 and 4 consisting of an open plan kitchen, dining and living room, separate lounge room, 3 bedrooms, laundry and bathroom and a double garage.

### Site and context suitability

The site is about 1.15 km from the main street of Young. The site area is 1993 sqm.

The total floor area of the 4 proposed dwellings including the garages is 850 M2.

Site coverage (gross building area)  is 42% of the site area

The ground falls from the East boundary down to the West boundary 4.0 M.

Excavation and filling is required.

The site is not subject to flooding, slope restraints, bushfire hazard or mine subsidence. There are no areas biodiversity.

The proposed dwellings are set back 3.0 M – 4.0 M. from the Northern boundary, and between 4.0 M. to 5.0 M. from the Southern boundary. Set back from the street boundary is 6.0 M to 8.5 M. and the set back from the rear or Eastern boundary is 2.1 M. Windows in the Eastern boundary wall have a sill height of 1.5 M.

Vehicle and pedestrian access is from Briggs Street.

Public transport, shops, community and recreational facilities are nearby in the township.

Adjoining properties are of a similar lot size and R1 zone.

The proposed use is compatible with the R1 zone.

### Present and previous use

The lot is vacant land Previous use is unknown.

### Development Standards

Hilltops Local Environmental Plan 2022

Objectives of the zone:

To provide for the housing needs of the community.

• To provide for a variety of housing types and densities.

• To enable other land uses that provide facilities or services to meet the day to day needs of residents.

• To provide and protect a high quality residential amenity that adds value to the existing character and amenity of the town, neighbourhood and street.

• To provide fully serviced residential lots linked to town water and sewer networks.

Permitted with consent:

Dwelling houses, Duplex house

Young Development Control Plan 2011

Chapter 2.3 Multi Dwelling Residential Development:

Controls

PMD1.1 Cut and fill will be less than 1.0 M. deep

The cut and fill will exceed 1.0 M. The deepest cut is 2.0 M at the SE corner of the site and deepest fill is 1200 mm.

PMD1.2 There are no easements on the site.

PMD1.3 There are no natural water courses or drainage lines on the site.

PMD1.4 The property access does not encroach on street trees structures or fixtures on the road reserve. The vehicle access will be via a new concrete kerb crossing.

PMD1.5 The land is not a former orchard.

AMD2.1.1 Fencing forward of the front building lie is a 1.2 M. high steel picket fence

AMD2.1.2 The primary entrance door to House No. 1 faces the Street frontage.

Entrance doors to Houses 2, 3 and 4 face the internal driveway.

AMD 2.1.3 The materials are brick veneer walls and a corrugated steel roof with a light colorbond finish. There are not large areas of glass or other reflective materials.

AMD 2.1.4 Landscaping to the street frontage will include low water use grass and 2 trees.

AMD 2.1.5 The driveway is not fenced.

AMD 2.2 Public access to each dwelling is clearly defined by the driveway from Briggs Street. Letterboxes are provided at the Briggs Street entrance.

AMD2.3.1 The dwellings are set back 6-8.5 M. from the primary street frontage.

AMD2.3.2 The House 1 garage directly address Briggs Street and do not project in front of other parts of the dwellings. Garages for Houses 2, 3 and 4 face the internal driveway and are set back from the front building line.

AMD2.4.1 The dwellings are setback 3.0 – 4.0 M. from the North side boundary, between 4 and 5 M. from the South side boundary and 2.1 M. from the rear boundary. Windows in the Eastern boundary wall have a sill height of 1.5 M.

AMD2.4.2 Not applicable

AMD3.1.1 The dwellings are single storey

AMD3.1.2 Eastern side wall lengths on all 4 houses exceed 8.0 M. and include windows.

AMD3.2.1 Permeable areas total 493 sqm or 24% of the site area., these are in the private out door space of each house.

AMD3.2.2 The area forward of the front building setback (to Briggs Street) is 138 sqm. The concrete driveway area is 67.5 sqm or 48 % of the total area. And the landscaped area is 76 sqm or 52% of the area. The concrete paved area is 18% in excess of the maximum 30% control.

AMD3.2.3 The site coverage is 429 sqm or 42 % of the site area.

AMD 3.3.1 Private open space is provide for each new dwelling.

House 1 has 131 sqm, House 2 has 98 sqm, House 3 has 78 sqm and House 4 has 115 sqm.

Each area is able to contain a rectangle of 8m x 4m. No part of the minimum required area of 48 sqm has a width less than 3.0 M.

AMD3.4 Not applicable

AMD4.1.1 Car parking is provided on site, 2 car space within a double garage for each dwelling.

2 visitor parking spaces are provided on site.

AMD4.1.2 Double car garage are 6.0 M wide x 6.0 M long. The visitor parking spaces are 2.8 M wide by 5.5 M. long

AMD4.1.3 Not applicable

AMD4.1.4 Visitor parking is accessible from the internal driveway.

AMD4.1.5 The driveway shall be paved with concrete

AMD4.2.1 The width of the internal driveway serving garages is 3.0 M.– 3.5  M. A passing lane is provided in front of House 2 which is 5.0 M wide by 14.0 M. long

AMD4.3.1 Vehicle access for House 1 is directly from the street. Vehicles enter the garage in a forward direction and reverse out to the street. Vehicle access to Houses 2, 3 and 4 is from the internal driveway. Vehicles enter the garages in a forward direction and leave the site in a forward direction. Vehicles can turn on the site in no more than a 2 point turn.

AMD 4.4.1 Vehicle access is provided by a new single consolidated driveway from Briggs Street. A new concrete driveway will be built from the kerb crossing to the site boundary.

AMD 4.5.1 and 4.5.2 The access driveway is 5.5 M in width at the property boundary.

AMD 4.5.3 The access driveway from the road edge to the property boundary shall be constructed in accordance with Council guidelines.

AMD4.5.4 Cross grades to driveways and visitor parking areas do not exceed 4 degrees.

AMD4.6 A 1.5 M. wide concrete footpath will be provided to the Briggs Street frontage.

AMD4.7 There is existing kerb and guttering along Briggs Street.

AMD5.1.1 Reticulated water will connected to the site.

AMD5.1.2 Reticulated sewer service will be connected to the site

AMD 5.2.1 Stormwater from the roofs will be collected in rainwater tanks. Overflow from the tanks will drain to Councils existing stormwater drainage system. Stormwater falling over the open space will be absorbed into the ground, stormwater falling on the paved driveway will be collected in pits and conveyed to Councils street gutter storm water drainage system.

AMD5.2.2 Civil Engineering design will be provided.

AMD6.1 AMD6.2 AMD 6.3 Letter boxes will be provided for each dwelling on the Briggs Street boundary

AMD6.4 Cloths drying facilities are provided outdoors for each dwelling in the form of a folding cloths line attached to the side of the house. The drying areas are set well back from the front of the dwellings and do not detract from the minimum area for private space.

AMD6.5 Garbage bin storage is provided within the curtilage of each dwelling and are not within the minimum area for private space.

AMD7 Compliance notes are included on floor plans.

- Design Guidelines

The dwellings use similar building materials, massing and roof forms as seen in the area. A small front entry porch and Dutch gables are features seen on many existing dwellings.

Setbacks and fence styles respect the existing local context

### Operation and Management

The houses will be used for residential occupation.

### Access and traffic

The site has a road access from Briggs Street. There is parking on site for 10 cars. Briggs Street is a busy suburban street.

- General Accessibility

The dwellings provide easy access to the street for a person with a disability. The ground floor entry to each house is 100 mm above the entry porch level. The entry porches are 100 mm above ground finished level

### Environmental Issues

### Air and noise

The residential use proposed will not generate noise or air pollution.

The construction stage will generate some noise from excavation machinery used during the excavation stage and building stages as would be normally expected with the building process.

### Soils and Water

The proposed work will have little or no impact on the soils and water.

The development will have no impact on existing surface water or ground water conditions.

Surface water will be absorbed through the permeable areas on site.

### Flora and Fauna

8 mature trees will be removed in the course of this development.

The development has minimal impact on existing flora and fauna other than the trees.

### Waste management

Domestic waste will be picked up by Councils Waste disposal service.

### Water and Sewer

The site will be connected to mains water supply and sewer system. The main sewer line and water mains run along the Briggs Street in front of the property. Rainwater tanks will be installed for each new dwelling with a storage capacity for 3000 L. The tank water will be connected to at least one toilet, the laundry washing machine tap and one outdoor tap in each residence.

All taps, shower heads and toilets will comply with the BASIX Certificate.

### Privacy, views and sunlight

The proposed development will have no adverse impact on the privacy, views and sunlight of adjoining properties.

### Visual Privacy

The proposed development does not overlook private outdoor space of properties nearby.

### Acoustic Privacy

The proposed development does not impact on the acoustic privacy of any properties nearby.

Views

The proposed development does not impact on the views of any properties nearby.

### Sunlight

The proposed development does not impact on sunlight or shadows cast on any properties nearby.

### Drainage

Rainwater from the roof of the houses will drain into a rain water tanks. Overflow from the rainwater tanks will discharge to Councils stormwater collection system.

Stormwater from the paved driveway will be collected in sumps and conveyed to Council’s stormwater system in the street.

The proposed building works will not create significant erosion or sediment run off.

Some of the excavated material will be retained on site for fill. Surplus material will disposed of in a land fill tip.

Measures will be taken by the builder to ensure sediment run off is trapped and does not enter Councils storm water drainage system or the natural water courses or neighbours properties.

### Heritage

There are no heritage items on the site or nearby the site.

### Environmental Sustainability

### Energy and Ventilation

The proposed design has living areas on the North side of the houses. The open plan  living areas have natural ventilation provided by doors and windows. All bedrooms and bathrooms have natural ventilation through windows. The ensuite bathrooms have mechanical ventilation and natural lighting and ventilation via windows.

### Sun Control

Eaves on all sides of the house are 450 mm deep.  All new glazing and window shading devices will comply with the BASIX certificate requirements.

### Insulation

Insulation is provided to all walls and ceilings as required by the BASIX certificate.

### Natural Ventilation

Windows and doors provide adequate cross ventilation throughout the house.

### Heating Cooling and Lighting

### Heating and cooling is provided by reverse cycle air conditioning throughout.

### All light fittings will be fitted with L.E.D. or fluorescent lamps as required by the BASIX Certificate.

### Cloths drying

An outdoor cloths drying area is provided.

### Water heating

An Electric Heat Pump water heater will be installed for each house to provide hot water to the kitchen, bathrooms and laundry.

- Waste

Building waste will be minimised as much as possible and waste materials will be sorted and disposed of at a waste processing and recycling facility.

- Site Management

The construction site will be secured with barrier fencing to restrict public access. No hoardings are required. There is adequate site area for site amenities, storage of building materials and equipment, bulk waste containers and material stock piles. Builder’s vehicles access the site from Briggs Street.

### CONCLUSION

The proposed development has an extremely low impact on the environment and adjoining properties.

There are no adverse impacts resulting from the proposed development

The residential use is entirely compatible with the adjoining residential properties and the R1 zoning.

The proposed development complies with the Young Development Control Plan 2011 for multi-dwelling residential development and the Hilltops Local Environmental Plan 2022 with only 2 exceptions, the minimum area of concrete paving forward of the front building line is exceeded by 18%. And the depth of excavation and fill is increased from the 1.0 M. maximum control, the deepest cut is 2.0 M at the SE corner of the site and the deepest fill is 1.2 M