Following the Section 34 conference, the Applicant provided Council with a suite of amended architectural drawings and materials The amendments resulted in a complete redesign made in response to the Respondent’s SOFAC and broadly the changes included the following;

FLOOR PLANS

LOWER GROUND  & BASEMENT PLANS

The basement has been reconfigured such that the eastern building face aligns in part with the adjoining edge of Jamison Lane also stepping back further to allow for basement fire stair egress.

This provided 52.4sqm of deep soil planting and an excellent landscaping buffer to property 2 Robert Street Sans Souci.

The redesigned basement has located the retail and visitor parking on the Lower Ground floor level with the residential parking located on the Basement floor level.

The repositioned passenger lifts (1 Residential + 1 Retail) enables efficiency with planning and apartments entries throughout the building.

GROUND FLOOR PLAN

The Ground floor has been reconfigured principally to observe a 9m setback from the centre of Jameson Lane and to provide a 3m setback in part along the northern boundary.

The residential entrance has bee relocated from the northern side to the southern side which is now shorter in length.

The reconfiguration has resulted in;

3/1 Bedroom apartments with generous podium planting and

1/ Retail area being much smaller facing Rocky Point Road.

LEVEL 1 FLOOR PLAN

The Level 1 floor has been reconfigured principally to observe a 9m setback from the centre of Jameson Lane and to provide a 3m setback in part along the northern boundary.

The revised lift lobby location has natural light and ventilation.

The reconfiguration has resulted in;

3/2 Bedroom apartments and

1/1 Bedroom apartment

LEVEL 2  FLOOR PLAN

The Level 2 floor has been reconfigured principally to observe a 12m setback from the centre of Jameson Lane and to provide a sensitive solution with a 3m setback in part along the northern boundary.

The revised lift lobby location has natural light and ventilation

The reconfiguration has resulted in

3/2 Bedroom apartments and

1/1 Bedroom apartment

LEVEL 3  FLOOR PLAN

The Level 3 floor has been reconfigured principally to observe a 12m setback from the centre of Jameson Lane and to provide a 3m setback along the northern boundary.

The revised lift lobby location is naturally light and ventilated.

The reconfiguration has resulted in

2/3 Bedroom apartments and

1/1 Bedroom apartment

ELEVATIONS

WESTERN (STREET FRONT) ELEVATION

The planning reconfiguration has generated a simpler façade where the retail area and the building elements above projects  as a three storey building form as the penthouse level setback and obscured with planting.

The removal of the roof terrace has made a significant reduction in the visual scale of the building.

The reconfiguration introduced a small break between the northern neighbour providing a relaxed visual separation which assists the relationship between the older neighbouring building and subject development.

SOUTHERN ELEVATION

The planning reconfiguration has generated a simpler façade with precast concrete panels displaying a casual yet distinctive panel joint layout.

The removal of the roof terrace has made a significant reduction in the visual scale of the building.

NORTHERN ELEVATION

The planning reconfiguration has generated a simpler façade with precast concrete panels displaying a casual yet distinctive panel joint layout. In addition the northern penthouse level has been setback providing greater curtilage between the neighbouring properties and allowing both the old and new building forms to sit comfortably side by side within the street.

The removal of the roof terrace has made a significant reduction in the visual scale of the building.

EASTERN ELEVATION

The planning reconfiguration and in particular adhering to the 9m and 12M setbacks from the centerline of the road at the various levels has generated a simpler and more respectful façade with less requirements for screens for audible and visual privacy.

The Eastern façade is further softened with landscaping within the deep soil planting area at the end of the lane and further soften with  two levels of planting on structures, one on Ground Level and one on Level 1 above.

The removal of the roof terrace has made a significant reduction in the visual scale of the building.