Consistency with Northern Councils E Zone Review Final Recommendations

Planning Proposal: PP_2017_BALLI_001_00 – Ballina Stage 1 for integration of deferred land into Ballina LEP 2012

| Recommendation | Key Issues | Comment |
| --- | --- | --- |
| - When will E Zones be applied? - E2 and E3 zones will only be applied if the primary use of the land is considered to be environmental conservation (E2) or environmental management(E3) and the land contains attributes which meet one or more of the criteria for an E2 or E3 zone (Tables 1 and 2). - An E4 zone can be applied if the land contains attributes that are consistent with the Department’s Practice Note PN09-002 Environment Protection Zones. | Does the proposed zone meet the primary use of land test? Is an E4 zone proposed and is it consistent with PN09-002. | Complies, subject to the primary use of land for the proposed E2 zone being confirmed by consultation with the land owners. The planning proposal intends to rezone the majority of the subject land RU1 Primary Production. The proposed E2 and E3 zones have been applied to land which has been determined to have a primary use of environmental conservation or environmental management and contains native vegetation that meets the criteria. Council intends to consult with the land owners after the Gateway has been issued and prior to community consultation to confirm the primary use of land (as discussed further below). Further detail is provided in the Planning Team Report. An E4 zone is not proposed as part of this planning proposal. |
| - How will the Primary use of land be determined? - The primary use of the land is the main use for which the land has been used for the last two (2) years. This may mean that land which is currently zoned rural will continue to have a rural zone but it may have parts of that land which have attributes that meet the criteria for an E2 or E3 zone included in a mapped planning control. - The primary use of the land may vary across a particular property depending on the characteristics of the land. This may result in more than one zone being applied to the land. - The primary use of land will be identified during the preparation of a planning proposal. | Is the verification methodology for determining the primary use of land consistent with the Secretary’s letter of 1 March 2016? | Complies, subject to consultation with land owners prior to community consultation. Council’s approach to determining primary use of land is also consistent with the requirements of the Secretary’s letter of 1 March 2016. Council has made a preliminary assessment of the primary use of the land proposed to be zoned E2, based on aerial photography, visual inspections and other property information available to Council, consistent with the requirements of the Final Recommendations, and concluded that the primary use of the land is either environmental conservation or environmental management. Council has acknowledged that the input of the land owner is necessary to confirm the primary use of the land proposed to be zoned E2. Council intends to write to each affected land owner after the issue of a Gateway determination and prior to community consultation and seek their input on the primary use of that portion of their land proposed to be zoned E2. The land proposed to be zoned E3 is an existing public reserve which contains subtropical rainforest that is an endangered ecological community and application of an E3 Zone to this type of public land is consistent with the E Zone Review recommendations. |
| - What are the E Zone Criteria? E2 Criteria - SEPP 26 Littoral Rainforest - SEPP 14 Coastal Wetlands - Endangered Ecological Communities - Key Threatened Species Habitat - Over-cleared vegetation communities - Culturally significant lands E3 Criteria - Riparian and estuarine vegetation - Rare endangered and vulnerable forest ecosystems - Native vegetation on coastal foreshores | Does land meet the E Zone criteria in Tables 1 and 2? Which criterion does the land satisfy to have an E zone applied to it? | Complies. The land proposed to be zoned E2 contains an endangered ecological community being subtropical rainforest and therefore meets the criteria for an E2 zone. The land proposed to be zoned E3 is an existing public reserve which contains subtropical rainforest that is an endangered ecological community. While satisfying the attributes to meet the E2 criteria, the primary use of the land as a public reserve is considered as environmental management |
| - What is the procedure for applying an E2 or E3 zone? - Councils will assess land against the E zone criteria and consider the primary use of the land before proposing an E2 or E3 zone. - An E2 or E3 zone can only be applied to land with a primary use of environmental conservation or environmental management and, which has attributes that have been verified to meet the E zone criteria. - If the land has attributes that meet the E2 criteria, however the primary use of the land is environmental management rather than environmental conservation, a council may apply an E3 zone. - If a council believes the primary use of the land does not warrant an E zone, and the land meets the E zone criteria, then a LEP Map and associated clauses can be applied. - The E zones will not include buffers to the vegetation attributes that meet the E zone criteria. | Has Council satisfied the procedure for Applying E Zones? | Complies. Council’s approach to identifying the primary use of land, and confirmation of attributes that satisfy the criteria has complied with the procedure for applying an E zone. |
| - How is the E Zone Criteria verified? An E2 or E3 zone or other mapped planning controls cannot be applied to land unless the attributes that meet the E2 or E3 criteria have been verified on that land. Verification of the presence of attributes that meet theE2 or E3 criteria on the site must be undertaken by one or a combination of the following: - biodiversity field inspections and ground surveys conducted by an appropriately qualified person. - Aboriginal heritage field inspections and ground surveys conducted by an appropriately qualified person or someone with extensive field experience and in accordance with the Code of Practice for Archaeological Investigation of Aboriginal Objects in NSW, NSW Department of Environment, Climate Change & Water (2010). - supporting flora and fauna reports conducted by a suitably qualified person and guided by the Draft Framework for Biodiversity Assessment, NSW Office of Environment and Heritage (2014) and the Draft Threatened Biodiversity Survey and Assessment: Guidelines for Developments and Activities, NSW Department of Environment and Conservation(2004). Such reports will only be acceptable where the field work is not more than five years old. - review of current (not more than five years old) high resolution digital aerial photography that has been verified by another one of these verification techniques. - supporting cultural heritage reports conducted by a suitably qualified person and in accordance with a Guide to investigating, assessing and reporting on Aboriginal cultural heritage in NSW, NSW Department of Environment, Climate Change &Water (2011). - consultation with Aboriginal cultural knowledge holders in regard to culturally significant lands in accordance with a Guide to investigating, assessing and reporting on Aboriginal cultural heritage in NSW, NSW Department of Environment, Climate Change & Water (2011). | Has the presence of attributes which meet the criteria been verified by one or more of the methods listed in Recommendation 5. | Complies. The presence of the endangered ecological community has been verified by Council using an aerial photograph from 2015, on ground verification by Council’s Ecologist and the use of reports used by the NSW Roads and Maritime Services relating to the Pacific Highway upgrade and the Rous Water draft report “Emigrant Creek River Reach Plan: Pacific Highway to Emigrant Creek Dam”. This approach to verifying the presence of vegetation attributes that meet the criteria for an E2 zone is considered to be consistent with the requirements of recommendation 5 of the Final Recommendations. |
| - Transferring Environmental Zones. - The areas of land to which the current environmental protection zones listed in the Table 3 (below) apply, may be zoned E2 or E3 once councils have verified the attributes of the land against the criteria. Ballina LEP 1987 7(a) Environmental Protection (Wetlands) 7(l) Environmental Protection (Habitat) Byron LEP 1988 7(a) Environmental Protection (Wetlands) 7(b) Environmental Protection (Coastal Habitat) 7(j) Environmental Protection (Scientific) 7(k) Environmental Protection (Habitat) Lismore LEP 2000 7(a) Environment Protection (Natural Vegetation and Wetlands) 7(b) Environment Protection (Habitat) Tweed LEP 2000 7(a) Environmental Protection (Wetlands and Littoral Rainforest) 7(l) Environmental Protection (Habitat) - Kyogle Council should apply a rural zone, equivalent to the zone in the superseded Interim Development Order, to the land which was proposed to be zoned E2 or E3 and was deferred from the Kyogle LEP 2012, until such time as investigations are completed to identify appropriate E zones or additional mapped planning controls. | Is the land already zoned one of the existing “7” zones in Table 3? | Complies. A portion of the land proposed to be zoned part E2 east of Killen Falls Drive is currently zoned 7(l) Environmental Protection - Habitat in the Ballina LEP 1987. In accordance with recommendation 6 of the Final Recommendations the 7(l) zoned land can be zoned E2 once its attributes have been verified. Council has verified the presence of the attributes that meet the criteria as discussed in the previous requirement. |
| - Public and Private Land inconsistent with the criteria. - Public land may be zoned E2 or E3 despite being inconsistent with the criteria, if the primary use of the land is environmental conservation or environmental management. - Private land may be zoned E2 or E3 despite being inconsistent with the criteria, only if it is consistent with a negotiated development outcome (master plan, rezoning, development consent, designated offset areas) or at the request of the landowner. | If an E zone has been applied to public land inconsistent with the criteria, is the primary use environmental conservation or environmental management? If an E zone has been applied to private land inconsistent with the criteria, is it the result of a negotiated development outcome or at the request of the landowner? | Complies. None of the land proposed to be zoned E2 or E3 is considered to be inconsistent with the criteria. |
| - Voluntarily revegetated land. - Land which has been voluntarily revegetated by the current landowner, will not have an E2 or E3 zone applied to it without the agreement of the current landowner providing: − the revegetation has been actively undertaken and is not the result of natural regrowth; − active revegetation includes a combination of planting, seeding, weed control, fencing, removing stock, watering, ripping, mulching and soil improvement to encourage the natural regeneration of native vegetation; and − the primary use of the land is agriculture. - Land which has been voluntarily revegetated can be included on a Vegetation Map without the agreement of the current landowner if the attributes have been verified to meet the criteria for an E2 or E3 zone and the primary use of the land is environmental conservation or environmental management. - If revegetation has been undertaken with the support of grant funding, and a condition of that funding was the ongoing conservation or management of the vegetation, then an E2 or E3 zone may be applied to the land. | Is any voluntarily revegetated land affected by this planning proposal? Does it comply with the requirements? | Not applicable. None of the land proposed to be zoned E2 or E3 has been identified as being voluntarily revegetated. |
| - Zoning of State and regionally significant farmland. - When zoning State or regionally significant farmland, councils will have to take account of the primary use of the land before applying an E zone or a rural zone. | Has the zoning of State or Regionally significant farmland taken into account the primary use of land test? | Complies. The subject land includes both State and Regionally significant farmland. The application of the RU1 and E2 zones has taken into account the primary use of the land. |
| - Application of multiples zones to a single property (split zoned lots). - More than one zone can be applied to properties where the characteristics of different areas of the land reflect the different primary uses of the land. - Councils should consider the suitability of alternative zones or including the land on a Vegetation Map when considering more than one zone for a property. - As a general principle, the use of multiple zones on a property should be minimised as far as possible. | Are multiple zones applied to a single property? Do these zones reflect the primary use of the land? | Complies. In some instances properties have an RU1 and E2 zone applied. These proposed zones reflect the primary use of land. |
| - Application of the E4 Zone in Byron Council. - Byron Shire Council is to prepare a planning proposal to apply a suitable residential zone to that land where an E4 zone was proposed under the draft Byron LEP. | Does the proposal apply to land proposed to be zoned E4 in Byron LGA? Is the proposal applying a suitable residential zone to this land? | Not applicable. The proposal does not relate to either an E4 zone or land in Byron LGA. |
| - Application of additional mapped planning controls. - Matters of public health, safety, risk and hazard such as drinking water catchments, flooding, coastal risk areas and land subject to strict development controls such as steep land may be included in a LEP Map. - A LEP Map is not to be used for areas of scenic protection or aesthetic values. - Land that has been verified to meet the criteria for an E2 or E3 zone where the primary use of the land is not environmental conservation or environmental management may be included in a mapped planning control, such as a Vegetation Map. | Does the proposal seek to introduce overlays or other mapped planning controls? Do the mapped planning controls apply to matters of public health, safety, risk and hazard? Do the mapped planning controls apply to land with attributes which meet the E zone criteria but does not have a primary use of land that warrants an E2 or E3 zone? | Complies. The planning proposal does not introduce any mapped planning controls. The proposal seeks to introduce a heads of consideration clause for land to be zoned E2 or E3. The proposal also applies the existing drinking water catchment clause in Ballina LEP 2012 to identified drinking water catchment land, and introduces a heads of consideration clause for the E and W Zones. These clauses are consistent with the Final Recommendations subject to E and W Zone clause being amended by the Gateway to apply only to these lands and not adjoining lands to create a buffer type situation. |
| - Aesthetic Value. - Councils on the Far North Coast will not be permitted to use scenic values as an attribute for the application of an E2 or E3 zone or mapped planning controls. | Has aesthetic value been used as an attribute for the application of an E zone? | Complies. Aesthetic value has not been used as a criteria for applying a proposed E zone. |
| - Permissibility of agriculture in E Zones. - Extensive agriculture is to be listed as permissible with consent in the E2 zone for all Far North Coast LEPs. - Extensive agriculture is to be listed as permissible without consent in the E3 zone for all Far North Coast LEPs. | Is extensive agriculture permissible without consent in the E3 zone? Is extensive agriculture permissible with consent in the E2 zone? | Complies. The proposed E3 zone table lists extensive agriculture as permissible without consent. The proposed E2 zone table lists extensive agriculture as permissible with consent. |
| - Coastal Zone Management. - Far North Coast councils are to use a Coastal Risk Map and associated clause to manage land affected by coastal hazards. | Has a Coastal Risk Map and associated clause been used to manage land affected by coastal hazards? | Not applicable. The land which is the subject of this planning proposal is not affected by coastal hazards. |
| - Section 117 Direction. - A Section 117 Ministerial Direction specific to applying E zones and mapped planning controls in Far North Coast LEPs will ensure the consistent application of the final recommendations of the Northern Councils E Zone Review for Ballina, Byron, Kyogle, Lismore and Tweed Local Government Areas. | Is the proposal consistent with S117 Direction 2.5 Application of E2 and E3 Zones and Environmental Overlays in Far North Coast LEPs . | To be determined. The consistency of the proposal with Direction 2.5 is to be reassessed after community consultation has been undertaken and Council has assessed the submissions received by land owners in relation to primary use. Further detail on this matter is contained in the planning team report. |