series/program name (insert space if not needed)

# 5-9 Gordon Avenue, Chatswood – Gateway Determination Report

Greater Sydney, Place and Infrastructure

November 2020

Published by NSW Department of Planning, Industry and Environment

dpie.nsw.gov.au

Title: 5-9 Gordon Avenue, Chatswood – Gateway Determination Report Greater Sydney, Place and Infrastructure

Department reference number: IRF20/5613

© State of New South Wales through Department of Planning, Industry and Environment 2020. You may copy, distribute, display, download and otherwise freely deal with this publication for any purpose, provided that you attribute the Department of Planning, Industry and Environment as the owner. However, you must obtain permission if you wish to charge others for access to the publication (other than at cost); include the publication in advertising or a product for sale; modify the publication; or republish the publication on a website. You may freely link to the publication on a departmental website.

Disclaimer: The information contained in this publication is based on knowledge and understanding at the time of writing (April 2020) and may not be accurate, current or complete. The State of New South Wales (including the NSW Department of Planning, Industry and Environment), the author and the publisher take no responsibility, and will accept no liability, for the accuracy, currency, reliability or correctness of any information included in the document (including material provided by third parties). Readers should make their own inquiries and rely on their own advice when making decisions related to material contained in this publication.

Contents

Glossary	2

Summary	3

1.	Introduction	3

1.1	Description of planning proposal	3

1.2	Site description	4

1.3	Surrounding area	6

1.4	Current planning controls	7

1.5	Summary of recommendation	10

2.	Background	11

2.1	Original Planning Proposal	11

3.	Planning Proposal	11

3.1	Objectives or intended outcomes	11

3.2	Explanation of provisions	12

3.3	Mapping	14

3.4	Concept Design	17

4.	Need for the planning proposal	21

5.	Strategic Assessment	21

5.1	Greater Sydney Region Plan	21

5.2	North District Plan	22

5.3	Chatswood CBD Strategy	23

5.4	Willoughby Local Strategic Planning Statement	27

5.5	Willoughby Local Housing Strategy	27

5.6	Section 9.1 Ministerial Directions	28

5.7	State Environmental Planning Policies (SEPPs)	30

6.	Site-specific Assessment	31

6.1	Built Form	31

6.2	Social	34

6.3	Environmental	35

6.4	Economic	35

6.5	Infrastructure	36

7.	Consultation	37

7.1	Community	37

7.2	Agencies	37

8.	Timeframe	37

9.	Local Plan Making Authority	38

10.	Conclusion	38

11.	Recommendation	38

Attachments	41

# Glossary

| Abbreviation | Reference |
| --- | --- |
| ADG | Apartment Design Guide |
| Council | Willoughby City Council |
| DA | Development Application |
| DCP | Development Control Plan |
| Department/DPIE | Department of Planning, Industry and Environment |
| DP | Deposited Plan |
| FSR | Floor Space Ratio |
| GFA | Gross Floor Area |
| LEP | Local Environmental Plan |
| LHS | Local Housing Strategy |
| LSPS | Local Strategic Planning Statement |
| PANS-OPS | Procedures for Air Navigation Services – Aircraft Operations |
| PPA | Planning Proposal Authority |
| RMS | Roads and Maritime Services |
| SEPP | State Environmental Planning Policy |
| SNPP/Panel | Sydney North Planning Panel |

# Summary

| Local Government Area | Willoughby |
| --- | --- |
| Planning Proposal Authority | Willoughby City Council |
| Planning Proposal Name | PP_2020_WILLO_004_00 |
| Dwellings and Jobs | 103 dwellings, 50 jobs |
| LEP to be Amended | Willoughby Local Environmental Plan 2012 |
| Address | 5-9 Gordon Avenue, Chatswood |
| Lot and DP | SP 57091 |
| Date Received | 1 October 2020 |
| File Number | IRF20/5613 |
| Political Donations | There are no donations or gifts to disclose and a political donation disclosure is not required. |
| Lobbyist Code of Conduct | There have been no meetings or communications with registered lobbyists with respect to this proposal. |

# Introduction

## Description of planning proposal

The planning proposal (Attachment A) seeks to amend the Willoughby LEP 2012 for the subject site by:

- Rezoning the site from R3 Medium Density Residential to B4 Mixed Use.

- Increasing the maximum permitted building height from 12m to 90m.

- Increasing the maximum floor space ratio (FSR) from 0.9:1 to 6:1.

- Including a minimum commercial FSR component of 1:1.

A concept design has been provided with the planning proposal depicting a mixed-use development comprising of a two-storey commercial/retail podium with 25-storey a residential tower above.

The concept development could deliver approximately:

- 103 total dwellings including 4% affordable housing

- 1,522sqm of commercial/retail floor space.

## Site description

The site is located at 5-9 Gordon Avenue, Chatswood (Figure 1) and has a total area of 1,522sqm comprising a single lot with frontages of 42.92m along Gordon Avenue and a secondary frontage of 48.635m to Hammond Lane. Vehicular access is currently obtained from Gordon Avenue.

The site is currently occupied by a three-storey brick apartment building that contains 12 dwellings and basement carparking (Figures 2-4).

Figure 1: Site map (source: Nearmap).

Figure 2: Existing site as viewed from Gordon Avenue looking north (source: Google Maps).

Figure 3: Existing site as viewed from Gordon Avenue looking north (source: Google Maps).

Figure 4: Existing site as viewed from Hammond Lane looking south (source: Ingham Planning)

## Surrounding area

The site is located within the expanded Chatswood CBD area which includes Chatswood Station and transport interchange and three major shopping centres including Westfield, Chatswood Chase and the Mandarin Centre (Figure 5).

Chatswood Station was upgraded in 2019 to include the new Sydney Metro network. The upgrade provided access to new trains operating between Rouse Hill and Chatswood, with trains running every four minutes at peak times. Chatswood Station is an integrated transport exchange and includes direct links to the strategic centres of Macquarie Park, North Sydney and the Sydney CBD.

The upgrades will also connect Chatswood to Sydney Metro City and South West. This network is due to be completed in 2024 and will provide an extension of Sydney Metro Northwest from Chatswood under Sydney Harbour, through new CBD stations and southwest Bankstown, with the capacity to provide trains every two minutes in each direction.

To the north of the site, on the opposite side of Hammond Lane is the Chatswood Bowling Club, associated bowling greens and carparking.

To the south of the site, on the opposite site of Gordon Avenue are two, 3 storey residential flat buildings.

To the east of the site, is a 3 storey residential flat building with basement carparking with the T1 North Shore Rail Line and a walking/cycling path further east.

To the west of the site, on the opposite site of Hammond Lane is a three-storey residential flat building.

Figure 5: Site context map (source: WMK Architecture).

## Current planning controls

Under the Willoughby LEP 2012 the site is subject to the following planning provisions:

- A R3 Medium Density Residential zoning (Figure 6).

- A maximum building height of 12m (Figure 7).

- A maximum FSR of 0.9:1 (Figure 8).

A comparison of the current and proposed controls has been provided at Table 1.

It is also noted that while the site is not a listed heritage item, it is within close proximity of the South Chatswood Heritage Conservation Area (HCA) (Figure 9).

Table 1: Comparison between current and proposed planning provisions.

| Control | Current | Proposed | CBD Strategy |
| --- | --- | --- | --- |
| Zone | R3 Medium Density Residential | B4 Mixed Use | B4 Mixed Use |
| Height | 12m | 90m | 90m |
| FSR | 0.9:1 | 6:1 (1:1 minimum commercial) | 6:1 (1:1 minimum commercial) |
| Affordable Housing | N/A | Minimum 4% | Minimum 4% |

Figure 6: Willoughby LEP 2012 Land zoning map (source: DPIE).

Figure 7: Willoughby LEP 2012 Maximum building height map (source: DPIE).

Figure 8: Willoughby LEP 2012 Maximum FSR map (source: DPIE).

Figure 9: Willoughby LEP 2012 Heritage map (source: DPIE).

## Summary of recommendation

The Department has considered the submitted documentation as part of the proposal and concludes that there is sufficient strategic merit in issuing a Gateway determination as:

- It is consistent with the actions of the North District Plan for Chatswood by providing additional commercial development capacity, maximises public transport patronage, promotes employment growth within an existing commercial zone and increases residential capacity in an accessible location.

- It is consistent with the key elements of the endorsed Chatswood CBD Strategy.

- The introduction a genuine mixed-use development would help to service the daytime and night-time economies of the retail precinct and further activates the CBD core area.

- The proposal can facilitate 103 dwellings including affordable housing within the Chatswood CBD on a site that supports the 30-minute city outcomes sought by the North District Plan.

The Department considers the proposal to have site-specific merit because:

- The proposed maximum building height and is consistent with the desired future maximum building heights and solar access planes for the Chatswood CBD.

- It is considered that the proposal is capable of adequate building separation, amenity and solar access outcomes to existing and proposed future development near the site.

- The proposal will enable the delivery of additional affordable housing and job growth in the Chatswood CBD.

- The proposal will deliver a mix of uses on the site within 500m walking distance of the Chatswood transport interchange and CBD core.

# Background

## Original Planning Proposal

On 11 February 2019, Council at its meeting resolved to support a planning proposal for the subject site to facilitate a mixed-use development. Specifically, the proposal sought to amend the Willoughby LEP 2012 by:

- Rezoning the site from R3 Medium Density to B4 Mixed Use.

- Increasing the maximum building height from 12m to 90m.

- Increasing the maximum FSR from 0.9:1 to 6:1.

- Modifying Clause 4.4(2A)(b) for parts of a building that do not count towards the calculation of FSR.

- Inclusion of Clause 6.23 requiring a minimum commercial FSR requirement of 1:1 on the subject site.

- Inclusion of a design excellence clause to adopt the Willoughby Design Excellence policy into the Willoughby LEP 2012.

- Modifying clause 6.8 Affordable housing.

On 9 August 2019, the planning proposal was returned to Council along with seven other planning proposals within the B4 Mixed Use areas of the Chatswood CBD Strategy as part of the Department’s partial endorsement letter.

Attachment G1 provides an overview and background of the Chatswood CBD Strategy endorsement process. Attachments G2-4 include the finalised Chatswood CBD Strategy and the Department’s partial and full endorsement letters.

# Planning Proposal

## 3.1	Objectives or intended outcomes

The planning proposal identifies that it achieves the following objectives and intended outcomes:

- Provide a floor space and building height yield that is consummate with the site’s location within convenient walking distance of the Chatswood CBD and transport interchange and in a manner consistent with the maximum development density and building height recommended in the Chatswood CBD Planning and Urban Design Strategy.

- Provide an increased floor space and building height that improves viability of redevelopment and reflects enhanced site capability for accommodating higher density development, arising from current planning strategies.

- Ensure environmental and amenity impacts associated with increased development yield are not unreasonable increased having regard to the desired future character of the locality and likely future redevelopment of neighbouring properties in accordance with the Chatswood CBD Strategy.

- Enable more economic and efficient use of land and inclusion of additional affordable housing accessible to retail, public transport and other services.

- Provide 293sqm of affordable housing on a site that currently is not required to provide affordable housing.

- Provide for developer levies, in addition to Council’s Section 7.12 levies, to assist with funding of infrastructure and community facilities and services required to meet demand for such infrastructure, facilities and services, arising from proposed increased development density in and near the Chatswood CBD.

- Improve pedestrian amenity and safety by way of providing a wider footpath on the northern side of Gordon Avenue fronting the site and provide a 1200mm wide footpath on the eastern side of Hammond Lane.

- Remove and existing low rise medium density housing development that is not compatible with the desired future mixed use high rise character planned for the area.

While these objectives are considered clear, it is recommended that the objective to pay developer contributions/levies be removed as the proposal should not be reliant on payment of funds to achieve a reasonable development outcome.

Additionally, as Council’s Local Strategic Planning Statement (LSPS) and Local Housing Strategy (LHS) have been finalised since Council original considered this proposal it is recommended that objectives regarding its consistency with these strategies are included.

## 3.2	Explanation of provisions

The proposal seeks to amend the Willoughby LEP 2012 for the site by:

- Rezoning the site from R3 Medium Density Residential to B4 Mixed Use.

- Increasing the maximum permitted building height from 12m to 90m.

- Increasing the maximum FSR from 0.6:1 to 6:1.

- Modifying the Active Street Frontages Map to include the Gordon Avenue and Hammond Lane frontages.

- Modifying the Special Provisions Area Map to show the site as Area 8 (architectural roof features), Area 9 (affordable housing), Area 11 (minimum commercial FSR) and Area 12 (design excellence).

- Modifying a provision within Clause 6.23 Minimum commercial FSR in mixed use zones:

  - Development consent must not be granted for the purpose of erecting a building on land identified as ‘Area 11’ unless commercial floor space equating to at least 1:1 is included.

- Replacing Clause 4.4 Floor space ratio (2A)(b) with the following:

  - any part of the building that:

    - is to be used for community facilities, or

    - is a heritage item, or

    - is to be used for affordable housing purposes if located within Area 3 of the Special Provisions Area Map is taken not to be part of the gross floor area of the building for determining the maximum floor space ratio of the building.

- Including Clause 5.6 Architectural roof features (2A) as follows:

  - Despite subclause (2), development within Area 8 of the Special Provisions Area Map may only be carried out in accordance with the maximum height of Clause 4.3 Height of buildings.

- To replace Clause 6.8 Affordable housing (2) with the following:

  - Development consent must not be granted to the erection of residential accommodation on land identified as Area 3 and Area 9 on the Special Provisions Area Map unless the consent authority has taken the following into consideration:

    - The Willoughby Affordable Housing Principles,

    - The likely impact the development would have on the existing mix and likely future mix of residential housing stock in Willoughby,

    - Whether one of the affordable housing conditions should be imposed on the consent for the purpose of providing affordable housing in accordance with the Willoughby Affordable Housing Principles.

- Note: the affordable housing principles are set out in Schedule 2 to State Environmental Planning Policy No.70 – Affordable Housing (Revised Schemes) may also apply to the development.

- To replace Clause 6.8 Affordable housing (7) with the following:

  - In this clause:

- Accountable total floor space means:

  - If in Area 3 of the Special Provisions Area Map, the gross floor space of the residential component of the development to which the development application relates, excluding any residential floor area of the building that is used for affordable housing purposes.

  - If in Area 9 on the Special Provisions Area Map, the gross floor space of the residential component of the development to which the development application relates, including any residential floor area for the building that is used for affordable housing.

- Including Clause 6.23 Minimum commercial floor space as follows:

  - Clause 6.23 Minimum commercial floor space within the Mixed Use zone

- Land zoned B4 Mixed Use is to contain a minimum commercial floor space component of 1:1 if located within Area 11 on the Special Provisions Area Map.

- Including Clause 6.24 Design excellence as follows:

  - Clause 6.24 Design excellence

    - The objective of this clause is to deliver the highest standard of architectural, urban and landscape design.

    - This clause applies to development involving the erection a new building on land shown in Area 12 on the Special Provisions Area Map.

    - Development consent must not be granted to development to which this clause applies unless, in the opinion of the consent authority, the proposed development exhibits design excellence.

The proposed provisions are generally clear, concise and are consistent with the recommendations of the now finalised Chatswood CBD Strategy and Council’s affordable housing and design excellence policies.

However, as Council’s resolution includes several clauses (e.g. affordable housing, design excellence) which have been included within previous planning proposals which have already been exhibited (PP_2019_WILLO_001_00 and PP_2019_WILLO_002_00) it is recommended that these clauses be removed prior to public exhibition to avoid unnecessary duplication and confusion during exhibition.

It is noted that PP_2019_WILLO_002_00 was submitted back to the Department with a request that the plan be made on 24 September 2020 and includes both the affordable housing and design excellence clauses as sought under the subject planning proposal.

It is recognised that multiple additional planning proposals within the Chatswood CBD have been submitted to the Department for Gateway determination. The majority of these proposals have similar clause inserted regarding the implementation of several of the Chatswood CBD provisions including but not limited to minimum commercial floor space, affordable housing and design excellence.

While the affordable housing and design excellence provisions are recommended to be altered given the previous planning proposal, which have been exhibited (one of which submitted for plan-making) includes similar clauses for implementation, it is recognised that the planning proposals within the Chatswood CBD’s B4 Mixed Use areas will all include the minimum commercial floor space provision.

Therefore, depending on when the current planning proposal is exhibited this provision should be removed or altered to a site specific ‘area’ only to avoid confusion and duplication during the exhibition.

## 3.3	Mapping

Draft LEP mapping (Attachment D) has been provided as part of the planning proposal to demonstrate the proposed changes to the Willoughby LEP 2012. The draft maps submitted include:

- An amended Land Zoning Map (sheet LZN_004) showing the site as B4 Mixed Use (Figure 10).

- An amended Maximum Height of Buildings Map (sheet HOB_004) showing the site as Area AB2 – 90m (Figure 11).

- An amended Maximum FSR Map (sheet FSR_004) showing the site as Area AA – 6:1 
(Figure 12).

- An amended Special Provisions Area map (SPA_004) showing the site as Area 8, Area 9, Area 11 and Area 12 (Figure 13).

- An amended Active Street Frontages Map (ASF_004) showing Gordon Avenue and Hammond Lane as active frontages (Figure 14).

No changes are proposed to any other LEP maps.

The maps submitted are considered appropriate for Gateway assessment and public exhibition. The maps will be required to be consistent with Department standard should the plan proceed to finalisation.

Figure 10: Draft Willoughby LEP 2012 Land zoning map (source: Willoughby City Council).

Figure 11: Draft Willoughby LEP 2012 Maximum height of buildings map (source: Willoughby City Council).

Figure 12: Draft Willoughby LEP 2012 Maximum FSR map (source: Willoughby City Council).

Figure 13: Draft Willoughby LEP 2012 Special Area Provisions map – Note: blue outline indicates Areas 8, 9, 11 and 12 (source: Willoughby City Council).

Figure 14: Draft Willoughby LEP 2012 Active street frontages map (source: Willoughby City Council).

## Concept Design

A concept design submitted with the documentation indicates that the proposal could potentially deliver a mixed-use development comprising a two storey commercial/retail podium with a 25-storey residential above.

The planning proposal states that the concept development could provide 103 dwellings and the delivery of up to around 50 jobs.

The breakdown of GFA based on the type of land use has been provided in Table 2.

Table 2: Proposed GFA allocation of concept design.

| Land use type | GFA |
| --- | --- |
| Commercial Office/Retail | 1,522sqm |
| Residential | 7,610sqm (including affordable housing) |
| Total | 9,132sqm |

Drawings of the proposed concept development have been provided at Figures 15-19 and at (Attachment A5).

Figure 15: Proposed concept development plan (source: WMK Architects).

Figure 16: Proposed concept podium (source: WMK Architects).

Figures 17: Proposed concept building envelope against existing skyline (source: WMK Architects).

Figure 18: Indicative ground floor plan (source: WMK Architects).

Figure 19: Indicative mid-level tower floor plan (source: WMK Architects).

# Need for the planning proposal

The planning proposal is the result of Council’s Chatswood CBD Strategy (Attachment G4) which has been endorsed by the Department and finalised by Council. An assessment of the proposal’s consistency with the strategy’s key recommendations has been included within Table 5 of this report.

Council’s intention is for site-specific planning proposals to be prepared to support the strategy’s recommendations prior to its comprehensive LEP amendment, which would rezone all land within the Chatswood CBD area being submitted to the Department for Gateway in late 2020.

# Strategic Assessment

## 5.1	Greater Sydney Region Plan

The Greater Sydney Commission’s (GSC) Greater Sydney Region Plan – A Metropolis of Three Cities was released in March 2018 and provides a vision for the growth and development of Greater Sydney by instilling the idea of the 30 minute city where people live no further than 30 minutes from their jobs, education, health facilities, services and great places.

Chatswood is located within the Eastern Economic Corridor of the Eastern Harbour City and is identified as a strategic centre being a major commercial precinct.

The Greater Sydney Region Plan establishes a series of directions to ensure planning proposals are consistent with the intended vision of Sydney. Directions relevant to this planning proposal are included in Table 3 below.

Table 3: Consistency of planning proposal with Greater Sydney Region Plan Directions.

| Region Plan Direction | Consistency |
| --- | --- |
| Housing the City | The planning proposal seeks to deliver 103 total dwellings in a highly accessible location. |
| A City of Great Places | The planning proposal would seek to enable the redevelopment of aged commercial buildings at the gateway of the Chatswood CBD. |
| A Well-Connected City | The site is located approximately 500m walking distance from the Chatswood transport interchange which has been recently upgraded to now include the Sydney Metro. Chatswood is well-connected to the neighbouring strategic centres of Macquarie Park and St Leonards and Crows Nest. |
| Jobs and Skills for the City | The planning proposal seeks to enable the redevelopment of the site to provide approximately 50 new jobs on the site. |
| A City in its Landscape | The planning proposal seeks to enhance the quality of landscaping on site including an above podium community garden and improved public domain to Hammond Land and Gordon Avenue |

## 5.2	North District Plan

The North District Plan reinforces the desired to strengthen and grow Chatswood as a strategic centre. Relevant to the site and proposal, the District Plan recommends the following actions for Chatswood:

- Maximise land-use opportunities provided by the Sydney Metro.

- Promote the role of the centre as a destination for cultural and leisure opportunities.

- Promote and encourage connectivity, and update and increase public open spaces.

- Investigate interchange options on both sides of the railway line to increase capacity and efficiency of modal changes.

- Improve pedestrian connectivity between the eastern and western side of the railway line.

The District Plan also sets a target increase of 6,300-8,300 jobs by 2036.

In relation to strategic centres, the District Plan states that housing contributes to a sense of vibrancy as long as this does not constrain the ongoing operation and expansion of commercial and retail activities.

The planning proposal documentation (Attachment A) has estimated that based on the concept for the current proposal, development in accordance with the proposal has the capacity to deliver around 50 total jobs. This translates to between 0.6% - 0.8% of the low to high 2036 jobs targets set for Chatswood, established by the District Plan.

The floor area allocated to non-residential development would be at least 1,522sqm. This is based on the proposed amendments and assuming the development is fully developed in accordance with the maximum FSR and minimum commercial FSR sought by the proposal.

Based on the above and subject to a future detailed design at the development application stage, it is considered that the proposal can deliver a development that is consistent with the North District Plan’s intention for Chatswood to grow as a commercial centre.

Table 4 outlines the relevant priorities of the District Plan and how the proposal demonstrates consistency with these priorities.

Table 4: Consistency of planning proposal with North District Plan priorities.

| District Plan Priority | Consistency |
| --- | --- |
| N3 Providing services and social infrastructure to meet people’s changing needs | By providing a land use mix that is co-located with a new retail shops and offices. |
| N5 Providing housing supply, choice and affordability, with access to jobs services and public transport | By providing 103 additional dwellings including a 4% affordable housing component within proximity of the Chatswood transport interchange. |
| N6 Creating and renewing great places and local centres, and respecting the District’s heritage | By redeveloping aged housing stock and providing active street frontages within a strategic centre. |
| N8 Eastern Economic Corridor is better connected and more competitive | By growing employment generating development in the economic corridor that is highly accessible to the well-connected Chatswood public transport interchange. |
| N10 Growing investment, business opportunities and jobs in strategic centres | By providing the opportunity to contribute between 0.6%-0.8%% of the District Plan’s 2036 job target for Chatswood as a key strategic centre. |
| N12 Delivering integrated land use and transport planning and a 30-minute city | By facilitating the delivery of a mix of land uses on a site that is within 500m of the Chatswood transport interchange. |

## 5.3	Chatswood CBD Strategy

The now finalised and endorsed Chatswood CBD Strategy (Attachment G4) represents Council’s 20-year development and land use vision for the future of the Chatswood CBD. The strategy has been designed to give strategic merit to site specific planning proposals and to align Council’s strategic planning work regarding its LSPS, LHS and comprehensive LEP amendment with the actions and priorities of the North District Plan.

The strategy includes a series of ‘key elements’ to guide the assessment of planning proposals and development against the intention and goals of the strategy. Table 5 includes the Department’s assessment of the proposal’s consistency with these key elements.

Table 5: Summary of proposal’s consistency with Chatswood CBD Planning and Urban Design Strategy.

| Key Element | Consistent | Assessment of Consistency |
| --- | --- | --- |
| CBD Boundary | Yes | Proposal is within the proposed expanded B4 Mixed Use area. |
| Land Use | Yes | The proposal intends to rezone the land from R3 Medium Density Residential to B4 Mixed Use. |
| Removal of DCP provisions for size of office and retail tenancies within B3 core | N/A | Not relevant to subject site. |
| Serviced apartments to be removed as a permissible use within B3 core | N/A | Not relevant to subject site. |
| Increased FSR above current controls are to be linked to a contributions scheme for the delivery of local infrastructure | Yes | The Council’s proposed value capture scheme is not supported by the Department. Council has advised that they are no longer seeking to include value capture as a mechanism for development contributions. Council has recently exhibited its revised contributions scheme and is currently considering submissions received. The proponent has included a letter of offer to enter into a Voluntary Planning Agreement (VPA) once Council finalises its contributions plan. |
| Design excellence required for all development above 35m | Yes | Council would require all development above 35m to be subject to a design excellence process. This would be undertaken at DA stage. The proposal includes a design excellence clause and would be identified as Area 12 where the design excellence policy applies. This is consistent with Council’s intended wording. Note: that this clause is intended to be implemented under PP_2019_WILLO_002_00 which has been submitted to the Department with a request that the plan be made. |
| Higher building sustainability standards | Yes | This could be determined at the DA stage. Furthermore, recent changes to the National Building Code of Australia now require improved performance of new buildings such as may be constructed in accordance with the proposal. |
| Base FSR | Yes | The proposal would achieve an FSR above 0.9:1 the minimum (base) for this site. |
| Minimum site area | Yes | Site is approximately 1,522sqm which is above the minimum site area requirement of 1,200sqm for sites within the B4 Mixed Use areas. |
| Maximum FSR | Yes | The site includes a maximum FSR of 6:1 which is consistent with the strategy. |
| Affordable Housing | Yes | The proposal seeks to include a local provision to ensure the site delivers a minimum of 4% affordable housing included within the total residential FSR component of 5:1. |
| Minimum commercial FSR in mixed use zones is to be 1:1 | Yes | The proposal includes a clause to ensure the delivery of a minimum commercial FSR of 1:1. |
| Maximum tower floor plate sizes | Yes | The proposed tower is below the maximum recommended floor plate size of 700sqm for residential development being approximately 400sqm. |
| Width of the side of each side of any tower to be minimised | Yes | The sides of the tower have been minimised to ensure Council’s goal of creating slender tower forms. |
| Tower separation | Yes | The proposal only includes a single residential tower and demonstrates that it can achieve a minimum building separation of 18m to neighbouring development. |
| Sun access to key public spaces and conservation areas | Yes | No key public open spaces identified within the strategy will be impacted by this proposal in regard to solar access. The concept shadow diagrams prepared to support the proposal indicate that a shadow will be cast to the South Chatswood HCA in the late afternoon (3pm) during mid-winter. However due to the slender tower form this shadow is fast moving and is not considered to unreasonably impact upon solar access received by the properties within the HCA. |
| Building Heights | Yes | The maximum building height for the subject site is 90m which the proposal is consistent with the strategy. |
| Lift over runs and architectural features integrated into building form | Yes – capable at DA stage | The proposed concept design integrates the lift over run and architectural features within the tower form. |
| Links and open space | Yes | While the site itself is not subject to the provision of through site links as part of the strategy, the proposal will not impede public access to the nearby links of Hammond Lane or the nearby cycle and pedestrian walkway. |
| Publicly accessible open space and landscaping | Yes – capable at DA stage | The proposed concept design includes widened footpaths and landscaping which could be delivered as part of a future DA or within a VPA. |
| All roofs up to 30m from the ground are to be green roofs | Yes | Detailed landscape drawings at a future DA stage would be able to consider and provide any desired planting requirements. The proposed concept identifies that the roof above the podium form is intended to be a green roof and includes a communal recreation area and community garden. |
| Soft landscaping is to apply to 20% of the site | Yes – capable at DA stage | Able to be resolved through the submission of a landscape plan at DA Stage. The concept podium form for the proposal shows that there is capability to achieve this outcome. |
| Roof top communal open space | Yes | Communal open space provided above the commercial/retail podium. |
| Street wall heights and setbacks | Yes – capable at DA stage | The strategy indicates a 6-14m street wall height with a minimum 3m setback above the street wall to the tower form above. The proposal includes a max street wall height of 8-9m and the tower is setback a minimum of 3m from the podium. |
| Active street frontages | Yes | Gordon Avenue and Hammond Lane are both identified as active street frontages and the proposal includes both of these frontages on the active street frontages map. |
| Site isolation | Yes | The site is not isolated and at 1,522sqm is well above minimum lot size of 1,200sqm for the B4 Mixed Use area. While the proposal recognises that the adjoining site at 1-3 Gordon Avenue is below the minimum 1,200sqm threshold (being 1,122sqm) concept plans of a potential future development on this site have been provided within the proposal demonstrating that a development on this site could still maintain consistency with the majority of the Chatswood CBD Strategy key elements. |
| Traffic and transport | Yes | All access and parking requirements are consistent with the strategy including a single entry and exit in a forward direction, basement loading and parking requirements. The widening of Hammond Lane is also proposed as part of the proposal to enable an improved access arrangement for the subject site and neighbouring properties. |

## 5.4	Willoughby Local Strategic Planning Statement

On 10 February 2020, Council’s Final Local Strategic Planning Statement (LSPS) was adopted at its meeting. The Willoughby LSPS sets out the 20-year vision for land use in the LGA, the special character and values that are to be preserved and how change will be managed into the future.

On 20 March 2020, the LSPS was endorsed by the Greater Sydney Commission (GSC) and published to the planning portal on 31 March 2020.

Relevant to the subject site, Council’s LSPS supports the delivery of the key elements of the Chatswood CBD Planning and Urban Design Strategy as this is the key driver for housing, jobs and investment within the Willoughby LGA. An assessment of the proposal’s consistency against the key elements of the strategy is provided further within this report.

The planning proposal does not include any commentary on Council’s now endorsed LSPS as it was prepared and supported by Council prior to its LSPS being finalised.

It is therefore recommended that prior to public exhibition the proposal be updated to address and assess the proposal against all the relevant priorities and actions of Council’s Final LSPS.

## 5.5	Willoughby Local Housing Strategy

In May 2020, Council’s Local Housing Strategy (LHS) was finalised and forwarded to the Department for endorsement.

Council’s LHS targets three growth/focus areas for the delivery of its housing supply over the next 20 years including:

- Existing R3 Medium Density Residential and R4 High Density Residential zones which have not been developed to their full potential.

- The proposed expanded B4 Mixed Use area of the Chatswood CBD as identified in the Chatswood CBD Strategy.

- The local centres identified in Council’s Local Centres Strategy.

Of relevance to the subject proposal, the site falls within the Chatswood CBD Strategy area and is expected to deliver the majority of Council’s future dwelling capacity.

The proposal is consistent with Council’s finalised LHS as it supports the delivery of the key elements of the Chatswood CBD Strategy, however the actions and priorities within the LHS are not addressed within the planning proposal at it was prepared prior to Council’s LHS being finalised.

Therefore, a Gateway condition is recommended to be included which requires the planning proposal to be updated to include an assessment of the consistency of the proposal with Council’s LHS.

## 5.6	Section 9.1 Ministerial Directions

Direction 2.6 Remediation of Contaminated Land

Direction 2.6 aims to reduce the risk of harm to human health and the environment by ensuring that contamination and remediation are considered at the planning proposal stage.

This direction applies as there is intent to carry out residential development on land where there is no knowledge of whether the land is contaminated.

To the Department’s knowledge there is no reason to expect that the subject site is contaminated as it has been used for residential purposes for an extended period of time. The proposal is supported by a preliminary site investigation that confirms the site has been used for residential purposes since the early 1900s and the potential for contamination is likely to be low.

However, the report recommends additional groundwater testing following the excavation of the existing basement carparking noting that development to the south of the site consists of light-industrial uses including vehicle body repair and other automotive business.

Should any land contamination be identified during the detailed development application stage, groundwater testing and preparation of a remediation action plan or extent of the contamination and methods to remediate the site.

It is noted that the proposal incorrectly refers to SEPP 55 Remediation of Land as opposed to the current s9.1 direction. While the proposal is considered to achieve consistency with this direction, it is recommended that the proposal be updated to correctly address Direction 2.6.

Direction 3.1 Residential Zones

Direction 3.1 aims to encourage a variety of housing types, make efficient use of infrastructure and service and minimise the impact of residential development on the environment and resource lands.

This direction applies to the planning proposal as the proposal seeks to enable the provision of residential development.

The proposal will be increasing the potential residential yield in a strategic location in an existing B4 Mixed Use zone where residential uses are permissible. The proposal is considered to be consistent with this direction.

Direction 3.4 Integrating Land Use and Transport

Direction 3.4 aims to ensure that urban structures, building forms, land use locations, development designs, subdivision and street layouts:

- Improve active and public transport access to homes and jobs.

- Increase transport choices and reduce car dependency.

- Reduce travel demand.

- Support the operation of public transport services.

- Provide for the efficient movement of freight.

This direction applies to the planning proposal as the proposal seeks to alter a provision relating to urban land.

The proposal seeks to increase the overall density of the zone for both additional commercial and new residential purposes. The site which the proposal relates is located approximately 500m from the Chatswood transport interchange which includes rail, metro and bus services connecting Chatswood to the local area and nearby strategic centres of Macquarie Park, St Leonards, North Sydney and the Sydney CBD.

As the proposal will encourage the delivery of significant job and housing growth within close proximity to a major public transport interchange it is considered to be consistent with this direction.

Direction 3.5 Development Near Regulated Airports and Defence Airfields

Direction 3.5 aims at ensuring the effective and safe operation of airports and to minimise the impact of aircraft on noise sensitive land.

The maximum height of the planning proposal falls below the Radar Terrain Clearance (RTCC) as prescribed by the PANS-OPS (Procedures for Air Navigation Services, Operations), which varies in Chatswood from RL 246.8m AHD to RL 300m AHD.

The proposal is considered consistent with this direction.

Direction 4.1 Acid Sulfate Soils

Direction 4.1 aims to prevent significant adverse environmental impacts from the use of land that is affected by acid sulfate soils.

Site is subject to Class 5 acid sulphate soils as indicated by the Willoughby LEP 2012. As a result there is minimum risk of encountering acid sulfate soils under a future development application.

While the proposal is recognised to have a minimum risk in encountering acid sulphate soils, the proposal report does not address this direction. For the proposal to be considered consistent with this direction it is recommended that the report be updated to include an assessment against this direction.

Direction 4.3 Flood Prone Land

Direction 4.3 aims to ensure that any potential impacts associated with development on flood prone land are appropriately considered.

A desktop flood study has been prepared and submitted to support the planning proposal and identifies the site as being potentially being affected by the 100 year overland flow path. The report indicates that this is likely to result in a low/medium risk, without the inclusion of any stormwater infrastructure within the proposed development.

Future flood modelling is recommended to be provided during the detailed design stage where necessary flood infrastructure can be included within the development to minimise the potential flood risk to the subject site and neighbouring land.

While the proposal is considered to be able to achieve consistency with this direction, the planning proposal report does not address this direction. Therefore it is recommended that the report be updated to appropriately consider direction 4.3.

Direction 6.3 Site Specific Provisions

This direction aims to discourage unnecessarily restrictive site-specific planning controls.

The direction applies to the planning proposal at it seeks to implement additional specific provisions which only apply to the subject site.

The planning proposal seeks to include site specific provisions as recommended by the Chatswood CBD Strategy in relation to minimum commercial FSR, affordable housing and design excellence in addition to the specific height of buildings, FSR and zoning requirements included within the strategy.

These requirements are not considered unnecessarily restrictive as they are reflective of the holistic strategy recommendations which are currently being prepared through site specific planning proposal and will be implemented at a wider level within Council’s future planning comprehensive LEP amendment. This arrangement is considered acceptable.

However, while the proposal is considered consistent with this direction the proposal report does not address it adequately. Therefore it is recommended that the report be updated to provide an assessment against this direction.

## 5.7	State Environmental Planning Policies (SEPPs)

SEPP (Infrastructure) 2007

The planning proposal would likely result in a development application where substantial excavation would be required for the basement parking. As the site is in proximity of the T1 North Shore Rail Line corridor, concurrence to the rail authority may be required with a resulting development application.

The planning proposal is recommended to be referred to the Department of Transport to provide commentary on any potential impact to rail infrastructure.

SEPP (Affordable Rental Housing) 2009

- Clause 4.4(2A)(b) of Willoughby LEP 2012 allows for any part of the floor area of a building that is to be used for affordable housing, not be included within the overall gross floor area for the purposes of calculating FSR. This allows the developer to contribute towards the additional affordable housing. Under Willoughby LEP 2012 affordable housing is to be provided in Area 3 as shown in the Special Provisions Area Map.

The proposal seeks to introduce a new area ‘Area 9’ within the existing affordable housing provisions of clause 6.8 which will include any additional residential floor space used for the purposes of affordable housing within the floor space ratio calculation and is consistent with the recommendations of the Chatswood CBD Strategy.

While affordable housing is generally provided and offered as a bonus floor space within Council’s LEP, considering the substantial uplift that has been offered for the B4 Mixed Use sites within the strategy area, the inclusion of affordable housing within the FSR calculation is considered appropriate and can be successfully delivered in a future development.

- Council has consistently supported the provision of affordable housing, in accessible locations where practical and this clause is considered to generate a positive social benefit for the community by contributing towards the delivery of affordable housing in area where there is critical need for more affordable housing and longer term options.

SEPP 65 Design Quality of Residential Flat Building Development

SEPP 65 applies to the residential component of any future mixed-use building on the site.

A concept plan has been submitted having regard to the Apartment Design Guide (ADG) which provides detail on how residential development proposals can meet SEPP 65’s design quality principles. The concept design achieves general compliance with this code in relation to site planning, overshadowing and building separation.

Part 2 of the ADG recommends developing the appropriate building height controls for a site, which include:

- Responding to the desired future scale and character of the street and local area;

- Considering the height of existing buildings that are unlikely to change (for example, a heritage item(s) or strata subdivided buildings);

- Providing adequate daylight and solar access to apartments within the development, but also adjoining properties and public domain; and

- Providing architectural roof design and roof-top communal open spaces, where appropriate.

The maximum scale of development proposed for the site is consistent with adjoining sites to the south and west and the future intention of the surrounding Chatswood CBD area.

# Site-specific Assessment

## 6.1	Built Form

Building height

The proposed height increase would result in a concept development of approximately 27 storeys including a two-storey commercial podium and 25-storey residential tower above

The concept design for the proposal identifies that the residential tower could achieve a maximum height of approximately RL 180m AHD.

The proposed residential tower is generally consistent with Council’s desired maximum height of buildings for the site as recommended under the Chatswood CBD Strategy (Figure 22). The site is not subject to a solar access plane and is consistent with the desired future character of buildings within the expanded CBD area along the western side of the T1 North Shore Rail Line.

The slender building form ensures that the amenity of neighbouring properties including views and solar access are retained where possible. Considering the significant strategic importance of Chatswood as a regional centre, development of existing underutilised sites is inevitable where the uplift is considered appropriate.

Figure 20: Chatswood CBD Strategy proposed maximum height map (source: Willoughby City Council).

Building separation and setbacks

It is considered that the concept design demonstrates that sufficient building separation and setbacks can be achieved on the site under the proposed controls, subject to a detailed design at development application stage. The concept design and supporting DCP details that:

- The proposed residential tower is capable of achieving compliance with the Apartment Design Guide for all neighbouring developments.

- Any commercial uses above street wall height will be setback a minimum of 6m from the site boundaries.

- Ground floor floorspace is intended to be maximise with a zero setback podium to allow for sufficient internal circulation, access and services.

It is also noted that under a future development application a SEPP 65 Design Report will be required to be submitted to ensure that the subsequently development is consistent with SEPP 65.

Any amenity impacts such as visual privacy and view loss can be appropriately addressed as part any future development application, although considering the site is located in a regionally significant growing centre it is likely that there will be unavoidable amenity impacts to neighbouring properties.

Solar access

The proposal indicated that approximately 89% of apartments within a residential tower built in accordance with the proposal would be capable of receiving a minimum of two hours of solar access to living areas in midwinter. This is consistent with Part 4A of the ADG of SEPP 65 and is considerable acceptable for the purposes of this planning proposal.

Overshadowing

- Clause 4.3A(8) of the Willoughby LEP 2012 requires that development consent must not be granted for the erection of a building within 50m of the Victoria Avenue/Chatswood Mall if that development would cause increased overshadowing impact in midwinter between 12pm and 2pm.

The Chatswood CBD Strategy seeks to further refine these protections with the inclusion of a solar access plane within its maximum height of buildings map (Figure 20) which seeks to protect the key public spaces as identified within the strategy (Figure 21). Although a supporting LEP clause has not been included within this proposal to support the intent of the strategy, it has been recommended as part of the Gateway conditions for PP_2020_WILLO_002_00.

The planning proposal is not affected by the solar access plane as indicated in the recommended height of buildings map under the Chatswood CBD Strategy and based on shadow diagrams submitted with the proposal (Attachments A5-A6), will not impact upon any of Chatswood’s key public spaces.

The shadow diagrams also indicated that due to the concept building’s slender tower design, any shadow created by the concept design will be fast moving and will maintain reasonable solar access and amenity to existing and proposed future development as part of the Chatswood CBD Strategy.

Figure 21: Proposed solar access protected areas under Chatswood CBD Strategy (source: Willoughby City Council).

View sharing

Any development on the site above the existing permitted building height would impact on the views currently obtained from the surrounding residential towers to the north, west and south of the site. The proposal states that the concept scheme has been designed to minimise impacts to views through the delivery of a slender tower form. While it is recognised that some views are likely to be lost as a result of a future development on site, the proposal is consistent with Council’s desired uplift in this location.

The loss of regional and district views from existing development is considered inevitable within a regional strategic centre such as Chatswood that seeks uplift for both the height of buildings and maximum FSR. However, an assessment of the extent of view loss has yet to be carried out as part of the planning proposal.

Prior to exhibition, Council as the planning proposal authority should determine whether there are likely to be significant regional or district views lost as a result of the planning and proposal and whether a view loss analysis is to be prepared as part of the exhibition material. This is recommended to be included as a Gateway condition.

As the proposal would deliver a development in excess of 35m in height, any future development application would be required to progress through a design review panel which could recommend refinements to the design to reduce amenity impacts where appropriate.

Noise

An acoustic assessment was prepared by Resonate, in support of the planning proposal has indicated that as the site is nearby the Pacific Highway and the T1 North Shore Rail Line that specific acoustic treatments for the glazing to ensure appropriate internal noise amenity can be achieved.

The report identifies that specific treatment requirements will be further considered during the detailed design stage as part of a development, although there are no adverse impacts expected to be created by the proposal.

Wind

The proposal is supported by a wind environment statement can be achieved at all trafficable outdoor areas within the site as a result of the proposal development, subject to the inclusion of several design features including landscaping and awnings at podium level and impregnable balustrades for balconies within the mixed-use tower.

Wind tunnelling has been recommended to be undertaken at the detailed design stage to ensure any additional wind mitigation measures can be included as part of any design. This wind analysis is considered acceptable for the purposes of this planning proposal noting that additional testing will be completed during the detailed design stage.

## 6.2	Social

Affordable housing

- Under clause 4.4(2A)(b) of Willoughby LEP 2012 any part of the floor area of a building that is to be used for affordable housing, is not calculated as overall gross floor area and does not contribute to FSR. This allows the developer to contribute towards the additional affordable housing. Under Willoughby LEP 2012 affordable housing is to be provided in Area 3 as shown in the Special Provisions Area Map.

- The proposal however seeks to include a new area ‘Area 9’ within this clause to enable the delivery of additional affordable housing in accordance with clause 4.4 of the LEP, where any affordable housing floor area is included within the total FSR measurement.

- While generally affordable housing is excluded for any floor space calculation, considering the extent of the proposed uplift above the existing base FSR for the site this outcome is still considered to generate a positive social benefit for the community by contributing towards the delivery of affordable housing in area where there is critical need for more affordable housing options.

Public amenity

The concept design of the proposal intends to improve and activate the existing footpaths surrounding the site to improve connectivity through the Chatswood CBD towards nearby public spaces and the Chatswood transport interchange.

Council is currently developing its Section 7.11 and 7.12 Contributions plans for the provision of social infrastructure within the Chatswood CBD which is expected to be adopted by Council following its recent exhibition that ended in mid-October.

The proposal is supported by a signed letter of offer to enter into a voluntary planning agreement (VPA) based on Council’s previous value capture scheme which was not supported by the Department within its partial endorsement letter for the Chatswood CBD Strategy. A revised offer is likely to be submitted by the proponent prior to the proposal being placed on public exhibition.

## 6.3	Environmental

Landscape

The site has previously been developed for urban purposes. No known critical habitat or threatened species populations or ecological communities or their habitats would be adversely affected by the proposal.

As the proposed concept intends provide a minimum 20% soft landscaped area as part of any future development (ground and above podium level) this loss of landscaping is considered acceptable.

Flooding

A desktop flood analysis has been prepared for the subject proposal to determine if the land is flood prone or impacted by overland flow paths. The report identifies that the subject site likely low risk of being affected by the 100 year overland flow path with depths up to around 0.2m.

The report recognises that the impact to this flow path is minor, it recommends that further testing at the detailed design stage is required to determine if specific flood mitigation techniques are to be incorporated in any future development application. These have been recommended, although not limited to include, raising the proposed building floor level 500m above the 100 year flood level and the crest of the basement set 300mm above the 100 year flood level.

It is noted that the subject site falls within the Scott’s Creek Catchment and that detailed flood information is not currently available for this property.

## 6.4	Economic

Employment

The proposal is not supported by an economic feasibility report, however the proposal states that it will enable the delivery of approximately 103 dwellings and 50 jobs within the site.

The planning proposal would result in a substantial increase to the current commercial offering on the subject site to align more closely with its current and proposed future use as a B4 Mixed Use zone as proposed within the Chatswood CBD Strategy.

This outcome has considerable benefit as it seeks to create additional jobs, which is a key outcome for Chatswood CBD and a key objective of the North District Plan. The proposal will also ensure that the development can accommodate diverse forms of employment by supporting both retail, office and business development.

The increase in opportunities for the provision of employment floorspace is considered to have positive economic impacts and support the key objective of the Chatswood CBD Strategy to deliver new jobs in a strategic location.

## 6.5	Infrastructure

Public transport

The site is well serviced with public transport infrastructure as it is within 150m walking distance of the Chatswood rail and bus interchange. The upgrading of the interchange to now accommodate the Sydney Metro has further enhanced Chatswood’s status as a strategic centre and key public transport interchange. This will continue to improve when the Metro network is extended to the Sydney CBD and Bankstown, which is due to open in 2024.

The planning proposal supports the principles of integrated land use and transport outcomes as it will:

- Rely upon existing and future transport capacity and services to support and encourage the use of public transport.

- Maintain good accessibility to the station and the associated interchange.

- Facilitate development that will deliver co-located housing and employment to provide opportunities to live and work in Chatswood.

Traffic

A traffic impact assessment report, as prepared by Varga Traffic Planning, has been submitted with the planning proposal and provides an assessment of the transport impacts resulting from the proposed concept development.

The report identifies that that based on Council’s DCP guidelines and RMS’s Guide to Traffic Generating Developments (RMS Guide):

- 80.8 car parking spaces (including visitor spaces) and 3 motorcycle spaces would be required to be provided on site with access via Hammond Lane.

- 21 bicycle spaces/lockers will be provided.

- A maximum of 40.3 vehicle trips per hour in the AM peak, 31.2 vehicle trips per hour in the PM peak and 379 trips in total on any given day.

The number of parking spaces is based on the RMS guide as Council’s DCP would require a higher provision of spaces at 98. The RMS guide is considered to be appropriate in the context of the site given its excellent accessibility to a major public transport node.

Based on the SIDRA analysis, the assessment finds that based on the existing traffic volumes near the site, the additional traffic generated by the amended planning proposal is not expected to compromise the safety or function of the surrounding road network.

All surrounding intersections (Pacific Highway and Gordon Avenue) would continue to operate with a level of service A during the peak commuter periods.

It is noted that the Department’s Chatswood CBD Strategy endorsement requires Council to engage Transport for NSW to establish if the strategy can accommodate the proposed growth and work out if a Transport Management Action Plan (TMAP) is required for all areas outside the CBD core. This was included as a condition of the endorsed strategy to ensure that the Chatswood road network could handle the proposed growth and future capacity within the CBD.

This analysis has been completed by Arup in conjunction with Transport for NSW and is currently with Transport for NSW for finalisation and adoption. As part of the Gateway determination it is recommended that a condition to consult with Transport for NSW is included to ensure that the planning proposal is consistent with the wider Chatswood CBD context and ongoing traffic and transport analysis.

Infrastructure and services

Any future development may require utility services to be upgraded and/or augmented to enable the future residential population to be accommodated.

As the proposal would intensify development on the site, it is recommended that relevant state infrastructure service providers are consulted as part of the Gateway determination, including Sydney Water, Ausgrid and the NSW Department of Education. These have been included as conditions.

# Consultation

## 7.1	Community

The planning proposal has recommended a 28 day consultation. This is adequate for the purposes of this plan.

## 7.2	Agencies

Consultation is to be undertaken with the relevant public agencies and adjoining landowners as per statutory requirements. It is recommended that consultation with the following agencies are recommended for the purposes of this Gateway:

- Transport for NSW.

- Ausgrid.

- Sydney Water Corporation.

- NSW Department of Education.

- NSW Department of Health.

- Sydney Airport Corporation.

# Timeframe

Council proposes a 12 month time frame to complete the LEP.

The Department recommends a time frame of 12 months to ensure it is completed in line with its commitment to reduce processing times. It is recommended that if the gateway is supported it also includes conditions requiring council to exhibit and report on the proposal by specified milestone dates.

# Local Plan Making Authority

As the Chatswood CBD Strategy has been endorsed by the Department and the proposal is able to demonstrate consistency with all key elements of the strategy, Council is recommended to be the local plan making authority for this proposal.

# Conclusion

The planning proposal seeks to redevelop the existing aged residential flat buildings for the purposes of a mixed-use development comprising 103 dwellings including affordable housing and 1,522sqm of commercial floor space (50 jobs).

The Department has considered the submitted documentation as part of the proposal and concludes that there is sufficient strategic merit in issuing a Gateway determination as:

- It is consistent with the actions of the North District Plan for Chatswood by providing additional commercial development capacity, maximises public transport patronage, promotes employment growth within an existing commercial zone and increases residential capacity in an accessible location.

- It is consistent with the key elements of the endorsed Chatswood CBD Strategy.

- The introduction a genuine mixed-use development would help to service the daytime and night-time economies of the retail precinct and further activates the CBD core area.

- The proposal will provide 103 dwellings including affordable housing within the Chatswood CBD on a site that supports the 30 minute city outcomes sought by the North District Plan.

The Department considers the proposal to have site-specific merit because:

- The proposed maximum building height and is consistent with the desired future maximum building heights and solar access planes for the Chatswood CBD.

- It is considered that the proposal is capable of adequate building separation, amenity and solar access outcomes to existing and proposed future development near the site.

- The proposal will enable the delivery of additional affordable housing and job growth in the Chatswood CBD.

- The proposal will deliver a mix of uses on the site within 500m walking distance of the Chatswood transport interchange and CBD core.

# Recommendation

It is recommended that the delegate of the Minister determine that the planning proposal should proceed subject to the following conditions:

- Prior to community consultation the proposal is to be updated to include:

  - An updated project timeline based on this Gateway determination.

  - Removal of clauses 6.8 and 6.24 relating to affordable housing and design excellence as planning proposal PP_2019_WILLO_002_00 includes these clauses and has been submitted to the Department for finalisation.

  - Removal of discussion regarding A Plan for Growing Sydney.

  - Removal of section 9.1 Ministerial Direction 7.1 A Plan for Growing Sydney.

  - The objectives of the planning proposal are to be updated to include the proposals consistency with the Willoughby Council’s Local Strategic Planning Statement and Local Housing Strategy.

  - The objectives of the planning proposal are to be updated to remove the payment of developer contributions or levies as this is not an appropriate objective for the purposes of a planning proposal.

  - Include an assessment of the proposal against the relevant priorities and actions of the endorsed Willoughby Local Strategic Planning Statement and the finalised Willoughby Local Housing Strategy.

  - Willoughby LEP 2012 Clause 4.3A(8) is to be updated to be consistent with the recommendations of the now finalised Chatswood CBD Strategy regarding solar access protection to heritage conservation areas for a minimum of 3 hours between 9am-3pm during mid winter.

  - The planning proposal is to be updated to include an assessment against the following s9.1 Ministerial directions:

    - 2.6 Remediation of contaminated land

    - 4.3 Flood prone land

  - Council should consider whether the planning proposal is likely to impact upon significant regional or district views for neighbouring properties and if a view sharing analysis should be prepared to support the proposal during exhibition.

- The planning proposal should be made available for community consultation for a minimum of 28 days.

- Consultation is required with the following public authorities:

- Transport for NSW.

- Ausgrid.

- Sydney Water.

- NSW Department of Education.

- NSW Department of Health.

- The planning proposal must be exhibited 6 months from the date of the Gateway determination.

- The planning proposal must be reported to council (or Planning Panel delete as required) for a final recommendation 9 months from the date of the Gateway determination.

- The timeframe for completing the LEP is to be 12 months from the date of the Gateway determination.

- Given the nature of the planning proposal and its consistency with the Chatswood CBD Strategy, Council authorised to be the local plan-making authority to make this plan.

Brendan Metcalfe

Acting Director, North District

Eastern Harbour City

Assessment officer: Nick Armstrong

Acting Manager, North District

Phone:8275 1219

# Attachments

Attachment Gateway – Gateway Determination

Attachment Letter – Letter to Council advising of decision

Attachment A – Planning proposal documentation

- A1 – Planning proposal cover letter

- A2 – Returning of planning proposal to Department for Gateway

- A3 – Planning proposal report

- A4 – Draft DCP

- A5 – Urban design report

- A6 – Concept plans

- A7 – Traffic report

- A8 – Acoustic report

- A9 – Wind impact assessment

- A10 – Flood impact assessment

- A11 – Phase 1 land contamination report

- A12 – Draft VPA

- A13 – Proposed written LEP amendments

Attachment B – Site map

Attachment C – Locality context map

Attachment D – Existing LEP maps

Attachment E – Proposed LEP maps

Attachment F – Council correspondence

- F1 – Council report

- F2 – Detailed assessment report

- F3 – Willoughby Local Planning Panel minutes

- F4 – Council resolution

Attachment G – Chatswood CBD Strategy

- G1 – Chatswood CBD Strategy background summary

- G2 – Department’s partial endorsement letter

- G3 – Department’s full endorsement

- G4 – Final Chatswood CBD Planning and Urban Design Strategy