(2016SYW169) - Draft Conditions of Consent– DA-767/2016

A. 	THE DEVELOPMENT

Approved Plans/Reports

- Development the subject of this determination notice must be carried out strictly in accordance with the following plans/reports marked as follows:

| Plan Name | Plan Number | Date | Issue | Prepared By |
| --- | --- | --- | --- | --- |
| Architectural Plans | DA10 – DA61 | 18/12/2018 | A | Saunders Global |
| Landscape Concept Plan – Podium (L4) | - | 21/11/2018 | B | Habitat8 |
| Plant Schedule | - | 21/11/2018 | B | Habitat8 |
| Swept Path Analysis Top: B99 Vehicle Bottom: 10.5m Rigid Truck (Substation) | SK.01 | 16/12/2019 | - | TRAFFIX traffic and transport planners |

| Report name | Date | Reference | Prepared by |
| --- | --- | --- | --- |
| Acoustic Assessment | 10/07/2019 | 20170647.1/2405A/R5/YK | Acoustic Logic |
| BASIX Certificate | 21/12/2018 | 960386M | SLR Consulting Australia Pty Ltd |
| Design Verification Statement | 21/12/2018 | - | Eric Pui Tong LEUNG On behalf of Saunders Global |
| Geotechnical Report | 10/12/2018 | E23665.G03_Rev2 | EI Australia |
| Section J Report | 15/12/2018 | 610.18183-R03-v1.0 | SLR Consulting Australia Pty Ltd |
| Traffic Impact Assessment Report | 16/12/2018 | 18.031r01v03 | TRAFFIX traffic and transport planners |
| Environmental Wind Tunnel Testing | 13/12/2018 | 610.18375‐R01‐v1.0 | SLR Consulting Australia Pty Ltd |
| Operational Waste Management Plan | 18/12/2018 | 18051 Revision D | Elephants Foot Recycling Solutions |

Works at no cost to Council

- All roadworks, drainage works and dedications, required to effect the consented development shall be undertaken at no cost to Liverpool City Council.

Fire Safety – Cladding

- For all Buildings of Type A and B construction having finishes or claddings other than concrete or masonry, a fire safety report prepared by an accredited C10 fire engineer, must be submitted to the Principal Certifying Authority prior to issue of a construction certificate, demonstrating that the proposed external wall cladding material and system for the building complies with the NCC and relevant Australian Standards. The fire safety report is to include evidence of suitability of all proposed external wall cladding materials as per clause A2.2 of the NCC.

Inconsistency between Plan and Reports

- The architectural plans approved under Condition 1 of this consent is to be prevail to the extent of any inconsistency between those plans, drainage concept plans and the reports approved under Condition 1 of this consent.

Heritage

- All relevant staff, contractors and subcontractors should be made aware of their statutory obligations for heritage under NSW National Parks and Wildlife Act 1974, NSW Heritage Act 1977 and best practice as outlined in The Burra Charter 2013. This may be implemented as a heritage induction in relation to the significance of the street grid and the potential for historical archaeology.

- In the unlikely event that any Aboriginal objects or places of Aboriginal heritage significance are identified in the project area during proposed clearance and excavation tasks, all works in the area should cease. The area should be cordoned off and contact made with the Heritage Division, Office of Environment and Heritage, NSW Department of Planning and Environment (131 555), a suitably qualified archaeologist and the relevant Aboriginal stakeholders, so that the Aboriginal heritage can be adequately assessed and managed.

- In the unlikely event that skeletal remains are identified during the proposed works, work in the vicinity of the remains must cease immediately. The area must be cordoned off. The NSW Police Coroner must be contacted in order to determine if the material is of Aboriginal origin. If determined to be of Aboriginal origin, the Client must contact the Heritage Division, Office of Environment and Heritage, NSW Department of Planning and Environment (131 555), along with a suitably qualified archaeologist and the relevant Aboriginal stakeholders, so that the remains can be adequately assessed and managed.

- If, during the proposed works suspected archaeological relics, as defined by the Heritage Act 1977 are uncovered, work should cease in that area immediately. The Heritage Division, Office of Environment and Heritage, NSW Department of Planning, Industry and Environment (131 555) should be notified and works only recommence when relevant permits and an appropriate and approved management strategy instigated.

- If, during the proposed works the external building design is altered, then the changes should be the subject of a review by Council Planning officers, so as to ensure that the design intent in so far as the heritage impact of the works is acceptable.

- The design for the landscaped space at the Bathurst Street frontage on ground level including the materials, finishes and details should be reconsidered. An urban landscape which includes a predominantly a traditional granite styled paved surface with trees set in traditional grates and selected bench seating would be appropriate. Garden beds should be limited to linear elements used to define the property boundary. The revised design to be submitted for review and approval to Council to ensure that the cultural heritage significance of the central Liverpool street grid is protected and enhanced.

- The design for the landscaped space at the Bathurst Street frontage on ground level should include an interpretive device within the pavement or street furniture which provides an indication of the heritage significance of the Street grid and Bathurst Street and should be prepared in conjunction with the Liverpool Historical Society and an experienced Interpretation consultant.

#### Section 7.12 Payment (Liverpool Contributions Plan 2018 Liverpool City Centre)

- As a consequence of this development, Council has identified an increased demand for public amenities and public services. The following payment represents 3% of the cost of the development and is imposed in accordance with Liverpool Contributions Plan 2018 Liverpool City Centre. 

The total contribution is $827,970.00 and will be adjusted at the time of payment in accordance with the contributions plan.

A breakdown of the contribution’s payable is provided in the attached payment form.

The Contributions Plan may be inspected online at www.liverpool.nsw.gov.au

Payment must be accompanied by the attached form.

In response to Covid-19, Council is providing for deferred payment options, for applications lodged or approved between 16 April 2020 and 31 December 2020. These provide for a deferral of 50% of the contribution amount to the occupation certificate stage. Please contact Council to discuss these options.

B. 	PRIOR TO ISSUE OF A CONSTRUCTION CERTIFICATE

The following conditions shall be complied with prior to issue of a CC by the PCA:

Long Service Payment

- Long Service Levy payment is applicable on building work having a value of $25,000 or more, at the rate of 0.35% of the cost of the works. The required Long Service Levy payment, under the Building and Construction Industry Long Service Payments Act 1986, is to be forwarded to the Long Service Levy Corporation or the Council, prior to the issuing of a Construction Certificate, in accordance with Section 109F of the Environmental Planning & Assessment Act 1979.

Compliance with the National Construction Code

- In accordance with section 4.17 of the Environmental Planning & Assessment Act 1979 and clause 98 of the Environmental Planning & Assessment Regulation 2000, it is a prescribed condition that all building work must be carried out in accordance with the applicable Performance Requirements of the National Construction Code. Compliance with the Performance Requirements can only be achieved by:

(a)	Complying with the Deemed to Satisfy Provisions; or

(b)	Formulating an Alternative Solution, which complies with the Performance Requirements or is shown to be at least equivalent to the Deemed to Satisfy Provision, or a combination of (a) and (b).

- Access must be provided to the building for people with a disability in accordance with the relevant requirements of the Building Code of Australia, Disability (access to Premises – Buildings) Standard 2010 and Australian Standard – AS1428.1 (2009), Design for Access and Mobility – General requirements for new building work, to the satisfaction of the Certifying Authority.

- A fire safety report prepared by an accredited C10 fire engineer, must be submitted to the PCA prior to issue of construction certificate, demonstrating that all proposed external wall cladding materials used for the building complies with the NCC and relevant Australian Standards. The fire safety report is to include evidence of suitability of all proposed external wall cladding materials as per clause A2.2 of the NCC.

- A schedule specifying all of the essential fire safety services, both existing and proposed, which are required for the building, shall be attached to the CC and submitted to Council, in compliance with the provisions of the EP&A Regulation.

The schedule must distinguish between the measures that are currently implemented in the building premises; and

(a) The measures that are to be proposed to be implemented in the building premises;

and

(b) The minimum standard of performance for each measure.

Provision of Services

- An application to obtain a Section 73 Compliance Certificate under the Sydney Water Act 1994, must be lodged with Sydney Water. To facilitate this, an application must be made through an authorised Water Servicing Coordinator. Please refer to the “building and developing” section of Sydney Water’s web site at www.sydneywater.com.au, or telephone 13 20 92.

Following receipt of the application, a ‘Notice of Requirements’ will detail water and sewer extensions to be built and charges to be paid. Please make early contact with the Coordinator, since building of water/sewer extensions can be time consuming and may impact on other services and building, driveway or landscape design. A copy of the ‘Notice of Requirements’ must be submitted to the PCA.

- Written clearance from Endeavour Energy, stating that electrical services have been made available to the development or that arrangements have been entered into for the provision of services to the development must be submitted to the PCA.

- Prior to the issue of a Construction Certificate, the Principal Certifying Authority shall be satisfied that telecommunications infrastructure may be installed to service the premises which complies with the following:

- The requirements of the Telecommunications Act 1997;

- For a fibre ready facility, the NBN Co’s standard specifications current at the time of

- installation; and

- For a line that is to connect a lot to telecommunications infrastructure external to the premises, the line shall be located underground.

Unless otherwise stipulated by telecommunications legislation at the time of construction, the development must be provided with all necessary pits and pipes, and conduits to accommodate the future connection of optic fibre technology telecommunications.

Design Verification Statement

- In accordance with the Environmental Planning and Assessment Regulation 2000 and State Environmental Planning Policy (SEPP) 65 “Design Quality of Residential Apartment Development”, the subject development must be undertaken or directed by a ‘qualified designer’ (i.e., a registered architect under the Architects Act). In this regard, a design verification statement shall be submitted to the PCA. The PCA shall ensure that the statement prepared by the qualified designer provides the following:

- A valid and current architect’s registration number (as issued by the NSW Architects Registration Board);

- That the qualified designer has designed or directed the design of the subject development; and

- That the plans and specifications lodged with the CC achieve or improve the design quality of the development for which the subject development consent was granted, having regard to the design principles set out in Part 2 of SEPP 65.

Note: The design verification statement must provide an explanation of the design in terms of the design quality principles set out in Part 2 of SEPP 65.

Recommendations of Acoustic Report

- The recommendations provided in the approved acoustic report 64 Bathurst Street, Liverpool DA Acoustic Assessment (Doc ref. 20170647.1/2405A/R5/YK, Revision 5) prepared by Acoustic Logic Consultancy Pty Ltd dated 10 July 2019 are to be implemented and incorporated into the design and construction of the development and be shown on plans accompanying the Construction Certificate application.

The construction methodology and plans accompanying the Construction Certificate application shall be assessed and certified in writing by a suitably qualified acoustic consultant to verify conformance with the requirements of the aforementioned acoustic report. The written certification from the suitably qualified acoustic consultant shall be submitted to and approved by the PCA prior to the Construction Certificate being issued.

Note: ‘Suitably qualified acoustic consultant’ means a consultant who possesses the qualifications to render them eligible for membership of the Australian Acoustic Society or by an Association of Australasian Acoustical Consultants (AAAC) member firm.

Construction Environmental Management Plan (CEMP)

- Prior to issue of a construction certificate, a Construction Environmental Management Plan (CEMP) for the development must be provided to the Principal Certifying Authority for approval. The environmental site management measures must remain in place and be maintained throughout the period of the development. The CEMP must address all environmental aspects of the development’s construction phases, and include (where relevant), but not be limited to, the following:

- Asbestos Management Plan;

- Project Contact Information;

- Site Security Details;

- Timing and Sequencing Information;

- Site Soil and Water Management Plan;

- Dust Control Plan;

- Air Monitoring;

- Odour Control Plan;

- Health and Safety Plan;

- Waste Management Plan;

- Incident management Contingency; and

- Unexpected Finds Protocol.

The CEMP must be kept on site for the duration of the works and must be made available to Council Officers upon request.

Waste Storage Area

- All waste management facilities shall comply with the Liverpool Development Control Plan 2008. Prior to the issue of a Construction Certificate, detailed floor and section plans shall be submitted to and approved by the Principal Certifying Authority for the waste storage area/s. The plans shall identify the location of the waste storage area/s and incorporate the following requirements:

- Sufficient space for access, storage and easy manoeuvring of bins;

- The areas are to be fully enclosed and include a solid roof and concrete or cement rendered walls coved to the floor;

- A concrete floor graded to an approved sewer connection comprising a sump and galvanised fine grade drain cover sufficient to prevent coarse pollutants from entering the sewer;

- Adequate ventilation to the external air by natural or mechanical means;

- The door to the rooms must be tight fitting and self-closing;

- A hose cock adjacent to the garbage storage areas to facilitate cleaning of bins and the storage areas. If the hose cock is located inside the waste storage areas, it shall not protrude into the space indicated for the placement of bins;

- Sufficient lighting to permit usage at night; and

- Should garbage chutes be integrated into the design of the building, operational instructions shall be located prominently next to the chute.

Any modifications to the construction of the waste storage area require Council’s prior written Approval.

Construction Noise and Vibration Management Plan

- A Construction Nosie and Vibration Management Plan is to be prepared by a suitably qualified acoustic consultant in accordance with relevant planning policies, guidelines and standards (whether that be Australian, British or German) and is to include, but not be limited to, the following:

- Site description

- Nature of the development

- Staging of construction

- Hours of construction

- A quantitative assessment of the airborne and ground-borne noise generated by the work for the proposed development and its impact on nearby receivers

- A quantitative vibration assessment and the impact to surrounding structures and on nearby receivers

- Proposed methods to mitigate the noise and vibration impact during the construction works

- Complaints handling and community liaison procedures. This is to include but not be limited to a complaints register with complainant details, date and time of complaint, nature of complaint and how the complaint was resolved or handled.

Once the noise and vibration impact on the surrounding community and structures is adequately assessed, all necessary action is to be taken to eliminate such impacts as recommended and suggested by the suitably qualified acoustic consultant.

S138 Roads Act – Minor Works in the public road

- Prior to the issue of a Construction Certificate, a Section 138 Roads Act 1993 application/s, including payment of fees, shall be lodged with Liverpool City Council, as the Roads Authority for any works required in a public road.  These works may include but are not limited to:

- Vehicular crossings (including kerb reinstatement of redundant vehicular crossings),

- Road opening for utilities and stormwater (including stormwater connection to Council infrastructure), or

- Road occupancy or road closures.

All works shall be carried out in accordance with the Roads Act approval, the development consent including the stamped approved plans, and Liverpool City Council’s.

Liverpool CBD – Street Lighting Upgrade

- The applicant/developer shall upgrade street lighting system for entire frontage of the development including any side streets. The street light pole shall be multi-function pole including all necessary accessories. The specification and accessories details are to be obtained from Infrastructure and Environment Group of Council. The applicant/developer shall engage services of Endeavour Energy accredited ASP Level 3 service provider to prepare electrical design and obtain certification from Endeavour Energy. The electrical design shall include undergrounding of existing aerial power lines and communication cables for entire frontage of the development including frontage to side streets. The approved design shall be submitted to Council for review and approval prior to any public domain construction works.

Liverpool CBD – Communication Conduits

- The applicant/developer shall also supply and install two 50mm white communication conduit with draw wires approximately 300mm behind kerb and gutter connecting multifunction poles. Details can be obtained from Infrastructure and Environment Group of Council. Please contact the Coordinator – Roads and Transport on 1300 36 2170.

Liverpool CBD – Footpath Paving and Landscaping Works

- Periphery Type/ Core Type paving shall be installed along the entire Bathurst Street and Huckstepp Serviceway frontages, as part of this development. Footpath paving and Landscaping works shall be strictly in accordance with the Liverpool CBD Street Tree and Landscape Strategy 2005 and The Liverpool CBD Streetscape and Paving Guidelines 2005 as amended in Implementation Note 12/2015 – Liverpool CBD Paving

Detailed plans are required to be submitted to and approved by Council showing the proposed tree locations, species and planting sizes, paving location and layout, including references to the relevant details and specifications as contained in the abovementioned documents.

To ensure that the street tree planting size, quantity and quality is maintained throughout the Liverpool CBD, please contact Council’s Land Development Section on 1300 36 2170 for further information.

A Section 138 Roads Act Approval for all works within Council’s road reserve will be required.

Retaining Walls on Boundary

- All retaining walls shall be of masonry construction and must be wholly within the property boundary, including footings and agricultural drainage lines.  Construction of retaining walls or associated drainage works along common boundaries shall not compromise the structural integrity of any existing structures.

Where a retaining wall exceeds 600mm in height, the wall shall be designed by a practicing structural engineer and a construction certificate must be obtained prior to commencement of works on the retaining wall.

S138 Roads Act – roadworks requiring approval of civil drawings

- Prior to the issue of a Construction Certificate for building or subdivision works the Certifying Authority shall ensure that a S138 Roads Act application, including the payment of application and inspection fees, has been lodged with Liverpool City Council (being the Roads Authority under the Roads Act), for construction of stormwater pipe and CBD footpath paving), in (Bathurst Street and Huckstepp Serviceway).

Engineering plans are to be prepared in accordance with the development consent, Liverpool City Council’s Design Guidelines and Construction Specification for Civil Works, Austroad Guidelines and best engineering practice.

Note: Where Liverpool City Council is the Certifying Authority for the development the Roads Act approval for the above works may be issued concurrently with the Construction Certificate.

Stormwater Concept Plan

- A stormwater drainage system shall be provided generally in accordance with the concept plan/s lodged for development approval, prepared by Xavier Knight, reference number 180201, revision 01, dated 26/10/2018:

- The proposed development and stormwater drainage system shall be designed to ensure that stormwater runoff from upstream properties is conveyed through the site without adverse impact on the development or adjoining properties.

- Engineering plans and supporting calculations for the stormwater drainage system are to be prepared by a suitably qualified engineer and shall accompany the application for a Construction Certificate.  The plan shall indicate the method of disposal of all stormwater and must include rainwater tanks, existing ground levels, finish surface levels and sizes of all pipes.

- Prior to the issue of a Construction Certificate the Certifying Authority shall ensure that the stormwater drainage system has been designed in accordance with Liverpool City Council’s Design Guidelines and Construction Specification for Civil Works.

Stormwater Discharge – Basement Car parks

- Prior to the issue of a Construction Certificate the Certifying Authority shall ensure that the stormwater drainage system for the basement car park has been designed in accordance with the requirements for pumped systems in AS3500.3:2003 and Council’s Stormwater Drainage Design Specifications for pump out systems for basement carparks.

Water Quality

- Prior to the issue of a Construction Certificate, the Certifying Authority shall ensure that details of a stormwater pre-treatment system have been provided on the stormwater plans and that the design meets pollutant retention criteria in accordance Council’s Development Control Plan.The Construction Certificate must be supported by:

- Specification & installation details of the stormwater pre-treatment system

- The approval of an operation and maintenance manual/  schedule for the stormwater pre-treatment system

- A copy of the approved operation and maintenance manual/ schedule shall be submitted to Liverpool City Council with notification of the Construction Certificate issue.

Peer Review of Geotechnical Report

- An independent peer review by a structural engineer/geotechnical engineer of the geotechnical report and future structural design of the development will be required at no cost to Council. The peer review shall be submitted to the Principal Certifying Authority for approval. This is to ensure that there are no impacts to the existing embankment at the rear of the property within the Council reserve.

Construction Traffic Management Plan

- A Construction Traffic Management Plan (CTMP) detailing construction vehicle routes, number of trucks, hours of operation, access arrangements and traffic control, if any, is to be submitted to Council for review prior to the issue of CC.

Access, Car Parking and Manoeuvring - General

- Prior to the issue of a Construction Certificate the Certifying Authority shall ensure that vehicular access, circulation, manoeuvring, pedestrian and parking areas associated with the subject development are in accordance with AS 2890.1, AS2890.2, AS2890.6 and Liverpool City Council’s Development Control Plan.

Dilapidation Report

- Prior to the Commencement of Works a dilapidation report of all infrastructure fronting the development in Bathurst Street and Huckstepp Serviceway is to be submitted to Liverpool City Council.  The report is to include, but not limited to, the road pavement, kerb and gutter, footpath, services and street trees and is to extend 15m either side of the development.

Dilapidation Report Private Property (Excavations)

- A full dilapidation survey and report on the visible and structural condition of all neighbouring structures within the ‘zone of influence’ of the required excavations must be submitted to the Certifying Authority for approval prior to the issue of any Construction Certificate. The zone of influence is to be defined as the horizontal distance from the edge of the excavation face to twice the excavation depth.

The dilapidation report and survey is to be prepared by a consulting structural or a geotechnical engineer agreed to by both the applicant and the owner of any affected adjoining property.

All costs incurred in achieving compliance with this condition shall be borne by the person entitled to act on this Consent.

In the event that access for undertaking the dilapidation survey is denied by an adjoining owner, the applicant MUST DEMONSTRATE, in writing, to the satisfaction of Council that all reasonable steps have been taken to obtain access and advise the affected property owner of the reason for the survey and that these steps have failed. Written concurrence must be obtained from Council in such circumstances.

Note: This documentation is for record keeping purposes only, and may be used by the developer or affected property owner to assist in any action required to resolve any dispute over damage to adjoining properties arising from the works. It is in the applicant’s and adjoining owner’s interest for it to be as full and detailed as possible.

Garbage Services

- On site collection of waste and recycling must be provided and integrated with the design of high density residential development. This must comply with the specifications detailed in the Fact Sheet: Waste Management Services for Residential Flat Buildings and Multi Dwelling Housing.

- The developer/owner of the site is to contact Liverpool Council- Waste Management Section to determine the required number of waste and recycle bins for the residential component of the development as well as servicing requirements. These waste and recycle bins are to be kept at all times within the residential waste storage rooms except before and after collection days. Waste and Recycle bins are to be returned to the storage rooms within 24 hours of collection.

- Any bin bays must be:

- Provided with mechanical ventilation,

- Provided with a hose cock for hosing the garbage bin bay and a sewered drainage point in or adjacent to the bin storage area. The drainage point should have a fine grade drain cover sufficient to prevent coarse pollutants from entering the sewer. If the hose cock is located inside the bin storage bay, it is not to protrude into the space indicated for the placement of bins,

- Provided with sufficient light to permit usage at night,

- Allocated with sufficient space within the bin bay to allow for access to all required bins by residents and waste collectors, as well as manoeuvring of bins within the bay and for the removal and return of bins by the waste collector,

- Bin bay signs are available from Council,

- Should garbage chutes be incorporated into the design of the building, signage on how to use the chutes is to located prominently next to the chute,

- Operation, maintenance and cleaning of the garbage compactor and associated equipment is the responsibility of the strata management or body corporate and not of Council,

- Maximum compaction ratio is 2:1,

- Operation, maintenance and cleaning of the garbage chutes and associated waste cupboards, rooms, or equipment is the responsibility of the strata management or body corporate, and not of Council, and

- Provided with signage to be prominently displayed in each bin bay, or waste service room, as appropriate indicating that:

- Garbage is to be placed wholly within the garbage bins provided,

- Only recyclable materials accepted by Council are to be placed within the recycling bins,

- The area it to be kept tidy,

- A phone number for arranging disposal of bulky items, and

- Graphic illustrative content to be 50%.

C. PRIOR TO WORKS COMMENCING

The following conditions shall be complied with prior to works commencing on the subject site:

- Prior to the commencement of any building works, the following requirements must be complied with:

  - A Construction Certificate must be obtained from the Council or an accredited certifier, in accordance with the provisions of the Environmental Planning & Assessment Act 1979. Where a Construction Certificate is obtained from an Accredited Certifier, the applicant shall advise Council of the name, address and contact number of the Accredited Certifier, in accordance with Section 81A (4) of the Act.

  - A copy of the Construction Certificate, the approved development consent plans and consent conditions must be kept on the site at all times and be made available to the Council officers and all building contractors for assessment.

  - A Principal Certifying Authority (PCA) must be appointed to carry out the necessary building inspections and to issue an occupation certificate; and

  - The PCA must advise Council of the intended date to commence work which is the subject of this consent by completing a notice of commencement of building works or subdivision works form, available from Council’s Customer Service Centre. A minimum period of two (2) working days’ notice must be given.

Residential Building Work

- Building work that involves residential building work (within the meaning of the Home

Building Act 1989) must not be commenced unless the principal certifying authority for the development to which the work relates (not being the council) has given the council written notice of the name and licence number of the principal contractor; and the name of the insurer by which the work is insured under Part 6 of that Act.

Notification of Service Providers

- The approved development must be approved through the ‘Sydney Water Tap In’ service to determine whether the development will affect any Sydney Water wastewater and water mains, stormwater drains and/or easement, and if any requirements need to be met. A receipt must be provided to Council.

Please refer to the website www.sydneywater.com.au for more information.

Complaints Register

- Prior to the commencement of construction, the Applicant must ensure that the following is available for the life of the Development:

(a) a postal address to which written complaints may be sent;

(b) an email address to which electronic complaints may be transmitted; and

(c) a telephone contact line to enable complaints associated with the Development to

be registered by the community.

The Complaints Register must be made available for inspection on request by Liverpool City Council.

Waste Classification and Disposal of Contaminated Soil and Material

- All soils and material(s), liquid and solid, to be removed from the site must be analysed

and classified by an appropriately qualified and certified consultant, in accordance with the Protection of the Environment Operations (Waste) Regulation 2014 and related guidelines, in particular the NSW EPA Waste Classification Guidelines, prior to off­site disposal.

All Waste material(s) must be disposed of at an appropriately licensed waste facility for the specific waste. Receipts for the disposal of the waste must be submitted to the Principal Certifying Authority within 30 days of the waste being disposed of.

All waste must be transported by a contractor licenced to transport the specific waste, and in vehicles capable of carting the waste without spillage, and meeting relevant requirements and standards. All loads must be covered prior to vehicles leaving the site.

Sediment and Erosion Control Measures

- Prior to commencement of works sediment and erosion control measures shall be installed in accordance with the approved Construction Certificate and to ensure compliance with the Protection of the Environment Operations Act 1997 and Landcom’s publication “Managing Urban Stormwater – Soils and Construction (2004)” – also known as “The Blue Book”.

The erosion and sediment control measures shall remain in place and be maintained until all disturbed areas have been rehabilitated and stabilised.

Visible signage identifying key personnel

- Clearly visible signage with the Site or Construction Managers name and contact details are to be erected on the sites fencing in a prominent position along the perimeter of the building site so that any community concerns can be adequately addressed.

The signage will contain the following wording or similar: “For enquires, complaints or emergencies relating to this site at any time please contact ……”.

The signs are to remain erected for the duration of construction works.

Traffic Control Plan

- Prior to commencement of works a Traffic Control Plan including details for pedestrian management, prepared in accordance with AS1742.3 “Traffic Control Devices for Works on Roads” and the Roads and Traffic Authority’s publication “Traffic Control at Worksites” and certified by an appropriately accredited Roads and Traffic Authority Traffic Controller is to be submitted to Council’s Traffic and Transport Section for approval.

The approval traffic control measures shall be implemented during the construction phase of the development in accordance with the certified plan. A copy of the plan shall be available on site at all times.

Note: A copy of the Traffic Control Plan shall accompany the Notice of Commencement to Liverpool City Council.

Construction Noise

- Construction noise shall not exceed the management levels defined within the Interim Construction Noise Guideline published by the NSW Department of Environment and Climate Change dated July 2009.

D. 	DURING CONSTRUCTION

The following conditions shall be complied with during construction:

Identification Survey Report

- The building and external walls are not to proceed past ground floor/reinforcing steel level until such time as the PCA has been supplied with an identification survey report prepared by a registered surveyor certifying that the floor levels and external wall locations to be constructed, comply with the approved plans, finished floor levels and setbacks to boundary/boundaries. The slab shall not be poured, nor works continue, until the PCA has advised the builder/developer that the floor level and external wall setback details shown on the submitted survey are satisfactory.

In the event that Council is not the PCA, a copy of the survey shall be provided to Council within three (3) working days. On placement of the concrete, works again shall not continue until the PCA has issued a certificate stating that the condition of the approval has been complied with and that the slab has been poured at the approved levels.

Site Facilities

- Toilet facilities must be available or provided at the work site and must be maintained until the works are completed at a ratio of one toilet plus one additional toilet for every 20 persons employed at the site. Each toilet must:

(a) be a standard flushing toilet connected to a public sewer, or

(b) have an on-site effluent disposal system approved under the Local Government Act

1993, or

(c) be a temporary chemical closet approved under the Local Government Act 1993.

- Adequate refuse disposal methods and builders storage facilities shall be installed on the site. Builders’ wastes, materials or sheds are not to be placed on any property other than that which this approval relates to.

Site Notice Board

- A sign must be erected in a prominent position on the premises on which work is to be carried out. The sign is to be maintained during work, and removed at the completion of work. The sign must state:

(a) The name, address and telephone number of the principal certifying authority for

the work;

(b) The name of the principal contractor (if any) for any building work and a telephone

number on which that person may be contacted outside working hours; and

(c) Unauthorised entry to the premises is prohibited.

Excavation Works

- All excavation works to be carried in accordance with recommendations of the Geotechnical Report by EI Australia dated 10/12/2018	(reference E23665.G03_Rev2).

Details of compliance must be provided to the PCA prior to and during excavation works. The developer must demonstrate the following:

(a) Protect and support the adjoining premises from possible damage from the excavation, and

(b) Where necessary, underpin the adjoining premises to prevent any such damage.

(c) Retaining walls or other approved methods necessary to prevent the movement of excavated or filled ground, together with associated subsoil drainage and surface stormwater drainage measures, shall be designed strictly in accordance with the manufacturers details or by a practising structural engineer.

Hoardings

- If the work is likely to cause pedestrian or vehicular traffic in a public area to be obstructed or rendered inconvenient; or if craning of materials is to occur across a public area or road reserve area a construction hoarding must be erected to prevent any substance from, or in connection with the construction site, falling onto a public area:

Such hoarding or barrier must be designed and erected in accordance with Council’s guidelines on hoarding construction. Relevant application under the Roads Act approval must be completed and fees paid prior to the construction of a hoarding on Council road reserve area.

- Lifting or craning materials over a public footway or roadway is not permitted unless a “B” class construction hoarding has been installed in compliance with Work Cover authority requirements.

Security Fence

- A temporary security fence to WorkCover Authority requirements is to be provided to the property during the course of construction.

Note: Fencing is not to be located on Council’s reserve area.

Hours of Construction Work

- Construction work/civil work/demolition work, including the delivery of materials, is only permitted on the site between the hours of 7:00am to 6:00pm Monday to Friday and 8:00am to 1:00pm on Saturday. No work will be permitted on Sundays or Public Holidays, unless otherwise approved by Council.

Waste Management

- The Waste Management Plan submitted to and approved by Council must be adhered to at all times throughout all stages of the development. Supporting documentation (receipts/dockets) of waste/recycling/disposal methods carried out, is to be kept and must be produced upon the request of Council or any other authorised officer.

Note: Any non-compliance with this requirement will result in penalties being issued.

- All disturbed areas shall be progressively stabilised and/or revegetated so that no areas remain exposed to potential erosion damage for a period of greater than 14 days.

- Sediment and erosion control measures are to be adequately maintained during the works until the establishment of grass.

- Vehicular access to the site shall be controlled through the installation of wash down bays or shaker ramps to prevent tracking of sediment or dirt onto adjoining roadways. Where any sediment is deposited on adjoining roadways it shall be removed by means other than washing. All material is to be removed as soon as possible and the collected material is to be disposed of in a manner which will prevent its mobilisation.

- All topsoil, sand, aggregate, spoil or any other material shall be stored clear of any drainage line, easement, water body, stormwater drain, footpath, kerb or road surface and there shall be measures in place in accordance with the approved erosion and sediment control plan.

- All construction waste must be separated as it is generated and kept separate bays, builder’s site bins and/or skips.

- All demolition and construction waste must be either recycled, re-used or disposed of according to the approved Waste Management Plan. Any variation from the approved Waste Management Plan to the locations to which the materials will be taken must be advised in writing to Council or the PCA.

Construction Noise

- Construction noise shall not exceed the management levels defined within the Interim Construction Noise Guideline published by the NSW Department of Environment and Climate Change dated July 2009.

- Construction activities, including operation of vehicles, shall be conducted so as to avoid unreasonable noise or vibration and cause no interference to adjoining or nearby occupations. Special precautions must be taken to avoid nuisance in neighbouring residential areas, particularly from machinery, vehicles, warning sirens, public address systems and the like. In the event of a noise or vibration problem arising at the time, the person in charge of the premises shall when instructed by Council, cause to be carried out, an acoustic investigation by an appropriate acoustical consultant and submit the results to Council. If required by Council, the person in charge of the premises shall implement any or all of the recommendations of the consultant and any additional requirements of Council to Council’s satisfaction.

Complaints Register

- The Applicant must keep a legible record of all complaints received in an up-to date

Complaints Register. The Complaints Register must record, but not necessarily be limited to:

(a) the date and time, where relevant, of the complaint;

(b) the means by which the complaint was made (telephone, mail or email);

(c) any personal details of the complainant that were provided, or if no details were

provided, a note to that effect;

(d) the nature of the complaint;

(e) any action(s) taken by the Applicant in relation to the complaint, including any

follow-up contact with the complainant; and

(f) if no action was taken by the Applicant in relation to the complaint, the reason(s)

why no action was taken.

(g) allocate an individual “complaint number” to each complaint received.

Car Parking Areas

- Car parking spaces and driveways must be constructed of a minimum of two coat finish seal or better. The spaces must be clear of obstructions and columns, permanently line marked and provided with adequate manoeuvring facilities. The design of these spaces must comply with Council’s DCP 2008, and Australian Standard 2890.1 Parking Facilities – Off Street Car Parking.

- The applicant is to provide a sign posting and line marking plan for the Bathurst Street, Buttman Lane and Huckstepp Service way at the property frontages.

- All car parking areas to be appropriately line marked and sign posted in accordance with the approved plans. All resident/visitor parking areas are to be clearly signposted limiting car parking for resident/visitor only. The applicant is to cover the costs of installation and maintenance of the signage.

- Directional signage indicating the location of customer parking, “in” and “out” crossings and directional arrows are to be provided in accordance with the approved plans.

Traffic Management

- If a works zone is required, an application must be made to Council’s Transport Planning section. The application is to indicate the exact location required and the applicable fee is to be included. If parking restrictions are in place, an application to have the restrictions moved, will need to be made.

- Notice must be given to Council’s Transport Planning section of any interruption to pedestrian or vehicular traffic within the road reserve, caused by the construction of this development. A Traffic Control Plan, prepared by an accredited practitioner must be submitted for approval, 48 hours to prior to implementation. This includes temporary closures for delivery of materials, concrete pours etc.

- Applications must be made to Council’s Transport Planning section for any road closures. The applicant is to include a Traffic Control Plan, prepared by a suitably qualified person, which is to include the date and times of closures and any other relevant information

General Site Works

- Building operations such as brick cutting, mixing mortar and the washing of tools, paint brushes, form­work, concrete trucks and the like shall not be performed on the public footway or any other locations which may lead to water pollution.

- The loading and unloading of all vehicles associated with the development must be

undertaken within the property boundary of the premises subject to this consent.

Measures must be implemented to prevent tracking of sediment by vehicles onto road.

Vehicle loads must be covered when entering and exiting the site with material.

- Vehicular access to the site shall be controlled through the installation of wash down bays or shaker ramps to prevent tracking of sediment or dirt onto adjoining roadways. Where any sediment is deposited on adjoining roadways is shall be removed by means other than washing. All material is to be removed as soon as possible and the collected material is to be disposed of in a manner which will prevent its mobilisation.

- Dust screens shall be erected and maintained in good repair around the perimeter of the subject land during land clearing, demolition, and construction works.

- Erosion and sediment control measures shall remain in place and be maintained until all disturbed areas have been rehabilitated and stabilised.

- All vehicles involved in the delivery, demolition or construction process departing from the property shall have their loads fully covered before entering the public roadway.

- The developer is to maintain all adjoining public roads to the site in a clean and tidy state, free of excavated “spoil” material.

- All dangerous and/or hazardous material shall be removed by a suitably qualified and experienced contractor licensed by SafeWork NSW. The removal of such material shall be carried out in accordance with the requirements of SafeWork NSW and the material shall be transported and disposed of in accordance with NSW Environment Protection Authority requirements.

- The development, including construction, shall not result in any increase in sediment deposition into any water body, wetland, bushland or environmentally significant land.

- Where operations involve excavation, filling or grading of land, or removal of vegetation, including ground cover, dust is to be suppressed by regular watering until such time as the soil is stabilised to prevent airborne dust transport. Where wind velocity exceeds five knots the PCA may direct that such work is not to proceed.

Ventilation

- The premises shall be ventilated in accordance with the requirements of the BCA (if using deemed to satisfy provisions: AS 1668, Parts 1 & 2).

- To ensure that adequate provision is made for ventilation of the building, the design, construction, installation and commissioning of the mechanical ventilation systems(s) shall be carried out in accordance with Australian Standard 1668 Parts 1 & 2.

The mechanical exhaust discharge point shall be designed and installed by an appropriately qualified person and shall be positioned to comply with AS 1668 Part 2, Section 3.7.

Vegetation

- All topsoil, sand, aggregate, spoil or any other material shall be stored clear of any drainage line, easement, water body, stormwater drain, footpath, kerb or road surface and there shall be measures in place in accordance with the approved erosion and sediment control plan.

- Any runoff entering the areas of vegetation to be retained shall be of an equivalent or better quality, and of a similar rate of flow to present levels.

- Within the area of land indicated on the approved plans for the development as fauna habitat, all existing native vegetation and habitat features shall be retained as habitat for native fauna.

- Cleared (weed free) native vegetation (timber, small branches and leaf litter) shall be reserved and stockpiled for re-use in rehabilitation works, such as mulching.

- Mulch generated from exotic trees or other weed species cleared shall not be used on site. It shall be removed from the site and disposed of appropriately and in accordance with legislative requirements.

- No known environmental or noxious weeds or known invasive plant species shall be included in the landscaping/revegetation.

- Any imported soil and/or mulch shall be free of contaminants, seed and propagules of weeds and undesirable species. Mulch shall not be used on flood liable land.

External

- Switchboards for utilities shall not be attached to the elevations of the development that are visible from the street.

- Any external lighting is to incorporate full cut-off shielding and is to be mounted so as to not cause any glare or spill over light nuisance within the development, neighbouring properties or road users.

- Illumination of the site is to be arranged in accordance with the requirements of Australian Standard 4282 1997 so as not to impact upon the amenity of the occupants of adjoining and nearby premises.

Contamination and Importation of Fill Material

- The development, including all civil works and demolition, must comply with the requirements of the Contaminated Land Management Act, 1997, State Environmental Planning Policy No. 55 – Remediation of Land, and Managing Land Contamination – Planning Guidelines (Planning NSW/EPA 1998).

Imported Fill Material

- Filling material must be limited to the following:

1. Virgin excavated natural material (VENM).

2.  Excavated natural material (ENM) certified as such in accordance with Protection o

the Environment Operations (Waste) Regulation 2014; and/or

- Material subject to a Waste Exemption under Clause 91 and 92 Protection of the Environment Operations (Waste) Regulation 2014 and recognised by the NSW Environment Protection Authority as being “fit for purpose” with respect to the development subject of this application.

Certificates proving that the material imported is ENM or VENM must be provided to the

Principal Certifying Authority prior to filling. Certificates are to be provided to Council officers if and when requested.

Fill imported on to the site must be compatible with the existing soil characteristic for site   drainage purposes.

Unidentified Contamination

- Any new information which comes to light during remediation, demolition or construction works which has the potential to alter previous conclusions about site contamination and remediation must be immediately notified to Council and the Principal Certifying Authority in writing.

A Section 4.55 Application under the EP&A Act shall be made for any proposed works outside the scope of the approved development consent.

Crime Prevention Through Environmental Design

- The following Crime Prevention Through Environmental Design principles are to be incorporated into the building.

(a) Back to base alarm systems shall be installed;

(b) Basement parking areas shall be painted a light colour;

(c) CCTV for the ground level, entry/exit points, car parks, lifts and the exterior of the building shall be implemented;

(d) ‘Way finding’ signage should be utilised at all major interchanges such as lifts and stair wells;

(e) Lighting is required to be designed in accordance with the Australian and New

Zealand Lighting Standard AS 1158. A lighting maintenance policy should be established. Security lighting should be installed in and around the building, and such shall not impact on any adjoining premises. The lighting should be vandal resistant, especially external lighting; and

(f) Access to the parking levels of the building shall be controlled via a security controlled device.

Waste Management Plan

- The Construction Waste Management Plan submitted to and approved by Council must be adhered to at all times throughout all stages of the development. Supporting documentation (receipts/dockets) of waste/recycling/disposal methods carried out, is to be kept and must be produced upon the request of Council or any other authorised officer.

Note: Any non-compliance with this requirement will result in penalties being issued.

Drainage Connection

- Prior to the connection of private drainage to Council’s drainage system, an inspection is to be carried out by Liverpool City Council’s Development Engineering Unit. A fee will be charged in accordance with Council’s adopted Fees and Charges, and is to be paid prior to the inspection.

Major Filling/ Earthworks

- All earthworks shall be undertaken in accordance with AS 3798 and Liverpool City Council’s Design Guidelines and Construction Specification for Civil Works.

The level of testing shall be determined by the Geotechnical Testing Authority/ Superintendent in consultation with the Principal Certifying Authority.

Archaeological discovery during excavation

- Should any relics be unexpectedly discovered on the site during excavation, all excavation or disturbance to the area is to stop immediately and the Heritage Council of NSW should be informed in accordance with section 146 of the Heritage Act 1977.

- Should any archaeological remains be discovered, a copy of recording of the finds and the final archaeological summary report is to be submitted to Council prior to the Occupational Certificate. If the discovery is on Council’s land, Council must be informed.

Erosion Control – Measures

- Erosion and sediment control measures shall remain in place and be maintained until all disturbed areas have been rehabilitated and stabilised.

E. 	PRIOR TO ISSUE OF OCCUPATION CERTIFICATE

The following conditions shall be complied with prior to issue of either an Interim or Final Occupation Certificate by the Principal Certifying Authority:

Certificates

- An Occupation Certificate must be obtained from the Principal Certifying Authority prior to any occupation of the building work encompassed in this development consent (including alterations and additions to existing buildings), in accordance with the relevant provisions of the Environmental Planning & Assessment Act 1979.

- A single and complete Fire Safety Certificate, certifying the installation and operation of all of the fire safety measures within the building must be submitted to Council with the Occupation Certificate.

- Details of critical stage inspections carried out by the principal certifying authority together with any other certification relied upon must be provided to Council with the occupation certificate.

Landscaping

- Upon completion of the approved landscape works associated with the development and prior to the issue of any OC, an Implementation Report is to be submitted to the PCA attesting to the satisfactory completion of the landscape works in accordance with the approved landscape plan. The report is to be prepared by a suitably qualified person.

Design Verification Statement

- In accordance with the Environmental Planning and Assessment Regulation 2000 and State Environmental Planning Policy (SEPP) 65 “Design Quality of Residential Apartment Development”, the subject development must be undertaken or directed by a ‘qualified designer’ (i.e., a registered architect under the Architects Act). In this regard, a design verification statement shall be submitted to the PCA assessing the development, upon completion of all works subject of this consent and its accompanying CC. The PCA shall ensure that the statement prepared by the qualified designer provides the following:

(a) A valid and current architect’s registration number (as issued by the NSW Architects Registration Board);

(b) That the completed development achieves the design quality of the development as shown in the plans and specifications submitted and approved with the CC, having regard to the design principles set out in Part 2 of SEPP 65.

Road Works and Road Reserve Works

- All traffic related works including intersection improvements works, line-marking and signposting are to be completed to the satisfaction of Council.

Recommendation of Acoustic Report

- Upon completion of works and prior to the issue of an Interim or Final Occupation Certificate, written certification prepared by a suitably qualified acoustic consultant shall be submitted to and approved by the Principal Certifying Authority (PCA).

The written certification prepared by the suitably qualified acoustic consultant shall confirm that the development complies with all requirements and recommendations

detailed within the approved acoustic report titled (Report Title), report reference (Project Number/Job Number, Revision) prepared by (Author) dated (date).

The acoustic consultant shall confirm that the development or use is capable of

operating in accordance with the design criteria.

Note: ‘Suitably qualified acoustic consultant’ means a consultant who possesses the qualifications to render them eligible for membership of the Australian Acoustical Society or employed by an Association of Australasian Acoustical Consultants

(AAAC) member firm.

Regulated Systems

- Regulated systems installed, such as air­handling systems, heated water systems and cooling water systems, must comply with the Public Health Act and Public Health

(Microbial Control) Regulation thereunder, including AS3666.1:2011 & AS1668.

The PCA is to ensure a Liverpool City Council approved registration form is completed and submitted to the Council with any relevant fee for the system.

Service Providers

- A Section 73 Compliance Certificate under the Sydney Water Act 1994 must be submitted to the PCA.

- Notification of arrangement for the development from Endeavour Energy shall be submitted to Council.

- Prior to the issue of an occupation certificate, written certification from all relevant service providers that the telecommunications infrastructure is installed in accordance with:

(a) The requirements of the Telecommunications Act 1997;

(b) For a fibre ready facility, the NBN Co’s standard specifications current at the time of installation; and

(c) For a line that is to connect a lot to telecommunications infrastructure external to the premises, the line shall be located underground.

Unless otherwise stipulated by telecommunications legislation at the time of construction, the development must be provided with all necessary pits and pipes, and conduits to accommodate the future connection of optic fibre technology telecommunications.

BASIX

- Supporting documentation issued by a suitable qualified person who has installed or carried out the works associated with the BASIX commitments shall be submitted to the PCA.

Liverpool City Council Clearance – Roads Act / Local Government Act

- Prior to the issue of an Occupation Certificate, the Principal Certifying Authority shall ensure that all works associated with a S138 Roads Act approval or S68 Local Government Act approval have been inspected and signed off by Liverpool City Council.

Works as Executed – General

- Prior to the issue of an Occupation Certificate, works-as-executed drawings and compliance documentation for the subject stage, shall be submitted to the Principal Certifying Authority in accordance with Liverpool City Council’s Design Guidelines and Construction Specification for Civil Works.

An original set of works-as-executed drawings and copies of compliance documentation shall also be submitted to Liverpool City Council with notification of the issue of the Occupation Certificate where Council is not the Principal Certifying Authority.

Compliance Documentation

- Prior to the issue of an Occupation Certificate the following compliance documentation shall be submitted to the Principal Certifying Authority. A copy of the following documentation shall be provided to Council where Council is not the Principal Certifying Authority:

(a) Work as Executed (WAE) drawings of all civil works. The WAE drawings shall be marked in red on copies of the stamped Construction Certificate drawings signed, certified and dated by a registered surveyor or the design engineer. The Work as

Executed drawings shall be prepared in accordance with Council’s Design

Guidelines. Electronic copies of the WAE shall be provided in PDF format and a DXF format to Council along with two hard copies of the WAE plans.

(b) Surveyor’s Certificate certifying that all pipes and services are located wholly within the property or within appropriate easements and that no services encroach boundaries.

Garbage Services

- Prior to the issue of an Occupation Certificate, if onsite waste and recycling collection is required the following restriction as to user must be registered on the title of the property: 

The owner/applicant of any property requiring on site collection of waste and recycling is to enter into a legally binding indemnity with Council to control the access and operation of the waste and recycling service. This shall be inclusive on successive owners in perpetuity. All costs involved are to be borne by the applicant.

- Prior to the issue of an OC, if onsite waste and recycling collection is required the following restriction as to user must be registered on the title of the property:

(a) “The owner/applicant of any property requiring on site collection of waste and recycling is to enter into a legally binding indemnity with Council to control the access and operation of the waste and recycling service. This shall be inclusive on successive owners in perpetuity. All costs involved are to be borne by the applicant.”

The restriction as to user may not be extinguished or altered except with the consent of Liverpool Council.

- The developer/owner of the site is to contact Liverpool Council- Waste Management Section to determine the required number of waste and recycle bins for the residential component of the development as well as servicing requirements. These waste/garbage bins are to be kept at all times within the residential waste/garbage compartment rooms except before and after collection days. Waste/garbage bins are to be returned to the compartment room as soon as practical after waste has been collected.

- Since the approved waste management plan states that any green (garden) waste will be collected and disposed of by the property maintenance contractor engaged by the building’s proprietors, the following restriction as to user shall be placed on the title of the property at the applicant’s expense, and this restriction cannot be altered or removed without Council’s consent:

‘Liverpool City Council will not supply green (garden) waste removal services to this property, nor any waste bins associated with the removal of garden waste.’

The restriction as to user may not be extinguished or altered except with the consent

of Liverpool Council.

- All signage relating to the use and operation of the waste facilities and equipment will be installed prior to the occupation certificate being issued.

Restrictions on Title

- Prior to the issue of an OC, the following restriction as to user must be registered on the title of the property:

“A static compaction unit for waste management shall be provided and maintained at all times within the waste storage area”

The restriction as to user may not be extinguished or altered except with the consent of Liverpool City Council.

- Prior to the issue of an OC, the following restriction as to user must be registered on the title of the property:

“The hanging of washing, including any clothing, towels, bedding or other article of a similar type of any balcony is not to be visible from any street”

The restriction as to user may not be extinguished or altered except with the consent of

Liverpool City Council.

Rectification of Damage

- Prior to the issue of an Occupation Certificate any damage to Council infrastructure not identified in the dilapidation report, as a result of the development shall be rectified at no cost to Liverpool City Council.

Any rectification works within Bathurst Street and Huckstepp Serviceway will require a Roads Act application. The application is to be submitted and approved by Liverpool City Council prior to such works commencing.

Dilapidation Report

- Any rectification works required by Council regarding the condition of Council infrastructure shall be undertaken, at full cost to the developer.

Stormwater Compliance

- Prior to the issue of an Occupation Certificate, the Principal Certifying Authority shall ensure that the:

- Stormwater pre-treatment system/s,

- Basement Carpark pump-out system:

- Have been satisfactorily completed in accordance with the approved Construction Certificate and the requirements of this consent,

- Have met the design intent with regard to any construction variations to the approved design, and

- Any remedial works required to been undertaken have been satisfactorily completed.

Details of the approved and constructed system/s shall be provided as part of the Works-As-Executed drawings.

Restriction as to User and Positive Covenant

- Prior to the issue of an Occupation Certificate a restriction as to user and positive covenant relating to the following shall be registered on the title of the property:

- Stormwater pre-treatment system/s,

- Basement Carpark pump-out system:
The restriction as to user and positive covenant shall be in Liverpool City Council’s standard wording as detailed in Liverpool City Council’s Design and Construction Guidelines and Construction Specification for Civil Works.

Laneway

140. The section of the proposed laneway adjoining the northern portion of the development site is to dedicated to Council.

F. 	GENERAL CONDITIONS

The following general conditions shall be complied with at all times:

Council’s Infrastructure

- Council’s on-street assets should be protected at all times. Any damages should be rectified to Council’s satisfaction.

- All vehicles enter and exit the site in forward direction at all times.

Parking Requirements

- Parking spaces shall be allocated in accordance with the Architectural Plans and Traffic Impact Assessment as follows:

(a) A minimum of 63 parking spaces for residents including 7 accessible spaces;

(b) A minimum of 10 parking spaces for visitors;

(c) A minimum of 17 parking spaces for the ground floor commercial premises including 1 accessible space

(d) A minimum of 4 parking spaces for motorcycles; and

(e) A minimum of 44 parking spaces for bicycles are to be provided

(f) A Minimum of 1 car wash bay.

- All parking areas shown on the approved plans must be used solely for this purpose.

- The operator of the development must not permit the reversing of vehicles onto or away from the road reserve. All vehicles must be driven forward onto and away from the development and adequate space must be provided and maintained on the land to permit all vehicles to turn in accordance with AS 2890.1 Parking Facilities – Off Street Car Parking.

- All line marking and sign posting is to be maintained in good condition at all times, to the satisfaction of Council.

Noise and Environmental Emissions

- The intruder alarm/s associated with the development shall only be permitted to operate in accordance with the requirements of Clause 53 of the Protection of the Environment Operations (Noise Control) Regulation 2000 under the POEO Act.

- The use of the dwellings including music, mechanical plant and equipment and the like shall not give rise to the emission of "offensive noise" as defined under the Protection of the Environment Act 1997.

- The use of the site shall not give rise to the emission into the surrounding environment of gases, vapours, dusts, odours or other impurities which are a nuisance, injurious or prejudicial to health.

Noise – General

- Noise associated with the use of the premises, including mechanical plant and equipment, shall not give rise to any one or more of the following:

- The use of the premises including the use of the loading dock and the cumulative operation of any mechanical plant shall not give rise to the emission of ‘offensive noise’ as defined by the Protection of the Environment Operations Act 1997.

- The operation of any mechanical plant, equipment and/or use of the loading dock on the premises shall not cause:

  - The emission of noise as measured over a 15 minute period (LAeq (15 minute)) that exceeds the LA90 (15 minute) background noise level by more than 5 dB(A) when measured at the most affected residential boundary. Modifying factor corrections must be applied for tonal, impulsive, low frequency or intermittent noise in accordance with the NSW Environment Protection Authority’s ‘Noise Policy for Industry’ (2017);

  - An internal noise level in any adjoining occupancy that exceeds the recommended design sound levels specified in Australian/New Zealand Standard AS/NZS 2107:2016 Acoustics – Recommended design sound levels and reverberation times for building interiors;

  - ‘offensive noise’ as defined by the Protection of the Environment Operations Act 1997; and

  - Transmission of vibration to any place of different occupancy greater than specified in AS 2670.

Should an offensive noise complaint be received and verified by Council an acoustic assessment is to be undertaken (by a suitably qualified acoustic consultant), and an acoustic report is to be submitted to Liverpool City Council for review. Any noise attenuation recommendations approved by Council must be implemented.

Waste Management

- Waste bins must be stored in the designated garbage refuse area, which must be kept clean and tidy at all times. Bins must not be stored or allowed to overflow into parking, driveway or landscaped areas, must not obstruct the exit of the building, must not leave the site onto neighbouring public or private properties and must be adequately secured.

- Waste must be adequately secured and contained within designated waste areas and must not leave the site onto neighbouring public or private properties.

- All waste products associated with the use of the development are to be placed in containers and stored within the building.

- All solid waste stored on site is to be covered at all times. Furthermore, all solid and liquid waste is to be removed from the site by a registered waste contractor.

- All waste materials generated as a result of the development are to be disposed at a facility licensed to receive such waste.

- A separate storage area must be allocated for the holding of bulk waste prior to collection.

- Waste bins must be stored in designated garbage/ trade refuse areas, which must be kept tidy at all times. Bins must not be stored or allowed to overflow in parking or landscaping areas, must not obstruct the exit of the building, and must not leave the site onto neighbouring public or private properties. 

Operation, maintenance and cleaning of the garbage compactor and associated equipment is the responsibility of the strata management or body corporate. Maximum compaction ratio is 2:1.

Operation, maintenance and cleaning of the garbage chutes and associated waste cupboards, rooms, or equipment is the responsibility of the strata management or body corporate. 

Signage is to be prominently displayed in each waste storage area, or waste service room, as appropriate, indicating that:

- Garbage is to be placed wholly within the garbage bins provided,

- Only recyclable materials accepted by Council are to be placed within the recycling bins,

- The area it to be kept tidy,

- A phone number for arranging disposal of bulky items, and

- Graphic illustrative content to be 50%.

- All garbage rooms must be:

(a) Provided with mechanical ventilation;

(b) Provided with a hose cock for hosing the garbage bin bay and a sewered drainage point in or adjacent to the bin storage area. The drainage point should have a fine grade drain cover sufficient to prevent coarse pollutants from entering the sewer. If the hose cock is located inside the bin storage bay, it is not to protrude into the space indicated for the placement of bins;

(c) Provided with sufficient light to permit usage at night;

(d) Allocated with sufficient space within the bin bay to allow for access to all required bins by residents and waste collectors, as well as manoeuvring of bins within the bay and for the removal and return of bins by the waste collector;

(e) Provided with signage to be prominently displayed in each bin bay, or waste service room, as appropriate indicating that:

- Only recyclable materials accepted by Council are to be placed within the recycling bins;

- A phone number for arranging disposal of bulky items;

(f) Should garbage chutes be incorporated into the design of the building, signage on how to use the chutes is to located prominently next to the chute;

(g) Operation, maintenance and cleaning of the garbage compactor and associated equipment is the responsibility of the strata management or body corporate and not of Council;

(h) Maximum compaction ratio for waste shall be 2:1; and

(i) Operation, maintenance and cleaning of the garbage chutes and associated waste cupboards, rooms, or equipment is the responsibility of the strata management or body corporate, and not of Council.

- The waste and recycle bins are to be kept at all times within the residential waste storage rooms except before and after collection days. Waste and Recycle bins are to be returned to the storage rooms within 24 hours of collection.

- After the issue of an occupation certificate but prior to residents commencing moving in, the Council must be contacted to deliver the required waste bins for the property.

- Bins are to be presented to the designated waste collection point by agents of the building management, bins will be emptied by Council’s waste contractors and the bins will be replaced promptly in the waste storage area.

- The temporary bin storage area adjacent to the building’s loading dock must be used and kept secured to ensure that any bins kept in that area remain free of contamination either from the building’s commercial tenants or from the residential occupants.

- Waste collection services and deliveries are restricted to between the hours of 7 am and 10 pm Monday to Saturday and 8am and 10pm Sunday.

Landscaping

- Landscaping shall be maintained in accordance with the approved plan, in a healthy state and in perpetuity by the existing or future owners and occupiers of the development. If any of the vegetation comprising the landscaping dies or is removed, it is to be replaced with vegetation of the same species, and similar maturity as the vegetation which has died or was removed.

An annual report shall be submitted to Council, for the 3 years following issue of the OC, certifying that the landscaping works have been satisfactorily maintained.

Graffiti

- Any graffiti carried out on the property shall be removed, within 48 hours, at full cost to the owner/occupier of the site.

Lighting

- Illumination of the site is to be arranged in accordance with the requirements of Australian Standard 4282 1997 so as not to impact upon the amenity of the occupants of adjoining and nearby premises.

Use of the Premises - General

- The portion of the site not the subject of this approval shall be maintained in a clean and tidy state at all times.

- The operation of the premises shall be conducted so as to avoid unreasonable noise and cause no interference to adjoining or nearby residences.

- The use of the premises shall not give rise to “offensive noise” as defined under the Protection of the Environment Operations Act 1997.

- The use of the premises is not to interfere with the amenity of the residential area.

- A permanent record of receipts for the removal of both liquid and solid waste from the site shall be kept and maintained up to date at all times. Such record shall be made available to the Consent Authority and Liverpool City Council’s Officers, upon request.

- Waste and recyclable material generated from the operations of the development shall be managed in a satisfactory manner that does not give rise to offensive odour or encourage pest activity. All waste material shall be regularly removed from the premises. Waste shall not be permitted to accumulate near the waste storage bins.

Separate Application for Fit out and Use

- This application does not approve the use or fit-out of the retail/commercial premises for food and drink premises.

A separate application is to be submitted and approved for the fit out and use.

G.	ADVISORY

- Section 8.2, 8.3, 8.4 & 8.5 allows Council to reconsider your proposal. Should you wish to have the matter reconsidered you should make an application under that section with the appropriate fee.

- Under Section 8.7 & 8.10 of the Act applicants who are dissatisfied with the outcome of a consent authority have a right of appeal to the Land and Environment Court. This right must be exercised within six (6) months from the date of this notice. The Court's Office is situated at Level 1, 225 Macquarie Street, Sydney (Telephone 9228 8388), and the appropriate form of appeal is available from the Clerk of your Local Court.

- In accordance with Section 4.53 of the Environmental Planning and Assessment Act 1979, unless otherwise stated by a condition of this consent, this consent will lapse unless the development is commenced within five years of the date of this notice.

- To confirm the date upon which this consent becomes effective, refer to Section 4.20 of the Environmental Planning and Assessment Act, 1979. Generally, the consent becomes effective from the determination date shown on the front of this notice. However, if unsure applicants should rely on their own enquiries.

- To confirm the likelihood of consent lapsing, refer to Section 4.53 of the Act. Generally, consent lapses if the development is not commenced within five years of the date of approval. However, if a lesser period is stated in the conditions of consent, the lesser period applies. If unsure applicants should rely on their own enquiries.

- In accordance with Section 8.8 and 8.10 of the Environmental Planning and Assessment Act 1979, an objector who is dissatisfied with the determination of a consent authority to grant consent to a development application for designated development (including designated development that is integrated development), may, within 28 days after the date on which the application is taken to have been determined, appeal to the Land and Environment Court.

- The approval of this application does not imply or infer compliance with the Disability Discrimination Act and that the developer should investigate their liability under the Act.

- The requirements of all authorities including the Environmental Protection Authority and the Work Cover Authority shall be met in regards to the operation of the building.

- “DIAL BEFORE YOU DIG”

Underground assets may exist in the area that is subject to your application. In the interest of health and safety and in order to protect damage to third party assets please contact Dial before you dig at www.1100.com.au or telephone 1100 before excavating or erecting structures (This is the law in NSW). If alterations are required to the configuration, size, form or design of the development upon contact the Dial before You Dig service, an amendment to the development consent (or a new development application) may be necessary. Individuals owe asset owners a duty of care that must be observed when working in the vicinity of plant or assets. It is the individual’s responsibility to anticipate and request the nominal location of plant or assets on the relevant property via contacting the Dial before you dig service in advance of any construction or planning activities.

- TELECOMMUNICATIONS ACT 1997 (COMMONWEALTH)

Telstra (and its authorised contractors) are the only companies that are permitted to conduct works on Telstra’s network and assets. Any person interfering with a facility or installation owned by Telstra is committing an offence under the Criminal Code Act 1995 (Cth) and is liable for prosecution. Furthermore, damage to Telstra’s infrastructure may result in interruption to the provision of essential services and significant costs. If you are aware of any works or proposed works which may affect or impact on Telstra’s assets in any way, you are required to contact: Telstra’s Network Integrity Team on Phone Number 1800 810 443.

- The Liverpool City Council Local Government area soils and ground water may be subject to varying levels of Salinity. Whilst Council may require applicants to obtain Salinity reports relating to some developments, no assessment may be made by Council in that regard. Soil and ground water salinity levels can change over time due to varying factors. It is recommended that all applicants make their own independent inquiries as to appropriate protection against the current and future potential affect of Salinity to ensure the ongoing structural integrity of any work undertaken.  Liverpool City Council will not accept any liability for damage occurring to any construction of any type affected by soil and or ground water Salinity.

- Care shall be taken by the applicant and the applicant’s agents to prevent any damage to adjoining properties.  The applicant or applicant’s agents may be liable to pay compensation to any adjoining owner if, due to construction works, damage is caused to such an adjoining property.

- Letter boxes must be provided in accordance with the requirements of Australia Post.  In this regard, the developer is required to obtain approval from Australia Post for address numbering, and letter box positioning and dimensions.

- The cost of any necessary adjustments to utility mains and services shall be borne by the applicant.

ATTACHMENT 2

CONTRIBUTIONS PURSUANT TO SECTION 7.12 OF THE ENVIRONMENTAL

PLANNING & ASSESSMENT ACT, 1979

Liverpool Contribution Plan 2018 – Liverpool City Centre

Note to the applicant:	When remitting payment as specified in the Conditions of Consent to the approval, this Form must be submitted with your payment.

These figures have been calculated to the current CPI quarter and will be adjusted at the time of payment in accordance with the conditions of consent.

APPLICATION NO: 	DA-767/2016

## APPLICANT:			Stirling 203 Northumberland

PROPERTY:			64 Bathurst Street and 203-209 Northurmberland Street Liverpool

PROPOSAL: 	Construction of a twenty-one (21) storey mixed-use development with five (5) levels of basement for parking, storage and plant. Four (4) retail tenancies on the ground floor facing Bathurst Street, four (4) storey podium comprising two (2) commercial tenancies with a courtyard, a mezzaninge and communal open space for residents and a seventeen (17) storey tower with 66 residential units.

| Facilities | Amount ($) | Job No. |
| --- | --- | --- |
| Georges River Foreshore | $154,554 | GL.10000001869.10105 |
| Pioneer Park | $22,079 | GL.10000001869.10105 |
| Apex Reserve | $11,040 | GL.10000001869.10217 |
| Georges River Pedestrian Crossing | $27,599 | GL.10000001869.10218 |
| Discovery Park | $27,599 | GL.10000001869.10219 |
| Community Facility Upgrade | $66,238 | GL.10000001870.10099 |
| Car parking | $220,792 | GL.10000001868.10108 |
| Access, bike facilities and bus priority | $165,594 | GL.10000001865.10220 |
| Peripheral Streetscape works | $110,396 | GL.10000001865.10221 |
| Footpath widening in City Centre | $22,079 | GL.10000001865.10222 |
| TOTAL | $827,970 |  |

---------------------------------------------- OFFICE USE ONLY -------------------------------------------

RECORD OF PAYMENT

# Total Amount paid: ______________________________

# Date:________________________

Receipt No.:_____________________________Cashier:_____________________