Attachment A – Draft Conditions of consent regarding DA/2076/2018 at

121 Portland Drive, Cameron Park

(Approved subject to the conditions specified in this notice and in accordance with

the stamped approved plans.)

Reason for the Imposition of Conditions

The reason for the imposition of the following conditions shall ensure, to Council’s satisfaction, the objects of the Environmental Planning and Assessment Act 1979 (as amended) are achieved:

To encourage:

- The proper management, development and conservation of natural and artificial resources, including agricultural land, natural areas, forest, minerals, water, cities, towns, and villages for the purpose of promoting the social and economic welfare of the community and a better environment;

- The promotion and co-ordination of the orderly and economic use of development of land;

- The protection, provision, and co-ordination of communication and utility services;

- The provision of land for public purposes;

- The provision and co-ordination of community services and facilities;

- The protection of the environment, including the protection and conservation of native animals and plants including threatened species, populations, and ecological communities and their habitats;

- Ecologically Sustainable Development; and

- The provision and maintenance of affordable housing.

- To promote the sharing of the responsibility for environmental planning between the different levels of government in the State.

- To provide increased opportunity for public involvement and participation in environmental planning and assessment.

Administrative Conditions

- Prescribed Conditions

- The work shall be carried out in accordance with the requirements of the Building Code of Australia.

- In the case of residential building work for which the Home Building Act 1989 requires there to be a contract of insurance in force in accordance with Part 6 of that Act, that such a contract of insurance is in force before any building work authorised to be carried out by the consent commences.

- A sign shall be erected in a prominent position on any site on which building work, subdivision work or demolition work is being carried out:

- showing the name, address and telephone number of the Certifying Authority for the work, and

- showing the name of the principal contractor (if any) for any building work and a telephone number on which that person may be contacted outside working hours, and

- stating that unauthorised entry to the work site is prohibited.

Any such sign shall be maintained while the building work, subdivision work or demolition work is being carried out, but shall be removed when the work has been completed.

- Residential building work within the meaning of the Home Building Act 1989 shall not be carried out unless the Certifying Authority for the development to which the work relates (not being the Council) has given the Council written notice of the following information:

- in the case of work for which a principal contractor is required to be appointed:

- the name and licence number of the principal contractor, and

- the name of the insurer by which the work is insured under Part 6 of that Act,

- in the case of work to be done by an owner-builder:

- the name of the owner-builder, and

- if the owner-builder is required to hold an owner-builder permit under that Act, the number of the owner-builder permit.

If arrangements for doing the residential building work are changed while the work is in progress so the information notified under (d) becomes out of date, further work must not be carried out unless the Certifying Authority for the development to which the work relates (not being the Council) has given the Council written notice of the updated information.

- Approved Documentation

The development consent incorporates this schedule of conditions and the plans and documents referenced and stamped as follows:

a)	Plans Reference:

| Architectural Plans prepared by: Integrated Design Group; Project Number: SAL18007 |  |  |  |
| --- | --- | --- | --- |
| Name of Plan | Drawing Number | Issue | Date |
| Site plan | DA0100 | K | 20 May 2019 |
| Car park plan | DA0101 | C | 20 May 2019 |
| Lower ground floor | DA1100 | K | 21 May 2019 |
| Upper ground floor | DA1101 | J | 21 May 2019 |
| First floor plan | DA1102 | I | 21 May 2019 |
| Service area elevation | DA9300 | A | 27 February 2019 |
| Roof plan | DA1103 | A | 20 November 2018 |
| Elevations 01 | DA2000 | D | 20 November 2018 |
| Elevations 02 | DA2001 | D | 20 November 2018 |
| Sections | DA3000 | E | 20 November 2018 |
| External finishes schedule | DA9600 | A | 20 November 2018 |

| Engineering plans prepared by: ADW Johnson; Project Number. 239840 |  |  |  |
| --- | --- | --- | --- |
| Name of Plan | Drawing Number | Issue | Date |
| Overall site plan | 002 | E | 1 November 2019 |
| Site arrangement plan | 101 | E | 1 November 2019 |
| Stormwater arrangement plan | 102 | E | 1 November 2019 |
| Stormwater longitudinal section and detail | 201 | E | 1 November 2019 |
| Accessway longitudinal section | 202 | E | 1 November 2019 |
| Typical cross sections and turning path: Sheet 1 | 211 | E | 1 November 2019 |
| Turning path sheets: Sheet 2 | 212 | E | 1 November 2019 |
| Turning path plans: Sheet 3 | 213 | E | 1 November 2019 |
| Cut and fill plan | 501 | E | 1 November 2019 |
| Erosion and sediment control | 801 | E | 1 November 2019 |
| Erosion and sediment control notes | 802 | E | 1 November 2019 |
| Erosion and sediment control details | 803 | E | 1 November 2019 |

| Landscaping Plans prepared by: Wallman Partners Pty Ltd |  |  |  |
| --- | --- | --- | --- |
| Name of Plan | Drawing Number | Issue | Date |
| Landscape concept plan | L1 of 3 | E | 17 May 2019 |
| Landscape finishes plan | L2 of 3 | D | 26 September 2019 |
| Landscape planting plan | L3 of 3 | D | 26 September 2019 |

b)	Document Reference:

| Document | Reference | Author | Date |
| --- | --- | --- | --- |
| Slope Stability Assessment | Project 91313.00. | Douglas Partners | 18 April 2018 |
| Waste Management Plan; as amended by additional information | - - | - ADW Johnson | - 27 September 2019 |
| Ecological Assessment Report | AEP Ref 1778 | Anderson Environment and Planning | November 2018 |
| Bushfire Threat Assessment | AEP Ref 1778 | Anderson Environment and Planning | November 2018 |
| Operational Plan of Management | - | The Salvation Army | November 2018 |
| Traffic and Parking Assessment | 18/093 | Intersect Traffic Pty Ltd | November 2018 |
| Noise Assessment | 181677 | Spectrum Acoustics | November 2018 |
| Access report | 218330 | Accessible Building Solutions | 21 February 2019 |
| CPTED report | - | James Marshall & Co | November 2018 |
| Visual Impact Statement | - | Integrated Design Group | - |

Details of the development shown in the approved plans and documents referenced are altered in the manner indicated by:

i.	Any amendments made by Council on the approved plans or documents;

ii.	Any notes, markings, or stamps on approved plans or documents, and

iii	Any conditions contained in this consent.

General Conditions

The person having the benefit of the consent shall comply with each of the following conditions.

- Disability Access Requirements

The recommendations contained in the approved Access report shall be incorporated into the design and construction of the development.

Upon completion of the works and prior to the issue of an Interim or Final Occupation Certificate, whichever occurs first, documentation shall be provided to the Principal Certifying Authority from a suitably qualified consultant certifying that the works have been completed in accordance with the recommendations of the report.

General Terms of Approval and Concurrence Requirements

- General Terms of Approval

The following approval bodies have given general terms of approval in relation to the development: (List Approval bodies).

- Subsidence Advisory NSW

- Subsidence Advisory NSW – General Terms of Approval

General

Plans, standards and guidelines.

- The development being undertaken strictly in accordance with the details set out on the application form, any information submitted with the application and the plans submitted, as amended or as modified by the conditions of this approval.

Note: 	Any proposal to modify the terms or conditions of this approval, whilst still maintaining substantially the same development to that approved, will require the submission of a formal application for consideration by Subsidence Advisory NSW.  If amendments to the design result in the development not remaining substantially the same as that approved by this approval, a new application must be submitted to Subsidence Advisory NSW.

- This approval expires 5 years after the date the approval was granted if construction work has not physically commenced.

Prior to Commencement of Construction

- Prescribed Design Parameters

The proposed structure(s) is to be designed to be “safe, serviceable and any damage from mine subsidence shall be limited to ‘slight’ in accordance with AS2870 (Damage Classification) and readily repairable” using the subsidence parameters outlined below:

a) Maximum Horizontal Strains (+/-): 2 mm/m

b) Maximum Tilt: 2 mm/m

c) Maximum Radius of Curvature: 5 km

4.	Submit an “Engineering Impact Statement” prior to commencement of detailed design for acceptance by Subsidence Advisory NSW, which shall identify the:

a. Mine subsidence parameters used for the design.

b. Main building elements and materials.

c. Risk of damage due to mine subsidence.

d. Design measures proposed to control the risks.

e. Comment on the:

- likely building damage in the event of mine subsidence.

- sensitivity of the design to greater levels of mine subsidence.

5. 	Submit a final design incorporating the design methodology contained in the “Engineering Impact Statement”, for acceptance by Subsidence Advisory NSW prior to commencement of construction. It shall include certification by a qualified structural engineer to the effect that the improvements will remain “safe, serviceable and any damage from mine subsidence shall be limited to ‘slight’ damage in accordance with AS2870 (Damage Classification), and readily repairable” taking into consideration the mine subsidence parameters outlined above.

Post Construction

6. 	Upon completion of construction, works-as-executed certification by a qualified engineer is to be forwarded to Subsidence Advisory NSW confirming that construction was in accordance with the plans approved by Subsidence Advisory NSW.

- Bushfire – Development on Bushfire Prone Land

The development shall conform to the specifications and requirements of the guide Planning for Bushfire Protection ISBN 0 9585987 8 9, produced by the NSW Rural Fire Service and the following conditions:

Asset Protection Zones

The intent of measures is to provide sufficient space and maintain reduced fuel loads so as to ensure radiant heat levels of buildings are below critical limits and to prevent direct flame contact with a building. To achieve this, the following conditions shall apply:

- At the commencement of building works and in perpetuity the entire property shall be managed as an inner protection area (IPA) as outlined within section 4.1.3 and Appendix 5 of 'Planning for Bush Fire Protection 2006' and the NSW Rural Fire Service's document 'Standards for asset protection zones'.

Water and Utilities

The intent of measures is to provide adequate services of water for the protection of buildings during and after the passage of a bush fire, and to locate gas and electricity so as not to contribute to the risk of fire to a building. To achieve this, the following conditions shall apply:

- Water, electricity and gas are to comply with sections 4.2.7 and 4.1.3 of 'Planning for Bush Fire Protection 2006'.

Access

The intent of measures for internal roads is to provide safe operational access for emergency services personnel in suppressing a bush fire, while residents are accessing or egressing an area. To achieve this, the following conditions shall apply:

- Internal Roads shall comply with section 4.2.7 of 'Planning for Bush Fire Protection 2006'.

Evacuation and Emergency Management

The intent of measures is to provide suitable emergency and evacuation (and relocation) arrangements for occupants of special fire protection purpose developments. To achieve this, the following conditions shall apply:

- A Bush Fire Emergency Management and Evacuation Plan shall be prepared consistent with 'Development Planning- A Guide to Developing a Bush Fire Emergency Management and Evacuation Plan December 2014'.

Design and Construction

The intent of measures is that buildings are designed and constructed to withstand the potential impacts of bush fire attack. To achieve this, the following conditions shall apply:

- New construction shall comply with Sections 3 and 5 (BAL 12.5) Australian Standard AS3959-2009 'Construction of buildings in bush fire-prone areas' or NASH Standard (1.7.14 updated) ‘National Standard Steel Framed Construction in Bushfire Areas – 2014’ as appropriate and section A3.7 Addendum Appendix 3 of 'Planning for Bush Fire Protection 2006'.

Landscaping

- Landscaping to the site is to comply with the principles of Appendix 5 of 'Planning for Bush Fire Protection 2006'.

Fees, Charges and Contributions

- Fixed Development Contributions

From the date this determination is made until payment, the amount of the contribution payable shall be indexed and adjusted at the close of business on:

- 14 August,

- 14 November,

- 14 February, and

- 14 May;

In each year in accordance with indexation provisions within the Section 7.12 Contributions Plan. The first date for indexation shall occur on the first abovementioned date after the Notice of Determination becomes effective.

The contribution payable shall be the amount last indexed and adjusted in accordance with the above. However, if no amount has been indexed and adjusted because the first date for indexation and adjustment has not arrived, the contributions payable shall be that contribution nominated below.

The contribution shall be paid to Council as follows:

- Development Applications involving subdivision – prior to the release of the Subdivision Certificate;

- Development Applications involving building work – prior to the release of the first Construction Certificate;

- Development Applications involving both subdivision and building work – prior to the release of the Subdivision Certificate or first Construction Certificate, whichever occurs first;

- Development Applications where no Construction Certificate or Subdivision Certificate is required – prior to the commencement of any construction work or prior to any occupation, whichever occurs first;

- Complying Development Certificates – prior to any work authorised by the application or certificate commencing.

It is the professional responsibility of the Certifying Authority to ensure the monetary contributions have been paid to Council in accordance with the above provisions.

Please note for payments made by cheque or electronic transfer – the release of any documentation shall be subject to the clearing of those funds.

Indexation details are available from Council’s Development Contributions Team.  A copy of the Section 7.12 Contributions Plan can be viewed on Council’s website, and at Council’s Administrative Building during Council’s normal business hours.

In accordance with the provisions of the Environmental Planning and Assessment Act 1979 – Section 7.12 and as at the date of this consent (and subject to indexation), the monetary contribution payable is:

TOTAL $142,880.00

Conditions to be satisfied prior to the issue of the Construction Certificate

The person having the benefit of the consent shall comply with each of the following conditions prior to the issue of a Construction Certificate.

Any documentation required to be submitted for the Construction Certificate shall be submitted to the Certifying Authority unless otherwise specified.

- Construction Certificate

Prior to the commencement of building work or subdivision work, a Construction Certificate shall be obtained.

Note:	If the Construction Certificate is issued by a Certifying Authority that is not Council it shall be necessary to lodge the Construction Certificate and other approved documents with Council within two days of such approval. (Clause 142(2) Environmental Protection Authority Regulation 2000).

- Erosion and Sediment Control

A final Erosion and Sediment Control Plan or Soil and Water Management Plan shall be submitted. The plan shall contain information required for the area of disturbance of the development or its distinct and separate stages in accordance with Development Control Plan 2014.

The final plan shall include a signed and dated Statement of Compliance stating (in full):

- This plan has been developed, certified and signed off by an appropriately qualified and experienced professional in erosion and sediment control;

- The plan complies with the requirements for the area of disturbance in accordance with Development Control Plan 2014;

- The plan and associated documents, calculations and drawings, have been prepared to a standard which, if properly implemented, shall achieve the water release criteria of 50mg/L of total suspended solids (TSS); and

- All erosion and sediment control measures are in accordance with Development Control Plan 2014.

- Stormwater Disposal and Harvesting

A Stormwater Management and Harvesting Plan shall be submitted. The plan shall be generally in accordance with the plans approved by this development consent prepared by ADW Johnson, Ref: 239840, Rev.E dated 01/11/19. The stormwater plans approved by this development consent are not approved for construction. The stormwater plans shall comply with the following:

- Stormwater shall be disposed of through a piped system designed in accordance with Australian Standard AS 3500 by a suitably qualified professional. The design shall be undertaken by a practising Civil Engineer deemed to be suitably experienced by Council and qualified so as to be accepted as a Member of the Institution of Engineers Australia or a Registered Surveyor deemed to be suitably experienced by Council shall be accepted as qualified to prepare plans for drainage works (excluding flood control structures and bridges). Qualifications demonstrating the above may be requested by Council.

- Stormwater harvesting measures shall be constructed and maintained in accordance with Development Control Plan 2014 Water Cycle Management Guideline. Stormwater drainage plans shall include details of the harvesting system, including rainwater tank details, pump details and reticulation diagrams.

- Vehicles Access Crossing and Kerb Layback

An approval for the vehicle access crossing and kerb layback under s138 of the Roads Act 1993 is required and “Application for Access to Property” shall be obtained from Council.

A paved vehicular access including kerb layback from the property boundary to the street shall be designed in accordance with Councils Standard Drawings: EGSD-103, EGSD-201 or EGSD-104 (available from Councils website).

- Geotechnical Report Compliance

The recommendations of the Geotechnical Report Reference 91313.00.R.001.Rev0 prepared by Douglas Partners dated 18/04/18 shall be complied with. Any works undertaken in relation to the development shall embody all relevant recommendations of the Geotechnical Report.

The engineering plans shall be certified as being designed in accordance with the approved Geotechnical Report.

- Car Parking and Allocation of Spaces

Plans demonstrating a total of 114 car parking spaces in accordance with Australian Standard AS/NZS 2890 shall be submitted.

- Parking Areas and Access Ways

A design plan for parking areas and access ways shall be submitted. The design shall include pavement design, stormwater drainage, line marking and signage. The design shall meet the standards nominated in Development Control Plan 2014, Engineering Guidelines and Australian Standard AS2890.

- Ausgrid Requirements

The written requirements of Ausgrid for the development shall be obtained and complied with.

- Building Over Drainage Easements (Council or Interallotment)

The construction plans shall demonstrate any works within or over an easement for drainage do not affect or interfere with the structural integrity or operation of the pipe system within that easement. Access for maintenance shall be preserved within the easement.

Note: The terms of the easement shall be consulted prior to commencing any works.

- Driveway Design

A design plan for the driveway to the garage or car parking area of the development shall be submitted. The design shall be in accordance with the following requirements and Council Standard Drawing: EGSD-104 (available from Councils website).

- The driveway design levels at the front boundary shall be obtained from Council’s Asset Management department prior to design of the driveway.

- The design plan shall include a driveway longitudinal section, incorporating the design level provided by Council and other construction details (i.e. concrete thickness and reinforcement).

- In accordance with AS 2890.2:2002 the maximum gradient of any driveway or ramp shall not exceed 1V:6.5H (15.4%).

- Where changes of grade occur, suitable transitions shall be designed to meet the Design Vehicle Ground Clearance Diagrams in Figure A1 of AS2890.2:2002.

- Acoustic Certification

Plans shall be certified as being designed in accordance with the approved Acoustic Report, reference 181677, prepared by Spectrum Acoustics, dated November 2018.

The consultant shall be engaged to assist with the preparation of the final building plans and specifications to ensure the proper attenuation measures such as barriers, acoustic suppression material, window laminated glass requirements and the correct sound power levels of specified mechanical plant are installed within the development including provisions preventing drainage grates rattling. This is to confirm compliance with any acoustic conditions and that no other noise nuisance is created to the neighbourhood.

The existing 1.8 metre high acoustic fence along the eastern boundary of the car park shall be maintained to meet the performance standard recommended by the consultant.

- Construction and Fit-out of Food Premises

The food premises shall comply with the following:

- General Construction

The fit-out of areas used for food preparation, storage or display, shall comply with the requirements of the Food Act 2003, Food Regulation 2010, and Australian Standard AS4674 Design, construction and fit-out of food premises.

Note: Particular attention shall be paid to:

- Routing of plumbing and electrical conduit

- Installation of hand wash basins and cleaners sinks

- Construction of floors, walls and ceilings

- Finishes of floors, walls and ceilings

- Cool room construction

- Installation of fixtures fittings and equipment

- Toilet facilities and airlocks

- Installation of light fittings

- Installation of floor wastes

- Ventilation and exhaust systems

- Plans and Specifications

Plans and specification shall demonstrate compliance with the Food Act and Regulations, and relevant Australian Standards.

- Mechanical Ventilation

Installation of any mechanical ventilation systems shall comply with the provisions of Part 2 of the Australian Standard AS1668.

- Partition Wall Construction

Any partition wall (that do not extend to the ceiling), sills or other ledges located within food preparation areas shall be splayed on top at an angle of 45 degrees to prevent storage of articles and reveal build-up of food waste, dirt, grease or other visible matter.

- Waste Traps

Any bucket traps, grease traps and associated sewer connections shall be installed in accordance with the requirements of Hunter Water Corporation.

- Location of Plant and Equipment

To minimise the visual impact of plant and equipment, all plant and equipment (including but not limited to air conditioning equipment) shall not be mounted to the roof of the building where visible from the street. Details demonstrating compliance with this condition shall be submitted to the Certifying Authority for approval with the first Construction Certificate.

- Roadways and Drainage Works Standards

All works within the public domain, including public infrastructure, shall be designed and constructed in accordance with the following publications (as amended or updated), as applicable:-

- Australian Rainfall and Runoff, 1987.

- Development Control Plan 2014 and supporting guidelines.

- AUSTROADS Guide to Road Design Guide.

- Roads and Maritime Services Delineation Guidelines.

- Managing Urban Stormwater documents (2004) by Landcom.

- The Constructed Wetlands Manual - Department of Land and Water Conservation, 1998.

- WSUD Technical Design Guidelines for South East Queensland.

- Healthy Waterways – Water by Design Guidelines.

- Australian Standards including, but not limited to:-

- AS1428 - Design for Access and Mobility, Part 1 General Requirements for Access and Part 4 Tactile Indicators,

- AS2890 - Off Street Parking

Where any inconsistency exists between these documents the relevant standard to be adopted shall be verified in writing with Council.

- Construction Certificate for Public Works

An application for a Construction Certificate and the associated fee shall be submitted to Council for the realignment of the Council drainage infrastructure and any adjustments to other Council assets.

The application shall include detailed engineering plans and specifications (including a Design Certification Report and Checklists in accordance with the Lake Macquarie City Council Engineering Guidelines) relating to the works.

- Stormwater Standards

A design shall be submitted for the realignment of the Council stormwater infrastructure in accordance with the requirements contained in Development Control Plan 2014 and Guidelines. The stormwater design shall meet the requirements of the publications and standards identified in this consent.

- Compliance Certificate for Works

An application for a Compliance Certificate and the associated application fee shall be submitted to Council prior to the commencement of works identified on the respective Construction Certificate for Public Works.

- Application Fees for Required Certificates

Applications for the following Certificates shall be submitted. For Council to process applications for these certificates the following fees shall be payable:

| Construction Certificate for Public Works | Please contact Council for fees | Plus $75 Archival Fee | (inc GST) |
| --- | --- | --- | --- |
| Compliance Certificate | Please contact Council for fees | Plus $75 Archival Fee | (inc GST) |

Applications for these certificates shall be lodged on the approved application form and be accompanied by the appropriate fee.

Where the development includes public construction works valued at $25,000.00 or more, the person having the benefit of the consent shall pay the Long Service Levy, as detailed in the Building and Construction Industry Long Service Payments Scheme. The Levy shall be paid prior to the issue of the Construction Certificate for Public Works. The Levy shall be paid directly to the Long Service Payments Corporation or to Council as agent for the Corporation. The Levy rate is 0.35% of the cost of building and construction works.

The above fees are current for the 2019/2020 financial year and are subject to change each financial year without notice. Confirmation of the relevant fee shall be obtained from Council prior to the lodgement of any application.

- Waste Management

Garbage washing and bin storage area shall be constructed of, or lined with materials that are durable, impervious to moisture, and capable of being easily cleaned. Construction details shall be provided to the Principal Certifying Authority prior to the issue of the first construction certificate.

Documentation confirming arrangements/agreements have been made with either a private contractor or Lake Macquarie City Council for the collection of general waste, green waste and recycling materials shall be submitted to the Principal Certifying Authority prior to the issue of with the first Construction Certificate. This documentation shall demonstrate the waste contractor can service the development for all waste streams, and within designated loading dock and waste storage areas.

Conditions to be satisfied prior to the commencement of works

The person having the benefit of the consent shall comply with each of the following conditions prior to the commencement of works.

- Erosion and Sediment Control

Appropriate erosion and sediment controls shall be installed in accordance with Development Control Plan 2014.

- Dilapidation Survey Report

A Dilapidation Report shall be submitted to Council and the Certifying Authority prior to any works, including demolition works, commencing. The report shall be prepared by an engineer or other suitably qualified person.

The report shall cover all properties, Craighill Crescent and public land that adjoin the development or that could be potentially damaged by the works.

The report shall include a description of the location and nature of any existing observable defects to the properties and existing public infrastructure, including a photographic record.

- Hoarding and Construction Site Safety Fencing

Construction site safety fencing and/or hoarding shall be provided in accordance with WorkCover requirements. Such fencing and/or hoarding shall be erected wholly within the property boundary unless prior approval from Council is obtained.

Council approval is required to install hoarding, site fencing or overhead protective structures over or adjoining a public place i.e. a footpath or a Public Reserve. No work shall commence until written approval is obtained.

- Retention of Trees and Native Vegetation

All native trees and vegetation on the site shall be retained and protected in accordance with Development Control Plan 2014 Guidelines – Tree Preservation and Native Vegetation Management Guidelines (Section 6) and the Australian Standard AS4970-2009 – Protection of Trees on Development Sites unless it:

- has been identified for removal on the approved plans or documentation; or

- has been identified for selective removal, or is exempt under the 10/50 Clearing Entitlement Area as deemed by the NSW Rural Fire Service.

Exclusion fencing shall be installed around all native vegetation that shall be retained on and adjoining the site to minimise damage, prior to the commencement of works. The person having the benefit of the consent shall notify Councils Development Planner Flora and Fauna when exclusion fencing has been installed. Vegetation exclusion fencing shall be maintained in good working order for the duration of works.

- Filling Importation and Compaction

Documentary evidence shall be provided to Council demonstrating the proposed fill material is either:

- Virgin excavated natural material (VENM) as defined under the provisions of the NSW Protection of the Environment Operations Act 1997; or

- The material is from a known origin and composition, free of contamination from manufactured chemicals, process residues, building debris, sulfidic ores, or other foreign matter; or

- Fill which has been characterised and validated by a suitably qualified and experienced site contamination consultant, in accordance with the NSW Office of Environment and Heritage publication Contaminated Sites - Sampling Design Guidelines dated September 1995.

Written certification from a suitably qualified geotechnical engineer that the material is suitable for the proposed use on the site, shall be provided to Council. Only that material certified by the geotechnical engineer shall be imported to the site.

Written details shall be kept of address of the origin of the fill; quantities, dates, and times of delivery from each location; registration numbers and driver’s identification details; and laboratory test results/consultants reports and available for inspection by the Certifying Authority or Council upon receipt of a written request.

- Building Waste

An area for the containment of building waste materials shall be provided within the boundaries of the building site, above natural or excavated ground level, by a screened area of silt stop fabric or shade cloth, having minimum dimensions of 2.4 x 2.4 x 1.2 metres high OR equivalent size waste disposal bin.

- Works Within a Public Road Reserve

Approval is granted to open a grassed or natural surface footpath for the installation of all water services, cables, or mains. Upon completion of the work, the footpath shall be restored to its original state and no hazards shall remain that may impact on the public.

For any other works within the road reserve, an approval under s138 of the Roads Act 1993 shall be obtained from Council. The road shall not be opened until the approval has been issued.

- Dial Before You Dig (Advice)

The free national community service “Dial Before You Dig” shall be contacted on 1100 regarding the location of underground services in order to prevent injury, personal liability and even death. Enquiries shall provide the property details and the nearest cross street/road.

- Details Required Prior to Commencement of Public Works

Construction works in accordance with this development consent shall not commence until:-

- a Construction Certificate has been issued by Council

- the person having the benefit of the development consent has given at least two days notice to Council of the intention to commence works.

In accordance with Council's authority under Section 138 of The Roads Act 1993, a Private Certifier shall not issue a Construction Certificate for any works within an existing public road. A Construction Certificate application shall be lodged with Council for the Public works.

A fee for applications for Construction Certificates shall be required to be paid in accordance with Council’s fees and charges for Construction Certificates.

- Construction Management Plan

The contractor engaged to undertake the construction of the development and the contractor engaged to undertake the public works shall provide a Construction Management Plan (CMP) to Council, a minimum of seven days prior to commencing any works. The CMP shall be submitted to the City Projects Department at the following email address projectmanagementpat@lakemac.nsw.gov.au. The CMP shall be approved by Council prior to works commencing on site. The CMP shall set out the construction approach for the works and should seek to minimise disruption to the local community.

As a minimum, the CMP must address the following areas:

Health and Safety

- Public safety, amenity and site security;

- Traffic Control and Management;

- Pedestrian management;

- Construction hours;

- Noise control;

- Contractor vehicle parking;

- Locating existing utilities and services;

- Health and Safety requirements.

Environment

- Air quality management;

- Erosion and sediment control- base information, monitoring and management;

- Waste management;

- Material stockpiling;

- Vegetation management;

- No go zones;

- Heritage management (if applicable).

Quality

- Submission of current insurance certificates;

- Work method description;

- Construction equipment to be used;

- Inspection and testing requirements;

- CCTV survey of pipework;

- Earthworks methodologies;

- Haulage routes;

- Retaining structure construction methodologies;

- Concrete jointing methodologies;

- Subsoil drainage installation methodologies;

- Stormwater drainage infrastructure installation methodologies;

- Stormwater Quality Improvement Device installation methodologies;

- Road construction methodologies;

- Accessways and footway construction methodologies;

- Landscaping installation methodologies;

- Utility and services installation methodologies

- Construction and installation methodologies of other structures not otherwise covered above.

All works on site shall be undertaken in accordance with the approved CMP.

- Notice of Commencement of Public Works

Construction works shall not commence until a meeting between the contractor and a representative of the Council or Certifying Authority has taken place on site.

In accordance with Section 6.12 of the Environmental Planning and Assessment Act the Council or Certifying Authority shall be given at least two days notice in writing prior to such meeting taking place.

The notice shall also include the names of the contractor undertaking construction and the developer's supervising officer.

Conditions to be satisfied during demolition and construction works

The person having the benefit of the consent shall comply with each of the following conditions during demolition and construction works.

- Erosion and Sediment Control

All erosion and sediment controls shall be appropriately managed throughout the development to prevent pollution until the land is considered erosion resistant.

Any pollution from site shall be cleaned up immediately and appropriate repairs made to onsite controls.

The final plan shall include a signed and dated Statement of Compliance stating (in full):

- This plan has been developed, certified and signed off by an appropriately qualified and experienced professional in erosion and sediment control;

- The plan complies with the requirements for the area of disturbance in accordance with Development Control Plan 2014;

- The plan and associated documents, calculations and drawings, have been prepared to a standard which, if properly implemented, shall achieve the water release criteria of 50mg/L of total suspended solids; and

- All erosion and sediment control measures are in accordance with Development Control Plan 2014.

- Topsoil and Stockpiles of Materials

Topsoil shall only be stripped from approved areas. It may be stockpile onsite for re-use during site rehabilitation and landscaping. Stockpiles of any material including but not limited to, soil, sand, aggregate, and spoil, stored on the site that is capable of being moved by water shall be stored clear of any drainage line or easement, natural watercourse, footpath, kerb, and/or road surface. Suitable erosion and sediment controls shall be installed. The stockpile shall be treated so its surface is resistant to water and wind erosion. No stockpiles shall be located on the public footpath or road reserve without prior written approval from Council.

- Fix Damage Caused by Construction Works

Any damage to a public road or associated structures including footpaths, drains, kerb and gutter and utility services caused as a consequence of the construction works shall be made good to the satisfaction of Council.

Any disused kerb and gutter and footpath crossing shall be removed and replaced with full kerb and gutter in accordance with Council's standards to match finished adjoining kerb and gutter.

- Geotechnical Report Compliance

Where the geotechnical report prepared for this development requires inspections, a suitably qualified engineer shall inspect the works at the stages specified in that report.

- Retention of Trees and Native Vegetation

All reasonable measures shall be undertaken to protect native vegetation on, and adjoining the site, that shall be retained from damage during construction. Such measures shall include, but not be limited to:

- maintaining exclusion fencing around vegetation that adjoins the construction area to minimise damage to vegetation that shall be retained;

- prohibiting compaction and the placement of fill within five metres of trees and native vegetation that shall be retained;

- keeping all vehicles, construction materials and refuse within areas approved for buildings, structures, access ways and car parks;

- limiting the number of access points;

- salvaging useable trees and shrubs which are felled for re-use, either in log form, or as woodchip mulch for erosion control and/or site rehabilitation. Non-salvageable material such as roots and stumps shall only be disposed of at an approved site;

- notifying all contractors, sub-contractors, and personnel of vegetation protection requirements of this condition.

A separate application shall be made to Council in accordance with the guidelines referenced above for the removal of any other trees or native vegetation. This includes application for the removal of any understorey vegetation or the stripping of ground cover vegetation that is outside those areas approved for construction.

- Landscape Works

Landscape works shall be constructed in accordance with the approved landscape plans and specification.

A minimum of 18 Eucalyptus/Angophora trees, as scheduled, shall be planted across the Portland Drive frontage including the embankment to the north of the church building.  A minimum of 23 Eucalyptus/Angophora trees, as scheduled, shall be planted along the southern boundary adjacent to George Booth Drive.

Street trees along Craighill Crescent shall comprise five 75 litre Waterhousia floribunda planted at centres along the streetscape as per the approved landscape plan and PLA-LSD-02 Lake Macquarie City Council Landscape Standard Drawings March 2019. Street trees shall have standard timber edging installed and all pot stakes, labels and wire/rubber/plastic tags removed. All street trees shall be grown to AS2303:2018.

All landscape planting works shall be implemented under the full supervision of a contractor with a current NSW Department of Fair Trading endorsed license in Structural Landscaping.

All paving works shall be implemented under the full supervision of a contractor with a current NSW Department of Fair Trading endorsed license in Structural Landscaping, Building or Minor Trade – Paving.

Landscape works shall conform to planting densities as scheduled with all plants at nominated pot sizes and spacing’s and be maintained for a minimum of 52 weeks to achieve continuous healthy growth. A hard garden retaining edge (timber or concrete) shall be installed to all planting areas adjoining turf surfaces. All planted areas shall be covered with minimum 100mm mulch to aid plant establishment.

- Building Over Drainage Easements (Council or Interallotment)

Any damage to the stormwater pipe system caused by the works, within or over the easement shall be repaired.

- Excavation and Retaining

Only retaining walls indicated on the approved plans shall be constructed under this consent.

No additional excavation/fill shall occur outside the area as shown on the approved plans.

No fill or retaining walls shall be located within any drainage easement located upon the subject property.

Retaining walls, footings and associated drainage works shall be located wholly within the subject property boundaries and shall be connected to the existing stormwater system or other approved stormwater system on the subject property.

Note:	Some retaining walls are able to be erected without consent, as Exempt or Complying Development pursuant to State Environmental Planning Policy Exempt and Complying Development Codes 2008. Prior to erection of any retaining wall not approved under this consent, reference to the State Environmental Planning Policy Exempt and Complying Development Codes 2008 shall be undertaken to ascertain whether approval is required.

- Filling Importation and Compaction

All fill shall be placed in accordance with the standards specified in Table 5.1 of AS 3798-2007 Guidelines on Earthworks for Commercial and Residential Developments.

- Building Waste

The enclosure or bin shall be maintained for the term of the construction to the completion of the development.

The enclosure or bin shall be regularly cleaned to ensure proper containment of the building wastes generated on the site.

- Waste Management

All requirements of the approved Waste Management Plan shall be implemented during the demolition, excavation and construction of the development.

- Works Within a Public Road Reserve

When works are being undertaken within a public road reserve, all necessary precautions shall be taken to protect the public while work is in progress, this shall include traffic control in accordance with Australian Standard AS1742 – Manual Uniform Traffic Control Services – Parts 1, 2 and 3.

- Unobstructed Footpath Access

Unobstructed footpath access shall be maintained within the public road reserve at all times. Building materials shall not be placed or stored within the road reserve.

In the case of sites where it is not possible to keep the footpath or road reserve clear during construction works written approval from Council shall be obtained prior to any closing of the road reserve or footpath area. The closure shall take place in accordance with Council’s written approval. The area shall be signposted and such signposting be maintained in a way that ensures public safety at all times.

- Site Amenities

Toilet facilities shall be available or provided at the work site before works begin and shall be maintained until the works are completed at a ratio of one toilet plus one additional toilet for every 20 persons employed at the site.

Each toilet shall:

- be a standard flushing toilet connected to a public sewer, or

- have an approved on-site effluent disposal system under the Local Government Act 1993, or

- be a temporary chemical closet.

- Dust Suppression

During the extraction, removal, and transportation of material associated with the works, the person having the benefit of the consent shall ensure that airborne dust is contained within the work site or transport vehicles, and does not impact on the amenity of the surrounding environment.

Effective environmental controls and practices shall be implemented and maintained to the satisfaction of Council or the Certifying Authority.

- Noise - Construction Sites

The operating noise level of construction site operations, including machinery, plant and equipment when measured at any affected premises, shall be evaluated and comply with the requirements of the NSW Office of Environment and Heritage publication Interim Construction Noise Guideline July 2009.

Approved Construction Times

- The approved hours for construction of this development are –

Monday to Friday - 7.00am to 6.00pm.

Saturday – 8am to 1pm.

- No construction work shall take place on Sundays or Public Holidays.

Construction Periods in Excess of 26 Weeks

- If the construction period is in excess of 26 weeks, a Noise Management Plan shall be provided to Council prior to the issue of the first construction certificate. Such plan shall be prepared with the assistance of a suitably qualified acoustic engineer, indicating whether the use of machinery, plant and equipment during those operations can be completed without causing offensive noise (as defined in the Protection of the Environment Operations Act 1997) in the neighbouring area. The Noise Management Plan shall be complied with at all times during the construction period and shall identify any mitigation measures to control noise, noise monitoring techniques and reporting methods, likely potential impacts from noise and a complaints handling system.

- Operational times may be amended with the written advice of Council’s General Manager or delegate.

- Construction Site Vibration

Vibration on surrounding land from construction site operations shall comply with the Office of Environment and Heritage publication Assessing Vibration: a technical guideline February 2006.

Conditions to be satisfied prior to issue of an Occupation Certificate

The person having the benefit of the consent shall comply with each of the following conditions prior to the issue of the Occupation Certificate, Interim or Final, as stated in each condition.

- Occupation Certificate

The development shall not be occupied or used prior to the issuing of a Final Occupation Certificate or Interim Occupation Certificate by the Certifying Authority.  Where an Interim Occupation Certificate has been issued, only that part of the building to which the Certificate applies may be occupied or used.

- Commencement of the Use of the Land

The approved use of the land shall not commence until all relevant conditions of this consent have been complied with and a Final or Interim Occupation Certificate has been issued. Where an Interim Occupation Certificate has been issued, only that part of the building to which the Certificate applies may be occupied or used.

- Stormwater Disposal and Harvesting

All drainage works shall be carried out in accordance with the approved Construction Certificate plans.

Prior to the issue of an Interim or Final Occupation Certificate, whichever comes first, a Works As Executed Plan shall be prepared by the plumber undertaking the works and shall be submitted to the Certifying Authority that demonstrates compliance with the approved Construction Certificate. If there are any changes from the Construction Certificate these shall be highlighted in a different colour on the plan and certification shall be provided from the design engineer the changes do not affect the stormwater design outcomes.

- Provision of Council Easement (Public)

An easement to drain water, (minimum three metres wide or the width of the 100 year flow path, whichever is greater) shall be created at no cost to Council in favour of Council in accordance with the approved Stormwater Drainage Plan by ADW Johnson, Ref: 239840, Rev.D, dated 27/09/19.

An accompanying 88B instrument shall also include:

- The Registered Proprietor of the lot hereby burdened will in respect of the drainage system conveying municipal road drainage water;

  - Keep the system clean and free from silt, rubbish and debris.

  - Maintain and repair at the sole expense of the Registered Proprietors the whole of the system within the property, so that it functions in a safe and efficient manner.

  - Permit the Council or its authorised agents from time to time and upon giving reasonable notice (but at any time and without notice in case of an emergency) to enter and inspect the land for the compliance with the requirements of this covenant.

  - Comply with the terms of any written notice issued by the Council in respect of this covenant within the time stated in the notice.

- Pursuant to Section 88F(3) of the Conveyancing Act 1919 the Council shall have the following additional powers:

  - In the event that the Registered Proprietor fails to comply with the terms of the written notice issued by Council as set out above the Council or its authorised agents may enter the land with all necessary equipment and carry out any work which the Council in it discretion considers reasonable to comply with the said notice referred to in Part 1 (d) above.

  - The Council may recover from the Registered Proprietor in a court of competent jurisdiction:

    - Any expense reasonably incurred by it in exercising its powers under sub-paragraph (a) hereof together with costs, reasonably estimated by the Council for the use of machinery, tools, and equipment in conjunction with the said work. Such expense shall include without limitation reasonable wages for the work and administering the said work.

    - Legal cost on an indemnity basis for issue of the said notices and recovery of the said cost and expenses together with the cost and expenses of registration of a covenant charged pursuant to Section 88F of the Act or providing any certificate required pursuant to Section 88F of the Act or obtaining any injunction pursuant to Section 88H of the Act.

The easement shall be registered on the title of the lot under Section 88B of the Conveyancing Act 1919 as amended. A copy of the registered Section 88B Instrument shall be provided to Council prior to the issue of an interim or final occupation certificate, whichever occurs first.

The existing easement to drain water benefited by Council, shall be released at the same time as the creation of the new easement to drain water.

Council shall be the authority with the power to release, vary or modify the terms of the easement.

- Dilapidation Survey Report

A final Dilapidation Report shall be prepared by an engineer or suitably qualified person at the completion of the works. The report shall identify if any structural or cosmetic damage has occurred to the properties specified in the earlier report. A copy of the report shall be submitted to Council, the Certifying Authority and owners of potentially affected properties and public infrastructure. Any identified damage shall be repaired prior to the issue of the Final Occupation Certificate.

- Fix Damage Caused by Construction Works

Any damage to a public road, associated structures or drainage infrastructure through the site caused as a consequence of the construction works shall be made good to the satisfaction of Council.

Any disused kerb and gutter and footpath crossing shall be removed and replaced with full kerb and gutter in accordance with Council's standards.

These works shall be undertaken prior to the issue of the Final Occupation Certificate.

- Vehicles Access Crossing and Kerb Layback

Prior to the issue of the Final Occupation Certificate, the paved crossing shall be completed for the building, at the owner's cost. Any disused kerb and gutter and footpath crossing shall be removed and replaced with full kerb and gutter to match adjoining kerb and gutter to the satisfaction of Council.

- Geotechnical Report Compliance

Prior to the issuing of the Interim or Final Occupation Certificate, whichever comes first, the suitably qualified engineer who inspected the works shall provide written confirmation the constructed works are in accordance with the recommendations of the Slope Stability Geotechnical report prepared for the development.

- Tactile Indicators

Prior to the issue of an interim or final occupation certificate, whichever occurs first, the development shall have tactile indicators installed in accordance with AS 1428.4.

Note:	The areas of the development to have tactile indicators installed are varied and include kerb ramps and footpaths along the street frontage; carpark pedestrian crossings (street frontage and internal); lifts; stairways and ramps.

- Car Parking

All car parking spaces, line marking and signage shall be completed prior to the issue of the Interim or Final Occupation Certificate, whichever comes first.

- Bicycle Parking Racks

Secure bicycle parking/storage shall be provided to accommodate 10 bicycles on the site. The installation and dimensions of the bicycle parking/storage shall be in accordance with the Austroads Cycling Aspects of Austroads Guides and Australian Standard AS2890.3:1993. These works shall be completed prior to the issue of the Final Occupation Certificate.

- Parking Areas and Access Ways

All parking areas and access ways shall be fully constructed and sealed in accordance with the approved Construction Certificate plans, prior to the issue of the Interim or Final Occupation Certificate, whichever comes first.

- Retention of Trees and Native Vegetation

Vegetation to be retained shall be inspected by Council’s Development Planner Flora and Fauna, prior to the issue of the Final Occupation Certificate. Any areas requiring rehabilitation shall be identified and remediation works completed prior to the release of the Final Occupation Certificate.

- Landscape Works

At the completion of landscape works, the consulting landscape architect who prepared the documentation shall submit to the Certifying Authority a Landscape Compliance Report. This report shall certify shrub and tree species, pot size, and planting densities and landscape area setout comply with approved documentation and practical completion of the landscaping works has occurred. This report shall certify any required street trees are planted specifically as per the nominated locations on the approved documentation and these conditions, and have edging constructed as per the Lake Macquarie City Council Landscape Standard Drawings March 2019. The Certifying Authority shall not issue the Final Occupation Certificate without receipt of the Landscape Compliance Report.

- Boundary Fencing

Fencing shall be erected and maintained in accordance with the approved Landscape Plans.

All costs associated with the removal and replacement of any fence shall be borne by the person with the benefit of the consent and not the relevant neighbouring property owners.

- Driveway Construction

Prior to the issue of the Interim or Final Occupation Certificate, whichever comes first, the driveway to the garage or car parking area of the development shall be constructed in accordance with the approved Construction Certificate plan.

- Works Within a Public Road Reserve

Where a road surface has been disturbed or damaged the calculated road restoration fee shall be paid to Council. Evidence shall be provided the relevant fee has been paid prior to the issue of the Final Occupation Certificate.

- Removal, Management and Transportation of Fill

For all excavated fill material that has been removed from the site during construction:

- A copy of the receipts from the waste disposal facility shall be kept and provided to the Certifying Authority prior to the issue of the Final Occupation Certificate, or authorised officer of Council upon request; or

- A copy of the current development consent for the site to which the material has been distributed shall be provided to the Certifying Authority prior to the issue of the Final Occupation Certificate, or authorised officer upon request.

- Acoustic Certification

Prior to the issue of the Interim or Final Occupation Certificate, whichever comes first, written confirmation shall be provided from a suitably qualified acoustic consultant certifying works have been completed in accordance with the recommendations of the Acoustic Report prepared for the development and the development is capable of operating in accordance with the design criteria.

- Certification of Food Premises Fit-out

Prior to the issue of the Interim or Final Occupation Certificate, whichever comes first, the food premises shall be inspected by an appropriately qualified person who shall certify the premises, including the construction and installation of all equipment, fixtures, fittings and finishes therein, complies with the Food Act 2003, Food Regulation 2010, and Australian Standard AS 4674 Design, construction and fit-out of food premises.

- Works as Executed Plan – Public Works

An electronic copy of the Works as Executed Plans, certified by the Consulting Civil Engineer or Registered Surveyor supervising the works shall be supplied to Council.

The Works as Executed Plan shall, in addition to construction details, show limits and depths of filling, locations of service conduits and street names.

- Compliance Certificate for Works

All public construction works required for the development shall be completed and a Compliance Certificate shall be obtained for these works. The Compliance Certificate shall certify that all public construction works and associated development have been constructed in accordance with this Development Consent, the Construction Certificate for Public Works and all other standards specified in this consent.

- Crime Prevention Through Environmental Design

The development shall be constructed and maintained in accordance with the recommendations of the approved Crime Prevention Through Environmental Design Report, and the following:

- A management plan shall be prepared for the site that includes, but is not limited to:

  - Where graffiti occurs at the site, it shall be removed within 24 hours of its appearance;

  - Where lighting is broken or damaged, it shall be repaired within 48 hours.

The plan shall be completed prior to the issue of the Final Occupation Certificate and be available upon request.

- No Parking Restrictions

No Parking restrictions shall be provided to the western side of Craighill Crescent. The restrictions shall apply from the intersection of Craighill Crescent and Northridge Drive, to the western boundary of 33 Craighill Crescent.

The restrictions are subject to the approval of Lake Macquarie Traffic Facilities and Road Safety Committee.

Plans showing signposting and line marking on the public street system/road related area shall be submitted to Council for approval by Lake Macquarie Traffic Facilities and Road Safety Committee.

All restrictions, signposting and line marking shall be finalised and installed prior to the issue of the first Occupation Certificate.

Following approval of the restrictions, the person having the benefit of this consent shall notify all affected properties in Craighill Crescent of the restrictions.

Operational Conditions

The person having the benefit of the consent shall comply with each of the following conditions during the operation of the development.

- Use and Allocation of Car Parking

The car parking provided shall only be used in conjunction with the approved use contained within the development and except as provided for in these conditions, shall not be used other than by an occupant or tenant of the development.

The spaces shall be allocated in the following proportions:

| Place of Public Worship | 111 |
| --- | --- |
| Disabled car spaces | 3 |
| Total | 114 car spaces |

No consent is given or implied to utilise the Village Green for over flow car parking.

- Retention of Trees and Native Vegetation

All native trees and vegetation on the site shall be retained and protected in accordance with Development Control Plan 2014 Guidelines – Tree Preservation and Native Vegetation Management Guidelines (Section 6) and the Australian Standard AS4970-2009 – Protection of Trees on Development Sites unless it:

- has been identified for removal on the approved plans or documentation; or

- has been identified for selective removal, or is exempt under the 10/50 Clearing Entitlement Area as deemed by the NSW Rural Fire Service.

- Landscape Works

All landscape works required under this consent shall undergo an establishment maintenance period of a minimum of 52 weeks to achieve continuous healthy growth improving amenity and aesthetics over the site and meeting the aims for development in the zone.

All landscaping shall then be permanently maintained in good condition in accordance with the approved landscape plan and the adopted Development Control Plan 2014 Guidelines – Landscape Design Guidelines.

- Hours of Operation

Following commencement of occupation, the premises shall operate or trade only between the times stated as follows:

Mondays to Sundays and Public Holidays 7am to 10pm

Other internal operations such as cleaning, preparation, and office administration may be undertaken outside of the above hours provided no disturbance to the amenity of the neighbourhood occurs.

A minimum of 30 minutes must be allowed between the end of one service and the beginning of the next to minimise noise and traffic.

- External Storage of Products

The external storage or display of any products on the development site is not permitted.

- Advertising Structures and Signs

Development consent shall be obtained from Council prior to the erection of any advertising structures or signs on the site. This requirement does not apply to any exempt, complying, or previously approved signage.

- Lighting

Lighting shall comply with Australian Standard AS4282-1997.

- Onsite Loading Area

The on-site loading / service area shall be kept clear of goods and is not permitted to be used for any storage purposes.

All loading operations associated with servicing the site, shall be carried out within the boundaries of the site, and shall not obstruct other properties, access driveways, public roads or footpaths.

- External Material Reflection

External cladding materials such as roofs, walls and windows shall have low-reflective properties.

- Acoustic Certification

At 90 days of operation a suitably qualified acoustic consultant shall test, measure and certify the development is operating, at that time, in accordance with the approved Acoustic Report.

- Liquid Gaseous Wastes, Emissions and Odour Control

Emissions shall be responsibly managed at all times, so as not to cause a danger to public health or loss of amenity or damage to the environment.

Any liquid gaseous wastes, emissions or odours shall be controlled and disposed of in accordance with the NSW Protection of the Environment Operations Act 1997 and Regulations 1997, as amended.

Where it is proposed to treat and discharge waters to the Hunter Water Corporations Sewer, formal approval from the Corporation shall be obtained.

- Emissions

There shall be no interference with the amenity of the neighbourhood by reason of the emission of any "offensive noise" as defined in the Protection of the Environment Operations Act 1997, vibration, smell, fumes, smoke, vapour, steam, soot, ash or dust, or otherwise as a result of the development.

- Approved numbers

The approved Place of Public Worship is limited to a maximum capacity of 342 seats only at upper ground floor level.

The maximum total occupancy of the premises allowed by this consent is 342 people.

No consent is given or implied for the 110 seats at mezzanine level. This area shall be utilised for storage purposes only.