SCHEDULE 1 – CONDITIONS OF CONSENT

1.0 - General Conditions of Consent

Pursuant to Clause 100 of the Environmental Planning & Assessment Regulation 2000, this Notice of Determination relates to a concept development application. Subsequent development applications are required prior to commencement of any development, subdivision of land or associated subdivision construction works on the site.

While this consent remains in force, the determination of any further Development Application(s) (‘Stage 2’ Development Application(s)) in respect of the site cannot be inconsistent with this consent.

This development consent will lapse after a period of 5 years effective from the date of this Notice, unless subsequent consent is granted to a stage 2 Development Application(s) to carry out development on the site and that development has commenced.

The following conditions of consent are general conditions applying to the Concept Development and Stage 1 Initial Works.

(1)	General terms of approval – The General Terms of Approval from state authorities must be complied with prior to, during, and at the completion of the development.

The General Terms of Approval are:

- Rural Fire Service, reference 2018-04800-DL55-1 dated 2 March 2020.

- Natural Resource Regulator, reference IDAS1111622 dated 8 August 2019.

- Transport NSW, reference 2019/002430 dated 20 December 2019.

- Department of Primary Industries, reference FE19/796 dated 19 September 2019.

A copy of the General Terms of Approval is attached to this determination notice.

(2)	Approved plans and documentation – Development must be carried out in accordance with the following plans and documentation, and all recommendations made therein, except where amended by the conditions of this development consent:

| Plan/Doc.Title | Plan Ref. No | Date | Drawn By |
| --- | --- | --- | --- |
| Scope of Concept Approval | SK-64-C1.00. Rev 11. | 14/12/20 | Northrop |
| Scope of Concept Approval - Proposed Stage 1 – Initial Subdivision Works – Clearing Phases | DA-08-8.00. Rev. P. | 21/09/20 | Northrop |
| Scope of Concept Approval - Proposed Stage 1 – Initial Subdivision Works – Staging Plan | DA-08-8.01. Rev. K. | 22/09/20 | Northrop |
| Preliminary Basin Layout | CSK67. Rev. 3. | 22/09/20 | Northrop |
| Proposed Detention Basins | CSK67 – Sheet 1 & 2 - Attachment D | 22/09/20 | Northrop |

| Document title | Prepared by | Date |
| --- | --- | --- |
| Engineering Report – Kings Hill Master Plan (reference NL120526) | Northrop | 11 December 2019 |
| Biodiversity Management Plan (reference PR130430) | RPS Group | 24 July 2020 |
| Vegetation Management Plan (reference PR130430) | RPS Group | 24 July 2020 |
| Species Impact Statement (reference 130430) | RPS Group | 24 July 2020 |
| VPA Letter of Offer | Colin Biggers and Paisley | 10 December 2020 |

In the event of any inconsistency between the approved plans and the conditions, the conditions will prevail.

(3)	External agency approvals – The requirements from the following agencies must be complied with prior to, during, and at the completion of the development.

The Requirements are:

- Hunter Water Corporation, reference HW2019-776/24.039 dated 22 June 2020.

- Ausgrid, reference C03048-Z0D6X7 dated 6 June 2019.

A copy of the Requirements is attached to this determination notice.

2.0 – Conditions for Concept Approval

Pursuant to Clause 100 of the Environmental Planning & Assessment Regulation 2000, this Notice of Determination relates to a concept development application. Subsequent development applications are required prior to commencement of any development, subdivision of land or associated subdivision construction works on the site.

- Any future detailed development application shall be prepared generally in accordance with the approved Scope of Concept Approval Plan (SK-64-C1.00. Rev 11).

- Subsequent development applications shall be prepared generally in accordance with the approved Preliminary Basin Layout (CSK67. Rev. 3) and the Engineering Report – Kings Hill Master Plan prepared by Northrop (reference NL120526).

- Subsequent applications for residential subdivision in the Pacific Highway noise impact area, or any land in proximity to the Pacific Highway as identified by Council that has the potential to be impacted by adverse road noise, must be supported by acoustic modelling and noise impact assessment.

Any required noise barriers must include a buffer between residential areas and the Pacific Highway and include landscaping and visual treatment measures.

- All development within 250m of the Newline Road Waste Disposal Facility and Raymond Terrace Resource Recovery Park and landfill or any land in proximity as identified by Council has the potential to have methane concentrations of greater than 1.25% (v/v) in the subsurface, is to be tested with a tested/calibrated methane detector over regular intervals for 12 months prior to a development application being lodged with Council for determination.

Development and monitoring should comply with the relevant sections of the NSW Environmental Protection Agency 'Environmental Guidelines: Solid Waste Landfills', or its successor.

Where buildings are approved within a landfill gas migration footprint associated with a land fill that has been operational up to 30 years prior to the occupation of the building, unless data is presented demonstrating otherwise, the structure should be required to be designed and constructed to a level that would protect inhabitants from the risk of gas accumulating within the structure.

- Subsequent development applications for subdivision of land within 1km of the Raymond Terrace Resource Recovery Park and landfill are to be accompanied by a detailed noise impact assessment and air quality impact assessment.

- Subsequent development applications for subdivision of land is to be accompanied by confirmation that satisfactory arrangements for State Public Infrastructure have been made, relevant to that application. Future development must be carried out in accordance with State Voluntary Planning Agreement executed on 25 October 2019, reference SVPA-2014-9939.

- Within six months of the date of the determination by the consent authority, the Applicant must enter into a Voluntary Planning Agreement with Port Stephens Council in accordance with:

- Division 7.1 of Part 7 of the EP&A Act; and

- the terms of the offer set out in the letter from the Applicant to Port Stephens Council dated 10 December 2020 (document reference: 16121054_3) at Schedule 6.

This Development Consent is not to operate until the Voluntary Planning Agreement is executed and registered on title.

- Subsequent development applications for subdivision of land is to be accompanied by preliminary design specifications and servicing strategies for the provision of public and private utility infrastructure, relevant to that application.

- Subsequent development applications for subdivision of land is to be accompanied by detailed plans for precinct and lot layout, including road network, lot areas and dimensions generally in accordance with the approved Scope of Concept Approval (SK-64-C1.00. Rev 11).

- Subsequent development applications for subdivision of land is to be accompanied by detailed plans for open space and recreation areas, including furnishings and landscaping detail. The provision of community and recreation assets is to be detailed in the application in accordance with the Kings Hill Urban Release Area Community and Recreation Infrastructure Study (GHD, April 2020), Section D14.C of the Port Stephens DCP 2014. The Study specifies the following community and recreation infrastructure to be provided within the Kings Hill Urban Release Area as it relates to the site:

- One community centre and library (200m2) to be located in town centre/district park;

- Two long dare care centres to be co-located with community centre and/or public school;

- One preschool to be co-located with public school;

- One RFS Building (to be planned in consultation with RFS);

- Six local parks (total 3.5ha) - four to contain playgrounds;

- One district park (3.5ha) near town centre to contain Skate Park and two multipurpose courts.

Documentation is to be provided that identifies community and recreation infrastructure that is of a size and location consistent with the recommendations of the Kings Hill Recreation Infrastructure Study. The obligation for the delivery of the community and recreation infrastructure will be in accordance with the Port Stephens Local Infrastructure Contributions Plan 2020.

- Subsequent development applications for development and subdivision of land must be in accordance with the approved Biodiversity Management Plan and Vegetation Management Plan prepared by the RPS Group (dated 24 July 2020, reference PR130430).

- Subsequent development applications for subdivision works is to be accompanied by detailed geotechnical and site contamination assessments.

- Subsequent development applications for subdivision in Precinct 7 must include a North-South Collector road that provides connectivity to Precinct 8.

- Subsequent development applications must be accompanied by stormwater plans, generally in accordance with the approved stormwater master plans and reports, Council’s Infrastructure Specifications and the current Australian Rainfall and Runoff guidelines using the Hydrologic Soil Mapping data for Port Stephens (available from Council), including:

- Stormwater drainage plans under subsequent applications must demonstrate stormwater infrastructure will prevent stormwater entering Grahamstown Dam in rainfall events up to the 0.2% Annual Exceedance Probability (AEP) and that Landcom Stretch Water Quality Targets are being achieved for water draining from the site to Irrawang Swamp (Wetland 804), either directly or indirectly via the stormwater channel.

- Subsequent development applications are to demonstrate Landcom Stretch Water Quality Targets can be achieved at each stage of the development, with the exception of the stretch target for Total Phosphorus (TP). For Total Phosphorus, each stage of the development must meet the modelled Reduction % in TP as presented in Table 5-16: Treatment Train Effectiveness of the approved engineering report (Northrop, reference NL120526, December 2019).

- Measures including disconnecting impervious areas, oversized BASIX rainwater tanks, infiltrating biofiltration systems, stormwater retention and harvesting systems. All water quality measures are to be included as positive covenants on 88B Instruments for future lots.

- The first development application must be accompanied by a Water Quality Monitoring and Management Plan for stormwater leaving the site and draining into Irrawang Swamp (Wetland 804). The Plan must outline monitoring locations (flow and quality), water quality parameters (to be assessed against Landcom Water quality stretch targets), and monitoring program (frequency and duration). Baseline monitoring of Irrawang Swamp (Wetland 804) must be undertaken at least three months prior to any site disturbance works.

The Water Quality Monitoring and Management Plan (WQMMP) should outline an adaptive management framework describing the process to be undertaken if monitoring indicates water quality targets are not being achieved. The WQMMP and ongoing monitoring should be undertaken by a suitably qualified independent environmental consultant at the proponent’s expense.

A Water Quality Monitoring and Management Plan must be submitted with each subsequent development application for subdivision. All Water Quality Monitoring and Management Plans must be endorsed by Hunter Water prior to determination for each development application for subdivision.

- Subsequent development applications for subdivision of land is to be accompanied by detailed plans, in accordance with approved plans and Council’s Infrastructure specification, for road layout, including road dimensions, traffic impacts, intersection detail, public transport facilities and street tree detail.

- Subsequent development applications for subdivision of land is to be accompanied by a Construction Management and Staging Plan – including details for Bulk Earthworks, Construction and Environmental Management, Erosion and Sediment Control Plans.

- Subsequent development applications for subdivision of land is to be accompanied by heritage, Aboriginal and archaeological survey investigations.

- Development consent is not to be issued under subsequent applications for works in the Grahamstown Dam Drinking Water Catchment Area prior to the completion of the Stormwater Diversion Channel to protect Grahamstown Dam drinking water supply.

- Vegetation clearing as approved under this Development Consent, including underscrubbing and tree removal, is to occur in conjunction with Development Consent for subdivision under subsequent applications. Site clearing, including underscrubbing and tree removal, must not occur beyond the extent of land approved for development and/or subdivision works under relevant subsequent applications.

- Subdivision is to be completed in numerical stages in accordance with the approved Scope of Concept Approval - Proposed Stage 1 – Initial Subdivision Works – Staging Plan (DA-08-8.01. Rev. K).

- Future applications are to include easements / restrictions on the Plan of Subdivision for right of access and servicing to adjoining land holdings in the Kings Hill URA. Future applications must also acknowledge all existing easements and/or restrictions on the use of the land on the final plan of subdivision.

- Future applications for subdivision are to demonstrate the development has an appropriate flood access route with 5% AEP flood immunity to the Pacific Highway.

- Development Consent under subsequent applications is not to be issued for more than 250 lots unless completion of the Pacific Highway Interchange and Grahamstown Dam Stormwater Channel has occurred.

- The dedication or transfer of roads is to occur in accordance with the State Voluntary Planning Agreement executed on 25 October 2019, reference SVPA-2014-9939 and Transport NSW Advice, reference 2019/002430 dated 20 December 2019.

2.0 - Prior to the Commencement of Works for Stage 1

The following conditions of consent shall be complied with prior to any clearing or vegetation works commencing on the development site.

- Staging of Clearing – Clearing is to occur in accordance with the following:

- Development Consent for subdivision or subdivision works must first be granted prior to Partial Vegetation Clearing or Complete Vegetation Clearing on the land subject of the Application (as defined by Step 2 and 3 clearing in Section 3 of the RPS Vegetation Management Plan, 24 July 2020, reference PR130430) occurring in each respective stage.

- The approved Step 2 and Step 3 clearing must only occur in a manner consistent with the Development Consent to carry out development or subdivision, subject to the stormwater management devices being designed in accordance with the recommendations of the Northrop Engineers Stormwater Management Report (dated 11 December 2019, version G), and constructed within the drainage catchment prior to the Step 2 and Step 3 clearing.

- Works for the establishment and maintenance of fire trails and designated conservation maintenance tracks as outlined in the Biodiversity Management Plan (RPS, 24 July 2020, reference PR130430) are permitted to occur as part of Step 1 works.

- No tree thinning practices (outside of track maintenance/establishment) proposed under Step 1 is to occur until Development Consent to carry out development or subdivision is granted for the respective stage.

- Prior to each clearing ‘Phase’ as identified in the VMP, a detailed clearing plan (Clearing Method Statement (CMS)), must be prepared and approved by Council. The CMS must be prepared in accordance with the recommendations of the approved VMP and BMP as well as Section 4.4 and 4.5 of the ‘Port Stephens Technical Specification Vegetation’ (May 2014), and provide guidance for the progressive and staged clearing of vegetation within each phase. The CMS is required to identify threatened species breeding periods and specific clearing directions.

- Pre-clearance survey - A pre-clearance survey must be undertaken by a suitably qualified and experienced ecological consultant in accordance with the approved VMP and BMP under this consent. The fauna ecologist must inspect all native trees approved for removal prior to felling within the disturbance footprint and advise the site manager and tree clearing staff of any habitat potential and precautions necessary during tree felling. The following strategies need to be employed to mitigate the effect of this clearing on native fauna that occur on the site:

- The fauna ecologist must supervise the removal of all trees and provide Council with a pre-clearance survey report at the completion of tree felling works;

- The process of removal should be as per the ecologists’ directions and include shaking the tree 24 hours prior to removal and leaving the felled tree in situ for 48 hours to allow any undetected fauna in hollows to escape;

- If a threatened species is identified as occurring within a tree requiring removal, clearing activities must cease immediately and the animal provided a period of at least 24 hours to vacate the area. If the animal has not vacated the area within this time, a procedure for the relocation of the animal must be developed and implemented under the guidance of the project ecologist;

- Remove and modify hollows and nests from felled trees and re-establish these into suitable adjacent native vegetation within the site to provide compensatory habitat. This includes the mounting of hollows and nests on trees or poles, or on other buildings or structures. Re-use of hollows will be carried out in preference to the construction of artificial nesting boxes. The replacement ratio for retained tree hollows is 1:1;

- Where original hollows cannot be retained, artificial nesting boxes must be provided to provide compensatory habitat in accordance with Council's Nest Box Technical Specification; and

- In circumstances where native fauna is detected during the removal of habitat trees, clearing is to cease until a licensed wildlife carer or consultant relocates the fauna species.

Note: If the pre-clearance survey identifies any threatened species inhabiting a tree flagged for removal, all clearing works shall cease until a plan of management for the relocation of the species.

- Construction Environmental Management Plan (CEMP) – A Construction Environmental Management Plan (CEMP) is to be submitted to and approved by Council. The required CEMP must include the following:

- sequence and construction methodology and specify mitigating measures to ensure all works are carried out with minimal environmental impact in relation to project staging,

- waste management,

- noise and vibration management,

- traffic and transport management,

- soils and contamination,

- environmental management,

- water and hydrology,

- Aboriginal and non-aboriginal heritage,

- biodiversity,

- air quality and odour,

- bushfire,

- hazard and risk,

- rehabilitation,

- legislative requirements relating to environmental management of the construction works, and allocation of responsibilities for environmental management procedures.

- Stormwater Diversion Channel - Prior to the commencement of any form of clearing works in the Grahamstown Dam Drinking Water Catchment Area, the stormwater diversion channel is to be constructed.

Vegetation planting and enrichment works in the conservation areas can be undertaken, however no clearing of vegetation, either Partial or Complete Vegetation Clearing described as Step 2 and Step 3 in the RPS Vegetation Management Plan dated 24 July 2020, reference PR130430) is to occur within the development areas (Management Area) to the extent it is within the Catchment Area (as shown in Figure 1 of RPS Vegetation Management Plan dated 24 July 2020, reference PR130430) prior to completion of the stormwater diversion channel.

- Soil erosion and sediment control – A detailed soil erosion and sediment control management plan is to be submitted to and approved by Council for each stage of works. Each soil erosion and sediment control management plan must specifically include mitigation measures to prevent sediment and other pollutants from entering Grahamstown Dam and Irrawang Swamp.

Soil erosion and sediment controls must be implemented prior to works commencing on the site in accordance with ‘Managing Urban Stormwater – Soils and Construction ('the blue book') and any Sediment and Erosion plans approved with this Development Consent.

Soil erosion and sediment control measures must be maintained during construction works and must only be removed upon completion of the project when all landscaping and disturbed surfaces have been stabilised (for example, with site turfing, paving or re-vegetation).

- Archaeology – Prior to the commencement of any works in the Conservation Area (as mapped in the SIS, July 2020), ground surveys and a Plan of Management is to be prepared for the areas of cultural significance identified in the conservation area, including the Kings Hill ridgeline, caves and rock outcrops as identified in the Archaeological Report prepared by Myall Coast Archaeological Services. The Plan of Management is to be submitted to Council for approval.

The Plan of Management and areas identified as being culturally significant are to be protected and managed by a positive covenant/restriction under Section 88B of the Conveyancing Act 1919. Council is to be nominated as the only authority who can modify this positive covenant/restriction.

No clearing or works for maintenance tracks are to occur in the areas of cultural significance unless consistent with the approved Management Plan.

- Site is to be secured – The site must be secured and fenced to the satisfaction of Council prior to commencement of works. All hoarding, fencing or awnings (associated with securing the site during construction) is to be removed upon the completion of works.

- All weather access – A 3m wide all-weather vehicle access is to be provided from the kerb and gutter to the site for use by trades people and contractors.

No materials, waste or the like are to be stored on the all-weather access at any time.

- Protection of trees /existing street trees – Protection of trees to be retained must be in accordance with AS490 ‘Protection of Trees on Development Sites’.

- Weed management – Weed removal and suppression must be undertaken using approved bush regeneration techniques under the supervision of a suitably qualified and approved bush regenerator and in accordance with the requirements for the NSW Biosecurity Act 2015, associated Regulations and NSW Weed Control Handbook.

The site is to be inspected by a representative of Council’s Invasive Species team one week prior to works commencing.

- Public liability insurance – The owner or contractor must take out a Public Liability Insurance Policy with a minimum cover of $20 million in relation to the occupation of, and works within, public property (i.e. kerbs, gutters, footpaths, walkways, reserves, etc.) for the full duration of the proposed works.

Evidence of this Policy must be provided to Council.

- Rubbish generated from the development – Where not already available, a waste containment facility is to be established on site. The facility is to be regularly emptied and maintained for the duration of works.

No rubbish must be stockpiled in a manner which facilitates the rubbish to be blown or washed off site. The site must be cleared of all building refuse and spoil immediately upon completion of the development.

- Flood Evacuation Plan – A flood evacuation plan indicating that permanent, fail-safe, maintenance free measures to ensure that timely, orderly and safe evacuation of people and potential pollutant material from the site should a flood occur. Details demonstrating compliance must be provided to Council.

3.0 - During Works

The following conditions of consent shall be complied with during the construction phase of the development.

- Staging of Stage 1 Works – Clearing is to be completed in stages in accordance with the approved Staging Plan (DA-08-8.01. Rev. K., Northrop dated 22 September 2020), approved Vegetation Management Plan and Biodiversity Management Plan (RPS, reference PR130430, dated 24 July 2020) and conditions of this consent. No clearing is to occur on E2 zoned land except as otherwise permitted by the approved plans (such as tracks, trails, roads and servicing corridors).

- Construction hours – All work (including delivery of materials) must be restricted to the hours of 7.00am to 5.00pm Monday to Saturday inclusive. Work is not to be carried out on Sundays or Public Holidays.

- Toilet facilities – Temporary toilet(s) must be provided and maintained on site from the time of commencement of construction work to completion. The number of toilets provided must be one toilet per 20 persons or part thereof employed on the site at any one time.

The temporary toilet is to be either connected to the sewerage system or an approved septic tank or otherwise may be a chemical toilet supplied by a licensed contractor.

- Vegetation and Biodiversity Management – All works must be undertaken in accordance with the approved Biodiversity Management Plan and Vegetation Management Plan prepared by the RPS Group (dated 24 July 2020, reference PR130430). Flora and Fauna Monitoring Reports must be prepared and submitted to Council every three months during clearing works.

- Ecological Burns - Prior to undertaking any strategic ecological burns within the conservation area as identified within the approved BMP, a detailed burn plan must be prepared and approved by Council.

- Location of stockpiles – Stockpiles of materials or waste must not be located on / near any drainage lines or easements, natural watercourses or water bodies, footpath or roadway without first providing suitable protective measures adequate to protect these water bodies.

All stockpiles must be suitably covered to prevent dust and odour nuisance. No contaminated materials are to be stored on / near any drainage lines or easements, natural watercourses or water bodies.  All stockpiles of contaminated materials must be suitably covered to prevent dust and odour nuisance. No off-site release of soil or other materials via water or land is permitted.

- Truck wash down – Provision must be made for all trucks to be effectively washed down after loading and unloading, prior to leaving the site. This must be within a suitably contained and designated area.

- Tree protection measures – All trees to be retained must be protected in accordance with AS4970 ‘Protection of Trees on Development Sites’ for the duration of works.

- Soil, erosion, sediment and water management – All requirements of the Erosion and Sediment Control Plan or Soil and Water Management Plan must be maintained at all times during the works and any measures required by the plan must not be removed until the site has been stabilised.

Monitoring reports every three months are to be submitted to Council that demonstrate the erosion and sediment control measures are adequately implemented.

- Offensive noise, dust, odour and vibration – All work must not give rise to offensive noise, dust, odour or vibration as defined in the Protection of the Environment Operations Act 1997 when measured at the nearest property boundary.

- Construction Environmental Management Plan implementation - All construction environmental management procedures and systems identified in the approved Construction Environmental Management Plan must be introduced and maintained during works.

Monitoring reports every three months are to be submitted to Council that demonstrate the CEMP measures are being adequately implemented.

- Weed management – All machinery that has operated in affected areas shall be cleaned thoroughly prior to entering and leaving the site. A wash down area shall be established, and subsequently monitored for the presence of Biosecurity Matter. Cleaning must include the removal of all mud and plant matter, followed by washing with high pressure water.

An area for storage of contaminated soil that is separate from clean material shall be provided during construction.

- Unexpected finds contingency (general) – Should any suspect materials (identified by unusual staining, odour, discolouration or inclusions such as building rubble, asbestos, ash material, etc.) be encountered during any stage of works (including earthworks, site preparation or vegetation works, etc.), such works must cease immediately until a qualified environmental specialist has be contacted and conducted a thorough assessment.

In the event that contamination is identified as a result of this assessment and if remediation is required, all works must cease in the vicinity of the contamination and Council must be notified immediately.

Where remediation work is required, the applicant will be required to obtain consent for the remediation works.

Advice Note(s):

(1)	'Dial Before you Dig' – Before any excavation work starts, contractors and others should phone the “Dial Before You Dig” service to access plans/information for underground pipes and cables.

(2)	Aboriginal archaeological objects – In the event of any Aboriginal object being unearthed, all work must cease immediately in the affected area, and the Biodiversity Conservation Division (BCD) shall be informed of the discovery. Work must not recommence until the material has been inspected and permission has been given by BCD to proceed.

(3)	Flood information is subject to change – You are advised that flood information is subject to change if more accurate data becomes available to Council. It is the responsibility of the applicant to use the most up-to-date flood information. Prior to the lodgement of subsequent applications, Council should be contacted to verify the currency of the flood information.

(4)	Works near/adjoining electricity network assets – There are underground electricity network assets adjacent to the proposed development. Any works undertaken adjacent to Ausgrid underground assets must be undertaken with care in accordance with Ausgrid Network Standard Document NS 156 – Work Near or Around Underground Cables.

(5)	Council must be nominated as PCA for subdivision works – Under Section 6.5 of the Environmental Planning & Assessment Act 1979, Council shall be nominated as the Principal Certifying Authority for subdivision work and has the option of undertaking inspection of physical construction works.

(6)	Responsibility for damage for tree removal/pruning – The applicant is responsible for any damage caused to existing public utilities, footpaths or public roads during the cutting down, grinding, removal and disposal of the timber and roots. Care must also be taken by the applicant and the applicant’s agents to prevent any damage to adjoining properties. The applicant or the applicants’ agent may be liable to pay compensation to any adjoining owner if, due to tree works, damage is caused to such adjoining property.

(7)   Development permitted by this Consent has been assessed and determined under the provisions of the TSC Act through the Biodiversity Conservation (Savings and Transitional) Regulations 2017. This means that the impact on listed threatened species, populations and ecological communities and their habitats has been properly assessed and requires no further consideration in the assessment of subsequent Development Applications, provided they are generally consistent with this Consent.

(8)   If approval under the Environment Protection and Biodiversity Conservation Act 1999 is required, Step 2 and Step 3 (as defined by Step 2 and 3 clearing in Section 3 of the RPS Vegetation Management Plan, 24 July 2020) clearing is not to occur until the respective approval is obtained from the relevant Commonwealth authority.