| CONSENT CONDITIONS – SOLAR FARMS NORTHERN REGIONAL PLANNING PANEL |  |
| --- | --- |

| PANEL REFERENCE & DA NUMBER | PPSNTH-128 - DA2021/47 |
| --- | --- |
| PROPOSAL | Erection of a 5MW Solar Farm |
| ADDRESS | Lot 2 DP 707260 and Lot 1 DP 705438 – Rosedale Road Ashley |
| APPLICANT | SLR Consulting Australia Pty Ltd |
| APPLICATION TYPE | Development application |

PART A: 	GENERAL

A1	Approved Plans and Supporting Documentation

The development must be implemented in accordance with the approved plans, specifications and supporting documentation listed below which have been endorsed by Council’s approved stamp, except where amended by conditions of this consent:

| Plan/Report Title | Reference No | Revision | Prepared by | Date |
| --- | --- | --- | --- | --- |
| Development Plans | 10160-G-GAD-01-1 | B | Providence Asset Group | 07/05/2021 |
| Statement of Environmental Effects | 631-30115-01 | -v0.3 | SLR | July 2021 |
| Stormwater Management Plan | 210748 | B | DRB Consulting Engineers | 13 May 2021 |
| Traffic Impact Assessment | 21/031 | E | Intersect Traffic | 21 May 2021 |
| Visual Impact Assessment | 631.30115 | -v1.0 | SLR | June 2021 |
| Acoustic Impact Assessment | MAC20118607RP1 | Final | Muller Acoustic Consulting | 29 April 2021 |

In the event of any inconsistency between conditions of this approval and the drawings/documents referred to above, the conditions prevail. In the event of any inconsistency between the approved plans and the supporting documentation, the approved plans prevail.

Reason: To ensure the development proceeds in the manner assessed by Council and all parties are aware of the approved plans and supporting documentation that applies to the development.

A2	Signage

A separate application shall be submitted to Council prior to the erection of any signage unless the proposed signage is ‘exempt development’ under State Environmental Planning Policy (Exempt and Complying Development Codes) 2008 or any other applicable environmental planning instrument.

Reason: To ensure any signage is assessed in accordance with the planning controls.

A3	Compliance with the Building Code of Australia

All building work shall be carried out in accordance with the provisions of the Building Code of Australia. A reference to the Building Code of Australia is a reference to that Code as in force on the date the application is made for the relevant construction certificate.

Reason: To ensure the building work complies with the Building Code of Australia.

A4	Lapsing of consent

This consent is limited to a period of 5 years from the date of the Notice of Determination unless the works associated with the development have physically commenced.

Reason: To ensure compliance with Section 4.53 of the Environmental Planning and Assessment Act 1979

A5	Tree Retention and Removal

The Development Application does not identify any vegetation removal. Existing trees on the site are to be retained and protected from damage during work. A separate application shall be submitted to Council prior to any vegetation removal.

Reason: To protect trees on the site to be retained.

A6	Requirements of Government Agencies

The following requirements have been imposed by government agencies:

- Essential Energy submission dated 21 October 2021

- NSW Rural Fire Service submission dated 18 October 2021

Reason: To ensure compliance with Section 4.47(3) of the Environmental Planning and Assessment Act 1979 and the conditions of Government agencies are complied with.

PART B: 	PRIOR TO THE ISSUE OF ANY CONSTRUCTION CERTIFICATE

B1	Construction Certificate

A Construction Certificate is required for the development in accordance with Section 6.7(1) of the Environmental Planning and Assessment Act 1979.

Reason: To ensure compliance with the Environmental Planning and Assessment Act 1979.

B2	Utility Services

Prior to the commencement of work the Applicant is to negotiate with the utility authorities in connection with the relocation and/or adjustment of the services affected by the development. Any necessary alterations to, or relocations of, utility services must be carried out at no cost to the council.

Reason: Protection of infrastructure

B3	Heavy Vehicle Permit for Local Roads

The applicant shall obtain a permit from the National Heavy Vehicle Regulator (NHVR) to utilise North Street, Heber Street and Rosedale Road for B- Double and /or Road Train trucks. This permit must be obtained prior to any such vehicles traversing these streets /roads.

Reason: Road use compliance

B4	Aboriginal Object Discovery

All site workers and personnel involved in site impact works associated with the proposal should be inducted and briefed on the possible identification of Aboriginal sites and objects during construction and their responsibilities according to the provisions of the NPW Act 1974, in case any additional unknown objects or items are uncovered during proposed works.

Reason: To preserve historic Aboriginal objects

B5	Payment of Security Deposits, Levies and Contributions

The fees listed in the table below must be paid in accordance with the conditions of this consent and Council’s adopted Fees and Charges applicable at the time of payment. Payments must be made prior to the issue of the Construction Certificate or prior to the commencement of work (if there is no associated Construction Certificate).

- Payment of building and construction industry long service levy - The applicant must pay the long service levy as calculated at the date of this consent to the Long Service Corporation or Council under Section 34 of the Building and Construction Industry Long Service Payments Act 1986 and provides proof of this payment to the certifier.

- Public liability insurance - Prior to the commencement of any works on Council land including a public road, the applicant is to obtain Public Liability Insurance in the minimum amount of $20 million. This insurance is to note Council’s interest and is to remain current for at least the period from the issue of the Construction Certificate until the issue of a Compliance Certificate/Occupation Certificate for the works. Documentary evidence of the Certificate of Currency is to be provided to Council prior to the issuing of any Construction Certificate for access.

- Payment of development contributions – A total monetary contribution of $76,344.62 is to be paid to Council, pursuant to Section 94A of the Environmental Planning and Assessment Act 1979, prior to the issue of any Construction Certificate.

- This condition is imposed in accordance with the provisions Section 94A Development Contributions Plan (Adopted by Moree Plains Shire Council on Friday 5th May 2006, as amended on 7 January, 2016) A copy of the document is available on Council’s website at

https://www.mpsc.nsw.gov.au/index.php/165-development-applications/planning-controls/777-local-infrastructure-contributions]

or may be inspected at Council’s Administration Building.

- The amount of contribution payable under this condition has been calculated at the date of consent. In accordance with the provisions of the Contributions Plan, this amount shall be indexed at time of actual payment in accordance with the Consumer Price Index. Indexation of contributions for payment occurs quarterly. Any party intending to act on this consent should contact Council prior to the date of payment to determine the indexed amount payable.

Reason: To ensure payments are made in accordance with legislative requirements.

B6	Construction Site Management Plan

Prior to the issue of a Construction Certificate, the applicant must ensure a construction site management plan is prepared before it is provided to and approved by the certifier. The plan must include the following matters:

- Sediment and Erosion Control Plan

- Construction Traffic Management Plan

  - Heavy vehicle access not permitted on Rosedale Road during school Bus commuting hours. Heavy vehicle movements will be restricted to:

Before 7am

9am to 2.30pm

After 5pm

Monday to Friday.

  - Directional signage

  - Truck movements to and from the site and estimated frequency

- Location and materials for protective fencing and hoarding to the perimeter on the site

- Provisions for public safety

- Pedestrian and vehicular site access points and construction activity zones

- Protective measure for on-site tree preservation (including in accordance with AS4970-20009 Protection of trees on development sites

- Details of any bulk earthworks to be carried out

- Location of site storage areas and sheds

- Equipment to carry out all works

- Complaints management mechanism including phone and email contacts and a complaints register.

- Waste Management Plan

- Noise management Plan in accordance with the recommendations made Noise Impact Assessment (Muller Acoustic Consulting, 2021)

Reason: To require details of measures that will protect the public and the surrounding environment during site works and construction.

B7	Other Approvals

The following approvals are required:

- Roads Act 1993 approval - The applicant is to submit an application to Council for any work within the road reserve (e.g. vehicular footpath crossings, utilities including stormwater, footpath paving, kerb and gutter etc) for local and regional roads or Transport for NSW for state roads, pursuant to Section 138 of the Roads Act 1993. Details must be provided with the Construction Certificate application.

B8	Servicing

In relation to services, the following must be undertaken by the applicant where relevant:

- Adequate services – the applicant must demonstrate that adequate services for water, electricity and wastewater are available to the site,

- Dial before your dig - the applicant shall contact “Dial Before You Dig on 1100” to obtain a Service Diagram. The sequence number obtained from “Dial Before You Dig” shall be forwarded to the Principal Certifying Authority (PCA) and Council for their records.

Details shall be provided with the Construction Certificate application.

Reason: To ensure work is carried out having regard to existing services and underground infrastructure for safety and efficiency.

B9	Stormwater Management

Final design plans of the stormwater drainage systems, prepared by a qualified practicing professional and in accordance with the requirements of council shall be submitted to the certifier prior to issue of a Construction Certificate. The hydrology and hydraulic calculations shall be based on models described in the current edition of Australian Rainfall and Runoff

The development must not result in the diversion of overland surface waters onto adjoining properties and where necessary shall construct appropriate surface drainage systems that connect to Council’s stormwater system.

Reason: To ensure stormwater is appropriately managed on the site.

B10	Bush fire Management

Where the site is identified as bush fire prone land, the solar farm shall comply with the relevant specifications and requirements of Planning for Bushfire Protection (2019) prepared by the NSW Rural Fire Service as outlined in the RFS General Terms of Approval dated 18 October 2021.

- Details are to be demonstrated on the Construction Certificate plans.

- Bush Fire Emergency Management and Operations Plan is to be prepared prior to the issue of a Occupation Certificate.

Reason: To ensure the development complies with the relevant requirements for bushfire management and to protect the safety of occupants on the site.

B11	Vehicle Access Requirements

The applicant shall be responsible for lodging a Driveway Access Application with Council’s Engineering Department for access to Rosedale Road. The Driveway Access Application must be approved by Council’s Engineering Department prior to the issue of a Construction Certificate. The design and construction of the driveway access shall be in accordance with Council’s specification in the Driveway Access Policy.

Entry and exit points are to be clearly signposted and visible from both the street and the site at all times. All required works are to be completed in accordance with Council/Transport for NSW standards prior to the commencement of construction works on the site

Heavy vehicle access not permitted on Rosedale Road during school Bus commuting hours. Details must be provided in the Construction Management Plan.

Reason: To ensure safe, practical and legal vehicle access is provided to the site.

B12	Directional Signage

The vehicular entrance and exit driveways and the direction of traffic movement within the site shall be clearly indicated by means of suitable signs. Directional signs shall be installed prior to the issue of a Final Occupation Certificate.

Reason: Safety and access

B13	Reflection Mitigation Measures

To ensure reflection and glare from the solar panels is minimised, the following shall be implemented:

- Installation of the measures outlined in the Glint and Glare report prepared by SLR dated June 2021;

Panels are left in either a horizontal or slight eastwards tilt if they need to be left in a fixed position during construction, maintenance, etc.

- A separate application shall be submitted to Council for night time illumination. This Development Consent does not include night time illumination.

Details are to be shown on the Construction Certificate plans.

Reason: To ensure the solar panels do not cause a nuisance, disturbance or hazard to adjoining properties, nearby aircraft and the public using nearby public roads from glint and glare.

B14	Car Parking

All driveways, access ramps, vehicular crossings and car parking spaces shall be designed and constructed in accordance with the current version of Australian Standards, AS 2890.1-2004: Parking Facilities – Off-street Car Parking and AS 2890.2:2018 - Parking facilities Off-street commercial vehicle facilities. All car parking for the operation of the solar farm is to be provided on an all-weather surface and line marked consistent with the approved development plans. Details are to be provided with the Construction Certificate application.

Reason: To ensure driveways, access ramps, vehicular crossings and car parking complies with the relevant Australian Standards.

B15	Flood Prone Land

A report shall be provided to certify the following:

- Any damage to the proposed development sustained in a flood will not generate debris capable of causing damage to downstream buildings or property;

- The building/structure will be able to withstand the force of flood waters (including buoyancy forces) and the impact of debris; and

- Finishes, plant fittings and equipment subject to inundation will be of materials and functional capability resistant to the effects of flood waters.

Reason: To ensure structural works are safe and built to the relevant specifications.

B16	Dilapidation report

Before the issue of a Construction Certificate, a suitably qualified engineer must prepare a dilapidation report detailing the structural condition of roads and public land, to the satisfaction of the certifier.

Any damage caused to other properties during construction shall be rectified by the applicant. A copy of the dilapidation survey and an insurance policy that covers the cost of any rectification works shall be submitted to Council prior to the commencement of the demolition works. The insurance cover shall be a minimum of $10 million.

Reason: To ensure any damage caused by the development is documented prior to works commencing on the site.

B17	Footing System Requirements – General

The Applicant shall provide the Certifying Authority with detailed design drawings for the footing system certified by a practising structural engineer as compliant with the relevant sections of Part 3.2 ‘Footings and Slabs’ of the BCA prior to the issue of a Construction Certificate.

The footing system shall be designed for an ‘E-D’ (Extremely reactive clay sites which can experience extreme deep-seated ground movement from moisture changes) class site in accordance with Part 3.2.4 ‘Site classification’ of the BCA unless accompanied by a detailed Site Classification Report from an appropriately qualified and accredited professional.

Reason: Structural safety

PART C: 	PRIOR TO WORKS COMMENCING

C1	Appointment of Principal Certifying Authority

Prior to the commencement of work, the person having the benefit of the development consent and a Construction Certificate shall:

- Appoint a Principal Certifying Authority and notify the Council of the appointment (if Council is not appointed); and

- Notify Council of their intention to commence building work (at least 2 days’ notice is required).

Reason: To ensure compliance with the Environmental Planning and Assessment Act 1979.

C2	Signs on site

A sign must be erected in a prominent position on any site on which building work or demolition work is being carried out:

- showing the name, address and telephone number of the principal certifier for the work, and

- showing the name of the principal contractor (if any) for any building work and a telephone number on which that person may be contacted outside working hours, and

- stating that unauthorised entry to the work site is prohibited.

Any such sign is to be maintained while the building work or demolition work is being carried out, but must be removed when the work has been completed.

Reason: Prescribed Condition under Clause 98A(2) and (3) of the Regulation.

C3	Toilet Facilities during Construction

Toilet facilities must be provided on the work site during construction at the rate of one toilet for every 20 persons or part of 20 persons employed at the work site. Each toilet provided must:

- Be a standard flushing toilet, connected to a public sewer, or

- If connection to a public sewer is not available, to an on-site effluent disposal
system approved by the council, or

- A portable toilet.

Reason: To provide appropriate on-site amenities during demolition and construction work.

C4	Implementation of the Construction Site Management Plan and Waste Management Plan

The approved Construction Site Management Plan (incorporating the Sediment and Erosion Control Plan and Construction Traffic Management Plan) and the Waste Management Plan must be implemented and maintained prior to, and during, the construction woks on the site until works are completed and all exposed surfaces are landscaped/sealed.

Reason: To ensure measures that will protect the public, and the surrounding environment, during site works and construction are implemented prior to works commencing on the site.

PART D:	DURING WORKS

D1	Construction Hours

The hours of demolition and/or building work shall be limited to the following hours:

- Monday to Friday:  			7 am to 6pm;

- Saturday:  				8am to 1pm;

- No Construction on Sundays or Public Holidays.

Unless otherwise approved within the Construction Site Management Plan, construction vehicles, machinery, goods or materials must not be delivered to the site outside the approved hours of site works. Note: Any variation to the hours of work requires Council’s approval.

Reason: To ensure the amenity of the area is maintained during construction.

D2	Compliance with Construction Site Management Plan

The requirements of the approved Construction Site Management Plan must be complied with and maintained for the duration of the construction works. This includes, but is not limited to, the following:

- All practicable measures shall be undertaken to prevent and minimise harm to the environment and the amenity of the area as a result of the construction and operation of the development, particularly from wind-blown dust, debris, noise, erosion and soil management and the like.

- All building materials and equipment must be stored wholly within the site unless an approval to store them elsewhere has been granted.

- During construction, care must be taken to protect Council’s infrastructure, including street signs, footpath, kerb, gutter and drainage pits etc.

- Protection measures shall be maintained in a state of good and safe condition throughout the course of demolition.

- The area fronting the site and in the vicinity of the development shall also be made safe for pedestrian and vehicular traffic at all times.

The applicant must ensure a copy of these approved plans is kept on site at all times and made available to Council officers upon request.

Reason: To ensure the required site management measures are implemented during construction.

D3	Waste Management

While building work is being carried out, the principal certifier must be satisfied all waste management is undertaken in accordance with the approved waste management plan. Upon disposal of waste, the applicant is to compile and provide records of the disposal to the Principal Certifier, detailing the following:

- The contact details of the person(s) who removed the waste

- The waste carrier vehicle registration

- The date and time of waste collection

- A description of the waste (type of waste and estimated quantity) and whether the waste is expected to be reused, recycled or go to landfill

- The address of the disposal location(s) where the waste was taken

- The corresponding tip docket/receipt from the site(s) to which the waste is transferred, noting date and time of delivery, description (type and quantity) of waste.

Note: If waste has been removed from the site under an EPA Resource Recovery Order or Exemption, the applicant is to maintain all records in relation to that Order or Exemption and provide the records to the principal certifier and Council.

Reason: To require records to be provided, during construction, documenting that waste is appropriately handled.

D4	Responsibility for Changes to Public Infrastructure

While building work is being carried out, the applicant must pay any costs incurred as a result of the approved removal, relocation or reconstruction of infrastructure or services affected by the required construction works. Council and other service authorities should be contacted for specific requirements prior to commencement of any works.

Reason: To ensure payment of approved changes to public infrastructure.

D5	Discovery of Aboriginal Objects

While excavation, demolition or building work is being carried out, all such works must cease immediately if a relic or Aboriginal object is unexpectedly discovered. The applicant must notify the Heritage Council of NSW in respect of a relic and notify the Secretary of the Department of Planning, Industry and Environment and the Heritage Council of NSW in respect of an Aboriginal object. Building work may recommence at a time confirmed by either the Heritage Council of NSW or the Secretary of the Department of Planning, Industry and Environment.

In this condition:

- “relic” means any deposit, artefact, object or material evidence that:

- relates to the settlement of the area that comprises New South Wales, not being Aboriginal settlement, and

- is of State or local heritage significance; and

- “Aboriginal object” means any deposit, object or material evidence (not being a handicraft made for sale) relating to the Aboriginal habitation of the area that comprises New South Wales, being habitation before or concurrent with (or both) the occupation of that area by persons of non-Aboriginal extraction and includes Aboriginal remains.

Reason: To ensure the protection of objects of potential significance during works.

D6	Discovery of Contamination

Should any contaminated, scheduled, hazardous or asbestos material be discovered before or during construction works, the applicant and contractor shall ensure the appropriate regulatory authority is notified and that such material is contained, encapsulated, sealed, handled or otherwise disposed of to the requirements of such Authority.

Reason: To ensure contamination discovered during construction is dealt with as quickly as possible and to protect the health of the community and the environment.

D7	Construction Noise

While building work is being carried out, and where a noise and vibration management plan is approved under this consent as part of the Construction Site Management Plan, the applicant must ensure that any noise generated from the site is controlled in accordance with the requirements of that plan.

Reason: To protect the amenity of the neighbourhood.

D8	Imported Fill

While construction work is being carried out, the principal certifier must be satisfied all soil removed from or imported to the site is managed in accordance with the following requirements:

- All excavated material removed from the site must be classified in accordance with the EPA’s Waste Classification Guidelines before it is disposed of at an approved waste management facility and the classification and the volume of material removed must be reported to the principal certifier,

- All fill material imported to the site must be Virgin Excavated Natural Material as defined in Schedule 1 of the Protection of the Environment Operations Act 1997 or a material identified as being subject to a resource recovery exemption by the NSW EPA.

Reason: To ensure soil removed from the site is appropriately disposed of and soil imported to the site is safe for future occupants.

D9	Critical Stage Inspections

Building work must be inspected on the occasions set out in clause 162A (Critical stage inspections for building work) under the Environmental Planning and Assessment Regulation 2000.

Reason: To require approval to proceed with building work following each critical stage inspection and comply with the Regulation.

D10	Tree Protection

While site or building work is being carried out, the applicant must maintain all required tree protection measures in good condition in accordance with the Construction Site Management Plan required under this consent, the relevant requirements of AS 4970-2009 Protection of Trees on Development Sites and any Arborist’s report approved under this consent.

This includes maintaining adequate soil grades and ensuring all machinery, builders refuse, spoil and materials remain outside tree protection zones.

Reason: To protect trees during construction.

D11	Native Vegetation

There must be no removal or disturbance of native vegetation except as authorised by this consent, including canopy trees, understorey and ground cover vegetation without the prior written consent of Council.

Reason: To ensure vegetation is maintained on the site.

D12	Clearing for Asset Protection Zones (APZ)

While building work is being carried out, the applicant must ensure the clearance of vegetation to establish the APZ is confined to within the marked APZ boundary, to the satisfaction of the principal certifier.

Reason: To ensure vegetation clearance during construction is confined within the APZ.

D13	Water Pollution

The applicant must ensure that the development does not cause any water pollution, as defined under Section 120 of the Protection of the Environment Operations Act 1997.

Reason: To ensure the development does not cause any water pollution during construction.

PART E: 	PRIOR TO THE ISSUE OF THE OCCUPATION CERTIFICATE

E1	Occupation Certificate

Occupation and operation of the solar farm is not to occur until all work has been completed, all of the conditions of consent have been satisfied and an Occupation Certificate has been issued by the Principal Certifying Authority pursuant to Section 6.10 of the Environmental Planning and Assessment Act 1979.

Reason: To ensure compliance with the Environmental Planning and Assessment Act 1979.

E2	Completion of Public Utility Services

Before the issue of the relevant Occupation Certificate, the principal certifier must ensure any adjustment or augmentation of any public utility services including gas, water, sewer, electricity, street lighting and telecommunications, required as a result of the development, and is completed to the satisfaction of the relevant authority. Before the issue of the Occupation Certificate, the certifier must request written confirmation from the relevant authority that the relevant services have been completed.

Reason: To ensure required changes to public utility services are completed, in accordance with the relevant agency requirements, before occupation.

E3	Completion of all Works

All works must be completed in accordance with the conditions of this consent prior to the issue of an Occupation Certificate including, but not limited to, the following

- Car parking areas, directional signage and vehicle access points;

- Fencing;

- Bushfire Protection Measures and standards as outlined in this consent shall be installed and completed;

- All drainage works required to be undertaken in accordance with this consent shall be completed prior to the issue of an Occupation Certificate for the development.

Following any construction or upgrading on site, the applicant must restore the ground cover of the site as soon as practicable, using suitable species and maintain ground cover.

Reason: To ensure adequate arrangements have been made for the development.

E4	Post-Construction Dilapidation Report and Repair of Infrastructure

Before the issue of an Occupation Certificate, a suitably qualified engineer must prepare a post-construction dilapidation report which must include a photographic survey, to the satisfaction of the principal certifier, detailing whether:

- any roads and/or public infrastructure have been damaged as a result of the carrying out of construction works (including damage caused by, but not limited to, delivery vehicles, waste collection, contractors, sub-contractors, concreting vehicles) and if so, it is to be fully repaired to the written satisfaction of Council, and at no cost to Council.

Before the issue of an Occupation Certificate, the Principal Certifier is to provide a copy of the post-construction dilapidation report to Council (where Council is not the principal certifier).

Reason: To identify damage to adjoining properties resulting from building work on the development site.

E5	Removal of Waste upon Completion

Before the issue of an Occupation Certificate, the Principal Certifier must ensure all refuse, spoil and material unsuitable for use on-site is removed from the site and disposed of in accordance with the approved waste management plan. Written evidence of the removal must be supplied to the satisfaction of the Principal Certifier. Before the issue of a partial Occupation Certificate, the applicant must ensure the temporary storage of any waste is carried out in accordance with the approved Waste Management Plan to the Principal Certifier’s satisfaction.

Reason: To ensure waste material is appropriately disposed or satisfactorily stored

E6	Electricity Grid Connection

The Applicant shall enter into a Connection Agreement for the development with Essential Energy. The Connection Agreement must be in place prior to the issue of an Occupation Certificate.

Reason: Electricity grid connection process

PART F: 	OPERATIONAL CONDITIONS

F1 	Emergency Management Plan

The approved Emergency Management Plan (‘EMP’) for the development shall be activated when required throughout the operation of the development and shall be updated if required.

During occupation of the development, the applicant must ensure the site is managed, in accordance with Planning for Bushfire Protection 2019 and the NSW Rural Fire Service’s document Standards for Asset Protection Zones. Any required bushfire protection measures are to be maintained throughout the operation of the solar farm at the site.

Reason: To ensure the protection of human life, the environment and adjoining property in the event of fire or other emergency generated by the development.

F2	Vehicle Management

The premises shall be operated in accordance with the following vehicle management requirements:

- All loading and unloading of vehicles must be undertaken wholly within the site and all vehicles must enter and leave the site a forward direction.

- Sufficient car parking is to be provided on the site, with no car parking to occur on the public road network in the vicinity of the site.

- The vehicle entry and exit points are to be clearly signposted and visible from both the street and the site at all times and must be maintained in good condition for the life of the development.

Reason: To ensure the operation of the solar farm does not adversely affect the surrounding road network and has adequate car parking provided on the site.

F3	Operation of Plant and Equipment

The Applicant must ensure that all plant and equipment used on the site or in connection with the development is maintained in a proper and efficient condition and operated in a proper and efficient manner.

Reason: To ensure all plant and machinery on the site is in good working order and is operated in a safe manner.

F4	Minimise Harm to the Environment

The applicant must implement all reasonable and feasible measures to prevent and/or minimise any material harm to the environment or to the amenity of the area that may result from the construction, operation or decommissioning of the development, including weed control.

Reason: To minimise harm to the environment.

F5	Visual Impacts

The applicant must:

- Minimise the off-site visual impacts of the development, including the potential for any glare or reflection from the solar panels;

- Ensure the visual appearance of all ancillary infrastructure (including paint colours) blends in as far as possible with the surrounding landscape. Material and colour choice are to be non-reflective muted tones; and

- Not mount any advertising signs or logos on site, except where this is required for safety purposes.

Reason: To minimise the visual impact of the development.

F6	Storage of Hazardous Materials

The applicant must store and handle all dangerous and hazardous materials on site in accordance with AS 1940-2004: The storage and handling of flammable and combustible liquids. The storage of any dangerous and hazardous materials must be provided in a suitably bunded and impervious area and in such a way as to minimise spills of hazardous materials or hydrocarbons. Clean up any spills must occur as soon as possible.

Reason: To minimise harm to the environment.

F7	Maintenance of Stormwater Treatment Device

During occupation and ongoing use of the solar farm, the applicant must ensure all stormwater treatment devices (including drainage systems, sumps and traps, and on-site detention) are regularly maintained, to remain effective and in accordance with any positive covenant (if applicable).

Reason: To protect stormwater systems.

F8	Noise Control During Operation

Any noise generated from the operation of the solar farm, including noise from any substation and associated infrastructure, must not be intrusive or constitute offensive noise as defined by the Protection of the Environment Operations Act 1997 at any private residential receiver.

The operation of the solar farm must satisfy the EPA maximum noise criteria pursuant to the EPA’s Noise Policy for Industry (2017). If, at any time, these levels are exceeded, operation of the solar farm shall immediately be modified, including suspension of operations if necessary, to ensure compliance.

Reason: To protect the amenity of the area while the solar farm is in operation.

F9	Waste Materials

The Applicant must:

- minimise the waste generated by the development;

- classify all waste generated on site in accordance with the NSW EPA Waste Classification Guidelines, Part 1: Classifying Waste, November 2014 (or its latest version);

- store and handle all waste on site in accordance with its classification;

- not receive or dispose of any waste on site; and

- remove all waste from the site as soon as practicable, and ensure it is sent to an appropriately licensed waste facility for disposal.

Reason: To protect the environment.

GENERAL TERMS OF APPROVAL

Insert Essential Energy’s response

Insert RFS GTAs