A2026/42401

| Attachment A – Conditions of Consent NORTHERN REGIONAL PLANNING PANEL |  |
| --- | --- |

| PANEL REFERENCE & DA NUMBER | PPSNTH-427 – BYRON – 10.2025.212.1 |
| --- | --- |
| PROPOSAL | Amended DA - Demolition of public toilets (heritage item), car park and tree removal. Affordable housing mixed use development comprising shop top housing (28 apartments above ground level), one ground floor commercial premises and ground floor boarding house, carparking, landscaping and associated services |
| ADDRESS | Lots 6, 7 & 8, Section 3 DP 2772 – 57 Station Street Mullumbimby |

CONDITIONS OF CONSENT

| - Schedule |  | - CONDITIONS OF CONSENT |
| --- | --- | --- |
| Parameters of consent |  |  |
|  | - Approved plans and supporting documentation Development must be carried out in accordance with the following approved plans and supporting documentation (stamped by Council), except where the conditions of this consent expressly require otherwise. Title; Plan No.; Drawn by; Dated. Cover Page; A0.01, Rev D; Architect Hill Thalis; 28/09/2026. Site Controls and Calculations; A1.04, Rev C; Architect Hill Thalis; 28/09/2026. Demolition Plan; A2.01, Rev A; Architect Hill Thalis; 26/03/2025. Ground Floor Plan; A2.02, Rev D; Architect Hill Thalis; 28/09/2026. Level 1 Floor Plan; A2.03, Rev C; Architect Hill Thalis; 28/09/2026. Level 2 Floor Plan; A2.04, Rev C; Architect Hill Thalis; 28/09/2026. Roof Plan; A2.05, Rev B; Architect Hill Thalis; 28/05/2026. Elevations; A2.21, Rev D; Architect Hill Thalis; 28/09/2026. Elevations; A2.22, Rev D; Architect Hill Thalis; 28/09/2026. Sections; A2.31, Rev D; Architect Hill Thalis; 28/09/2026. 11.7m Height Plane; A5.01, Rev B; Architect Hill Thalis; 28/05/2026. Materials; A6.01, Rev C; Architect Hill Thalis; 28/05/2026. C-DA 010 P6; Civil Services Site Layout Plan; JHA; 2/6/26. C-DA 050 P5; Civil Services Erosion and Sediment Control Plan; JHA; 2/6/26. C-DA 100 P5; Civil Services Ground Floor Drainage Plan; JHA; 2/6/26. 240430; Additional Flood Assessment; JHA; 12/6/26. In the event of any inconsistency between the approved plans and the supporting documentation, the approved plans prevail. In the event of any inconsistency between the approved plans and a condition of this consent, the condition prevails. Note: an inconsistency occurs between an approved plan and supporting documentation or between an approved plan and a condition when it is not possible to comply with both at the relevant time. |
|  | - Payment of security deposits Before the commencement of any works on the site or the occupation of the building, a security deposit (bond) is to be paid to Council as guarantee against damage to surrounding public land and infrastructure during construction of the proposed development. Evidence is to be provided to Council indicating the predevelopment condition of the surrounding public land and infrastructure. Such evidence must include photographs. The proponent will be held responsible for the repair of any damage to roads, kerb and gutters, footpaths, driveway crossovers or other assets. The applicant must make all the following payments to Council and provide written evidence of these payments to the certifier: Security deposit; $ 10,000. Inspection fee; In accordance with the current fees and charges.. The payments will be used for the cost of: - making good any damage caused to any council property (including street trees) as a consequence of carrying out the works to which the consent relates. - completing any public work such as roadwork, kerbing and guttering, footway construction, stormwater drainage and environmental controls, required in connection with this consent, and - any inspection carried out by Council in connection with the completion of public work or the making good any damage to council property. - Note: The inspection fee includes Council’s fees and charges and includes the Asset Inspection Fee (under the Roads Act 1993). The amount payable must be in accordance with council’s fees and charges at the payment date. The security deposit (bond) will be held until Council is satisfied that the infrastructure is maintained/repaired to pre development conditions and that no further work is to be carried out that may result in damage to Council’s roads, footpaths etc. |
|  | - Support for neighbouring buildings If an excavation extends below the level of the base of the footings of a building on an adjoining allotment of land, the person causing the excavation to be made or builder must: - Inform the neighbouring property owner immediately. - Engage a structural engineer to determine any remedial works that may need to be undertaken. - Preserve and protect the adjoining building from damage. - If necessary, underpin and support the building in an approved manner. |  |
|  | - Signage - Any signage that is not in accordance with State Environmental Planning Policy (Exempt and Complying Development Codes) 2008 will require separate consent. |  |
|  | - Conditions prescribed by the Regulation - This development consent is subject to the conditions prescribed by the regulations in accordance with subsection 4.17(11) of the Environmental Planning and Assessment Act 1979. Conditions are provided in a Schedule to this consent. |  |
|  | - Waste Management Waste Management on the site is to be in accordance with the Waste Management Plan, prepared by MRA Consulting Group and dated 29/6/26, with the exception that the following changes are made to the plan: - Amend Table 5, page 15 so that only “Timber (non-treated)” is reused or has recovery potential as treated timber is sent to landfill. - Amend Section 4.3 Signage so that the link/web address or reference is included for Standard signage requirements and guidance for application. |  |
|  | - Crime prevention - The development is to include the recommendations of the Crime Risk Assessment, dated April 2025, prepared by Wilson Planning. |  |
|  | - Silver LHA Dwellings and Adaptable Housing - Three (3) of the dwellings are to be constructed in accordance with AS4299:1995 Adaptable Housing; and six dwellings in the development are to satisfy the LHA Silver design requirements in accordance with the Access Report, prepared by iAccess Residential, dated 29/5/2026. |  |
|  | - Supply of Electricity to the Development Satisfactory arrangements must be made with Essential Energy for the provision of power with respect to the proposed development. It is the Applicant’s responsibility to make the appropriate application with Essential Energy for the supply of electricity to the development, which may include the payment of fees, contributions and if required, new designated electrical infrastructure. If it is deemed that designated electrical infrastructure is required, then all fees for such infrastructure will be borne by the Applicant. Refer Essential Energy’s Contestable Works Team for requirements via email contestableworks@essentialenergy.com.au. |  |
|  | - Acid Sulfate Soil Management Plan - Acid sulfate soils are to be managed in accordance with the Acid Sulfate Soil Management Plan prepared by Raw Earth Environmental, dated November 2024. |  |
|  | - Noise Management - Construction phase and operational phase noise is to be managed in accordance with the recommendations contained within the Noise Impact Assessment Report, prepared by JHA Services, dated 18/3/2025. |  |
|  | - Façade Colour Palate FC cladding/Hardiflex planks or similar are also to be used as a cladding material in conjunction with the proposed cladding. In this regard, level 1 and 2 of all elevations with balconies are to include FC cladding/ Hardiflex planks. A colour palette, consistent with the requirements of Byron Development Control Plan 2014 Chapter C1, is to be used for the FC cladding/Hardiflex planks. Where galvanised steel is proposed for wall cladding or roofing, it is not to be substituted with Zincalume as galvanised steel dulls faster than Zincalume cladding. |  |
|  | - Metal fences and gates – Station Street and McGoughans Lane - Metal fencing and gates used along Station Street and McGoughans Lane is to have a permeability of at least 60%. This will facilitate the flow of flood waters and enhance streetscape appeal. |  |
| The following conditions are to be complied with before certification that Crown building works comply with the National Construction Code |  |  |
|  | Certification that Crown building works comply with the National Construction Code Certification by a suitably qualified person is to be provided to the Crown prior to commencement of any building work on the site. |  |
|  | External lighting Before the certification that the Crown building works comply with the NCC, plans detailing external lighting must be prepared by a suitably qualified person. The lighting plan must be consistent with the approved plans and documents, and the following requirements: - comply with AS 1158: Lighting for Roads and Public Spaces; - comply with AS 4282: Control of Obtrusive Effects of Outdoor Lighting - lighting must be placed at all entrances to, and exits from the premises - lighting must provide coverage of the premises and surrounding areas for visibility and to reduce hidden areas; - lighting must not interfere with traffic safety; - lighting must not give rise to obtrusive light or have adverse impacts on the amenity of surrounding properties; and - external lighting must not flash or intermittently illuminate unless required for safe ingress/egress of vehicles crossing a pedestrian footway or approved vehicle entrance. Note: All above documents refer to the version in effect at the time the consent is granted. |  |
|  | Demolition of heritage listed public lavatory Before the certification that the Crown building works comply with the NCC the following matters are to be finalised: - The submission to Byron Shire Council of an archival report, prepared by a suitably qualified person, of the heritage listed public lavatory building located at 57 Station Street Mullumbimby; - The suitable arrangements are to be confirmed in writing with Byron Shire Council regarding the relocation of the entire roof of the public lavatory to a specified location for short term storage, or to a permanent site for repurposing (if approvals are in place). The roof is to be transported intact or in modular components and may require temporary bracing. This is to be installed, if needed, by a suitably qualified person without compromising the integrity of the structure. |  |
|  | Relocation of existing streetlight/s and powerlines Prior to the issue of any Crown Certificate, the Applicant is to engage a Level 3 Accredited Service Provider to prepare and obtain approval from Essential Energy for the relocation of existing streetlight/s and powerlines located adjacent to the boundary of the property that do not meet minimum safety clearance requirements. Refer Essential Energy’s policy CEOM7106.25. Minimum Clearance Requirements for NSW and ISSC 20 Guideline for the Management of Activities within Electricity Easements and Close to Infrastructure. All relocated works, including electrical and other utility services, must be undertaken in accordance with the requirements of the relevant service authority and infrastructure provider. |  |
|  | - Construction site management plan The applicant must ensure a construction site management plan is prepared before it is provided to and approved by the certifier. The plan must include the following matters: - location and materials for protective fencing and hoardings to the perimeter on the site - provisions for public safety - pedestrian and vehicular site access points and construction activity zones - details of construction traffic management, including proposed truck movements to and from the site and estimated frequency of those movements, and measures to preserve pedestrian safety in the vicinity of the site - protective measures for on-site tree preservation (including in accordance with AS 4970:2025 Protection of trees on development sites and Council’s DCP, if applicable) and trees in adjoining public domain (if applicable) - details of any bulk earthworks to be carried out - location of site storage areas and sheds - equipment used to carry out all works - a garbage container with a tight-fitting lid - dust, noise and vibration control measures - location of temporary toilets. The applicant must ensure a copy of the approved construction site management plan is kept on-site at all times during construction. |  |
|  | - Sediment and Erosion Control Management Plan required The application is to include plans and specifications that indicate the measures to be employed to control erosion and loss of sediment from the site. Control over discharge of stormwater and containment of run-off and pollutants leaving the site/premises must be undertaken through the installation of erosion control devices such as catch drains, energy dissipaters, level spreaders and sediment control devices such as filter fences and sedimentation basins. NOTE: The plans must be in compliance with Council's current “Northern Rivers Local Government Development Design & Construction Manuals and Standard Drawings”. |  |
|  | - Stormwater Management Plan – Connection to Existing Drainage System The application is to include plans and specifications for stormwater drainage in accordance with AS/NZS 3500.3:2003, Plumbing and drainage, Part 3: Stormwater drainage. All stormwater drainage for the development must be conveyed by a gravity system to the existing stormwater drainage system within the site in accordance with the approved plan. The following must be included in the plan: - - The applicant shall revise the stormwater drainage design to demonstrate a positive surcharge relief mechanism for pit SWP 04/01. The revised design shall provide a clearly defined overflow path, overflow weir, bypass drainage arrangement, or other suitable means of conveying surcharge flows in the event of blockage of the OG-200 basket, obstruction of downstream pipework, surcharge of the StormFilter treatment device, or backwatering from the downstream drainage system. The design shall demonstrate that major storm flows can be safely conveyed to a lawful point of discharge without causing adverse flooding, ponding, or inundation of buildings, pedestrian accessways, service infrastructure, or neighbouring land, and without adversely affecting the approved overland flow regime. Details shall be certified by a suitably qualified civil engineer. - The Stormwater Quality Improvement Device (SQID) Maintenance Plan be revised to include maintenance requirements for all treatment elements forming part of the approved MUSIC treatment train, including vegetated swales and associated drainage infrastructure. The revised plan should also include post-flood inspection and maintenance procedures, record keeping requirements, and details of the mechanism by which maintenance obligations will be transferred to future owners, operators or managing entities for the life of the development - An executed 10-year maintenance agreement (JHA – SQIDs Maintenance Plan and Ocean protect Agreement No 24386/1) for all approved Stormwater Quality Improvement Devices (SQIDs) be provided to Council for endorsement. The agreement shall be prepared by the system supplier or a suitably qualified maintenance provider and shall ensure the ongoing operation and maintenance of the StormFilter system, OceanGuard devices, vegetated swales and associated stormwater treatment infrastructure for the life of the development. Evidence of renewal of the agreement shall be retained by the asset owner and made available to Council upon request. - All fencing, gates, barriers and landscaping located within identified overland flow paths be designed and maintained so as not to impede flood conveyance, increase flood afflux, divert floodwaters onto adjoining properties or create debris accumulation hazards. Detailed design documentation should demonstrate that the flood modelling assumptions remain valid for the life of the development |  |
|  | - Consent required for works within the road reserve Consent from Council must be obtained for works within the road reserve pursuant to Section 138 of the Roads Act 1993. Three (3) copies of engineering construction plans must accompany the application for consent for works within the road reserve. Such plans are to be in accordance with Council’s current Design & Construction Manuals and are to provide for the following works: Roadworks Station St; - Road, footpath and landscape works generally in accordance with the approved plans. - The footpath must be designed to provide a cross fall of 1 % or 1:100 (maximum 2.5% or 1 in 40) to provide for pedestrians with access disabilities. - Works are to include the adjustment and/or relocation of services as necessary to the requirements of the appropriate service authorities and to ensure the services have suitable cover and are constructed flush with the finished surface levels. - Regulatory signage and line markings (note: details to be included on separate plan/s for submission to Local Transport Forum). - The Roads Act application is to include separate plans and specifications that indicate the landscaping works within the road reserve. The landscaping plan must be prepared by suitably qualified person/s and incorporate adequate detail to demonstrate compliance with the provisions of Chapter B9 Byron DCP 2014 and Water Sensitive Urban Design Guidelines. The street trees are to be small to medium sized endemic (to the north coast of NSW) rainforest trees with appropriate crown development for screening the development and shade for pedestrians and must not obstruct pedestrian or vehicle sight lines. The use of native palms not supported.. Roadworks McGoughans Ln; - The existing misaligned dish gutter along the McGoughans Lane frontage shall be removed and replaced with a continuous dish gutter extending from the existing grated inlet pit located south of the development site for the full length of the site frontage. The dish gutter shall be constructed to match the existing profile, line and grade, in accordance with Council's standards and specifications, and to the satisfaction of Council. - Carpark driveway: - The proposed driveway crossover to McGoughans Lane shall be constructed in accordance with the civil engineering plans approved under the consent, and the relevant Byron Shire Council standard drawings and specifications. - Road pavement widening and adjustment works shall be undertaken as necessary to provide a smooth transition between the existing pavement, proposed driveway crossing and drainage infrastructure, including the reconstructed dish gutter. Any failed, damaged or deteriorated pavement along the full frontage of the site shall be removed and reconstructed to the satisfaction of Council... |
|  | Public Safety Management Plan required Consent from Council must be obtained for a Public Safety Management Plan for those works within the road reserve pursuant to Section 138 of the Roads Act 1993. This public safety management plan is to include provision for (but not be limited to): - a pedestrian barrier, alternative footpaths and ramps as necessary; - an awning sufficient to prevent any substance from, or in connection with, the work falling into the road reserve; - lighting of the alternative footpath between sunset and sunrise; - the loading and unloading of building materials; - parking space for tradesman’s vehicles, where such vehicles must be located near the site due to tools and equipment contain within the vehicle; - Removal of any such hoarding, fence or awning as soon as the particular work has been completed. |  |
|  | Traffic Management Plan Consent from Council must be obtained for a Traffic Management Plan pursuant to Section 138 of the Roads Act 1993. The plans and specifications are to include the measures to be employed to control traffic (inclusive of construction vehicles) during construction of the development. The traffic management plan is to be designed in accordance with the requirements of the Roads and Traffic Authority’s Manual, Traffic Control at Work Sites Version 2, and the current Australian Standards, Manual of Uniform Traffic Control Devices Part 3, ‘Traffic Control Devices for Works on Roads’. The report must incorporate measures to ensure that motorists using road adjacent to the development, residents and pedestrians in the vicinity of the development are subjected to minimal time delays due to construction on the site or adjacent to the site. The traffic management plan must be prepared by a suitably qualified and RTA accredited Work Site Traffic Controller. |  |
|  | Car parking layout, vehicle circulation and access plans required. The application for a Crown Certificate is to include plans and specification that indicate access, parking and manoeuvring details in accordance with the plans approved by this consent. The access, parking and manoeuvring for the site is to comply with the requirements of AS 2890.1-2004: Parking facilities, Part 1: Off-street car parking and AS 2890.2 – 2010 - Parking facilities, Part 2: Off-street commercial vehicle facilities. Plans are to include, but not be limited to, the following items: - 19 parking spaces including 5 user class 4 (3-sole occupancy units, 1-boarding house and 1 commercial), 3 user class 3 and user 11 class 1A; - 1 SRV loading bay; - Wheel stops, other physical control, blind aisle in accordance with AS2890.1 - pavement design, comprising an all-weather surface, such as asphalt, bitumen seal, concrete, pavers or other similar treatment; - site conditions affecting the access; - existing and design levels; - longitudinal section from the road centreline to the car space(s); - cross sections every 15 metres; - drainage details; - turning paths; - line marking and signage; - Security gating that does not inhibit commercial car parking for employees and customers at the shop within the development. The engineering plans and specifications are to be designed by a qualified practising Civil Engineer. The Civil Engineer is to be a corporate member of the Institution of Engineers Australia or is to be eligible to become a corporate member and have appropriate experience and competence in the related field. NOTE: The plans must be in compliance with Council's current “Northern Rivers Local Government Development Design & Construction Manuals and Standard Drawings”. |  |
|  | EV parking spaces - The four EV parking spaces are to be Level 2 charging output as a minimum; - All EV charging infrastructure to be designed and installed in accordance with relevant Australian Standards and electrical regulations. |  |
|  | - Flood Planning Level The flood planning level for this development is 5.06m A.H.D. The plans and specifications to accompany the crown certificate application are to indicate a minimum habitable floor level that is at, or up to 0.3m above, the flood planning level unless a greater floor level has been shown on the plans approved by the development consent. The plans and specifications to accompany the crown certificate application are to also indicate: - a minimum floor level of 4.07m A.H.D for non-habitable building or room (eg. Shed, carport, garage, laundry, shelter, etc.); and - the use of flood compatible materials and services below the flood planning level in accordance with the requirements of Council’s Development Control Plan 2014 – Chapter C2 – Areas Affected by Flood. |  |
|  | - Flood Emergency Response Plan - A site-specific Flood Emergency Response Plan is required and be maintained for the development and made available to building managers and residents. The plan should include procedures for flood warning, evacuation, post-flood recovery and management of vulnerable occupants. |  |
|  | Fibre-ready Facilities and Telecommunications Infrastructure Unless exempted from Part 20A of the Telecommunications Act 1997, evidence satisfactory to the Certifying Authority must be submitted prior to the issue of the Crown Certificate in connection with a development, that the developer (whether or not a constitutional corporation) has made arrangements for: - the installation of fibre-ready facilities to all individual lots and/or premises in a real estate development project so as to enable fibre to be readily connected to any premises that is being or may be constructed on those lots. Demonstrate that the carrier has confirmed in writing that they are satisfied that the fibre ready facilities are fit for purpose; and - the provision of fixed-line telecommunications infrastructure in the fibre-ready facilities to all individual lots and/or premises in a real estate development project demonstrated through an agreement with a carrier. Notes: - real estate development project has the meanings given in section 372Q of the Telecommunications Act; and - exemptions only apply if published on the Register of developments exempted from Part 20A of the Telecommunications Act 1997 issued by the NSW Government Department of Infrastructure, Transport, Regional Development, Communications and the Arts, or similar. |  |
|  | Easement required over pipelines In all new subdivisions and/or developments, an easement in gross benefitting Byron Shire Council shall be provided for all pipelines in accordance with the Council Policy: Building in the Vicinity of Underground Infrastructure 2020. Easement documentation shall be prepared in accordance with NSW Land Registry Services requirements over the pipeline/s within the allotment/s. Easement documentation must be submitted to Council for execution, together with a survey showing location, size and depth of the pipeline/s and payment of fees in accordance with Council’s adopted fees and charges. Prior to issue of a certification that the Crown building works comply with the Building Code of Australia, proof of lodgement to NSW Land Registry Services must be provided to the Principal Certifying Authority. The minimum width of the required easement shall be 3.6 metres along the southern boundary. |  |
|  | Compliance required with Building in the Vicinity of Underground Infrastructure Policy All developments must comply with Policy: Building in the Vicinity of Underground Infrastructure. Foundations within a pipeline’s zone of influence will require piering below the zone of influence of the affected pipeline. The use of displacement and screw pile construction methods will require approval by Council. Any retaining wall / brick / masonry fence crossing a pipeline requires Council approval. The fence must be supported such that it does not cause loading on the pipes zone-of-influence. A minimum horizontal clearance of 1.5m from the centre is required around existing access structures as well as minimum vertical clearance of 3m from finished ground level is required. |  |
|  | - Landscape Plans The plans and specifications to accompany the certification application are to include a detailed landscape plan prepared by a suitably qualified landscape architect. - Landscaping with the subject site is to be in accordance with the Landscape Design Intent, dated 27/5/2025, prepared by Ivy May; and the Specifications contained within the Arboricultural Impact Assessment Report; dated 21/5/2025, prepared by Catherine Russo SHEgreen. - Landscaping within the sewer easement in the southern portion of the site is to be in Compliance Council’s Building in the Vicinity of Underground Infrastructure Policy. Refer to Appendix C of the Policy. |  |
|  | Minimum clearance between underground services Minimum horizontal and vertical clearances are required between gravity sewer, pressure sewer and water pipelines, and other services such as underground electrical cables. Clearance requirements between underground services are specified in the Water Services Association of Australia Codes. Access to the WSAA clearances can be obtained by contacting Byron Shire Council’s Utilities Manager. |  |
|  | - Water and Sewerage - Section 68 Part B approval If required, an Approval under Section 68 of the Local Government Act 1993 to carry out water supply work and sewerage work must be obtained. Water metering to be in accordance with Byron Shire Council’s water metering requirements See: https://www.byron.nsw.gov.au/Services/Building-development/Other-approvals-and-permits/Plumbing-and-Drainage-Applications |  |
|  | - Liquid Trade Waste - Section 68 Part C approval required If required, an approval under Section 68 Part C of the Local Government Act 1993 to discharge liquid trade waste into Council’s sewer must be obtained in accordance with current NSW Liquid Trade Waste Guidelines, Council’s Liquid Trade Waste Policy and Liquid Trade Waste Guidelines. Commercial, business, trade and industrial activities discharging or proposing to discharge to the sewer are required to notify Council and complete the Liquid Trade Waste Registration Form available at the Mullumbimby Office and from Council’s website at: Liquid Trade Waste registration form - Byron Shire Council (nsw.gov.au) Liquid Trade Waste approval must be obtained prior to gaining Section 68 Part B approval to carry out water supply work and sewerage work. |  |
|  | Certificate of Compliance – s307 Water Management Act 2000 Prior to the issue of a certification that the Crown building works comply with the Building Code of Australia, a Certificate of Compliance under Section 307 of the Water Management Act 2000 must be obtained upon payment of developer charges for water and sewer as calculated in accordance with Byron Shire Council and Rous County Council Development Servicing Plans. Where staging occurs, a separate s307 Compliance Certificate must be obtained for each stage. Byron Shire Council acts as Rous County Council’s agent in this matter and will issue a Certificate of Compliance on behalf of Rous County Council upon payment of the Rous County Council Development Servicing Charge to this Council. Note: For issue of the Certificate of Compliance, an application form and associated fee must be lodged via Council’s website. Copies of Byron Shire Council’s Development Servicing Plans are available via Council’s website. Developer charges will be calculated in accordance with the Development Servicing Plan applicable at the date of payment. Applicable charges can be found on Council’s website: https://www.byron.nsw.gov.au/Services/Water-sewer/Plumbers-and-developers/Calculate-the-cost-of-an-Equivalent-Tenement#section-3 The contributions payable will be adjusted in accordance with relevant plan and the amount payable will be calculated on the basis of the contribution rates that are applicable at the time of payment. Payment by Personal or Company Cheque will not be Accepted |  |
|  | - Long Service Levy to be paid In accordance with Section 6.8 of the Environmental Planning and Assessment Act 1979 (as amended), a Crown Certificate for SUBDIVISION WORKS OR BUILDING WORKS shall NOT be issued until any Long Service Levy payable under Section 34 of the Building and Construction Industry Long Service Payments Act, 1986 (or where such levy is payable by instalments, the first instalment of the levy) has been paid (as applicable). These payments can be made online at www.longservice.nsw.gov.au. Proof of payment is required to be submitted with the Crown Certificate application. For further information regarding the Long Service Payment please refer to the website above. |  |
|  | Provision of Affordable Housing Shop top housing apartments - From the date of the occupation of the building, twelve (12) studio, ten (10) one bedroom and two (2) two bedroom dwellings are to be used only for the purposes of ‘affordable housing’ (as defined in the Environmental Planning and Assessment Act 1979 and State Environmental Planning Policy [Housing] 2021), and these dwellings are to be managed by a registered community housing provider (registered community housing provider has the same meaning as in the Housing Act 2001). Boarding house - From the date of the occupation of the building, the boarding house is to be used only for the purposes of ‘affordable housing’ (as defined in the Environmental Planning and Assessment Act 1979 and of State Environmental Planning Policy [Housing] 2021), and the boarding house is to be managed by a registered community housing provider (registered community housing provider has the same meaning as in the Housing Act 2001) and is used for affordable housing in perpetuity. A restriction will be registered, prior to Crown Certification, against the title of the property, in accordance with section 88E of the Conveyancing Act 1919, that will ensure that these requirements of this condition are met. |  |
|  | Developer Contributions deferred for Affordable Dwellings Contributions set out in the schedule below are to be paid to Council. Contributions are levied in accordance with the Byron Shire Developer Contributions Plan 2012 (as mended). The Plan may be viewed on line at http://www.byron.nsw.gov.au/ or during office hours at the Council Offices located at Station Street, Mullumbimby. These contributions are to fund public amenities and services as listed in the schedule. Additional details on the specific amenities are to be found in the Byron Shire Developer Contributions Plan 2012 (as amended). These Contributions are not payable if whilst ever the twelve (12) studio, ten (10) one bedroom and two (2) two bedroom dwellings are permanently used for the provision of affordable housing as defined by State Environmental Planning Policy (Housing) 2021. Should these dwellings cease to be used for affordable housing then all contributions owing under this condition shall become payable and shall be indexed to the date of payment. The contributions in the schedule are current at the date of this consent. The contributions payable will be adjusted in accordance with the relevant plan and the amount payable will be calculated on the basis of the contribution rates that are applicable at the time of payment. The schedule contains a date for which the schedule remains valid, after this date you will have to contact Council for an updated schedule. |  |
|  | Developer Contributions to be paid for Standard Dwellings (Contributions for the twenty four (24) affordable shop top housing apartments are deferred) Contributions set out in the schedule below are to be paid to Council prior to the release of a crown certificate. Contributions are levied in accordance with the Byron Shire Developer Contributions Plan 2012 (as amended). The Plan may be viewed on line at www.byron.nsw.gov.au or during office hours at the Council Offices located at Station Street, Mullumbimby. These contributions are to fund public amenities and services as listed in the schedule. Additional details on the specific amenities are to be found in the Byron Shire Developer Contributions Plan 2012 (as amended). The contributions in the schedule are current at the date of this consent. The contributions payable will be adjusted in accordance with the relevant plan and the amount payable will be calculated on the basis of the contribution rates that are applicable at the time of payment. The schedule contains a date for which the schedule remains valid, after this date you will have to contact Council for an updated schedule. |  |
|  | - Tree Removal No trees or vegetation to be cleared or removed until a Crown Certification is complete. |  |
|  | Construction Environmental Management Plan including Noise and Vibration Management Plan A Construction Environmental Management Plan must be prepared before certification that Crown works comply with the NCC. The plan must be prepared by a qualified professional and include the following minimum requirements: Details of measures to be undertaken to ensure that construction impacts on surrounding environments are minimised including but not limited to: Identification of the specific activities likely to generate noise or vibration impacts, mitigation measures required, monitoring and noncompliance reporting, Identification of sensitive receptors, neighbourhood engagement strategies and complaints response procedures, Details of measures to be undertaken to ensure that construction impacts on surrounding environments are minimised. |  |
|  | Compliance with BASIX Certificate requirements The BASIX Certificate used for Crown certification is in accordance Certificate number 1788945M_03, dated Monday 01 June 2026 or an updated certificate if required. The plans submitted must clearly indicate all windows numbered or identified in a manner that is consistent with the identification on the Basix Certificate. Minor changes to the measures may be undertaken without the issue of any amendment under Section 4.55 of the Act, provided that the changes do not affect the form, shape or size of the building. Such plans and specifications must be part of the certification that Crown building works comply with the Building Code of Australia. |  |
| The following conditions are to be complied prior to any building or construction works commencing |  |  |
|  | - Heritage listed public lavatory – roof relocation The entire roof of the public lavatory is to be delivered to an agreed specified location for short term storage and or permanent repurposing. |  |
|  | - Public Safety Management Plan The approved public safety management plan is to be implemented. |  |
|  | - Traffic Management Plan The approved traffic management plan is to be implemented. |  |
|  | - Public safety requirements All care is to be taken to ensure the safety of the public in general, road users, pedestrians and adjoining property. The public liability insurance cover, for a minimum of $20 million, is to be maintained for the duration of the construction of the development. Council is to be nominated as an interested party on the policy. Council is not held responsible for any negligence caused by the undertaking of the works. |  |
|  | - Dilapidation Reports required Prior to the commencement of any excavation works requiring rock drilling, blasting or breaking, a pre-construction Dilapidation Report is to be submitted to Council detailing the current condition of all adjoining buildings, infrastructure and roads. A second Dilapidation Report must be submitted to Council, prior to occupation of the building (whole or partial), to ascertain if any structural damage has occurred to any adjoining building, infrastructure or roads. |  |
|  | - Erosion and Sediment measures Erosion and sedimentation controls are to be in place in accordance with the Sediment and Erosion Control Management Plan. Sediment and erosion control measures must be maintained at all times until the site has been stabilised by permanent vegetation cover or hard surface. |  |
|  | Toilet facilities Toilet facilities are to be provided, at or in the vicinity of the work site at the rate of one toilet for every 20 persons or part of 20 persons employed at the site. Each toilet provided must be a toilet connected to an accredited sewage management system approved by the Council or be a building and construction site portable chemical toilet. Only one (1) such portable chemical toilet may be used during construction, should additional toilets be required during the construction they must be either: - Connected to an accredited sewage management system approved by the Council. or - Not installed or used until such time that approval under Section 68 of the Local Government Act 1993 is obtained for the installation of a human waste storage facility. Note: The chemical toilet must be installed and serviced by a licensed contractor (including pump-outs). |  |
|  | - Water service to be connected A water service must be connected to the property using an approved backflow prevention device. It is the applicant’s responsibility to engage a licensed plumber who shall liaise with council during this process. Any new water service will be at the applicant’s cost. |  |
|  | - Site Location - The location of the building on the site to be established by a suitably qualified Surveyor and must comply with this approval including any required setbacks to boundaries. |  |
| The following conditions are to be complied with during any building or construction works |  |  |
|  | - Hours of work The principal certifier must ensure that building work, demolition or vegetation removal is only carried out between: - 7am to 6pm on Monday to Friday. - 8am to 1pm on Saturday. The principal certifier must ensure building work, demolition or vegetation removal is not carried out on Sundays and public holidays, except where there is an emergency. Unless otherwise approved within a construction site management plan, construction vehicles, machinery, goods or materials must not be delivered to the site outside the approved hours of site works. Note: Any variation to the hours of work requires Council’s approval. |  |
|  | - Construction Noise While building work is being carried out, and where a noise and vibration management plan is approved under this consent, the applicant must ensure that any noise generated from the site is controlled in accordance with the requirements of that plan. |  |
|  | - Signs to be erected on building and demolition sites A sign must be erected in a prominent position on the work site: - stating that unauthorised entry to the work site is prohibited, and - showing the name of the person in charge of the work site and a telephone number at which that person may be contacted outside working hours. Any such sign is to be removed when the work has been completed. |  |
|  | - Unexpected Findings during excavations In the event of any unexpected finds including the discovery of Underground Storage Tanks (UST), Asbestos Containing Material (ACM), odorous and/or stained soil material during excavation activities, all site work must cease and Raw Earth Environmental must be notified immediately for further investigation. |  |
|  | Demolition and Construction waste management: Waste management during demolition and construction must be consistent with the waste management plans for those phases in the waste management plan provided. |  |
|  | Acoustic Controls - All acoustic controls required by the Noise impact Assessment report and Construction Environmental Management Plan must be incorporated into the design and construction of the development. |  |
|  | - Builders rubbish to be contained on site All builder’s rubbish is to be contained on the site in a ‘Builders Skips’ or an enclosure. Footpaths, road reserves and public reserves are to be maintained clear of rubbish, building materials and all other items. |  |
|  | - Fill to be retained on the subject land Fill material must not encroach onto any adjoining land. |  |
|  | - Prevention of water pollution Only clean and unpolluted water is to be discharged to Council’s stormwater drainage system or any watercourse to ensure compliance with the Protection of Environment Operations Act. NOTE: Council may impose on-the-spot fines for non-compliance with this condition. |  |
|  | - Removal of asbestos All asbestos wastes associated with removal of the existing building to be disposed of in accordance with the requirements of the Workcover Authority. The applicant/owner is to produce documentary evidence that this condition has been met. Please note the Byron Resource Recovery Centre cannot accept asbestos. You will need to arrange disposal at an alternate landfill site. |  |
|  | - Maintenance of sediment and erosion control measures - Sediment and erosion control measures must be maintained at all times until the site has been stabilised by permanent vegetation cover or hard surface. |  |
|  | - Demolition - Any required demolition works must be undertaken in accordance with the relevant requirements of Australian Standard AS 2601–1991: The Demolition of Structures published by Standards Australia, and the Work Cover Authority of NSW. |  |
|  | - Removal of excavated soils, demolition and other wastes Any excavated soils to be disposed of off-site and in accordance with NSW EPA Waste Classification Guidelines (2014) and approved environmental management plans. - All wastes, including asbestos and lead-contaminated wastes, associated with these works are to be handled and disposed of in accordance with the requirements of the Work Cover Authority. The applicant/owner is to produce documentary evidence that this condition has been met. Wastes must be disposed of at a Licenced Waste Facility. All wastes removed from the site must be managed and disposed of in accordance with the NSW DECC Waste Classification Guidelines (2008). Demolition and builders waste transported from the site must be accompanied (a copy kept with the transporter) by a NSW Protection of The Environment Operations Act s143 Notice. |  |
|  | - Aboriginal Relics While demolition or building work is being carried out, all such works must cease immediately if a relic or Aboriginal object is unexpectedly discovered. The applicant must notify the Heritage Council of NSW in respect of a relic and notify the Secretary of the Department of Planning, Industry and Environment and the Heritage Council of NSW in respect of an Aboriginal object. Building work may recommence at a time confirmed by either the Heritage Council of NSW or the Secretary of the Department of Planning, Industry and Environment. In this condition: - “relic” means any deposit, artefact, object or material evidence that: - relates to the settlement of the area that comprises New South Wales, not being Aboriginal settlement, and - is of State or local heritage significance; and - “Aboriginal object” means any deposit, object or material evidence (not being a handicraft made for sale) relating to the Aboriginal habitation of the area that comprises New South Wales, being habitation before or concurrent with (or both) the occupation of that area by persons of non-Aboriginal extraction and includes Aboriginal remains. |  |
| The following conditions are to be complied with prior to occupation or use |  |  |
|  | - Works to be completed prior to Occupation or use of the building All the works indicated on the plans and approved by this consent, including any other consents that are necessary for the completion of this development including approvals issued under the Local Government Act 1993 and the Roads Act 1993; and a Building Code of Australia completion statement, are to be completed by an appropriately qualified person appointed by the Crown. Any Security bond paid for this application will be held until Council is satisfied that no further works are to be carried out that may result in damage to Council’s road/footpath reserve. |  |
|  | Easement required over pipelines The easement over Council pipelines required by conditions of this consent must be registered on title and a copy of the title search and registered easement documentation, including plan of easement must be submitted to the Principal Certifying Authority prior to occupation or use. |  |
|  | - Roadworks, access and parking areas to be completed. Prior to occupation of the development, the roadworks, access arrangements, driveway crossover and parking areas shall be constructed in accordance with the approved engineering plans and any applicable Roads Act approval. Certification from a suitably qualified practising engineer confirming that the works have been constructed in accordance with the approved engineering design and accompanying Work-As-Executed (WAE) documentation shall be provided. Evidence that all required Roads Act works have been completed to the satisfaction of Council, including written confirmation from Council, shall also be provided prior to occupation of the development. |  |
|  | - Stormwater Drainage Work Prior to occupation of the development, all stormwater drainage infrastructure associated with the development, including stormwater treatment devices, pipelines, pits, swales, surcharge relief measures, and any on-site stormwater detention works (if required), shall be constructed in accordance with the approved engineering plans and specifications. Work-As-Executed (WAE) plans and a Certificate of Compliance for the stormwater drainage works shall be prepared and certified by a suitably qualified practising engineer. The certification shall confirm that the stormwater drainage system has been constructed in accordance with the approved design and is capable of achieving the intended hydraulic and stormwater quality performance outcomes. The WAE plans and associated certification shall be submitted to Council and retained as part of the development records. The documentation shall demonstrate compliance with Council's Comprehensive Guidelines for Stormwater Management and include the as-constructed levels, locations and details of all stormwater infrastructure forming part of the approved drainage system. |  |
|  | - Floor Levels – Certification for Flooding Prior to occupation of the development, a Registered Surveyor shall certify that the as-constructed habitable and non-habitable floor levels comply with the minimum flood planning levels nominated in the approved Flood Impact Assessment and associated engineering documentation. The certification shall include levels (RLs) for all relevant floor areas and confirm compliance with the adopted flood immunity requirements for the development. A copy of the certification shall be submitted to Council for the attention of Council’s Drainage and Flood Engineer. |  |
|  | - Plumbing Works All works in relation to any associated Section 68 Water and Sewerage approval must be fully complied with and have a Final Plumbing Certificate issued prior to the Occupation or use of the building. |  |
|  | Mosquito & Biting Midges Management Provide effective insect screening to all windows, doors and other openings to all parts of the development used for residential purposes. |  |
|  | Acoustic compliance report An acoustic compliance report prepared by an acoustic engineer shall be submitted to Council as development related advice for approval. The report shall certify that the development has been designed and constructed in accordance with the established noise criteria and recommendations outlined in the approved acoustic report. The report must specify what works are required if necessary to bring acoustic performance into compliance. |  |
|  | - Site Location Survey A final survey of the structures showing setbacks to boundaries to be submitted to the Private Certifier with the Occupation Certificate demonstrating compliance with the approved plans. |  |
|  | - Landscaping - Landscaping to be completed in accordance with the approved landscaping plan. |  |
| The following conditions are to be complied with at all times |  |  |
|  | - Car Parking spaces are to be available for the approved use 19 car parking spaces and 1 SRV loading bay are to be provided and maintained, together with all necessary access driveways and turning areas, to the satisfaction of Council. Tenants and customers of the development must have unrestricted access to the car parking spaces on a daily basis during business hours of the development. No car parking spaces are to be reserved (generally or specifically) for any tenant or customer. Only carparking spaces required for the residential component of the development are to be reserved and made available for particular residents. |  |
|  | - Vehicles to enter/leave in a forward direction Vehicles using the parking area must enter and leave in a forward direction. All driveways and turning areas must be kept clear of obstructions that prevent compliance with this condition. |  |
|  | - Maintenance of Stormwater Quality Improvement Devices (SQIDs) and OSD - All SQIDs & OSD must be maintained in accordance with maintenance plan at all times. |  |
|  | - Overland Flow Path - The overland flow paths identified in the approved Flood Impact Assessment shall be maintained at all times for the life of the development. No walls, landscaping, retaining structures, storage areas, garden edging, structures, fill, debris, equipment or other obstructions shall be installed, placed or permitted within the designated overland flow paths where such works would impede flood conveyance, increase flood afflux, alter flow distribution, reduce flood storage, divert floodwaters, or result in adverse flood impacts on the development, adjoining properties or the surrounding road network. Any fencing or gates in the overland flow path are to have fencing panels with a permeability of 60% or greater. - All overland flow paths shall be maintained in a condition consistent with the flood modelling assumptions and design intent of the approved Flood Impact Assessment and associated civil engineering documentation. |  |
|  | - Flood Emergency Response Plan (FERP) - The FERP must be implemented in any flood events and emergencies. |  |
|  | Night-time lighting External lighting arrangements should be in accordance with AS4282-1997: Control of the obtrusive effects of outdoor lighting and NSW Work Cover OH&S requirements. External lighting must be directed downwards and limited to ensure minimal dispersion of luminosity to the surrounding area and to protect neighbouring properties, the dark night sky and the rural atmosphere of the locality. |  |
|  | Landscaping The landscaping of the site must be maintained to ensure the establishment and successful growth of plants at all times. |  |
|  | - Boarding House Management - The boarding house is to be managed in accordance with the Boarding House Plan of Management, prepared by Wilson Planning, dated June 2026. |  |
|  | - Must not interfere with the amenity of the neighbourhood The use of the development must not interfere with the amenity of the neighbourhood by reason of noise, vibration, smell, fumes, smoke, dust, wastewater or otherwise. In particular: - Any complaints to Council about ‘offensive’ noise will be dealt with under the provisions of the Protection of the Environment Operations Act 1997. - Only clean and unpolluted water is permitted to be discharged to Councils’ stormwater drainage system or any waters. - All wastes shall be contained within appropriate containers fitted with a tight-fitting vermin-proof lid. - All waste storage and sewage facilities shall be serviced and maintained to ensure that all relevant environment protection standards are satisfied. - Goods deliveries shall be restricted to daytime operating hours. |  |

| - Schedule |  | - PRESCRIBED CONDITIONS |
| --- | --- | --- |
| The prescribed conditions in accordance with Division 2, Subdivision 1 of the Environmental Planning and Assessment Regulation 2021 as at the date of this development consent as are of relevance to this development must be complied with: |  |  |
| 69 | Compliance with Building Code of Australia and insurance requirements under the Home Building Act 1989 |  |
| 70 | Erection of signs |  |
| 71 | Notification of Home Building Act 1989 requirements |  |
| 72 | Entertainment venues |  |
| 73 | Maximum capacity signage |  |
| 74 | Condition relating to shoring and adequacy of adjoining property |  |
| 75 | Fulfilment of BASIX commitments |  |
| Refer to the Environmental Planning and Assessment Regulation 2021, Division 2, Subdivision 1 of for full text of the above clauses. This can be accessed at http://www.legislation.nsw.gov.au. |  |  |

| - Schedule | - REASONS FOR DECISION, HOW COMMUNITY VIEWS WERE ADDRESSED |
| --- | --- |

Delete Reason/s not required

Council’s are required to give and publicly notify reasons for a range of planning decisions where they are deciding if development should proceed to help community members to see how their views have been taken into account and improve accountability to stakeholders. A statement of reasons for the determination of this application is provided below.

Statement of Reasons

The proposed development complies with the provisions of Byron Local Environmental Plan 2014.

The proposed development complies with relevant State Environmental Planning Policies

The proposed development complies with relevant provisions of Development Control Plan 2014

The proposed development complies with Environmental Planning & Assessment Regulation 2021 considerations.

The proposed development will not have significant adverse impact on the natural, built or social environment or economic impacts on the locality.

The proposed development is considered suitable for the proposed site.

The application was publicly exhibited in accordance with Council’s Community Participation Plan. Issues raised in the submissions have been addressed during assessment of the application.

The proposed development is unlikely to prejudice or compromise the public interest.

The proposed development will not have a detrimental impact on the heritage character of the precinct or the buildings contained on the site. It is considered consistent with the heritage provisions contained within Clause 5.10 of Local Environmental Plan 2014.

The proposed development will not have a detrimental impact on the heritage character of the precinct or the buildings contained on the site. It is considered consistent with the heritage provisions contained within Development Control Plan 2014.

How community views were addressed

The application was publicly exhibited in accordance with Council’s Community Participation Plan. The submissions received were considered on merit and addressed during assessment of the application. To view the considerations, please contact Council to view a copy of the assessment report relating to this application.

| - Schedule | - NOTES |
| --- | --- |
| Protection of the Environment Operations Act 1997: It is an offence under the provisions of the Protection of the Environment Operations Act 1997 to act in a manner causing, or likely to cause, harm to the environment. Anyone allowing material to enter a waterway or leaving material where it can be washed off-site may be subject to a penalty infringement notice (“on-the-spot fine”) or prosecution. |  |
| Penalties apply for failure to comply with development consents Failure to comply with conditions of development consent may lead to an on-the-spot fine being issued pursuant to section 4.2(1) of the Environmental Planning & Assessment Act 1979 or prosecution pursuant to section 9.50 of the Environmental Planning & Assessment Act 1979. |  |
| Plumbing Standards and requirements. All Plumbing, Water Supply, Sewerage and Stormwater Works shall be installed in accordance with the Local Government Act 1993, Plumbers Code of Australia and AS/NZS 3500 Parts 0-5, the approved plans (any notations on those plans) and the approved specifications. Any plumbing inspections required under a Section 68 Approval are to occur in accordance with that approval. |  |
| Relics Provisions- Advice Attention is directed to the NSW Heritage Act 1977 and the provisions of the Act in relation to the exposure of relics. The Act requires that if: - a relic is suspected, or there are reasonable grounds to suspect a relic in ground, that is likely to be disturbed damaged or destroyed by excavation; and/or - any relic is discovered in the course of excavation that will be disturbed, damaged or destroyed by further excavation. Those responsible for the discovery must notify nominated management personnel who will in turn notify the Heritage Council of New South Wales or its delegate, the Office of Environment and Heritage, NSW Heritage Branch, and suspend work that might have the effect of disturbing, damaging or destroying such relic until the requirements of the NSW Heritage Council have been satisfied (ss139, 146). |  |
| Fire Ants The importation of any of the following material from Queensland invasive ant biosecurity zones must be in accordance with the Biosecurity (Invasive Ant Carriers) Control Order 2023 (including any revised orders made under the Biosecurity Act 2015) and meet the requirements of NSW Department of Primary Industries: - organic mulch (which includes manure, bark, wood chips, hay, straw, sileage, and sugar cane bagasse); - baled materials; - potted plants; - agricultural or earth-moving machinery; - fill or soil (which includes anything with soil on it such as turf); and - mining or quarrying materials. Prior to the importation of each material type, the supplier must provide the receiver and the Principal Certifying Authority with the relevant Certificate as identified within the Biosecurity (Invasive Ant Carriers) Control Order 2023 or revised biosecurity control orders. All material shall meet the requirements of the relevant Certificate. It is an offence under the Biosecurity Act 2015 if this material comes from within five kilometres of a known invasive ant infested area (e.g. identified Fire Ant Biosecurity Zones in Queensland), or any other place at which the person knows, or ought reasonably to know, that an invasive ant has been detected, unless the carrier material has been managed and treated to reduce the risk and meets the certification requirements listed in the Control Order. |  |
| Illegal dumping in reserves, bushland or on the kerbside All construction, demolition and green waste must be disposed of appropriately. Council has a ZERO tolerance policy towards illegal dumping in reserves, bushland or on the kerbside. On-the-spot fines of a minimum of $8 000 for corporations apply and up to $5 000 000 where waste causes actual or likely harm to the environment. Council will be monitoring all DA approved sites to ensure compliance. Council’s bush regeneration team regularly work in native reserves to improve the habitats and will also be monitoring for green waste dumping. |  |
| Water payments under the Water Management Act 2000 Charges will be calculated based on the additional water and sewerage load that the proposed development generates, shown in Equivalent Tenements (ET) by the following table: ADDITIONAL WATER & SEWER LOAD OF DEVELOPMENT (ET Policy 2022) Water; 10.55 ET. Bulk Water; 10.55 ET. Sewer; 14.23 ET. NB: Information regarding Development Servicing charges can be found on the Byron Shire Council website (https://www.byron.nsw.gov.au/Development-Business/Building-Construction/Plumbing-Sewage/Calculate-the-cost-of-an-Equivalent-Tenement). These charges will enable you to calculate the total contribution charges payable when you are ready to pay them. Developer charges will be calculated in accordance with the Development Servicing Plan applicable at the date of payment. |
| S7.11 Schedule of Development Contributions The following contributions are current at the date of this consent. The contributions payable will be adjusted in accordance with the relevant plan and the amount payable will be calculated based on the contribution rates that are applicable at the time of payment. The current contribution rates are available from Council offices during office hours. Developer Contributions to be paid for Dwellings (not including twenty four (24) affordable shop top housing apartments – payment deferred) Developer Contributions deferred for twenty four (24) Affordable Dwellings |  |
| Essential Energy |  |