### FOR USE BY PLANNERS/SURVEYORS TO PREPARE LIST OF PROPOSED CONDITIONS

# NOTE:  THESE ARE DRAFT ONLY

### DA NO:  2019/546  		DATE: 19/11/2019

PRESCRIBED CONDITIONS

The development is to be undertaken in accordance with the prescribed conditions of Part 6 - Division 8A of the Environmental Planning & Assessment Regulations 2000.

A – GENERAL MATTERS

(1)	(A001) The development is to be carried out in accordance with the plans and supporting documents set out in the following table, as stamped and returned with this consent, except where modified by any conditions of this consent.

| Plan / Supporting Document | Reference | Prepared by | Date |
| --- | --- | --- | --- |
| Cover sheet | AR-2B-DA-A01-01 | BVN | 10 July 2019 |
| Site plan | AR-2B-DA-A10-01 | BVN | 10 July 2019 |
| Level 1 plan | AR-2B-DA-B10-01 | BVN | 10 July 2019 |
| Level 2 plan | AR-2B-DA-B10-02 | BVN | 10 July 2019 |
| Roof plan | AR-2B-DA-B10-03 | BVN | 10 July 2019 |
| Elevations | AR-2B-DA-C10-01 | BVN | 10 July 2019 |
| Sections | AR-2B-DA-D10-02 | BVN | 10 July 2019 |
| Materials board | AR-2B-DA-P10-01 | BVN | 10 July 2019 |
| Stage 2B Building works - Noise impact assessment |  | WSP | 31 July 2019 |
| Bushfire Assessment Report - Tertiary Educations Development CSU |  | Blackash Bushfire Consulting | 30 July 2019 |
| Plan of detail and levels over part of Lot 1 DP 1240488 and adjacent road | Sheet 1 Revision C | King and Campbell | 17 August 2018 |
| Flora and fauna assessment | Version - V3.1 | SLR | August 2019 |
| Stage 2B Landscape concept | Sheets 0 to 4 | Oculus | July 2019 |
| Statement of Environmental Effects | 6325 | King and Campbell | July 2019 |
| Civil Engineering Report | 171878 CAAC | TTW | 26 September 2019 |

In the event of any inconsistency between conditions of this development consent and the plans/supporting documents referred to above, the conditions of this development consent prevail.

(3)	(A008) Any necessary alterations to, or relocations of, public utility services to be carried out at no cost to council and in accordance with the requirements of the relevant authority including the provision of easements over existing and proposed public infrastructure.

(4)	(A009) The development site is to be managed for the entirety of work in the following manner:

- Erosion and sediment controls are to be implemented to prevent sediment from leaving the site. The controls are to be maintained until the development is complete and the site stabilised with permanent vegetation;

- Appropriate dust control measures;

- Building equipment and materials shall be contained wholly within the site unless approval to use the road reserve has been obtained. Where work adjoins the public domain, fencing is to be in place so as to prevent public access to the site;

- Building waste is to be managed via appropriate receptacles into separate waste streams;

- Toilet facilities are to be provided on the work site at the rate of one toilet for every 20 persons or part of 20 persons employed at the site.

- Building work being limited to the following hours, unless otherwise permitted by Council;

- Monday to Saturday from 7.00am to 6.00pm

- No work to be carried out on Sunday or public holidays

The builder to be responsible to instruct and control his sub-contractors regarding the hours of work.

(5)	(A011) The design and construction of all public infrastructure works shall be in accordance with Council’s adopted AUSPEC Specifications.

(6)	(A014) This approval does not provide any indemnity to the owner or applicant under the Disability Discrimination Act 1992 with respect to the provision of access and facilities for people with disabilities.

(7)	(A029) The provision, at no cost to Council, of 2.0m wide concrete foot paving as illustrated on the Concept Siteworks Plan Sheets 1 & 2 by TTW, Job No 171878, Drawing Number SKC421 revision P8 & SKC422 Revision P8 with design details in accordance with AUSPEC and Council Standard drawings. The design plans must be approved by Council pursuant to Section 138 of the Roads Act.

(8)	(A032) The developer is responsible for any costs relating to minor alterations and extensions to ensure satisfactory transitions of existing roads, drainage and Council services for the purposes of the development.

(9)	(A033) The applicant shall provide security to the Council for the payment of the cost of the following:

- making good any damage caused to any property of the Council as a consequence of doing anything to which the consent relates,

- completing any public work (such as road work, kerbing and guttering, footway construction, utility services, stormwater drainage and environmental controls) required in connection with the consent,

- remedying any defects in any such public work that arise within twelve (12) months after the work is completed.

Such security is to be provided to Council prior to the issue of the Section 138 of the Roads Act, 1993 approval.

The security is to be for such reasonable amount as is determined by the consent authority, being an amount that is the estimated cost plus 30% for building development of public works or $5000, whichever is the greater of carrying out the development by way of:

- deposit with the Council, or

- an unconditional bank guarantee in favour of the Council.

The security may be used to meet any costs referred to above and on application being made to the Council by the person who provided the security any balance remaining is to be refunded to, or at the direction of, that person. Should Council have to call up the bond and the repair costs exceed the bond amount, a separate invoice will be issued. If no application is made to the Council for a refund of any balance remaining of the security within 6 years after the work to which the security relates has been completed the Council may pay the balance to the Chief Commissioner of State Revenue under the Unclaimed Money Act 1995.

(10)	(A057) The applicant is to ensure the proposed development will drain to an approved point of connection to Council’s sewerage system.

(11)	(A063) The disposal of wastewater from a commercial or industrial business to Port Macquarie-Hastings Council’s sewage system requires specific approval under Section 68 of the Local Government Regulation, 1999.

In this regard, whilst you have indicated that trade waste will not be discharged from the subject development, should you wish to discharge liquid trade wastes to Council’s sewer in the future, a further application under Section 68 of the Local Government Regulation, 1999 will be required.

(12)	The proposal shall comply with the approved Koala Plan of Management.

(13)	(A195)  The intersection with the proposed access road shall be designed in accordance with Austroad Part 4A and Council’s adopted AUSPEC Design and Construction Guidelines. The intersection shall provide a channelised right turn and an auxiliary left turn off Major Innes Road. Details shall be provided as part of the Roads Act application.

(14)	(A196)  A concrete entry treatment is required on the proposed entry driveway which intersects with Major Innes Road to delineate between the public and private roads. Details of the entry treatment shall be provided as part of the Roads Act application.

B – PRIOR TO WORKS COMMENCING

(1)	(B001) Prior to works commencing, approval pursuant to Section 68 of the Local Government Act, 1993 to carry out water supply, stormwater and sewerage works is to be obtained from Port Macquarie-Hastings Council.  The following is to be clearly illustrated on the site plan to accompany the application for Section 68 approval:

- Position and depth of the sewer (including junction)

- Stormwater drainage termination point

- Easements

- Water main

(2)	(B003) Submission to Council prior to works commencing of detailed design plans for the following works associated with the developments.  Public infrastructure works shall be constructed in accordance with Port Macquarie-Hastings Council’s current AUSPEC specifications and design plans are to be accompanied by AUSPEC DQS:

- Road works along the frontage of the development.

- Public parking areas including;

  - Driveways and access aisles;

  - Parking bays;

  - Delivery vehicle service bays & turning areas 
in accordance with AS2890.

- Sewerage reticulation.

- Water supply plans shall include hydraulic plans for internal water supply services and associated works in accordance with AS 3500, Plumbing Code of Australia and Port Macquarie-Hastings Council Policies.

- Stormwater systems.

- Erosion & Sedimentation controls.

- Detailed driveway profile in accordance with Australian Standard 2890, AUSPEC D1, and ASD, Port Macquarie-Hastings Council current version.

- The provision of a 2.0m wide concrete foot paving as illustrated on the Concept Siteworks Plan Sheets 1 & 2 by TTW, Job No 171878, Drawing Number SKC421 revision P8 & SKC422.

- Detailed intersection layout at the junction of new road and Major Innes Drive in accordance with the current version of the AUSTROADS guidelines for Intersection design, giving particular attention to sight distance.

(3)	(B006) An application pursuant to Section 138 of the Roads Act, 1993 to carry out works required by the Development Consent on or within public road is to be submitted to and obtained from Port Macquarie-Hastings Council prior to works commencing.

Such works include, but not be limited to:

- Civil works

- Traffic management

- Work zone areas

- Hoardings

- Concrete foot paving (width)

- Footway and gutter crossing

- Functional vehicular access

(4)	(B011) As part of Notice of Requirements by Port Macquarie-Hastings Council as the Water Authority under Section 306 of the Water Management Act 2000, the payment of a cash contribution, prior to works commencing, of the Section 64 contributions, as set out in the “Notice of Payment – Developer Charges” schedule attached to this consent unless deferral of payment of contributions has been approved by Council.  The contributions are levied in accordance with the provisions of the relevant Section 64 Development Servicing Plan towards the following:

- augmentation of the town water supply headworks

- augmentation of the town sewerage system headworks

(5)	(B030) Prior to public infrastructure road works commencing, a pavement design report shall be prepared by a suitably qualified geotechnical or civil engineer and submitted to Council, including soil test results and in-situ CBR values (NATA certified). Council’s minimum pavement compaction testing criteria are as follows:

- 98% (modified) base layers - Maximum Modified Dry Density test in accordance with AS1289.5.2.1

- 95% (modified) sub-base layers - Maximum Modified Dry Density test in accordance with AS1289.5.2.1

- 100% (standard) subgrade/select layers - Maximum Standard Dry Density test in accordance with AS1289.5.1.1 (or for in-situ subgrade soils only, wet density testing may be used)

(6)	(B038) Footings and/or concrete slabs of buildings adjacent to sewer lines or stormwater easements are to be designed so that no loads are imposed on the infrastructure. Detailed drawings and specifications prepared by a practising chartered professional civil and/or structural engineer are to be submitted to Council prior to works commencing.

(7)	(B085) The location of electricity substations are to be clearly illustrated on the construction plans. All substations are to remain on private property unless otherwise agreed to by Port Macquarie-Hastings Council.

(8)	(B053) The design of the carpark and accesses is to be in accordance with Australian Standard 2890 (including AS 2890.1, AS 2890.2 and AS 2890.6).  Certification of the design by a suitably qualified consultant is to be provided to Council prior to works commencing.

(9)	(B054) A driveway longitudinal section shall accompany the section 138 application pursuant to section 138 of the Roads Act, 1993. The section shall demonstrate compliance with Council’s adopted AUSPEC Design and Construction Guidelines.

(10)	Prior to building works commencing, the approved landscape plan is to be amended to reflect consistency with the approved carparking layout and site plan.

(11)	(C001) A minimum of one (1) week’s notice in writing of the intention to commence works on public land is required to be given to Council together with the name of the principal contractor and any major sub-contractors engaged to carry out works.  Works shall only be carried out by a contractor accredited with Council.

(12)	(C013) Where a sewer manhole and Vertical Inspection Shaft exists within a property, access to the manhole/VIS shall be made available at all times.  Before during and after construction, the sewer manhole/VIS must not be buried, damaged or act as a stormwater collection pit.  No structures, including retaining walls, shall be erected within 1.0 metre of the sewer manhole or located so as to prevent access to the manhole.

(13)	Prior to building works commencing the construction plans are to include details of BAL 12.5 construction in accordance with AS3959, 2009 Construction of Buildings in Bushfire Prone Areas and Appendix 3 of Planning for Bushfire Protection 2006.

C – DURING WORK

(1)	(D001) Development works on public property or works to be accepted by Council as an infrastructure asset are not to proceed past the following hold points without inspection and approval by Council.  Notice of required inspection must be given 24 hours prior to inspection, by contacting Council’s Customer Service Centre on (02) 6581 8111. You must quote your Section 138 approval number and property description to ensure your inspection is confirmed:

- at completion of installation of traffic management works

- before commencement of any filling works;

- when the sub-grade is exposed and prior to placing of pavement materials;

- when trenches are open, stormwater/water/sewer pipes and conduits jointed and prior to backfilling;

- at the completion of each pavement (sub base/base) layer;

- before pouring of kerb and gutter;

- prior to the pouring of concrete for sewerage works and/or works on public property;

- on completion of road gravelling or pavement;

- during construction of sewer infrastructure;

- prior to sealing and laying of pavement surface course.

All works at each hold point shall be certified as compliant in accordance with the requirements of AUSPEC Specifications for Provision of Public Infrastructure and any other Council approval, prior to proceeding to the next hold point.

(2)	(D006) A copy of the current stamped approved construction plans must be kept on site for the duration of site works and be made available upon request to either the Principal Certifying Authority or an officer of the Council.

(3)	(D051) Prior to commencement of any public infrastructure pavement works a material quality report from the proposed supplier shall be submitted to Council. The pavement materials shall meet Council’s current specifications at the time of construction.

(4)	(D052) Prior to laying of Asphaltic Concrete (AC) or wearing surface course on public infrastructure road works, submission to Council of pavement and soil test results prepared by a NATA registered person for all road pavement construction, including:

- CBR test results, and

- Subgrade / select fill, sub-base and base pavement compaction reports in accordance with AS1289.5.1.1 & AS1289.5.2.1 as applicable.

(5)	(D040) Wastes shall not be disposed of by burning.

(6)	(D003) The site is in an area known to contain rock that may contain naturally occurring asbestos (NOA). Should potential NOA be located on site notification shall be provided to Council and Workcover prior to works proceeding. No work shall recommence until a NOA management plan has been approved by Council or Workcover.

D – PRIOR TO OCCUPATION

- (E005) Prior to the release of any bond securities held by Council for infrastructure works associated with developments, a formal written application is to be submitted to Council specifying detail of works and bond amount.

- (E007) The owner/applicant is responsible for ensuring that any imported fill is either Virgin Excavated Natural Material (VENM) or Excavated Natural Material (ENM).

(3)	(E013) Restrictions and/or positive covenant must be provided over the overland flow path for on site detention storage areas with appropriate public awareness signage.

(4)	(E030) Vehicle ramps, driveways, turning circles and parking spaces being paved, sealed and line marked prior to occupation or commencement of the approved land use.

(5)	(E034) Prior to occupation documentation is to be obtained from Port Macquarie-Hastings Council being the local roads authority certifying that all matters required by the approval issued pursuant to Section 138 of the Roads Act have been satisfactorily completed.

(6)	(E036) Certification by a suitably qualified consultant is to be submitted to the Principal Certifying Authority (PCA) confirming that the car park and internal accesses have been constructed in accordance with Australian Standard 2890 (including AS 2890.1, AS 2890.2 and AS 2890.6) prior to occupation.

(7)	(E050) Prior to Council accepting new stormwater infrastructure, a CCTV inspection of all new and modified stormwater assets must be undertaken in accordance with the Conduit Inspection Reporting Code of Australia WSA 05.

A copy of the CCTV inspection footage and inspection report prepared and certified by a suitably qualified person shall be provided to Council prior to the acceptance of works into the nominated ‘into maintenance period’.

(8)	(E051) Prior to occupation a section 68 Certificate of Completion shall be obtained from Port Macquarie-Hastings Council.

(9)	(E056) A Certificate of Compliance under the provisions of Section 307 of the Water Management Act must be obtained prior to occupation.

(10)	(E066) Ancillary works shall be undertaken at no cost to Council to make the engineering works required by this Consent effective to the satisfaction of Director of Council’s Infrastructure Division. Such works shall include, but are not limited to the following:

- The relocation of underground services where required by civil works being carried out.

- The relocation of above ground power and telephone services

- The relocation of street lighting

- The matching of new infrastructure into existing or future design infrastructure

(11)	(E072) Lodgement of a security deposit with Council upon practical completion of the public infrastructure works.

(12)	(E082) Submission of a compliance certificate accompanying Works as Executed plans for public infrastructure works with detail included as required by Council’s current AUSPEC Specifications. The information is to be submitted in electronic format in accordance with Council’s “CADCHECK” requirements detailing all infrastructure for Council to bring in to account its assets under the provisions of AAS27. This information is to be approved by Council prior to Council accepting the public infrastructure work in to the maintenance/defects liability period. The copyright for all information supplied, shall be assigned to Council.

(14)	Prior to occupation, a copy of the ‘CSU Port Macquarie Campus Green Travel Plan’ is to be provided to Council and Roads and Maritime Services.

F – OCCUPATION OF THE SITE

(1)	(F001) On site car parking in accordance with the approved plans to be provided at all times during the operations of development for use by both staff and patrons.

(2)	(F003)	All loading and unloading operations associated with servicing the site must be carried out within the confines of the site, at all times and must not obstruct other properties/units or the public way.

(3)	(F013) All garbage areas are to be screened from the street, create no adverse odour impact on adjoining properties and be kept free of pests at all times.

(4)	(F019) Contaminated wastes shall be removed from the site by an approved contaminated waste contractor for disposal at an approved facility.

(5)	(F021) All solid waste generated by the development which cannot be reused or recycled, shall be disposed of at Council’s waste management facilities.

(6)	(F024) Offensive noise as defined under the Protection of the Environment Operations Act 1997, shall not be generated as a result of the operation of the development.

(7)	(F036)	Any exterior lighting on the site shall be designed and installed so as not to cause a nuisance or adverse impact on the amenity of the surrounding area by light overspill. The lighting shall be the minimum level of illumination necessary for safe operation and must be designed, installed and used in accordance with AS 4282 - 1997 control of the obtrusive effects of outdoor lighting. No flashing, moving or intermittent lighting is permitted on the site.