| COUNCIL ASSESSMENT REPORT SYDNEY EASTERN CITY PLANNING PANEL |  |
| --- | --- |

| PANEL REFERENCE & DA NUMBER | PPSSEC-343 – DA413/2024 |
| --- | --- |
| PROPOSAL | Demolition of the existing buildings and construction of a new four storey mixed use building comprising a Woolworths supermarket at Ground floor and Level 1 and 13 residential apartments at Levels 1-3, three levels of basement car parking and associated services; site remediation; earthworks; landscaping; signage and siteworks. |
| ADDRESS | LOT: 1 DP:1009799 and LOT: 30 SEC: B DP: 4567 488 – 492 Old South Head Road 30 Albemarle Avenue, Rose Bay |
| APPLICANT | Fabcot Pty Ltd |
| OWNER | Fabcot Pty Ltd |
| DA LODGEMENT DATE | 21/10/2024 |
| APPLICATION TYPE | Development Application (Integrated) |
| REGIONALLY SIGNIFICANT CRITERIA | Section 2.19(1) and Clause 2 of Schedule 6 of State Environmental Planning Policy (Planning Systems) 2021 declares the proposal regionally significant development as: the cost of works exceeds $30 million |
| CIV | $58,630,000.00 (excluding GST) |
| CLAUSE 4.6 REQUESTS | Clause 4.3 and Clause 6.12(3) of WLEP 2014 (height control) |
| LIST OF ALL RELEVANT PLANNING CONTROLS (S4.15(1)(A) OF EP&A ACT) | - State Environmental Planning Policy (Sustainable Buildings) 2022 - State Environmental Planning Policy (Housing) 2021 - State Environmental Planning Policy (Industry and Employment) 2021 - State Environmental Planning Policy (Planning Systems) 2021 - State Environmental Planning Policy (Resilience and Hazards) 2021 - State Environmental Planning Policy (Transport and Infrastructure) 2021 - State Environmental Planning Policy (Biodiversity & Conservation) 2021 - Woollahra Local Environmental Plan 2014; - Woollahra Development Control Plan 2015. |
| AGENCY REFERRALS | Water NSW Transport for NSW Ausgrid |
| TOTAL & UNIQUE SUBMISSIONS | 46 submissions |
| DOCUMENTS SUBMITTED FOR CONSIDERATION | Architectural Plans, Survey Plan, Landscape Plan, Aboriginal Cultural Heritage or Assessment Report, Access report, Acoustic & Vibration Assessment, Arborists report, BASIX certificate, BCA Report, Clause 4.6 – Height, Construction Management Plan, Remediation Action Plan, Design Verification Statement, Flood Risk Management Report, Geotechnical Report, Plan of management, Statement of environmental effects, Stormwater Management Plan, Structural Engineers Report, Traffic Impact Assessment, Traffic Impact Assessment, Waste Management Plan |
| DOCUMENTS SUBMITTED WITH THIS REPORT FOR THE PANEL’S CONSIDERATION | Attachment 1 – Draft Conditions Attachment 2 – Revised Architectural Plans Attachment 3 – Revised Clause 4.6 Written Request Attachment 4 – Design Verification Statement Attachment 5 – Referral Response Development Engineering Attachment 6 – Referral Response Traffic Engineering Attachment 7 – Referral Response Trees and landscaping Attachment 8 – Referral Response Heritage Attachment 9 – Referral Response Urban Design Attachment 10 – Referral Response Environmental Health Attachment 11 – Referral Response Drainage Engineering Attachment 12 – Referral Response Transport for NSW Attachment 13 – Referral Comment Ausgrid Attachment 14 – WaterNSW General Terms of Approval |
| INFRASTRUCTURE CONTRIBUTIONS (S7.24) | Yes |
| RECOMMENDATION | Conditional Approval |
| PREPARED BY | Ms L Samuels |
| PANEL BRIEFING | 27 February 2025 |
| DATE OF REPORT | 5 August 2025 |

REASON for report to Sydney Eastern City Planning Panel (SECPP)

In accordance with the State Environmental Planning Policy (Planning Systems) 2021, the application is to be determined by the Sydney Eastern City Planning Panel as it is regionally significant development with a Capital Investment Value over $30 million.

REASONS for recommendation

The application has been assessed within the framework of the matters for consideration under section 4.15 of the Environmental Planning and Assessment Act 1979 and is recommended for approval because:

- It is considered to be satisfactory with all relevant planning policies including the objectives of WLEP 2014 and WDCP 2015

- The proposal is generally consistent with the building envelope, built form and setbacks outlined at Planning Proposal stage and the specific controls prepared for the site under Chapter G9 of WDCP 2015

- It will not have adverse effects on the local built and natural environment nor any adverse social and economic impacts in the locality such that refusal is warranted.

- All likely impacts to adjoining properties including any submissions made have been addressed in the report, or are considered to be satisfactory. Subject to the recommended conditions of consent, the proposal will not result in an unreasonable amenity impact to adjoining and nearby properties such that refusal would be warranted.

- The site is suitable for the proposed development as the proposed land use is permissible on the site

- The proposal is in the public interest as it will have positive social impacts within the locality due to the provision of a supermarket, will generate economic benefits and will contribute to housing supply.

LOCALITY PLAN

Objectors’ properties are listed in Section 9 of the report Subject site North

PROPOSAL

The proposal involves the following works:

- Demolition of the existing building, remediation and earthworks.

- Construction of a four storey mixed-use building comprising the following:

    - Ground floor and part of Level 1: Woolworths supermarket

    - Levels 1 – 3: 13 residential apartments, comprising a mix of 2, 3 and 4 bedroom dwellings.

- Construction of three basement levels containing 65 supermarket parking spaces (plus 5 ‘Direct to Boot’ spaces), 19 residential parking spaces, 3 residential visitor parking spaces, a car wash bay, as well as 9 motorcycle and 15 resident bicycle spaces and 30 visitor bicycle spaces.

- Installation of business and building identification signage.

- Landscaping works including a buffer zone at ground level and communal and private open spaces for the upper level units.

Additional information was submitted on 22/04/2025 and included the following:

- Acoustic Statement (in relation to the loading dock) prepared by E-LAB consulting.

- Traffic Statement prepared by JMT consulting.

- Revised Landscape Plan.

- Revised Operational Management Plan.

- A Design Verification Statement.

- Substation location review prepared by Neuron and Projen.

- The Architectural Plans were updated as follows:

    - The electrical substation at ground level has been setback 0.8m from the front boundary and moveable planter boxes have been provided as screening.

    - Updated sightline diagrams have been provided.

    - Additional screening has been provided to the fire hydrant at ground floor level.

    - The plans note that Level 2 electric vehicle charging points will be provided to all off-street car parking spaces (residential and non-residential) to ensure that 100% of car spaces can install electric vehicle charging points in the future.

- Modifications to the layout of the communal open space at ground floor and level 2.

- The addition of a glass balustrade atop the planters to the residential balconies.

Planning Proposal

The site was the subject of a Planning Proposal (PP-2022-731) which received Gateway Determination from the Department of Planning, Housing and Infrastructure (DPHI) on 23 February 2024. The planning proposal was placed on Public Exhibition by DPHI from 2 April 2024 to 7 May 2024. The Planning Proposal sought the following amendments to the Woollahra LEP 2014:

- Insert a new clause in Schedule 1 Additional permitted uses to permit retail premises at 30 Albemarle Avenue, provided it is as part of a shop top housing development at 488-492 Old South Head Road.

- Create a new local provisions clause that applies only if 488-492 Old South Head Road and 30 Albemarle Avenue are developed together that: Allows a maximum Gross Floor Area of 3,720m² on 488-492 Old South Head Road and 480m² on 30 Albemarle Avenue.

- Permits a maximum Height of Building of 14.5m at 30 Albemarle Avenue.

The Planning Proposal was approved by the Department of Planning, Housing and Infrastructure (DPHI) on 18 March 2025 and came into effect on 25 July 2025.

Figure 1 - Site Plan

ISSUES

Exceptions to Development Standards in Woollahra Local Environmental Plan 2014

| Clause | Development Standard | Departure from Control | Conclusion |
| --- | --- | --- | --- |
| Part 4.3 and Part 6.12(3) | Height of Buildings | 14.89m (roof) or 2.7% departure from the 14.5m control 15.53m (solar panels/AC screen) or 7.1% departure from the 14.5m control | Satisfactory |

Primary Issues

| Issue | Conclusion | Section |
| --- | --- | --- |
| Traffic and parking | Council’s Traffic Engineer has determined that the proposal is acceptable, subject to conditions requiring the allocation of on-site parking spaces for staff, no parking on Albemarle Ave adjacent to the intersection, an updated Green Travel Plan, an updated Plan of Management in relation to delivery vehicles and a Construction Traffic Management Plan (CTMP) in relation to construction vehicles. | 18.3 |
| Streetscape impact of the substation | The plans were amended to set back the electrical substation 0.8m from the front boundary and moveable planter boxes have been provided as screening. The amended proposal is considered acceptable. | 18 |
| Hours of operation | Condition G.1 limits the hours of operation to 7am – 10pm everyday. Subject to conditions, the hours of operation are consistent with similar developments within the Rose Bay South centre. | 18 |
| Tree protection | Additional information has been provided to indicate that the four mature street trees within the Albemarle Avenue and Old South Head Road streetscape can be retained. Subject to the Root Mapping Report, Pruning and Impact Statement and conditions requiring a setback between the proposed awnings and the trees, the proposal is considered acceptable. | 18 |
| Operational management | The implementation of a trolley locking system, limited hours of operation for the supermarket and loading dock, traffic and parking management have all been addressed by conditions of consent. | 18 |

Summary of Submissions

| Issue | Conclusion | Section |
| --- | --- | --- |
| Height non-compliance | The proposal involves a 7.1% non-compliance with the height control, which relates to a small portion of the roof, the solar panels, lift overrun and AC screen. The areas of non-compliance do not result in any adverse amenity or streetscape impacts. This is discussed further within the report. | 17 |
| Bulk and scale is incompatible with the local character Overdevelopment of the site | The density, bulk and scale of the proposed development is consistent with the specific DCP controls for the site and is compatible with the desired future character of the Rose Bay South Mixed Use Centre. | 18 |
| A Woolworths metro would be more appropriate. The size of the Woolworths in this residential area is excessive No need for additional Woolworths (due to existing Woolworths, Harris Farm, Parisi’s, IGA) | This is not a relevant consideration under Section 4.15 of the Environmental Planning & Assessment Act. Nevertheless, the proposal is considered acceptable with regard to maintaining an active ground floor retail frontage and providing a use which will service the needs of the local community. | - |
| Privacy impacts to the neighbouring properties | The proposal is considered to maintain the privacy of the neighbouring properties. This is discussed under Section 18.2 of the report. | 18 |
| Solar access impacts to the neighbouring properties | The proposal maintains solar access to the neighbouring properties. This is discussed under Sections 10 and 18.2 of the report. | 18 |
| The proposed northern elevation results in the balcony at 7/494 Old South Head Road being bricked up/enclosed | This issue has been reviewed by the Urban Design Officer and the following comments are provided: - The DCP allows development to the side boundary in this location. - The balcony loses sky views which would be obstructed even if the proposal was setback from the boundary. - The impacted balcony is accessed from a secondary bedroom and is not the primary private open space. - The property has a primary north facing balcony accessed from the living room and master bedroom. | - |
| A 2.5-meter high, solid brick boundary fence should be provided between the site and 28a Albemarle Avenue | A 1.8m high palisade fence is provided along the side boundary with 28a Albemarle Avenue. This is a typical side boundary fence height and complies with the Site Specific DCP. | 18 |
| Traffic lights should be installed at the intersection of Newcastle Street and Old South Head Road, divert traffic away from Albemarle Avenue | This is beyond the scope of the Development Application. | - |
| Traffic, parking and pedestrian safety impacts including: - Impact on street parking availability - No on-site staff parking - Traffic impacts - Exacerbate existing queuing at intersections - Inadequate/inaccurate Traffic Report - Driveway access to the site should be from Old South Head Road - Exit from the carpark should be left turn only onto Albemarle Avenue - School children walking in the area - Limited access for emergency vehicles due to congestion - Construction traffic should not access Albemarle Avenue and Wilberforce Avenue during school traffic times | Council’s Traffic Engineer has determined that the proposal is acceptable, subject to a deferred commencement condition and conditions of consent. These issues are addressed under Section 18 of the report. | 18 |
| Hours of operation are excessive | Condition G.1 limits the hours of operation to 7am – 10pm everyday. Subject to conditions, the hours of operation are consistent with similar retail premises within the Rose Bay South centre. | Conditions of consent |
| The communal open space to the north-west should not be used for smoking, drinking, large gatherings | Condition G.3 has been imposed to address this issue. | Conditions of consent |
| Trees in separation zone not high enough to screen the development. 100L pots are too small, mature trees should be used | Council’s Trees and Landscaping Officer has provided the following comment with regard to replacement planting tree height; “Ten trees are proposed to be planted within the separation zone, three of which have mature dimensions of 8 metres in height or taller with a combined tree canopy greater than 40m2. The total canopy cover within the separation zone is estimated to be approximately 10% of the site area.” A 3m high Lillypilly hedge is provided along the boundary with 28A Albemarle Avenue. | 18 |
| Any street trees being removed should be replaced with mature trees | The proposal allows for the retention of four street trees (2 x brush box and 2 x Honey Locust). Only one street tree (Norfolk Island Hibiscus) is being removed, this is an exempt species and is conditioned to be replaced. | 18 |
| Root mapping should be provided for the two Weeping Fig’s within 39-41 Wilberforce Ave | The proposed fence along the side and rear boundary is the closest building work to the Fig Trees and these works are located outside of the Structural Root Zone of the Figs and are not considered a major encroachment. Any impacts from the proposed fence will be mitigated through the tree protection measures within the conditions of consent. The conditions include no level changes, hand excavation and the constriction of footings within the vicinity of the figs so that any roots 50mm or greater in diameter are retained and bridged over to avoid severing. | 18 |
| The substation should not be visible from the streetscape/adjacent to neighbouring properties | The plans were amended to set back the electrical substation 0.8m from the front boundary and moveable planter boxes have been provided as screening. The amended proposal is considered acceptable and advice has been provided that a basement substation is not feasible in the circumstances. | 18 |
| Outside lights and signage should be limited to the hours of operation | - Subject to Condition G.4 the illumination of any signage between 10pm and 7am is prohibited and all lighting must comply with the Australian Standards. | Conditions of consent |
| A Construction Management Plan (CMP) should be prepared which considers impacts to the school | The CMP states that construction deliveries will be scheduled to minimise disruption to Albemarle Avenue. Traffic controllers will be stationed on Albemarle Avenue to manage vehicle movements, particularly during peak hours. Furthermore, Council’s Traffic Engineer has imposed a condition requiring a detailed Construction Traffic Management Plan, which requires the management of construction impacts and vehicles. The conditions also require that a pedestrian crossing is provided across both driveway entries. | 18 |
| The proposal does not include any studio or 1 bedroom apartments | While there are no studio or 1 bedroom apartments, the development offers a good variety of apartment sizes (2, 3 and 4 bedrooms). | 10 |
| Truck deliveries should not occur during peak traffic times | The Plan of Management states that deliveries should occur outside of peak times (including peak school pick-up/drop-off). | 18 |
| Activities in the loading dock should occur with the door shut | The Acoustic Report states that ‘roller doors to the loading dock will be shut during its use’ and this is reinforced by conditions. Furthermore, the Acoustic states that ‘the roller door shall be selected to achieve an acoustic performance of Rw 20 & the roller door installed with no visible gaps’. | Conditions of consent |
| Delivery trucks should enter and exit the site in a forward direction | A turntable has been provided which allows delivery vehicles to enter and exit the site in a forward direction. |  |
| The loading dock and mechanical plant should be in the basement to mitigate noise Noise from the loading dock Noise from the turntable | Council’s Environmental Health Officer has noted that acoustic mitigation measures for mechanical plant and equipment are to be finalised during detailed design once equipment selections and layouts are further progressed. As such, conditions are recommended requiring the following “Before the issue of any construction certificate, an Acoustic Report, prepared by a suitably qualified acoustic consultant, shall assess the environmental noise impacts associated with the operation of all mechanical plant services. The Acoustic Report shall have regard to noise criteria of Chapter D3, Section D3.10 (Site Facilities) of the WDCP 2015, which states, ‘noise emissions from mechanical plant equipment must not exceed the background noise levels when measured at the boundary of the development site’. The cumulative noise emissions from all mechanical plant services, at all times shall comply with Chapter D3, Section D3.10 (Site Facilities) of the WDCP 2015”. | Conditions of consent |
| Dilapidation Report should be prepared for 26 Albemarle | The conditions of consent require a Dilapidation Report for 26, 28 & 28A Albemarle Avenue and 494-496 Old South Head Road. | Conditions of consent |
| Environmental impacts during construction | Conditions of consent are recommended to minimise impacts during construction. The conditions also require a site specific Construction Noise Management Plan to ensure that appropriate work practices are implemented during the demolition, excavation and construction to minimise noise and vibration impact. | Conditions of consent |
| The land should be used for a community benefit | The land is privately owned and Council is not able to stipulate that the land is used for community benefit. | - |
| The application should be refused | Subject to the amended plans and detailed conditions of consent, the application is recommended for approval. | - |
| Complaint management | This issue has been adequately addressed in the Woolworths Plan of Management October 2024. Residents also have the opportunity to lodge a Customer Service Request with Council for investigation of any noise issues. | - |

PROPERTY DETAILS AND REFERRALS

SITE AND LOCALITY

Physical features

The site comprises two lots – the existing Woolworths Direct to Boot at 488 – 492 Old South Head Road and the adjoining residential dwelling at 30 Albemarle Avenue. The site is legally described as Lot 1 in DP 1009799 and Lot 30, Section B in DP4567. The site has a total area of 2,257m² (1,560.3m² and 696.7m²) and is irregular in shape. It has a frontage of approximately 40 metres to Old South Head Road and approximately 70 metres to Albemarle Avenue.

Topography

The site is relatively flat.

Existing buildings and structures

The site is currently occupied by a Woolworths Direct to Boot Operation, which repurposed the previous Caltex Service Station building. 30 Albemarle Avenue is currently occupied by a detached dwelling house.

Surrounding Environment

The surrounding development comprises the following: - To the north of the site is 494-496 Old South Head Road, which is occupied by four-storey mixed-use development. There are retail/office premises at ground floor level and residential apartments above. - To the south of the site (on the opposite side of Albemarle Avenue) is a single storey retail development (Gaslight pharmacy) and associated carpark. - To the west of the site is 28 Albemarle Avenue which is occupied by a two storey detached dwelling. - To the east of the site (on the opposite side of Old South Head Road) are three storey residential flat buildings Rose Bay Public School is located to the west along Albemarle Avenue. Rose Bay South Mixed Use Centre is located to the north along both sides of Old South Head Road.

Figure 2 - Photomontage of the proposed development viewed from the corner of Albemarle Avenue and Old South Head Road

Figure 3 - Photomontage of the proposed development from Old South Head Road

Figure 4 - Photomontage of the proposed development from Albemarle Avenue

Figure 5 – Subject site and existing ‘direct to boot’ use

Figure 6 – Subject site

Figure 7 – Subject site as viewed from Albemarle Avenue

Figure 8 – Subject site as viewed from Old South Head Road and adjacent RFB at 494 – 496 Old South Head Rd

Figure 9 – Adjacent properties at 26 – 28a Albemarle Avenue, Rose Bay

Figure 10 – Rose Bay South Mixed Use Centre

RELEVANT PROPERTY HISTORY

Current use

Woolworths ‘Direct to Boot’ premises, defined in the Woollahra LEP 2014 as a ‘local distribution premises’

Relevant Application History

- DA502/2019 was approved on 08/09/2020 for the demolition of the existing service station and Category 2 remediation works. - DA52/2023 was approved on 19/05/2023 for repurposing of the former service station/convenience store and change of use to a ‘local distribution premises’. The application involved alterations to the existing building on site, the provision of 13 on-site parking spaces and remediation works.

Pre-DA

A pre-DA meeting was held on 13/08/2024. The information requested through the pre-DA process has been submitted with the application.

Requests for Additional Information and Replacement Applications

- On 6/11/2024 the following information was requested: - Revised Stormwater Management Plan and Drains File - Arborist Report - Aboriginal Heritage Impact Assessment This information was submitted on 2 December 2024. - On 27 February 2025 the following information was requested: - Revised Acoustic Statement - Revised Traffic Statement - Revised Landscape Plan - Revised Operational Management Plan. - Revised Design Verification Statement. - Revised Architectural Plans. - Substation location review. This information was submitted on 22 April 2025.

REFERRALS

| Referral | Summary of Referral Response | Annexure |
| --- | --- | --- |
| Development Engineering | Acceptable, subject to conditions. | 5 |
| Traffic Engineering | Acceptable, subject to conditions. | 6 |
| Trees and Landscaping | Acceptable, subject to conditions. | 7 |
| Heritage | Acceptable, subject to conditions. | 8 |
| Urban Design | Acceptable. | 9 |
| Environmental Health | Acceptable, subject to conditions. | 10 |
| Drainage Engineering | Acceptable, subject to conditions. | 11 |
| Transport for NSW (TfNSW) | The proposed development will increase the number of road users turning right from Old South Head Road into Albemarle Avenue to access the Woolworths carpark. Cars driving southbound on Old South Head Road will be unable to overtake right turning vehicles due to the street parking in the southbound kerbside lane of Old South Head Road. As such, the applicant should investigate mitigation measures, which may include restricting parking in the southbound lane of Old South Head Road (between Beaumont Street and the existing ‘No Stopping’ sign at 509 Old South Head Road), between 3-7pm weekdays. The subject parking spaces are on the Waverley Council side of the road and therefore the applicant is required to liaise with Waverley Council. This is addressed by Condition D.19. | 12 |
| Ausgrid | Acceptable, subject to conditions. | 13 |
| WaterNSW | Acceptable, subject to conditions. | 14 |

ENVIRONMENTAL ASSESSMENT UNDER SECTION 4.15

The relevant matters for consideration under Section 4.15 of the Environmental Planning and Assessment Act 1979 include the following:

- The provisions of any environmental planning instrument

- The provisions of any proposed instrument that is/has been the subject of public consultation

- The provisions of any development control plan

- Any planning agreement that has been entered into

- Any draft planning agreement that a developer has offered to enter into

- The regulations

- Any coastal zone management plan

- The likely impacts of that development:

- Environmental impacts on the natural and built environments

- Social and economic impacts

- The suitability of the site

- Any submissions

- The public interest

ADVERTISING AND NOTIFICATION

Submissions

The application was advertised and notified from 13 November 2024 to 13 December 2024 in accordance with Chapter 6 of the Woollahra Community Participation Plan 2019. Submissions were received from:

- David Sanday, 19 Liverpool Street, Rose Bay

- Barbara Todes, 2/30 Liverpool Street, Rose Bay

- Tracy Newfield, 26 Albemarle Avenue, Rose Bay

- Julie Feller, 7/494 Old South Head Road, Rose Bay

- Thomas Hamilton-Matthews, 1/2b Liverpool Street, Rose Bay

- Frances and Hugh Dixson, 3 Albemarle Avenue, Rose Bay

- Jacquie Hilmer, 31 Albemarle Avenue, Rose Bay

- Katrina Lesnie, 33 Albemarle Avenue, Rose Bay

- Anne & David Moore, 9/6 Liverpool Street, Rose Bay

- Yianoulla Shaw & Christopher Moser, 2B Bangalla Road, Rose Bay

- Lincoln Lawler on behalf of the Department of Education, Level 8, 259 George Street GPO Box 33, Sydney

- Michele Esplin, PO Box 768, Rose Bay

- John Beck, 17 Albemarle Avenue, Rose Bay

- Tony and Esther Diamond, 23 Albemarle Avenue, Rose Bay

- Charles Beck, 31 Beaumont Street, Rose Bay

- Bary and Melisa White, 25 Beaumont Street, Rose Bay

- Alexander Armstrong, Address not provided

- Sarah Chipman, 4/26b Liverpool Street, Rose Bay

- Evelyn Palmer, 40 Old South Head Road, Vaucluse

- Georgie Beck, 17 Albemarle Ave, Rose Bay

- E. Uguccioni, 18 Faraday Ave, Rose Bay

- Jamie Freed, 1/2A Liverpool St, Rose Bay

- Lyndall Holman and Ron Norman, 70 Newcastle St Rose Bay

- Jozef Zlekha, 15/494-496 Old South Head Road, Rose Bay

- Aviva Levine, Address not provided

- Rosemary Marando, 28a Albemarle Avenue, Rose Bay

- Vicky Gonda and Terry Wengdal, Address not provided

- Lucinda Regan, Residents First Bellevue Hill Ward Councillor, Address not provided

- Merrill Witt, Woollahra Councillor for the Vaucluse Ward, Address not provided

- Marissa Gold, Address not provided

- Isa Harris, 20/9-11 Princess St, Rose Bay.

- Florette Cohen, Address not provided

- Belinda Skurnik, 25 Albemarle Avenue Rose Bay

- Simon Anderson, 12 Albemarle Avenue, Rose Bay

- Kerry Dreyer, 26 Faraday Ave, Rose Bay

- Frank Femia, 28 Albemarle Avenue, Rose Bay

- Mark Skurnik, 25 Albemarle Avenue, Rose Bay

- Marcel & Bonnie Collignon, 1 Albemarle Ave, Rose Bay

- Lee Dreyer, 75 Dover Road, Rose Bay

- Sue and Stephen Cassrels, 117 Dover Road, Rose Bay

- Chris, Address not provided

- Monica Shor, 12a Beaumont Street, Rose Bay

- Rachael Scharrer, 16 Albemarle Avenue, Rose Bay

- Stephanie Phillips, 22 Liverpool Street, Rose Bay

- Juliet Ingham, 50 Newcastle St, Rose Bay

- Letter of support – name details withheld

Replacement Application

The additional information and design amendments noted in Section 4 were not renotified to surrounding residents and previous objectors under Schedule 1 of the Woollahra Community Participation Plan 2019 because the proposal, as amended, will have no greater impacts than the previously advertised application.

Statutory Declaration

The applicant has completed the statutory declaration dated 23/06/2025 declaring that the site notice for DA413/2024/1 was erected and maintained during the notification period in accordance with Schedule 1 of the Woollahra Community Participation Plan 2019.

STATE ENVIRONMENTAL PLANNING POLICY (HOUSING) 2024

The following assessment is provided against the SEPP (Housing) 2024 and the Apartment Design Guide. The comments have been extracted from the review undertaken by Council’s Consultant Urban Designer.

SEPP (Housing) 2021 Chapter 4

Environmental Planning and Assessment Regulation 2021

| Part 3 Clause 29 Design Verification Statement | Comment | Complies |
| --- | --- | --- |
| (1) A development application that relates to residential apartment development must be accompanied by a statement by a qualified designer. (2) The statement must— (a) verify that the qualified designer designed, or directed the design of, the development, and (b) explain how the development addresses— (i) the design principles for residential apartment development, and (ii) the objectives in Parts 3 and 4 of the Apartment Design Guide. | The Design Verification Statement provided in the development application is dated 18 September 2024 It was prepared prior to the date of the submitted plans. The accompanying ADG Compliance Table is incomplete and has been prepared by the author of the Statement of Environmental Effects. It must be prepared by the qualified designer. | No. Design statement and ADG compliance statement need to be updated to address plans and prepared by designer. |

Assessment Against Schedule 9 Design Principles

| Schedule 9 Principle & Statement | Comment | Complies |
| --- | --- | --- |
| Principle 1: Context and Neighbourhood Character (1) Good design responds and contributes to its context, which is the key natural and built features of an area, their relationship and the character they create when combined and also includes social, economic, health and environmental conditions. (2) Responding to context involves identifying the desirable elements of an area’s existing or future character. (3) Well designed buildings respond to and enhance the qualities and identity of the area including the adjacent sites, streetscape and neighbourhood. (4) Consideration of local context is important for all sites, including sites in the following areas— (a) established areas, (b) areas undergoing change, (c) areas identified for change. | The site of the proposed development is located in a precinct zoned and proposed to be zoned for mixed use shop housing development. It enjoys convenient access to, and supports, complementary commercial activities to serve its residential locality. Neighbourhood character is defined by essentially three eras: when the suburb was established post 1900, the 1930s inter war period; and the current era of contemporary infill development where opportunities exist. The site presents opportunities for site planning that are capitalised upon. The close proximity of the neighbouring apartment buildings to the side boundary has been considered in site planning and architectural design. | Yes. |
| Principle 2: Built Form and Scale (1) Good design achieves a scale, bulk and height appropriate to the existing or desired future character of the street and surrounding buildings. (2) Good design also achieves an appropriate built form for a site and the building’s purpose in terms of the following— (a) building alignments and proportions, (b) building type, (c) building articulation, (d) the manipulation of building elements. (3) Appropriate built form— (a) defines the public domain, and (b) contributes to the character of streetscapes and parks, including their views and vistas, and (c) provides internal amenity and outlook | The proposed building envelope scale, bulk and height are greater than the existing buildings. | Yes |
| However, the building envelope height is generally consistent with those of surrounding buildings and while the bulk and scale are also greater it provides an appropriate address and contribution to the public domain within the confines of its infill siting. The approach to the architectural design is an appropriate response to the fragmented historic streetscape character of the road. The design of ground floor commercial frontages, upper level elevations, signage and siting of setbacks appropriately define the public domain and contribute to the centre’s character. The building design prioritises its frontages and streetscape character. While the proposed driveways in Albermarle are extensive, they are an improvement on the existing driveways of the former service station. Configuration of living areas provides good internal amenity and outlook. | Yes. | Yes |
| Principle 3: Density (1) Good design achieves a high level of amenity for residents and each apartment, resulting in a density appropriate to the site and its context. (2) Appropriate densities are consistent with the area’s existing or projected population. (3) Appropriate densities are sustained by the following— (a) existing or proposed infrastructure, (b) public transport, (c) access to jobs, (d) community facilities, (e) the environment | The site is accessible to services and facilities. The proposed density is consistent with that in the centre. The level of amenity for the proposed apartments is good. Most apartments are dual aspect, enjoy good solar access and ventilation and have generous balcony sizes. The proposed FSR is consistent with the proposed maximum standard in the WLEP. This is discussed in Part 3.3 below. | Yes. |
| Principle 4: Sustainability (1) Good design combines positive environmental, social and economic outcomes. (2) Good sustainable design includes— (a) use of natural cross ventilation and sunlight for the amenity and liveability of residents, and (b) passive thermal design for ventilation, heating and cooling, which reduces reliance on technology and operation costs. (3) Good sustainable design also includes the following— (a) recycling and reuse of materials and waste, (b) use of sustainable materials, (c) deep soil zones for groundwater recharge and vegetation. | The proposal adopts a number of measures that facilitate a good response to the need for sustainability. Apartments enjoy good solar access and cross ventilation. Facilities for roof top solar are proposed. No apparent conflicts exist between the deep soil zone and stormwater infrastructure. | Yes. |
| Principle 5: Landscape (1) Good design recognises that landscape and buildings operate together as an integrated and sustainable system, resulting in development with good amenity. (2) A positive image and contextual fit of well-designed development is achieved by contributing to the landscape character of the streetscape and neighbourhood. (3) Good landscape design enhances the development’s environmental performance by retaining positive natural features that contribute to the following— (a) the local context, (b) co-ordinating water and soil management, (c) solar access, (d) micro-climate, (e) tree canopy, (f) habitat values, (g) preserving green networks. (4) Good landscape design optimises the following— (a) usability, (b) privacy and opportunities for social interaction, (c) equitable access, (d) respect for neighbours’ amenity. (5) Good landscape design provides for practical establishment and long term management. | The Landscape Plan proposes an appropriate approach to site landscaping. There is a sufficient provision of landscaping, with sufficient dimensions. | Yes. |
| Principle 6: Amenity (1) Good design positively influences internal and external amenity for residents and neighbours. (2) Good amenity contributes to positive living environments and resident well-being. (3) Good amenity combines the following— (a) appropriate room dimensions and shapes, (b) access to sunlight, (c) natural ventilation, (d) outlook, (e) visual and acoustic privacy, (f) storage, (g) indoor and outdoor space, (h) efficient layouts and service areas, (i) ease of access for all age groups and degrees of mobility. | The design of each floor plan and building siting contributes to a development that offers a good standard of amenity. A majority of apartments are dual (corner) aspect, and all enjoy direct access to an easterly or westerly aspect. There is provision of communal open space. Separation of windows addressing the northern, eastern and south eastern side boundaries ensure that appropriate levels of internal visual privacy will be achieved. Pedestrian access to the development is convenient and accessible. | Yes. |
| Principle 7: Safety (1) Good design optimises safety and security within the development and the public domain. (2) Good design provides for quality public and private spaces that are clearly defined and fit for the intended purpose. (3) Opportunities to maximise passive surveillance of public and communal areas promote safety. (4) A positive relationship between public and private spaces is achieved through clearly defined secure access points and well-lit and visible areas that are easily maintained and appropriate to the location and purpose. | The proposed development effectively addresses the public domain. Pedestrian and vehicle entrances enjoy good exposure, are legible and will be comfortable to use. | Yes. |
| Principle 8: Housing Diversity and Social Interaction (1) Good design achieves a mix of apartment sizes, providing housing choice for different demographics, living needs and household budgets. (2) Well designed residential apartment development responds to social context by providing housing and facilities to suit the existing and future social mix. (3) Good design involves practical and flexible features, including— (a) different types of communal spaces for a broad range of people, and (b) opportunities for social interaction among residents. | The proposed development offers a good variety of apartment sizes. | Yes. |
| Principle 9: Aesthetics (1) Good design achieves a built form that has good proportions and a balanced composition of elements, reflecting the internal layout and structure. (2) Good design uses a variety of materials, colours and textures. (3) The visual appearance of well-designed residential apartment development responds to the existing or future local context, particularly desirable elements and repetitions of the streetscape. | The modern architectural style displays a suitable aesthetic. | Yes. |

Apartment Design Guide Assessment

Part 3: Siting the Development

This part provides guidance on the design and configuration of apartment development at a site scale. It is to be used during the design process and in the preparation and assessment of development applications.

| Requirement Objective or Guidance | Comment | Complies |
| --- | --- | --- |
| 3A – Site analysis Responsive to opportunities and constraints of site conditions and streetscape | The proposal responds well to the local centre context of the site and surrounding development. | Yes. |
| 3B – Orientation Responsive to streetscape character while optimising solar access within the development. Overshadowing of neighbouring properties in minimised during mid-winter. Where an adjoining property does not currently receive the required hours of solar access, the proposed building ensures solar access to neighbouring properties is not reduced by more than 20%. If the proposal will significantly reduce the solar access of neighbours, building separation should be increased beyond minimums. A minimum of 4 hours of solar access should be retained to solar collectors on neighbouring buildings. | Building orientation prioritises solar access to, and outlook for, apartments and preserves solar access to adjoining developments (achieved primarily by the terraced stepping back of upper levels, proposed separation and existing separation offered by adjoining roads). While the recessed lightwell in No 494-496 is partly obscured solar access appears to be preserved. | Yes. |
| 3C – Public domain interface Transition between private and public domain is achieved without compromising safety and security. Upper level balconies and windows should overlook the public domain. Amenity of the public domain is retained and enhanced. Length of solid walls should be limited along street frontages. Terraces, balconies and courtyard apartments should have direct street entry, where appropriate. Opportunities for people to be concealed should be minimised. Substations, pump rooms, garbage storage areas and other service requirements should be located in basement car parks or out of view. Where development adjoins public parks, open space or bushland, the design positively addresses this interface. | The proposed building design, articulation, setbacks and siting present an appropriate relationship to Old South Head Road and Albermarle Avenue. No Crime Prevention Through Environmental Design (CPTED) concerns are evident. Building services are hidden and the necessary presence of fire equipment at the front boundary has been thoughtfully sited. | Yes. |
| 3D - Communal and public open space Minimum communal space area 25% of site area. Minimum 50% direct sunlight to the principal usable part of the communal open space for a minimum of 2 hours between 9am and 3pm on 21 June (mid-winter). Communal open space should have a minimum dimension of 3m, and larger developments should consider greater dimensions. Communal open space should be consolidated into a well-designed, easily identified and usable area. Where communal open space cannot be provided at ground level, it should be provided on a podium or roof. | The communal open space area exceeds 25%. Appropriate solar access is provided. | Yes. |
| 3E – Deep soil zones Deep soil zones that allow for and support healthy plant and tree growth. Site area; Min Dim.; Deep soil zone (% of site area). < 650m2; -; 7%. 650 m2 – 1,500m2; 3m. > 1,500m2; 6m. > 1,500m2 with significant existing tree cover.; 6m. | The site has an area of 2,257sqm. There is a minimum width requirement of 6 metres for a site of this size. The total areas of deep soil nominated in the development application is 228.36 sqm (architecture plans), being circa 10%. | Yes. |
| 3F – Visual privacy Minimum separation distances from buildings to side and rear boundaries: Building height; Habitable rooms and balconies; Non-habitable rooms. Up to 12m (4 storeys); 6m; 3m. Up to 25m (5-8 storeys); 9m; 4.5m. Over 25 m (+9 storeys); 12m; 6m. New development adjacent to existing buildings should provide adequate separation distances to the boundary in accordance with the design criteria. Gallery access circulation should be treated as habitable space when measuring privacy separation distances between neighbouring properties. For residential buildings next to commercial buildings, separation distances should be measured as follows: for retail, office spaces and commercial balconies use the habitable room distances. Apartment buildings should have an increased separation distance of 3m (in addition to the requirements set out in design criteria 1) when adjacent to a different zone that permits lower density residential development to provide for a transition in scale and increased landscaping. Communal open space, common areas and access paths should be separated from private open space and windows to apartments, particularly habitable room windows. Windows should be offset from the windows of adjacent buildings. | The building has a height of 4 storeys for this assessment. Generally, windows and balconies to habitable rooms address all boundaries. The locations of the northeast and southeast facing windows where they overlook the side boundaries achieve the minimum setback and separation distances. A potential overlooking corner from the balcony of Unit 205 on level 2 into the recessed light well of No. 494-496 is addressed by the presence of a nib wall, planter box and palisade screen fencing. | Yes. |
| 3G – Pedestrian access and entries Building entries and pedestrian access connects to and addresses the public domain. Access areas clearly visible from public domain. Multiple entries (including communal building entries and individual ground floor entries) should be provided to activate the street edge. The design of ground floor and underground car parks minimise level changes along pathways and entries. Pedestrian links should be direct, have clear sight lines, be overlooked by habitable rooms or private open spaces of dwellings, be well lit and contain active uses, where appropriate. | The proposed development offers a good level of connectivity, entry, access and visibility with Albermarle Avenue and Old South Head Road. | Yes. |
| 3H – Vehicle access Vehicle access points designed and located to achieve safety. Car park access should be integrated with the building’s overall facade. The width and number of vehicle access points should be limited to the minimum. Car park entry and access should be located on secondary streets or lanes where available. Garbage collection, loading and servicing areas are screened. Designed to minimise conflict with pedestrians and vehicles. Create high quality streetscapes. | The proposed development adopts a thoughtful approach to vehicle access. Streetscape and pedestrian impact /amenity from driveway access is minimised compared to the existing situation presented by the former service station use. | Yes. |
| 3J – Bicycle and car parking Car parking needs of the development provided off-street. Protrusion of car parks should not exceed 1m above ground level. Design solutions may include stepping car park levels or using split levels on sloping sites. | The proposed development provides undercover bicycle and bike storage. It is hidden from view. | Yes. |

Part 4: Designing the Building

This part addresses the design of apartment buildings in more detail. It focuses on building form, layout, functionality, landscape design, environmental performance and residential amenity. It is to be used during the design process and in the preparation and assessment of development applications.

| Requirement | Comment | Complies |
| --- | --- | --- |
| 4A – Solar and daylight access Living rooms and private open spaces of at least 70% of apartments in a building receive a minimum of 2 hours direct sunlight between 9am and 3pm at mid-winter in the Sydney Metropolitan Area. A maximum of 15% of apartments in a building receive no direct sunlight between 9am and 3pm at mid-winter. | The proposed location of apartments meets the solar and daylight criteria. The orientation of the site precludes design solutions that achieve solar access to south east sited apartments. However, the amenity offered suggests that appropriate levels of amenity are achieved, notwithstanding the lack of solar access. Given this context I consider that the proposal is acceptable. | No, but considered acceptable in the circumstances. |
| 4B – Natural ventilation At least 60% of apartments are naturally cross ventilated in the first 9 storeys. Overall depth of a cross-over or cross-through apartment does not exceed 18m, measured glass line to glass line. | The majority of apartments are corner or dual aspect and offer good opportunities for cross ventilation. | Yes. |
| 4C – Ceiling heights Measured from finished floor level to finished ceiling level, minimum ceiling heights are: Apartment; Minimum ceiling height. Habitable rooms; 2.7m. Non-habitable; 2.4m. Attic spaces; 1.8m with 30° minimum ceiling slope. Minimum floor to floor height 3.1m (4C.5). | The nominated floor to floor height is 3.2 metres which may achieve the 2.7 metre floor to ceiling height once floor materials and possible plumbing or ceiling AC ducting is installed. | Yes. |
| 4D – Apartment size and layout Apartments are required to have the following minimum internal areas: Apartment type; Minimum internal area. Studio; 35m2. 1 bedroom; 50m2. 2 bedrooms; 70m2. 3 bedrooms; 90m2. Every habitable room must have a window in an external wall with a total minimum glass area of at least 10% of the floor area of the room. Habitable room depths are limited to a maximum of 2.5 x the ceiling height. In open plan layouts (where the living, dining and kitchen are combined) the maximum habitable room depth is 8m from a window. Master bedrooms have a minimum area of 10m2 and other bedrooms 9m2 (excluding wardrobe space). A window should be visible from any point in a habitable room. Bedrooms have a minimum dimension of 3m (excluding wardrobe space). Living rooms or combined living/dining rooms have a minimum width of: Apartment type; Minimum width. 1 bedroom; 3.6m. 2 bedrooms; 4m. 3 bedroom; 4m. The width of cross-over or cross-through apartments are at least 4m internally to avoid deep narrow apartment layouts. | All apartments achieve and generously exceed the minimum area. | Yes. |
| 4E – Private open space and balconies All apartments are required to have primary balconies as follows: Apartment type; Min. width; Min. depth. 1 bedroom; 8m²; 2m. 2 bedroom; 10m²; 2m. 3+ bedroom; 12m²; 2.4m. For apartments at ground level, a private open space area shall be provided instead of a balcony with minimum area of 15m² and minimum depth of 3m. | All balconies meet the minimum area and depth requirements. | Yes. |
| 4F – Common circulation and spaces Maximum number of apartments off a circulation core on a single level is eight (8). Daylight and natural ventilation should be provided to all common circulation spaces that are above ground. Longer corridors greater than 12m in length from the lift core should be articulated. Design solutions may include: a series of foyer areas with windows and spaces for seating; wider areas at apartment entry doors and varied ceiling heights. | The circulation core (essentially the lift lobby area) on each floor provides access to less than eight apartments. | Yes. |
| 4G – Storage In addition to storage in kitchens, bathrooms and bedrooms, the following storage is provided: Dwelling type; Storage size volume. Studio; 4m3. 1 bedroom; 6m3. 2 bedroom; 8m3. 3+ bedrooms; 10m3. Note: At least 50% of the required storage is to be located within the apartment. | Storage for apartments is proposed internally within units and basement. | Yes. |
| 4H – Acoustic Privacy Noise transfer is minimised through the siting of buildings and building layout. Noise impacts are mitigated within apartments through layout and acoustic treatments. Adequate building separation is provided within the development and from neighbouring buildings/adjacent uses (see also section 2F Building separation and section 3F Visual privacy). | A noise assessment is provided. The potential source of any external noise issues would most likely be from the traffic using Old South Head Road. The proposal residential use is not expected to generate any unreasonable or unexpected noise. The activities that often cause noise concerns (roof top communal areas, swimming pool, carparking and driveways/ roller doors) are generally undercover, obscured by screening/ built form, have adequate separation or are absent. This limits any potential acoustic concerns for residents in both the site and the neighbouring sites. That said, the basement car park door and loading dock are directly below Apartment 101 on level 1. Potential acoustic impact on this apartment is not addressed in the Acoustic study. It will require consideration by the noise report to ensure that noise impacts are minimised given the location of bedrooms in close proximity. Apartment sizes are large and separation between apartments is high. | No. |
| 4J – Noise and Pollution The impacts of external noise and pollution are minimised through careful siting and layout of buildings. Appropriate noise shielding or attenuation techniques for the building design, construction and choice of materials are used to mitigate noise transmission. | No air quality assessment is provided. The potential source of any air quality and external noise issues would most likely be from the traffic using Old South Head Road. Given the observed use of the road, together with the upper setbacks of the proposed building from the road, no air quality concerns are apparent. | Yes. |
| 4K – Apartment mix A range of apartment types and sizes is provided. | The proposal offers a mix of housing sizes and variety. | Yes. |
| 4L – Ground floor apartments Street frontage activity is maximised where ground floor apartments are located. Apartments deliver amenity and safety for residents. Direct street access should be provided to ground floor apartments | N/A | N/A. |
| 4M – Facades Building facades provide visual interest along the street while respecting the character of the local area. Entries are clearly defined. Building services should be integrated within the overall façade. | The approach to the design of the building façade respects the character of the area. Building services are generally not visible from the public domain. | Yes. |
| 4N – Roof design Roof treatments are integrated into the building design and positively respond to the street | The roof is not trafficable, and rooftop plant is proposed to be integrated into the building design. | Yes. |
| 4O – Landscape design Landscape design is viable and sustainable. Landscape design contributes to the streetscape and amenity. | The approach to landscape design is acceptable. | Yes. |
| 4P – Planting on structure Appropriate soil profiles are provided. Plant growth is optimised with appropriate selection and maintenance. Planting on structures contributes to the quality and amenity of communal and public open spaces | Some planting is proposed on the roof top. Soil profiles and depth details are provided for these locations. | Yes. |
| 4Q – Universal design Universal design features are included in apartment design to promote flexible housing for all community members. A variety of apartments with adaptable designs are provided. Apartment layouts are flexible and accommodate a range of lifestyle needs. Developments achieve a benchmark of 20% of the total apartments incorporating the Liveable Housing Guideline's silver level universal design features. | The private open space areas, vertical access, apartment sizes and layout and amenity of each unit generally provide a high level of flexibility to evolve as households evolve. | Yes. |
| 4R – Adaptive reuse New additions to existing buildings are contemporary and complementary and enhance an area's identity and sense of place. Adapted buildings provide residential amenity while not precluding future adaptive reuse. | The application is for a new development. | N/A |
| 4S – Mixed use Mixed use developments are provided in appropriate locations and provide active street frontages that encourage pedestrian movement. Residential levels of the building are integrated within the development, and safety and amenity are maximised for residents. | This is achieved as discussed elsewhere. | Yes |
| 4T – Awnings and signage Awnings are well located and complement and integrate with the building design. Signage responds to the context and desired streetscape character. | Awnings at street level are proposed and signage is of the appropriate scale and form. | Yes. |
| 4U – Energy efficiency Development incorporates passive environmental design. Development incorporates passive solar design to optimise heat storage in winter and reduce heat transfer in summer. Adequate natural ventilation minimises the need for mechanical ventilation. | The proposed development offers high levels of natural ventilation and there are opportunities for rooftop solar provision. The proposal satisfies the relevant objectives or design criteria prescribed by this Part. | Yes. |
| 4V – Water management and conservation Potable water use is minimised. Urban stormwater is treated on site before being discharged to receiving waters. Flood management systems are integrated into site design. | The Stormwater Plan provides no information to demonstrate appropriate rainwater collection and reuse. | No. Subject to condition. |
| 4W – Waste management Waste storage facilities are designed to minimise impacts on the streetscape, building entry and amenity of residents. Domestic waste is minimised by providing safe and convenient source separation and recycling. | Arrangements for waste management are proposed. | Yes |
| 4X – Building maintenance Building design detail provides protection from weathering. Systems and access enable ease of maintenance. Material selection reduces ongoing maintenance costs. | While no information has been provided with regards to the building maintenance, I consider the proposed materials selected, will not require excessive maintenance. | Yes. |

Assessment Officer’s comment:

The Urban Design referral raised the following issues:

- The Design Statement and ADG Compliance Statement need to be updated to address the most recent set of plans.

- The Acoustic Report should consider the impact to Apartment 101 on level 1, which is directly above the basement carpark door and loading dock.

- A condition should be imposed to ensure that the Stormwater Plan demonstrates appropriate rainwater collection and reuse.

- An amended survey should provide additional information on window locations at No. 39-41 Wilberforce Avenue and the recessed light well/balconies No. 494-498 Old South Head Road.

These issues were addressed by the provision of amended plans and additional information and the following comments are provided:

- The Design Verification Statement has been updated in conjunction with the current plan revisions.

- As requested, the ADG Compliance Table has now been prepared by the architect.

- Based on the plans, it is evident that the development on the northern boundary adjoins the balcony/light well at 7/494 Old South Head Road. This is unavoidable given the DCP allows development to have a nil side setback from the boundary in this location. Furthermore, the impacted balcony is accessed from a secondary bedroom and is not the primary private open space and the property has a primary north facing balcony accessed from the living room and master bedroom.

Overall, the proposed building design, articulation and setbacks present an appropriate relationship to Old South Head Road and Albermarle Avenue. The design of the ground floor commercial frontage and upper level elevations appropriately define the public domain and contribute to the centre’s character. The building provides a supermarket that will meet the needs of the local community, and the residential apartments are provided with good internal amenity.

- STATE ENVIRONMENTAL PLANNING POLICY ( Sustainable Buildings) 2022

SEPP (Sustainable Buildings) 2022 applies to the proposed development. It relates to commitments within the proposed development in relation to thermal comfort, water conservation and energy efficiency sustainability measures.

The development application was accompanied by a BASIX Certificate demonstrating compliance with the SEPP. These requirements are imposed by standard condition.

- STATE ENVIRONMENTAL PLANNING POLICY (TRANSPORT AND INFRASTRUCTURE) 2021

In accordance with Schedule 3 of the Transport and Infrastructure SEPP, the proposed land use requires referral to Transport for NSW as the site is located on Old South Head Road (which is identified as a State registered Classified Road) and is over 500m² in gross floor area. The application has been referred, as noted in Section 8 of the report.

Part 2.119 ‘Development with frontage to classified road’ and Part 2.120 ‘Impact of road noise or vibration on non-road development’ under the SEPP (Transport and Infrastructure 2021) are relevant to the assessment of the application.

Clause 2.119(2) requires the following:

(2)  The consent authority must not grant consent to development on land that has a frontage to a classified road unless it is satisfied that—

(a)  where practicable and safe, vehicular access to the land is provided by a road other than the classified road, and

(b)  the safety, efficiency and ongoing operation of the classified road will not be adversely affected by the development as a result of—

(i)  the design of the vehicular access to the land, or

(ii)  the emission of smoke or dust from the development, or

(iii)  the nature, volume or frequency of vehicles using the classified road to gain access to the land, and

(c)  the development is of a type that is not sensitive to traffic noise or vehicle emissions, or is appropriately located and designed, or includes measures, to ameliorate potential traffic noise or vehicle emissions within the site of the development arising from the adjacent classified road.

The following comments are provided in this regard:

- In accordance with the SEPP, the development provides vehicular access to the site from Albermarle Avenue (and not from the Classified Road).

- The proposed development will increase the number of road users turning right from Old South Head Road into Albemarle Avenue, to access the Woolworths carpark. Cars driving southbound on Old South Head Road will be unable to overtake right turning vehicles due to the street parking in the southbound kerbside lane of Old South Head Road. As such, the applicant should investigate mitigation measures, which may include restricting parking in the southbound lane of Old South Head Road (between Beaumont Street and the existing ‘No Stopping’ sign at 509 Old South Head Road), between 3-7pm weekdays.

- Subject to conditions the proposal will not have an impact on efficiency and safety of the road.

- The Acoustic Report details noise mitigation measures to be incorporated into the design of the building, which ensure that the classified road will not have an adverse impact upon the residential units.

Clause 2.120(3) requires the following:

(3)  If the development is for the purposes of residential accommodation, the consent authority must not grant consent to the development unless it is satisfied that appropriate measures will be taken to ensure that the following LAeq levels are not exceeded—

(a)  in any bedroom in the residential accommodation—35 dB(A) at any time between 10 pm and 7 am,

(b)  anywhere else in the residential accommodation (other than a garage, kitchen, bathroom or hallway)—40 dB(A) at any time.

Council’s Environmental Health Officer is satisfied that the internal residential accommodation will meet the above noise criteria, subject to the noise mitigation measures detailed in Section 6.2, 6.3 & 6.4 of the Acoustic Report being incorporated into the design of the building. This is addressed by conditions of consent.

- STATE ENVIRONMENTAL PLANNING POLICY (RESILIENCE AND HAZARDS) 2021

The provisions of SEPP (Resilience and Hazards) 2021 that are relevant to the subject site and application involve managing development in terms of the following:

- Chapter 2 Coastal Management

- Chapter 4 Remediation of land

- Chapter 2 Coastal Management

The provisions of this chapter that are relevant to the subject application involve managing development in the coastal zone and protecting the environmental assets of the coast. It is considered that the proposal, as conditioned, will not have any significant adverse environmental impact upon the harbour coastal locality and it is therefore satisfactory with regard to the relevant provisions of the planning instrument.

- Chapter 4 Remediation of land

Clause 4.6 requires that a consent authority must not consent to the carrying out of any development on land unless:

(a)  it has considered whether the land is contaminated, and

(b)  if the land is contaminated, it is satisfied that the land is suitable in its contaminated state (or will be suitable, after remediation) for the purpose for which the development is proposed to be carried out, and

(c)  if the land requires remediation to be made suitable for the purpose for which the development is proposed to be carried out, it is satisfied that the land will be remediated before the land is used for that purpose.

In accordance with Clause 4.11, the proposal involves Category 2 Remediation work (not requiring consent).

Given the history of 488 – 492 Old South Head Road as a service station, the site is identified as being potentially contaminated. The proposal involves remediation works including in-situ decommissioning of underground storage tanks and associated infrastructure and removal of lead impacted soil. 30 Albermarle has also been identified as being contaminated by uncontrolled fill and hazardous building materials.

Council’s Environmental Health Officer is satisfied that the land can be made suitable for the proposed development subject to “the preparation of an updated RAP to outline a revised data gap investigation scope, in consideration of the updated proposed development (which includes multiple basement levels) and the inclusion of 30 Albemarle Avenue. The RAP should also provide options for removal of the UST and potential contamination.” This is addressed by conditions of consent, ensuring that the development is carried out in accordance with the recommendations in revised RAP and the reporting requirements of the instrument.

Subject to conditions, the proposal is therefore acceptable with regard to SEPP (Resilience and Hazards) 2021.

- SEPP (Biodiversity and Conservation) 2021

  - Chapter 6 – Water Catchments

Chapter 6 (Water Catchments) of the SEPP applies to the subject land which is located within a regulated catchment being the Sydney Harbour Catchment.

The land is within the Sydney Harbour Catchment but is outside the Foreshores and Waterways Area and therefore only the provisions in Part 6.2 of the SEPP applies.

In deciding whether to grant development consent to development on land in a regulated catchment, matters relating to water quality and quantity, aquatic ecology, flooding, recreation and public access and total catchment management must be considered.

The proposal will have no significant adverse impacts on the Sydney Harbour Catchment, subject to standard conditions including erosion and sedimentation, stormwater and flood risk management control.

- State Environmental Planning Policy (Industry and Employment) 2021

Chapter 3 of the Industry and Employment SEPP applies to all signage that can be displayed with or without development consent and is visible from any public place or public reserve. The aims and objectives of signage are:

(a) To ensure that signage (including advertising) –

Is compatible with the desired amenity and visual character of an area, and

Provides effective communication in suitable locations, and

Is of high quality design and finish, and

(b) To regulate signage (but not content) under Part 4 of the Act, and

(c) To provide time-limited consents for the display of certain advertisements, and

(d) To regulate the display of advertisements in transport corridors, and

(e) To ensure that public benefits may be derived from advertising in and adjacent to transport corridors.

Schedule 5 of the Industry and Employment SEPP contains assessment criteria. The relevant criteria are considered below:

The proposal involves business identification and building identification signage as follows:

- ‘Woolworths’ supermarket signage x 4 (ground and first floor levels)

- Parking signage x 3

- Parking entry/exit signage

- Loading bay signage

- Residential entry signage

The details of the proposed signage are summarised in the table below:

1   Character of the area

- Is the proposal compatible with the existing or desired future character of the area or locality in which it is proposed to be located?

- Is the proposal consistent with a particular theme for outdoor advertising in the area or locality?

The proposed signage is unobtrusive, low scale and is primarily proposed at ground level. As such, the signage is compatible with the existing and desired future character of the locality.

2   Special areas

- Does the proposal detract from the amenity or visual quality of any environmentally sensitive areas, heritage areas, natural or other conservation areas, open space areas, waterways, rural landscapes or residential areas?

The proposed signage will not detract from the amenity or visual quality of the nearby residential area.

3   Views and vistas

- Does the proposal obscure or compromise important views?

- Does the proposal dominate the skyline and reduce the quality of vistas?

The subject signage does not obscure or compromise existing private or public views and does not dominate the skyline.

4   Streetscape, setting or landscape

- Is the scale, proportion and form of the proposal appropriate for the streetscape, setting or landscape?

- Does the proposal contribute to the visual interest of the streetscape, setting or landscape?

- Does the proposal protrude above buildings, structures or tree canopies in the area or locality?

The signage is integrated into the design of the building and the scale and proportion of the signage is appropriate for the building and within the streetscape. The signage is located at ground and first floor levels and does not protrude above the building or tree canopies in the area.

5   Site and building

- Is the proposal compatible with the scale, proportion and other characteristics of the site or building, or both, on which the proposed signage is to be located?

- Does the proposal respect important features of the site or building, or both?

- Does the proposal show innovation and imagination in its relationship to the site or building, or both?

The Urban Design Officer has provided the following comment in this regard: “The design of the signage is sensitive to the scale of the proposed development and the surrounding context, as the colours, size and visual appearance corresponds with the supermarket land use, residential context and the overall design characteristic of the development.” The proposed signage is compatible with the site and the subject building.

6   Associated devices and logos with advertisements and advertising structures

- Have any safety devices, platforms, lighting devices or logos been designed as an integral part of the signage or structure on which it is to be displayed?

The proposal incorporates the Woolworths logo which is appropriate given the proposed use of the site as a Woolworths supermarket.

7   Illumination

- Would illumination result in unacceptable glare?

- Would illumination affect safety for pedestrians, vehicles or aircraft?

- Would illumination detract from the amenity of any residence or other form of accommodation?

- Can the intensity of the illumination be adjusted, if necessary?

- Is the illumination subject to a curfew?

Condition G.4 prohibits the illumination of any signage between 10pm and 7am. Subject to this condition the proposal is satisfactory in relation to illumination.

8   Safety

- Would the proposal reduce the safety for any public road?

- Would the proposal reduce the safety for pedestrians or bicyclists?

Would the proposal reduce the safety for pedestrians, particularly children, by obscuring sightlines from public areas?

Given the proposal does not involve any signage that obstructs sightlines, the proposal is acceptable with regard to safety for vehicles, pedestrians (including children) and cyclists.

- State Environmental Planning Policy (Planning Systems) 2021

Under Schedule 6 of the SEPP, the proposal is defined as ‘regionally significant development’ as it has a cost of works greater than $30 million. As such, Section 2.15 of the EP&A Act applies and the functions of the consent authority under Part 4 of the Act will be exercised by the Sydney Eastern Planning Panel.

WOOLLAHRA LOCAL ENVIRONMENTAL PLAN 2014

Part 1.2: Aims of Plan

The following aims are relevant to the proposal:

(b)  to promote the management, development, conservation and economic use of property,

(c)  to provide for an appropriate balance and distribution of land for commercial, retail, residential and tourist development and for recreation, open space, entertainment and community facilities,

(d)  to provide greater population densities in and around centres that are well serviced by public transport,

(e)  to facilitate opportunities, in suitable locations, for diversity in dwelling density and type,

(g)  to protect and enhance amenity and the natural environment,

(h)  to minimise and manage stormwater and flooding impacts,

(j)  to promote a high standard of design in the private and public domain,

(k)  to minimise and manage traffic and parking impacts,

(l)  to ensure development achieves the desired future character of the area,

(m)  to minimise excavation and manage impacts, including the potential impact of construction dewatering,

(n)  to encourage the retention and planting of trees and other vegetation as part of development and minimise the urban heat island effect,

The proposal is consistent with the aims in Part 1.2(2) of the Woollahra LEP 2014 as follows:

- The subject site is currently underutilised, and the proposal facilitates the redevelopment of the site and the economic use of the land.

- The proposed development provides a balance of retail and residential development that is appropriate in this location.

- The proposed residential units increase density within a mixed use centre where a variety of services are available.

- The residential units range in size/bedrooms, thereby providing diversity in the housing stock.

- Subject to conditions, the proposal will not have an adverse impact upon the natural environment.

- Subject to conditions, the proposal will minimise and manage stormwater and flooding on the site.

- Council’s Traffic Engineer has reviewed the proposal and is satisfied in relation to traffic and parking impacts, subject to conditions.

- Subject to conditions, the proposal is satisfactory with regard to the provision of on-site car parking.

- The proposed development is consistent with the desired future character of the mixed use centre as the proposal provides a supermarket to meet the needs of the local community and contributes to upgrading the building stock with a well-designed contemporary building. The proposed four storey built form with residential units above the retail space is consistent with the desired future character.

- The proposed volume of excavation accommodates necessary retail and residential parking. Subject to conditions, the proposed excavation and dewatering is considered to be acceptable.

- The proposal allows for the retention of four existing street trees and provides replacement planting on Albemarle Avenue and within the separation zone. Council’s Trees and Landscaping Officer is satisfied that the proposed landscaping will provide adequate canopy cover and screening. Furthermore, additional planting is provided within the building face to enhance the streetscape presentation.

Land Use Table and Schedule 1, Clause 17

The proposal is defined as a shop top housing development and is permitted and is consistent with the objectives of the MU1 Mixed Use zone. While part of the site (30 Albemarle Avenue) is zoned R2, the Planning Proposal amended Schedule 1 (Clause 17) of WLEP 2014 to permit retail premises at 30 Albemarle Avenue, provided it is for the purpose of shop top housing and developed in conjunction with 488-492 Old South Head Road. Accordingly, the proposed use is permissible.

The zone objectives for the MU1 zone are as follows:

- To encourage a diversity of business, retail, office and light industrial land uses that generate employment opportunities.

- To ensure that new development provides diverse and active street frontages to attract pedestrian traffic and to contribute to vibrant, diverse and functional streets and public spaces.

- To minimise conflict between land uses within this zone and land uses within adjoining zones.

- To encourage business, retail, community and other non-residential land uses on the ground floor of buildings.

- To provide for development of a scale and type that is compatible with the amenity of the surrounding residential area.

- To ensure development is of a height and scale that achieves the desired future character of the centre.

- To maximise public transport patronage and encourage walking and cycling.

- To encourage the retention and planting of trees and other vegetation as part of development to minimise the urban heat island effect and to improve microclimates.

The proposal is acceptable with regard to the zone objectives for the following reasons:

- The proposed Woolworths supermarket contributes to the diversity of retail uses that generate employment opportunities. The proposal would provide up to 70 full and part time employment positions.

- The proposal has been designed to provide an active street frontage, by addressing the building to both the Old South Head Road and Albermarle Avenue frontages. The frontage is well articulated through brick arches, glazed walls and a curved design that responds to the corner location of the site.

- The design creates an appropriate transition to the adjacent R2 zone through the stepped design of the western elevation, whereby the upper level is setback 13m from the western boundary. The 9m separation zone also provides a landscape buffer between the mixed use development the neighbouring dwelling.

- The retail land use has been provided at ground floor level. There are no residential uses provided at ground floor level.

- As discussed further within the report, the height and scale of the development is consistent with the desired future character of the area.

- Subject to conditions, the Green Travel Plan is required to be updated to encourage staff to use modes of transport other than driving to the site.

- The proposal allows for the retention of four existing street trees and provides replacement planting on Albemarle Avenue and within the separation zone. Furthermore, additional planting is provided within the building face to enhance the streetscape presentation.

Part 4.3: Height of Buildings and Part 6.12(3) (Development at Albemarle Avenue and Old South Head Road, Rose Bay)

Part 4.3 limits development to a maximum height of 14.5m on 488-492 Old South Head Road, Rose Bay. Part 6.12(3) allows development to a height of 14.5m on 30 Albemarle Avenue when the site is developed in conjunction with 488-492 Old South Head Road for the purpose of retail and residential premises.

|  | Proposed | Control | Complies |
| --- | --- | --- | --- |
| Maximum Building Height | 14.89m (roof) 15.53m (Solar panels, lift overrun, AC screen) | 14.5m | NO |

The proposal does not comply with Part 4.3 and Part 6.12(3) of Woollahra LEP 2014 as detailed and assessed in Section 17.5.

Part 6.12(2) (Development at Albemarle Avenue and Old South Head Road, Rose Bay)

Part 6.12(2) allows a maximum Gross Floor Area of 3,720m² on 488 – 492 Old South Head Road and 480m² on 30 Albemarle Avenue. This clause applies when 30 Albemarle Avenue and 488-492 Old South Head Road are developed in conjunction and for the purpose of retail and residential premises.

| Site Area: 2,257m2 | Proposed | Control | Complies |
| --- | --- | --- | --- |
| Floor Space Ratio (whole site) | 1.8:1 (4,055.4m²) | 1.86:1 (4,200m2) | YES |

The proposal complies with the maximum floor space ratio prescribed by Part 6.12(2) of Woollahra LEP 2014. It is also acceptable with regard to the relevant Floor Space Ratio objectives under Part 4.4(1) of Woollahra LEP 2014 in the following manner:

- The density, bulk and scale of the proposed development is compatible with the desired future character of the Rose Bay South Mixed Use Centre.

- The scale of the development does not have an adverse impact upon the enjoyment of adjoining properties and the public domain.

- The proposal complies with the requirement for deep soil planting, tree canopy cover and open space.

As such, the proposal is acceptable with regard to Part 4.4(1) and Part 6.12(2) of WLEP 2014.

Part 4.6: Exceptions to Development Standards

Departure

|  | Proposed | Control | Complies |
| --- | --- | --- | --- |
| Maximum Building Height | 14.89m (roof) 15.2m – 15.53 (Solar panels, lift overrun, AC screen) | 14.5m | NO |

The proposal attains a height of 14.89m - 15.53m which involves a maximum 1.03 (7.1%) departure from the 14.5m height control under Part 4.3 and Part 6.12(3) of Woollahra LEP 2014. The height non-compliance is indicated by the area shown in white in the 3D Diagrams below.

Figure 5 - Long Section showing the maximum height of the building relative to the 14.5m height limit

- Purpose

Clause 4.6 allows for the contravention of a development standard (provided that the standard is not expressly excluded from the section), with the objectives of the clause being:

- to provide an appropriate degree of flexibility in applying certain development standards to particular development,

- to achieve better outcomes for and from development by allowing flexibility in particular circumstances.

- Applicant’s Written Request

Section 35B of the Environmental Planning and Assessment Regulation 2021 requires a development application for development that proposes to contravene a development standard to be accompanied by a document [the written request] setting out the grounds on which the applicant seeks to demonstrate the matters in clause 4.6(3) paragraphs (a) and (b).

The applicant has provided a written request (Annexure 4) in accordance with Section 35B of the Environmental Planning and Assessment Regulation 2021.

- Council’s Assessment

Clause 4.6(3) requires Council to be satisfied that the applicant has demonstrated that compliance with the development standard is unreasonable or unnecessary in the circumstances, and that there are sufficient environmental planning grounds to justify the contravention.

Council is required to undertake a critical review of the written request, and any other material that the Applicant has provided seeking to demonstrate the matters in clause 4.6(3). This is set out below.

- Step 1: Are the planning instrument, development standard and proposed variation identified in the written request accurate?

In determining whether the information contained within the written request is accurate, the following questions have been considered:

- Is the provision proposed to be varied a development standard?

- Is the development standard proposed to be varied one that can be varied, and not excluded from the operation of section 4.6 by section 4.6(8)?

- Is the correct LEP or SEPP section (and objectives if relevant) referenced?

- Is the extent of the variation correctly identified?

### Assessment:

Having regard to the definition of ‘development standards’ under the EP&A Act, Council is satisfied that the provision proposed to be varied is a development standard and is not excluded from the operation of Clause 4.6 by Clause 4.6(8).

The submitted written request titled Clause 4.6 Variation Request has accurately referenced Clause 4.3 Height of Buildings and the associated objectives under WLEP 2014 and Clause 6.12(3). The extent of variation with Clause 4.3 and Clause 6.12(3) has been accurately identified in the submitted written request.

- Step 2: Section 4.6(3)(a): Is compliance unreasonable or unnecessary?

In Wehbe v Pittwater Council (2007) 156 LGERA 446 (Wehbe), Preston CJ established five potential tests (the Wehbe test) for determining whether a development standard could be considered unreasonable or unnecessary:

- The first is to establish that compliance with the development standard is unreasonable or unnecessary because the objectives of the development standard are achieved notwithstanding non-compliance with the standard.

- The second is to establish that the underlying objective or purpose is not relevant to the development with the consequence that compliance is unnecessary.

- The third is to establish that the underlying objective or purpose would be defeated or thwarted if compliance was required with the consequence that compliance is unreasonable.

- The fourth is to establish that the development standard has been virtually abandoned or destroyed by the Council’s own actions in granting consents departing from the standard and hence compliance with the standard is unnecessary and unreasonable.

- The fifth is to establish that “the zoning of particular land” was “unreasonable or inappropriate” so that “a development standard appropriate for that zoning was also unreasonable or unnecessary as it applied to that land” and that “compliance with the standard in that case would also be unreasonable or unnecessary.

However, the five tests set out in Wehbe above are not the only ways that an applicant can demonstrate that compliance with a development standard is unreasonable or unnecessary. In Initial Action Pty Ltd v Woollahra Municipal Council [2018] 236 LGERA 256 (Initial Action), Preston CJ stated at paragraph 22:

These five ways are not exhaustive of the ways in which an applicant might demonstrate that compliance with a development standard is unreasonable or unnecessary; they are merely the most commonly invoked ways. An applicant does not need to establish all the ways. It may be sufficient to establish only one way, although if more ways are applicable, an applicant can demonstrate that compliance is unreasonable or unnecessary in more than one way.

### Assessment:

The submitted Section 4.6 Written Request to Clause 4.3 Height of Buildings and Clause 6.12(3) and the arguments provided are relevant to Test 1 – in that the objectives of the development standard are achieved notwithstanding non-compliance with the standard - are largely agreed with and supported.

An assessment against the objectives of the subject development standard is included below.

- Objectives underlying Clause 4.3 Height of Buildings are as follows:

(a)	To establish building heights that are consistent with the desired future character of the neighbourhood

(b)	To establish a transition in scale between zones to protect local amenity

(c)	To minimise the loss of solar access to existing buildings and open space

(d)	To minimise the impacts of new development on adjoining or nearby properties from disruption of views, loss of privacy, overshadowing or visual intrusion

(e)	To protect the amenity of the public domain by providing public views of the harbour and surrounding areas

The proposal is assessed against the objectives of Clause 4.3 as follows:

- As shown in the diagrams above, the non-compliance relates to part of the roof parapet, the lift overrun, screen around the AC equipment and the solar panels on the roof. These structures are primarily located in the central part of the roof, which minimises amenity impacts to the surrounding properties.

- The proposed four storey built form is consistent with the desired future character of the mixed use centre.

- The proposal creates a transition in scale between the MU1 zone and the R2 zone by incorporating a two storey built form to the streetscape and providing recessed third and fourth levels. The 9m separation/buffer zone between the site and the R2 zone also facilitates this transition.

- The areas of height non-compliance result in minimal additional overshadowing that primarily falls on the roof of the subject development or the road. The height non-compliance has a negligible impact upon solar access.

- The plant enclosure and lift overrun are centrally located on the roof and would not be highly visible from the streetscape and would not have an adverse visual impact upon the adjacent properties.

- The non-compliant height will not have an adverse impact upon the amenity of the public domain and will not impact upon any public views of the harbour and surrounding areas.

- The floor to floor heights within the development have been minimised as much as possible while still providing space for the required services.

- Step 3: Section 4.6(3)(b) Are there sufficient environmental planning grounds to justify the contravention?

In Initial Action Preston CJ provided the following guidance (at paragraphs 23 and 24) concerning whether there will be sufficient environmental planning grounds to justify a contravention of the development standard:

23	As to the second matter required by cl 4.6(3)(b), the grounds relied on by the applicant in the written request under cl 4.6 must be ‘environmental planning grounds’ by their nature: See Four2Five Pty Ltd. v Ashfield Council. The adjectival phrase “environmental planning” is not defined, but would refer to grounds that relate to the subject matter, scope and purpose of the EPA Act including the objects in s1.3 of the EPA Act.

24	The environmental planning grounds relied on in the written request under cl 4.6 must be “sufficient”. … the environmental planning grounds advanced in the written request must be sufficient “to justify contravening the development standard”. The focus of cl 4.6(3)(b) is on the aspect or element of the development that contravenes the development standard, not on the development as a whole, and why that contravention is justified on environmental planning grounds. The environmental planning grounds advanced in the written request must justify the contravention of the development standard, not simply promote the benefits of carrying out the development as a whole: see Four2Five Pty Ltd v Ashfield Council [2015] NSWCA 248 at [15].

Section 1.3 of the EPA Act reads as follows:

1.3 Objects of Act

The objects of this Act are as follows:

- to promote the social and economic welfare of the community and a better environment by the proper management, development and conservation of the State's natural and other resources,

- to facilitate ecologically sustainable development by integrating relevant economic, environmental and social considerations in decision-making about environmental planning and assessment,

- to promote the orderly and economic use and development of land,

- to promote the delivery and maintenance of affordable housing,

- to protect the environment, including the conservation of threatened and other species of native animals and plants, ecological communities and their habitats,

- to promote the sustainable management of built and cultural heritage (including Aboriginal cultural heritage),

- to promote good design and amenity of the built environment,

- to promote the proper construction and maintenance of buildings, including the protection of the health and safety of their occupants,

- to promote the sharing of the responsibility for environmental planning and assessment between the different levels of government in the State,

- to provide increased opportunity for community participation in environmental planning and assessment.

### Assessment:

To determine whether the applicant has demonstrated that there are sufficient environmental planning grounds to justify the contravention of the development standard, the following questions have been considered:

- What environmental planning grounds have been put forward to justify the variation?

- Are the environmental planning grounds specific to the proposed variation?

- Are there sufficient environmental planning grounds to justify the proposed variation to the development standard?

The applicant has provided the following environmental planning grounds to justify the variation:

- The extent of the variation is considered to be minor, particularly in the context of surrounding development and prominent corner location within the Rose Bay South Local Centre.

- The variation allows for more housing in a well-located and service area, as well as delivery of a functional supermarket to serve an identified need, making the proposal in the public interest.

- The proposed height variation does not preclude compliance with the floor space ratio (FSR) standard under the Woollahra LEP.

- The additional height will not result in adverse environmental impacts to surrounding development in regard to overshadowing, privacy and views.

- The proposed height variation will not result in adverse visual impacts from the public domain.

- The proposed development, inclusive of the variation, achieves key aims of the Woollahra LEP 2014, as well as key objects of the EP&A Act.

The environmental planning grounds forwarded by the applicant are specific to the proposed variation against Clause 4.3 Height of Buildings and Clause 6.12(3).

The written request provides explanation of how the proposed development and in particular the breach of the Height standard is reasonable and supportable in the circumstances. In doing so, the written request provides sufficient environmental planning grounds to justify contravention of the Clause 4.3 Height of Buildings development standard and 6.12(3), as it demonstrates that the proposal achieves aims (c) and (g) of Section 1.3 of the Act.

There are sufficient environmental planning grounds to justify the proposed variation to the Clause 4.3 Height of Buildings development standard and Clause 6.12(3) of the Woollahra LEP 2014.

- Conclusion

Council is satisfied that the applicant has demonstrated that compliance with the development standard is unreasonable or unnecessary in the circumstances, and that there are sufficient environmental planning grounds to justify the contravention.

Part 5.10: Heritage Conservation

Parts 5.10(2) and 5.10(4) require Council to consider the effect of works proposed to a heritage item, building, work, relic or tree, within a heritage conservation area or new buildings or subdivision in a conservation area or where a heritage item is located.

The subject site is not a heritage item and is not located within a Heritage Conservation Area. The property is located in an area of Aboriginal Sensitivity. An Aboriginal Heritage Impact Assessment has been prepared.

Council’s Heritage Officer has provided the following comment:

“Significance as per the Woollahra LEP 2014

The subject site is not a listed heritage item in Schedule 5 of the Woollahra Local Environmental Plan 2014 ‘Woollahra LEP 2014’ and is not within a heritage conservation area. The 2018 demolition report assessed the heritage significance and concluded that No. 30 Albermarle Avenue, Rose Bay forms part of the later subdivision of the Cooper Estate in Rose Bay, in the early 20th century. The dwelling constructed c.1912, is a modest example of a Federation period dwelling with later alterations and additions. The extant building was not innovative for its day in terms of layout, form siting or architectural detailing. It is not a landmark site and is located within a mixed residential setting. No person of note has been associated with the site. It does not meet the threshold for listing on the LEP (Weir Phillips, 2018: 21).

Significance of items in the vicinity

There are no listed heritage items in close proximity that would be adversely affected by the proposal. The closest heritage items are located a few blocks distance from the subject property and are:

- ‘Rose Bay Uniting Church and Wesley Hall group of buildings at 518a Old South Head Road, LEP Item No. I683

- Royal Sydney Golf Club, Kent Road, LEP Item No. I318

Significance as per the Woollahra DCP 2015

The subject site is not located within a heritage conservation area or a residential precinct outlined in the DCP.”

Subject to conditions, the proposal is acceptable with regard to the objectives in Parts 5.10 of the Woollahra LEP 2014.

  - Part 5.21: Flood Planning

The objectives of Part 5.21 are:

- to minimise the flood risk to life and property associated with the use of land,

- to allow development on land that is compatible with the flood function and behaviour on the land, taking into account projected changes as a result of climate change,

- to avoid adverse or cumulative impacts on flood behaviour and the environment,

- to enable the safe occupation and efficient evacuation of people in the event of a flood.

Development consent must not be granted to development on land the consent authority considers to be within the flood planning area unless the consent authority is satisfied the development:

- is compatible with the flood function and behaviour on the land, and

- will not adversely affect flood behaviour in a way that results in detrimental increases in the potential flood affectation of other development or properties, and

- will not adversely affect the safe occupation and efficient evacuation of people or exceed the capacity of existing evacuation routes for the surrounding area in the event of a flood, and

- incorporates appropriate measures to manage risk to life in the event of a flood, and

- will not adversely affect the environment or cause avoidable erosion, siltation, destruction of riparian vegetation or a reduction in the stability of river banks or watercourses.

In deciding whether to grant development consent on land to which Part 5.21 applies, the consent authority must consider the following matters:

- the impact of the development on projected changes to flood behaviour as a result of climate change,

- the intended design and scale of buildings resulting from the development,

- whether the development incorporates measures to minimise the risk to life and ensure the safe evacuation of people in the event of a flood,

- the potential to modify, relocate or remove buildings resulting from development if the surrounding area is impacted by flooding or coastal erosion

Council’s Drainage Engineer is satisfied that the proposal is acceptable with regard to Part 5.21 of Woollahra LEP 2014, subject to conditions requiring the following modifications to the plans:

Fencing

- The development shall be protected by a waterproof wall/facade designed to protect the development to the flood planning level,

Below Ground Car parking

- The driveway entry shall be protected by a physical crest with the threshold set to the flood planning level of 12.7m AHD,

- The loading dock pedestrian  entry shall be protected by an automatic mechanical flood barrier with the threshold set to the flood planning level of 12.8m AHD,

- The  main loading dock entry shall be protected by an automatic mechanical flood barrier with the threshold set to the flood planning level of 12.8m AHD,

- Permanent brass plaques shall be mounted adjacent to all automatic mechanical flood barriers explaining their purpose and operation,

- Emergency self-powered lights, indicting the safe exit to a flood free area above the probable maximum flood (PMF) shall be installed in the car parking area,

Floor levels

- The pedestrian entry to the lobby shall be protected by an automatic mechanical flood barrier with the threshold set to the flood planning level of 12.9m AHD,

- The  main pedestrian entry from Old South Head Road shall be protected by an automatic mechanical flood barrier with the threshold set to the flood planning level of 13.2m AHD,

- The  minor pedestrian entry from Old South Head Road shall be protected by an automatic mechanical flood barrier with the threshold set to the flood planning level of 13.2m AHD,

- Permanent brass plaques shall be mounted adjacent to all automatic mechanical flood barriers explaining their purpose and operation.

Part 6.1: Acid Sulfate Soils

Part 6.1 requires Council to consider any potential acid sulfate soil affectation so that it does not disturb, expose or drain acid sulfate soils and cause environmental damage.

Council’s Environmental Health Officer has provided the following comment in this regard:

“The site is mapped to be within 250 m of an area of low probability of occurrence of ASS, likely related to the Woollahra Golf Club, however, the site itself sits within an area of ‘no known occurrence’ of ASS.

Reference to the Woollahra Local Environmental Plan (LEP) (2014), the site is located within ‘Class 5’ land. For ‘Class 5’ land, development consent is required for works within 500 m of adjacent Class 1, 2, 3, or 4 land that is below 5 m AHD and by which the water table is likely to be lowered by 1 m AHD.

Based on the above, there is no requirement for the preparation of an Acid Sulfate Soils Management Plan.”

The proposal is therefore acceptable with regard to Part 6.1.

Part 6.2: Earthworks

Part 6.2(1) requires Council to ensure that any earthworks will not have a detrimental impact on environmental functions and processes, neighbouring uses, cultural or heritage items or features of the surrounding land.

The proposal involves excavation to accommodate to a maximum depth of about 11m for the proposed basement level car park.

Council’s Development Engineer has no objection to the proposed excavation on technical grounds, subject to the recommended conditions of consent. The following comments are made in this regard:

“A desktop Geotechnical Report by Douglas Partners, Ref: 229319.00, dated 2 October 2024, has been submitted in support of the application. The proposal involves excavation with a maximum depth of about 11m for the proposed basement level car park.

Based upon the available data, the report identified that the subsurface conditions depth of natural sand with various density from a depth beneath the fill to a depth of at least 25m. Sandstone bedrock is expected to at depths in excess of 25m and also likely to be highly variable. The sandstone could be interbedded with bands of shale and laminate. Groundwater has been measured and the bulk excavation will be about 7m below the maximum groundwater table previously measured. Temporary dewatering will be required for the basement excavation and construction.

The report made comments and recommendations on the following:

- Shoring and support,

- Vibration Monitoring,

- Excavation method,

- Further Geotechnical input.

Council’s Infrastructure & Sustainability Services Division has no objections to the proposed excavation on technical grounds.  Notwithstanding this, Council’s Planning Officer is also to undertake an assessment of the proposed excavation against the relevant excavation objectives and controls prescribed under the LEP and DCP.”

In relation to the matters for consideration under 6.2(3), the following comments are provided:

- In accordance with the advice of Council’s Development Engineer, impacts on soil stability and structural risks can be appropriately mitigated by conditions of consent;

- The proposed earthworks would not compromise the future development potential of the land;

- The approved Remediation Action Plan appropriately deals with the treatment of potentially contaminated excavated material;

- The review by Council’s Heritage Officer concluded that the site was of low archaeological potential;

- The proposal would not give rise to impacts on waterways, drinking water catchment or environmentally sensitive areas;

- Conditions of consent are recommended to mitigate the impact of the development.

Subject to conditions, the proposal is acceptable with regard to Part 6.2 of the Woollahra LEP 2014.

  - Part 6.9: Tree canopy cover in Zones R2 and R3

Part 6.9 aims to conserve and enhance tree canopy cover in the R2 Low Density Residential zone and R3 Medium Density Residential zone.

Part 6.9(3) requires Council to consider whether the development incorporates planning and design measures to enable the retention and planting of trees to minimise the urban heat island effect, and will avoid, minimise or mitigate adverse impacts on the existing tree canopy.

Council’s Trees and Landscaping Officer has provided the following comment in this regard:

“Located on the Old South Head Road and Albemarle Avenue verge areas in front of the subject site are four trees. Trees 5 and 6 are Lophostemon confertus (Brush Box) growing in Albemarle Avenue. Trees 7 and 8 are Gleditsia triacanthos cvs (Honey Locust) growing in Old South Head Road. These trees are proposed to be retained.

Ten trees are proposed to be planted within the separation zone, three of which have mature dimensions of 8 metres in height or taller with a combined tree canopy greater than 40m2. The total canopy cover within the separation zone is estimated to be approximately 10% of the site area.

An additional nine trees are proposed to be planted on the level 2 common area adding to the canopy cover of the subject site however not planted in deep soil.”

The proposal is acceptable with regard to Part 6.9 of Woollahra LEP 2014.

  - Part 6.10: Development in local centres

Part 6.10 aims to ensure the scale and function of development in local centres is appropriate for the location, and to make certain development in local centres is compatible with the desired future character and amenity of surrounding residential areas.

Part 6.10(3) requires Council to consider the impact of the development on the amenity of any surrounding residential areas, and the desired future character of the local centre. Furthermore, Council must consider whether the development is consistent with the hierarchy of centres.

The detailed assessment within this report indicates that the scale and function of the development is acceptable in this location and is compatible with the desired future character of the area and the local centre. The proposal does not have a significant adverse impact upon the amenity of the surrounding residential area, subject to conditions. The proposal is acceptable with regard to Part 6.10 of Woollahra LEP 2014.

WOOLLAHRA DEVELOPMENT CONTROL PLAN 2015

Chapter D2.4: Rose Bay South

D2.4.2 Desired Future Character

The desired future character for the mixed use precinct is as follows:

The Rose Bay South centre provides a good mix of cafés and restaurants and personal services to address the local community’s needs. As with the Rose Bay North mixed use centre, the existing building stock is due for upgrading or replacement. The focus for future development is on upgrading or replacing the existing building stock with well designed contemporary buildings. These will take the form of four storey mixed use development containing residential or office space above ground floor retailing.

The proposal provides a supermarket to meet the needs of the local community and contributes to upgrading the building stock with a well-designed contemporary building. The proposal has a four storey form with residential units above the retail space, which is consistent with the desired future character.

The proposal meets the streetscape character and key elements of the precinct and desired future character objectives of the Rose Bay South precinct.

D2.4.3 Objectives and Controls

Chapter G9 overrides the objectives and controls within this section of the DCP.

Chapter G9 – Site Specific Controls for 488 – 492 Old South Head Road & 30 Albemarle Avenue, Rose Bay

This site specific chapter applies to 488-492 Old South Head Road and 30 Albemarle Avenue, if both lots are developed together.

G8.1.4 Objectives

The objectives of this chapter are to ensure that development on the site:

- O1 Supports growth of the Rose Bay South centre through provision of a new supermarket and new dwellings.

- O2 To positively contribute to the architectural design and character in the Rose Bay South centre.

- O3 Provides active frontages to Old South Head Road and Albemarle Avenue.

- O4 Incorporates a high standard of landscape design to encourage urban greening, contribute to the streetscape, and soften the visual impact of the built form.

- O5 Has a scale, bulk and design that respects the adjoining and nearby residences and the low scale streetscape character of Albemarle Avenue, particularly in terms of the building setback, articulation and transition of building height.

- O6 Does not unreasonably compromise the amenity of adjoining and nearby residences having regard to visual and acoustic privacy, landscape setting, and separation between the building and 28A Albemarle Avenue.

- O7 Minimises traffic and parking impacts on Old South Head Road, Albemarle Avenue and surrounding streets.

The proposal is acceptable with regard to the objectives for the following reasons:

- The proposed supermarket and new residential dwellings will support the growth of the Rose Bay South Centre.

- The proposed building design, articulation, setbacks and siting present an appropriate architectural design to Old South Head Road and Albermarle Avenue.

- The proposal provides an active street frontage to both Old South Head Road and Albemarle Avenue. This is achieved through large glazed areas allowing visibility to the retail premises, which is accessible from the Old South Head Road frontage.

- The integrated planter boxes will soften the visual presentation of the built form and the retention of the street trees and planting of a new street tree will contribute to the streetscape.

- The proposal provides a 9m – 13.5m separation from the low scale dwellings on Albemarle Avenue. The separation, setbacks and articulation provide an appropriate transition.

- The proposal does not have an adverse impact upon privacy to the neighbouring dwellings. The ADG requires a 6m separation between habitable rooms and balconies and the boundaries. The proposed balconies and windows are separated from the north-eastern and north-western boundaries by 9m – 13.5m, thereby maintaining visual privacy. Planter boxes are also integrated into the design of the parapets, to minimise lines of sight.

- The proposal does not have an adverse impact upon neighbouring amenity with regard to solar access. Additional overshadowing primarily falls on the roof of the subject development or the road. Between 9am-10am on 21 June, the proposal results in some additional overshadowing to the front yards of the neighbouring dwellings on Albemarle Avenue. Overall, the additional shadowing impact to all the neighbouring dwellings is negligible.

- Council’s Traffic Officer is satisfied that the proposal is acceptable with regard to traffic and parking impacts. This is discussed further under Section 18.3.

Figure 6 - Lines of sight towards the north-western boundary

G8.2.1 Desired future character

Objectives

O1 To facilitate a built form that is consistent with the desired future character of the site and surrounding area.

Controls

C1 Development on the site must align with the following:

- A building of no more than four storeys;

- Active street frontages along Old South Head Road and Albemarle Avenue;

- Articulated facades that contribute to the visual interest of the building and the character of the local area;

- Respect the landscape character and low scale residential character of Albemarle Avenue, by providing building setbacks, height transition and a landscape setting;

- Planting and a deep soil zone to soften its interface with the adjoining residential uses on Albemarle Avenue;

- A diverse range of apartment sizes;

- A single residential entry from the Albemarle Avenue frontage only; and

- No vehicular access from Old South Head Road.

The proposal is acceptable with regard to the desired future character controls for the following reasons:

- The proposed development is four storeys, with the fourth level being set back from boundaries. The built form is consistent with the desired future character of the site and the mixed-use centre.

- The proposal provides an active street frontage to both Old South Head Road and Albemarle Avenue. This is achieved through large glazed areas allowing visibility to the retail premises, which is accessible from the Old South Head Road frontage.

- The façade is articulated through vertical forms, recesses and setbacks and varied materials.

- The development respects the low scale residential character of Albemarle Avenue by providing a 9m – 13.5m separation from the boundary with 28A Albemarle Avenue (R2 zone) and by providing a 9m wide landscape buffer zone.

- Planting within the landscape buffer and deep soil zone will improve the amenity to the neighbouring dwellings.

- The development provides a mix of 2, 3 and 4 bedroom dwellings.

- A single residential entry is provided from the Albemarle Avenue frontage only.

- In accordance with the controls, vehicular access is provided from Albemarle Avenue (and not Old South Head Road).

G8.2.2. Built Form

| Site Area: 2,257m2 | Proposed | Control | Complies |
| --- | --- | --- | --- |
| Number of storeys | 4 storeys | 4 storeys | YES |
| Building design | Contemporary building with defined horizontal elements and curved edges | Contemporary incorporating defined horizontal and vertical elements, arches and curved edges | YES |
| Setbacks from OSH Rd and Albemarle Ave | The provided setbacks are consistent with the controls | Ground Level and Level 1: 0m from the street boundary. Level 2: 0m from the street boundary with a minimum 3m articulation zone. Level 3: 3m from the street boundary with a minimum 2m articulation zone from Old South Head Rd and 2.5m articulation zone from Albemarle Ave. | YES |
| Setbacks from the north-eastern boundary | The provided setbacks are consistent with the controls | Ground Level and Level 1: 0m from the north-western boundary. Level 2: 0m from the north-western boundary (adjoins blank wall) and 10m between habitable rooms/balconies and non-habitable rooms. Level 3: 7m between non-habitable rooms and 10m between habitable rooms/balconies and non-habitable rooms and 13m between habitable rooms/balconies. | YES |
| Setbacks from the north-western boundary (adjoining Albemarle Ave) | The provided setbacks are consistent with the controls | Ground Level and Level 1: 9m to the building. Level 2: 9m to the building and 12m to any habitable rooms/balconies. Level 3: 12m to the building and 13.5m to any habitable rooms/balconies. | YES |
| Basement walls setback | 1.5m – 8.7m | 8m from NW boundary | NO (partial) |
| Awnings | Awnings provided along OSH and Albemarle retail frontages | Awnings provided along OSH and Albemarle retail frontages | YES |
| Entrances | Weather protection provided | Wet weather protection provided | YES |

The proposal is consistent with the objectives and controls within the DCP as follows:

- The built form is well defined and the design responds to the corner position of the site.

- The proposal complies with the built form controls in relation to the number of storeys and setbacks from all boundaries. The greater setback at the upper levels reduces the apparent bulk of the building.

- The design incorporates high quality finishes, a variety of materials and integrated planter boxes.

- Articulation is provided within the façade and a high level of amenity and connectivity to the streetscape is provided through the provision of awnings and pedestrian entrances.

- As discussed within the report, the proposal provides an appropriate separation from the R2 zone.

- The proposal achieves an appropriate relationship with the surrounding pedestrian areas and the retention of existing street trees will enhance streetscape amenity.

The only non-compliance relates to the setback of the basement walls and the following comments are provided:

- The excavation non-compliance relates to the western corner of the site and allows for the provision of 10 additional parking spaces (2 residential and 4 retail spaces).

- C6 states that exceptions to the 8m setback control may be considered if the minimum deep soil landscaped area and canopy cover is achieved. The proposal complies with the deep soil landscaping control and Council’s Trees Officer has determined that sufficient tree planting and canopy cover is provided.

- Council’s Development Engineer has determined that the proposed excavation is acceptable, subject to conditions.

G8.2.3 Uses

| Site Area: 2,257m2 | Proposed | Control | Complies |
| --- | --- | --- | --- |
| Retail area | 2022.8m² retail GFA | Maximum 2400m² retail GFA | YES |
| Apartment mix | 13 apartments 2, 3 and 4 bed | Minimum of 10 apartments Mix of studio/1bed, 2 bed and 3 bed units | YES NO |
| Amenity | Acceptable with regard to SEPP and ADG | Compliance with Housing SEPP and Apartment Design Guide | YES |

The proposal is consistent with the objectives and controls within the DCP as follows:

- The proposal provides a mix of residential and retail land uses that is appropriate within the Rose Bay South centre.

- The proposed supermarket will meet the needs of the local community.

- The proposal complies with GFA for retail uses.

- The Urban Design Officer is satisfied that the proposal achieves a good level of residential amenity and is acceptable with regard to the Housing SEPP and the ADG.

- The only non-compliance relates to apartment mix as the proposal does not provide any studio or 1 bedroom apartments. However the provision of 2, 3 and 4 bedroom units is considered to provide an appropriate mix of dwelling types.

G8.2.4 Ground Level Active Frontage

| Site Area: 2,257m2 | Proposed | Control | Complies |
| --- | --- | --- | --- |
| Retail uses | Ground and part of first floor level | Ground level to OSH Rd and Albemarle Ave | YES |
| Active frontages | Residential entry on Albemarle retail entry on OSH Rd | Shopfront and building entries to OSH Rd and Albemarle Ave | YES |
| Entrances | Residential entry on Albemarle retail entry on OSH Rd | Separate entrances for residential and commercial uses | YES |
| Building facade | Large areas of glazing to the street | No blank walls to the street | YES |
| Trolleys wheel locking system | Trolley locking system provided | Provided at all supermarket and carpark exit points | YES |

The proposal is consistent with the objectives and controls within the DCP as follows:

- The proposed retail use, active frontages and façade design is compatible with the desired future character of the area and responds to the site’s corner location.

- The proposal provides an active frontage at street level, with a retail premises at ground floor level, retail and residential entries and glazed areas which create an appropriate interface between the building and the street.

- The use of extensive glazing within the façade and the provision of an Old South Head Road pedestrian entrance, will improve street level activation.

- A trolley locking system will be provided in order to keep trolleys within the site, subject to conditions.

G8.2.5 Separation zone and landscaping

| Site Area: 2,257m2 | Proposed | Control | Complies |
| --- | --- | --- | --- |
| Separation zone from the north-western boundary (Albemarle Avenue) | 9m | 9m | YES |
| Separation zone landscaping | - 10.1% of site area - 8m wide - Screen planting along boundary - Screen planting provided | - Deep soil landscaping for 10% of the site area - 8m wide consolidated landscaped area - Canopy trees with 40m² canopy - Screen planting along boundary - Planting to screen the development | YES YES YES YES YES |
| Separation zone safety | - Passive surveillance - Secured by fencing - Lighting and CCTV by condition | - Incorporates active and passive surveillance - Lighting - CCTV | YES YES YES |
| Separation zone fencing | 1.8m palisade fence | 1.8m high palisade style fencing to the boundary | YES |
| Side and rear fence height | 1.8m | 1.8m | YES |
| Tree canopy | 10% | 10% of the site area | YES |
| Planting within the façade | Planters within the façade | Greens roofs, green walls or planter boxes within the façade | YES |
| Streetscape planting | Provided along OSH Rd and Albermarle Ave | Provided along OSH Rd and Albermarle Ave | YES |

The proposal provides a landscaped separation zone that is consistent with the objectives within the DCP as follows:

- The separation zone protects the amenity of the adjoining R2 zoned land, minimises the sense of enclousre to 28 Albemarle Avenue and provides a consolidated landscaped area that will enhance the streetscape.

- The separation zone incorporates deep soil landscaping, screen planting and provides tree canopy cover that will enhance the streetscape along Albemarle Avenue.

- The separation zone will be a safe environment as it is fully fenced and only accessible from within the development. Conditions have been imposed requiring the provision of lighting and CCTV.

- Planting has been integrated into the façade of the building through planter boxes.

- Stormwater runoff will be discharged to the Council’s underground drainage system by gravity.

- The proposal complies with all the DCP controls relating to the separation zone.

- G8.2.6 Vehicular Access and Servicing

| Site Area: 2,257m2 | Proposed | Control | Complies |
| --- | --- | --- | --- |
| Number of parking spaces | 19 spaces – residential 3 spaces – visitors 70 spaces – retail 15 spaces – bicycle resi. 14 spaces – bicycle ret. 9 spaces - motorbike | 19 spaces (max) – residential 3 spaces (max) – residential visitors 43 spaces (min) – retail/supermarket 14 spaces (min) – bicycle residential 29 spaces (min) – bicycle retail 6 spaces (min) - motorbike | YES |
| Location of vehicular access | Albemarle Ave | From Albemarle Avenue | YES |
| Vehicular/pedestrian access | Vehicular and pedestrian access is separate and clearly defined | Vehicular and pedestrian access is separate and clearly defined | YES |
| Bicycle parking | Located in the carpark | Located within the building | YES |
| Loading dock door | Enclosed with a door no larger than required | Enclosed with a door no larger than required | YES |
| Noise from loading dock door | Acceptable, subject to conditions | Should not exceed 5dB(a) above background | YES |
| Plan of Management | Plan of management provided | Traffic and Paring Report and Car Parking and Loading Dock Plan of Management provided | YES |

The proposal provides vehicular access that is consistent with the objectives and controls within the DCP as follows:

- Council’s Traffic Officer is satisfied that the proposal allows for the safe and efficient movement of vehicles within, entering and leaving the site.

- Pedestrian safety is maintained.

- The service entry and egress is located on Albemarle Avenue to minimise impacts to traffic flow on the classified road.

- In order to achieve a positive streetscape presentation, the parking and loading dock entrances have been integrated into the design of the façade.

- Council’s Traffic Officer is satisfied that the proposal provides adequate residential, retail and staff parking on site, subject to conditions.

- The proposal contributes towards creating a continuous active retail street frontage.

- Council’s Trees and Landscaping Officer is satisfied that the street trees along Old South Head Road will be protected and maintained by the proposal.

The proposal complies with all the DCP controls relating to vehicular access and servicing.

- G8.2.7 Site facilities

| Site Area: 2,257m2 | Proposed | Control | Complies |
| --- | --- | --- | --- |
| Mechanical equipment | Located on the roof and in basement | Location protects visual and acoustic amenity | YES |
| Electricity substation | Located within the Albemarle Ave frontage (screened) | Located within the basement | NO |

The proposal provides vehicular access that is consistent with the objectives and controls within the DCP as follows:

- Solar panels, plant and the lift overrun are centrally located on the roof, to minimise any visual impacts. A screen is provided around the services on the roof.

- Plant rooms are also located within the basement and internally at ground floor level.

- The location of the substation is contrary to O3 and C2 which requires that the substation is not visible from the street and is located within the basement. The plans were amended to set back the electrical substation 0.8m from the front boundary and moveable planter boxes have been provided as screening. The amended proposal is considered acceptable given advice has been provided that a basement substation is not feasible in the circumstances. The modified location of the substation is consistent with O1, as it will not have an negative impact upon the streetscape or the amenity of the adjoining residential dwellings.

- G8.2.8 Ecologically Sustainable Development

C1 requires that the building achieves a minimum 4 Green Star design and as-built rating.

The proposed development achieves a 4-star Green Star rating, as outlined in the ESD Report

Chapter E1: Parking and Access

Council’s Traffic Engineer has provided the following assessment:

Parking Provision

The parking provision for the proposed development has been assessed in accordance with Council’s DCP 2015 Chapter E1 Parking and Access:

Table 1: Car Parking Provision

| Residential Component | Quantity | DCP Maximum Requirement per Unit | DCP Maximum Permitted Parking | Proposed Provision |
| --- | --- | --- | --- | --- |
| 2 bedrooms | 2 | 1 | 2 | 19 |
| 3 or more bedrooms | 11 | 1.5 | 16.5 (17) |  |
| Visitors | 13 | 0.2 | 2.6 (3) | 3 |
| Total permitted |  |  | 22 | 22 |
| Non-Residential Component | Quantity | DCP Minimum Requirement per 100m2 GFA* | DCP Minimum Required Parking | Proposed Provision |
| Supermarket | 2022.8m2 | 3.5 | 42.5 (43) | 70 |
| Total required |  |  | 65 | 92 |

*Note: Parking multiplier of 0.6 applies to Rose Bay South MU1 zone

It is noted that majority of this site is zoned as MU1 which consists of a parking multiplier of 0.6 for non-residential use. It is also noted that a draft site specific DCP has been endorsed by Council where it stipulates that the parking multiplier for non-residential uses applies to the whole development site including 30 Albemarle Avenue, Rose Bay. Onsite parking requirement for this proposal is therefore calculated and shown in Table 1.

Table 2: Bicycle and Motorbike Parking Provision

| BICYCLE |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Quantity | DCP Minimum Requirement | DCP Minimum Required Parking | Proposed Provision |
| Residential Residents | 13 dwellings | 1 per dwelling | 13 | 15 + storage for each residential unit |
| Residential Visitors | 13 dwellings | 1 per 10 dwellings | 1.3 (1) |  |
| Shop & restaurant employees | 2022.8m2 | 1 per 250m2 | 8.1 (8) | 14 |
| Shop & restaurant customers | 2022.8m2 | 2 + 1 per 100m2 over 100m2 GFA | 21.2 (21) |  |
| Total required |  |  | 43 | 29 |
| MOTORBIKE |  |  |  |  |
|  | Quantity | DCP Minimum Requirement | DCP Minimum Required Parking | Proposed Provision |
| Car Spaces | 64 | 1 per 10 car spaces | 6.4 (6) | 9 |
| Total required |  |  | 6 |  |

It is clear from Table 2 that the proposal would result in a shortfall of 15 bicycle parking spaces for retail use. It is however noted that compliant provision can be readily made per the proposal car park layout. Revised architectural plans should be submitted to clearly depict sufficient onsite bicycle parking for further assessment, noting Green Travel Plan lists provision of sufficient bike parking in easily accessible and secure locations as one of the actions to promote sustainable travel and reduce private car use.

Conditions have been imposed to ensure compliant levels of parking are provided.

EV Parking

It is stipulated in E1.11 of Council’s DCP that 10% of car parking spaces, for both residential and non-residential use, should provide electric vehicle charging points to encourage and support the increased use of electric vehicles. It is therefore requested that the proposal be updated to include a minimum of two (2) electric vehicle parking spaces for residential use and seven (7) for retail use.

Staff Parking & Green Travel Plan

Additional information was requested in relation to the Plan of Management, Traffic Report and Green Travel Plan to demonstrate future staff parking, including number of staff expected to be onsite, parking generation rate of staff parking, number of parking spaces allocated to staff, etc.

Council’s Traffic Engineer provided the following further comment:

The updated traffic letter did not address the issues raised in relation to traffic modelling and future traffic performance at the intersection of Old South Head Road and Albemarle Avenue, Rose Bay. It is however considered that this can be addressed by way of conditions and a deferred commencement condition to update operational plans and provide traffic controls to the satisfaction of Council’s Engineering Services Department.

Specifically, the applicant is required to:

- Allocate sufficient parking spaces to future staff to minimise additional parking pressure on surrounding road network;

- Incorporate car share parking and car share scheme;

- Update Green Travel Plan with goals and measures accordingly to achieve these goals to be consistent with onsite parking provision;

- Update Plan of Management to:

  - place cap on the number of customers for direct-to-boot services at peak hours to be consistent with queuing analysis in the traffic statement, dated 15 April 2025;

  - request all oversized vehicles, including deliveries and service vehicles, to access and exit the site from and to Old South Head Road before turning into Albemarle Lane to minimise adverse impacts on local roads; and

  - request all oversized vehicles, including deliveries and service vehicles, to access the site outside of peak hours, including school peak hours, to reduce delays and queues.

- Provide Construction Traffic Management Plan to request all oversized vehicles accessing and exiting the site using the existing access off Old South Head Road; other vehicles accessing the site via Albemarle Avenue may only be supported outside of school peak hours.

- Liaise with TfNSW, Woollahra Council and Waverly Council on the additional parking restrictions to improve traffic performance and reduce queues approaching the intersection of Old South Head Road and Albemarle Avenue, Rose Bay.

Subject to the deferred commencement condition and the conditions of consent, the proposal is acceptable with regard to Chapter E1 of the Woollahra DCP 2015.

Chapter E2: Stormwater and Flood Risk Management

Council’s Development Engineer has provided the following comment in this regard:

The applicant has satisfactorily addressed the stormwater issue which was raised previously. As such, the submitted revised concept stormwater plans are considered acceptable in principle subject to refinements at the CC stage. Stormwater runoff will be discharged to the Council’s underground drainage system by gravity via the provision of stormwater filtration/treatment system. Condition will be imposed to ensure this filtration pit will not be subject to backwater effect from the Council’s underground drainage system.

Generally, Council’s Infrastructure & Sustainability Services Division is satisfied that adequate provision could be made for the disposal of stormwater from the land it is proposed to develop and complies with Chapter E2 “Stormwater and Flood Risk Management” DCP.

Subject to conditions, the proposal is acceptable with regard to Chapter E2 of the Woollahra DCP 2015.

Chapter E3: Tree Management

Council’s Trees and Landscaping Officer has provided the following comment in this regard:

Located on the Old South Head Road and Albemarle Avenue verge areas in front of the subject site are four trees. Trees 5 and 6 are Lophostemon confertus (Brush Box) growing in Albemarle Avenue. Trees 7 and 8 are Gleditsia triacanthos cvs (Honey Locust) growing in Old South Head Road. These trees are proposed to be retained.

Existing trees located within 30 Albemarle Avenue are of low to moderate landscape value and are proposed to be removed.

Located on the Council verge at the front of 30 Albemarle Avenue Rose Bay is a Lagunaria patersonia (Norfolk Island Hibiscus). The tree is proposed for removal as part of the proposed development. The species of tree is listed on the exempt tree species list in Chapter E.3 of Council’s Development Control Plan (DCP). A street tree replacement planting condition is included in my recommended conditions to ensure the loss of canopy cover is replaced with a tree species in accordance with Council’s Street Tree Master Plan.

Root mapping and pruning investigations have been provided by the applicant as requested in my previous referral responses dated 05/02/2025 and 05/05/2025.

The submitted Root Investigation Statement – Tree 5 and Pruning and Impact Statement written by Naturally Trees, dated 16/06/2025 provide sufficient detail of the impacts the proposed development will have on Trees 5, 6, 7 and 8 located on the Council verge areas along the frontage of the subject property.

The proposed development indicates excavation within the Structural Root Zone of Tree 5 Lophostemon confertus (Brush Box). The root mapping investigation for Tree 5 has shown small diameter woody roots that will require severing to accommodate the proposed development. Based on the information provided the number and diameter size of roots required to be severed should not adversely impact on Tree 5.

The submitted Pruning and Impact Statement identifies a percentage of the canopies of Trees 5, 6, 7 and 8 that will require pruning to accommodate the proposed development. The Pruning and Impact Statement is however incomplete by not including photos of Tree 8 however does provide a written description of the percentage of pruning required and a limit on branch diameter size to be pruned. Additionally, a written description of the pruning required for Tree 9 has not been included however photographs have been provided which identify the branches required for pruning to accommodate the proposed development. Tree 5 will be the most impacted upon by the proposed pruning. Considering the species of Tree 5 (Brush Box), the level of pruning should be tolerable and it is expected the tree should recover from the proposed pruning.

To minimise pruning of Trees 5, 6, 7 and 8 the proposed awnings around the trees should be set back a minimum of 1.5 metres from the trunks and 1st and 2nd order branches of each tree. Accordingly, the architectural plans should be modified prior to the issuing of any construction certificate to include notation of this requirement.

Should the proposed development be approved, a full tree bond is recommended to be included for Trees 5, 6, 7 and 8 for the duration of the development works.

The proposed landscape design satisfies G8.2.5 Separation zone and landscaping of the Draft Woollahra Development Control Plan 2015 (Amendment No 27). Ten trees are proposed to be planted within the separation zone, three of which have mature dimensions of 8 metres in height or taller with a combined tree canopy greater than 40m2. The total canopy cover within the separation zone is estimated to be approximately 10% of the site area.

An additional nine trees are proposed to be planted on the level 2 common area adding to the canopy cover of the subject site however not planted in deep soil.

Understory planting in the separation zone and roof top planting comprises a mix of Australian native and exotic plants and appear to be planted densely.

Acmena smithii minor (Lillypilly) is proposed to be planted along the length of the north western side boundary adjoining 28A Albemarle Avenue Rose Bay. The proposed hedge planting is expected to reach a mature height of approximately 3 metres.

Street level and façade soft landscaping is proposed along the Old South Head Road and Albemarle Avenue frontage.

Subject to conditions, the proposal is acceptable with regard to Chapter E3 of the Woollahra DCP 2015.

Chapter E4: Contaminated Land

As discussed under Section 13 of the report, the site is identified as being potentially contaminated. Council’s Environmental Health Officer has reviewed the documentation and found the application to be acceptable, subject to conditions.

The land can be made suitable for the proposed development subject to “the preparation of an updated RAP to outline a revised data gap investigation scope, in consideration of the updated proposed development (which includes multiple basement levels) and the inclusion of 30 Albemarle Avenue. The RAP should also provide options for removal of the UST and potential contamination.” This is addressed by conditions of consent, ensuring that the development is carried out in accordance with the recommendations in revised RAP and the reporting requirements of the instrument.

The proposal is therefore acceptable with regard to the objectives and controls in Chapter E4 of the Woollahra DCP 2015.

Chapter E5: Waste Management

Chapter E5 is applicable to all development and seeks to establish waste minimisation and sustainable waste management during demolition and construction phases and throughout the on-going use of the building.

The SWMMP addresses volume and type of waste and recyclables to be generated, storage and treatment of waste and recyclables on site, disposal of residual waste and recyclables and operational procedures for ongoing waste management once the development is complete.

The applicant provided a SWMMP with the development application and it was found to be satisfactory.

Part E5.7: Mixed Use Developments

|  | Proposed | Control | Complies |
| --- | --- | --- | --- |
| Commercial Waste Storage Area | Retail and residential waste are separate | Separated from Residential | YES |

C1 requires the separation of the waste storage area for the commercial component of the building from the residential component. There is a separate room provided for retail waste and residential waste, in accordance with the control.

The proposal is acceptable with regard to the objectives and controls in Chapter E5 of the Woollahra DCP 2015.

CONTRIBUTION PLANS

Contributions plans allow funds to be raised from approved development applications. The funds are used for the intended provision, extension or augmentation of public facilities, or towards recouping the cost of facilities that have been provided, extended or augmented. These contributions relate to sections 7.11 and 7.12 of the EP&A Act, formerly known as section 94 and section 94A.

Section 7.12 Contributions Plan

A 1% levy applies with the monies being used for a variety of works as outlined in Schedule 1 of the Section 7.12 Contributions Plan 2022.

Subdivision 4 Housing and Productivity Contributions

Section 7.24 of the Environmental Planning and Assessment Act 1979 identifies that the object of this subdivision is to facilitate the provision of regional infrastructure that supports and promotes housing and economic activity in a region by enabling a housing and productivity contribution to be required.

Division 1 of the Environmental Planning and Assessment (Housing and Productivity Contribution) Ministerial Order 2024 sets out the classes of development, which require a housing and productivity contribution.

The proposal does require a housing and productivity contribution, which is addressed by condition.

Application of Contributions

The consent authority cannot impose conditions under both Section 7.11 and Section 7.12 on the same development consent.

In this instance, Section 7.12 contributions have been applied. Accordingly, Section 7.11 contributions are not applicable.

APPLICABLE ACTS/REGULATIONS

Environmental Planning and Assessment Regulation 2021

Clause 61(1) Additional matters that consent authority must consider

Clause 61(1) of the EPA Regulation 2021 requires Council to take into consideration Australian Standard AS 2601-2001: The demolition of structures.  This requirement is addressed by Council’s standard condition.

THE LIKELY IMPACTS OF THE PROPOSAL

All likely impacts have been addressed elsewhere in the report, or are considered to be satisfactory and not warrant further consideration.

THE SUITABILITY OF THE SITE

The site is suitable for the proposed development for the reasons outlined within the report.

THE PUBLIC INTEREST

The proposal is considered to be in the public interest.

CONCLUSION

The proposal for a new mixed use building and Woolworths supermarket is acceptable against the relevant considerations under Section 4.15 for the following reasons:

- It is considered to be satisfactory with all relevant planning policies including the objectives of WLEP 2014 and WDCP 2015

- The proposal is generally consistent with the building envelope, built form and setbacks outlined at Planning Proposal stage and the specific controls prepared for the site under Chapter G9 of WDCP 2015

- It will not have adverse effects on the local built and natural environment nor any adverse social and economic impacts in the locality such that refusal would be warranted.

- All likely impacts to adjoining properties including any submissions made have been addressed in the report, or are considered to be satisfactory. Subject to the recommended conditions of consent, the proposal will not result in an unreasonable amenity impact to adjoining and nearby properties such that refusal would be warranted.

- The site is suitable for the proposed development as the proposed land use is permissible on the site.

- The proposal is in the public interest as it will have positive social impacts within the locality due to the provision of a supermarket, will contribute to local housing supply and will generate economic benefits.

DISCLOSURE STATEMENTS

There have been no disclosure statements regarding political donations or gifts made to any Councillor or to any council employee associated with this development application by the applicant or any person who made a submission.

RECOMMENDATION: Pursuant to Section 4.16 of the Environmental Planning and Assessment Act 1979

THAT the Sydney Eastern City Planning Panel, as the consent authority, is satisfied that the applicant has demonstrated under Part 4.6 of the Woollahra Local Environmental Plan 2014 that compliance with the Building Height development standard under Clause 4.3 and Clause 6.12(3) of Woollahra LEP 2014 is unreasonable or unnecessary in the circumstances, and there are sufficient environmental planning grounds to justify the contravention of the development standard.

AND

THAT the Sydney Eastern City Planning Panel, as the consent authority, grant development consent to Development Application No. 413/2024/1 for the demolition of the existing buildings and construction of a new four storey mixed use building comprising a Woolworths supermarket at ground floor and level 1 and 13 residential apartments at Levels 1-3, three levels of basement car parking and associated services; site remediation; earthworks; landscaping; signage and siteworks on land at 488-492 Old South Head Road ROSE BAY, subject to the following conditions:

# GENERAL CONDITIONS

Deferred Commencement - (section 4.16(3) of the Act, clause 76 of the Regulation)

Development consent is granted subject that this consent is not to operate until the Applicant satisfies the Council, in accordance with the Regulations, as to all matters specified in this condition:

- A Green Travel Plan (GTP) in accordance with Section E1.12 of the Woollahra DCP 2015 must be submitted to, and approved by, Council’s Traffic & Transport Engineers. The GTP should take the form of a control document to be implemented in the ongoing operation of the site, and must provide information including but not limited to:

- Targets and goals of plan to be consistent with onsite parking provision and minimise adverse impacts on surrounding road network in terms of parking implications and traffic flow performance.

- Strategies and measures to achieve proposed targets and goals.

- The implementation of plan, including measures intended to take and representative responsible for implementing and enforcing the plan, with a yearly review for at least five years from the operation date to assess the effectiveness of the plan and provide adjustment and improvement measures, if applicable.

- An updated Plan of Management (POM), must be submitted to, and approved by, Council’s Engineering Services Department, to provide efficient and safe environment for Direct-To-Boot customers and service vehicles accessing/egressing the site, including but not limited to:

- A maximum of 20 Direct-To-Boot customers can be permitted to book the service and attend the site within an hour window.

- All oversized vehicles (longer than 7.5m or heavier than 4.5T) accessing/egressing the site must travel from and to Old South Head Road before turning into Albemarle Avenue to minimise impact on local roads.

- All oversized vehicles (longer than 7.5m or heavier than 4.5T) attending the site must only access/egress the site outside of peak hours, including school peak hours, to reduce traffic delays and queues.

Incorporation of a car share scheme and provision of a minimum of two (2) designated car share parking spaces in order to reduce reliance on private vehicles and facilitate sustainable development. The car share parking spaces must be accessible to the general public. Documentation demonstrating this is to be submitted to, and approved by, Council’s Traffic & Transport Engineers.

Condition Reason: To ensure all parties are aware this consent does not operate until the Council is satisfied all relevant matters specified in this condition are addressed.

Conditions

Consent is granted subject to the following conditions imposed under section 4.16 of the Environmental Planning and Assessment Act 1979 (“the Act”), and the provisions of the Environmental Planning and Assessment Regulation 2021 (“the Regulations’) and the provisions of the Environmental Planning and Assessment (Development Certification and Fire Safety) Regulation 2021 (“the Development Certification and Fire Safety Regulations”), such conditions being reasonable and relevant to the development as assessed under section 4.15 of the Act.

Notes:

- Failure to comply with this development consent and any condition of this consent is a criminal offence.  Failure to comply with other environmental laws is also a criminal offence.

- Where there is any breach Council may without any further warning:

a)	Issue Penalty Infringement Notices (On-the-spot fines);

b)	Issue notices and orders;

c)	Prosecute any person breaching this consent; and/or

d)	Seek injunctions/orders before the courts to restrain and remedy any breach.

- Maximum penalties under NSW environmental laws include fines up to $1.1 Million and/or custodial sentences for serious offences.

- Should Council have to take any action to enforced compliance with this consent or other environmental laws Council’s policy is to seek from the Court appropriate orders requiring the payments of its costs beyond any penalty or remedy the Court may order.

- This consent and this specific advice will be tendered to the Court when seeking costs orders from the Court where Council is successful in any necessary enforcement action.

- The payment of environmental penalty infringement notices does not result in any criminal offence being recorded.  If a penalty infringement notice is challenged in Court and the person is found guilty of the offence by the Court, subject to section 10 of the Crimes (Sentencing Procedure) Act 1999, a criminal conviction is recorded.  The effect of a criminal conviction beyond any fine is serious.

Condition Reason: To ensure all parties are aware of the relevant legislation that applies to the development.

Definitions

Unless specified otherwise, words have the same meaning as defined by the Act, the Regulations, the Development Certification and Fire Safety Regulations and the Interpretation Act 1987 as in force at the date of consent.

Applicant means the applicant for this consent.

Approved Plans mean the plans endorsed by Council referenced by this consent as amended by conditions of this consent.

Local native plants means species of native plant endemic to Sydney’s eastern suburbs.

Owner-builder has the same meaning as in the Home Building Act 1989.

PC means the Principal Certifier under the Act.

Principal Contractor has the same meaning as in the Act, or where a Principal Contractor has not been appointed by the Owner of the land being developed Principal Contractor means the Owner of the land being developed.

Professional engineer has the same meaning as in the BCA.

Public place has the same meaning as in the Local Government Act 1993.

Road has the same meaning as in the Roads Act 1993.

SEE means the final version of the Statement of Environmental Effects lodged by the Applicant.

Site means the land being developed subject to this consent.

Site work means any work that is physically carried out on the land to which the development the subject of this development consent is to be carried out, including but not limited to building work, subdivision work, demolition work, clearing of vegetation or remediation work.

Woollahra LEP means Woollahra Local Environmental Plan 2014

Woollahra DCP means Woollahra Development Control Plan 2015

Work for the purposes of this consent means:

- the use of land in connection with development,

- the subdivision of land,

- the erection of a building,

- the carrying out of any work,

- the use of any site crane, machine, article, material, or thing,

- the storage of waste, materials, site crane, machine, article, material, or thing,

- the demolition of a building,

- the piling, piering, cutting, boring, drilling, rock breaking, rock sawing or excavation of land,

- the delivery to or removal from the site of any machine, article, material, or thing, or

- the occupation of the site by any person unless authorised by an occupation certificate.

Condition Reason: To ensure all parties are aware of the relevant definitions.

Approved Plans and Supporting Documents

Those with the benefit of this consent must carry out all work and maintain the use and works in accordance with both the architectural plans to which is affixed a Council stamp “Approved” and supporting documents listed below unless modified by any following condition.

Where the plans relate to alterations or additions only those works shown in colour or highlighted are approved.

| Reference | Description | Author | Date |
| --- | --- | --- | --- |
| Project No. P013 74 | Acoustic Report | E-LAB Consulting | 25 March 2025 |
| DA000 Rev 3 DA001 Rev 6 DA002 Rev 4 DA003 Rev 3 DA004 Rev 3 DA100 Rev 6 DA101 Rev 7 DA102 Rev 9 DA103 Rev 9 DA104 Rev 5 DA105 Rev 8 DA106 Rev 5 DA107 Rev 5 DA200 Rev 5 DA201 Rev 5 DA300 Rev 6 DA301 Rev 5 DA302 Rev 6 DA303 Rev 3 DA400 Rev 2 DA700 Rev 4 DA801 Rev 4 DA810 Rev P1 DA820 Rev P1 DA821 Rev P1 DA822 Rev P1 DA823 Rev P1 DA824 Rev P1 DA825 Rev P1 DA826 Rev P1 DA827 Rev P1 DA830 Rev P1 DA831 Rev P1 DA832 Rev P1 | Architectural Plans | PBD Architects | 14/04/2025 26/09/2024 26/09/2024 26/09/2024 26/09/2024 14/04/2025 14/04/2025 14/04/2025 14/04/2025 28/09/2024 14/04/2025 26/09/2024 26/09/2024 26/09/2024 14/04/2025 14/04/2025 26/09/2024 14/04/2025 14/04/2025 28/09/2024 28/08/2024 04/10/2024 13/01/2025 13/01/2025 13/01/2025 13/01/2025 13/01/2025 13/01/2025 13/01/2025 13/01/2025 13/01/2025 13/01/2025 13/01/2025 13/01/2025 |
| 1766799M | BASIX Certificate | NSW Department of Planning and Environment | 30 September 2024 |
| New Woolworths Rose Bay | Plan of Management | Woolworths | April 2025 |
| 240341 | ESD Report | JHA Services | 30/09/2024 |
| 229319.00 | Geotechnical Report | Douglas Partners | 02 October 2024 |
| 240961-Rev B | Stormwater Report | Northrop | 01 October 2024 |
| 240961 DA-C01.01-C DA-C04.01-C DA-C10.01-C | Stormwater Management Plan | Northrop | 02/12/2024 02/12/2024 02/12/2024 |
| Document Reference No. 229319.01.R.001 – Revision 1 | Land Contaminated Report (Report on Preliminary Site Investigation) | Douglas Partners Pty Ltd | 19 September 2024 |
| Unreferenced | Traffic Statement | JMT Consulting | 15 April 2025 |
| Plan No.s 000/E, 100/E, 101/E, 102/G, 103/E, 501/C, 502/B | Landscape Plan | Site Image Landscape Architects | C, E, G – 08/04/2025 |
|  | Arboricultural Impact Assessment and Method Statement | Naturally Trees | 02/10/2024 |
|  | Root Investigation Statement – Tree 5 | Naturally Trees | 16/06/2025 |
|  | Pruning and Impact Statement | Naturally Trees | 16/06/2025 |

Notes:

- Warning to Principal Certifier – You must always insist on sighting the original Council stamped approved plans.  You must not rely solely upon the plan reference numbers in this condition.  Should the Applicant not be able to provide you with the original copy Council will provide you with access to its files so you may review our original copy of the approved plans.

- These plans and supporting documentation may be subject to conditions imposed under section 4.17(1)(g) of the Act modifying or amending the development.

Condition Reason: To ensure all parties are aware of the approved plans and supporting documentation that applies to the development.

Ancillary Aspects of Development (section 4.17(2) of the Act)

The Owner must procure the repair, replacement or rebuilding of all road pavement, kerb, gutter, footway, footpaths adjoining the site or damaged as a result of work under this consent or as a consequence of work under this consent. Such work must be undertaken to Council's satisfaction in accordance with Council’s Specification for Roadworks, Drainage and Miscellaneous Works (2012) unless expressly provided otherwise by these conditions at the Owner’s expense.

Notes:

- This condition does not affect the Principal Contractor's or any sub-contractors obligations to protect and preserve public infrastructure from damage or affect their liability for any damage that occurs.

Condition Reason: To ensure all parties are aware of works required to public infrastructure and to ensure payment for works.

No Underpinning works

This development consent does NOT give approval to any works outside the boundaries of the subject property including any underpinning works to any structures on adjoining properties.

Condition Reason: To ensure all works are located within the boundaries of the site and to confirm that no consent is granted for underpinning works to any structures on adjoining properties.

Vehicular Access and Garaging

Driveways and vehicular access ramps shall be designed to provide adequate ground clearance to the underside of B99 vehicles. In all respects, the proposed vehicular access including any parking spaces must be designed and constructed to comply with the minimum requirements of AS2890.1, AS2890.2, AS2890.6 and the Council’s DCP.

Condition Reason: To prevent car scraping and to ensure vehicular access ramps and parking spaces are designed in accordance with the Australian Standard.

SEPP (Resilience and Hazards) 2021 – Hazards Materials Survey, Preparation of an updated Remediation Action Plan (RAP) & Waste Classification

The recommendations made in Section 10 of the Detailed Site Investigation: prepared by EI Australia – Report No. E26397.E02_Rev2 – 30 July 2024 are to be completed to satisfy clause 4.6 of the State Environmental Planning Policy

(Resilience and Hazards) 2021.

Hazards Building Materials Survey

  - Before commencement of any demolition works, a Hazardous Building Materials Survey shall be completed by a suitably qualified consultant, to confirm the presence/location of any hazardous materials within the existing building fabrics.

An asbestos clearance inspection and certificate shall be completed by a suitably qualified professional (SafeWork NSW Licensed Asbestos Assessor) following any removal of all ACM. Where clearance inspection indicates hazardous materials remain on the site, further removal and re-clearance inspection, must be undertaken.

  - Following building, pavement and vegetation removal, an inspection of the exposed surface shall be performed by a suitably qualified environmental consultant, for any visible signs of contamination. An asbestos clearance inspection and certificate shall be completed by a suitably qualified professional (SafeWork NSW Licensed Asbestos Assessor). Where (if) clearance inspection indicates the presence of hazardous materials remaining on the site, further removal and clearance inspection works must be undertaken.

Updated Remediation Action Plan (RAP)

An updated Remediation Action Plan (RAP) shall be prepared and forwarded to Council, based on the findings of the Land Contamination Report (Report on Preliminary Site Investigation): prepared by Douglas Partners Pty Ltd – Document Reference 229319.01.R.001.Rev1 – 19 September 2024.

The updated RAP is to outline a revised data gap investigation scope in consideration of the updated proposed development (which includes multiple basement levels), and the inclusion of 30 Albemarle Avenue. The RAP should also provide options for removal of the UST and potential contamination.

Waste Classification & Imported Soils

  - All waste materials are to be classified and transported to NSW EPA-licensed waste facilities by the appointed waste contractors. All tipping dockets supplied by the landfill companies are to be retained, to confirm the appropriate (lawful) disposal of wastes and included in the site validation report.

  - All soil materials that are designated for off-site disposal, including any virgin excavated natural material (VENM), shall be managed in accordance with an appropriate waste management plan, which references the POEO (Waste) Regulations 2014. The waste classification process will require representative soil sampling and analysis, in accordance with the EPA (2014) Waste Classification Guidelines.

Any material being imported to the site should be validated as suitable for the intended residential use, in accordance with NSW EPA guidelines, including imported filling landscaping material must be certified to meet the VENM classification, prior to importation.

Condition Reason: To satisfy clause 4.6 of the State Environmental Planning Policy (Resilience and Hazards) 2021.

Recording of Buildings with Little or No Heritage Significance that are to be Demolished – 30 Albermarle Avenue, Rose Bay

Prior to any site works and prior to the issue of any Construction Certificate, a photographic archival record of the residence and property at 30 Albermarle Avenue Rose Bay to be demolished is to be submitted, to the satisfaction of Council’s heritage officer.

The photographic archival recording is to be submitted in a digital format and is to include the following:

a)	A concise history of the property – which is also contained within the 2018 heritage assessment / demolition report

b)	Site plan at a scale of 1:200 (or 1:500 if appropriate) of all structures and major landscape elements including their relationship to the street and adjoining properties and directional details of photographs taken.

c)	Coloured photographs of:

each elevation,

each structure and landscape feature, and

views to the subject property from each street and laneway or public space.

Notes:

Refer to the NSW Office of Environment and Heritage website for the free publication ‘Photographic Recording of Heritage Items using Film or Digital Capture’ available at https://www.environment.nsw.gov.au/-/media/OEH/Corporate-Site/Documents/Heritage/photographic-recording-of-heritage-items-using-film-or-digital-capture.pdf

Condition Reason: To ensure existing building and landscape elements are recorded to add to our understanding of the history of Rose Bay.

Aboriginal Heritage Induction

Prior to any site works:

a)	All construction staff and contractors must be made aware of their statutory obligations for Aboriginal heritage under the National Parks and Wildlife Act 1974;

b)	An Aboriginal heritage induction is to be delivered by the La Perouse Local Aboriginal Land Council, or by a heritage consultant with Aboriginal heritage expertise (if a representative of the Local Land Council is not able to provide the induction), to explain what Aboriginal heritage may be found and outline the unexpected findings procedures; and

c)	Documentary evidence demonstrating compliance with a) and b) above must be submitted to Council and the Principal Certifier.

Condition Reason: To protect Aboriginal heritage and to comply with NPW Act 1974 and with the Woollahra LEP 2014.

Aboriginal Heritage Due Diligence Responsibilities

While site work is being carried out, nothing in this approval allows to cause harm to an Aboriginal object as defined in the National Parks & Wildlife Act 1974. Under the National Parks & Wildlife Act 1974, it is an offence to harm Aboriginal ‘objects’ (consisting of any material evidence of the Aboriginal occupation of NSW) without a valid Aboriginal Heritage Impact Permit under Section 90 of the Act. This applies whether the harm occurs either knowingly [s86(1)] or unknowingly [s86(2)].

It is a defence to the strict liability offence of harm to an Aboriginal object under s86(2) if a process of Due Diligence was followed which reasonably determined that the proposed activity would not harm an Aboriginal object.

Condition Reason: To protect Aboriginal heritage and to comply with NPW Act 1974 and with the Woollahra LEP 2014.

Heritage Fabric – 30 Albermarle Avenue, Rose Bay

Before the issue of any construction certificate, the Site Waste Minimisation and Management Plan must detail the recycling/salvaging of any significant heritage fabric from the subject site. This may include, but is not limited to, doors, windows, joinery, lighting, plumbing, bricks, stone, roofing, flooring and hardware.

Whilst the existing building is not Heritage listed, it incorporates significant heritage fabric worthy of recycling/salvaging – such as the tessellated tiles on the front verandah and any plaster ceiling roses.

Condition Reason: To ensure significant heritage fabric is recycled/salvaged.

Tree Preservation and Landscaping Works

While site work is being carried out, all landscape works must be undertaken in accordance with the approved landscape plan, arborist report, tree management plan and transplant method statement as applicable.

- The following trees must be retained:

  - Trees on Council land:

| Council Ref No | Species | Location | Dimension (metres) | Tree value |
| --- | --- | --- | --- | --- |
| 5 | Lophostemon confertus (Brush Box) | Most north western specimen along the Albemarle Avenue frontage of the site | 12 x 10 | $23,000.00 |
| 6 | Lophostemon confertus (Brush Box) | Most south eastern specimen along the Albemarle Avenue frontage of the site | 8 x 6 | $6,700.00 |
| 7 | Gleditsia triacanthos cvs (Honey Locust) | Most south western specimen along the Old South Head Road frontage of the site | 9 x 7 | $7,800.00 |
| 8 | Gleditsia triacanthos cvs (Honey Locust) | Most north eastern specimen along the Old South Head Road frontage of the site | 9 x 8 | $8600.00 |

The tree/s required to be retained must appear coloured green on the Construction Certificate plans.

- The following trees may be removed:

| Council Ref No | Species | Location | Dimension (metres) |
| --- | --- | --- | --- |
| 1 | Lagunaria patersonia (Norfolk Island Hibiscus)* | Council verge in front of 30 Albemarle Avenue Rose Bay | 9 x 7 |
| 2 | Michelia figo (Port-wine Magnolia) | Front western corner of 30 Albemarle Avenue Rose Bay | 5 x 4 |
| 3 | Celtis sinensis (Chinese Nettle)* | Front yard north western side boundary | 9 x 8 |
| 4 | X Cupressocyparis leylandii (Leyland Cypress)* | Front southern corner of 30 Albemarle Avenue Rose Bay | 8 x 6 |
| 9 | Camellia japonica (Camellia) | Front yard north western side boundary | 4 x 3 |
| 10 | Dracaena fragrans (Happy Plant) | North western side boundary of 30 Albemarle Avenue Rose Bay | 4 x 3 |
| 11 | Eribotrya japonica (Loquat tree) | North western side boundary of 30 Albemarle Avenue Rose Bay | 8 x 6 |
| 12 | Celtis sinensis (Chinese Nettle)* | Rear yard south eastern side boundary of 30 Albemarle Avenue Rose Bay | 5 x 6 |
| 13 | Cactus tree | South eastern side boundary of 30 Albemarle Avenue Rose Bay | 8 x 3 |

The tree/s that may be removed must appear coloured red on the Construction Certificate plans.

The species marked (*) is exempt from the WMC DCP 2015 and can be removed without requiring consent from Council.

- The following trees may be pruned in accordance with Australian Standard Pruning of Amenity Trees (AS 4373) and Workcover NSW Code of Practice Amenity Tree Industry, to the minimum extent necessary to provide clearance to the new development:

| Council Ref No | Species | Location | Approved pruning (extent of pruning |
| --- | --- | --- | --- |
| 5 | Lophostemon confertus (Brush Box) | Most north western specimen along the Albemarle Avenue frontage of the site | Pruning branches no greater than 130mm diameter as indicated in the Pruning and Impact Statement, written by Naturally Trees, dated 16/06/2025. No more than 20% of live foliage can be removed. |
| 6 | Lophostemon confertus (Brush Box) | Most south eastern specimen along the Albemarle Avenue frontage of the site | Pruning branches no greater than 100mm diameter as indicated in the Pruning and Impact Statement, written by Naturally Trees, dated 16/06/2025. No more than 15% of live foliage can be removed |
| 7 | Gleditsia triacanthos cvs (Honey Locust) | Most south western specimen along the Old South Head Road frontage of the site | Pruning branches no greater than 100mm diameter as indicated in the Pruning and Impact Statement, written by Naturally Trees, dated 16/06/2025. No more than 10% of live foliage can be removed |
| 8 | Gleditsia triacanthos cvs (Honey Locust) | Most north eastern specimen along the Old South Head Road frontage of the site | Pruning branches no greater than 100mm diameter as indicated in the Pruning and Impact Statement, written by Naturally Trees, dated 16/06/2025. No more than 10% of live foliage can be removed |

The tree/s required to be pruned must appear coloured blue on the Construction Certificate plans.

This consent does not permit the holder, or any other person or agent, to enter any property for the purpose of undertaking approved tree works, without the consent of the owner of the property.

Only a Council authorised contractor will be allowed to undertake the pruning of trees located on Council managed land under the instruction of Council’s Coordinator of Trees Maintenance in accordance with Council’s Tree Management Policies and AS 4373-2007 Pruning of Amenity Trees. All costs associated with the approved pruning will be the responsibility of the applicant. Contact Council’s Coordinator of Trees Maintenance on 9391 7000 to arrange the approved pruning works.

To facilitate the assessment of Council’s tree assets for pruning, a physical outline of the approved works (if not already constructed) must be provided at the time of inspection via the use of height poles or scaffolding. The outline of the approved works will enable Council’s Arboricultural Technical Officer to correctly identify the branches that require pruning to provide clearance for the approved works.

Condition Reason: To ensure all landscape works are undertaken in accordance with the approved plans and documents.

Standard Condition A.22

General Terms of Approval – Water Management Act 2000 Water Licenses and Approval (WaterNSW)

The following general terms of approval have been imposed by WaterNSW:

Dewatering

- GT0115-00001 
Groundwater must only be pumped or extracted for the purpose of temporary construction dewatering at the site identified in the development application. For clarity, the purpose for which this approval is granted is only for dewatering that is required for the construction phase of the development and not for any dewatering that is required once construction is completed.

- GT0117-00001 
A water access licence, for the relevant water source, must be obtained prior to extracting more than 3ML per water year of water as part of the construction dewatering activity.

Advisory Notes:

1. This approval is not a water access licence.

2. A water year commences on 1 July each year.

3. This approval may contain an extraction limit which may also restrict the ability to take more than 3ML per water year without further information being provided to WaterNSW. 4. Note that certain water sources may be exempted from this requirement - see paragraph17A, Schedule 4 of the Water Management (General) Regulation 2018.

- GT0118-00001 
If no water access licence is obtained for the first 3ML / year (or less) of water extracted, then, in accordance with clause 21(6), Water Management (General) Regulation 2018, the applicant must:

(a) record water taken for which the exemption is claimed, and

(b) record the take of water not later than 24 hours after water is taken, and

(c) make the record on WAL exemption form located on WaterNSW website "Record of groundwater take under exemption", and

(d) keep the record for a period of 5 years, and

(e) give the record to WaterNSW either via email to Customer.Helpdesk@waternsw.com.au or	post completed forms to - PO Box 398 Parramatta NSW 2124

(i) not later than 28 days after the end of the water year (being 30 June) in which the	water was taken, or

(ii) if WaterNSW directs the person in writing to give the record to WaterNSW on an earlier date, by that date.

- GT0119-00001 
All extracted groundwater must be discharged from the site in accordance with Council requirements for stormwater drainage or in accordance with any applicable trade waste agreement.

- GT0120-00001

The design and construction of the building must prevent:

    - any take of groundwater, following the grant of an occupation certificate (and completion of construction of development), by making any below-ground levels that may be impacted by any water table fully watertight for the anticipated life of the building. Waterproofing of below-ground levels must be sufficiently extensive to incorporate adequate provision for unforeseen high water table elevations to prevent potential future inundation;

    - obstruction to groundwater flow, by using sufficient permanent drainage beneath and around the outside of the watertight structure to ensure that any groundwater mounding shall not be greater than 10 % above the pre-development level; and

    - any elevated water table from rising to within 1.0 m below the natural ground surface.

- GT0123-00001

    - Prior to the issuing of the occupation certificate, and following the completion of the dewatering activity, and any monitoring required under the Approved Monitoring Programme, the applicant must submit a completion report to WaterNSW.

    - The completion report must, unless agreed otherwise in writing by WaterNSW, include matters set out in any guideline published by the NSW Department of Planning Industry and Environment in relation to groundwater investigations and monitoring. Where no guideline is current or published, the completion report must include the following (unless otherwise agreed in writing by WaterNSW): 
1) All results from the Approved Monitoring Programme; and 
2) Any other information required on the WaterNSW completion report form as updated from time to time on the WaterNSW website.

    - The completion report must be submitted using "Completion Report for Dewatering work form" located on WaterNSW website www.waternsw.com.au/customer-service/water- licensing/dewatering

- GT0150-00001 
Any dewatering activity approved under this approval shall cease after a period of two (2) years from the date of this approval, unless otherwise agreed in writing by WaterNSW (Term of the dewatering approval).

Advisory note: an extension of this approval may be applied for within 6 months of the expiry of Term.

- GT0155-00001 
The following construction phase monitoring requirements apply (Works Approval):

    - The monitoring bores must be installed in accordance with the number and location shown, as modified by this approval, unless otherwise agreed in writing with WaterNSW.

    - The applicant must comply with the monitoring programme as amended by this approval (Approved Monitoring Programme).

    - The applicant must submit all results from the Approved Monitoring Programme, to WaterNSW, as part of the Completion Report

- GT0174-00001 
Construction phase monitoring bore requirements GTA:

    - Monitoring bores are required to be installed and collecting data prior for at least 3 months prior to submitting a water supply work approval

    - A minimum of three monitoring bore locations are required at or around the subject property, unless otherwise agreed by WaterNSW.

    - The location and number of proposed monitoring bores must be submitted for approval, to WaterNSW and should be submitted prior to the application for a water supply work approval.

    - The monitoring bores should be used to develop a water table map for the site and its near environs.

    - The monitoring bores must be protected from construction damage. Advisory note: no approval under the Water Management Act 2000 is required for these monitoring bores provided that they extract less than 3ML/water year.

- GT0241-00001 
A construction certificate can be issued for excavation work in accordance with a valid development consent, however dewatering cannot take place without an Approval being granted by Water NSW for any water supply works required by the development. If the excavation work will or is likely to require dewatering, the applicant must apply and obtain, an approval under the Water Management Act 2000 prior to any dewatering taking place and notify WaterNSW of the programme for the dewatering activity including the commencement and proposed completion dates of the dewatering activity.

Advisory Note: An approval under the Water Management Act 2000 is required to construct and/or install the water supply works. For the avoidance of doubt, these General Terms of Approval do not represent any authorisation for the take of groundwater, nor do they constitute the grant or the indication of an intention to grant, any required Water Access Licence (WAL). A WAL is required to lawfully take more than 3ML of water per water year as part of the dewatering activity. A water use approval may also be required, unless the use of the water is for a purpose for which a development consent is in force.

Condition Reason: To ensure all parties are aware of the general terms of approval.

Ausgrid

Ausgrid Underground Cables are in the vicinity of the development.

Special care should be taken to ensure that driveways and any other construction

activities do not interfere with existing underground cables located in the footpath or

adjacent roadways. It is recommended that the developer locate and record the depth of all known underground services prior to any excavation in the area. Information regarding the

position of cables along footpaths and roadways can be obtained by contacting Before

You Dig Australia (BYDA).

In addition to BYDA the proponent should refer to the following documents to support

safety in design and construction:

- SafeWork Australia – Excavation Code of Practice.

- Ausgrid’s Network Standard NS156 which outlines the minimum requirements for

working around Ausgrid’s underground cables.

The following points should also be taken into consideration.

- Ausgrid cannot guarantee the depth of cables due to possible changes in ground levels from previous activities after the cables were installed.

- Should ground levels change above Ausgrid’s underground cables in areas such as footpaths and driveways, Ausgrid must be notified, and written approval provided

- prior to the works commencing.

- Should ground anchors be required in the vicinity of Ausgrid underground cables, the anchors must not be installed within 300mm of any cable, and the anchors must not pass over the top of any cable.

Ausgrid Overhead Powerlines are in the vicinity of the development.

The developer should refer to SafeWork NSW Document – Work Near Overhead Powerlines: Code of Practice. This document outlines the minimum separation requirements between electrical mains (overhead wires) and structures within the development site throughout the construction process. It is a statutory requirement that these distances be maintained throughout the construction phase. Consideration should be given to the positioning and operating of cranes, scaffolding, and sufficient clearances from all types of vehicles that are expected be entering and leaving the site.

The “as constructed” minimum clearances to the mains must also be maintained. These distances are outlined in the Ausgrid Network Standard, NS220 Overhead Design Manual. This document can be sourced from Ausgrid’s website at www.ausgrid.com.au.

It is the responsibility of the developer to verify and maintain minimum clearances onsite. In the event where minimum safe clearances are not able to be met due to the

design of the development, the Ausgrid mains may need to be relocated in this

instance. Any Ausgrid asset relocation works will be at the developer’s cost.

New Driveways - Proximity to Existing Poles

Proposed driveways shall be located to maintain a minimum clearance of 1.5m from the nearest face of the pole to any part of the driveway, including the layback, this is to allow room for future pole replacements. Ausgrid should be further consulted for any deviation to this distance.

New or modified connection

To apply to connect or modify a connection for a residential or commercial premises.

Ausgrid recommends the proponent to engage an Accredited Service Provider and

submit a connection application to Ausgrid as soon as practicable. Visit the Ausgrid

website for further details; https://www.ausgrid.com.au/Connections/Get-connected

Additional information can be found in the Ausgrid Quick Reference Guide for Safety

Clearances “Working Near Ausgrid Assets - Clearances". This document can be found

by visiting the following Ausgrid website: www.ausgrid.com.au/Your-safety/Working-Safe/Clearance-enquiries

Green Star Certification

The development must achieve a 4 star rating under the Green Star – Design & As Built certification. Design Review Certification must be submitted to Council and the Principal Certifier once obtained. The As Built Certification must be submitted to Council and the Principal Certifier prior to the occupation of any completed building on the site, or the use of the overall development.

# BEFORE DEMOLITION WORK COMMENCES

Construction Certificate Required Prior to Any Demolition

Where demolition is associated with an altered portion of, or an extension to an existing building the demolition of any part of a building is "commencement of erection of building" under section 6.6 of the Act.

In such circumstance all conditions included at the following development stages of this consent must be satisfied prior to any demolition work:

- Before issue of a construction certificate

- Before building work commences

This includes, but is not limited to, the issue of a Construction Certificate, appointment of a Principal Certifier, and Notice of Commencement under the Act.

Note:

- See Over our Dead Body Society Inc v Byron Bay Community Association Inc [2001] NSWLEC 125.

Condition Reason: To ensure appropriate conditions are complied with for development for the alteration and extension of an existing building.

Erosion and Sediment Controls – Installation

Before any site work commences, water pollution, erosion and sedimentation controls must be installed and maintained in accordance with:

a)	“Do it Right On Site, Soil and Water Management for the Construction Industry” and accompanying factsheets published by the Southern Sydney Regional Organisation of Councils, and

b)	“Managing Urban Stormwater - Soils and Construction” 2004 published by the NSW Government (The Blue Book).

Where there is any conflict The Blue Book takes precedence.

Notes:

- The “Do it Right On Site, Soil and Water Management for the Construction Industry” publication and accompanying factsheets can be downloaded from www.woollahra.nsw.gov.au and The Blue Book is available at www.environment.nsw.gov.au

- A failure to comply with this condition may result in penalty infringement notices, prosecution, notices and orders under the Act and/or the Protection of the Environment Operations Act 1997 without any further warning.  It is a criminal offence to cause, permit or allow pollution.

- Section 257 of the Protection of the Environment Operations Act 1997 provides inter alia that “the occupier of premises at or from which any pollution occurs is taken to have caused the pollution”.

- Warning: Irrespective of this condition any person occupying the site may be subject to proceedings under the Protection of the Environment Operations Act 1997 where pollution is caused, permitted or allowed as the result of their occupation of the land being developed.

Condition Reason: To prevent potential water pollution and dust nuisance.

Identification of Hazardous Material

Prior to any site works, and in accordance with Australian Standard AS2601: The Demolition of Structures, all hazardous substances located on the site must be identified, including asbestos, polychlorinated biphenyls (PCBs), lead paint, underground storage tanks, chemicals, etc.

In this regard, prior to any site works, Council must be provided with a written report prepared by a suitably qualified competent person detailing:

- all hazardous materials identified on the site,

- the specific location of all hazardous materials identified,

- whether the hazardous materials are to be removed from the site as part of the works to be undertaken, and

- safety measures to be put in place.

Condition Reason: To protect the health and safety of all persons while works are being undertaken and to ensure all safety measures have been identified and are in place to protect all parties in the immediate vicinity of the site.

Public Road Assets Prior to Any Work/Demolition

Prior to any site works, a full record of the condition of the public infrastructure on public land adjacent to the development site must be submitted to Council.

The report must include photographs and/or CCTV footage showing the current condition and any existing damage fronting and adjoining the site to the:

- road pavement,

- street signage including street lights,

- kerb and gutter,

- footway including pedestrian crossings, footpath, and driveways,

- retaining walls, or other significant structures,

- Heritage Items, including street name inlays,

- utility service items including historical utility covers, and

- drainage structures/pits/pipes (CCTV footage).

The reports are to be supplied in electronic format in Word and if applicable accompanied by CCTV footage. Photographs are to be in colour, digital and date stamped.

If the required report is not submitted then Council will assume there was no damage to any infrastructure in the immediate vicinity of the site prior to the commencement of any site works under this consent.

Condition Reason: To clarify the condition of the existing public infrastructure prior to the commencement of any site works.

Payment of Security and Fees

Prior to any site works, the following security and fees must be paid in full:

| Description | Amount | Indexed | Council Fee Code |
| --- | --- | --- | --- |
| SECURITY under section 4.17(6) of the Environmental Planning and Assessment Act 1979 |  |  |  |
| Property Damage Security Deposit -making good any damage caused to any property of the Council | $1,441,453.00 | No | T115 |
| Tree Damage Security Deposit – making good any damage caused to any public tree | $46,100.00 | No | T114 |
| INSPECTION FEES under section 608 of the Local Government Act 1993 |  |  |  |
| Public Tree Management Inspection Fee | $242.00 | No | T95 |
| Street Tree Planting and Maintenance Fee | $2,156.00 | No | T95 The Reference for the payment must include the development application number, eg. DA413/2024/1. |
| Security Deposit Administration Fee | $235.00 | No | T16 |
| TOTAL SECURITY AND FEES | $1,490,186.00 |  |  |

How must the payments be made?

Payments must be made by:

- cash deposit with Council,

- credit card payment with Council, or

- bank cheque made payable to Woollahra Municipal Council.

- The payment of a security may be made by a bank guarantee where:

- the guarantee is by an Australian bank for the amount of the total outstanding contribution,

- the bank unconditionally agrees to pay the guaranteed sum to the Council on written request by Council on completion of the development or no earlier than 12 months from the provision of the guarantee whichever occurs first [NOTE: a time limited bank guarantee or a bank guarantee with an expiry date is not acceptable],

- the bank agrees to pay the guaranteed sum without reference to the Applicant or landowner or other person who provided the guarantee and without regard to any dispute, controversy, issue or other matter relating to the development consent or the carrying out of development in accordance with the development consent,

- the bank guarantee is lodged with the Council prior to any site works being undertaken, and

- the bank’s obligations are discharged when payment to the Council is made in accordance with the guarantee or when Council notifies the bank in writing that the guarantee is no longer required.

Notes:

- An application must be made to Council by the person who paid the security for release of the securities held under section 4.17 of the Act.

- The securities will not be released until the Occupation Certificate has been lodged with Council, Council has inspected the site and Council is satisfied that the public works have been carried out to Council’s requirements. Council may use part or all of the security to complete the works to its satisfaction if the works do not meet Council’s requirements.

- Council will only release the security upon being satisfied that all damage or all works, the purpose for which the security has been held have been remedied or completed to Council’s satisfaction as the case may be.

- Council may retain a portion of the security to remedy any defects in any such public work that arise within 6 months after the work is completed.

- Upon completion of each section of road, drainage and landscape work to Council's satisfaction, 90% of the bond monies held by Council for these works will be released upon application. 10% may be retained by Council for a further 6 month period and may be used by Council to repair or rectify any defects or temporary works during the 6 month period.

- The Refund of Security Bond Application form can be downloaded from www.woollahra.nsw.gov.au

Condition Reason: To ensure any relevant security and fees are paid.

Security Fencing, Hoarding (including ‘Creative Hoardings’) and Overhead Protection

Before any site work commences, security fencing must be provided around the perimeter of the development site, including any additional precautionary measures taken to prevent unauthorised entry to the site at all times during the demolition, excavation and construction period. Security fencing must be the equivalent 1.8m high chain wire as specified in AS 1725.

Type A Hoarding

Where the development site adjoins a public thoroughfare, the common boundary between them must be fenced for its full length with a hoarding, unless the least horizontal distance between the common boundary and the nearest parts of the structure is greater than twice the height of the structure. The hoarding must be constructed of solid materials (chain wire or the like is not acceptable) to a height of not less than 1.8m adjacent to the thoroughfare.

Type B Hoarding

Where a development site adjoins a public thoroughfare with a footpath alongside the common boundary then, in addition to the hoarding required above, the footpath must be covered by an overhead protective structure and the facing facade protected by heavy-duty scaffolding, unless either:

a)	the vertical height above footpath level of the structure being demolished is less than 4.0m; or

b)	the least horizontal distance between footpath and the nearest part of the structure is greater than half the height of the structure.

The overhead structure must consist of a horizontal platform of solid construction and vertical supports, and the platform must:

a)	extend from the common boundary to 200mm from the edge of the carriageway for the full length of the boundary,

b)	have a clear height above the footpath of not less than 2.1m,

c)	terminate 200mm from the edge of the carriageway (clearance to be left to prevent impact from passing vehicles) with a continuous solid upstand projecting not less than 0.5m above the platform surface, and

d)	together with its supports, be designed for a uniformly distributed live load of not less than 7 kPa.

The overhead protective structures must be installed and maintained in accordance with the NSW “Code of Practice - Overhead Protective Structures 1995”.  This is code available at www.safework.nsw.gov.au/__data/assets/pdf_file/0008/52883/Overhead-protective-structures-Code-of-practice.pdf

All Hoardings

Security fencing, hoarding and overhead protective structure must not obstruct access to utilities services including but not limited to man holes, pits, stop valves, fire hydrants or the like.

Hoardings on Public Land including ‘Creative Hoardings’

All fees associated with the application and occupation and use of the road (footway) for required hoarding or overhead protection must be paid in full.

A creative hoarding (i.e. an approved artwork or historic image affixed to the hoarding) is required if the hoarding meets the criteria in Council’s Creative Hoardings Policy (adopted March 2020). The cost of printing and affixing the creative hoarding is the responsibility of the person with the benefit of this consent. The Creative Hoardings Policy can be downloaded from Council’s website www.woollahra.nsw.gov.au

Notes:

- A minimum of two (2) weeks from the date of making a hoarding application to determination must be allowed. Any approval for a hoarding or overhead protection under the Roads Act 1993 will be subject to its own conditions and fees.

- Council seeks to increase public art in the public domain by requiring artwork or historic images on hoardings located on public land. Under the Creative Hoardings Policy an application for a hoarding proposed on public land will require an approved artwork or historic image affixed to the hoarding if the hoarding meets the criteria in section 3 of the Policy:

a)	Hoardings proposed on land zoned E1 Local Centre, or MU1 Mixed Use, or SP2 Infrastructure under Woollahra Local Environmental Plan 2014 AND erected for 8 weeks or more

OR

b)	Hoardings proposed on land located along a State classified road (regardless of the zone) AND erected for 8 weeks or more

OR

c)	Hoardings proposed in any other location than that referred to in A. and B. above AND erected for 12 weeks or more, except where:

1.	the capital investment value of the work to which the hoarding relates is less than $1 million, or

2.	the land is zoned R2 Low Density Residential, or

3.	the land is zoned R3 Medium Density Residential and the hoarding is located in a lane or street that does not have through traffic (e.g. a cul-de-sac or no through road).

- Artwork and historic images for the hoardings are assessed and approved in accordance with the Creative Hoardings Policy. Details of the artwork or images proposed to be affixed to the hoardings must be submitted with Council’s form “Application for a permit to use a footpath for the erection of a hoarding/scaffolding”. The Creative Hoardings Policy can be downloaded from www.woollahra.nsw.gov.au

Condition Reason: To ensure public safety.

Site Signs

Before any site work commences, the sign/s required by clauses 70 of the Regulation and 75 of the Development Certification and Fire Safety Regulation must be erected and maintained at all times.

Clause 70 of the Regulation provides:

Erection of signs

- For the purposes of section 4.17(11) of the Act, the requirements of subclauses (2) and (3) are prescribed as conditions of a development consent for development that involves any building work, subdivision work or demolition work.

- A sign must be erected in a prominent position on any site on which building work, subdivision `work or demolition work is being carried out:

a)	showing the name, address and telephone number of the principal certifier for the work, and

b)	showing the name of the principal contractor (if any) for any building work and a telephone number on which that person may be contacted outside working hours, and

c)	stating that unauthorised entry to the work site is prohibited.

- Any such sign is to be maintained while the building work, subdivision work or demolition work is being carried out, but must be removed when the work has been completed.

- This clause does not apply in relation to building work, subdivision work or demolition work that is carried out inside an existing building that does not affect the external walls of the building.

- This clause does not apply in relation to Crown building work that is certified, in accordance with section 6.28 of the Act, to comply with the Building Code of Australia.

Clause 75 of the Development Certification and Fire Safety Regulation provides:

Signs on development sites

If there is a person who is the Principal Certifier or the Principal Contractor for any building work, subdivision work or demolition work authorised to be carried out on a site by a development consent or complying development certificate:

- Each such person MUST ensure that a rigid and durable sign showing the person’s identifying particulars so that they can be read easily by anyone in any public road or other public place adjacent to the site is erected in a prominent position on the site before the commencement of work, and is maintained on the site at all times while this clause applies until the work has been carried out.

Notes:

- Clause 75 of the Development Certification and Fire Safety Regulations imposes a maximum penalty of 55 penalty units if these requirements are not complied with.

- If Council is appointed as the Principal Certifier it will provide the sign to the Principal Contractor or Owner-builder who must ensure that the sign is erected and maintained as required by clause 70 of the Regulation and clause 75 of the Development Certification and Fire Safety Regulation.

Condition Reason: To ensure that contact details for the principal certifier and principal contractor are provided on a sign at the development site.

Establishment of Boundary Location, Building Location and Datum

Before any site work commences, a surveyor registered under the Surveying and Spatial Information Act 2002 must:

a)	set out the boundaries of the site by permanent marks (including permanent recovery points),

b)	set out the location and level of foundation excavations, footings, walls and slabs by permanent marks, pegs or profiles relative to the boundaries of the land and relative to Australian Height Datum (AHD) in compliance with the approved plans,

c)	establish a permanent datum point (bench mark) within the boundaries of the site relative to AHD, and

d)	provide a copy of a survey report, prepared by the registered surveyor, detailing the title boundaries, pegs/profiles, recovery points and bench mark locations as established under this condition to the Principal Certifier.

Notes:

- Where there is any discrepancy between the approved development consent and the Construction Certificate, especially in relation to the height, location or external configuration of the building (but not limited to these issues) the site works must not proceed until the variations as shown are consistent with the consent.  Failure to do so may result in a breach of development consent.

- On larger developments, or where boundary redefinition is required, the placement of new State Survey Marks as permanent marks must be considered by the registered surveyor.

Condition Reason: To ensure that the boundary locations, building location, and a datum point is established by a surveyor.

Compliance with Australian Standard for Demolition

While site work is being carried out, the demolition of buildings and structures must comply with Australian Standard AS 2601—2001: The Demolition of Structures.

Condition Reason To control the risks of demolition work.

Skeletal Remains

While site work is being carried out, if any skeletal remains suspected of being human are found, work must cease immediately and no further disturbance of the site must occur. The following must be notified:

a)	NSW Police, and

b)	The person who is the authority for the protection of Aboriginal objects under the National Parks and Wildlife Act 1974, section 85.

Details of the remains and their precise location are to be provided.

Site work may recommence at a time confirmed in writing by the NSW Police and the person who is the authority for the protection of Aboriginal objects under the National Parks and Wildlife Act 1974, section 85.

Condition Reason: To ensure the appropriate management of skeletal remains.

Aboriginal Objects – Unexpected Findings

While site work is being carried out, if unexpected Aboriginal objects or bones are found, you must:

a)	Not further disturb or move these objects or bones.

b)	Immediately cease all work at the particular location.

c)	In the case of suspected human remains, notify NSW Police.

d)	Notify the Heritage NSW Environment Line on 131 555 and the La Perouse Land Council (LALC) on (02) 9311 4282 as soon as practicable and provide available details of the objects or remains and their location.

e)	Notify the person who is the authority for the protection of Aboriginal objects under the National Parks and Wildlife Act 1974, section 85.

f)	Not recommence any work at the particular location unless authorised in writing by the police (in the case of human remains) and the person who is the authority for the protection of Aboriginal objects under the National Parks and Wildlife Act 1974, section 85. Additional assessment and approval under the National Parks and Wildlife Act 1974 may be required prior to works continuing in the affected area(s) based on the nature of the discovery.

Notes:

- The Definition of Aboriginal object, as per the National Parks & Wildlife Act 1974, is any deposit, object or other material evidence (not being a handicraft made for sale) relating to the Aboriginal habitation of an area of New South Wales, being habitation before or concurrent with (or both) the occupation of that area by persons of non-Aboriginal extraction, and includes Aboriginal remains.

Condition Reason: To protect Aboriginal objects

Dilapidation Reports for Existing Buildings

Before any site work commences, dilapidation surveys and dilapidation reports must be conducted and prepared by a professional structural engineer for all buildings and/or structures that are located within the likely “zone of influence” of any excavation, dewatering and/or construction induced vibration as determined applicable by the structural engineer.

These properties must include (but is not limited to):

No. 28 & 28A Albemarle Avenue

No. 494-496 Old South Head Road

Where access is not granted to any adjoining properties to prepare the dilapidation report, the report must be based on a survey of what can be observed externally and it must be demonstrated, in writing, to the satisfaction of the Principal Certifier, that all reasonable steps were taken to obtain access.

The completed dilapidation reports must be submitted to the Principal Certifier for approval, and an approved copy of the reports must be submitted to Council with the Notice of Commencement prior to the commencement of any development work.

No less than two (2) days before any site work commences, neighbouring building owner(s) must be provided with a copy of the dilapidation report for their property(ies).

Notes:

- The dilapidation report will be made available to affected property owners on request and may be used by them in the event of a dispute relating to damage allegedly caused by the carrying out of the development.

- This condition cannot prevent neighbouring buildings being damaged by the carrying out of the development.

- Council will not be held responsible for any damage which may be caused to adjoining buildings as a consequence of the development being carried out.

- Council will not become directly involved in disputes between the developer, its contractors and the owners of neighbouring buildings.

Condition Reason: To establish and document the structural condition of adjoining properties for comparison as site work progresses and is completed and ensure neighbours and Council are provided with the dilapidation report.

Adjoining Buildings Founded on Loose Foundation Materials

The report is to be supplied in both paper copy and electronic format in PDF. Photographs are to be in colour, digital, and date stamped.

The dilapidation report must specify (with supporting photographic/DVD evidence) the exact location and extent of any damaged or defective public infrastructure prior to the commencement of any site work.  If the required report is not submitted, Council will assume there was no damage to any infrastructure in the immediate vicinity of the site prior to the commencement of any site work under this consent.

Notes:

- If a dilapidation report is not submitted as required by this condition, and damage is occasioned to public assets, which adjoin the site, Council will deduct from security any costs associated with remedying, repairing or replacing damaged public infrastructure.

- Nothing in this condition prevents Council making any claim against security held for this purpose.

Condition Reason: To ensure professional engineering advice is obtained to confirm that appropriate underpinning and support to adjoining land is maintained.

Piezometers for the Monitoring of Ground Water Levels

Before any site work commences, 2 piezometers within the excavation area and a further 2 piezometers around the perimeter of the wall must be provided. The piezometers are to be installed to monitor ground water levels (GWL) before and during all dewatering works for the construction phase.

The GWL monitoring wells and monitoring program must be maintained until the issue of the occupation certificate.

The GWL are to be regularly monitored during the course of the works as required by the work method statement for the control of GWL. Any damaged piezometers are to be replaced to allow uninterrupted monitoring.

Where there are any movements in the GWL outside a safe range set by the work method statement for the control of GWL, corrective action must be undertaken under the direction of the professional engineer (hydrological/geotechnical engineer).

Condition Reason: To ensure that piezometers are provided to monitor ground water levels.

Construction Traffic Management Plan

Before any site work commences, and as a result of the site constraints, limited space and access, a Construction Traffic Management Plan (CTMP) is to be submitted to Council for approval. Also, due to lack of on-street parking a Work Zone may be required during construction.

An application for the CTMP must be submitted for approval, and all associated application fees must be paid.

The CTMP must be submitted as a self-contained document that outlines the nature of the construction project and as applicable, include the following information:

a)	Detail the scope of the works to be completed including details of the various stages, e.g. demolition, excavation, construction etc. and the duration of each stage.

b)	Identify local traffic routes to be used by construction vehicles and state that the existing access in Old South Head Road will be maintained and will be utilised as the main access for construction vehicles. All oversized vehicles (longer than 7.5m or heavier than 4.5T) must only access/egress the site from and to Old South Head Road.

c)	Identify ways to manage construction works to address impacts on local traffic routes.

d)	Identify other developments that may be occurring in the area and identify ways to minimise the cumulative traffic impact of these developments. Should other developments be occurring in close proximity (500m or in the same street) to the subject site, the developer/builder is to liaise fortnightly with the other developers/builders undertaking work in the area in order to minimise the cumulative traffic and parking impacts of the developments.

e)	Detail how construction workers will travel to and from the site and parking arrangements for those that drive.

f)	Identify any proposed road closures, temporary traffic routes, loss of pedestrian or cyclist access, or reversing manoeuvres onto a public road, and provide Traffic Guidance Schemes (TGSs) prepared by an accredited SafeWork NSW Control Work Training Card holder to manage these temporary changes.

g)	Detail the size (including dimensions), numbers and frequency of arrival of the construction vehicles that will service the site for each stage of works.

h)	Provide for the standing of vehicles during construction.

i)	If construction vehicles are to be accommodated on the site, provide a scaled drawing showing where these vehicles will stand and the vehicle swept path to show that these vehicles can access and egress the site in a forward direction (including dimensions and all adjacent traffic control devices, such as parking restrictions, pedestrian facilities, kerb extensions, etc.).

j)	If trucks are to be accommodated on Council property, provide a scaled drawing showing the location of any proposed Works Zone (including dimensions and all adjacent traffic control devices, such as parking restrictions, pedestrian facilities, kerb extensions, etc.).

k)	Show the location of any site sheds and any anticipated use of cranes and concrete pumps and identify the relevant permits that will be required.

l)	If a crane/s are to be accommodated on site, detail how the crane/s will be erected and removed, including the location, number and size of vehicles involved in the erection/removal of the crane/s, the duration of the operation and the proposed day and times, any full or partial road closures required to erect or remove the crane/s and appropriate Traffic Guidance Schemes (TGSs) prepared by an approved SafeWork NSW Control Work Training Card holder.

m)	Make provision for all materials, plant, etc. to be stored within the development site at all times during construction.

n)	State that any oversized vehicles proposed to operate on Council property (including Council approved Works Zones) will attain a Permit to Stand Plant on each occasion (Note: oversized vehicles are vehicles longer than 7.5m or heavier than 4.5T.)

o)	Show the location of any proposed excavation and estimated volumes.

p)	When demolition, excavation and construction works are to be undertaken on school days, all vehicular movements accessing/egressing from and to Albemarle Avenue must only be undertaken between the hours of 9.30am and 2.30pm, in order to minimise disruption to the traffic network during school pick up and drop off times.

q)	Show the location of all Tree Protection (Exclusion) zones (Note: storage of building materials or access through Reserve will not be permitted without prior approval by Council).

Notes:

A minimum of eight weeks will be required for assessment. Site work must not commence until the Construction Traffic Management Plan is approved.

Failure to comply with this condition may result in fines and proceedings to stop work.

Council and NSW Police approval is required prior to a partial or full temporary road closure.  If you are seeking a partial or full temporary road closure you must comply with the relevant conditions of this consent and you must also gain the approval of the Eastern Suburbs Police Area Command.

If you partial or full close a road without compliance with Council and NSW Police requirements Council Rangers or the Police can issue Penalty Infringement Notices or Court Attendance Notices leading to prosecution.

Traffic Supervisors at the Eastern Suburbs Police Area Command can be contacted on eastsubtraffic@police.nsw.gov

Condition Reason: To facilitate the efficient operation of construction projects, minimise traffic disruption, and protect the public, and the surrounding environment, during site works and construction.

Works (Construction) Zone – Approval and Implementation

If the Construction Management Plan relies upon a Works Zone, before any site work commences, a Works Zone application must be made.

If the works zone is approved, all fees for the Works Zone must be paid before it can be installed.

All Works Zone signs must have been erected by Council to permit enforcement of the Works Zone by Council’s Rangers and NSW Police before commencement of any site work.  Signs are not erected until full payment of Works Zone fees is made.

Notes:

- A minimum of four to six weeks must be allowed (for routine applications) from the date of making an application to the Traffic Committee (Woollahra Local Traffic Committee) constituted under clause 20 of the Transport Administration (General) Regulation 2018 to exercise those functions delegated by Transport for New South Wales under section 31(3) of the Transport Administration Act 1988.

- The enforcement of the Works Zone is at the discretion of Council’s Rangers and the NSW Police Service.  Any breach of the Works Zone must be reported to either Council or the NSW Police Service.

Condition Reason: To facilitate the efficient operation of construction projects and to minimise traffic disruption.

Establishment of Tree Protection Measures within the Tree Protection Zones (TPZ) of Existing Trees

Prior to any site works, tree protection measures must be established around all trees to be retained in accordance with Section 4 of the Australian Standard Protection of Trees on Development Sites (AS 4970).

The Tree Protection Zones must be calculated in accordance with Section 3 of the Australian Standard Protection of Trees on Development Sites (AS 4970).

A Construction Site Management Plan, which clearly details the tree protection measures, must be prepared before the issue of a construction certificate. The tree protection measures must comply with the following requirements;

- Tree Protection Fencing:

| Council Ref No | Species | Tree Location | Fence Radius from Centre of Trunk (Metres)* |
| --- | --- | --- | --- |
| 9 | Ficus benjamina (Weeping Fig) | Rear yard of 39-41 Wilberforce Avenue Rose Bay | 6 |

Where this condition relates to street trees, and the fence cannot be placed at the specified radius, the fencing is to be positioned so that the entire verge (nature strip) area in front of the subject property, excluding existing driveways and footpaths, and bus stops is protected.

Where this condition relates to trees on private property, the radial distance of fencing must be positioned only within the subject property relating to the development consent.

- Tree Protection Zones must be fenced with a 1.8 metre high chainmesh or weldmesh fence and secured to restrict access. The fence must be established prior to any materials being bought onto the site and before the commencement of works including demolition. The area within the fence must be mulched and the mulch layer maintained to a depth of 75mm. The soil within the TPZ area must be kept in a moist condition for the duration of the construction works. Unless approved by the site arborist there must be no access within the TPZ area.

- Trunk protection must be installed around the trunks of the following trees:

| Council Ref No | Species | Tree Location |
| --- | --- | --- |
| 5 | Lophostemon confertus (Brush Box) | Most north western specimen along the Albemarle Avenue frontage of the site |
| 6 | Lophostemon confertus (Brush Box) | Most south eastern specimen along the Albemarle Avenue frontage of the site |
| 7 | Gleditsia triacanthos cvs (Honey Locust) | Most south western specimen along the Old South Head Road frontage of the site |
| 8 | Gleditsia triacanthos cvs (Honey Locust) | Most north eastern specimen along the Old South Head Road frontage of the site |
| 9 | Ficus benjamina (Weeping Fig) | Rear yard of 39-41 Wilberforce Avenue Rose Bay |

Trunk protection must consist of a padding material such as hessian or thick carpet underlay wrapped around the trunk. Hardwood planks (50mm x100mm or similar) must be placed over the padding and around the trunk of the tree at 150mm centres. The planks must be secured with 8 gauge wire at 300mm spacing. Trunk protection must extend a minimum height of 2 metres or to the maximum possible length permitted by the first branches.

- Signs identifying the Tree Protection Zone area must be erected on each side of the protection fence indicating the existence of a TPZ area. Signage must be visible from within the development site.

- No excavation, construction activity, grade changes, storage of materials, stockpiling, siting of works sheds, preparation of mixes or cleaning of tools is permitted within Tree Protection Zones, unless specified in this consent.

- Temporary access within the TPZ area for pedestrian and machinery movements must only be permitted with the approval of the site arborist or unless specified in this consent.

- The site supervisor must be made aware of all tree protection requirements associated with these conditions of consent by the project arborist. Any subsequent site personnel and contractors to the site must be made aware of all tree protection requirements by the site foreman.

- The project arborist must provide written certification of compliance to the Principal Certifier with the above conditions.

Condition Reason: To ensure the protection of existing trees

Standard Condition B.5

Construction Management Plan Arborist Review

Prior to any site works, the Construction Management Plan must be reviewed and certified by the Project Arborist confirming that appropriate tree protection measures are implemented. The Construction Management Plan must be prepared in accordance with all tree protection measures specified within this consent.

The plan must address:

- Drawings and method statement showing details and the location of hoarding and scaffold and any pruning required to accommodate the hoarding and scaffolding;

- The movement and positioning of heavy machinery, lifting cranes, pier drilling gantry etc.;

- Site construction access, temporary crossings and movement corridors on the site defined;

- Contractors car parking;

- Phasing of construction works;

- The space needed for all foundation excavations and construction works;

- All changes in ground level;

- Space for site sheds and other temporary structures such as toilets;

- Space for sorting and storing materials (short or long term), spoil and fuel and the mixing of cement and concrete; and

- The effects of slope on the movement of potentially harmful liquid spillages towards or into tree protection areas.

Condition Reason To ensure the Construction Management Plan incorporates measures for the protection of existing trees.

Standard Condition B.28

Arborists Documentation and Compliance Checklist

Prior to any site works, the project arborist must provide written certification that all tree protection measures and construction techniques relevant to this consent are implemented. Documentation for each site visit must include:

- A record of the condition of trees to be retained prior to and throughout development.

- Recommended actions to improve site conditions and rectification of non-compliance.

- Recommendations for future works which may impact the trees.

All compliance certification documents must be kept on site.

As a minimum the following intervals of site inspections must be made:

| Stage of arboricultural inspection and supervision | Compliance documentation and photos must include |
| --- | --- |
| Prior to any site works | - Project Arborist to hold pre construction site meeting with the principal contractor to discuss methods and importance of tree protection measures and resolve any issues in relation to feasibility of tree protection requirements that may arise. Project Arborist to mark all trees approved for removal under DA consent. - The project arborist must install or supervise the installation of tree protection fencing and trunk protection |

Inspections and compliance documentation must be made by an arborist with AQF Level 5 qualifications.

Additional site visits must be made when required by the site arborist and/or site foreman for ongoing monitoring/supervisory work.

Condition Reason To ensure that written certification that all tree protection measures and construction techniques relevant to this consent have been implemented.

Standard Condition B.29

Permissible work within Tree Protection Zones

Prior to any site works, the following works are permissible within the Tree Protection Zone:

| Council Ref No | Species | Radius from Centre of Trunk (Metres) | Approved works |
| --- | --- | --- | --- |
| 5 | Lophostemon confertus (Brush Box) | 7.2 | Proposed building |
| 6 | Lophostemon confertus (Brush Box) | 3.6 | Proposed building |
| 7 | Gleditsia triacanthos cvs (Honey Locust) | 4.8 | Proposed building |
| 8 | Gleditsia triacanthos cvs (Honey Locust) | 4.2 | Proposed building |
| 9 | Ficus benjamina (Weeping Fig) | 6 | Proposed boundary fence using tree sensitive techniques ensuring no roots equal to or greater than 50mm diameter are severed. Proposed soft landscaping |

The project arborist must provide written certification of compliance to the Principal Certifier with the above condition.

Condition Reason To establish the works which are permissible within the Tree Protection Zones.

Standard Condition B.30

# ON COMPLETION OF REMEDIATION WORK

Nil

# BEFORE THE ISSUE OF A CONSTRUCTION CERTIFICATE

Modification of Details of the Development (section 4.17(1)(g) of the Act

Before the issue of any construction certificate, the approved plans and the construction certificate plans and specification, required to be submitted to the Principal Certifier under clause 7 of the Development Certification and Fire Safety Regulations, must detail the following amendments:

- CCTV and low-level garden lighting should be provided within the landscaped separation zone (adjacent to the north-western boundary). The lighting should be directed away from the adjacent residential property and should achieve compliance with AS/NZS 4282: Control of the obtrusive effects of outdoor lighting.

- A minimum of ten (10) retail parking spaces must be reallocated to staff parking and must be clearly depicted on the architectural drawings.

- A minimum of two (2) retail parking spaces must be reallocated to designated car share parking spaces and must be clearly depicted on the architectural drawings.

- A yellow epoxy zebra crossing be provided at both vehicle access points to minimise any conflicts and improve pedestrian safety.

- The architectural plans for the proposed development must be modified to include notes on the plans indicating a minimum 1.5 metre setback of any proposed awning from the trunks, 1st and 2nd order branches of Trees 5, 6, 7 and 8 for the incorporation into construction drawings.

Notes:

- Clause 20 of the Development Certification and Fire Safety Regulations prohibits the issue of any construction certificate subject to this condition unless the Principal Certifier is satisfied that the condition has been complied with.

- Clause 19 of the Development Certification and Fire Safety Regulations prohibits the issue of any construction certificate that is inconsistent with this consent.

Condition Reason: To require design changes and/or further information to be provided to address specific issues identified during the assessment under section 4.15 of the Act.

Payment of Long Service Levy

Before the issue of any construction certificate, the original receipt(s) for the payment of the following levy must be provided to the Principal Certifier:

| Description | Amount | Indexed | Council Fee Code |
| --- | --- | --- | --- |
| LONG SERVICE LEVY under Building and Construction Industry Long Service Payments Act 1986 |  |  |  |
| Long Service Levy www.longservice.nsw.gov.au/bci/levy/other-information/levy-calculator | Contact LSL Corporation or use online calculator | No |  |

Building and Construction Industry Long Service Payment

The long service levy under section 34 of the Building and Construction Industry Long Service Payment Act 1986, must be paid and proof of payment provided to the Principal Certifier prior to the issue of any construction certificate. The levy can be paid directly to the Long Service Corporation or to Council.  Further information can be obtained from the Long Service Corporation website www.longservice.nsw.gov.au or the Long Service Corporation on 131 441.

How must the payments be made?

Payments must be made by:

- cash deposit with Council,

- credit card payment with Council, or

- bank cheque made payable to Woollahra Municipal Council.

Condition Reason: To ensure any relevant levy is paid.

Hydraulic Fire Services

Before the issue of any construction certificate, the construction certificate plans and specifications required by clause 7 of the Development Certification and Fire Safety Regulation, must detail all hydraulic fire safety services required by this condition.

Fire hydrants, booster valve assembly installations, sprinkler valves and associated hydraulic equipment, must be:

a)	enclosed with doors, if located in the building façade; or

b)	housed in a cabinet or enclosure, if located external to the building.

The location, design, colour and material of the doors, cabinet or enclosure must be visually unobtrusive and suitably integrated with the development, including any fencing and landscaping.

Notes:

- These conditions apply in addition to requirements in AS 2419.1 Fire hydrant installations.

Condition Reason: To ensure that, in accordance with clause 7 of the Development Certification and Fire Safety Regulation, that hydraulic fire safety services are provided, but not visually intrusive when viewed from the street.

Design Principles for Residential Apartment Development – Design Verification Statement

Before the issue of any construction certificate, as required under clause 15 of the Development Certification and Fire Safety Regulation, a statement by a qualified designer verifying that the relevant building work plans and specifications achieve or improve the design quality of the development for which development consent was granted, having regard to the design quality principle in State Environmental Planning (Housing) 2021 —Design Principles for Residential Apartment Development must be provided to the Principal Certifier.

Notes:

- Although a Principal Certifier may under clause 73(2)(b) of the Development Certification and Fire Safety Regulation be satisfied to any matter that relates to the external finish of a building the specific provisions of clause 15 of the Development Certification and Fire Safety Regulation overrides the Principal Certifier’s powers under clause 73(2)(b).  No Principal Certifier can set aside this requirement.

- Qualified designer means a person registered as an architect in accordance with the Architects Act 2003.

- There are several methods of verifying the status of an individual or corporation or firm offering architectural services. Ask the individual for their four digit NSW Architects Registration Number, ask a corporation or firm for the name and registration number of their “nominated architect”, check to see whether an individual is listed on the NSW Register of Architects or the corporation or firm is listed on the list of architect corporations and firms on the Board’s website www.architects.nsw.gov.au or call the NSW Architects Registration Board on 9241 4033 to check the status of an individual or corporation or firm.

Condition Reason: To ensure a design verification statement from a qualified designer is provided for the development.

Waste Storage – Mixed Developments (both commercial and residential)

Before the issue of any construction certificate, the construction certificate plans and specifications required by clause 7 of the Development Certification and Fire Safety Regulation, must include detailed plans and specifications must make provision for:

a)	A dedicated and enclosed waste and recycling storage area within non-habitable areas of the building, as per the approved Architectural Plans DA109 (Rev 9) (prepared by PBD Architects).

b)	Two separate centralised waste and recycling rooms or areas, one for commercial waste and one for residential waste. These must be self-contained and have separate keys and locking systems.

c)	The path for wheeling bins between the waste and recycling storage area and the collection point must be free of steps and kerbs and having a maximum grade of 1:8. The waste storage area must be as close as possible to the service road collection point.

d)	Bins to be stored with lids down to prevent vermin from entering the waste containers.

e)	Smooth impervious floor graded to a floor waste and provided with a tap and hose to facilitate regular cleaning of the bins. A waste storage area that is located internal to the building must be fitted with both a hot and cold water supply and hose cocks. Wastewater must be discharged to the sewer in accordance with the requirements of Sydney Water.

f)	Walls and ceilings of the waste storage area must be constructed of an impervious material with a smooth finish. The junction between the walls and the floor must be covered with a minimum radius of 25mm to prevent the accumulation of waste matter.

g)	The garbage storage area must be well lit to enable use at night. A timer switch must be fitted to the light fitting to ensure the light is turned off after use.

h)	Odour problems must be minimised by exhaust ventilation.

i)	Both putrescible and recycling bins/crates must be stored together. Recycling bins must never stand alone. They must always be located beside putrescible waste bins. Putrescible bins must be located closest to the entrance to the waste storage room.

j)	Signage on the correct use of the waste management system and what materials may be recycled must be posted in the communal waste storage cupboard/ room or bin bay.

Condition Reason: To ensure a waste and recycling storage area is provided.

Utility Services Generally

Before the issue of any construction certificate, the construction certificate plans and specifications required by clause 7 of the Development Certification and Fire Safety Regulation, must demonstrate that all utility services (telecommunications, electricity, gas, water and waste water) will be provided underground.  All service ducts, pipes and conduits must be provided within the fabric of the building (excluding stormwater down pipes).

A customer's electrical supply connection and point of connection must comply with the NSW Service and Installation Rules: https://www.energy.nsw.gov.au/sites/default/files/2022-08/2018_07_NSW_ServiceAndInstallationRulesOfNSW_underlined.pdf and comply with Ausgrid's relevant Networks Standards relating to customer connections and the type of connection.

Where adequate provision has not been made for an electrical substation within the building, this may necessitate the lodgement of an application to amend this consent under section 4.55 of the Act to detail the location, landscape/streetscape impacts and compliance with AS2890 as applicable.

The applicant shall provide to the Principal Certifier a true copy of the plans developed by the applicant's Accredited Service Provider outlining the design and requirements for network modification and customer connection for the proposed development. The plans must have been reviewed and certified by Ausgrid to be deemed compliant of meeting this requirement.

Proposed water pipes, waste pipes, stack work, duct work, mechanical ventilation plant and the like must be located within the building unless expressly shown upon the approved DA plans.  Details confirming compliance with this condition must be shown on the construction certificate plans and/or detailed within the construction certificate specifications.  Required external vents or vent pipes on the roof or above the eaves must be shown on the construction certificate plans.

The construction certificate plans and specifications, required to be submitted to the Principal Certifier under clause 7 of the Development Certification and Fire Safety Regulation, must detail the replacement of all private sewer pipes between all sanitary fixtures and Sydney Water’s sewer main where they are not found by inspection to be UPVC or copper with continuously welded joints.

Notes:

- This condition has been imposed to ensure that where private sewer pipes are old, may leak or may be subject to root invasion (whether from existing or proposed private or public landscaping) that existing cast iron, concrete, earthenware or terracotta pipes be replaced with new UPVC or copper continuously welded pipes between all sanitary fixtures and Sydney Waters sewer main.

- Leaking sewer pipes are a potential source of water pollution, unsafe and unhealthy conditions which must be remedied in the public interest.

Condition Reason: To ensure the adequate provision of utility services, and to ensure that any proposed cables, pipes, conduits, ducts, plant, equipment, electricity substations or the like does not compromise the design quality of the development.

Payment of S7.12 Contributions Levy

A payment of a levy authorised by section 7.12 of the Environmental Planning and Assessment Act 1979 must be paid prior to the issue of any Construction Certificate or Subdivision Works Certificate. The Principal Certifier is to be provided with the original receipt for payment under the Woollahra Section 7.12 Development Contributions Plan 2022.

A cost estimate report, no more than 3 months old, demonstrating the proposed cost of carrying out the development must be completed and submitted to Council for determination of the costs of work. This report must incorporate all approved modification applications. The costs and expenses of the proposed cost of development must be established in accordance with clause 208 of the Environmental Planning and Assessment Regulation 2021.

The cost estimate report must be in the form of:

- A cost summary report, prepared by the applicant or a person acting on the behalf of the applicant for a development up to $150,000;

- A cost summary report, prepared by a suitably qualified person, at the applicant’s cost, where the cost of development is between $150,000 and $749,999; or

- A quantity surveyor’s report, at the applicant’s cost, for development over $750,000.

Note: A ‘suitably qualified person’ is defined in the Woollahra Section 7.12 Development Contributions Plan 2022.

The applicable levy rate is to be calculated using the summary schedule below.

| Summary Schedule |  |
| --- | --- |
| Development Cost | Levy Rate |
| - Up to and including $100,000 | Nil |
| - More than $100,000 and up to and including $200,000 | 0.5% of the cost |
| - More than $200,000 | 1% of the cost |

How must the payments be made?

Payments must be made by:

- Cash deposit with Council,

- Credit card payment with Council, or

- Bank cheque made payable to Woollahra Municipal Council.

Deferred or periodic payment of section 7.12 levy

Where the Applicant makes a written request supported by reasons for payment of the section 7.12 levy other than as required by clause 2.9, the Council may accept deferred or periodic payment. The decision to accept a deferred or periodic payment is at the sole discretion of the Council, which will consider:

- the reasons given,

- whether any prejudice will be caused to the community deriving benefit from the public facilities,

- whether any prejudice will be caused to the efficacy and operation of the Plan, and

- whether the provision of public facilities in accordance with the adopted works schedule will be adversely affected.

Council may, as a condition of accepting deferred or periodic payment, require the provision of a bank guarantee where:

- the guarantee is by an Australian bank for the amount of the total outstanding contribution,

- the bank unconditionally and irrevocably agrees to pay the guaranteed sum to the Council on written request by Council prior to the issue of an occupation certificate,

- a time limited bank guarantee or a bank guarantee with an expiry date is not acceptable,

- the bank agrees to pay the guaranteed sum without recourse to the applicant or landowner or other person who provided the guarantee and without regard to any dispute, controversy, issue or other matter relating to the development consent or the carrying out of development in accordance with the development consent, and

- the bank’s obligations are discharged when payment to the Council is made in accordance with the guarantee or when Council notifies the bank in writing that the guarantee is no longer required.

Any deferred or periodic payment of the section 7.12 levy will be adjusted in accordance with clause 2.12 of the Plan. The Applicant will be required to pay any charges associated with establishing or operating the bank guarantee. Council will not cancel the bank guarantee until the outstanding contribution as indexed and any accrued charges are paid.

Do you need HELP indexing the levy?

Please contact Council’s Customer Service Team on 02 9391 7000. Failure to correctly calculate the adjusted development levy will delay the issue of any certificate issued under section 6.4 of the Act and could void any such certificate (e.g. construction certificate, subdivision certificate, or occupation certificate).

Condition Reason: To ensure any relevant contributions are paid.

BASIX Commitments

Before the issue of any construction certificate, BASIX Certificate No. 1766799Mmust be submitted to the Principal Certifier with any application for a construction certificate.

All commitments in the BASIX Certificate must be shown on the construction certificate plans and specifications prior to the issue of any construction certificate.

Notes:

- Where there is any proposed change in the BASIX commitments the Applicant must submit a new BASIX Certificate to the Principal Certifier and Council.  If any proposed change in the BASIX commitments are inconsistent with development consent (see: clauses 19 and 20 of the Development Certification and Fire Safety Regulation) the Applicant will be required to submit an amended development application to Council under section 4.55 of the Act.

- Clause 19(1)(a) of the Development Certification and Fire Safety Regulation 2021 provides: a certifier must not issue a construction certificate for building work unless: the relevant building work plans and specifications include the matters required by a relevant BASIX certificate, if any.

Condition Reason: To ensure all commitments in the BASIX Certificate are incorporated into the development.

Water and Waste Water - section 73 Developers Certificate and Upgrading of Existing System

Prior to the issue of any construction certificate, a Compliance Certificate must be obtained from Sydney Water under section 73 of the Sydney Water Act 1994. Sydney Water’s assessment will determine the availability of water and sewer services, which may require extension, adjustment or connection to their mains.

The construction certificate plans and specifications, required to be submitted to the Principal Certifier under clause 7 of the Development Certification and Fire Safety Regulation, must detail the replacement of all private sewer pipes between all sanitary fixtures and Sydney Water’s sewer main where they are not found by inspection to be sewer grade UPVC or copper with continuously welded joints.

Notes:

- Sydney Water will assess the development and if required will issue a ‘Notice of Requirements’ letter detailing all requirements that must be met. Applications can be made either directly to Sydney Water or through a Sydney Water accredited Water Servicing Coordinator (WSC).  Please make early contact with Sydney Water or a WSC, since building of water/sewer extensions can be time consuming and may impact on other services and building, driveway or landscape design.

- For more information go to www.sydneywater.com.au/section73 or call 1300 082 746 to learn more about applying through an authorised WSC or Sydney Water.

- Where private sewer pipes are old, may leak or may be subject to root invasion (whether from existing or proposed private or public landscaping) that existing cast iron, concrete, earthenware or terracotta systems are replaced with new UPVC or copper continuously welded pipes between all sanitary fixtures and Sydney Waters sewer main as part of the development.  Leaking sewer pipes are a potential source of water pollution, unsafe and unhealthy conditions which must be remedied in the public interest.

Condition Reason: To ensure that private sewer pipes are upgraded where required to prevent water pollution, and unsafe and unhealthy conditions.

Erosion and Sediment Control Plan – Submission and Approval

Before the issue of any construction certificate, an erosion and sediment control plan, prepared by a suitably qualified person in accordance with the following documents, must be submitted to the Principal Certifier.  The erosion and sediment control plan must comply with:

a)	“Do it Right On Site, Soil and Water Management for the Construction Industry” and the accompanying factsheets published by the Southern Sydney Regional Organisation of Councils; and

b)	“Managing Urban Stormwater - Soils and Construction” 2004 published by the NSW Government (The Blue Book).

Where there is any conflict The Blue Book takes precedence.

The Principal Certifier must be satisfied that the erosion and sediment control plan complies with the publications above prior to issuing any construction certificate.

Notes:

- The International Erosion Control Association – Australasia www.austieca.com.au lists consultant experts who can assist in ensuring compliance with this condition.  Where erosion and sedimentation plans are required for larger projects it is recommended that expert consultants produce these plans.

- The “Do it Right On Site, Soil and Water Management for the Construction Industry” publication and accompanying factsheets can be downloaded from www.woollahra.nsw.gov.au, and The Blue Book is available at www.environment.nsw.gov.au

- Under clause 73(2)(a)(v) of the Development Certification and Fire Safety Regulation an Accredited Certifier may be satisfied as to this matter.

Condition Reason: To prevent potential water pollution and dust nuisance.

Electric vehicle circuitry and electric vehicle charging point requirements

Before the issue of any construction certificate, the construction certificate plans and specifications required under clause 7 of the Development Certification and Fire Safety Regulation, must include an accurate electrical plan and specifications for all off-street car parking, prepared by a suitably qualified person, demonstrating the following;

a)	That each off-street car parking space will be provided with electrical circuitry to support the installation of a Level 2 electric vehicle charger point. The construction certificate plans are to:

- Identify the power capacity to each car parking space.

- Identify the load management system on each level of parking such as a distribution board.

- Identify the conduit system to allow each car space to install an electric vehicle charger point - such as cable trays and/or buried cables underground. This system must allow future installation of cabling to power electric vehicle charger points and allow internet access (run Ethernet cable or install 4G modem).

b)	A minimum of one Level 2 electric charger must be provided and Level 2 chargers must be provided to not less than 10% of all car parking spaces. The location of all electric vehicle chargers must be shown on the construction certificate plans.

c)	The Principal Certifier must be satisfied that the electrical plans and specifications are consistent with (a) and (b) prior to the issue of the construction certificate.

Notes:

- The minimum electric circuitry requirements for ‘Level 2’ electric vehicle charging points are:

-	Privately available spaces including visitor spaces: ‘Level 2’ slow – single phase 7kW power; and

-	Publicly available spaces: ‘Level 2’ fast – three-phase 11-22kW power.

Condition Reason: To ensure the provision of electric vehicle circuitry to enable the future installation of electric vehicle charging point(s), and electric charging points if 10 or more car parking spaces are provided.

Road and Public Domain Works

Before the issue of any construction certificate, a separate application under Section 138 of the Roads Act 1993 is to be made to, and be approved by Council, for the following infrastructure works. The infrastructure works must be carried out at the applicant's expense:

- The removal of all redundant vehicular crossings including layback and gutter and reinstatement to standard kerb and gutter in accordance with Council’s and TfNSW’s Specification, Council’s standard drawing RF2_D and to the satisfaction of Council’s Assets Engineers,

- The removal of the existing kerb and gutter and the construction of a new 5.5m wide vehicular crossing to access the proposed basement in accordance with Council’s standard drawing RF2_D, Crossing Specification and to the satisfaction of Council’s Assets Engineers. The new crossing shall be constructed at a right angle to the street kerb in plain concrete, with its centreline aligned with the centreline of the proposed driveway at the property boundary. Design longitudinal profiles along each edge/side of the proposed crossing, starting from the centreline of the road to the parking slab, must be submitted for assessment,

- The removal of the existing vehicular crossing including layback and gutter and the construction of a new 7.0 metres wide vehicular crossing for the northern driveway and a 4.5 metres wide vehicular crossing for the loading dock in accordance with Council’s Crossing Specification, standard driveway drawing RF2_D and to the satisfaction of Council’s Assets Engineers. The new vehicular crossings must be constructed at a right angle to the street kerb in plain concrete where the centreline of the new crossings must align with the centreline of the internal driveways at the property boundary. Design longitudinal surface profiles along each side/edge for the proposed driveway, starting from the road centreline to the parking slab must be submitted for assessment.

- The reconstruction of the existing 1.8m wide concrete footpath including pram ramps for the full frontage of the site in Albemarle Avenue in accordance with Council’s Specification for Roadworks, Drainage and Miscellaneous Works and to the satisfaction of Council’s Assets Engineers. A maximum crossfall of 3% must be provided for the footpath from the property boundary towards the top of kerb. A design longitudinal surface profile (scale 1:100) and cross sections (scale 1:50) at every 5 metres intervals must be submitted for assessment,

- The reconstruction of full width footpath with Rose Bay pavers for the full frontage of the site in Old South Head Road in accordance with Council’s Specification. A maximum crossfall of max. 3% shall be provided for the full width footpath from the property boundary to the top of kerb. A design longitudinal surface profile (scale 1:100) and cross sections (scale 1:50) at every 5 metres interval must be submitted,

- The reinstatement of all damaged kerb and gutter and road pavement to Council’s Specification and to the satisfaction of Council’s Assets Engineers,

- Where a grass verge exists, the balance of the area between the footpath and the kerb over the full frontage of the proposed development must be turfed.  The grass verge must be constructed to contain a uniform minimum 75mm of friable growing medium and have a total cover of Couch turf.

- The construction of a new kerb inlet pit (KIP) with 1.8m precast lintel over the existing Sydney Water’s underground pipeline in Albemarle Avenue with the extension of the Council’s underground drainage system using minimum Class 4, 375mm RRJ steel reinforced concrete pipes (RCP) with minimum 1% grade in accordance with Council’s Specification for Roadworks, Drainage and Miscellaneous Works, Council’s Standard Drawing DR1 and to the satisfaction of Council’s Assets Engineers. Detailed design including longitudinal section of the proposed 375mm RCP shall be prepared by a chartered professional civil engineer. Trench details shall be included in the design drawings to comply with Council’s Specification and AS3725. A copy of the approved plans from Sydney Water for the proposed stormwater connection must be submitted with this S138 application,

- The construction of a new kerb inlet pit (KIP) with 1.8m precast lintel for the proposed stormwater connection. The proposed KIP must be provided within the frontage of the site and be located at a minimum distance of 0.5m from the layback of the proposed crossing to comply with Council’s Specification. The grates shall be Class D, “bicycle friendly” type.

- The developer shall be responsible for carrying out all service investigations to allow a gravity connection,

- An infrastructure bond will be used as security to ensure the satisfactory completion of the infrastructure works. The security or bank guarantee must be the original unconditional bank guarantee with no expiry date.

- Council may use all or part of the Infrastructure Bond as well as the Property Damage Security Deposit to meet the cost of removing or completing the works if they do not meet Council’s requirements.

- The Deposit/Bond will not be released until Council has inspected the site and is satisfied that the Works have been completed in accordance with Council approved drawings and to Council requirements.

Before the issue of any construction certificate, the principal certifier must be provided with the original receipt(s) for the payment of all of the following security bonds and fees:

| Description | Amount | Indexed | Council Fee Code |
| --- | --- | --- | --- |
| SECURITY under section 4.17(6) of the Environmental Planning and Assessment Act 1979 |  |  |  |
| Infrastructure Works Bond - completing any public work required in connection with the consent. | $ 236,800.00 | No | T113 |
| Infrastructure Works Bond – remedying any defects in any public work that arise within 6 months after the work is completed | $ Nil | No | T113 |
| INSPECTION FEES under section 608 of the Local Government Act 1993 |  |  |  |
| Public Road and Footpath Infrastructure Inspection Fee | $ 1,348.00 | Yes | T45 |
| TOTAL SECURITY AND FEES | $ 238,148.00 |  |  |

How must the payments be made?

Payments must be made by:

- cash deposit with Council,

- credit card payment with Council, or

- bank cheque made payable to Woollahra Municipal Council.

The payment of a security may be made by a bank guarantee where:

the guarantee is by an Australian bank for the amount of the total outstanding contribution,

- the bank unconditionally agrees to pay the guaranteed sum to the Council on written request by Council on completion of the development or no earlier than 12 months from the provision of the guarantee whichever occurs first [NOTE: a time limited bank guarantee or a bank guarantee with an expiry date is not acceptable],

- the bank agrees to pay the guaranteed sum without reference to the Applicant or landowner or other person who provided the guarantee and without regard to any dispute, controversy, issue or other matter relating to the development consent or the carrying out of development in accordance with the development consent,

- the bank guarantee is lodged with the Council prior to any site works being undertaken, and

- the bank’s obligations are discharged when payment to the Council is made in accordance with the guarantee or when Council notifies the bank in writing that the guarantee is no longer required.

Notes:

- Road has the same meaning as in the Roads Act 1993.

- Changes in levels may arise from the detailed design of buildings, road, footpath, driveway crossing grades and stormwater. Changes required under Roads Act 1993 approvals may necessitate design and levels changes under this consent.  This may in turn require the Applicant to seek to amend this consent.

- Works or structures over, on or under public roads or footpaths are subject to sections 138, 139 and 218 of the Roads Act 1993 and specifically:

- Construction of driveways and/or new or alterations to footpath paving

- Alteration and/or extension to Council drainage infrastructure

- Alteration and/or addition of retaining walls

- Pumping of water to Council’s below ground stormwater system

- Installation of soil/rock anchors under the roadway

- Installation of Stormwater outlet pipes across the nature strip

- An “Application to Carry Out Works in a Public Road” form must be completed and lodged, with the application fee, at Council’s Customer Services.  Detailed plans and specifications of all works (including but not limited to structures, road works, driveway crossings, footpaths and stormwater drainage etc) within existing roads, must be attached, submitted to and approved by Council under section 138 of the Roads Act 1993, before the issue of any construction certificate.

- Detailed engineering plans and specifications of the works required by this condition must accompany the application form. The plans must clearly show the following:

- Engineering drawings (plan, sections and elevation views) and specifications of the footpath, driveways, kerb and gutter, new gully pit showing clearly the connection point of site outlet pipe(s). The connection drainage lines must be as direct as possible and generally run perpendicular to the kerb alignment.

- Engineering drawings of the new drainage line to be constructed joining the new and existing drainage pits including services.

- All driveways must include a design longitudinal surface profile for the proposed driveway for assessment. The driveway profile is to start from the road centreline and be along the worst case edge of the proposed driveway. Gradients and transitions must be in accordance with clause 2.5.3, 2.6 of AS 2890.1 – 2004, Part 1 – Off-street car parking. The driveway profile submitted to Council must be to (1:25) scale (for template checking purposes) and contain all relevant details: reduced levels, proposed grades and distances.

- The existing footpath level and grade at the street alignment of the property must be maintained unless otherwise specified by Council.  Your driveway levels are to comply with AS2890.1 and Council’s Standard Drawings. There may be occasions where these requirements conflict with your development and you are required to carefully check the driveway/garage slab and footpath levels for any variations.

- Any adjustments required from the garage slab and the street levels are to be carried out internally on private property.

- Drainage design works must comply with the Woollahra DCP 2015 Chapter E2 – Stormwater and Flood Risk Management.

- Temporary ground anchors may be permitted, in accordance with Council’s “Rock Anchor Policy”.

- Services: Prior to any excavation works, the location and depth of all public utility services (telephone, cable TV, electricity, gas, water, sewer, drainage, etc.) must be ascertained. The Applicant must be responsible for all public utility adjustment/relocation works, necessitated by the development work and as required by the various public utility authorities and/or their agents.

- All public domain works must comply with the latest version of Council’s “Specification for Roadworks, Drainage and Miscellaneous Works” unless expressly provided otherwise by these conditions.  This specification and the application form can be downloaded from www.woollahra.nsw.gov.au.

- When an application under the Roads Act is required, then four (4) weeks is to be allowed for assessment.

- An application must be made to Council by the person who paid the security for release of the securities held under section 4.17 of the Act.

- The securities will not be released until the Occupation Certificate has been lodged with Council, Council has inspected the site and Council is satisfied that the public works have been carried out to Council’s requirements. Council may use part or all of the security to complete the works to its satisfaction if the works do not meet Council’s requirements.

- Council will only release the security upon being satisfied that all damage or all works, the purpose for which the security has been held have been remedied or completed to Council’s satisfaction as the case may be.

- When determining whether the works within public land are satisfactory, Council will consider the ownership, construction quality, maintenance, operations, and public utility of such item/s.

- Upon completion of each section of road, drainage and landscape work to Council's satisfaction, 90% of the bond monies held by Council for these works will be released upon application. 10% may be retained by Council for a further 6 month period and may be used by Council to repair or rectify any defects or temporary works during the 6 month period.

Condition Reason: To ensure the design of the road, footpaths, driveway crossings and public stormwater drainage works are detailed and approved under section 138 of the Roads Act 1993 and to ensure the works are completed to Council’s satisfaction.

Provision for Energy Supplies

Before the issue of any construction certificate:

a)	A survey is to be carried out of all utility services within and adjacent to the site. If necessary, this shall include relevant information from utility authorities and excavation, to determine the position and level of services.

b)	The applicant is to obtain written approval from Ausgrid for the relocation, adjustment or installation of new services, or existing services affected by, or required for the development.

c)	The applicant shall provide to the Principal Certifier a true copy of the plans developed by the applicant's Accredited Service Provider outlining the design and requirements for network modification and customer connection for the proposed development. The plans must have been reviewed and certified by Ausgrid to be deemed compliant of meeting this requirement.

Any required substation must be located within the boundaries of the site. Where an electricity substation is required, but no provision has been made to place it within the building and such substation has not been detailed upon the approved development consent plans an application under section 4.55 of the Act is required to be submitted to Council.

The design and location of any substation should have regard to the electrical substation provisions within the Woollahra Development Control Plan 2015, which require substations to be located and/or concealed so they are not visible from the street. The design and placement of electrical substations shall comply with Ausgrid’s relevant Network Standards and will prioritise the placement of the substation at the front of the property.

Where an electrical pillar is required, the electrical designer must consider the impact of all existing and proposed work when preparing their connection or relocation design. Site specific conditions such as existing/proposed property boundaries, building setbacks, other street furniture, street trees and pedestrian pathways will all be taken into account when specifying the final location of the pillar. The design should be compliant with Ausgrid’s Network Standards and satisfy Council’s objective to maintain a safe and accessible public domain for pedestrians:

a)	The design and location of underground and aboveground utility infrastructure shall consider the finished streetscape and not adversely impact existing pedestrian footpath, or, where the only permissible location for aboveground infrastructure will impact the footpath, the applicant shall widen the footpath to meet minimum accessibility requirements.

The construction certificate plans and specifications, required to be submitted under clause 7 of the Development Certification and Fire Safety Regulation, must detail provisions to meet the requirements of Ausgrid.

Where an electricity pillar and/or substation is required, the construction certificate plans and specifications must provide:

a)	A setback not less than 3m from any side or rear site boundary (or fire source feature) and not within the areas required to be kept clear of obstructions to vehicle visibility under clause 3.2.4 of AS2890.1 (See: Figures 3.2 and 3.3).

b)	That vegetation does not overhang or encroach within the substation site.

c)	That the substation is installed outside of the mature growth root zone of any trees to be retained, or proposed to be planted, to prevent root damage to underground cables. A setback not less than the 10m from any NSW Fire Brigade booster connection as prescribed by clause 5.6.3(d)(iii) of AS 2419.1 or be separated from any booster connections by a construction with a fire resistance rating of not less than FRL 90/90/90 for a distance of not less than 2 m each side of and 3 m above the upper hose connections in the booster assembly under clause 5.6.3(c)(ii) of AS 2419.1, and

d)	The Owner must dedicate to the appropriate energy authority, free of cost, an area of land adjoining the street alignment to enable a substation to be established, if required.  The size and location of the substation is to be in accordance with the requirements of the appropriate energy authority and Council.  The opening of any access doors must not intrude onto the public road reserve.

Notes:

- If the substation is not located within the building its location, all screen walls or fire separating walls must have been approved by the grant of development consent or amended development consent prior to the issue of any construction certificate for those works. Documentary evidence of compliance, including correspondence from the energy authority is to be provided to the Principal Certifier prior to issue of the construction certificate. The Principal Certifier must be satisfied that the requirements of the energy authority have been met prior to issue of the construction certificate.

- Where it is proposed to shield any booster connection or any building from any electricity pillar and/or substation under clause 5.6.3(c)(ii) of AS 2419.1 or by fire resisting construction under the BCA respectively and this construction has not been detailed upon the approved development consent plans such works should be considered inconsistent with consent under clause 19 of the Development Certification and Fire Safety Regulation. The Applicant must lodge with Council details for any such construction under section 4.55 of the Act to allow assessment under section 4.15 of the Act.

- Electricity pillar and/or Substations must not be located within the minimum sight distance at driveway entrances under Australian Standard AS/NZS 2890 (Set) Parking Facilities Set whether such driveways service the site or any adjoining land.

Condition Reason: To ensure the adequate provision of energy supply, and to ensure that any proposed electricity substation or the like does not compromise the design quality of the development.

Professional Engineering Details

Before the issue of any construction certificate, the construction certificate plans and specifications, required under clause 7 of the Development Certification and Fire Safety Regulation, must include detailed professional engineering plans and/or specifications for all structural, electrical, hydraulic, hydrogeological, geotechnical, mechanical and civil work complying with this consent, approved plans, and supporting documentation.

Detailed professional engineering plans and/or specifications must be submitted to the Principal Certifier with the application for any construction certificate.

Notes:

- This does not affect the right of the developer to seek staged construction certificates.

Condition Reason: To ensure professional engineering details and technical specifications are provided.

Engineer Certification

Before the issue of any construction certificate, engineer certification must be submitted to the Principal Certifier confirming that the structural design does not incorporate any temporary or permanent underpinning works or ground anchors, bolts, etc. which encroach outside the boundaries of the subject property.

This development consent does NOT give approval to any works outside the boundaries of the subject property including any underpinning works to any structures on adjoining properties and Council’s property.

Condition Reason: To ensure certification is provided that demonstrates all structural works are located within the boundaries of the site and do not include underpinning works to any structures on adjoining properties.

Geotechnical and Hydrogeological Design, Certification and Monitoring

Before the issue of the construction certificate, the applicant must submit, for approval by the Principal Certifier, a detailed geotechnical report prepared by a Geotechnical Engineer with National Engineering Register (NER) credentials in accordance with Chapter E2 of Council’s DCP and Council’s document “Guidelines for Preparation of Geotechnical and Hydrogeological Reports”. The report must include a Geotechnical / Hydrogeological Monitoring Program together with civil and structural engineering details for foundation retaining walls, footings, basement tanking, and subsoil drainage systems, as applicable, prepared by a professional engineer, who is suitably qualified and experienced in geotechnical and hydrogeological engineering.

These details must be certified by the professional engineer to:

- Provide appropriate support and retention to ensure there will be no ground settlement or movement, during excavation or after construction, sufficient to cause an adverse impact on adjoining property or public infrastructure,

- Provide appropriate support and retention to ensure there will be no adverse impact on surrounding property or infrastructure as a result of changes in local hydrogeology (behaviour of groundwater),

- Provide details of cut-off walls or similar controls prior to excavation such that any temporary changes to the groundwater level, during construction, will be kept within the historical range of natural groundwater fluctuations. Where the historical range of natural groundwater fluctuations is unknown, the design must demonstrate that changes in the level of the natural water table, due to construction, will not exceed 0.3m at any time,

- Provide tanking to below ground structures to prevent the entry of seepage water such that subsoil drainage/ seepage water is NOT collected and discharged to the kerb and gutter,

- Provide a Geotechnical and Hydrogeological Monitoring Program that:

- will detect any settlement associated with temporary and permanent works and structures,

- will detect deflection or movement of temporary and permanent retaining structures (foundation walls, shoring bracing or the like),

- will detect vibration in accordance with AS 2187.2 Appendix J including acceptable velocity of vibration (peak particle velocity),

- will detect groundwater changes calibrated against natural groundwater variations,

- details the location and type of monitoring systems to be utilised,

- details the pre-set acceptable limits for peak particle velocity and ground water fluctuations,

- details recommended hold points to allow for the inspection and certification of geotechnical and hydrogeological measures by the professional engineer, and details a contingency plan.

Condition Reason: To ensure that geotechnical and hydrogeological impacts are appropriately managed.

Ground Anchors

This development consent does NOT give approval to works or structures over, on or under adjoining properties, public roads and/or footpaths.

Before the issue of any construction certificate, if ground anchors are proposed:

a)	Prior written consent must be obtained from all relevant adjoining property owner(s) for the use of any ground anchors extending beyond the boundaries of the subject property.

b)	The use of permanent ground anchors under Council land is not permitted. Temporary ground anchors under Council’s land may be permitted, in accordance with Council’s “Rock Anchor Policy", where alternative methods of stabilisation would not be practicable or viable, and where there would be benefits in terms of reduced community impact due to a shorter construction period, reduced disruption to pedestrian and vehicular traffic on adjacent public roads, and a safer working environment.

c)	If temporary ground anchors under Council land are proposed, a separate application, including payment of fees, must be made to Council under Section 138 of the Roads Act 1993.  Application forms and Council’s “Rock Anchor Policy" are available from Council’s website.  Approval may be granted subject to conditions of consent. A minimum of four weeks should be allowed for assessment.

Notes:

- To ensure that this work is completed to Council’s satisfaction, this consent by separate condition, may impose one or more Infrastructure Works Bonds.

- Road has the same meaning as in the Roads Act 1993.

- Clause 17 of the Roads (General) Regulation 2018 prohibits excavation in the vicinity of roads as follows: “Excavations adjacent to road - A person must not excavate land in the vicinity of a road if the excavation is capable of causing damage to the road (such as by way of subsidence) or to any work or structure on the road.”  Separate approval is required under the Roads Act 1993 for any underpinning, shoring, soil anchoring (temporary) or the like within or under any road.  Council will not give approval to permanent underpinning, shoring, soil anchoring within or under any road.

Condition Reason: To ensure the relevant approval is gained for any temporary ground anchors.

Parking Facilities

Before the issue of any construction certificate, the construction certificate plans and specifications required under clause 7 of the Development Certification and Fire Safety Regulation, must include detailed plans and specifications for all bicycle, car and commercial vehicle parking in compliance with AS2890.3: Parking Facilities - Bicycle Parking Facilities, AS 2890.6: Parking facilities - Off-street parking for people with disabilities, AS/NZS 2890.1: Parking Facilities - Off-Street Car Parking and AS 2890.2: Off-Street Parking: Commercial Vehicle Facilities respectively.

The plans must satisfy the following requirement(s):

- A 2m x 2.5m driveway sightline splay be provided, clear of sight obstruction, along exit side of the northern driveway and along both sides of the driveway for the loading dock to ensure visibility between vehicles leaving the site and traffic on the frontage road, including pedestrians on the frontage road footpath. This should be clearly depicted on the architectural plans.

- Sight distance requirements must comply with Clause 3.2.4 and Figure 3.3 of AS2890.1:2004.

- Concrete median island be provided for the northern driveway to allow separate and independent operations for ingress and egress traffic.

- ‘STOP’ sign and line marking be provided, wholly within the site, for vehicles exiting the site from both driveways.

- Signage and/or line marking be provided to ensure compliant allocation of parking spaces to retail and residential uses. A minimum of ten (10) parking spaces must be allocated for staff parking to minimise parking pressure on surrounding streets.

- Signage and line marking be provided within the basement car park to clearly guide the traffic flow to allow efficient movements. Convex mirrors should be provided at turning points to assist with visibility and avoid traffic conflicts.

- Yellow epoxy zebra crossing be provided within the basement car park to allow pedestrians to walk safely to and from the parking spaces, trolley areas and the lift.

- Accessible parking space and the adjacent shared space must be designed to comply with parking configurations stipulated in AS 2890.6.

- Access levels and grades must comply with access levels and grades required by Council under the Roads Act 1993.

The Principal Certifier has no discretion to reduce or increase the number or area of car parking or commercial parking spaces required to be provided and maintained by this consent.

Condition Reason: To ensure parking facilities are designed in accordance with the Australian Standard.

Woollahra Local Traffic Committee Approval

Before the issue of a construction certificate, the applicant is to liaise with Council’s Traffic and Transport Department for any required adjustment of parking restrictions. This matter must be referred to the Woollahra Local Traffic Committee for review and approval.

The parking restrictions relate to the following:

- No Parking signage required along Albemarle Avenue between the loading dock driveway and the traffic lights at the intersection of Albemarle Avenue and Old South Head Road. The purpose of this is to maintain traffic flow and reduce queuing at the Albemarle Avenue intersection by maintaining an unobstructed a left turn lane.

- No parking on the southbound kerbside lane of Old South Head Road (between Beaumont Street and the existing ‘No Stopping’ sign at 509 Old South Head Road), between 3-7pm weekdays. The purpose of this is to maintain traffic flow on Old South Head Road. These parking spaces are on the Waverley Council side of the road and therefore the applicant is required to liaise with Waverley Council.

Notes:

- This process can take up to 8 weeks.

- All works associated with the signage changes shall be carried out at the full cost to the applicant.

Stormwater Management Plan

Before the issue of any construction certificate, the applicant must submit, for approval by the Principal Certifier, detailed stormwater management plans prepared and certified by a chartered professional civil engineer, which detail the following:

- General design in accordance with the concept stormwater management plans, referenced 240961-Rev C, prepared by Northrop, dated 02/12/24, other than amended by this and other conditions,

- The discharge of stormwater from the site, by direct connection, to a new kerb inlet pit (KIP) over the existing Sydney Water’s underground pipeline in Albemarle Avenue,

- The provision of a minimum 1.2m x 1.2m boundary junction pit shall be provided prior to discharging stormwater from the site to the street drainage system. The stormwater outlet pipe must be made by using 300 diameter pipe which must be located within the frontage of the subject site with a minimum grade of 1% to comply with the Council’s DCP and AS3500.3,

- In order to prevent any backwater effects, invert level of the proposed stormwater outlet pipe at the stormwater treatment chamber must be designed so that it is set above the 1% AEP flood level in Albemarle Avenue,

- The installation of stormwater filtration /treatment system which includes but not limited to the installation of ATLAN Stormsack and 2 x ATLAN Filter (SF.30-EMC) to achieve the minimum the water quality targets stipulated in Chapter E2.2.3 of the Council’s DCP,

- All below ground structures are to be fully tanked such that subsoil drainage/ seepage water is not collected and discharged to the kerb and gutter in accordance with Chapter E2.2.5 and 2.2.10 of the Council’s DCP. Notation to this requirement shall be clearly depicted on the drawings,

- The dimensions of all drainage pits and access grates must comply with AS3500.3,

- Compliance the objectives and performance requirements of the BCA, and

- General compliance with the Council’s Woollahra DCP 2015 Chapter E2 – Stormwater and Flood Risk Management.

The Stormwater Management Plan must also include the following specific requirements:

Layout plan

A detailed drainage plan at a scale of 1:100 based on drainage calculations prepared in accordance with the Australian Government publication, Australian Rainfall and Run-off, 2019 edition or most current version thereof. It must include:

- All pipe layouts, dimensions, grades, lengths and material specification,

- Location of proposed rainwater tanks,

- All invert levels reduced to Australian Height Datum (AHD),

- Location and dimensions of all drainage pits,

- Point and method of connection to Councils drainage infrastructure, and

- Overland flow paths over impervious areas.

For the proposed stormwater connection to the Council’s drainage infrastructure, separate approval under Section 138 of the Roads Act 1993 must be obtained from Council for those works prior to the issue of any Construction Certificate.

All Stormwater Drainage System work within any road or public place must comply with Woollahra Municipal Council’s Specification for Roadworks, Drainage and Miscellaneous Works (2012).

Note: 	This Condition is imposed to ensure that site stormwater is disposed of in a controlled and sustainable manner.

Note: 	The collection, storage and use of rainwater is to be in accordance with Standards Australia HB230-2008 “Rainwater Tank Design and Installation Handbook”.

Condition Reason: To ensure that site stormwater is disposed of in a controlled and sustainable manner.

Non-Gravity Drainage Systems

Before the issue of any construction certificate, the construction certificate plans and specifications required under clause 7 of the Development Certification and Fire Safety Regulation, must include a Stormwater Management Plan for the site.

The Stormwater Management Plan must detail the non-gravity drainage systems (charged, siphon, pump/sump systems) being designed in accordance with Woollahra DCP 2015 Chapter E2 – Stormwater and Flood Risk Management.

Notes:

- The Woollahra DCP is available from Council's website www.woollahra.nsw.gov.au

Flood Protection

Before the issue of any construction certificate, the construction certificate plans and specifications required under clause 7 of the Development Certification and Fire Safety Regulation, must include a Flood Risk Management Plan on the basis of the Flood Planning Level (FPL).

Flood Warning:

- A permanent flood risk management plan shall be installed in a prominent area of the basement carpark,

- A permanent flood risk management plan shall be installed in a prominent area frequented by the shopping centre employees,

- A permanent flood risk management plan shall be installed in a prominent lobby,

Fencing

- The development shall be protected by a waterproof wall/facade designed to protect the development to the flood planning level,

Below Ground Car parking

- The driveway entry shall be protected by a physical crest with the threshold set to the flood planning level of 12.7m AHD,

- The loading dock pedestrian entry shall be protected by an automatic mechanical flood barrier with the threshold set to the flood planning level of 12.8m AHD,

- The main loading dock entry shall be protected by an automatic mechanical flood barrier with the threshold set to the flood planning level of 12.8m AHD,

- Permanent brass plaques shall be mounted adjacent to all automatic mechanical flood barriers explaining their purpose and operation,

- Emergency self-powered lights, indicting the safe exit to a flood free area above the probable maximum flood (PMF) shall be installed in the car parking area,

Floor levels

- The pedestrian entry to the lobby shall be protected by an automatic mechanical flood barrier with the threshold set to the flood planning level of 12.9m AHD,

- The main pedestrian entry from Old South Head Road shall be protected by an automatic mechanical flood barrier with the threshold set to the flood planning level of 13.2m AHD,

- The minor pedestrian entry from Old South Head Road shall be protected by an automatic mechanical flood barrier with the threshold set to the flood planning level of 13.2m AHD,

- Permanent brass plaques shall be mounted adjacent to all automatic mechanical flood barriers explaining their purpose and operation,

Flood Proof Material

- Flood compatible materials shall be used for all flood exposed construction,

Electricals

- All flood exposed electrical wiring and equipment shall be waterproofed or able to be easily isolatable,

Certification

- All flood protection measures shall be inspected and certified as fit for purpose after construction is complete by an engineer experienced in flood mitigation,

Flood protection is to comply with Woollahra DCP 2015, Part E General Controls for All Development, Chapter E2 –Stormwater and Flood Risk Management.

Notes:

- The revised driveway profile, gradients and transitions must be in accordance with Australian Standard 2890.1, Part 1: Off-street car parking. The driveway profile submitted to Council must contain all relevant details: reduced levels, proposed grades and distances. Council will not allow alteration to existing reduced levels within the road or any other public place to achieve flood protection.

Condition Reason: To ensure the development incorporates flood inundation protection measures.

Acoustics – Noise & Vibration Mitigation of Loading Dock

The recommendations for noise and vibration mitigation measures, as detailed in Section 4 of the Acoustic Assessment – Loading Dock Noise & Vibration Assessment prepared by E-LAB Consulting – Project No. P01374 – 25 March 2025, to ensure that loading dock noise impacts are controlled to residential apartments above the loading dock, shall be fully incorporated into the design and construction of the building envelope and be shown on plans accompanying the Construction Certificate application.

The construction methodology and plans accompanying the Construction Certificate application, shall be assessed and certified in writing by a suitably qualified acoustic consultant, to verify conformance with the requirements of the aforementioned acoustic assessment report. The written certification from the suitably qualified acoustic consultant shall be submitted to and approved by the Principal Certifying Authority (PCA) prior to issue of the Construction Certificate.

Note 1: Internal noise levels shall be designed to achieve the recommendations in Australian Standard AS2106:2016 Acoustics: Recommended design sound levels and reverberation times for building interiors.

Note 2: ‘Suitably qualified acoustic consultant’ means a consultant who possesses Australian Acoustical Society membership or are employed by an Association of Australasian Acoustical Consultants (AAAC) member firm.

Condition Reason:  To ensure interior design sound levels are maintained for occupants of the residential apartments above the loading dock.

Acoustics – Building Design and Construction

The recommendations for acoustic treatments of the building envelope provided for in the approved acoustic report prepared by E-LAB Consulting – Document Reference P01374 – Revision 002 – 02 October 2024, shall be incorporated into the design and construction of the development and be shown on plans accompanying the Construction Certificate application.

The construction methodology and plans accompanying the Construction Certificate application, shall be assessed and certified in writing by a suitably qualified acoustic consultant, to verify conformance with the requirements of the aforementioned acoustic report. The written certification from the suitably qualified acoustic consultant shall be submitted to and approved by the Principal Certifying Authority (PCA) prior to issue of the Construction Certificate.

Note: ‘Suitably qualified acoustic consultant’ means a consultant who possesses Australian Acoustical Society membership or are employed by an Association of Australasian Acoustical Consultants (AAAC) member firm.

Condition Reason: To ensure internal acoustic amenity is maintained for occupants of the building.

Acoustics – Mechanical Plant Services

Before the issue of any construction certificate, an Acoustic Report, prepared by a suitably qualified acoustic consultant, shall assess the environmental noise impacts associated with the operation of all mechanical plant services.

The Acoustic Report shall have regard to noise criteria of Chapter D3, Section D3.10 (Site Facilities) of the WDCP 2015, which states, ‘noise emissions from mechanical plant equipment must not exceed the background noise levels when measured at the boundary of the development site’.

The cumulative noise emissions from all mechanical plant services, at all times shall comply with Chapter D3, Section D3.10 (Site Facilities) of the WDCP 2015.

Where necessary, noise amelioration treatments will be incorporated in the mechanical plant services design to ensure that cumulative noise levels comply with the noise emission criteria. Compliance noise testing is to be undertaken following the installation of the mechanical plant equipment.

Note 1: Compliance with the Night-time (RBL) noise emission criteria will ensure compliance at all other times.

Note 2: Note: ‘Suitably qualified acoustic consultant’ means a consultant who possesses Australian Acoustical Society membership or are employed by an Association of Australasian Acoustical Consultants (AAAC) member firm.

Condition Reason: To protect the acoustic amenity of the surrounding environment.

Acoustics – Demolition and Construction

At the Construction Certificate stage, a Construction Noise Management Plan (CNMP) is to be prepared by a suitably qualified acoustic consultant. The construction noise will be assessed with consideration to DECCW Interim Construction Noise Guidelines (ICNG) (2009) to provide noise management levels for construction noise at residential and other potentially sensitive receivers. The management levels are to be calculated based on the adopted rating background level (RBL) at nearby locations.

The CNMP shall be a site specific plan developed to ensure that appropriate work practices are implemented during the demolition, excavation and construction to minimise noise and vibration impact. The CNMP shall include:

- Attended and unattended noise monitoring at locations indicative of noise sensitive receivers.

- Site description.

- Staging of construction.

- Establishment of project specific airborne noise construction goals based on monitored existing noise levels.

- Prediction of construction noise levels from proposed construction works.

- Recommendation of environmental noise control options/management practices.

- Complaints handling and community liaison procedures.

Condition Reason: To protect the acoustic amenity of the surrounding environment.

Light and Ventilation

Before the issue of any construction certificate, the construction certificate plans and specifications required under clause 7 of the Development Certification and Fire Safety Regulation, must detail all lighting, mechanical ventilation or air-conditioning systems complying with Part F.4 of the BCA or clause 3.8.4 and 3.8.5 of the BCA Housing Provisions, inclusive of AS 1668.1, AS 1668.2 and AS/NZS 3666.1.

If an alternate solution is proposed then the construction certificate application must include a statement as to how the performance requirements of the BCA are to be complied with and support the performance based solution by expert evidence of suitability.

This condition does not set aside the mandatory requirements for ‘Legionella Control’ under the Public Health Act 2010 and Public Health Regulation 2022 in relation to regulated systems. This condition does not set aside the effect of the Protection of the Environment Operations Act 1997 in relation to offensive noise or odour.

Notes:

- Clause 69 of the Regulation requires compliance with the BCA. Clause 19 of the Development Certification and Fire Safety Regulation prevents the issue of a construction certificate unless the Principal Certifier is satisfied that compliance has been achieved.

- Part 3, Division 1 of the Development Certification and Fire Safety Regulation details what information must be submitted with any construction certificate. It is the Applicant's responsibility to demonstrate compliance through the construction certificate application process.

- Applicants must also consider possible noise and odour nuisances that may arise.  The provisions of the Protection of the Environment Operations Act 1997 have overriding effect if offensive noise or odour arises from the use.

- Applicants must pay attention to the location of air intakes and air exhausts relative to sources of potentially contaminated air and neighbouring windows and air intakes respectively, see section 2 and 3 of AS 1668.2

Condition Reason: To ensure the development is provided with adequate light and ventilation.

Acoustic Certification of Mechanical Plant and Equipment

Before the issue of any construction certificate, the construction certificate plans and specifications required under clause 7 of the Development Certification and Fire Safety Regulation, must be accompanied by a certificate from a professional acoustic engineer certifying that the noise level measured at any boundary of the site at any time while the proposed mechanical plant and equipment is operating will not exceed the background noise level.  Where noise sensitive receivers are located within the site, the noise level is measured from the nearest strata, stratum or community title land and must not exceed background noise level, at any time.

The background noise level is the underlying level present in the ambient noise, excluding the subject noise source, when extraneous noise is removed. For assessment purposes the background noise level is the LA90, 15 minute level measured by a sound level meter.

Where sound attenuation is required this must be detailed.

Notes:

- Further information including lists of Acoustic Engineers can be obtained from:

-	Australian Acoustical Society - professional society of noise-related professional www.acoustics.asn.au

-	Association of Australian Acoustical Consultant - professional society of noise related professionals www.aaac.org.au

Condition Reason: To ensure the development does not result in any unreasonable acoustic impacts.

Ventilation - Enclosures used by Vehicles (Car parks, automotive service, enclosed driveways, loading docks and the like)

Before the issue of any construction certificate, the basement carpark in which vehicles powered by internal combustion engines are parked, serviced or operated are required to comply with Section 4 of Australian Standard 1668.2-2012. In general air distribution must achieve uniform dilution of contaminants in the garage and maintain contaminant concentrations below recommended exposure standards.

The basement carpark must be naturally ventilated or provided with a combination of both supply and exhaust mechanical ventilation. The applicant is to determine the method of ventilation of the basement carpark and provide details to the Certifying Authority accordingly. Except as varied, the basement carpark shall be mechanically ventilated by a combination of general exhaust and supply flow rates in accordance with Australian Standard 1668.2-2012.

Condition Reason: To ensure the development is adequately ventilated.

Ventilation - Internal Sanitary Rooms

Before the issue of any construction certificate, all internal sanitary rooms and laundry facilities not provided with natural ventilation must be provided with a system of mechanical exhaust ventilation in accordance with the Minimum Exhaust Ventilation Flow Rates of AS 1668.2-2012. Details of any proposed mechanical ventilation system(s) must be submitted with the Construction Certificate plans and specifications to the Certifying Authority demonstrating compliance with AS 1668 Parts 1 & 2.

Condition Reason: To ensure the development is adequately ventilated.

Tree Protection Plan and Specification

Before the issue of any construction certificate, the Principal Certifier must ensure the measures for tree protection detailed in this consent are in place. The construction certificate plans and specifications must show the following information:

- Trees to be numbered and coloured in accordance with these conditions:

  - shaded green where required to be retained and protected

  - shaded red where authorised to be removed

  - shaded yellow where required to be transplanted

  - shaded blue where required to be pruned

- Tree Protection Plan and Specification prepared by an arborist with a minimum qualification of AQF 5 including the following:

  - Tree Location Plan (to scale) based on/overlaid with the approved plans, indicating trees to be retained, removed or transplanted, and the location of tree protection zones (TPZ) and structural root zones (SRZ) of trees to be retained and protected.

  - The plan must also include site specific tree protection recommendations such as fencing, ground and trunk protection and other protection devices.

  - Tree Protection Specification (written document) with tree protection requirements included from this consent and in accordance with sections 4 & 5 of AS4970, providing guidance for the implementation of tree protection methods.

  - To minimise construction damage, the plan must show specific areas requiring works to be done under direct supervision of the project arborist.

- References to applicable tree management plan, arborists report or transplant method statement.

This plan must be kept on site until the issue of the occupation certificate for the whole building.

Condition Reason: To ensure the construction certificate includes the approved tree management details, and all measures are implemented.

Standard Condition D.30

# BEFORE BUILDING WORK COMMENCES

Compliance with Building Code of Australia and insurance requirements under the Home Building Act 1989

Before any building work commences, and under section 4.17(11) of the Act, the following conditions are prescribed in relation to a development consent for development that involves any building work:

a)	that the work must be carried out in accordance with the requirements of the Building Code of Australia,

b)	in the case of residential building work for which the Home Building Act 1989 requires there to be a contract of insurance in force in accordance with Part 6 of that Act, that such a contract of insurance is in force before any building work authorised to be carried out by the consent commences.

This condition does not apply:

a)	to the extent to which an exemption is in force under the Home Building Regulation 2014, or

b)	to the erection of a temporary building.

In this condition, a reference to the BCA is a reference to that code as in force on the date the application for the relevant Construction Certificate is made.

Notes:

- This condition must be satisfied prior to commencement of any work in relation to the contract of insurance under the Home Building Act 1989.  This condition also has effect during the carrying out of all building work with respect to compliance with the Building Code of Australia.

- All new guttering is to comply with the provisions of AS3500.

Condition Reason: To ensure that works are carried out in accordance with the Building Code of Australia and any required contract of insurance is in force.

(Autotext 1E)

Erosion and Sediment Controls – Installation

Before any building work commences, water pollution, erosion and sedimentation controls must be installed and maintained in accordance with:

a)	The Soil and Water Management Plan if required under this consent;

b)	“Do it Right On Site, Soil and Water Management for the Construction Industry” and accompanying factsheets published by the Southern Sydney Regional Organisation of Councils, and

c)	“Managing Urban Stormwater - Soils and Construction” 2004 published by the NSW Government (The Blue Book).

Where there is any conflict The Blue Book takes precedence.

Notes:

- The International Erosion Control Association – Australasia (www.austieca.com.au/) lists consultant experts who can assist in ensuring compliance with this condition.

- Where Soil and Water Management Plan is required for larger projects it is recommended that this be produced by a member of the International Erosion Control Association – Australasia.

- The “Do it Right On Site, Soil and Water Management for the Construction Industry” publication and the accompanying factsheets can be downloaded from www.woollahra.nsw.gov.au and The Blue Book is available at www.environment.nsw.gov.au

- A failure to comply with this condition may result in penalty infringement notices, prosecution, notices and orders under the Act and/or the Protection of the Environment Operations Act 1997 without any further warning.  It is a criminal offence to cause, permit or allow pollution.

- Section 257 of the Protection of the Environment Operations Act 1997 provides inter alia that “the occupier of premises at or from which any pollution occurs is taken to have caused the pollution”.

- Warning: Irrespective of this condition any person occupying the site may be subject to proceedings under the Protection of the Environment Operations Act 1997 where pollution is caused, permitted or allowed as the result of their occupation of the land being.

Condition Reason: To prevent potential water pollution and dust nuisance.

Building - Construction Certificate, Appointment of Principal Certifier, Appointment of Principal Contractor and Notice of Commencement (Part 6, Division 6.3 of the Act)

Building work must not commence, until:

a)	A construction certificate for the building work has been issued by the consent authority, the Council (if the Council is not the consent authority) or an accredited Certifier, and

b)	The person having the benefit of the development consent has:

- appointed a Principal Certifier for the building work, and

- notified the Principal Certifier that the person will carry out the building work as an Owner-builder, if that is the case, and

c)	The Principal Certifier has, no later than 2 days before the building work commences:

- notified the consent authority and the Council (if the Council is not the consent authority) of his or her appointment, and

- notified the person having the benefit of the development consent of any critical stage inspections and other inspections that are to be carried out in respect of the building work, and

d)	The person having the benefit of the development consent, if not carrying out the work as an Owner-builder, has:

- appointed a Principal Contractor for the building work who must be the holder of a contractor licence if any residential building work is involved, and

- notified the Principal Certifier of any such appointment, and

- unless that person is the Principal Contractor, notified the Principal Contractor of any critical stage inspections and other inspections that are to be carried out in respect of the building work, and

- given at least 2 days’ notice to the Council of the person’s intention to commence the erection of the building.

Notes:

- Building has the same meaning as in section 1.4 of the Act and includes part of a building and any structure or part of a structure.

- New building has the same meaning as in section 6.1 of the Act and includes an altered portion of, or an extension to, an existing building.

- The commencement of demolition works associated with an altered portion of, or an extension to, an existing building is considered to be the commencement of building work requiring compliance with section 6.6(2) of the Act (including the need for a Construction Certificate) prior to any demolition work. See: Over our Dead Body Society Inc v Byron Bay Community Association Inc [2001] NSWLEC 125.

- Construction Certificate Application, PC Service Agreement and Notice of Commencement forms can be downloaded from Council’s website www.woollahra.nsw.gov.au

- It is an offence for any person to carry out the erection of a building in breach of this condition and in breach of section 6.6(2) of the Act.

- Under the Home Building Act 1989 any property owner who intends undertaking construction work to a dwelling house or dual occupancy to the value of $12,000 or over must complete an approved education course and obtain an owner-builder permit from NSW Fair Trading.

Condition Reason: To ensure a construction certificate has been issued, a Principal Certifier is appointed, a Principal Contractor (if applicable) is appointed, and a notice of commencement has been submitted.

Housing and Productivity Contribution Order 2024

1)	The housing and productivity contribution (HPC) set out in the table below is required to be made:

| Housing and productivity contribution | Amount |
| --- | --- |
| Housing and productivity contribution (base component) | $180,171.69 |

2)	The HPC must be paid before the issue of the first construction certificate in relation to the development, or before the commencement of any work authorised by this consent (if no construction certificate is required). However, if development is any of the kinds set out in the table below, the total housing and productivity contribution must be paid as set out in the table:

| HPC component of development | Time by which HPC must be paid |
| --- | --- |
| Residential subdivision | Before the issue of the first subdivision certificate for the residential subdivision. Note: This is subject to clause 20 relating to staged subdivision |
| Residential subdivision and other residential development, or commercial or industrial development, that requires a construction certificate. | Before the issue of: - the first subdivision certificate for the residential subdivision, or - the first construction certificate for the other residential development or the commercial or industrial development, whichever is the earlier. |
| Medium or high-density residential development for which a construction certificate is not required. Note: Medium or high-density residential development may not require a construction certificate if the medium or high-density residential accommodation authorised by the consent will result from a change of use of existing building. | Before the issue of the first strata certificate for the medium or high density residential development. |
| Residential subdivision and medium or high-density residential development for which a construction certificate is not required. | Before the issue of: - the first subdivision certificate for the residential subdivision, or - the first strata certificate (if any is required) for the medium or high density residential development, whichever is the earlier. |
| Development for the purposes of manufactured home estate for which a construction certificate is not required | Before the installation of the first manufactured home on a dwelling site. |

3)	In this condition, HPC Order means the Environmental Planning and Assessment (Housing and Productivity Contribution) Order 2024.

4)	The HPC must be paid using the NSW planning portal (https://pp.planningportal.nsw.gov.au/).

5)	At the time of payment, the amount of the HPC is to be adjusted in accordance with the Environmental Planning and Assessment (Housing and Productivity Contributions) Order 2024 (HPC Order).

6)	The HPC may be made wholly or partly as a non-monetary contribution (apart from any transport project component) if the Minister administering the Environmental Planning and Assessment Act 1979 agrees.

7)	The HPC is not required to be made to the extent that a planning agreement excludes the application of Subdivision 4 of Division 7.1 of the Environmental Planning and Assessment Act 1979 to the development, or the HPC Order exempts the development from the contribution.

8)	The amount of the contribution may be reduced under the HPC Order, including if payment is made before 1 July 2025.

Notes:

- This condition is to be used for development consents (other than complying development certificates, concept DAs or staged residential subdivision).

- This condition is based upon the Department of Planning and Environment’s standard HPC condition.

# DURING BUILDING WORK

Compliance with BCA and Insurance Requirements under the Home Building Act 1989

While site work is being carried out:

a)	work must be carried out in accordance with the requirements of the Building Code of Australia (BCA),

b)	in the case of residential building work for which the Home Building Act 1989 requires there to be a contract of insurance in force in accordance with Part 6 of that Act, that such a contract of insurance is in force before any building work authorised to be carried out by the consent commences.

This condition does not apply:

a)	to the extent to which an exemption is in force under the Development Certification and Fire Safety Regulations, or

b)	to the erection of a temporary building.

In this clause, a reference to the BCA is a reference to that Code as in force on the date the application for the relevant construction certificate is made.

For the purposes of section 4.17(11) of the Act, the above condition is prescribed in relation to a development consent for development that involves any building work.

Notes:

- All new guttering is to comply with the provisions of AS 3500.

Condition Reason: To ensure compliance with the BCA and Home building Act 1989.

Requirement to Notify about New Evidence

While site work is being carried out, any new information that comes to light, which has the potential to alter previous conclusions about site contamination, heritage significance, threatened species or other relevant matters must be immediately notified to Council and the Principal Certifier.

Condition Reason: To ensure Council and the Principal Certifier are made aware of new information.

Critical Stage Inspections

While site work is being carried out, critical stage inspections must be called for by the Principal Contractor or Owner-builder as required by the Principal Certifier, any PC service agreement, the Act, the Development Certification and Fire Safety Regulation, and the Regulation.

Work must not proceed beyond each critical stage until the Principal Certifier is satisfied that work is proceeding in accordance with this consent, the construction certificate(s) and the Act.

Critical stage inspections means the inspections prescribed by the Development Certification and Fire Safety Regulations, and Regulations for the purposes of section 6.5 of the Act or as required by the Principal Certifier and any PC Service Agreement.

Notes:

- The Principal Certifier may require inspections beyond mandatory critical stage inspections in order that the Principal Certifier be satisfied that work is proceeding in accordance with this consent.

- The Principal Certifier may, in addition to inspections, require the submission of Compliance Certificates, survey reports or evidence of suitability in accordance with Part A2G2 of the BCA in relation to any matter relevant to the development.

Condition Reason: To ensure that building work progresses in accordance with the approved plans, conditions of consent, and requirements of the act.

Hours of Work –Amenity of the Neighbourhood

While site work is being carried out:

a)	No work must take place on any Sunday or public holiday.

b)	No work must take place before 7am or after 5pm any weekday.

c)	No work must take place before 7am or after 1pm any Saturday.

d)	The following work must not take place before 9am or after 4pm any weekday, or before 9am or after 1pm any Saturday or at any time on a Sunday or public holiday:

-	piling,

-	piering,

-	rock or concrete cutting, boring or drilling,

-	rock breaking,

-	rock sawing,

-	jack hammering, or

-	machine excavation.

e)	No loading or unloading of material or equipment associated with the activities listed in part d) above must take place before 9am or after 4pm any weekday, or before 9am or after 1pm any Saturday or at any time on a Sunday or public holiday.

f)	No operation of any equipment associated with the activities listed in part d) above must take place before 9am or after 4pm any weekday, or before 9am or after 1pm any Saturday or at any time on a Sunday or public holiday.

g)	No rock excavation being cutting, boring, drilling, breaking, sawing , jack hammering or bulk excavation of rock, must occur without a 15 minute interval break within every hour.

Notes:

- The use of noise and vibration generating plant and equipment and vehicular traffic, including trucks in particular, significantly degrade the amenity of neighbourhoods and more onerous restrictions apply to these activities.  This more invasive work generally occurs during the foundation and bulk excavation stages of development.  If you are in doubt as to whether or not a particular activity is considered to be subject to the more onerous requirement (9am to 4pm weekdays and 9am to 1pm Saturdays) please consult with Council.

- Each and every breach of this condition by any person may be subject to a separate penalty infringement notice or prosecution.

- The delivery and removal of plant, equipment and machinery associated with wide loads subject to Transport for NSW and NSW Police restrictions on their movement outside the approved hours of work will be considered on a case by case basis.

- Compliance with these hours of work does not affect the rights of any person to seek a remedy to offensive noise as defined by the Protection of the Environment Operations Act 1997, the Protection of the Environment Operations (Noise Control) Regulation 2017.

- NSW EPA Noise Guide is available at www.epa.nsw.gov.au/noise/nglg.htm

Condition Reason: To mitigate the impact of work upon the amenity of the neighbourhood.

Public Footpaths – Safety, Access and Maintenance

While site work is being carried out, any person acting with the benefit of this consent must:

a)	Not erect or maintain any gate or fence that swings out, or encroaches upon the road or the footway.

b)	Not use the road or footway for the storage of any article, material, matter, waste or thing.

c)	Not use the road or footway for any work.

d)	Keep the road and footway in good repair free of any trip hazard or obstruction.

e)	Any damage caused to the road, footway, vehicular crossing, nature strip or any public place must be immediately made safe and then repaired, to the satisfaction of Council.

f)	Not stand any plant and equipment upon the road or footway.

g)	If it is proposed to locate any site fencing, hoardings, skip bins or other articles upon any part of the footpath, nature strip or any public place, or operate a crane, hoist or concrete pump on or over Council land, an application must be submitted to and approved by Council beforehand.

h)	Provide a clear safe pedestrian route a minimum of 1.5m wide.

i)	Protect heritage listed street name inlays located in the footpath, kerb and gutter, and any other structure, to ensure they are not removed or damaged during development.

This condition does not apply to the extent that a permit or approval exists under the section 148B of the Road Transport Act 2013, section 138 of the Roads Act 1993 or section 68 of the Local Government Act 1993 except that at all time compliance is required with:

a)	Australian Standard AS 1742 (Set): Manual of uniform traffic control devices and all relevant parts of this set of standards.

b)	Australian Road Rules.

Notes:

- Section 148B of the Road Transport Act 2013 allows the NSW Police to close any road or road related area to traffic during any temporary obstruction or danger to traffic or for any temporary purpose.

- Section 138 of the Roads Act 1993 provides that a person must not:

-	erect a structure or carry out a work in, on or over a public road, or

-	dig up or disturb the surface of a public road, or

-	remove or interfere with a structure, work or tree on a public road, or

-	pump water into a public road from any land adjoining the road, or

-	connect a road (whether public or private) to a classified road,

-	otherwise than with the consent of the appropriate roads authority.

- Section 68 of the Local Government Act 1993 provides that a person may carry out certain activities only with the prior approval of the Council including:

-	Part C Management of waste:

a)	For fee or reward, transport waste over or under a public place

b)	Place waste in a public place

c)	Place a waste storage container in a public place.

-	Part E Public roads:

a)	Swing or hoist goods across or over any part of a public road by means of a lift, hoist or tackle projecting over the footway

b)	Expose or allow to be exposed (whether for sale or otherwise) any article in or on or so as to overhang any part of the road or outside a shop window or doorway abutting the road, or hang an article beneath an awning over the road.

Condition Reason: To ensure safe access is maintained to footpaths and roads during building works.

Maintenance of Environmental Controls

While site work is being carried out, the following monitoring, measures and

controls must be maintained:

a)	erosion and sediment controls,

b)	dust controls,

c)	dewatering discharges,

d)	noise controls,

e)	vibration monitoring and controls, and

f)	ablutions.

Support of Adjoining Land and Buildings

While site work is being carried out, a person must not to do anything on or in relation to the site (the supporting land) that removes the support provided by the supporting land to any other land (the supported land) or building (the supported building).

For the purposes of this condition, supporting land includes the natural surface of the site, the subsoil of the site, any water beneath the site, and any part of the site that has been reclaimed.

Notes:

- This condition does not authorise any trespass or encroachment upon any adjoining or supported land or building whether private or public.  Where any underpinning, shoring, soil anchoring (temporary or permanent) or the like is considered necessary upon any adjoining or supported land by any person the Principal Contractor or Owner-builder must obtain:

-	the consent of the owners of such adjoining or supported land to trespass or encroach, or

-	an access order under the Access to Neighbouring Land Act 2000, or

-	an easement under section 88K of the Conveyancing Act 1919, or

-	an easement under section 40 of the Land and Environment Court Act 1979 as appropriate.

- Section 177 of the Conveyancing Act 1919 creates a statutory duty of care in relation to support of land. Accordingly, a person has a duty of care not to do anything on or in relation to land being developed (the supporting land) that removes the support provided by the supporting land to any other adjoining land (the supported land).

- Clause 17 of the Roads Regulation 2018 prohibits excavation in the vicinity of roads as follows: “Excavations adjacent to road - A person must not excavate land in the vicinity of a road if the excavation is capable of causing damage to the road (such as by way of subsidence) or to any work or structure on the road.”  Separate approval is required under the Roads Act 1993 for any underpinning, shoring, soil anchoring (temporary) or the like within or under any road.  Council will not give approval to permanent underpinning, shoring, soil anchoring within or under any road.

- The encroachment of work or the like is a civil matter of trespass or encroachment and Council does not adjudicate or regulate such trespasses or encroachments except in relation to encroachments upon any road, public place, Crown land under Council’s care control or management, or any community or operational land as defined by the Local Government Act 1993.

Condition Reason: To ensure that the support of adjoining land is not removed.

Erosion and Sediment Controls – Maintenance

While site work is being carried out, water pollution, erosion, and sedimentation controls must be maintained in accordance with:

a)	the Soil and Water Management Plan required under this consent,

b)	“Do it Right On Site, Soil and Water Management for the Construction Industry” and the accompanying factsheets published by the Southern Sydney Regional Organisation of Councils, and

c)	“Managing Urban Stormwater - Soils and Construction” 2004 published by the NSW Government (The Blue Book).

Where there is any conflict The Blue Book takes precedence.

Notes:

- A failure to comply with this condition may result in penalty infringement notices, prosecution, notices and orders under the Act and/or the Protection of the Environment Operations Act 1997 without any further warning.  It is a criminal offence to cause, permit or allow pollution.

- Section 257 of the Protection of the Environment Operations Act 1997 provides that “the occupier of premises at or from which any pollution occurs is taken to have caused the pollution”.

- Warning: Irrespective of this condition any person occupying the site may be subject to proceedings under the Protection of the Environment Operations Act 1997 where pollution is caused, permitted or allowed as the result of the occupation of the land being developed whether or not they actually cause the pollution.

Condition Reason: To prevent potential water pollution and dust nuisance.

Disposal of Site Water During Construction

While site work is being carried out:

a)	Prior to pumping any water into the road or public stormwater system, approval must be obtained from Council under section 138(1)(d) of the Roads Act 1993.

b)	Water pollution, as defined by the Protection of the Environment Operations Act 1997, must not occur as the result of the discharge to the road, public stormwater system or other place of any site water.

c)	That stormwater from any roof or other impervious areas is linked, via temporary downpipes and stormwater pipes, to a Council approved stormwater disposal system immediately upon completion of the roof installation or work creating other impervious areas.

Condition Reason: To ensure that adjoining and neighbouring land is not adversely affected by unreasonable overland flows of stormwater and that site water does not cause erosion and water pollution.

Check Surveys - boundary location, building location, building height, stormwater drainage system and flood protection measures relative to Australian Height Datum

While site work is being carried out, a registered surveyor must carry out check surveys and provide survey certificates confirming the location of the building(s), ancillary works, flood protection works and the stormwater drainage system relative to the boundaries of the site and that the height of buildings, ancillary works, flood protection works and the stormwater drainage system relative to Australian Height Datum complies with this consent at the following critical stages.

Work must not proceed beyond each of the following critical stages until compliance has been demonstrated to the Principal Certifier’s satisfaction:

a)	Upon the completion of foundation walls prior to the laying of any floor or the pouring of any floor slab and generally at damp proof course level.

b)	Upon the completion of formwork for floor slabs prior to the laying of any floor or the pouring of any concrete and generally at each storey.

c)	Upon the completion of formwork or framework for the roof(s) prior to the laying of any roofing or the pouring of any concrete roof.

d)	Upon the completion of formwork and steel fixing prior to pouring of any concrete for any ancillary structure, flood protection work, swimming pool or spa pool or the like.

e)	Upon the completion of formwork and steel fixing prior to pouring of any concrete for driveways showing transitions and crest thresholds confirming that driveway levels match Council approved driveway crossing levels and minimum flood levels.

f)	Stormwater drainage Systems are in place prior to back filling over pipes confirming location, height and capacity of works.

g)	Flood protection measures are in place confirming location, height and capacity.

Condition Reason: To ensure that development occurs in the location and at the height approved under this consent, which is critical to ensure that buildings are constructed to minimum heights for flood protection and maximum heights to protect views and the amenity of neighbours.

Placement and Use of Skip Bins

While site work is being carried out, all waste storage containers, including but not limited to skip bins, must be stored within the site unless:

a)	Activity Approval has been issued by Council under section 68 of the Local Government Act 1993 to place the waste storage container in a public place; and

b)	where located on the road it is located only in a positions where a vehicle may lawfully park in accordance with the Australian Road Rules.

Notes:

- Waste storage containers must not be located on the footpath without a site specific activity approval.  Where such site specific activity approval is granted a 1.5m wide clear path of travel is maintained free of any trip hazards.

Condition Reason: To ensure waste storage containers are appropriately located.

Prohibition of Burning

While site work is being carried out, there must be no burning of any waste or other materials. The burning of copper chrome arsenate (CCA) or pentachlorophenol (PCP) treated timber is prohibited in all parts of NSW.  All burning is prohibited in the Woollahra local government area.

Notes:

- Under the Protection of the Environment Operations (Clean Air) Regulation 2021 all burning (including burning of vegetation and domestic waste) is prohibited except with approval.  No approval is granted under this consent for any burning.

Condition Reason: To ensure no burning of waste occurs.

Dust Mitigation

While site work is being carried out, dust mitigation must be implemented in accordance with “Dust Control - Do it right on site” and the accompanying facts sheets published by the Southern Sydney Regional Organisation of Councils.

This generally requires:

a)	Dust screens to all hoardings and site fences.

b)	All stockpiles or loose materials to be covered when not being used.

c)	All equipment, where capable, being fitted with dust catchers.

d)	All loose materials being placed bags before placing into waste or skip bins.

e)	All waste and skip bins being kept covered when not being filled or emptied.

f)	The surface of excavation work being kept wet to minimise dust.

g)	Landscaping incorporating trees, dense shrubs and grass being implemented as soon as practically possible to minimise dust.

Notes:

- “Dust Control - Do it right on site” and the accompanying factsheets can be downloaded from Council’s website www.woollahra.nsw.gov.au

- Special precautions must be taken when removing asbestos or lead materials from development sites.  Additional information can be obtained from www.safework.nsw.gov.au and www.epa.nsw.gov.au.  Other specific conditions and advice may apply.

- Demolition and construction activities may affect local air quality and contribute to urban air pollution. The causes are dust, smoke and fumes coming from equipment or activities, and airborne chemicals when spraying for pest management. Precautions must be taken to prevent air pollution.

Condition Reason: To mitigate the impact of dust upon the amenity of the neighbourhood and prevent water pollution.

Site Waste Minimisation and Management – Demolition

While site work is being carried out, in order to maximise resource recovery and minimise residual waste from demolition activities:

a)	the provisions of the Site Waste Minimisation and Management Plan (SWMMP) are to be implemented at all times during the course of the work,

b)	an area is to be allocated for the storage of materials for use, recycling and disposal (giving consideration to slope, drainage, location of waterways, stormwater outlets, vegetation and access and handling requirements),

c)	separate collection bins and/or areas for the storage of residual waste are to be provided,

d)	the purpose and content of the bins and/or storage areas are to be clearly ‘signposted’,

e)	measures to prevent damage by the elements, odour, health risks and windborne litter are to be implemented, and

f)	site disturbance must be minimised, and unnecessary excavation limited.

When implementing the SWMMP the Applicant must ensure:

a)	footpaths, public reserves and street gutters are not used as places to store demolition waste or materials of any kind without Council approval,

b)	any material moved offsite is transported in accordance with the requirements of the Protection of the Environment Operations Act 1997,

c)	waste is only transported to a place that can lawfully be used as a waste facility,

d)	generation, storage, treatment and disposal of hazardous waste and special waste (including asbestos) is conducted in accordance with relevant waste legislation administered by the NSW Environment Protection Authority, and relevant occupational health and safety legislation administered by SafeWork NSW, and

e)	evidence such as weighbridge dockets and invoices for waste disposal or recycling services are retained.

Notes:

- Materials that have an existing reuse or recycling market must not be disposed of in a land fill. Reuse and recycling opportunities are decreased when asbestos is not carefully removed and segregated from other waste streams.

Condition Reason: To maximise resource recovery and minimise residual waste from demolition activities.

Site Waste Minimisation and Management – Construction

While site work is being carried out, in order to maximise resource recovery and minimise residual waste from construction activities:

a)	the provisions of the Site Waste Minimisation and Management Plan (SWMMP) are to be implemented at all times during the course of the work,

b)	deliveries of materials must be arranged so that materials are delivered ‘as needed’ to prevent the degradation of materials through weathering and moisture damage,

c)	consideration must be given to returning excess materials to the supplier or manufacturer,

d)	an area must be allocated for the storage of materials for use, recycling and disposal (considering slope, drainage, location of waterways, stormwater outlets and vegetation),

e)	the purpose and content of the storage areas must be clearly ‘signposted’,

f)	contractors must be arranged for the transport, processing and disposal of waste and recycling and all contractors must be aware of the legal requirements for disposing of waste,

g)	separate collection bins or areas for the storage of residual waste must be promoted,

h)	measures to prevent damage by the elements, odour and health risks, and windborne litter must be implemented,

i)	site disturbance must be minimised and unnecessary excavation limited,

j)	all waste must be transported to a place that can lawfully be used as a waste facility, and

k)	records demonstrating lawful disposal of waste must be retained and kept readily accessible for inspection by regulatory authorities such as Council, the NSW EPA or SafeWork NSW.

Condition Reason: To maximise resource recovery and minimise residual waste from construction activities.

Asbestos Removal

While site work is being carried out, all asbestos removal work must be carried out safely according to NSW work health and safety legislation.

Where hazardous material, including bonded or friable asbestos has been identified in accordance with the conditions in Section B above, and such material must be demolished, disturbed and subsequently removed, all such works must comply with the following criteria:

a)	Be undertaken by contractors who hold a current SafeWork NSW “demolition licence” and a current SafeWork NSW “Class A licence” for friable asbestos removal.

b)	Be carried out in accordance with the relevant SafeWork NSW codes of practice.

c)	No asbestos products may be reused on the site.

d)	No asbestos laden skip or bins must be left in any public place.

Notes:

- Before starting work, a work site-specific permit approving each asbestos project must be obtained from SafeWork NSW. A permit will not be granted without a current SafeWork licence.

- All removal, repair or disturbance of or to asbestos material must comply with:

-	Work Health and Safety Act 2011,

-	Work Health and Safety Regulation 2017,

-	SafeWork NSW “Code of Practice: How to Safely Remove Asbestos” (2016), and

-	SafeWork NSW “Code of Practice: How to Manage and Control Asbestos in the Workplace” (2016).

- For more information go to the SafeWork NSW website on asbestos www.safework.nsw.gov.au/health-and-safety/safety-topics-a-z/asbestos, and www.safework.nsw.gov.au/law-and-policy/legislation-and-codes/codes-of-practice or call 131 050

Condition Reason: To ensure the safe removal of asbestos and protect the health and safety of persons working on the site and the public

Classification of Hazardous Waste

While site work is being carried out, and prior to the exportation of hazardous waste (including hazardous fill or soil) from the site, the waste materials must be classified in accordance with the provision of the Protection of the Environment Operations Act 1997 and the NSW EPA Waste Classification Guidelines, Part1: Classifying Waste, 2014.

Condition Reason: To ensure that where hazardous waste will be removed from a site an asbestos licensed contractor can definitively determine where the waste may be legally taken for disposal.

Disposal of Asbestos and Hazardous Waste

While site work is being carried out, asbestos and hazardous waste, once classified in accordance with the hazardous waste classification condition must only be transported to waste facilities licensed to accept asbestos and appropriate classifications of hazardous waste.

Condition Reason: To ensure that asbestos and other hazardous waste is disposed of lawfully under the Protection of the Environment Operations Act 1997 and relevant NSW EPA requirements.

Asbestos Removal Signage

While site work is being carried out and when asbestos is being removed, standard commercially manufactured signs containing the words "DANGER ASBESTOS REMOVAL IN PROGRESS" measuring not less than 400mm x 300mm are to be erected in prominent visible positions on the site.

Condition Reason: To ensure awareness of any hazard to the health and safety of persons working on the site and public.

Notification of Asbestos Removal

While site work is being carried out, in addition to the requirements for licensed asbestos removalists to give written notice to SafeWork NSW, all adjoining properties and those opposite the development site must be notified in writing of the dates and times when asbestos removal is to be conducted.

The notification is to identify the licensed asbestos removal contractor and include a contact person for the site together with telephone and facsimile numbers and email addresses.

Condition Reason: To ensure that local residents are informed and have adequate contact details for incidents of asbestos removal.

Compliance with Geotechnical / Hydrogeological Monitoring Program

While site work is being carried out, excavation must be undertaken in accordance with the recommendations of the Geotechnical / Hydrogeological Monitoring Program and any oral or written direction of the supervising professional engineer.

The Principal Contractor and any sub-contractor must strictly follow the Geotechnical / Hydrogeological Monitoring Program for the development including, but not limited to:

a)	the location and type of monitoring systems to be utilised,

b)	recommended hold points to allow for inspection and certification of geotechnical and

c)	hydrogeological measures by the professional engineer, and

d)	the contingency plan.

Notes:

- The consent authority cannot require that the author of the geotechnical/hydrogeological report submitted with the development application to be appointed as the professional engineer supervising the work however, it is the Council’s recommendation that the author of the report be retained during the construction stage.

Condition Reason: To ensure the geotechnical and/or hydrogeological impacts of the development are appropriately managed.

Vibration Monitoring

While site work is being carried out, vibration monitoring equipment must be installed and maintained, under the supervision of a professional engineer with expertise and experience in geotechnical engineering, between any potential source of vibration and any building identified by the professional engineer as being potentially at risk of movement or damage from settlement and/or vibration during the excavation and during the removal of any excavated material from the land being developed.

If vibration monitoring equipment detects any vibration at the level of the footings of any adjacent building exceeding the peak particle velocity adopted by the professional engineer as the maximum acceptable peak particle velocity an audible alarm must activate such that the Principal Contractor and any sub-contractor are easily alerted to the event.

Where any such alarm triggers all excavation works must cease immediately. Prior to the vibration monitoring equipment being reset by the professional engineer and any further work recommencing the event must be recorded and the cause of the event identified and documented by the professional engineer.

Where the event requires, in the opinion of the professional engineer, any change in work practices to ensure that vibration at the level of the footings of any adjacent building does not exceed the peak particle velocity adopted by the professional engineer as the maximum acceptable peak particle velocity these changes in work practices must be documented and a written direction given by the professional engineer to the Principal Contractor and any sub-contractor clearly setting out required work practice.

The Principal Contractor and any sub-contractor must comply with all work directions, verbal or written, given by the professional engineer.

A copy of any written direction required by this condition must be provided to the Principal Certifier within 24 hours of any event.

Where there is any movement in foundations such that damaged is occasioned to any adjoining building or such that there is any removal of support to supported land the professional engineer, Principal Contractor and any sub-contractor responsible for such work must immediately cease all work, inform the owner of that supported land and take immediate action under the direction of the professional engineer to prevent any further damage and restore support to the supported land.

Notes:

- Professional engineer has the same mean as in Schedule 1 of the BCA.

- Building has the same meaning as in section 1.4 of the Act i.e. “building includes part of a building and any structure or part of a structure….”

- Supported land has the same meaning as in the Conveyancing Act 1919.

Condition Reason: To monitor and manage vibration impacts from development.

Site Cranes

While site work is being carried out, site crane(s) and hoist(s) may be erected within the boundary of the land being developed subject to compliance with Australian Standards AS 1418, AS 2549 and AS 2550 and all relevant parts to these standards.

Cranes must not swing or hoist over any public place unless the relevant approvals have been obtained under the Local Government Act 1993, Crown Lands Act 1989 or Roads Act 1993.

The crane must not be illuminated outside approved working hours other than in relation to safety beacons required by the Civil Aviation Safety Authority under the Civil Aviation Act 1988 (Cth).

No illuminated sign(s) must be erected upon or displayed upon any site crane.

Notes:

- Where it is proposed to swing a crane over a public place a separate application to Council must be made under section 68 of the Local Government Act 1993 and obtain activity approval from Council prior to swinging or hoisting over the public place.

- Where it is proposed to swing a crane over private land the consent of the owner of that private land is required.  Alternatively, an access order under the Access to Neighbouring Land Act 2000 or easement under section 88K of the Conveyancing Act 1919 or section 40 of the Land and Environment Court Act 1979 as appropriate must be obtained.  The encroachment of cranes or the like is a civil matter of trespass and encroachment.  Council does not adjudicate or regulate such trespasses or encroachments.

Condition Reason: To ensure site cranes are used safely with the relevant approvals.

Compliance with Geotechnical / Hydrogeological Monitoring Program

While site work is being carried out, excavation must be undertaken in accordance with the recommendations of the Geotechnical / Hydrogeological Monitoring Program and any oral or written direction of the supervising professional engineer.

The Principal Contractor and any sub-contractor must strictly follow the Geotechnical / Hydrogeological Monitoring Program for the development including, but not limited to:

a)	the location and type of monitoring systems to be utilised,

b)	recommended hold points to allow for inspection and certification of geotechnical and

c)	hydrogeological measures by the professional engineer, and

d)	the contingency plan.

Notes:

- The consent authority cannot require that the author of the geotechnical/hydrogeological report submitted with the development application to be appointed as the professional engineer supervising the work however, it is the Council’s recommendation that the author of the report be retained during the construction stage.

Condition Reason: To ensure the geotechnical and/or hydrogeological impacts of the development are appropriately managed.

Vibration Monitoring

While site work is being carried out, vibration monitoring equipment must be installed and maintained, under the supervision of a professional engineer with expertise and experience in geotechnical engineering, between any potential source of vibration and any building identified by the professional engineer as being potentially at risk of movement or damage from settlement and/or vibration during the excavation and during the removal of any excavated material from the land being developed.

If vibration monitoring equipment detects any vibration at the level of the footings of any adjacent building exceeding the peak particle velocity adopted by the professional engineer as the maximum acceptable peak particle velocity an audible alarm must activate such that the Principal Contractor and any sub-contractor are easily alerted to the event.

Where any such alarm triggers all excavation works must cease immediately. Prior to the vibration monitoring equipment being reset by the professional engineer and any further work recommencing the event must be recorded and the cause of the event identified and documented by the professional engineer.

Where the event requires, in the opinion of the professional engineer, any change in work practices to ensure that vibration at the level of the footings of any adjacent building does not exceed the peak particle velocity adopted by the professional engineer as the maximum acceptable peak particle velocity these changes in work practices must be documented and a written direction given by the professional engineer to the Principal Contractor and any sub-contractor clearly setting out required work practice.

The Principal Contractor and any sub-contractor must comply with all work directions, verbal or written, given by the professional engineer.

A copy of any written direction required by this condition must be provided to the Principal Certifier within 24 hours of any event.

Where there is any movement in foundations such that damaged is occasioned to any adjoining building or such that there is any removal of support to supported land the professional engineer, Principal Contractor and any sub-contractor responsible for such work must immediately cease all work, inform the owner of that supported land and take immediate action under the direction of the professional engineer to prevent any further damage and restore support to the supported land.

Notes:

- Professional engineer has the same mean as in Schedule 1 of the BCA.

- Building has the same meaning as in section 1.4 of the Act i.e. “building includes part of a building and any structure or part of a structure….”

- Supported land has the same meaning as in the Conveyancing Act 1919.

Condition Reason: To monitor and manage vibration impacts from development.

Site Cranes

While site work is being carried out, site crane(s) and hoist(s) may be erected within the boundary of the land being developed subject to compliance with Australian Standards AS 1418, AS 2549 and AS 2550 and all relevant parts to these standards.

Cranes must not swing or hoist over any public place unless the relevant approvals have been obtained under the Local Government Act 1993, Crown Lands Act 1989 or Roads Act 1993.

The crane must not be illuminated outside approved working hours other than in relation to safety beacons required by the Civil Aviation Safety Authority under the Civil Aviation Act 1988 (Cth).

No illuminated sign(s) must be erected upon or displayed upon any site crane.

Notes:

- Where it is proposed to swing a crane over a public place a separate application to Council must be made under section 68 of the Local Government Act 1993 and obtain activity approval from Council prior to swinging or hoisting over the public place.

- Where it is proposed to swing a crane over private land the consent of the owner of that private land is required.  Alternatively, an access order under the Access to Neighbouring Land Act 2000 or easement under section 88K of the Conveyancing Act 1919 or section 40 of the Land and Environment Court Act 1979 as appropriate must be obtained.  The encroachment of cranes or the like is a civil matter of trespass and encroachment.  Council does not adjudicate or regulate such trespasses or encroachments.

Condition Reason: To ensure site cranes are used safely with the relevant approvals.

Compliance with Council’s Specification for Roadworks, Drainage and Miscellaneous Works, Road Works and, Work within the Road and Footway

While site work is being carried out, all work carried out on assets which are under Council ownership or will revert to the ownership, care, control or management of Council in connection with the development to which this consent relates must comply with Council’s Specification for Roadworks, Drainage and Miscellaneous Works (2012).

The person with the benefit of this consent must meet all costs associated with such works.

This condition does not set aside the need to obtain relevant approvals under the Roads Act 1993 or Local Government Act 1993 for works within roads and other public places.

Notes:

- A copy of Council’s Specification for Roadworks, Drainage and Miscellaneous Works can be downloaded from Council’s website www.woollahra.nsw.gov.au

Condition Reason: To ensure that any road, drainage, or miscellaneous works comply with Council’s specifications.

Shoring and Adequacy of Adjoining Property

While site work is being carried out, the person having the benefit of the development consent must, at the person’s own expense:

- protect and support the adjoining premises from possible damage from the excavation.

For the purposes of section 4.17(11) of the Act, the following condition is prescribed in relation to a development consent for development that involves an excavation that extends below the level of the base of the footings of a building on adjoining land.

Notes:

- This condition does not apply if the person having the benefit of the development consent owns the adjoining land or the owner of the adjoining land has given consent in writing to that condition not applying.

Condition Reason: To protect and support the adjoining premises from possible damage from the excavation.

Compliance with Construction Traffic Management Plan

While site work is being carried out, all development activities and traffic movements must be carried out in accordance with the approved Construction Traffic Management Plan (CTMP). All controls in the CTMP must be maintained at all times. A copy of the CTMP must be kept on-site at all times and made available to the Principal Certifier on request.

Notes:

- Irrespective of the provisions of the Construction Traffic Management Plan the provisions of traffic and parking legislation prevails.

Condition Reason: To ensure compliance with the Construction Traffic Management Plan.

Tree Preservation

While site work is being carried out, all persons must comply with Chapter E.3 – Tree Management of Council’s Development Control Plan (DCP) 2015, other than where varied by this consent. The DCP applies to any tree with a height greater than 5 metres or a diameter spread of branches greater than 3 metres.

General Protection Requirements:

- The tree protection measures must be maintained during all development work unless otherwise specified within these conditions of consent.

- Excavation must cease where tree roots with a diameter exceeding 50mm are exposed. The principal contractor must procure an inspection of the exposed tree roots by an arborist with a minimum AQF Level 5 qualification. Excavation must only recommence with the implementation of the recommendations of the arborist.

- Where there is damage to any part of a tree the principal contractor must procure an inspection of the tree by a qualified arborist immediately. The principal contractor must immediately implement treatment as directed by the arborist. The arborist is to supply a detailed report to the appointed certifier.

Trees must be pruned in accordance with Australian Standard AS 4373 “Pruning of Amenity Trees” and WorkCover NSW Code of Practice Amenity Tree Industry.

Condition Reason: To protect trees during the carrying out of sitework.

Standard Condition F.8

Arborists Documentation and Compliance Checklist

While site work is being carried out, the project arborist must provide written certification that all tree protection measures and construction techniques relevant to this consent have been implemented. Documentation for each site visit must include:

- a record of the condition of trees to be retained prior to and throughout development,

- recommended actions to improve site conditions and rectification of non-compliance, and

- recommendations for future works which may impact the trees.

All compliance certification documents must be kept on site by the site Supervisor.

As a minimum the following intervals of site inspections must be made:

| Stage of arboricultural inspection and supervision | Compliance documentation and photos must include |
| --- | --- |
| While site work is carried out | - The project arborist must supervise all demolition and excavation works within the Tree Protection Zones of nominated trees listed in this consent. - The project arborist must supervise the installation of any proposed hoarding within the Tree Protection Zones of nominated trees listed in this consent ensuring branches are tied back or the hoarding built around branches that do not require pruning for the purpose of clearance for the proposed building. - The project arborist must ensure pier holes within the Tree Protection Zones or specified distances of nominated trees listed in this consent are positioned to avoid the severance of and damage to roots greater than 50mm diameter when constructing boundary fences. - Ensure the proposed awnings are set back a minimum of 1.5 metres from the trunks, 1st and 2nd order branches of Trees 5, 6, 7 and 8. - Project Arborist to approve relocation of tree protection for landscaping. All landscaping works within the TPZ of trees to be retained are to be undertaken in consultation with the project Arborist to minimise the impact to trees. - Regular inspections as indicated in the Tree Management Plan. |

Inspections and compliance documentation must be made by an arborist with AQF Level 5 qualifications.

Additional site visits must be made when required by site arborist and/or site foreman for ongoing monitoring/supervisory work.

Condition Reason: To ensure that all tree protection measures and construction techniques relevant to this consent have been implemented.

Standard Condition F.45

Replacement/Supplementary trees which must be planted

While site work is being carried out, any replacement or supplementary tree must be grown in accordance with Tree stock for landscape use (AS 2303). The following replacement tree/s must be planted in deep soil landscaped area <delete this if planted in a planter box on a structure> and maintained in a healthy and vigorous condition. If the replacement tree is found to be faulty, damaged, dying or dead before it attains a size whereby it becomes a prescribed tree in accordance with Chapter E.3 of Council’s Development Control Plan, it must be replaced with another of the same species, which complies with the criteria outlined below.

| Species/Type | Planting Location | Container Size/Size of Tree (at planting) | Minimum Dimensions at Maturity (metres) |
| --- | --- | --- | --- |
| 1 x Melaleuca quinquenervia (Broad-leaved Paperbark) | In accordance with Landscape Plan No.s 000/E, 100/E, 101/E, 102/G, 103/E, 501/C, 502/B, designed by Site Image Landscape Architects, dated C, E, G – 08/04/2025 | 100 litre | 10 x 8 |
| 2 x Glochidion ferdinandii (Cheese tree) |  | 100 litre | 8 x 8 |
| 4 x Tristaniopsis laurina (Water gum) |  | 100 litre | 8 x 6 |
| 3 x Magnolia grandiflora ‘Little gem’ (Magnolia variety) |  | 100 litre | 5 x 3 |
| Level 2 planting |  |  |  |
| Species/Type | Planting Location | Container Size/Size of Tree (at planting) | Minimum Dimensions at Maturity (metres) |
| 4 x Banksia integrifolia (Coastal Banksia) | In accordance with Landscape Plan No.s 000/E, 100/E, 101/E, 102/G, 103/E, 501/C, 502/B, designed by Site Image Landscape Architects, dated C, E, G – 08/04/2025 | 100 litre | 8 x 6 |
| 4 x Magnolia grandiflora ‘Little gem’ (Magnolia variety) |  | 100 litre | 5 x 3 |
| 1 x Tristaniopsis laurina (Water gum) |  | 100 litre | 8 x 6 |

The project arborist must document compliance with the above condition.

Condition Reason: To ensure the provision of appropriate replacement planting.

Standard Condition F.46

Street tree planting

While site work is being carried out, the street tree as indicated in the table below must be planted prior to the completion of works.

The fees outlined in the table below associated with the planting and 12 months maintenance must be paid by the applicant to Council in full prior to the issue of a construction certificate, subdivision certificate or occupation certificate, as applicable, in accordance with the Street Tree Planting and Maintenance scheduled fee. The Reference for the payment must include the development application number, eg. DA413/2024/1.

Council’s Arboricultural Technical Officer (9391-7980) must be contacted a minimum of 2 months prior to the completion of works to procure tree stock and schedule the planting works.

The cost as indicated in the table below includes:

- Species selection (by Council) in line with the Woollahra Council Street Tree Master Plan (2014).

- Tree stock procured by Council compliant with Australian Standard 2303: Tree stock for landscape use.

- Installation by Council or a Council-approved contractor in a location determined by Council.

- Twelve months maintenance by Council or a Council approved contractor.

| Species/Type | Planting Location | Container Size/Size of Tree (at planting) | Cost/Fees |
| --- | --- | --- | --- |
| 1 x Lophostemon confertus (Brush Box) | 3 metres from the closest northern edge of the proposed driveway for carpark levels. | 100 litre | $2,156.00 |

Condition Reason: To ensure the provision of appropriate street tree planting.

Standard Condition F.47

Level changes in the vicinity of trees

While site work is being carried out, no level changes must occur within the specified radius from the trunks of the following trees.

| Council Ref No | Species | Location | Radius from centre of trunk (metres) |
| --- | --- | --- | --- |
| 9 | Ficus benjamina (Weeping Fig) | Rear yard of 39-41 Wilberforce Avenue Rose Bay | 6 |

The project arborist must document compliance with the above condition.

Condition Reason: To ensure level changes would not adversely impact upon the health of existing trees.

Standard Condition F.49

Hand excavation within tree root zones

While site work is being carried out, demolition of underground structures such as existing footings and approved excavation undertaken within the specified radius from the trunks of the following trees must be carried out by hand.

| Council Ref No | Species | Location | Radius from centre of trunk (metres) |
| --- | --- | --- | --- |
| 9 | Ficus benjamina (Weeping Fig) | Rear yard of 39-41 Wilberforce Avenue Rose Bay | 6 |

Small hand tools such as mattocks or using compressed air or water jetting only must be used. Roots with a diameter equal to or in excess of 50mm must not be severed or damaged unless approved in writing and documented by the project arborist.

Mechanical excavation is permitted beyond this radius when root pruning by hand along the hand excavated perimeter line is completed. Exposed roots to be retained must be covered with mulch or a geotextile fabric and kept in a moist condition and prevented from drying out.

All root pruning must be undertaken in accordance with the Australian Standard 4373 “Pruning of Amenity Trees” and carried out by a qualified Arborist (minimum qualification of Australian Qualification Framework Level 5 or recognised equivalent).

The project arborist must document compliance with the above condition.

Condition Reason: To ensure demolition and excavation works would not adversely impact upon the health of existing trees.

Standard Condition F.50

Footings in the vicinity of trees

While site work is being carried out, footings for any structure within the specified radius from the trunks of the following trees must be supported using an isolated pier and beam system.

| Council Ref No | Species | Location | Radius from centre of trunk (metres) |
| --- | --- | --- | --- |
| 9 | Ficus benjamina (Weeping Fig) | Rear yard of 39-41 Wilberforce Avenue Rose Bay | 6 |

Excavations for the installation of piers must be located so that no tree root with a diameter equal to or in excess of 50mm is severed or damaged. The smallest possible area must be excavated which allows construction of the pier. In consultation with the project engineer the piers must be offset a minimum 100mm from any root equal to or in excess of 50mm to accommodate future growth. The beam is to be placed a minimum of 100mm above ground level and is to be designed to bridge all tree roots with a diameter equal to or in excess of 50mm.

The project arborist must document compliance with the above condition.

Condition Reason: To ensure the proposed footings would not adversely impact upon the health of existing trees.

Standard Condition F.51

Awnings in the vicinity of trees

While site work is being carried out, the proposed awnings must be constructed to maintain a minimum 1.5 metre clearance from the trunks, 1st order and 2nd order branches of the following trees.

| Council Ref No | Species | Location |
| --- | --- | --- |
| 5 | Lophostemon confertus (Brush Box) | Most north western specimen along the Albemarle Avenue frontage of the site |
| 6 | Lophostemon confertus (Brush Box) | Most south eastern specimen along the Albemarle Avenue frontage of the site |
| 7 | Gleditsia triacanthos cvs (Honey Locust) | Most south western specimen along the Old South Head Road frontage of the site |
| 8 | Gleditsia triacanthos cvs (Honey Locust) | Most north eastern specimen along the Old South Head Road frontage of the site |

Any roots equal to or greater than 50mm diameter uncovered for the installation of stormwater pipes and pits must not be severed and remain in situ bridging across the excavated trench. Pipes must be guided under any roots equal to or greater than 50mm bridging across excavated trenches. Stormwater pits must be positioned so that no roots equal to or greater then 50mm diameter are severed.

The project arborist must document compliance with the above condition.

Condition Reason: To ensure the installation of the stormwater works would not adversely impact upon the health of existing trees.

Standard Condition F.52

- BEFORE THE ISSUE OF AN OCCUPATION CERTIFICATE

Occupation Certificate (section 6.9 of the Act)

A person must not commence occupation or use of the whole or any part of a new building (within the meaning of section 6.10 of the Act) unless an occupation certificate has been issued in relation to the building or part.

Notes:

- New building includes an altered portion of, or an extension to, an existing building.

Condition Reason: To ensure the building is suitable to occupy.

Plan of Management

The Plan of Management must be updated to incorporate the requirements within the conditions of consent. The updated Plan of Management must be submitted to Council for review.

Provision of Category 1 Fire Safety Provisions (clause 62 of the Regulation)

Before the issue of any occupation certificate, the Category 1 fire safety provisions, as are applicable to the building’s proposed new use, must be provided.

Condition Reason: To ensure that fire safety measures are provided prior to occupation.

Removal of Ancillary Works and Structures

Before the issue of any occupation certificate for the whole of the building, the following articles must be removed from the land and any adjoining public place:

a)	the site sign,

b)	ablutions,

c)	hoarding,

d)	scaffolding, and

e)	waste materials, matter, article or thing.

Condition Reason: To ensure that all ancillary matter is removed prior to occupation.

Commissioning and Certification of Systems and Works

Before the issue of any occupation certificate, works-as-executed (WAE) plans prepared by a registered surveyor, compliance certificates, and evidence of suitability in accordance with Part A5G1 of the BCA confirming that the works, as executed and as detailed, comply with the requirement of this consent, the Act, the Regulations, any relevant construction certificate, the BCA and relevant Australian Standards must be submitted to the satisfaction of the Principal Certifier.

Works-as-executed plans, compliance certificates, and evidence of suitability in accordance with Part A5G1 of the BCA must include, but may not be limited to:

a)	Certification from the supervising professional engineer that the requirement of the Geotechnical/Hydrogeological conditions and report recommendations were implemented and satisfied during development work.

b)	All flood protection measures.

c)	All garage/car park/basement car park, driveways and access ramps comply with Australian Standard AS 2890.1: Off-Street car parking.

d)	All stormwater drainage and storage systems.

e)	All mechanical ventilation systems.

f)	All hydraulic systems.

g)	All structural work.

h)	All acoustic attenuation work.

i)	All waterproofing.

j)	Such further matters as the Principal Certifier may require.

Notes:

- The PC may require any number of WAE plans, certificates, or other evidence of suitability as necessary to confirm compliance with the Act, Regulation, development standards, BCA, and relevant Australia Standards.  As a minimum WAE plans and certification is required for stormwater drainage and detention, mechanical ventilation work, hydraulic services (including but not limited to fire services).

- The PC must submit to Council, with any occupation certificate, copies of WAE plans, compliance certificates and evidence of suitability in accordance with Part A5G1 of the BCA upon which the PC has relied in issuing any occupation certificate.

Condition Reason: To ensure that systems and works as completed meet development standards as defined by the Act, comply with the BCA, and this consent, and to ensure a public record of works as executed is maintained.

Commissioning and Certification of Public Infrastructure Works

Before the issue of any occupation certificate, certification from a professional engineer that all public infrastructure works have been executed in compliance with this consent and with Council’s Specification for Roadworks, Drainage and Miscellaneous Works (2012) must be submitted to the satisfaction of Council, and the Principal Certifier must be provided with correspondence from Council to this effect.

The certification must be supported by closed circuit television / video inspection provided on DVD of all stormwater drainage together with works-as-executed engineering plans and a survey report detailing all finished reduced levels.

Condition Reason: To ensure that any road, drainage, or miscellaneous works have been completed in accordance with Council’s specifications to the satisfaction of Council.

Works within Public Land (including Council, State or Federal owned land or property)

Before the issue of any occupation certificate, the following works within public land, whether new/existing/renewed must be completed to the satisfaction of Council, in compliance with Council’s Specification for Roadworks, Drainage and Miscellaneous Works (2012) unless expressly provided otherwise by these conditions at the person with the benefit of this consents expense:

a)	stormwater pipes, pits, structures and connections to public stormwater systems within the road,

b)	driveways and vehicular crossings,

c)	renew/new retaining structures,

d)	overhang structures,

e)	encroachments or occupation or alienation of public land or property,

f)	removal of redundant driveways and any other structure,

g)	new footpaths, pathways, walkways, or dunny lanes,

h)	relocation of existing power/light pole, if applicable,

i)	relocation/provision of street signs, if applicable,

j)	new or replacement street trees, if applicable,

k)	verge landscape items, where a grass verge exists, the balance of the area between the footpath and the kerb or site boundary over the full frontage of the proposed development must be turfed.  The grass verge must be constructed to contain a uniform minimum 75mm of friable growing medium and have a total cover of turf predominant within the street,

l)	new or reinstated kerb and guttering within the road, and

m)	new or reinstated road surface pavement within the road.

Notes:

- When determining whether the works within public land are satisfactory, Council will consider the ownership, construction quality, maintenance, operations, and public utility of such item/s.

- Security held by Council under section 4.17(6) of the Act will not be released until compliance has been achieved with this condition.  An application for the refund of security must be submitted with the occupation certificate to Council.  This form can be downloaded from Council’s website www.woollahra.nsw.gov.au or obtained from Council’s customer service centre.

Condition Reason: To ensure road, drainage and miscellaneous works are completed to the satisfaction of Council prior to occupation.

Positive Covenant and Works-As-Executed Certification of Stormwater Systems

Prior to issue of any Occupation Certificate, stormwater drainage works are to be certified by a professional engineer with works-as-executed drawings prepared by a registered surveyor and submitted, for approval by the Principal Certifying Authority, certifying:

- compliance with conditions of development consent relating to stormwater,

- the structural adequacy of the on-site stormwater filter vault,

- that only one stormwater outlet pipe has been constructed in accordance with the approved stormwater plans,

- that a stormwater treatment system has been constructed in accordance with the approved plans and meets the water quality targets stipulated in the Council’s DCP,

- that all below ground structures are fully tanked such that subsoil drainage/ seepage water is NOT collected and discharged into the kerb and gutter in accordance with the approved stormwater drawings,

- pipe invert levels and surface levels to Australian Height Datum, and

- contours indicating the direction in which water will flow over land should the capacity of the pit be exceeded in a storm event exceeding design limits.

A positive covenant pursuant to section 88E of the Conveyancing Act 1919 must be created on the title of the subject property, providing for the indemnification of Council from any claims or actions and for the on-going maintenance of the on-site detention system, including any pumps and sumps incorporated in the development.  The wording of the Instrument must be in accordance with Council’s standard format and the Instrument must be registered with the NSW Land Registry Services.

Note: 	The required wording of the Instrument can be downloaded from Council’s website www.woollahra.nsw.gov.au.  The PC must supply a copy of the WAE plans to Council together with the Occupation Certificate.

Note:	Occupation Certificate must not be issued until this condition has been satisfied.

Positive Covenant for Mechanical Parking Installation & Work-As-Executed Certification of Mechanical Systems

Before the issue of any occupation certificate for the whole of the building, mechanical parking installations are to be certified by a professional engineer with works-as-executed drawings supplied to the Principal Certifier detailing:

a)	Compliance with conditions of development consent relating to mechanical parking installation including turntable for the loading dock.

b)	That the works have been constructed in accordance with the approved design.

c)	A positive covenant pursuant to Section 88E of the Conveyancing Act 1919 must be created on the title of the subject property, providing for the indemnification of Council from any claims or actions and for the on-going maintenance of the turntable incorporated in the development. The wording of the Instrument must be in accordance with Council’s standard format and the Instrument must be registered at the Land and Property Information NSW.

Notes:

- The PCA must supply a copy of the WAE Plans to Council together with the occupation certificate for the whole of the building.

- The occupation certificate for the whole of the building must not be issued until this condition has been satisfied.

Amenity Landscaping

Before the issue of any occupation certificate, all approved amenity landscaping (screen planting, soil stabilisation planting, etc.) and replacement/supplementary tree planting must be installed in accordance with the approved plans and documents and any relevant conditions of consent.

Condition Reason: To ensure that the environmental impacts of the development are mitigated by approved landscaping prior to the occupation of the development.

Standard Condition G.6

Landscaping

Before the issue of any occupation certificate, the Principal Certifier and Council must be provided with a works-as-executed landscape plan and certification from a qualified landscape architect/designer, horticulturist and/or arborist as applicable to the effect that the landscaping and replacement/supplementary tree planting works comply with this consent.

Condition Reason: To ensure that all landscaping work is completed prior to occupation.

Arborists Documentation and Compliance Checklist – Prior to any occupation certificate

Before the issue of any occupation certificate, the project arborist must provide written certification that all tree protection measures and construction techniques relevant to this consent have been implemented. Documentation for each site visit must include:

- A record of the condition of trees to be retained prior to and throughout development.

- Recommended actions to improve site conditions and rectification of non-compliance.

- Recommendations for future works which may impact the trees.

All compliance certification documents must be kept on site by the site Supervisor.

As a minimum the following intervals of site inspections must be made:

| Stage of arboricultural inspection and supervision | Compliance documentation and photos must include |
| --- | --- |
| Prior to the issue of any occupation certificate | - Ensure all trees conditioned to be planted as part of this consent have been planted in accordance with the details prescribed in this consent. |

Inspections and compliance documentation must be made by an arborist with AQF Level 5 qualifications.

Additional site visits must be made when required by site arborist and/or site foreman for ongoing monitoring/supervisory work.

Condition Reason: To ensure that all tree protection measures and construction techniques relevant to this consent are implemented.

Standard Condition G.34

Arborists Documentation and Compliance Checklist – Prior to an occupation certificate for the whole building

Before the issue of any occupation certificate for the whole of the building, the project arborist must provide written certification that all tree protection measures and construction techniques relevant to this consent have been implemented. Documentation for each site visit must include:

- A record of the condition of trees to be retained prior to and throughout development.

- Recommended actions to improve site conditions and rectification of non-compliance.

- Recommendations for future works which may impact the trees.

All compliance certification documents must be kept on site.

As a minimum the following intervals of site inspections must be made:

| Stage of arboricultural inspection and supervision | Compliance documentation and photos must include |
| --- | --- |
| Before the issue of any occupation certificate for the whole of the building | - The project arborist must supervise the dismantling of tree protection measures - After all demolition, construction and landscaping works are complete the project Arborist must assess that the subject trees have been retained in the same condition and vigour. If changes to condition are identified the project Arborist must provide recommendations for remediation. |

Inspections and compliance documentation must be made by an arborist with AQF Level 5 qualifications.

Additional site visits must be made when required by site arborist and/or site foreman for ongoing monitoring/supervisory work.

Condition Reason: To ensure that all tree protection measures and construction techniques relevant to this consent are implemented.

- OCCUPATION AND ONGOING USE

Trading Hours

In order to minimise impacts on the amenity of adjoining residential properties, the hours of operation are limited to the following:

Supermarket

- Monday to Friday: 7am-10pm

- Saturday, Sunday and Public Holiday: 7am-10pm

Loading dock operating hours

To ensure an adequate level of acoustic privacy is maintained to the neighbouring properties, the use of the loading dock area by trucks, refrigerated vans and other vehicles for the purpose of goods deliveries and the movement of such goods shall be strictly restricted to the following hours:

- Monday to Friday: 7am-10pm

- Saturday, Sunday and Public Holiday: 7am-10pm

Communal open space/separation zone

Signage should be provided at the gate to the area of communal open space (adjacent to the north-western boundary of the site) to indicate the following:

- The area should be accessible between 7am – 7pm only.

- The area should be designated as non-smoking.

- The area should not be used for large gatherings.

Lighting

To protect the amenity of the neighbouring properties and limit the obtrusive effects of signage illumination on public places, all external lighting and the illumination of any signage between 10pm and 7am is prohibited.

Outdoor Lighting – Residential

During the occupation and ongoing use, outdoor lighting must comply with AS/NZS 4282: Control of the obtrusive effects of outdoor lighting.  The maximum luminous intensity from each luminaire and threshold limits must not exceed the level 1 control relevant under tables in AS/NZS 4282.

Condition Reason: To protect the amenity of neighbours and limit the obtrusive effects of outdoor lighting.

Outdoor Lighting – Commercial

During the occupation and ongoing use, outdoor lighting must comply with AS/NZS 4282: Control of the obtrusive effects of outdoor lighting.  The maximum luminous intensity from each luminaire and threshold limits must not exceed the level 1 control relevant under tables in AS/NZS 4282.

Notes:

- Council may consider, subject to an appropriate Section 4.55 Application, relaxation of this condition where it can be demonstrated, by expert report, that the level of lighting in the existing area already exceeds the above criteria, where physical shielding is present or physical shielding is reasonably possible.

Condition Reason: To protect the amenity of neighbours and limit the obtrusive effects of outdoor lighting in public places.

Outdoor Lighting – Roof Terraces

During the occupation and ongoing use, outdoor lighting must comply with AS/NZS 4282: Control of the obtrusive effects of outdoor lighting.  The maximum luminous intensity from each luminaire and threshold limits must not exceed the level 1 control relevant under tables in AS/NZS 4282.

All lighting to be installed on the roof terrace will be recessed lights or will be surface wall/balustrade mounted lights at a maximum height of 600mm above the finished floor level of the roof terrace.

Notes:

- Council may consider, subject to an appropriate Section 4.55 Application, relaxation of this condition where it can be demonstrated, by expert report, that the level of lighting in the existing area already exceeds the above criteria, where physical shielding is present or physical shielding is reasonably possible.

Condition Reason: To protect the amenity of neighbours and limit the obtrusive effects of outdoor lighting.

Waste collection hours and requirements

To ensure an adequate level of acoustic privacy is maintained to the neighbouring properties, 	the collection of waste materials, recyclables and the collection of fat oil and grease trap 	services shall only occur during the following hours:

  - 7am - 3pm from Monday to Saturday.

Delivery vehicles

To ensure an adequate level of acoustic privacy is maintained to the neighbouring properties, no trucks, refrigerated vans or any other vehicles shall be parked within the vicinity of the loading dock area awaiting delivery or accepting of goods prior to or after the approved loading dock operating hours. Similarly, no trucks, refrigerated vans or any other vehicles shall leave engines idle when delivering or accepting goods within the loading dock area.

Noise management

The noise management recommendations within the Acoustic Report must be complied with during the ongoing use of the development.

Noise Control

During the occupation and ongoing use, the use of the premises must not give rise to the transmission of offensive noise to any place of different occupancy.  Offensive noise is defined in the Protection of the Environment Operations Act 1997.

Notes:

- Council will generally enforce this condition in accordance with the Noise Guide for Local Government (www.epa.nsw.gov.au/your-environment/noise/regulating-noise/noise-guide-local-government) and the NSW Industrial Noise Policy (www.epa.nsw.gov.au/your-environment/noise/industrial-noise) published by the NSW Environment Protection Authority. Other State Government authorities also regulate the Protection of the Environment Operations Act 1997.

- Useful links:

-	Community Justice Centres—free mediation service provided by the NSW Government www.cjc.nsw.gov.au.

-	NSW Environment Protection Authority— see “noise” section www.environment.nsw.gov.au/noise.

-	NSW Government legislation- access to all NSW legislation, including the Protection of the Environment Operations Act 1997 and the Protection of the Environment Noise Control Regulation 2017 is available at www.legislation.nsw.gov.au.

-	Australian Acoustical Society—professional society of noise related professionals www.acoustics.asn.au.

-	Association of Australian Acoustical Consultants—professional society of noise related professionals www.aaac.org.au.

-	Liquor and Gaming NSW—www.liquorandgaming.nsw.gov.au.

Condition Reason: To protect the amenity of the neighbourhood.

Compliance with the Trolley Management Plan

The owner and/or occupier must comply with the Operational Management Plan which requires a trolley locking system that prevents trolleys from leaving the site.

Maintenance of BASIX Commitments

During the occupation and ongoing use, all BASIX commitments must be maintained in accordance with the BASIX Certificate No. 1766799M.

This condition affects successors in title with the intent that environmental sustainability measures must be maintained for the life of development under this consent.

Condition Reason: To ensure the approved environmental sustainability measures are maintained for the life of development.

Maintenance of Landscaping

During the occupation and ongoing use, all landscaping must be maintained in general accordance with this consent.

This condition does not prohibit the planting of additional trees or shrubs subject that they are native species endemic to the immediate locality.

Notes:

- This condition also acknowledges that development consent is not required to plant vegetation and that over time additional vegetation may be planted to replace vegetation or enhance the amenity of the locality.

- Owners must have regard to the amenity impact of trees upon the site and neighbouring land.

Condition Reason: To ensure that the landscaping design intent is not eroded over time by the removal of landscaping or inappropriate exotic planting.

Noise from Mechanical Plant and Equipment

During the occupation and ongoing use, the noise level measured at any boundary of the site at any time while the mechanical plant and equipment is operating must not exceed the background noise level.  Where noise sensitive receivers are located within the site, the noise level is measured from the nearest strata, stratum or community title land and must not exceed background noise level at any time.

The background noise level is the underlying level present in the ambient noise, excluding the subject noise source, when extraneous noise is removed. For assessment purposes the background noise level is the LA90, 15 minute level measured by a sound level meter.

Notes:

- Words in this condition have the same meaning as in the Noise Policy for Industry (2017) www.epa.nsw.gov.au/your-environment/noise/industrial-noise/noise-policy-for-industry-(2017) and Noise Guide for Local Government (2013) www.epa.nsw.gov.au/your-environment/noise/regulating-noise/noise-guide-local-government

Condition Reason: To protect the amenity of the neighbourhood.

Ongoing Maintenance of the On-site Stormwater Treatment and Pumpout Systems

The owner(s) must in accordance with this condition and any positive covenant:

- permit stormwater to be detained and treated by the Systems;

- keep the systems clean and free of silt rubbish and debris;

- maintain renew and repair as reasonably required from time to time the whole or part of the treatment system so that it functions in a safe and efficient manner;

- carry out the matters referred to in paragraphs (b) and (c) at the Owners expense;

- not make any alterations to the system or elements thereof without prior consent in writing of the Council and not interfere with the system or by its act or omission cause it to be interfered with so that it does not function or operate properly;

- Not take any act, matter or thing which would prevent the overland flow path and flood protection measures (e.g. mechanical flood barriers) from operating in a safe and efficient manner;

- Not allow any structure to encroach upon the overland flow path;

- Not make any alterations to the flood protection measures and overland flow path or elements thereof without prior consent in writing of the Council and not interfere with the flood protection measures and overland flow path or by its act or omission cause it to be interfered with so that it does not function or operate properly;

- permit the Council or its authorised agents from time to time upon giving reasonable notice (but at any time and without notice in the case of an emergency) to enter and inspect the land with regard to compliance with the requirements of this covenant;

- comply with the terms of any written notice issued by Council in respect to the requirements of this clause within the time stated in the notice; and

- where the Owner fails to comply with the Owner’s obligations under this covenant, permit the Council or its agents at all times and on reasonable notice at the Owner’s cost to enter the land with equipment, machinery or otherwise to carry out the works required by those obligations.

The Owner

- indemnifies the Council from and against all claims, demands, suits, proceedings or actions in respect of any injury, damage, loss, cost, or liability (Claims) that may be sustained, suffered, or made against the Council arising in connection with the performance of the Owner’s obligations under this covenant except if, and to the extent that, the Claim arises because of the Council's negligence or default; and

- releases the Council from any Claim it may have against the Council arising in connection with the performance of the Owner’s obligations under this covenant except if, and to the extent that, the Claim arises because of the Council's negligence or default.

Note:	This condition has been imposed to ensure that owners are aware of require maintenance requirements for their stormwater systems.

Note: 	This condition is supplementary to the owner(s) obligations and Council’s rights under any positive covenant.

Provision of Off-street Public and Visitor Parking

During the occupation and ongoing use, in compliance with AS2890.3: Parking Facilities - Bicycle Parking Facilities, AS 2890.6: Parking facilities - Off-street parking for people with disabilities and AS/NZS 2890.1: Parking Facilities - Off-Street Car Parking and AS 2890.2: Off-Street Parking: Commercial Vehicle Facilities, unimpeded public access to off-street parking must be maintained as follows:

| Use | Number of spaces |
| --- | --- |
| Car parking (resident) | 19 (including 2 accessible) |
| Car parking (residential visitor) | 3 |
| Car parking (car wash) | 1 |
| Car parking (direct to boot) | 5 |
| Car parking (staff) | 10 |
| Car parking (car share) | 2 |
| Car parking (retail) | 53 (including 2 accessible) |
| Bicycle parking (residential) | 15 (within storage cage) |
| Bicycle parking (visitor) | 30 (bike racks) |
| Motorbike parking | 9 |

Notes:

Where there is a potential for the trespass of private motor vehicles upon private parking servicing the owner of the site may seek to enter into a free parking area agreement with Council.  Council may under such agreement enforce parking restrictions under section 650 of the Local Government Act 1993.

Further information can be obtained from Council’s Compliance Team by calling 9391 7000 or from the Office of Local Government at www.olg.nsw.gov.au or call 4428 4100.

Condition Reason: To ensure adequate on-site parking is maintained.

Provision of Off-street Commercial Vehicle Facilities

During the occupation and ongoing use, in compliance with AS 2890.2: Parking facilities - Off-street commercial vehicle facilities, unimpeded access to off-street parking must be maintained as follows:

| Use | Number of spaces |
| --- | --- |
| Loading Dock | 1 (including a turntable with a minimum clearance of 11m) |

All deliveries to and dispatch from the site, excluding retail sales, must occur within the curtilage of the development and within the commercial vehicle parking facilities required to be maintained by this condition.

Notes:

Where there is a potential for the trespass of private motor vehicles upon private parking servicing the owner of the site may seek to enter into a free parking area agreement with Council.  Council may under such agreement enforce parking restrictions under section 650 of the Local Government Act 1993.

Further information can be obtained from Council’s Compliance Team by calling 9391 7000 or from the Office of Local Government at www.olg.nsw.gov.au or call 4428 4100.

Condition Reason: To ensure that commercial vehicles use loading facilities within the site to reduce impacts on the neighbourhood including noise and parking impacts.

On-going Maintenance of the Mechanical Parking Installations

During the occupation and ongoing use, the Owner must ensure the ongoing maintenance of the mechanical parking installations in accordance with this condition and any positive covenant. The Owner must:

a)	keep the system clean and free of silt rubbish and debris;

b)	maintain renew and repair as reasonably required from time to time the whole of the system so that it functions in a safe and efficient manner;

c)	carry out the matters referred to in paragraphs (a) and (b) at the Owners expense;

d)	not make any alterations to the system or elements thereof without prior consent in writing of the Council and not interfere with the system or by its act or omission cause it to be interfered with so that it does not function or operate properly;

e)	permit the Council or its authorised agents from time to time upon giving reasonable notice (but at any time and without notice in the case of an emergency) to enter and inspect the land with regard to compliance with the requirements of this covenant;

f)	comply with the terms of any written notice issued by Council in respect to the requirements of this clause within the time stated in the notice; and

g)	where the Owner fails to comply with the Owner’s obligations under this covenant, permit the Council or its agents at all times and on reasonable notice at the Owner’s cost to enter the land with equipment, machinery or otherwise to carry out the works required by those obligations.

The Owner

a)	indemnifies the Council from and against all claims, demands, suits, proceedings or actions in respect of any injury, damage, loss, cost, or liability (Claims) that may be sustained, suffered, or made against the Council arising in connection with the performance of the Owner’s obligations under this covenant; and

b)	releases the Council from any Claim it may have against the Council arising in connection with the performance of the Owner’s obligations under this covenant.

Notes:

This condition is supplementary to the owner(s) obligations and Council’s rights under any positive covenant.

Condition Reason: To ensure the ongoing maintenance of the mechanical parking installations.

Operation in Accordance with Green Travel Plan (GTP) and Plan of Management (POM)

During the occupation and ongoing use:

a)	The operation and management of the premises shall be in accordance with the GTP/POM as approved under the deferred commencement condition.

b)	The GTP/POM cannot be altered without the written consent of Council.

c)	Monitoring annual reports must be submitted for a minimum of 5 years post occupation.

Condition Reason: To maximise road safety and performance.

Maintenance of Landscaping

During the occupation and ongoing use, all landscaping must be maintained in general accordance with this consent.

This condition does not prohibit the planting of additional trees or shrubs.

Notes:

- This condition also acknowledges that development consent is not required to plant vegetation and that over time additional vegetation may be planted to replace vegetation or enhance the amenity of the locality.

- Owners must have regard to the amenity impact of trees upon the site and neighbouring land.

Condition Reason: To ensure that the landscaping design intent is not eroded over time by the removal of landscaping or inappropriate exotic planting.

Standard Condition H.25

- BEFORE THE ISSUE OF A SUBDIVISION WORKS CERTIFICATE

Electricity Substations – Dedication as Road and/or Easements for Access

Before the issue of any subdivision certificate, if an electricity pillar and/or substation is required on the site the owner must dedicate to the appropriate energy authority (to its satisfaction), free of cost, an area of land adjoining the street alignment to enable an electricity pillar and/or substation to be established.  The size and location of the electricity substation is to be in accordance with the requirements of the appropriate energy authority and Council.  The opening of any access doors must not intrude onto the public road (footway or road pavement).

Documentary evidence of compliance, including correspondence from the network authority is to be provided to the Principal Certifier prior to issue of the construction certificate detailing energy authority requirements.

The Accredited Certifier must be satisfied that the requirements of the energy authority have been met prior to issue of the construction certificate.

Where an electricity substation is provided on the site adjoining the road boundary, the area within which the substation is located must be dedicated as public road.  Where access is required across the site to access an electricity pillar and/or substation an easement for access across the site from the public place must be created upon the linen plans burdening the subject site and benefiting the Crown in right of New South Wales and any statutory corporation requiring access to the electricity pillar and/or substation.

Condition Reason: To ensure that the energy authority is provided with access to electricity pillars and/or substations.