# GENERAL CONDITIONS

| Number | Condition |
| --- | --- |
|  | Approved Plans and supporting documentation |
|  | Approved plans and supporting documentation Development must be carried out in accordance with the following approved plans and documents, except where the conditions of this consent expressly require otherwise. Approved plans. Drawing number; Revision number; Plan title; Drawn by; Date of plan. Architectural Plans. 357_DA_112; 26; Site Plan; Collins and Turner; 15/10/2024. 357_DA_120; 30; Ground Floor Plan; Collins and Turner; 7/08/2024. 357_DA_121; 24; Roof Plan; Collins and Turner; 7/08/2025. 357_DA_122; 05; Demolition Floor Plan; Collins and Turner; 7/08/2025. 357_DA_201; 24; North & South Elevations; Collins and Turner; 7/08/2025. 357_DA_202; 23; East & West Elevations; Collins and Turner; 7/08/2025. 357_DA_310; 22; Sections; Collins and Turner; 16/07/2025. 357_DA_511; 22; Façade Details – SCR2; Collins and Turner; 16/07/2025. 357_DA_512; 22; Façade Details – SCR1; Collins and Turner; 16/07/2025. 357_DA_513; 22; Façade Details – SCR1; Collins and Turner; 16/07/2025. 357_DA_514; 22; Façade Details – SCR3; Collins and Turner; 16/07/2025. 357_DA_515; 10; Park Edge Detail Section; Collins and Turner; 16/07/2025. 357_DA_531; 22; Material Schedule; Collins and Turner; 16/07/2025. Civil Plans. CE100; G; Stormwater Plan; ADP; 14/11/2025. CE250; D; Catchment Plan and Results; ADP; 13/08/2025. CE300; E; Sediment & Erosion Control Plan; ADP; 10/10/2025. CE350; C; Sediment & Erosion Control Details; ADP; 08/07/2025. Landscape Plans. L-DA1-100; F; Tree Management Plan; Jane Irwin Landscape Architecture; 9/10/2025. L-DA1-400; F; Levels & Finishes 1; Jane Irwin Landscape Architecture; 9/10/2025. L-DA1-402; F; Trees + Planting; Jane Irwin Landscape Architecture; 9/10/2025. Approved Reports. Report number; Revision number; Title; Prepared by; Date. 24-0951; 7; Traffic Impact Assessment; PTC; 16/10/2025. SYD2966; G; Water Cycle and Flood Management Report; ADP; 14/11/2025. -; 2; Arboricultural Impact Assessment; Tree Survey; 8/07/2025. -; Griffith Park Community Centre – Operational Plan of Management; Canterbury Bankstown Council; 12/06/2025. JC22426A-r1; 1; Geotechnical, Contamination/Waste Classification and Acid Sulfate Soil Investifation; GepEnviro Consultancy Pty Ltd; 24/01/2023. 19758-Access-1; 1; Access Assessment; EBS Consultants; 27/05/2025. 237181; 00; Griffith Park Community Centre NCC (2022) Section-J Report; Steensen Varming; 15/05/2025. 237181; 01; Griffith Park Community Centre ESD DA Report; Steensen Varming; 26/05/2025. SYD3293; 01; Acoustic DA Report; ADP; 16/05/2025. 24-45; 3; Statement Of Environmental Effects; MG Planning; July 2025. In the event of any inconsistency with the approved plans and a condition of this consent, the condition prevails. |
|  | Condition reason: To ensure the development is built and remains consistent with approved plans and documentation. |
|  | Design Amendments |
|  | Before the issue of a construction certificate, the certifier must ensure the construction certificate plans and specifications detail the following required amendments to the approved plans and documents: Landscape Plan (Rev F) The following amendments are required relating to the Landscape Plan (Rev F): - Landscape construction details: Edge treatments (E1, K1), tree pits (typical and hardstand), and paving cross-sections (P1, P2, P5) must be documented to support constructability. - Planting maintenance guidance: A 12-month maintenance schedule covering the establishment period must be provided. - Tree location verification: All proposed tree locations must be checked against underground services with appropriate clearance distances, and root barriers must be specified where required. - Relocate landscaping to be directly north and south of the pedestrian crossing to ensure that the minimum No Stopping distances are adhering to TfNSW Standards TS05413 and TS05422, and any relevant Australian Standards. - Bioretention basin interface: The interface treatment and planting specification for the bioretention basin area shown on the landscape plans must be clarified. |
|  | Condition reason: To ensure that sight lines are appropriate. |
|  | Comply with Development Consent |
|  | The proposal shall comply with the conditions of this Development Consent. |
|  | Condition reason: To ensure the development is built and remains consistent with approved plans and documentation |
|  | Preliminary Site Investigation/Detailed Site Investigation |
|  | The preliminary site investigation prepared by GeoEnviro Consultancy Pty Ltd, titled ’ Geotechnical, Contamination/Waste Classification and Acid Sulfate Soil Investigation – Proposed Community Space Griffith Park Bankstown NSW’, dated 1/01/2023 reference JC22426A-r1and all the recommendations stated within the report forms part of the development consent. |
|  | Condition reason: To ensure compliance with the relevant New South Wales legislation. |
|  | Transport for New South Wales |
|  | The proposed development is to comply with the conditions imposed by Transport for NSW provided as an Appendix to this consent. |
|  | Condition reason: To ensure compliance with Transport for NSW requirements. |

# DEMOLITION WORK

|  | Hazardous Material Survey |
| --- | --- |
|  | Prior to demolition, the entire site (includes any structures) is to be assessed for hazardous materials, especially asbestos. A Hazardous Materials (HAZMAT) Report is to be prepared by an inspection body accredited by NATA. All recommendations in the above-mentioned report are to be implemented and a clearance certificate issued by an appropriately qualified occupational hygienist who has not previously been involved in the preparation of the Hazardous Materials (HAZMAT) Report, is to be submitted to Council prior to the issue of a construction certificate. |
|  | Condition reason: To ensure that the development complies with Council’s Environmental Health Policies. |
|  | Demolition |
|  | The demolition of the existing structures must be undertaken in accordance with all the following: - Written notice must be issued to adjoining residents a minimum of seven (7) days before the commencement of demolition advising of the commencement date, - Written notice must be issued to Council for inspection a minimum of (7) days before the commencement of demolition. Written notice must include the date when demolition work will commence and details of the name, address, contact telephone number and licence number of the demolition contractor, - Inspections being undertaken by Council including: - A pre-commencement demolition inspection when all site works required as part of this Determination Notice are installed on the site and before demolition work commencing, and - A final demolition inspection when the demolition work has been completed to ensure that the site is left in a satisfactory manner, in accordance with the conditions of this Determination Notice, - Payment of an inspection fee at Council’s current rate must be paid before inspection. Council requires 48 hours’ notice to carry out an inspection, - A sign must be erected at the front of the property showing the name of the licenced demolition contractor, the licence number and a telephone number on which that person can be contact outside of working hours, - All demolition work must be carried out in accordance with Australian Standard 2601 – ‘The Demolition of Structures’, - Where asbestos containing materials are to be removed, demolition must be carried out by contractors who have a current SafeWork NSW licence in asbestos removal, - An asbestos clearance certificate issued under the Work Health and Safety Act 2011 must be submitted to Council after the completion of demolition work, - Hazardous and intractable wastes arising from the demolition process must be removed and disposed of in accordance with the requirements of SafeWork NSW and the NSW Environment Protection Authority, - Demolition procedures must maximise the reuse and recycling of demolished materials to reduce the environmental impacts of waste disposal, - Adhere to the requirements stipulated in the approved Waste Management Plan, and - Demolition may only be carried out between 7.00 am and 5.00 pm on Monday to Saturday and no building work/s is to be carried out at any time on a Sunday or a public holiday. A copy of the final demolition inspection report must be submitted to the principal certifier before the commencement of any building work. 3.201 |
|  | Condition reason: To ensure compliance with the relevant Canterbury Bankstown policy |
|  | Tree Protection |
|  | ## All work carried out on or around protected trees during demolition, clearing and construction shall comply with Australian Standard AS4373-2007 Pruning of amenity trees and Australian Standard AS4970-2025 Protection of trees on development sites. Site specific conditions relating to tree protection shall take precedence over this requirement. |
|  | Condition reason: To ensure compliance with the relevant Australian Standard |

# BUILDING WORK

## BEFORE ISSUE OF A CONSTRUCTION CERTIFICATE

|  | Payment of Fees |
| --- | --- |
|  | Before Issuing a construction certificate, all the following must be paid to the relevant authority/entity and written evidence of these payments must be provided to the certifier. - Levies - Bonds - Contributions - Inspection fees 2.103.S |
|  | Condition reason: To ensure compliance with the relevant New South Wales legislation. |
|  | Long Service Levy |
|  | Before the issue of the relevant construction certificate, payment of the long service levy is required, under the Building and Construction industry Long Service Payments Act 1986, section 34. Evidence of the payment is to be provided to the Certifier. Payment can be calculated using the Levy Portal Calculator at https://longservice.force.com/bci/s/levy-calculator. Payment must be made via the Long Service Levy Portal at https://www.longservice.nsw.gov.au. 2.104.S |
|  | Condition reason: To ensure compliance with the relevant New South Wales legislation. |
|  | Submit Plans to Sydney Water |
|  | The Council approved plans, including demolition plans, must be submitted to Sydney Water for assessment. This will determine if the proposed structure(s) will affect any Sydney Water infrastructure or if there are additional requirements. Building plan approvals can be submitted online via Sydney Water Tap inTM. Refer to www.sydneywater.com.au/tapin for Sydney Water’s Guidelines for building over or next to assets, visit www.sydneywater.com.au ‘Plumbing, building & developing’ then ‘Building Plan Approvals’ or call 13000 TAPINTM. Sydney Water must issue either a Building Plan Assessment letter which states that the application is approved, or the appropriate plans must be stamped by a Water Servicing Coordinator. 2.202.S |
|  | Condition reason: To protect existing public and private infrastructure and building works during demolition, construction and ongoing use of the development. |
|  | Erosion and Sediment Control Plan |
|  | Before the issue of a construction certificate, the consent holder is to ensure that an erosion and sediment control plan is prepared in accordance with the following documents before it is provided to and approved by the certifier: - Council’s development control plan, - the guidelines set out in the NSW Department of Housing manual ‘Managing Urban Stormwater: Soils and Construction Certificate’ (the Blue Book), and - the ‘Do it Right On-Site, Soil and Water Management for the Construction Industry' (Southern Sydney Regional Organisation of Councils and the Natural Heritage Trust). The consent holder must ensure the erosion and sediment control plan is kept onsite at all times during site works and construction. 2.211.S |
|  | Condition reason: To protect existing public and private infrastructure and building works during demolition, construction and ongoing use of the development. |
|  | Retaining Walls |
|  | Where Council approved cut or fill exceeds 200mm and stable batter of 1 vertical to 3 horizontal maximum grade cannot be achieved, then a masonry or other proprietary material retaining wall, intended and suitable for that purpose, shall be constructed within the development site. Note, filling of the site needs specific approval from Council. - The retaining wall shall be located so that it will not impede or obstruct the natural flow of stormwater. Retaining walls exceeding 600mm in height shall be designed by an appropriately qualified person as defined in the Building and Development Certifiers Regulation 2020. Plans and details prepared and signed by an appropriately qualified person as defined in the Building and Development Certifiers Regulation 2020 are to be submitted to the certifier before the issue of the construction certificate. - All works associated with the construction of the wall, including backfilling and drainage, is to be located wholly within the allotment boundaries. |
|  | Condition reason: To ensure works undertaken are carried out in a safe manner in accordance with relevant policies. |
|  | Apply for Work Permit for Engineering Works |
|  | The consent holder shall apply for a Work Permit and obtain approval from Council, for any of the following applicable engineering works in front of the site, at their own expense: - Drainage connection to Council’s Kerb and Gutter drainage system. Minimum 0.3m clearance required between kerb outlet and VFC wing. - Removal of all driveway surfaces, reinstatement of laybacks to kerb and gutter and reshaping of the footway, all associated with redundant VFCs, - Repair of any damage to the public road including the footway occurring during building works, and - Reinstatement of the footway reserve and adjustment or relocation of existing public utility services to match the footway design levels as proposed on the approved Work Permit. Adjustment or relocation to any public utility services shall be carried out to the requirements of the public utility authority. Note: Council is required to prepare a site survey and design in order to determine the necessary information. The consent holder should make application and payment for the Work Permit at least twenty-one (21) days before the information being required and before the issue of the construction certificate. The Work Permit must be approved before any works commencing within the Council Road Reserve or on Council’s assets. 2.302 |
|  | Condition reason: To provide and maintain orderly and safe areas for vehicular and pedestrian movement / parking. |
|  | Works Requiring a Work Permit |
|  | As any works within, or use of, the footway or public road for construction purposes requires separate Council approval under Section 138 of the Roads Act 1993 and/or Section 68 of the Local Government Act 1993, Council requires the consent holder to obtain a Works Permit and or a Roadway / Footpath Building Occupation Permit before the issue of any construction certificate for this development being issued where one or more of the following will occur, within, on or over the public footway or public road: A PRIVATE CERTIFIER CANNOT ISSUE THESE PERMITS WORKS REQUIRING A 'WORKS PERMIT' - Dig up, disturb, or clear the surface of a public footway or public road, - Remove or interfere with a structure or tree (or any other vegetation) on a public footway or public road, - Connect a road (whether public or private) to a classified road, - Undertake footway, paving, vehicular crossing (driveway), landscaping or stormwater drainage works within a public footway or public road, - Install utilities in, under or over a public road, - Pump water into a public footway or public road from any land adjoining the public road, - Erect a structure or carry out a work in, on or over a public road, - Require a work zone on the public road for the unloading and or loading of vehicles, - Pump concrete from within a public road, - Stand a mobile crane within a public road, - Store waste and recycling containers, skips, bins and/or building materials on any part of the public road, - The work is greater than $25,000, and - Demolition is proposed. The consent holder shall construct all works approved within the public road and footway under the supervision and to the satisfaction of Council and shall arrange for necessary inspections by Council whilst the work is in progress. The consent holder shall ensure that the person or company carrying out the work will carry public liability insurance to a minimum value of twenty million dollars and shall provide proof of the policy to Council, before commencing any work approved by the Work Permit including the Road Opening Permit. The policy must remain valid for the duration of the works. 2.304 |
|  | Condition reason: To ensure the development is built and remains consistent with approved plans and documentation. |
|  | Finished surface levels |
|  | Finished surface levels of all internal works and at the street boundary, including driveways, landscaping and drainage structures, must be as shown on relevant construction certificate plans. The levels at the street boundary must be consistent with the Street Boundary Alignment Levels issued by Council. |
|  | Condition reason: To ensure the development is built and remains consistent with approved plans and documentation. |
|  | Compliance with Arborist Report |
|  | ## The consent holder shall ensure full compliance with all the recommendations numbered in Section 6 of the Arborist Report prepared by Tree Survey dated 8/7/25, which was submitted to Council as part of this application. |
|  | Condition reason: To ensure the development is carried out in accordance with the recommendations of the endorsed arborist report. |
|  | Easement |
|  | Sites located adjacent to Council’s drainage easement and affected by flooding shall comply with the following: a. Concrete pier and beam type footings shall be provided for all structures adjacent to Council’s stormwater pipe/easement in accordance with the requirements contained in Council's Development Engineering Standards. Plans and details prepared by a qualified practising structural engineer, with details suitable for construction in accordance with the above Policy shall be incorporated into the details for approval of the construction certificate. A copy of the approved details together with a report prepared by a registered surveyor showing location, size and depth of the stormwater system, shall be sent to Council for recordkeeping. b. The habitable floor level shall be constructed to RL 17.3 metres AHD as determined by the flood study including freeboard. All approved construction details shall be consistent with this requirement. c. The design of the building must ensure free flow of floodwater beneath the building slab, and the undercroft area must remain unobstructed and not enclosed by any structures. d. Landscaping within Council’s drainage easement shall be limited to grassed or paved surfaces only. Boundary fencing across floodway shall incorporate provision for the passage of overland stormwater runoff to cater for the 1:100-year A.R.I. storm. All approved construction details shall be consistent with this requirement. A copy of the approved landscaping details shall be submitted to Council for recordkeeping. e. For concrete pavements built over drainage easements full depth construction joints shall be provided along the easement boundary, in accordance with the requirements contained in Council's Development Engineering Standards. Details suitable for construction prepared by a qualified professional civil engineer shall be submitted to the principal before the issue of the construction certificate. A copy of the approved details shall be submitted to Council for recordkeeping. |
|  | Condition reason: To ensure the development is built and remains consistent with approved plans and documentation. |
|  | Stormwater Drainage |
|  | Stormwater drainage from the development shall be designed so as to comply with Council's Development Engineering Standards and the requirements of the BASIX Certificate. A final detailed stormwater drainage design shall be prepared by an appropriately qualified person as defined in the Building and Development Certifiers Regulation 2020 in accordance with the above requirements and shall generally be in accordance with the approved concept stormwater plan as shown in the table below. The final plan shall be certified by an appropriately qualified person as defined in the Building and Development Certifiers Regulation 2020 that it complies with Council's Development Engineering Standards, the BASIX Certificate and the relevant Australian Standards. Plan Number; Revision; Plan Name; Prepared By; Date. CE100; G; Stormwater Plan; ADP; 14/11/2025. CE250; D; Catchment Plan and Results; ADP; 13/08/2025. CE300; E; Sediment & Erosion Control Plan; ADP; 10/10/2025. CE350; C; Sediment & Erosion Control Details; ADP; 08/07/2025. |
|  | Condition reason: To ensure compliance with the relevant Australian Standard and National Construction Code. |
|  | Traffic Management Plan |
|  | Before the issue of any construction certificate for this development, the consent holder must obtain approval from Council’s Traffic Section for a Site, Pedestrian and Traffic Management Plan (SPTMP). This Plan must address the measures that will be implemented for pedestrian safety and traffic management as specified below. A PRIVATE CERTIFIER CANNOT APPROVE YOUR SITE, PEDESTRIAN & TRAFFIC MANAGEMENT PLAN . This plan shall include details of the following: - Proposed ingress and egress points for vehicles to and from the construction site; - Proposed protection of pedestrians, adjacent to the constructions site; - Proposed pedestrian management whilst vehicles are entering/exiting the construction site; - Proposed method of loading and unloading excavation machines, building material, construction materials and waste containers during the construction period; - Proposed traffic control measures such as advanced warning signs, barricades, warning lights, after hours contact numbers etc. are required to be displayed and shall be in accordance with Council's and Transport for New South Wales (TfNSW) requirements and AS1742.3 Traffic control devices for works on roads; - Proposed route for transportation of bulk and excavation materials to and from the development site. The route for transportation to and from the development site of bulk and excavation materials shall generally be by the shortest possible route to the nearest “State Road” via "Regional Roads", with every effort to avoid school zones on public roads. Alternate longer routes will only be considered in order to bypass school zones during school zone hours. If school zones cannot be avoided no heavy construction vehicle movements are to arrive or depart the site during signposted school zone periods 8:00am - 9:30am and 2:30pm - 4:00pm on school days. An Agreement to Council's satisfaction, signed by the consent holder/owner specifying the approved route and acknowledging responsibility to pay Council to rectify damages to public property adjacent to the site as a consequence of building works shall be lodged with Council before release of any construction certificate. Damage will be rectified as required by Council to remove unsafe conditions. All damage must be rectified upon completion of work to the satisfaction of Council. The approved Site, Pedestrian and Traffic Management Plan is to be implemented before the commencement of any works on the construction site. In addition, a TfNSW Approval / Road Occupancy Licence will be required for works on Regional or State Roads or within 100m of a traffic facility including roundabouts and traffic signals. Refer to Council's Development Engineering Standards for a list of Regional and State Roads. Note: Approval by Council of a SPTMP may take up to six (6) months. Accordingly, it is recommended the application to Council be submitted well in advance of the need for a construction certificate. |
|  | Condition reason: To provide and maintain orderly and safe areas for vehicular and pedestrian movement / parking. |
|  | Apply for Work Permit for Engineering Works |
|  | The consent holder shall apply for a Work Permit and obtain approval from Council, for any of the following applicable engineering works in front of the site, at their own expense: - Extra heavy duty splayed VFC (S-010A) of maximum width of 9.0 metres at the property boundary, - Drainage connection to Council's drainage system, - Removal of all driveway surfaces, reinstatement of laybacks to kerb and gutter and reshaping of the footway, all associated with redundant VFCs, - Repair of any damage to the public road including the footway occurring during building works, and - Reinstatement of the footway reserve and adjustment or relocation of existing public utility services to match the footway design levels as proposed on the approved Work Permit. Adjustment or relocation to any public utility services shall be carried out to the requirements of the public utility authority. Note: Council is required to prepare a site survey and design in order to determine the necessary information. The consent holder should make application and payment for the Work Permit at least twenty-one (21) days before the information being required and before the issue of the construction certificate. The Work Permit must be approved before any works commencing within the Council road reserve or on Council’s assets. |
|  | Condition reason: To provide and maintain orderly and safe areas for vehicular and pedestrian movement / parking. |
|  | Waste Management Plan |
|  | Before the issue of a construction certificate, a Waste Management Plan (WMP) must be prepared and provided to the certifier. The plan must include the following matters: The plan must be prepared - in accordance with: - the Environment Protection Authority’s Waste Classification Guidelines as in force from time to time, and - a development control plan that provides for waste management that applies to the land on which the work or the clearing of vegetation is carried out, and - include the following information— - the contact details of the person removing waste, - an estimate of the type and quantity of waste, - whether waste is expected to be reused, recycled or sent to landfill, - the address of the disposal location for waste. A copy of the waste management plan must be kept on-site at all times while work approved under the development consent is being carried out. 2.502 |
|  | Condition reason: To protect and enhance the amenity of the occupants of the development site and the occupants of adjoining sites. |
|  | Bin Carting Route |
|  | The certifier must not issue a construction certificate unless provided with detailed plans that form part of the construction certificate which identify that the bin carting route from the communal bin storage room to the waste collection point identified on stamped plans complies with the following requirements as well as the requirements of Council’s “Waste Management Guide for New Developments”: - Is direct and less than 10.0 metres in length, - Has a minimum width of 1.5 metres of hard surface; - Is of non-slip material and free from obstacles and steps; - Has a maximum grade of 1:30 (3%); - Has a kerbside ramp installed at the collection point; and - Has compliance with Work, Health and Safety legislation and standards |
|  | Condition reason: To ensure the orderly collection of waste from the site. |
|  | Bin Storage Room |
|  | The certifier must not issue a construction certificate unless provided with detailed plans that form part of the construction certificate for the communal bin storage room that comply with the following requirements: - Floors must be constructed of concrete at least 75mm thick and graded and drained to a Sydney Water approved drainage fitting; - Floors must be finished so as to be non-slip with a smooth and even surface; - A designated room or enclosure and is to have a roof, with a minimum 2.1m unobstructed room height. - Must be compatible with the overall design of the development; - Walls must be constructed of solid impervious material; - Ceilings must be finished with a smooth faced non-absorbent material capable of being cleaned; - Walls, ceiling and floors must be finished in a light colour; - An adequate supply of hot and cold water mixed through a centralised mixing valve with hose cock is to service the room; - A self-closing door openable from within the room; - Must be constructed to prevent the entry of birds and vermin; - Be provided with adequate light and ventilation. Light source must be through controlled light switches located both outside and inside the room; - Any doorways must be 2 metres wide and open outwards; and - Designed to fit the following bin allocations: Number; Bin Size; Bin Type. 2; 660L; Garbage (Red). 2; 660L; Recycling (Yellow). 1; 140L; Garden (Green). |
|  | Condition reason: To protect and enhance the amenity of the occupants of the development site and the occupants of adjoining sites. |
|  | Bin Carting Route |
|  | The certifier must not issue a construction certificate unless provided with detailed plans that form part of the construction certificate which identify that the bin carting route from the communal bin storage room to the waste collection point identified on stamped plans complies with the following requirements: - Is direct, - Has a minimum width of 2.0 metres of hard surface; - Is of non-slip material and free from obstacles and steps; - Has compliance with Work, Health and Safety legislation and standards |
|  | Condition reason: To protect and enhance the amenity of the occupants of the development site and the occupants of adjoining sites. |
|  | On-Site Waste Collection |
|  | Before the issue of any construction certificate, the certifier must receive written confirmation from Council, that Council has been provided to its satisfaction with the following: - Detailed plans showing that the waste storage rooms identified on stamped plans will be capable of being accessed from the designed access roadway by a Heavy Rigid Vehicle (HRV) in accordance with AS2890.2, and that a loading area is available for the vehicle when collecting waste that meets the following requirements: - Within 10 metres of the all waste storage rooms or temporary holding area; - A separate parking area for the collection vehicle; and - Include an extra 2 metres at the rear of the vehicle loading area. - Certification from a suitably qualified engineer that the route of travel, loading area and entry and exit points will be constructed so as to support the following loading requirements: - Heavy Rigid Vehicle can enter and exit the site in a forward position; - 30 tonne waste collection vehicles; - Length of 12 metres; |
|  | Condition reason: To ensure the orderly collection of waste from the site. |

## BEFORE BUILDING WORK COMMENCES

|  | Certifier details |
| --- | --- |
|  | A construction certificate is required for the erection of a building in accordance with this Determination Notice. This Determination Notice does not authorise building work until a principal certifier has been appointed as the principal certifier for the work by (or with the approval of) the consent holder. The following requirements apply before the commencement of building work in accordance with this Determination Notice: - the principal certifier has, no later than two (2) days before the building work commences, notified the Consent Authority and the Council (if the council is not the Consent Authority) of their appointment as the principal certifier, - the principal certifier has, no later than two (2) days before the building work commences, notified the consent holder of any inspections that are required to be carried out in respect of the building work, - the person carrying out the building work has notified the principal certifier that the person will carry out the building work as an owner-builder, if that is the case, - the consent holder, if not carrying out the work as an owner-builder, has: - appointed a principal contractor for the building work who must be the holder of a contractor licence if any residential building work is involved, and - notified the principal certifier of the appointment, and - unless that person is the principal contractor, notified the principal contractor of any inspections that are required to be carried out in respect of the building work, - the consent holder has given at least a 2-day notice to the Council, and the principal certifier if not the Council, of the person’s intention to commence the erection of the building. 3.202 |
|  | Condition reason: To ensure compliance with the relevant New South Wales legislation. |
|  | s73 Compliance Certificate |
|  | A Section 73 Compliance Certificate under the Sydney Water Act 1994 must be obtained from Sydney Water. It is recommended that the consent holder apply early for the certificate, as there may be water and sewer pipes to be built and this can take some time. This can also impact on other services and building, driveway or landscape design. Application can be made through an authorised Water Servicing Coordinator. For help either visit www.sydneywater.com.au > Plumbing, building and developing > Developing > Section 73 Compliance Certificates, or telephone 13 20 92. |
|  | Condition reason: To protect existing public and private infrastructure and building works during demolition, construction and ongoing use of the development. |

|  | Temporary fence or hoarding |
| --- | --- |
|  | A fence must be erected around the area of the development site, except where an existing 1.8 metres high boundary fence is in good condition and can secure the area. Any new fencing must be temporary and at least 1.8 metres high. All fencing must be maintained throughout the construction period to ensure that the work area is secured. Where the work is located within 3.6m of a public place, then a Type A or Type B hoarding must be constructed appropriate to the work proposed. An application for a Work Permit for such hoarding must be submitted to Council for approval before the commencement of building work/s. 3.204 |
|  | Condition reason: To ensure works undertaken are carried out in a safe manner in accordance with relevant policies. |
|  | WC temporary toilet facilities on site |
|  | Before the commencement of work, the consent holder must provide a temporary on-site toilet if access to existing toilets on site is not adequate/available. Temporary on-site toilet facilities must be installed and be adequately maintained throughout the construction period. 3.206 |
|  | Condition reason: To ensure compliance with the relevant New South Wales legislation. |
|  | Install Erosion control |
|  | Before the commencement of any site or building work, the principal certifier must be satisfied the erosion and sediment controls in the erosion and sediment control plan, (as approved by the principal certifier) are in place until the site is rectified (at least 70% ground cover achieved over any bare ground on site). 3.207 |
|  | Condition reason: To protect existing public and private infrastructure and building works during demolition, construction and ongoing use of the development. |
|  | Soil and water management warning sign |
|  | Council’s warning sign for Soil and Water Management must be displayed throughout the construction period at the most prominent point of the site, visible to both the street and site. 3.208 |
|  | Condition reason: To protect existing public and private infrastructure and building works during demolition, construction and ongoing use of the development. |
|  | Sign with principal certifier details |
|  | A sign must be erected in a prominent position on any site on which building work or demolition work is being carried out: - showing the name, address and telephone number of the principal certifier for the work, and - showing the name of the principal contractor (if any) for any building work and a telephone number on which that person may be contacted outside working hours, and - stating that unauthorised entry to the work site is prohibited. Any such sign is to be maintained while the building work or demolition work is being carried out and must be removed when the work has been completed. Note: This does not apply in relation to building work or demolition work that is carried out inside an existing building that does not affect the external walls of the building. 3.209.P |
|  | Condition reason: Prescribed condition under section 70 of the Environmental Planning and Assessment Regulation 2021. |
|  | Construction Traffic Management Plan (CTMP) / Site Pedestrian Traffic Management Plan (SPTMP) |
|  | A Construction Traffic Management Plan (CTMP) shall be required to be submitted six months prior to the commencement of works on the site, for both demolition and construction phases of the development / project. |
|  | Condition Reason: To ensure the management of traffic is upheld. |
|  | Tree protection measures |
|  | Before the commencement of any site or building work, the principal certifier must ensure the measures for tree protection are in place. 3.801.S |
|  | Condition reason: To protect the natural environment of the development site and adjoining lands. |
|  | Acid Sulfate Soil Preliminary Assessment |
|  | All recommendations made in the preliminary acid sulfate soil assessment prepared by GeoEnviro Consultancy Pty Ltd, titled ‘Geotechnical, Contamination/ Waste Classification and Acid Sulfate Soil Investigation Proposed Community Space’, dated January 2023, reference : JC22426A-r1 must be implemented. Any change made to the site that will or is likely to impact the recommendations of the preliminary site investigation will require a further assessment and is to be provided to Council prior to the commencement of works. |
|  | Condition reason: To protect the natural environment of the development site and adjoining land. |

## DURING BUILDING WORK

|  | Procedure for Critical Stage Inspections |
| --- | --- |
|  | While building work is being carried out, the work must not continue after each critical stage inspection unless the principal certifier is satisfied the work may proceed in accordance with this consent and the relevant construction certificate. 4.101.S |
|  | Condition reason: To ensure the development is built and remains consistent with approved plans and documentation. |
|  | Works in accordance with Building Code of Australia (BCA) |
|  | Building work must be carried out in accordance with the requirements of the Building Code of Australia (BCA). 4.201.P |
|  | Condition reason: Prescribed condition - EP&A Regulation clause 98(1)(a). |
|  | Hours of Work |
|  | Building work or demolition work may only be carried out between 7.00 am and 6.00 pm on Monday to Friday, or between 8.00 am and 1.00 pm on Saturdays, and no construction is to be carried out at any time on a Sunday or a public holiday. 4.204.S |
|  | Condition reason: To protect and enhance the amenity of the occupants of the development site and the occupants of adjoining sites. |
|  | Noise and Vibration |
|  | While site work is being carried out, noise generated from the site must be controlled in accordance with the requirements of the approved documentation. Where no noise and vibration management plan is approved under this consent, the certifier is to ensure that any noise caused by demolition, vegetation removal or construction does not exceed an LAeq (15 min) of 5dB(A) above background noise, when measured at any lot boundary of the property where the site work is being carried out. 4.205.S |
|  | Condition reason: To protect and enhance the amenity of the occupants of the development site and the occupants of adjoining sites. |
|  | Surveys by a registered surveyor |
|  | While building work is being carried out, the positions of the following must be measured and marked by a registered surveyor and provided to the principal certifier: - All footings / foundations in relation to the site boundaries and any registered and proposed easements - At other stages of construction – any marks that are required by the principal certifier. 4.206.S |
|  | Condition reason: To ensure the development is built and remains consistent with approved plans and documentation. |
|  | Civil and Hydraulic engineering works |
|  | All civil and hydraulic engineering works on site must be carried out in accordance with Council's Development Engineering Standards. All Civil and Hydraulic engineering works associated with Council's assets and infrastructure must be carried out in accordance with Council's Work Permit requirements and to Council's satisfaction. 4.301 |
|  | Condition reason: To ensure compliance with the relevant Canterbury Bankstown policy. |
|  | Stormwater drainage system |
|  | The stormwater drainage system shall be constructed in accordance with Council's Development Engineering Standards and the engineering plans and details approved by the principal certifier. Should the consent holder encounter any existing, live, underground stormwater drainage pipes, which carry flow from upstream properties, then they must maintain the stormwater flow and re-route the stormwater pipes around the subject building or structures at their own expense. 4.302 |
|  | Condition reason: To ensure compliance with the relevant Canterbury Bankstown policy. |
|  | Connection to existing water |
|  | The existing internal stormwater drainage system, and the stormwater drainage system draining the site shall be cleaned out, tested for leaks and repaired as necessary. A licensed plumber is to certify that the work has been carried out. |
|  | Condition reason: To protect and enhance the amenity of the occupants of the development site and the occupants of adjoining sites. |
|  | Driveway adequacy |
|  | A suitably qualified Professional Civil or Structural Engineer shall be engaged by the developer to carry out inspections relating to construction of internal driveways and parking areas. The work shall be carried out in accordance with the approved plans and specifications and certification from the Civil or Structural Engineer is to be provided upon completion. |
|  | Condition reason: To ensure compliance with the relevant Canterbury-Bankstown policy. |
|  | Responsibility for Changes to Public Infrastructure |
|  | While site work is being carried out, any costs incurred as a result of the approved removal, relocation or reconstruction of infrastructure (including ramps, footpaths, kerb and gutter, light poles, kerb inlet pits, service provider pits, street trees or any other infrastructure in the street footpath area) must be paid as directed by the consent authority. |
|  | Condition reason: To protect existing public and private infrastructure and building works during demolition, construction and ongoing use of the development. |

|  | Excavation safety |
| --- | --- |
|  | All excavations and backfilling must be executed safely and in accordance with the relevant Australian Standards. If the development involves an excavation that extends below the level of the base of the footings of a building, structure or work (including any structure or work within a road or rail corridor) on adjoining land, the consent holder must, at their own expense: - Protect and support the building, structure or work from possible damage from the excavation, and - Where necessary, underpin the building, structure or work to prevent any such damage. This consent condition does not apply if the consent holder owns the adjoining land or the owner of the adjoining land has given consent in writing to that condition not applying. The owner of any affected buildings is not liable for any part of the cost of work carried out for the purposes of this clause, whether carried out on the allotment of land being excavated or on an adjoining allotment of land. 4.308 |
|  | Condition reason: To ensure compliance with the relevant Australian Standard and National Construction Code. |
|  | Disposal of Excavated Soil |
|  | All soils to be excavated and disposed of from the site must be analysed and classified by a suitably qualified environmental consultant, in accordance with relevant NSW EPA guidelines including the ‘Waste Classification Guidelines’ (NSW EPA, 2014) prior to off-site disposal. All waste material for disposal to be transported to an appropriately licensed waste facility by an EPA licensed waste contractor in accordance with relevant NSW EPA guidelines. Receipts for the disposal of the waste must be submitted to Council prior to issuing of an occupation certificate. |
|  | Condition reason: To ensure compliance with the relevant New South Wales legislation. |
|  | Disposal of Acid Sulfate Soil |
|  | Any soils to be excavated and disposed of from the site must be analysed and classified by a suitably qualified environmental consultant, in accordance with relevant NSW EPA guidelines including Part 4 of the ‘Waste Classification Guidelines’ (NSW EPA, 2014) prior to off-site disposal. Any waste material is to be transported to an appropriately licensed waste facility by an EPA licensed waste contractor in accordance with relevant NSW EPA guidelines. |
|  | Condition reason: To ensure compliance with the relevant New South Wales legislation. |
|  | Importation of Fill |
|  | Any fill imported onto the site must be virgin excavated natural material or excavated natural material, classified as such in accordance the ‘Waste Classification Guidelines’ (NSW EPA, 2014) or excavated natural material meeting the requirements of the Excavated Natural Material Exemption (NSW EPA, 2014). Certificates verifying that imported fill is virgin excavated natural material or excavated natural material must be provided to Council prior to the issue of an occupation certificate. All imported fill must be compatible with the existing soil characteristics of the site. |
|  | Condition reason: To ensure compliance with the relevant New South Wales legislation. |

|  | Shoring and adequacy of adjoining properties |
| --- | --- |
|  | If the development involves an excavation that extends below the level of the base of the footings of a building, structure or work on adjoining land (including any structure or work within a road or rail corridor), the consent holder must, at their own expense — - Protect and support the building, structure or work from possible damage from the excavation, and - Where necessary, underpin the building, structure or work to prevent any such damage. This condition does not apply if the consent holder owns the adjoining land or the owner of the adjoining land has given consent in writing to that condition not applying. The owner of any affected buildings is not liable for any part of the cost of work carried out for the purposes of this clause, whether carried out on the allotment of land being excavated or on an adjoining allotment of land. |
|  | Condition reason: Prescribed condition under section 74 of the Environmental Planning and Assessment Regulation 2021. |
|  | Retaining walls |
|  | If soil conditions require it, retaining walls or other approved methods of preventing movement of the soil must be provided, and adequate provisions must be made for drainage. Separate approval may be required for retaining walls should they be required. 4.310 |
|  | Condition reason: To ensure works undertaken are carried out in a safe manner in accordance with relevant policies. |
|  | Waste management |
|  | While site work is being carried out: - all waste management must be undertaken in accordance with the waste management plan, and - upon disposal of waste, records of the disposal must be compiled and provided to , detailing the following: - The contact details of the person(s) who removed the waste - The waste carrier vehicle registration - The date and time of waste collection - A description of the waste (type of waste and estimated quantity) and whether the waste is to be reused, recycled or go to landfill - The address of the disposal location(s) where the waste was taken - The corresponding tip docket/receipt from the site(s) to which the waste is transferred, noting date and time of delivery, description (type and quantity) of waste. If waste has been removed from the site under an EPA Resource Recovery Order or Exemption, records in relation to that Order or Exemption must be maintained and provided to the principal certifier and council. 4.502.S |
|  | Condition reason: To protect and enhance the amenity of the occupants of the development site and the occupants of adjoining sites. |
|  | Waste Classification and Disposal of Contaminated Soil and Materials |
|  | All soils and material, liquid and solid, to be removed from the site must be analysed and classified by an appropriately qualified and experienced environmental consultant, in accordance with the Protection of the Environment Operations (Waste) Regulation 2014 and related guidelines, the ‘Waste Classification Guidelines’ (NSW EPA, 2014), prior to off-site disposal. The waste classification report, including the results of testing, must be compiled, or reviewed and approved by an appropriately qualified and certified consultant, and must be submitted to and approved by the principal certifier before off-site disposal. A copy of the waste classification report must be submitted to Council prior to issuing of an occupation certificate. All waste material must be disposed of at an appropriately licensed waste facility for the specific waste. Receipts for the disposal of the waste must be submitted to Council prior to issuing of an occupation certificate. All waste must be transported by a contractor licensed to transport the specific waste, and in vehicles capable of carting the waste without spillage and meeting relevant requirements and standards. All loads must be covered prior to vehicles leaving the site. |
|  | Condition reason: To ensure compliance with the relevant New South Wales legislation. |
|  | Demolition and Construction Waste |
|  | All waste material generated during demolition and construction must be disposed of at an appropriately licensed waste facility for the specific waste. All waste must be transported by a contractor licensed to transport the specific waste, and in vehicles capable of carting the waste without spillage and meeting relevant requirements and standards. All loads must be covered prior to vehicles leaving the site. |
|  | Condition reason: To ensure that the development complies with Council’s Environmental Health Policies. |
|  | Unexpected Finds |
|  | In the instance works cause the generation of odours or uncovering of unexpected contaminants works are to immediately cease, Council is to be notified and a suitably qualified environmental consultant appointed to further assess the site. The exposed material/excavation is to be evaluated by the supervising environmental consultant and an appropriate response determined in consultation with the applicant, which is agreed to by Council. Council may also request that an NSW EPA Accredited Site Auditor is involved to assist with the assessment of any unexpected finds, review any contamination information and prepare a site audit report and site audit statement. The applicant must adhere to any additional conditions which may be imposed by the NSW EPA Accredited Site Auditor. 4.601 |
|  | Condition reason: To ensure that the development complies with Council’s Environmental Health Policies. |
|  | Threatening of trees |
|  | If any trees to be retained and protected are threatened by demolition, building or other site works through accidental omission or unforeseen circumstances, the approving authority is to be notified immediately in writing before such work is continued so that this issue can be resolved. 4.810 |
|  | Condition reason: To protect the natural environment of the development site and adjoining lands. |
|  | Tree Protection |
|  | ## All work carried out on or around protected trees during demolition, clearing and construction shall comply with Australian Standard AS4373-2007 Pruning of amenity trees and Australian Standard AS4970-2025 Protection of trees on development sites. Site specific conditions relating to tree protection shall take precedence over this requirement. |
|  | Condition reason: To ensure compliance with the relevant Australian Standard |
|  | Tree Approval |
|  | Approval is granted for the removal of the following trees: - Any tree/s where the base of the trunk of the tree is located within 3 metres of the external wall of an approved dwelling; - Any declared noxious plant. The consent holder, builder and all contractors are to ensure that all noxious plants are properly identified, controlled and/or removed on this site without injury or death of any protected plants; - Any tree species listed under clause 2.4 of Canterbury-Bankstown Development Control Plan 2023, Chapter 2.3; - Any of the following tree/s: T1, T15, T18, T20, T23, T25, T27, T28, T29, T30, T34, T35, T36, T37, T38, T42 and T51 as per the Arboricultural Impact Assessment prepared by Tree Survey dated 8/7/25; - All tree removal works must be carried out by a qualified arborist - minimum qualification Australian Qualification Framework (AQF) Level 3 or equivalent; - All tree removal works must comply with the Amenity Tree Industry – Code of Practice, 1998 (Workcover, NSW) and Guide to Managing Risks of Tree Trimming and Removal Work (Safe Work Australia 2016). - All other vegetation not specifically identified above, and protected by Councils Tree Management Order, is to be retained and protected from construction damage and pruning. The Tree Management Order protects trees over 5m in height. - The tree removal contractor must hold a Public Liability Insurance Certificate of Currency with a minimum indemnity of $20 million, together with a NSW Workers Compensation Insurance Certificate of Currency; - The tree stump is to be ground to 300mm below ground level. Investigation to locate underground services shall be the responsibility of the consent holder. - All tree material shall be removed from site, and the nature strip / area shall be backfilled, compacted and restored to the original level. - The site must be maintained in a safe condition at all times; - Appropriate hazard signage to be in place at all times during the tree works |
|  | Condition Reason: |
|  | Discovery of relics and Aboriginal objects |
|  | While site work is being carried out, if a person reasonably suspects a relic for Aboriginal object is discovered: - the work in the area of the discovery must cease immediately; - the following must be notified - for a relic – the Heritage Council; or - for an Aboriginal object – the person who is the authority for the protection of Aboriginal objects and Aboriginal places in New South Wales under the National Parks and Wildlife Act 1974, section 85. Site work may recommence at a time confirmed in writing by: - for a relic – the Heritage Council; or - for an Aboriginal object – the person who is the authority for the protection of Aboriginal objects and Aboriginal places in New South Wales under the National Parks and Wildlife Act 1974, section 85. In this condition: “relic” means any deposit, artefact, object or material evidence that: - relates to the settlement of the area that comprises New South Wales, not being Aboriginal settlement, and - is of State or local heritage significance; and “Aboriginal object” means any deposit, object or material evidence (not being a handicraft made for sale) relating to the Aboriginal habitation of the area that comprises New South Wales, being habitation before or concurrent with (or both) the occupation of that area by persons of non-Aboriginal extraction and includes Aboriginal remains. 4.901.S |
|  | Condition reason: To ensure the protection of objects of potential significance during works. |

## BEFORE ISSUE OF AN OCCUPATION CERTIFICATE

|  | Occupation certificate |
| --- | --- |
|  | The occupation or use of the building must not commence unless an occupation certificate has been issued. 5.201 |
|  | Condition reason: To ensure compliance with the relevant New South Wales legislation. |
|  | Surveyor |
|  | A report as prepared by a registered surveyor must be submitted to the principal certifier verifying that the reduced level (RL) of the finished floor level at each level, the height of external walls, the maximum reduced level (RL) of roof structures / roof eaves overhangs and the external wall setbacks to the property boundaries all conform with the approved plans. |
|  | Condition reason: To ensure the development is built and remains consistent with approved plans and documentation. |
|  | Section 73 certificate |
|  | A Section 73 Compliance Certificate under the Sydney Water Act 1994 must be obtained from Sydney Water. It is recommended that the consent holder apply early for the certificate, as there may be water and sewer pipes to be built and this can take some time. This can also impact on other services and building, driveway or landscape design. Application can be made through an authorised Water Servicing Coordinator. For help either visit www.sydneywater.com.au > Plumbing, building and developing > Developing > Section 73 Compliance Certificates, or telephone 13 20 92. |
|  | Condition reason: To ensure compliance with the relevant New South Wales legislation. |
|  | Redundant Driveways |
|  | Any redundant driveways along Dale Parade and Northam Avenue are to be removed and reinstated with kerb and gutter and verge to match with remaining at the Applicant’s cost. |
|  | Condition Reason: To ensure the development remains consistent. |
|  | Access Control |
|  | Prior to issuing an occupation certificate, ensure measures are in place to restrict and channel access for people and vehicles. Utilize physical and symbolic barriers to guide pedestrians and increase crime prevention effectiveness. |
|  | Condition Reason: To ensure access is managed appropriately. |
|  | Master Lock |
|  | Install deadlocks to prevent theft and fraud in accordance with AS 4145.2—2008. |
|  | Condition Reason: To ensure the site is protected. |
|  | Landscaping |
|  | ## Landscaping is to be installed in accordance with the approved landscape plan. All works and methods nominated and materials and plants specified on the approved landscape plan are to be completed prior to the issue of an occupation certificate. The landscaping shall be maintained for the life of the development. |
|  | Condition Reason: To protect the natural environment of the development site and adjoining lands and to ensure the development is built and remains consistent with approved plans and documentation. |
|  | Confirmation of waste facilities |
|  | No occupation certificate may be issued until such time as written confirmation has been received from Council that all waste management facilities and associated access have been constructed in accordance with the requirements of this consent. The waste management facilities include the path of travel for all waste, from unit to point of waste collection, waste storage room sizing, access to water and sewer connections, finished materials, access and doorway dimensions. |
|  | Condition reason: To ensure the development is built and remains consistent with approved plans and documentation. |
|  | Acoustic Report |
|  | The acoustic report submitted in support of this application prepared by ADP Consulting: Engineering, titled ‘Griffith Park Community Centre Bankstown – Acoustic DA Report’, reference number SYD3293, dated 16 May 2025 and all the recommendations stated within the report, form part of the development consent. The principal certifier shall obtain a report from an appropriately qualified acoustic consultant, not previously involved with the development, stating that the recommendations outlined in the above-mentioned acoustic report have been implemented and that the relevant noise criteria have been satisfied prior to the issue of any occupation certificate. A copy of the report is to be submitted to Council prior to the issue of any occupation certificate. |
|  | Condition reason: To ensure the development is built and remains consistent with approved plans and documentation. |
|  | Work Permit Compliance required |
|  | An occupation certificate must not be issued until all conditions relating to demolition, building and subdivision works have been satisfied and Council has issued a Work Permit Compliance Certificate (where applicable). |
|  | Condition reason: To ensure the development is built and remains consistent with approved plans and documentation. |
|  | Redundant Driveways |
|  | Any redundant driveways along the site’s frontage to Waterloo Road are to be removed and reinstated with kerb and gutter and the verge to match with existing. |
|  | Condition reason: To ensure redundant driveways are removed. |
|  | Repair of infrastructure |
|  | Before the issue of an occupation certificate: - any public infrastructure damaged as a result of the carrying out of work approved under this consent (including damage caused by, but not limited to, delivery vehicles, waste collection, contractors, sub-contractors, concreting vehicles) must be fully repaired to the written satisfaction of council, and at no cost to council, or - if the works in (a) are not carried out to council’s satisfaction, council may carry out the works required and the costs of any such works must be paid as directed by council and in the first instance will be paid using the security deposit required to be paid under this consent. |
|  | Condition reason: To protect existing public and private infrastructure and building works during demolition, construction and ongoing use of the development. |
|  | Works-as-executed plans and any other documentary evidence |
|  | Before the issue of the relevant occupation certificate, works-as-executed plans, any compliance certificates and any other evidence confirming the following completed works must be submitted to the satisfaction of the principal certifier: - All stormwater drainage systems and storage systems, and - The following matters that council requires to be documented in accordance with the Councils relevant Development Control Plan. - A copy of the plans must be provided to council with the occupation certificate. |
|  | Condition reason: To ensure the development is built and remains consistent with approved plans and documentation. |
|  | Driveways certified |
|  | A suitably qualified professional civil engineer shall certify that the driveways, parking bays, and service areas have been constructed in accordance with the approved plans and specifications. Such Certification shall be submitted before the issue of the occupation certificate or occupation of the site. |
|  | Condition reason: To ensure the development is built and remains consistent with approved plans and documentation. |
|  | Business Registration |
|  | Prior to the issue of any occupation certificate, a food business registration form must be completed and submitted to Council. This form is available online at www.cbcity.nsw.gov.au. In the instance details on the original registration form change, Council is to be notified of the change within seven (7) days of the change occurring. |
|  | Condition reason: To ensure that the development complies with Council’s Environmental Health Policies. |

## OCCUPATION AND ONGOING USE

|  | Hours of operation |
| --- | --- |
|  | During ongoing use of the premises, the hours of operation of the Griffith Park Community Centre are restricted to: From; To. Monday; 8am; 10pm. Tuesday; 8am; 10pm. Wednesday; 8am; 10pm. Thursday; 8am; 10pm. Friday; 8am; 12am. Saturday; 8am; 12am. Sunday; 8am; 10pm. |
|  | Condition reason: To protect and enhance the amenity of the occupants of the development site and the occupants of adjoining sites. |
|  | Use of Car Parking Spaces and Driveways |
|  | The car parking spaces, driveways and manoeuvring areas must be used solely for vehicular access and for the parking of vehicles associated with the use of the premises. The car parking spaces, driveways and manoeuvring areas must not be used for the manufacture, storage or display of goods, materials or any other equipment (includes mobile food vending vehicles). |
|  | Condition reason: To provide and maintain orderly and safe areas for vehicular and pedestrian movement / parking. |
|  | Plan of Management |
|  | The Operational plan of management submitted in support of this application prepared by Canterbury-Bankstown Council, dated, 12 June 2025, forms part of the development consent. In the event of any inconsistency, the conditions of this consent will prevail over the plan of management. |
|  | Condition reason: To ensure the development is built and remains consistent with approved plans and documentation. |
|  | Food Safety Requirements |
|  | All parts of the premises used for the storage, preparation and handling of food must be designed, constructed and operated in accordance with the requirements of: - The Food Act 2003 and Food Regulation 2015; - Australia New Zealand Food Standards Code; - AS 4674-2004, ‘Design, construction and fit-out of food premises’; AS 1668.2 – ‘The Use of Ventilation and Air-Conditioning in Buildings – Mechanical Ventilation in Buildings’ |
|  | Condition reason: To ensure compliance with the relevant New South Wales legislation. |
|  | Method of Cooking – Solid Fuel (charcoal, wood fire etc.) |
|  | The installation of any solid fuel (charcoal, wood fire etc.) cooking appliances is not permitted on the premises. The cooking of food at the premises by means of solid fuel is not permitted. |
|  | Condition reason: To ensure that the development complies with Council’s Environmental Health Policies. |
|  | Waste Food Oil Storage |
|  | Where cooking by oil occurs, the premise shall contain adequate holding facilities for the reception of waste food oil. Waste food oil must be stored in a covered and bunded area and regularly removed by a waste oil recycler. |
|  | Condition reason: To ensure that the development complies with Council’s Environmental Health Policies. |
|  | Final Inspection |
|  | Prior to the issue of an occupation certificate, a pre-occupation premises inspection report is to be undertaken by Council’s Environmental Health Officer. |
|  | Condition reason: To ensure that the development complies with Council’s Environmental Health Policies. |
|  | No Mobile Food Vending Vehicles or Temporary Food Premises |
|  | No mobile food vending vehicles or temporary food premises are permitted to operate at the premises without prior development consent from Council. |
|  | Condition reason: To ensure that the development complies with Council’s Environmental Health Policies. |
|  | Complaint Handling Procedure |
|  | The Griffith Park Community Centre management must implement and maintain a complaint handling procedure. Every complaint must be recorded on a Complaint Form and kept in a Complaints Register. The Complaint Form must contain the following information: - Name, address and contact details of the Complainant; - Time and date the complaint was received; - The nature of the complaint or incident (see below what constitutes an incident); - The time and date the incident occurred; - The name of the employee that received the complaint; - Actions taken to investigate the complaint and the summary of the results of the investigation; - Indication of what was occurring at the time the incident was observed; - Required remedial action; - Validation of the remedial action; - Summary of feedback to the complainant and name of employee who gave the feedback. Incident includes: - Any breach of a plan of management (where applicable); - Any complaint by any person about the operation of the premises; - Any complaint by any person about noise emanating from the premises; or - Any event that may cause concern to any person as a result of the conduct and/or an act of any person(s) on, entering or exiting the premises. All complaints shall have an initial investigation commence within seven (7) days from the date of receipt of complaint. The Complaints Register must always be held on the premises and shall be reviewed monthly by Roberts Park Community Hub to ensure complaints are investigated and resolved in a timely manner. The Complainant must be notified of the results and actions arising from the investigation. The Complaints Register must contain a direction that all complaints of a criminal nature are to be reported to the Police immediately. A copy of the Complaints Register shall be provided to Council or Police immediately upon request. |
|  | Condition reason: To protect and enhance the amenity of the occupants of the development site and the occupants of adjoining sites. |
|  | Further Acoustic Assessment |
|  | Following occupation of the building/premises, should it be found that the measures recommended in the acoustic assessment are not sufficient, or have been incorrectly installed or a noise issue (relating to the development) not previously identified arises (through complaint or otherwise), the owner/ occupier must, upon request by Council, employ the services of a qualified acoustic consultant who has not previously been involved with the development to undertake a post-occupation assessment of the development and complete an acoustic report with recommendations to rectify the situation. A copy of this acoustic assessment report must be submitted to Council for approval and from there noise attenuation works must be implemented. Submission of the acoustic report must be within 30 days from the date requested by Council. |
|  | Condition reason: To protect and enhance the amenity of the occupants of the development site and the occupants of adjoining sites. |
|  | Acoustic Assessment |
|  | Should a noise issue (relating to the development) not previously identified arise (through complaint or otherwise), the owner/occupier must, upon request by Council, employ the services of a suitably qualified acoustic consultant who has not previously been involved with the development to undertake a post-occupation assessment of the development and complete an acoustic report with recommendations to rectify the noise issue. A copy of this acoustic assessment report must be submitted to Council for approval and from there noise attenuation works must be implemented. Submission of the acoustic report must be within 30 days from the date requested by Council. Note: Suitably Qualified Acoustic Consultant means a consultant who possesses the qualifications to render them eligible for membership of the Australian Acoustical Society, Institution of Engineers Australia or the Association of Australian Acoustical Consultants at the grade of member. |
|  | Condition reason: To protect and enhance the amenity of the occupants of the development site and the occupants of adjoining sites. |
|  | Compliance with Noise Control Legislation |
|  | The applicant must ensure that all activities within the premises comply with the relevant sections of the Protection of the Environment Operations Act 1997 and Regulations and the Noise Policy for Industry (NSW EPA, 2017). |
|  | Condition reason: To protect and enhance the amenity of the occupants of the development site and the occupants of adjoining sites. |
|  | Vibration |
|  | The use of the premises and the operation of plant and equipment must not give rise to the transmission of a vibration nuisance or damage to other premises as defined in the ‘Assessing Vibration: A Technical Guideline’ (NSW Department of Environment and Conservation, 2006). |
|  | Condition reason: To protect and enhance the amenity of the occupants of the development site and the occupants of adjoining sites. |
|  | Amplified Music |
|  | Music and other amplified sound played on the premises must not give rise to offensive noise as defined under the provisions of the Protection of the Environment Operations Act 1997. The sound level output must not exceed five (5) dB(A) above the ambient background level at the boundary of the premises. Speakers must not be installed, and music must not be played in any of the outdoor areas associated with the premises including the public domain. Speakers located within the premises must not be placed to direct the playing of music towards the outdoor areas associated with the premises. |
|  | Condition reason: To protect and enhance the amenity of the occupants of the development site and the occupants of adjoining sites. |
|  | Control of Noise upon Exiting |
|  | A clearly visible sign must be permanently erected immediately adjacent to the entry/exit doors indicating that patrons are to leave in an orderly fashion and must leave the vicinity of the premises in a manner that does not disturb the quiet and good order of the neighbourhood. |
|  | Condition reason: To protect and enhance the amenity of the occupants of the development site and the occupants of adjoining sites. |
|  | General Amenity |
|  | The operation of the development must not adversely affect the amenity of the neighbourhood or interfere unreasonably with the comfort or repose of a person who is outside the premises by reason of the emission or discharge of noise, fumes, vapour, odour, steam, soot, ash, dust, wastewater, waste products, grit, oil or other harmful products. |
|  | Condition reason: To protect and enhance the amenity of the occupants of the development site and the occupants of adjoining sites |
|  | Waste generated on site |
|  | All waste generated on the site is to be stored, handled and disposed of in such a manner as to not create offensive odour, offensive noise or pollution of land and/or water as defined under the Protection of the Environment Operations Act 1997. All waste generated shall be removed and disposed of by an authorised waste removal contractor. A copy of the waste removal agreement and receipts shall be made available to Council on request. 7.504 |
|  | Condition reason: To protect and enhance the amenity of the occupants of the development site and the occupants of adjoining sites. |
|  | Lighting |
|  | Any lighting on the site shall be designed so as not to cause nuisance to other residences in the area or to motorists on nearby roads and to ensure no adverse impact on the amenity of the surrounding area by light overspill. All lighting shall comply with the Australian Standard 4282-2019 Control of the Obtrusive Effects of Outdoor Lighting. No flashing, moving or intermittent lighting, visible from any public place may be installed on the premises or external sign associated with the development The use of floodlighting or the like, to advertise or attract attention or for the convenience of patrons must be controlled so as not to cause any distraction or disturbance to nearby or adjacent residents, pedestrians or motorists. The use of flashing lights is strictly prohibited. The implementation of this development shall not adversely affect the amenity of the neighbourhood or interfere unreasonably with the comfort or repose of a person who is outside the premises. |
|  | Condition reason: To ensure compliance with the relevant Australian Standard and National Construction Code. |
|  | POEO – General |
|  | Any activity carried out in accordance with this approval must not give rise to offensive odour, offensive noise or pollution of air, land or water as defined in the Protection of the Environment Operations Act 1997 and Regulations. |
|  | Condition reason: To ensure compliance with the relevant New South Wales legislation. |

General advisory notes

- This consent contains the conditions imposed by the consent authority which are to be complied with when carrying out the approved development. However, this consent is not an exhaustive list of all obligations which may relate to the carrying out of the development under the EP&A Act, EP&A Regulation and other legislation. Some of these additional obligations are set out in the Conditions of development consent: advisory notes. The consent should be read together with the Conditions of development consent: advisory notes to ensure the development is carried out lawfully.

- The approved development must be carried out in accordance with the conditions of this consent. It is an offence under the EP&A Act to carry out development that is not in accordance with this consent.

- Building work or subdivision work must not be carried out until a construction certificate or subdivision works certificate, respectively, has been issued and a principal certifier has been appointed.

- A document referred to in this consent is taken to be a reference to the version of that document which applies at the date the consent is issued, unless otherwise stated in the conditions of this consent.

#### Dictionary

The following terms have the following meanings for the purpose of this determination (except where the context clearly indicates otherwise):

Approved plans and documents means the plans and documents endorsed by the consent authority, a copy of which is included in this notice of determination.

AS means Australian Standard published by Standards Australia International Limited and means the current standard which applies at the time the consent is issued.

Building work means any physical activity involved in the erection of a building.

Certifier means a council or a person that is registered to carry out certification work under the Building and Development Certifiers Act 2018.

Construction certificate means a certificate to the effect that building work completed in accordance with specified plans and specifications or standards will comply with the requirements of the EP&A Regulation and Environmental Planning and Assessment (Development Certification and Fire Safety) Regulation 2021.

Council means Canterbury Bankstown Council.

Court means the Land and Environment Court of NSW.

EPA means the NSW Environment Protection Authority.

EP&A Act means the Environmental Planning and Assessment Act 1979.

EP&A Regulation means the Environmental Planning and Assessment Regulation 2021.

Independent Planning Commission means Independent Planning Commission of New South Wales constituted by section 2.7 of the EP&A Act.

Local planning panel means Canterbury Bankstown Local Planning Panel.

Occupation certificate means a certificate that authorises the occupation and use of a new building or a change of building use for an existing building in accordance with this consent.

Principal certifier means the certifier appointed as the principal certifier for building work or subdivision work under section 6.6(1) or 6.12(1) of the EP&A Act respectively.

Site work means any work that is physically carried out on the land to which the development the subject of this development consent is to be carried out, including but not limited to building work, subdivision work, demolition work, clearing of vegetation or remediation work.

Stormwater drainage system means all works and facilities relating to:

the collection of stormwater,

the reuse of stormwater,

the detention of stormwater,

the controlled release of stormwater, and

connections to easements and public stormwater systems.

Strata certificate means a certificate in the approved form issued under Part 4 of the Strata Schemes Development Act 2015 that authorises the registration of a strata plan, strata plan of subdivision or notice of conversion.

Subdivision certificate means a certificate that authorises the registration of a plan of subdivision under Part 23 of the Conveyancing Act 1919.

Subdivision works certificate means a certificate to the effect that subdivision work completed in accordance with specified plans and specifications will comply with the requirements of the EP&A Regulation.

Sydney district or regional planning panel means Sydney South Planning Panel.

#### Appendix – Transport for NSW Conditions