| SWCPP Ref. No. | PPSSWC-7 |
| --- | --- |
| DA Number | DA19/0470 |
| LGA | ##### Penrith City Council |
| Proposed Development | ##### Alterations to existing approved waste management & resource recovery facility including alterations to finished landform & increase in waste storage capacity |
| Street Address | 1725a Elizabeth Drive, Badgerys Creek |
| Applicant | SUEZ Recycling & Recovery Pty Ltd |
| Owner | SUEZ Recycling & Recovery Pty Ltd |
| Regional Planning Panel Criteria | Designated Development |
| Addendum Report Author | Paul Anzellotti, Senior Development Assessment Planner, Penrith City Council |
| Addendum Report Date | 12 June, 2020 |

Assessment Report Addendum

Penrith City Council has received commentary from Jamie McMahon of AECOM on behalf of the applicant (dated 11 June, 20200 in regard to the draft conditions of consent provided as part of the assessment report prepared for DA19/0470 (SWCPP Ref. No. PPSSWC-7) to be considered by the Sydney Western City Planning Panel on the 15 June, 2020.

In this regard, a table is provided below outlining which conditions have been addressed, commentary provided by the applicant for each specific condition and response by Penrith City Council.

In response to the matters raised, an updated draft conditions of consent are provided at Addendum A.

| Proposed condition to be modified | Commentary provided by applicant | Commentary in response provided by Council |
| --- | --- | --- |
| Condition 1 | Add: ‘…and as per GHD's Elizabeth Drive Landfill Expansion Technical Advice dated Feb 2020 and GHD's Technical Letter to EPA dated 16 April 2020’ | This modification to the condition is not supported as it requests inclusion of a document requesting amendments to the existing Environment Protection Licence (EPL), contrary to the requirements of draft condition 4, which require amendments to the EPL to be provided for as part of any forthcoming development consent. |
| Condition 1 | Amend to read ‘Final landscape plan as per condition 7’ | Agreed. While so, preferred additional wording is as follows; ‘as amended to satisfy condition 7 of this consent.’ |
| As part of condition 1. Condition 1 | Add four more documents to this list for completeness of the drawings: 1) GHD's Elizabeth Drive Landfill Expansion Technical Advice report dated Feb 2020, 2) GHD's Technical Letter to EPA dated 16th April 2020 3) SK003 Staging Schedule by GHD dated April 2020, 4) SK004 Final Cap Schedule and Proposed Final cap detail for General waste cells by GHD dated April 2020 | It is noted that requested item 1 is already included as part of condition 1. The inclusion of items 2 to 4 are not supported as discussed in the first item of this document under condition1. |
| Condition 8: The development is to be carried out in three (3) stages as per approved Sheet No. 60571292-SHT-CI-00007 (Indicative Stage 1 Fill - Layout), Sheet No. 60571292-SHT-CI-00008 (Indicative Stage 2 Fill - Layout) and Sheet No. 60571292-SHT-CI-00009 (Indicative Stage 3 Fill - Layout), all prepared by AECOM, all Issue B, all dated 4 February, 2019. | Add: ‘ …and in accordance with GHD's Technical Letter to EPA dated 16th April 2020’ | The modification of this Condition as requested is not supported as discussed in the first item of this document under condition1. |
| Condition 9: Any changes to the nature and volume of gas flaring which would increase the velocity of gas exiting the flare/combustion system is to be reported immediately by the applicant to the Western Sydney Airport Authority and the NSW Environment Protection Agency. If it is identified by the Western Sydney Airport Authority that a change to the nature and volume of gas flaring will create a safety concern in the operation of the Western Sydney Airport, all landfill operations on the subject site including flaring are to cease and not recommence until documentation in relation to the nature and volume of gas flaring is provided for review and to the satisfaction of the NSW Environment Protection Authority in relation to the safety of operating gas flaring on the subject site and impacts on the Western Sydney Airport. | Whilst we agree with the intent of this condition to require any new landfill gas burning to only occur in consultation with Western Sydney Airport, the condition currently goes too far in that it effectively allows WSA to stop all activities within the site if they ‘identify’ that a change to gas burning will create a safety issue. Given the DA doesn’t request or allow for the expansion of the rate of gas burning within the site this condition is unnecessarily forceful and provides WSA with a form of veto over all operations within the site, whether or not they have anything to do with gas burning. In addition, any changes to gas burning infrastructure would be the subject of a future DA in any case, at which point WSA would be consulted anyway. As such we propose the following: Landfill gas burning within the site is to remain consistent with the current capacity and the conditions of DA12/0515. Any changes to the nature or rate of gas burning may only be implemented if approved via development application. In the event of unexpected or emergency landfill gas burning (such as for safety reasons), the applicant is to report such events immediately to the Western Sydney Airport Authority and the NSW Environment Protection Agency. | Agreed. |
| Condition 12: A re-assessment of the site's restoration and bush fire protection measures is to be undertaken at the time that the site ceases to operate as an active landfill. Written evidence is to be provided from the Rural Fire Service identifying that any document prepared is in accordance with Planning for Bush Fire Protection 2006 and a copy provided to Penrith City Council at the time that the site ceases to operate. | Add a section to relate to the updated version of the Planning for Bush Fire Protection, noting this has now been updated to a 2019 version. Written evidence is to be provided from the Rural Fire Service identifying that any document prepared is in accordance with Planning for Bush Fire Protection 2006 (or the relevant document at the time) and a copy provided to Penrith City Council at the time that the site ceases to operate. | Agreed. |
| Condition 17 No retail sale of any materials extracted, produced or recycled on the site shall be conducted from the subject premises. | SUEZ’ view is that this condition is not necessary as the project does not in any way request that retail sales be undertaken on site, nor is this permitted by the existing consent. As such the conduct of retail sales would not be legal in the first place. SUEZ does however reserve the right to explore future options within the site relating to local waste management for the aerotropolis, recycling and the broader regional circular economy – all of which will be important in the context of the dramatic changes to land use within the region. Any such activity would be the subject of a future development application. If this future DA was lodged as a standalone application (not a modification of this consent), and was approved, it would then require an additional modification to this consent to amend this condition. As such the inclusion of this condition a) is irrelevant to the subject DA, and b) creates a potential unnecessary administrative burden in the future. On this basis we request it be removed entirely. | As retail sales are not identified to be provided as part of this application, it is considered appropriate that this condition be retained. |
| Condition 21: Mud and soil from vehicular movements to and from the site must not be deposited on the road. | Whilst we agree with the intent of this condition it is not practical in its current form in that it may be interpreted as no mud or soil whatsoever. This is simply not achievable. Instead we propose the same condition be imposed here as suggested to the EPA for internal sealed roads, being: During operating hours the site access road must be inspected twice per day and cleaned with a high-pressure water spray if clumps of dirt, deposited sediment or other soil or waste debris are present. This is not required if it is raining or has rained within the previous four hours. | The modification of this condition is not supported as it is considered that the proposed replacement wording is open for interpretation, for instance the requirement to not clean the road if it has been raining within a time period as well as the nature of any high pressure water spray to be used and its further impact upon the surrounding environment. |
| Condition 25: Run-off, including that from truck wash-down facilities (and sediment) from within the extraction area, must not be discharged from the site. and, Condition 26: All water needed for dust suppression, irrigation, fire fighting, etc must be collected from runoff from the site or sources satisfactory to Council. Sufficient water must be available for these purposes. Water is not to be pumped from or discharged into Badgerys Creek. | Both of these conditions state that no off site water discharges are acceptable. Section L2 of the EPL for the site however allows for some discharges (which typically only occur during wet weather when sediment basins overflow). We request that these conditions are updated to be consistent with the intent of the EPL in the following manner: 25. All runoff, including that from truck washdown facilities (and sediment) from within the extraction area, must be directed towards on-site sediment basins. 26. All water needed for dust suppression, irrigation, fire fighting, etc must be collected from runoff from the site or sources satisfactory to Council. Sufficient water must be available for these purposes. Water is not to be pumped from Badgerys Creek. Any excess surface water within site, including runoff, is to be directed towards on-site sediment basins. Also, condition 26 makes reference to ‘sources satisfactory to Council’. This should be clarified as there is no indication of what such sources would be. | Agreed. |
| .Condition 27: The facility must not accept waste or recyclable materials from members of the general public. | We see no specific reason why this needs to be a condition of consent given it has never been requested or even mentioned in the DA. SUEZ currently have no such plans but in the event that we did there is no sensible reason why these conditions should prevent any future application for such activities. | The removal of this condition is not supported as it is considered relevant to provide for a condition not allowing the general public to use the facility. |
| Condition 31: The following dust mitigation measures are to be actioned in the operation of the landfill at all times; - Sealing is to be provided to waste delivery haul routes. Sealed roads are to be progressively laid moving onto the landfill towards the tipping face. Only the final 50m of the haul/turnaround area is to remain unsealed. - Twice daily cleaning of sealed roads is to be provided using high-pressure water spray from on-site water carts. - Shale covers are to be placed on non-active areas of the general and restricted waste cells. - Tarps are to be used at night on the general waste cell batters for the life of the project. - Shale and clay cover is to be placed on non-active surfaces. - Chemical stabilisers are to be used on disused shale stockpiles. | - Update second dot point in line with changes requested to EPA GTAs i.e. During operating hours sealed roads must be inspected twice per day and cleaned with a high-pressure water spray if clumps of dirt, deposited sediment or other soil or waste debris are present. This is not required if it is raining or has rained within the previous four hours. - Fifth dot point makes reference to ‘shale and clay cover’ on non-active surfaces. The agreed plans with the EPA only require clay on final capped surfaces, and the requirement for shale as an intermediate cover is already captured in the third dot point. We suggest this fifth dot point is removed entirely. The final dot point refers to the use of chemical stabilisers on disused shale stockpiles. We request that this is updated as follows to reflect current industry standards of utilising hydromulch on these areas: Hydromulch or similar products with stabilising additives are to be used on disused shale stockpiles | For the request to the second dot point, this is not supported as no commentary has been returned by the EPA supporting the change to the GTA. Agreed. Agreed. |
| Condition 39: The applicant is to ensure that the access road from the site to Elizabeth Drive is kept clean and dust free. Any cost incurred by Penrith City Council in cleaning activities resulting from the development, must be funded by the applicant. | This condition doubles up condition 21. It also raises confusion in the event that soiling of the road is incurred by other users of this road and not by vehicles entering or exiting the SUEZ site. Suggest this condition is removed entirely. | Agreed, |
| Condition 40: Total vehicle movements are not to exceed 640 per day. | The body of the assessment report indicates that the proposed cap of 640 vehicles per day was derived from the traffic and transport study in the EIS. It should be noted these numbers in the EIS were based on long term averages and predictions of individual vehicle capacities, rather than on actual daily maximums. As such, even under current operations (750,000 tpa), the number of vehicle movements per day sometimes exceeds 640. The EIS demonstrated that traffic issues are not a problem based on the current fluctuating levels and RMS has also raised no objections to the proposal. In addition, as noted in the assessment report, the future upgrade of Elizabeth Drive and ‘bypass’ provided by the M12 will provide substantial additional capacity on Elizabeth Drive, hence rendering any traffic benefits from this mandated reduction in vehicle movements obsolete. Given this and the fact that we do sometimes have busier days above 640 movements, we request that the previous limit of 780 vehicle movements per day be retained. | The argument presented to provide for a modification of this condition is not supported. Section 3.1.5, ‘Summary of Future Daily Heavy Vehicle Trips with Project’ provided within the Traffic and Transport Impact Technical Report as part of the Environmental Impact Statement reads as follows; The existing consent for the Site limits the number of daily heavy vehicle movements per day to 780. Based on the future site operation with the Project, it is expected that the forecast daily heavy vehicle movements generated by the Site will remain under 780 (Table 3.7). Table 3.7 – Forecast total daily vehicle movements of SUEZ’s Elizabeth Drive Resources Recovery Park with this Project provides the following for a typical weekday; - Number of daily heavy vehicle movements – landfill without project = 444 - Number of additional daily heavy vehicle movements – landfill with project = 116 - Number of daily heavy vehicle movements – SAWT = 80 - Total daily heavy vehicle movements = 640 As the application was assessed and referred to relevant authorities based upon the figures provided within the accompanying Traffic and Transport Impact Technical Report and referrals returned based upon the figures within this report, the provision of a maximum 640 vehicle movements is considered appropriate. |
| Condition 41: All landscape works are to be constructed in accordance with the stamped approved plans and as amended by the conditions of this consent. Landscaping shall be maintained: - in accordance with the approved plans, and - in a healthy state, and in perpetuity by the existing or future owners and occupiers of the property. If any of the vegetation comprising that landscaping dies or is removed, it is to be replaced with vegetation of the same species and, to the greatest extent practicable, the same maturity as the vegetation which died or was removed. | We request the addition of text to time-limit the operation of this condition to the end of landfilling operations. This is because at this stage (final operations) the site will be well in its way to being fully grassed over and the need for strict vegetation management as a visual screen will become obsolete. | The modification of this condition is not considered acceptable. The vegetation to the perimeter of the subject site is considered to contribute the visual presentation of the landfill when viewed especially from the eastern and southern sides. As operations are not time restricted and noting the possibility for new uses adjoining the subject site with the development of the Aerotropolis, retention of this condition is considered appropriate. |
| Condition 43: All trees and landscaping along the perimeter of the site must be maintained in perpetuity to enable its growth to full maturity in healthy growing conditions. This includes mature height, spread and form, consistent with the tree species. Pruning must not alter the natural form and height of any trees. | As above, we request that this condition is time-limited to the end of landfilling operations. The carrying out of any consent condition ‘in perpetuity’ is highly impractical. A second issue with this condition is the limits on pruning. On occasion it is necessary to prune trees in order to maintain internal road access or to provide access to operational facilities such as sediment basins (particularly during emergencies in order to manage discharges). In such cases pruning is always carried out to the minimum possible, but inevitably ‘alters the natural form and height of trees’. This condition should be qualified by adding the following text: ‘…unless required to maintain normal operations such as access to roads and other operational facilities within the site’ | The removal of the wording ‘in perpetuity’ is considered appropriate for this condition but time limiting is not supported as discussed for condition 41. The inclusion of the additional text is supported. |

Addendum A – Revised draft conditions of consent for DA19/0470 in response to correspondence received on behalf of the applicant dated 11 June, 2020

General

- The development must be implemented substantially in accordance with the following approved plans and documents, the application form and any supporting information received with the application, except as may be amended in red on the approved plans and documents and by the following conditions.

| Plan No. | Issue | Title | Prepared by | Date |
| --- | --- | --- | --- | --- |
| 60571292-SHT-CI-00001 | B | Locality Plan and Drawing Index | AECOM | 4 January, 2019 |
| 60571292-SHT-CI-00004 | B | Proposed Final Landform Pre-Settlement Contours | AECOM | 4 February, 2019 |
| 60571292-SHT-CI-00005 | B | Pre-Settlement Final Landform Sections and Approved Cap Sections | AECOM | 4 February, 2019 |
| 60571292-SHT-CI-00006 | B | Pre-Settlement Final Landform Sections | AECOM | 4 February, 2019 |
| 60571292-SHT-CI-00007 | B | Indicative Stage 1 Fill - Layout | AECOM | 4 February, 2019 |
| 60571292-SHT-CI-00008 | B | Indicative Stage 2 Fill - Layout | AECOM | 4 February, 2019 |
| 60571292-SHT-CI-00009 | B | Indicative Stage 3 Fill - Layout | AECOM | 4 February, 2019 |
| 60571292-SHT-CI-000010 | B | Indicative Stage 1 Fill - Catchments and Stormwater Drainage Plan | AECOM | 4 February, 2019 |
| 60571292-SHT-CI-000011 | B | Indicative Stage 2 Fill - Catchments and Stormwater Drainage Plan | AECOM | 4 February, 2019 |
| 60571292-SHT-CI-000012 | B | Indicative Stage 3 Fill - Catchments and Stormwater Drainage Plan | AECOM | 4 February, 2019 |
| 60571292-SHT-CI-000013 | B | Proposed Final Access Road on Proposed Final Landform | AECOM | 4 February, 2019 |
| 60571292-SHT-CI-000014 | B | Proposed Final Access Road - Long Section | AECOM | 4 February, 2019 |
| 60571292-SHT-CI-000015 | B | Post-Settlement Final Landform | AECOM | 4 February, 2019 |
| 60571292-SHT-CI-000016 | B | Post-Settlement Final Landform - Section Sheet 01 of 02 | AECOM | 4 February, 2019 |
| 60571292-SHT-CI-000017 | B | Post-Settlement Final Landform - Section Sheet 02 of 02 | AECOM | 4 February, 2019 |
| L01 | - | Landscape Plan as amended to satisfy condition 7 of this consent. | AECOM | - une, 2019 |

- General Terms of Approval issued by the Environment Protection Authority, Notice Number: 1594697, dated 11 May, 2020,

- Elizabeth Drive Landfill Expansion Technical Advice prepared by GHD, dated February, 2020,

- Bushfire Risk Assessment prepared by Australian Bushfire Consulting Services, Reference No. 18-205/1, dated 3 October, 2018.

- Noise and Vibration Technical Report prepared by AECOM, Document Status: Final, dated 12 July, 2019 and Supplimentary Noise and Vibration Impact Assessment prepared by AECOM, dated 12 November, 2019 and Supplimentary Noise Impact Assessment prepared by AECOM, dated 25 February, 2020,

- Air Quality Impact Technical Report prepared by AECOM, Document Status: Final, dated 12 July, 2019 and Supplimentary Air Quality Impact Assessment prepared by AECOM, dated 12 November, 2019 and Supplimentary Air Quality Impact Assessment prepared by AECOM, dated 25 February, 2020, and

- Greenhouse Gas Calculations prepared AECOM, Document Status: Final, dated 12 July, 2019.

2. The development shall comply at all times with the General Terms of Approval, Notice Number 1594697, dated 11 May, 2020 issued by the Environment Protection Authority.

3. The applicant must deliver a signed 'Notice of Modification' to Penrith City Council in accordance with the requirements of Clause 97 of the Environmental Planning and Assessment Regulation 2000 within twelve (12) months of the date of this determination.

The 'Notice of Modification' is to be provided in relation to DA08/0958 (as amended) and identify that:

a)	DA08/0958 is to be limited to the geographical extent of the part of the site zoned E2 Environmental Conservation under the Penrith Local Environmental Plan 2010, and

b)	The conditions provided within DA08/0958 are to be modified so as to facilitate the integration between this Development Consent and Development Consent DA19/0470.

The provision of any 'Notice of Modification' to Penrith City Council is to be accompanied by written evidence from both Penrith City Council and the Environment Protection Authority indicating satisfaction with any amended conditions of consent, plans or documents.

4.	Prior to altering the existing approved waste management and resource recovery facility, altering finished landform and increasing waste capacity at the premises, the applicant must apply to the Environment Protection Authority and be issued with a variation to the existing Environment Protection Licence No. 4068.

A copy of the amended Environment Protection Licence No. 4068 is to be provided to Penrith City Council prior to any works commencing as part of this Development Consent.

The proposal is to operate in accordance with the amended Environment Protection Licence No. 4068 issued by the Environment Protection Authority and the conditions of this development consent at all times.

5.	All quarrying and waste compaction activities at the premises must only be conducted between the following hours:

Monday to Friday: 7.00am to 6.00pm,

Saturday: 7.00am to 5.00pm, and

Sunday and Public Holidays: 8.00am to 5.00pm.

All waste receipt activities at the premises must only be conducted between the following hours:

Monday to Friday: 6.00am to 6.00pm,

Saturday: 7.00am to 5.00pm, and

Sunday and Public Holidays: 8.00am to 5.00pm.

6.	Only non-putrescible solid wastes as defined by the Waste Classification Guidelines as provided by the NSW Environment Protection Authority are to be accepted for disposal on the site.

The maximum amount of non-putrescible solid wastes which may be imported onto the site for landfilling is 950,000 tonnes per annum.

7.	Prior to the commencement of any works associated with this consent, the approved landscape plan as provided in Condition 1 is to be amended to the satisfaction of Penrith City Council to provide for the following;

a) an identification of all new plant species, number of new plants to be planted and pot size to be provided (for each new species), to the existing vegetation buffers along the northern, eastern and southern perimeter of the subject site.

8.	The development is to be carried out in three (3) stages as per approved Sheet No. 60571292-SHT-CI-00007 (Indicative Stage 1 Fill - Layout), Sheet No. 60571292-SHT-CI-00008 (Indicative Stage 2 Fill - Layout) and Sheet No. 60571292-SHT-CI-00009 (Indicative Stage 3 Fill - Layout), all prepared by AECOM, all Issue B, all dated 4 February, 2019.

9.	Landfill gas burning within the site is to remain consistent with the current capacity and the conditions of DA12/0515. Any changes to the nature or rate of gas burning may only be implemented if approved via development application. In the event of unexpected or emergency landfill gas burning (such as for safety reasons), the applicant is to report such events immediately to the Western Sydney Airport Authority and the NSW Environment Protection Agency.

10.A noise complaint phone number and email address is to be provided on the SUEZ Kemps Creek Resource Recovery Park website. The details of each noise complaint received (including the person complaining, date, time and nature of complaint) are to be recorded. The actions taken to resolve the complaint and the time taken to resolve the complaint are to be recorded. The noise complaint records are to be made available to Penrith City Council on request.

11.The Emergency Response Plan for existing operations on the subject site is to be updated so as to also specifically address bush fires and include appropriate triggers and responses to a bush fire event onsite in accordance with Planning for Bush Fire Protection 2006. Details of the updated Emergency Response Plan are to show concurrence from the NSW Rural Fire Service and be provided to Penrith City Council prior to the operation of this consent.

12.A re-assessment of the site's restoration and bush fire protection measures is to be undertaken at the time that the site ceases to operate as an active landfill. Written evidence is to be provided from the Rural Fire Service identifying that any document prepared is in accordance with Planning for Bush Fire Protection 2006 (or the relevant document at the time) and a copy provided to Penrith City Council at the time that the site ceases to operate.

13.Upon request, the applicant is to provide, or arrange provision of, any information required by, or on behalf of Council, in relation to compliance or otherwise with any conditions of this consent. Information should be furnished within 2 weeks of any request except as agreed otherwise by Council.

14.The applicant shall indemnify and keep indemnified the Council from and against all damages, claims, demands, proceedings, costs and expenses which arise from any aspect of the site operations.

15.The applicant is to provide an annual report to Council in relation to compliance or otherwise with any of the conditions of this consent. This report must also indicate the response of the applicant and/or operator to any emergencies, incidents, etc.

16.A separate development approval shall be obtained for the erection of any signage, other than signage listed as exempt development.

17.No retail sale of any materials extracted, produced or recycled on the site shall be conducted from the subject premises.

18.The proponent shall ensure that lighting associated with the project:

a) complies with the latest version of Australian Standard AS 4282 (INT) - Control of Obrusive Effects of Outdoor Lighting; and

b) is mounted, screened and directed in such a manner that it does not create a nuisance to surrounding properties or the surrounding public road network.

19.All conditions of this consent shall be complied with throughout the lifetime of the site operations and until such time as a Statement of Completion is issued for the site by the Environment Protection Authority as provided by Environment Protection Licence No. 4068 (as amended).

20.Council staff or agents of Council may enter the site at all reasonable times to inspect the works and any other operational aspects as necessary.

Environmental Matters

21.Mud and soil from vehicular movements to and from the site must not be deposited on the road.

22.The Environment Protection Authority has the right to impose any additional control or treatment measures as may, in its opinion, become necessary upon the area of site operations.

23.All unsealed roads on or serving the development shall be dampened by a water cart to prevent dust generation. A water cart shall be available for on-site use at all times.

24.Radiator fan backwash and engine exhaust gases from all mobile equipment on the site must be directed away from the ground.

25.All runoff, including that from truck washdown facilities (and sediment) from within the extraction area, must be directed towards on-site sediment basins.

26.All water needed for dust suppression, irrigation, fire fighting, etc must be collected from runoff from the site or sources satisfactory to Council. Sufficient water must be available for these purposes. Water is not to be pumped from or discharged into Badgerys Creek. All water needed for dust suppression, irrigation, fire fighting, etc must be collected from runoff from the site or sources satisfactory to Council. Sufficient water must be available for these purposes. Water is not to be pumped from Badgerys Creek. Any excess surface water within site, including runoff, is to be directed towards on-site sediment basins.

27.The facility must not accept waste or recyclable materials from members of the general public.

28.All access gates are to be kept fully attended whilst the facility is operating and locked when the facility is not operating.

29.Burning of combustible material is not to be permitted on the site.

30.Operations are to comply with the NSW Smoke-free Environment Act 2000 and Smoke-free Environment Regulation 2000. Smoking is to be prohibited in proximity to any fuel storage and equipment maintenance facilities.

31.The following dust mitigation measures are to be actioned in the operation of the landfill at all times;

- Sealing is to be provided to waste delivery haul routes. Sealed roads are to be progressively laid moving onto the landfill towards the tipping face. Only the final 50m of the haul/turnaround area is to remain unsealed.

- Twice daily cleaning of sealed roads is to be provided using high-pressure water spray from on-site water carts.

- Shale covers are to be placed on non-active areas of the general and restricted waste cells.

- Tarps are to be used at night on the general waste cell batters for the life of the project.

- Hydromulch or similar products with stabilising additives are to be used on disused shale stockpiles.

32.A noise mitigation action plan is to be prepared within six (6) months of the date of this consent by a suitably qualified person to address future noise non-compliances or complaints as provided by Section 4.0 'Feasible and reasonable management of impacts', Supplementary Noise and Vibration Impact Assessment prepared by AECOM, dated 12 November, 2019. The plan is to be to the satisfaction of Penrith City Council.

BCA Issues

33.Access and sanitary facilities for persons with disabilities are to be provided and maintained in accordance with the requirements of the Building Code of Australia and Australian Standard AS 1428.1-2009 "Design for Access and Mobility". Details of compliance are to be provided to Penrith City Council prior to the commencement of works associated with this consent.

Engineering

34.All roadworks, stormwater drainage works, associated civil works and dedications, required to effect the consented development shall be undertaken at no cost to Penrith City Council.

35.An Infrastructure Restoration Bond is to be lodged with Penrith City Council for development involving works around Penrith City Council's Public Infrastructure Assets. The bond is to be lodged with Penrith City Council prior to commencement of any works on site or prior to the issue of any Construction Certificate, whichever occurs first. The bond and applicable fees are in accordance with Council’s adopted Fees and Charges.

An application form together with an information sheet and conditions are available on Council’s website.

Contact Penrith City Council's City Works Department on (02) 4732 7777 or visit Penrith City Council’s website for more information.

36.The stormwater management system shall be provided generally in accordance with the concept plans lodged for development approval, prepared by AECOM, sheet number 60571292-SHT-CI-00007 to sheet number 60571292-SHT-CI-000013, all revision B, all dated 4 February, 2019.

Engineering plans and supporting calculations for the stormwater management system are to be prepared by a suitably qualified person.

37.All vehicular access, circulation, manoeuvring and pedestrian and parking areas associated with the subject development must be in accordance with AS 2890.1, AS 2890.2, AS 2890.6 and Penrith City Council’s Development Control Plan.

38.Sediment and erosion control measures shall be installed in accordance with the approved plans and documents and are to ensure compliance with the Protection of the Environment Operations Act 1997.

The erosion and sediment control measures shall remain in place and be maintained until all disturbed areas have been rehabilitated and stabilised.

39.Deleted.

40.Total vehicle movements are not to exceed 640 per day.

Landscaping

41,All landscape works are to be constructed in accordance with the stamped approved plans and as amended by the conditions of this consent.

Landscaping shall be maintained:

- in accordance with the approved plans, and

- in a healthy state, and in perpetuity by the existing or future owners and occupiers of the property.

If any of the vegetation comprising that landscaping dies or is removed, it is to be replaced with vegetation of the same species and, to the greatest extent practicable, the same maturity as the vegetation which died or was removed.

42.All trees that are required to be retained as part of the development are to be protected in accordance with the minimum tree protection standards prescribed in Section C6 'Landscape Design' of Penrith Development Control Plan 2014.

43.All trees and landscaping along the perimeter of the site must be maintained to enable its growth to full maturity in healthy growing conditions. This includes mature height, spread and form, consistent with the tree species. Pruning must not alter the natural form and height of any trees unless required to maintain normal operations such as access to roads and other operational facilities within the site.

44.No trees are to be removed, ringbarked, cut, topped or lopped or willfully destroyed along the perimeter of the landfill area without the prior consent of Penrith City Council and in accordance with Council's Tree Preservation Order and Policy.