ATTACHMENT 3 – RECOMMENDED CONDITIONS OF CONSENT

A.	THE DEVELOPMENT

STAGED APPROVAL

The conditions referenced in the below table apply to each stage of the development as identified.

| Stage | Development works | Part Reference | Condition Reference |
| --- | --- | --- | --- |
| 1 | Demolition of the single storey brick administration building and associated structures within the southern portion of the site and construction of 4 large warehouse over two levels with ancillary office floor area. Retention of the existing warehouse within the northern portion of the site to operate concurrently with Stage 1 of the development with 159 temporary car parking spaces. | All Parts | All conditions |
| 2 | Demolition of the existing warehouse and associated structures within the northern portion of the site and Construction of 4 warehouse over two levels with ancillary office floor area and basement carpark with a total of 438 vehicular parking spaces including 4 accessible car parking spaces, 20 motorbike parking spaces and 36 bicycle parking spaces. | All Parts | All conditions |

Approved Plans

- Development the subject of this determination notice must be carried out strictly in accordance with the following approved plans/reports, except where modified by the undermentioned conditions.

| Plan Name | Plan Number | Date | Revision | Prepared By |
| --- | --- | --- | --- | --- |
| Cover Sheet | DA000 | 5/12/2019 | A | Bureau SRH Architecture |
| Site Analysis | DA001 | 5/12/2019 | A | Bureau SRH Architecture |
| Stage 1 – Site Plan | DA002 | 5/12/2019 | A | Bureau SRH Architecture |
| Stage 2 – Site Plan | DA003 | 5/12/2019 | A | Bureau SRH Architecture |
| Stage 1 – Demolition Plan | DA004 | 5/12/2019 | A | Bureau SRH Architecture |
| Construction Management Plan | DA005 | 5/12/2019 | A | Bureau SRH Architecture |
| Stage 2 – Demolition Plan | DA006 | 5/12/2019 | A | Bureau SRH Architecture |
| Key Plan – Stage 1 Ground | DA100 | 17/6/2020 | B | Bureau SRH Architecture |
| Key Plan – Stage 1 Ground Mezzanine | DA101 | 17/6/2020 | B | Bureau SRH Architecture |
| Key Plan – Stage 1 Level 1 | DA102 | 17/6/2020 | B | Bureau SRH Architecture |
| Key Plan – Stage 1 Level 1 Mezzanine | DA103 | 5/12/2019 | A | Bureau SRH Architecture |
| Key Plan – Stage 1 Level 1 Mezzanine 2 | DA104 | 5/12/2019 | A | Bureau SRH Architecture |
| Key Plan – Stage 1 Roof | DA105 | 5/12/2019 | A | Bureau SRH Architecture |
| Key Plan – Stage 2 Basement | DA106 | 17/6/2020 | B | Bureau SRH Architecture |
| Key Plan – Stage 2 Ground | DA107 | 17/6/2020 | B | Bureau SRH Architecture |
| Key Plan – Stage 2 Ground Mezzanine | DA108 | 17/6/2020 | B | Bureau SRH Architecture |
| Key Plan – Stage 2 Level 1 | DA109 | 17/6/2020 | B | Bureau SRH Architecture |
| Key Plan – Stage 2 Level 1 Mezzanine | DA110 | 5/12/2019 | A | Bureau SRH Architecture |
| Key Plan – Stage 2 Level 1 Mezzanine 2 | DA111 | 5/12/2019 | A | Bureau SRH Architecture |
| Key Plan – Stage 2 Roof | DA112 | 5/12/2019 | A | Bureau SRH Architecture |
| Stage 1 – Elevations | DA200 | 17/6/2020 | B | Bureau SRH Architecture |
| Stage 2 – Elevations | DA201 | 17/6/2020 | B | Bureau SRH Architecture |
| Stage 1 – Sections | DA300 | 17/6/2020 | B | Bureau SRH Architecture |
| Stage 2 – Sections | DA301 | 17/6/2020 | B | Bureau SRH Architecture |
| 3D Perspectives 1 | DA600 | 5/12/2019 | A | Bureau SRH Architecture |
| 3D Perspectives 2 | DA601 | 5/12/2019 | A | Bureau SRH Architecture |
| Materials & Finishes Schedule | DA602 | 5/12/2019 | A | Bureau SRH Architecture |
| Landscape Plans and Details | SK01-02 | 12/2019 | F | Carmichael Studios |
| Demolition Statement | -- | -- | -- | Bureau SRH Architecture |
| Road Works and Stormwater Drainage Plan Sheet 5 | 19-648-C045 | 28/11/19 | -- | AT & LP/L |
| Road Works and Stormwater Drainage Plan Sheet 6 | 19-648-C045 | 28/11/19 | -- | AT & LP/L |

| Report Name | Date | Reference | Prepared By |
| --- | --- | --- | --- |
| BCA Report | 22/11/19 | NW19/4620 | NW Building Certification |
| Building Services Report | 22/11/19 | SYD0923-2 | ADP Consulting P/L |
| Civil Stormwater Management Report | 12/19 | 19-683-FAVCO | AT & LP/L |
| Preliminary Site Investigation Report | 11/19 | P1907209 JR01_1 | Martens Consulting Engineers |
| Detailed Site Investigation Report | 11/19 | P1907209 JR03_1 | Martens Consulting Engineers |
| Remediation Action Plan | 12/19 | P1907209 JR04_1 | Martens Consulting Engineers |
| ESD Strategy Report | 25/11/19 | SYD0923-02 | ADP Consulting P/L |
| Fire Services Report | 25/11/19 | 19203-L01 | Innova Services P/L |
| Noise Impact Assessment Report | 17/06/19 | SYD0923-04 | ADP Consulting P/L |
| Preliminary Geotechnical Report | 11/19 | P1907209 JR02-02 | Martens Consulting Engineers |
| Waste Management Plan | 11/19 | 1.1 | Auswide Consulting |

Works at no Cost to Council

- All roadworks, drainage works and dedications, required to effect the consented development shall be undertaken at no cost to Liverpool City Council.

Sydney Water

- All comments provided by Sydney Water shall be complied with prior, during, and at the completion of construction. A copy of the Sydney Water comments are attached to this decision notice.

Transport for NSW (TfNSW)

- All comments provided by TfNSW shall be complied with prior, during, and at the completion of construction. A copy of the TfNSW comments are attached to this decision notice.

Comply with EP & A Act

- The requirements and provisions of the Environmental Planning & Assessment Act 1979 and Environmental Planning & Assessment Regulation 2000, must be fully complied with at all times.

Failure to comply with these legislative requirements is an offence and may result in the commencement of legal proceedings, issuing of ‘on-the-spot’ penalty infringements or service of a notice and order by Council.

National Code Construction

- In accordance with Section 4.16(11) of the Environmental Planning & Assessment Act 1979 and clause 98 of the Environmental Planning & Assessment Regulation 2000, it is a prescribed condition that all building work must be carried out in accordance with the applicable Performance Requirements of the National Construction Code. Compliance with the Performance Requirements can only be achieved by:

  - Complying with the Deemed to Satisfy Provisions; or

  - Formulating an Alternative Solution, which complies with the Performance Requirements or is shown to be at least equivalent to the Deemed to Satisfy Provision, or a combination of (a) and (b).

B.	PRIOR TO ISSUE OF A CONSTRUCTION CERTIFICATE

The following conditions shall be complied with prior to issue of a CC by the PCA:

Fee Payments

- Unless otherwise prescribed by this consent, all relevant fees or charges must be paid. Where Council does not collect these payments, copies of receipts must be provided. For the calculation of payments such as Long Service Levy, the payment must be based on the value specified with the DA/CC. The following fees are applicable:

- Damage Inspection Fee;

- Fee associated with Application for Permit to Carry Out Work Within a Road, Park and Drainage Reserve; and

- Long Service Levy – based on 0.35% of the cost of building work.

These fees are reviewed annually and will be calculated accordingly

Long Service Levy payment is applicable on building work having a value of $25,000 or more, at the rate of 0.35% of the cost of the works.  The required Long Service Levy payment, under the Building and Construction Industry Long Service Payments Act 1986, is to be forwarded to the Long Service Levy Corporation or the Council, prior to the issuing of a Construction Certificate, in accordance with Section 109F of the Environmental Planning & Assessment Act 1979.

Road Opening

- All fees associated with a Road Opening permit required for the connection, extension or amplification of any services within Council’s road reserve must be paid to Council and receipts provided to the Principal Certifying Authority. A separate form must be submitted in conjunction with payment of the fees. The fees include the standard Road Opening permit fee and any restoration fees that may be required as a result of the works.

Site Development Work

- Site development work in the form of excavation, underpinning or shoring works must not take place, until such time as a Construction Certificate has been issued.

- Building work shall not commence prior to the issue of a Construction Certificate. Building work as defined under Section 1.4 of the EP&A Act means any physical activity involved in the erection of a building and includes but is not limited to, the placement of any site shed/s or builders facilities, site grading, retaining walls, excavation, cutting trenches, installing formwork and steel reinforcement or, placing of plumbing lines.

Access for people with a disability

- Access must be provided to the building for people with a disability in accordance with the relevant requirements of the Building Code of Australia, Disability (access to Premises – Buildings) Standard 2010 and Australian Standard – AS1428.1 (2009), Design for Access and Mobility – General requirements for new building work, to the satisfaction of the Certifying Authority.

Recommendations of Acoustic Report

- The recommendations provided in the approved acoustic report titled: Noise Impact Assessment (Ref. SYD0923, Rev. 04) prepared by ADP Consulting Pty Ltd dated 17 June 2020 shall be implemented and incorporated into the design and construction of the development and be shown on plans accompanying the Construction Certificate application.

A suitably qualified acoustic consultant is to be appointed to assess and provide recommendations in relation to final plant and equipment selection to ensure compliance with the aforementioned acoustic report.

The construction methodology and plans accompanying the Construction Certificate application shall be assessed and certified in writing by a suitably qualified acoustic consultant to verify conformance with the requirements of the aforementioned acoustic report. The written certification from the suitably qualified acoustic consultant shall be submitted to and approved by the Principal Certifying Authority (PCA) prior to issue of the Construction Certificate.

Note: ‘Suitably qualified acoustic consultant’ means a consultant who possesses Australian Acoustical Society membership or are employed by an Association of Australasian Acoustical Consultants (AAAC) member firm.

Mechanical Plant and Equipment

- Mechanical plant and equipment shall be selected in consultation with a suitably qualified acoustic consultant in accordance with the recommendations of the approved acoustic report Acoustic report titled Noise Impact Assessment (Ref. SYD0923, Rev. 04) prepared by ADP Consulting Pty Ltd dated 17 June 2020.

Note: ‘Suitably qualified acoustic consultant’ means a consultant who possesses the qualifications to render them eligible for membership of the Australian Acoustical Society, Institution of Engineers Australia or the Association of Australasian Acoustical Consultants (AAAC) at the grade of member’.

Cladding

- Prior to issue of a construction certificate the certifier must be satisfied that all proposed attachments, cladding material and systems forming part of external walls comply with the NCC BCA and relevant Australian Standards.  The certifier must be able to demonstrate compliance with evidence of suitability as per clause A2.2 of the BCA for all products/systems proposed.

Construction Environmental Management Plan (CEMP)

- Prior to issue of a construction certificate, a Construction Environmental Management Plan (CEMP) for the development must be provided to the Principal Certifying Authority for approval. The environmental site management measures must remain in place and be maintained throughout the period of the development. The CEMP must address all environmental aspects of the development’s construction phases, and include (where relevant), but not be limited to, the following:

  - Asbestos Management Plan;

(b)	Project Contact Information;

(c)	Site Security Details;

(d)	Timing and Sequencing Information;

(e)	Site Soil and Water Management Plan;

(f) 	Noise and Vibration Control Plan;

(g) 	Dust Control Plan;

(h) 	Air Monitoring;

(i) 	Odour Control Plan;

(j) 	Health and Safety Plan;

(k) 	Waste Management Plan;

(l) 	Incident management Contingency; and

(m) 	Unexpected Finds Protocol.

The CEMP must be kept on site for the duration of the works and must be made available to Council Officers upon request.

Construction Noise, Vibration Assessment and Management Plan

- A site-specific Construction Noise, Vibration Assessment and Management Plan prepared by a suitably qualified acoustic consultant shall be submitted to and approved by the Principal Certifying Authority (PCA) prior to issue of the Construction Certificate. The Construction Noise, Vibration Assessment and Management Plan must include an assessment of expected noise impacts and detail feasible work practices to be adopted to avoid, remedy or mitigate construction noise and vibration impacts.

The Construction Noise, Vibration Assessment and Management Plan shall be consistent with the ‘Interim Construction Noise Guideline’ published by the Department of Environment and Climate Change NSW (DECC 2009/265) dated July 2009 and include, but not necessarily be limited to the following information:

•	Identification of nearby residences and other noise sensitive land uses;

•	Assessment of expected noise impacts;

•	Detailed examination of feasible and reasonable work practices that will be 	implemented to minimise noise impacts;

•	Strategies to promptly deal with and address noise complaints;

•		Details of performance evaluating procedures (for example, noise monitoring or checking work practices and equipment);

•	Methods for receiving and responding to complaints about construction noise;

•		Procedures for notifying nearby residents of forthcoming works that are likely to produce noise impacts; and

•	Reference to relevant licence and consent conditions.

The Construction Noise, Vibration Assessment and Management Plan must be made available for inspection when requested by the PCA or Liverpool City Council.

Substation (General)

- Should the Electrical Substation be located outside the building envelope, the location and any associated fire separation walls must comply with Integral Energy Substation Design Instruction Document No SDI 104 (Current Version).

Notification

- The certifying authority must advise Council, in writing of:

- The name and contractor licence number of the licensee who has contracted to do or intends to do the work, or

- The name and permit of the owner-builder who intends to do the work.

If these arrangements are changed, or if a contact is entered into for the work to be done by a different licensee, Council must be immediately informed.

Crime Prevention Through Environmental Design

- The following Crime Prevention Through Environmental Design principles are to be incorporated into the building.

- Back to base alarm systems shall be installed;

- Basement parking areas shall be painted a light colour;

- CCTV for the ground level, entry/exit points, car parks, lifts and the exterior of the building shall be implemented;

- ‘Way finding’ signage should be utilised at all major interchanges such as lifts and stair wells;

- Lighting is required to be designed in accordance with the Australian and New Zealand Lighting Standard AS 1158.  A lighting maintenance policy should be established.  Security lighting should be installed in and around the building, and such shall not impact on any adjoining premises.  The lighting should be vandal resistant, especially external lighting; and

- Access to the parking levels of the building shall be controlled via a security controlled device.

Regulated Systems

- Regulated systems installed, such as air-handling systems, heated water systems and cooling water systems, must comply with the Public Health Act and Public Health (Microbial Control) Regulation thereunder, including AS3666.1:2011 & AS1668.

The PCA is to ensure a Liverpool City Council approved registration form is completed and 	submitted to the Council with any relevant fee for the system.

Security Access to car park

- Secure access is to be provided to the basement car park to prevent any unauthorised entry. Details are to be provided with the Construction Certificate.

S138 Roads Act – Minor Works in the public road

- Prior to the issue of a Construction Certificate a S138 Roads Act application/s, including payment of fees shall be lodged with Liverpool City Council, as the Roads Authority for any works required in a public road.  These works may include but are not limited to the following:

- Vehicular crossings (including kerb reinstatement of redundant vehicular crossings)

- Road opening for utilities and stormwater (including stormwater connection to Council infrastructure)

- Road occupancy or road closures

All works shall be carried out in accordance with the Roads Act approval, the development consent including the stamped approved plans, and Liverpool City Council’s specifications.

Note:	Approvals may also be required from the Roads and Maritime Service (RMS) for classified roads.

S138 Roads Act – Roadworks requiring approval of civil drawings

- Prior to the issue of a Construction Certificate for building or subdivision works the Certifying Authority shall ensure that a S138 Roads Act application, including the payment of application and inspection fees, has been lodged with Liverpool City Council (being the Roads Authority under the Roads Act), for all works associated with the development in Yarrunga Street (half width road pavement 6.5m wide and street tree planting).

Engineering plans are to be prepared in accordance with the development consent, Liverpool City Council’s Design Guidelines and Construction Specification for Civil Works, Austroad Guidelines and best engineering practice.

Note:

- Where Liverpool City Council is the Certifying Authority for the development the Roads Act approval for the above works may be issued concurrently with the Construction Certificate.

- Approvals may also be required from the Roads and Maritime Service (RMS) for classified roads.

Retaining Walls on Boundary

- All retaining walls must be wholly within the property boundary, including footings and agricultural drainage lines.  Construction of retaining walls or associated drainage works along common boundaries shall not compromise the structural integrity of any existing structures.

Where a retaining wall exceeds 600mm in height, the wall shall be designed by a practicing structural engineer and a construction certificate must be obtained prior to commencement of works on the retaining wall.

Dilapidation Report Private Property (Excavations)

- A full dilapidation survey and report on the visible and structural condition of all neighbouring structures within the ‘zone of influence’ of the required excavations must be submitted to the Certifying Authority for approval prior to the issue of any Construction Certificate. The zone of influence is to be defined as the horizontal distance from the edge of the excavation face to twice the excavation depth.

The dilapidation report and survey is to be prepared by a consulting structural/geotechnical engineer agreed to by both the applicant and the owner of any affected adjoining property.

All costs incurred in achieving compliance with this condition shall be borne by the person entitled to act on this Consent.

In the event that access for undertaking the dilapidation survey is denied by an adjoining owner, the applicant MUST DEMONSTRATE, in writing, to the satisfaction of Council that all reasonable steps have been taken to obtain access and advise the affected property owner of the reason for the survey and that these steps have failed. Written concurrence must be obtained from Council in such circumstances.

Note: This documentation is for record keeping purposes only, and may be used by the developer or affected property owner to assist in any action required to resolve any dispute over damage to adjoining properties arising from the works. It is in the applicant’s and adjoining owner’s interest for it to be as full and detailed as possible.

Land Engineering

- Prior to the issue of a Construction Certificate for building works the Certifying Authority shall ensure that engineering plans are consistent with the stamped approved concept plan/s prepared by AT and I, reference number 19-648-C045, revision B, C, D & E, dated 28/11/19 and that all works have been designed in accordance with conditions of this consent, Liverpool City Council’s Design Guidelines and Construction Specification for Civil Works, any Roads Act approval issued, Austroad Guidelines and best engineering practice.

The works may include but are not limited to the following:

(a)	Public and private road

(b)	Stormwater drainage including water quantity and quality treatment measures

(c)	Interallotment drainage

(d)	Private access driveways

(e)	Sediment and erosion control measures

(f)	Overland flowpaths

(g) 	Flood control measures

(h) 	Traffic facilities including roundabouts, intersection treatments, car parks, bus stops, cycleways, pathways etc.

(i) 	Earthworks

(j) 	Bridges, culverts, retaining walls and other structures

(k) 	Landscaping and embellishment works

(l) 	All works required for conversion of the proposed sediment basin to a bio retention function

(m) All works required for the decommissioning temporary OSD systems including pipe removal, basin filling  and works to existing pit structures if required

The Construction Certificate must be supported by engineering plans, calculations, specifications and any certification relied upon.

Road Design Criteria

- Prior to the issue of a Construction Certificate the Certifying Authority shall ensure that the proposed roads have been designed in accordance with Liverpool City Council’s Design Guidelines and Construction Specification for Civil Works and the following criteria:

| Road No. | Road Reserve Width | Carriageway Width | Verge | Footpath (1.5m wide) | ESA |
| --- | --- | --- | --- | --- | --- |
| Yarrunga Street | 20.115m | 5.5m -6.5m (half road width) | 3.5m - 4.5m | 1.5m | 1 x 107 |

- The applicant is to provide ‘No Stopping’ parking restrictions along the Yarrunga Street frontage following. Approval is required to be obtained for these parking restrictions from Council’s Local Traffic Committee, prior to the issue of any CC. Signage is to be installed prior to any OC.

Stormwater Concept Plan

- A stormwater drainage system shall be provided generally in accordance with the concept plan/s lodged for development approval, prepared by AT & I Pty Ltd, reference number 19-648-C045, revision B, C, D & E, dated 28/11/2019.

The proposed development and stormwater drainage system shall be designed to ensure no adverse impact on adjoining properties by the diversion, damming or concentration of stormwater flows.

The proposed method of stormwater discharge shall be detailed in the Construction Certificate issued by the Certifying Authority. Prior to the issue of a Construction Certificate the Certifying Authority shall ensure that the stormwater drainage system has been designed in accordance with Liverpool City Council’s Design Guidelines and Construction Specification for Civil Works.

Stormwater Discharge – Basement Car Parks

- Prior to the issue of a Construction Certificate the Certifying Authority shall ensure that the stormwater drainage system for the basement car park has been designed in accordance with the requirements for pumped systems in AS3500.3:2003 and Council’s Stormwater Drainage Design Specifications for pump out systems for basement carparks.

On-Site Detention

- On-Site Detention shall be provided generally in accordance with the concept plan/s lodged for development approval, prepared by AT & I, reference number 19-648-C045, revision B, C, D & E, dated 28/11/2019.

The proposed development and stormwater drainage system shall be designed to ensure that stormwater runoff from upstream properties is conveyed through the site without adverse impact on the development or adjoining properties.

Engineering plans and supporting calculations for the OSD system are to be prepared by a suitably qualified person and shall accompany the application for a Construction Certificate.

Prior to the issue of a Construction Certificate the Certifying Authority shall ensure that the on-site detention system has been designed in accordance with Liverpool City Council’s Design Guidelines and Liverpool City Council’s OSD policy and Technical Specification.

Performance Bond

- Prior to the issue of any Construction Certificate, a performance bond is to be lodged with Liverpool City Council for half width road and drainage construction in Yarrunga Street.

The value of the bond shall be determined and administered in accordance with Liverpool City Council’s Bond Policy.

Note: Contact Council’s Land Development Section for further information relating to bond requirements.

Dilapidation Report

- Prior to the Commencement of Works a dilapidation report of all infrastructure fronting the development in Yarrunga Street is to be submitted to Liverpool City Council.  The report is to include, but not limited to, the road pavement, kerb and gutter, footpath, services and street trees and is to extend 40m either side of the development.

No loading on easements

- Prior to the issue of a Construction Certificate the Certifying Authority shall ensure that the foundations of proposed structures adjoining the drainage and/ or services easement have been designed clear of the zone of influence.

Water Quality

- Prior to the issue of a Construction Certificate the PCA shall ensure that details of a stormwater pre-treatment system have been provided on the stormwater plans and that the design meets pollutant retention criteria in accordance with the LDCP 2008. The CC must be supported by:

- Specification & installation details of the stormwater pre-treatment system

- The approval of an operation and maintenance manual/schedule for the stormwater pre-treatment system

A copy of the approved operation and maintenance manual/ schedule shall be submitted to Council with notification of the CC issue.

Access and Manoeuvring

- Prior to the issue of a Construction Certificate the Certifying Authority shall ensure that:

a)	Off street access and parking complies with AS2890.1.

b)	Sight Distance at the street frontage has been provided in accordance with AS 2890.1.

c)	All cars can enter and exit the site in a forward direction

d)	Arrangements shall be made with the relevant authority for the relocation of the power pole adjoining the proposed driveway/vehicular crossing. Alternatively, written acceptance with regard to the proposed location of the vehicular crossing and the existing asset, and/or any alternative arrangements, shall be obtained and provided to the Certifying Authority.

- Prior to the issue of a Construction Certificate, detailed design plans for the access driveway and car park including swept path analysis, line markings and sign posting in accordance with the DCP and AS2890 should be submitted to Council’s Traffic and Transport Branch for review and approval.

The design should demonstrate that all expected vehicles will be accommodated, including B-Doubles. This needs to be accompanied by a certification from a qualified professional that the design of the car parks, driveways, ramps and swept path comply with the relevant Australian Standards and Liverpool City Council’s Development Control Plan.

Detailed design of road reconstruction along the section of Yarrunga Street in front of the development site, including path paving shall also be submitted to Council for review and approval.

Prior to the issue of a Construction Certificate the Certifying Authority shall ensure that vehicular access, circulation, manoeuvring, pedestrian and parking areas associated with the subject development as well as road reconstruction along Yarrunga Street are in accordance with the plans approved by Council as required by this condition.

- Street light design is to be submitted to Council for street light category and endorsement for submission to Endeavor Energy for approval.

Construction Traffic Management Plan

- A Construction Traffic Management Plan (CTMP) is to be submitted to Liverpool City Council’s Traffic & Transport Section for review and endorsement. The CTMP is to be prepared by a qualified project manager/engineer.  The construction activities are not to commence prior to Council endorsing the CTMP.

All construction vehicles must enter and exit in forward direction, and no construction materials are to be stored on affected road and footpath reserves.

The construction sites must install shakers to prevent mud spilling from the construction sites being transported onto the road network.  Any spillage on the public road network must be cleaned as soon as possible.

Street Tree Planting

- Detailed plans are required to be submitted to and approved by Council’s Manager of Development Assessment showing the proposed street tree locations, species and planting sizes, paving location and layout, including references to the relevant details and specifications.

To ensure that the street tree planting size, quantity and quality is maintained throughout the Liverpool CBD, please contact Council’s Land Development Section on 1300 36 2170 for further information.

Prior to the issue of a Construction Certificate the Certifying Authority shall ensure that street tree planting associated with road reconstruction along Yarrunga Street are in accordance with the detailed plans approved by Council as required by this condition.

Note: A Section 138 Roads Act Approval for all works within Council’s road reserve will be required.

Bus Stops

- Prior to the issue of a Construction Certificate the Certifying Authority shall ensure that all bus stops have been designed in accordance with the requirements of the Disability Discrimination Act 2002, Disability Standards for Accessible Public Transport and the Guidelines for assessing compliance of bus stops with the Disability Standards for Accessible Public Transport 2002.

Provision of Services

- An application to obtain a Section 73 Compliance Certificate under the Sydney Water Act 1994, must be lodged with Sydney Water.  To facilitate this, an application must be made through an authorised Water Servicing Coordinator.  Please refer to the “building and developing” section of Sydney Water’s web site at www.sydneywater.com.au, or telephone 13 20 92.

Following receipt of the application, a ‘Notice of Requirements’ will detail water and sewer extensions to be built and charges to be paid. Please make early contact with the Coordinator, since building of water/sewer extensions can be time consuming and may impact on other services and building, driveway or landscape design. A copy of the ‘Notice of Requirements’ must be submitted to the PCA.

- Written clearance from the Electricity provider, stating that electrical services have been made available to the development or that arrangements have been entered into for the provision of services to the development must be submitted to the PCA.

- Should the Electrical Substation be located outside the building envelope, the location and any associated fire separation walls must comply with Integral Energy Substation Design Instruction Document No SDI 104 (Current Version). The colours and materials used in the construction of any wall of structure are to match those of the approved development.

- Prior to the issue of a Construction Certificate, the Principal Certifying Authority shall be satisfied that telecommunications infrastructure may be installed to service the premises which complies with the requirements of the Telecommunications Act 1997:

  - For a fibre ready facility, the NBN Co’s standard specifications current at the time of installation; and

  - For a line that is to connect a lot to telecommunications infrastructure external to the premises, the line shall be located underground.

Unless otherwise stipulated by telecommunications legislation at the time of construction, the development must be provided with all necessary pits and pipes, and conduits to accommodate the future connection of optic fibre technology telecommunications.

C.	PRIOR TO WORKS COMMENCING

The following conditions shall be complied with prior to works commencing on the subject site:

Commencement of Building Works

- Building work shall not commence prior to the issue of a Construction Certificate. Building work as defined under Section 1.4 of the EP&A Act means any physical activity involved in the erection of a building and includes but is not limited to, the placement of any site shed/s or builders facilities, site grading, retaining walls, excavation, cutting trenches, installing formwork and steel reinforcement or, placing of plumbing lines.

Construction Certificates

- Prior to the commencement of any building works, the following requirements must be complied with:

- Construction Certificate must be obtained from the Council or an accredited certifier, in accordance with the provisions of the Environmental Planning & Assessment Act 1979.

- Where a Construction Certificate is obtained from an Accredited Certifier, the applicant shall advise Council of the name, address and contact number of the Accredited Certifier, in accordance with Section 81A (4) of the Act.

- A copy of the Construction Certificate, the approved development consent plans and consent conditions must be kept on the site at all times and be made available to the Council officers and all building contractors for assessment.

- A Principal Certifying Authority (PCA) must be appointed to carry out the necessary building inspections and to issue an occupation certificate; and

- The PCA must advise Council of the intended date to commence work which is the subject of this consent by completing a notice of commencement of building works or subdivision works form, available from Council’s Customer Service Centre.  A minimum period of two (2) working days’ notice must be given.

Notification/Principal Certifying Authority

- Written notice of intention shall be given to the owners or the adjoining allotments of land, outlining the particulars of the work, which involves:

- Any excavation below the base of the footings of a building on an adjoining allotment of land.

- The notice shall be given seven (7) days prior to the commencement of work.

Site Notice Board

- A sign must be erected in a prominent position on the premises on which work is to be carried out.  The sign is to be maintained during work, and removed at the completion of work.  The sign must state:

- The name, address and telephone number of the principal certifying authority for

the work;

- The name of the principal contractor (if any) for any building work and a telephone number on which that person may be contacted outside working hours; and

- Unauthorised entry to the premises is prohibited.

Sediment and Erosion Control Measures

- Prior to commencement of works sediment and erosion control measures shall be installed in accordance with the approved Construction Certificate and to ensure compliance with the  Protection of the Environment Operations Act 1997 and Landcom’s publication “Managing Urban Stormwater – Soils and Construction (2004)” – also known as “The Blue Book”.

The erosion and sediment control measures shall remain in place and be maintained until all disturbed areas have been rehabilitated and stabilised.

Demolition Works

- Demolition works shall be carried out in accordance with the following:

(a)	Prior to the commencement of any works on the land, a detailed demolition work plan designed in accordance with the Australian Standard AS 2601-2001 – The Demolition of Structures, prepared by a suitably qualified person with suitable expertise or experience, shall be submitted to and approved by Council and shall include the identification of any hazardous materials, method of demolition, precautions to be employed to minimise any dust nuisance and the disposal methods for hazardous materials.

(b) 	Prior to commencement of any works on the land, the demolition Contractor(s) licence details must be provided to Council.

(c) 	The handling or removal of any asbestos product from the building/site must be carried out by a NSW Work Cover licensed contractor irrespective of the size or nature of the works. Under no circumstances shall any asbestos on site be handled or removed by a non-licensed person. The licensed contractor shall carry out all works in accordance with NSW Work Cover requirements.

Waste Classification and Disposal of Contaminated Soil and Material

- All soils and material(s), liquid and solid, to be removed from the site must be analysed and classified by an appropriately qualified and certified consultant, in accordance with the Protection of the Environment Operations (Waste) Regulation 2014 and related guidelines, in particular the NSW EPA Waste Classification Guidelines, prior to off-site disposal.

All Waste material(s) must be disposed of at an appropriately licensed waste facility for the specific waste. Receipts for the disposal of the waste must be submitted to the Principal Certifying Authority within 30 days of the waste being disposed of.

All waste must be transported by a contractor licenced to transport the specific waste, and in vehicles capable of carting the waste without spillage, and meeting relevant requirements and standards. All loads must be covered prior to vehicles leaving the site.

Traffic Management Plan

- No work or craning shall be undertaken within the adjoining public lands without the prior written consent of Council. In this regard Council may require a Traffic Management Plan to be submitted before giving its approval.

- The developer shall seek road occupancy, road opening permits, and works zone approval from Council if required prior to undertaking any works within public road reserve. The application forms are available on Council’s website or can requested from the Council’s Customer Services.

Traffic Control Plan

- Prior to commencement of works a Traffic Control Plan including details for pedestrian management, shall be prepared in accordance with AS1742.3 “Traffic Control Devices for Works on Roads” and the Roads and Traffic Authority’s publication “Traffic Control at Worksites” and certified by an appropriately accredited Roads and Traffic Authority Traffic Controller.

Traffic control measures shall be implemented during the construction phase of the development in accordance with the certified plan.  A copy of the plan shall be available on site at all times.

Note:   A copy of the Traffic Control Plan shall accompany the Notice of Commencement to Liverpool City Council.

Site Facilities

- Toilet facilities must be available or provided at the work site and must be maintained until the works are completed at a ratio of one toilet plus one additional toilet for every 20 persons employed at the site. Each toilet must:

  - be a standard flushing toilet connected to a public sewer, or

  - have an on-site effluent disposal system approved under the Local Government Act 1993, or

  - be a temporary chemical closet approved under the Local Government Act 1993.

- Adequate refuse disposal methods and builders storage facilities shall be installed on the site. Builders’ wastes, materials or sheds are not to be placed on any property other than that which this approval relates to.

Notification of Service Providers

- The approved development must be approved through the ‘Sydney Water Tap In’ service to determine whether the development will affect any Sydney Water wastewater and water mains, stormwater drains and/or easement, and if any requirements need to be met. A receipt must be provided to Council.

Please refer to the website www.sydneywater.com.au for more information.

Environmental Management

- An Environmental Management Plan (EMP) shall be developed and submitted to the Principal Certifying Authority for approval. The EMP shall provide a comprehensive and complete action and implementation plan to ensure that the anthropological and natural environment is not unacceptably impacted upon by the proposal. The EMP shall include but not be necessarily limited to the following measures:

(a)	Measures to control noise emissions from the site;

(b)	Measures to suppress odours and dust emissions;

(c)	Selection of traffic routes to minimise residential noise intrusions;

(d)	Soil and sediment control measures;

(e)	Measures to identify hazardous and industrial wastes and the procedures for removal and disposal including asbestos; and

(f)	Community consultation.

- Adequate soil and sediment control measures shall be installed and maintained.  Furthermore, suitable site practices shall be adopted to ensure that only clean and unpolluted waters are permitted to enter Council’s stormwater drainage system during construction/demolition. Measures must include, as a minimum:

(a)	Siltation fencing;

(b)	Protection of the public stormwater system; and

(c) 	Site entry construction to prevent vehicles that enter and leave the site from tracking loose material onto the adjoining public place.

Aboriginal Heritage

- All relevant on-site staff and contractors should be made aware of their statutory obligations for heritage under NSW National parks and Wildlife Act 1974 and the NSW Heritage Act 1977. They are to be informed of what the potential heritage on the site will be and the significant of the heritage. The site supervisor is to maintain a record of who has completed the heritage induction and this is to be provided to Council prior to Issue of Occupation Certificate.

D.	DURING CONSTRUCTION

The following conditions shall be complied with during construction:

Building Inspections

- The building works must be inspected by the Principal Certifying Authority, in accordance with sections 6.5 (3) of the Environmental Planning & Assessment Act 1979 and clause 162A of the Environmental Planning & Assessment Regulation 2000, to monitor compliance with the relevant standards of construction, Council’s development consent and the construction certificate.

- The Principal Certifying Authority must specify the relevant stages of construction to be inspected and a satisfactory inspection must be carried out, to the satisfaction of the Principal Certifying Authority, prior to proceedings to the subsequent stages of construction or finalisation of the works (as applicable).

Identification Survey Report

- The building and external walls are not to proceed past ground floor/reinforcing steel level until such time as the PCA has been supplied with an identification survey report prepared by a registered surveyor certifying that the floor levels and external wall locations to be constructed, comply with the approved plans, finished floor levels and setbacks to boundary/boundaries. The slab shall not be poured, nor works continue, until the PCA has advised the builder/developer that the floor level and external wall setback details shown on the submitted survey are satisfactory.

In the event that Council is not the PCA, a copy of the survey shall be provided to Council within three (3) working days.

On placement of the concrete, works again shall not continue until the PCA has issued a certificate stating that the condition of the approval has been complied with and that the slab has been poured at the approved levels.

Construction Requirements (Excavation)

- In the event the development involves an excavation that extends below the level of the base of the footings of a building on adjoining land, the following is to be undertaken at full cost to the developer:

(a)	Protect and support the building, structure or work from possible damage from the excavation, and

(b)	where necessary, underpin the building, structure or work to prevent any such damage.

(c)	a and b above does not apply if the person having the benefit of the development consent owns the adjoining land or the owner of the adjoining land has given consent in writing to that condition not applying.

(d)	Retaining walls or other approved methods necessary to prevent the movement of excavated or filled ground, together with associated subsoil drainage and surface stormwater drainage measures, shall be designed strictly in accordance with the manufacturers details or by a practising structural engineer.

Footpath and Road Carriageway

- The applicant/ builder shall be responsible to report to the Council any damage to Council’s footpath and road carriageway as a consequence of demolition or excavation or building activities or delivery/ departure of materials associated with this site.  The damage shall be reported to Council as soon as the damage becomes apparent to the builder/ site manager.  Arrangements to the satisfaction of Council are to be made for making safe by temporary repairs to the public way until permanent restoration and repair can be organised with Council.

Retaining Walls

- Retaining walls or other approved methods necessary to prevent the movement of excavated or filled ground, together with associated subsoil drainage and surface stormwater drainage measures, shall be designed strictly in accordance with the manufacturers details or by a practising structural engineer.  Retaining walls on any boundary are to be of masonry construction.

Security Fence

- A temporary security fence to WorkCover Authority requirements is to be provided to the property during the course of construction.

Note: 	Fencing is not to be located on Council’s reserve area.

Hours of Construction Work

- Construction work/civil work/demolition work, including the delivery of materials, is only permitted on the site between the hours of 7:00am to 6:00pm Monday to Friday and 8:00am to 1:00pm on Saturday. No work will be permitted on Sundays or Public Holidays, unless otherwise approved by Council

Drainage Connection

- Prior to the connection of private drainage to Council’s drainage system, an inspection is to be carried out by Liverpool City Council’s Development Engineering Unit. A fee will be charged in accordance with Council’s adopted Fees and Charges, and is to be paid prior to the inspection.

Removal/ Decommissioning of Underground Petroleum Storage System

- Decommissioning of the unused underground storage tank shall be undertaken in accordance with the following:

- Protection of the Environment Operations (Underground Petroleum Storage Systems) Regulation 2014;

- AS 4976-2008 (R2016) - The removal and disposal of underground petroleum storage tanks; and

- UPSS Technical note: Decommissioning, Abandonment and removal of UPSS by NSW Department of Environment, Climate Change and Water, now known as Office of Environment and Heritage.

A Validation Assessment of the soils upon removal of the UPSS is to be undertaken by a suitably qualified and certified contamination consultant and a report submitted to Liverpool City Council to confirm that the site is suitable for the proposed use.

Note: ‘Suitably qualified and experienced contaminated land consultant’ means someone who is certified under either the Environment Institute of Australia and New Zealand’s Certified Environmental Practitioner (Site Contamination) Scheme (CEnvP(SC)) or the Soil Science Australia Certified Professional Soil Scientist Contaminated Site Assessment and Management (CPSS CSAM) Scheme.

Major Earthworks

- All earthworks shall be undertaken in accordance with AS 3798 and Liverpool City Council’s Design Guidelines and Construction Specification for Civil Works.

The level of testing shall be determined by the Geotechnical Testing Authority/ Superintendent in consultation with the Principal Certifying Authority.

Construction Noise and Vibration

- Noise and vibration associated with excavation, demolition and construction activities shall comply with the management levels detailed within the ‘Interim Construction Noise Guideline’ published by the Department of Environment and Climate Change NSW (DECC 2009/265) dated July 2009 and acceptable vibration values prescribed within the Environmental Noise Management Assessing Vibration: A Technical Guideline (Department of Environment and Conservation, 2006).

All feasible and reasonable noise and vibration mitigation measures shall be implemented and any activities which may exceed the construction noise management levels and vibration criteria shall be identified and managed in accordance with the approved Construction Noise, Vibration Assessment and Management Plan.

- Construction activities, including operation of vehicles, shall be conducted so as to avoid unreasonable noise or vibration and cause no interference to adjoining or nearby occupations.  Special precautions must be taken to avoid nuisance in neighbouring residential areas, particularly from machinery, vehicles, warning sirens, public address systems and the like.  In the event of a noise or vibration problem arising at the time, the person in charge of the premises shall when instructed by Council, cause to be carried out, an acoustic investigation by an appropriate acoustical consultant and submit the results to Council.  If required by Council, the person in charge of the premises shall implement any or all of the recommendations of the consultant and any additional requirements of Council to Council’s satisfaction.

General Site Works

- Building operations such as brick cutting, mixing mortar and the washing of tools, paint brushes, form-work, concrete trucks and the like shall not be performed on the public footway or any other locations which may lead to water pollution.

- Erosion and sediment control measures shall remain in place and be maintained until all disturbed areas have been rehabilitated and stabilised.

- Vehicular access to the site shall be controlled through the installation of wash down bays or shaker ramps to prevent tracking of sediment or dirt onto adjoining roadways. Where any sediment is deposited on adjoining roadways is shall be removed by means other than washing. All material is to be removed as soon as possible and the collected material is to be disposed of in a manner which will prevent its mobilisation.

- Where operations involve excavation, filling or grading of land, or removal of vegetation, including ground cover, dust is to be suppressed by regular watering until such time as the soil is stabilised to prevent airborne dust transport. Where wind velocity exceeds five knots the PCA may direct that such work is not to proceed.

- All vehicles involved in the delivery, demolition or construction process departing from the property shall have their loads fully covered before entering the public roadway.

- The developer is to maintain all adjoining public roads to the site in a clean and tidy state, free of excavated “spoil” material.

- All disturbed areas shall be progressively stabilised and/or revegetated so that no areas remain exposed to potential erosion damage for a period of greater than 14 days.

Aboriginal Heritage - Unexpected finds

- If Aboriginal object/s or historic archaeological relics are identified during works, then all works in the immediate area must cease and the area secured. The Office of Environment and Heritage must be notified by ringing the Enviroline 131 555. No works are to commence until authorisation has been received from the Office of Environment and Heritage and the appropriate permits have been obtained.

- In the event that skeletal remains are uncovered, work must cease immediately in that area and the area secured. NSW Police must be contacted and no further action taken until written advice has been provided by the NSW Police. If the remains are determined to be of Aboriginal origin, the Office of Environment and Heritage must be notified by ringing the Enviroline 131 555 and a management plan prior to works re-commencing must be developed in consultation with relevant Aboriginal stakeholders.

- Copies of all Archaeological assessments and reports (including summary excavation and analysis reports) are to be provided to Liverpool City Council, Liverpool City Library and the relevant Local Aboriginal Land Councils (where applicable).

Pollution Control – Truck Movements

- The loading and unloading of all vehicles associated with the development must be undertaken within the property boundary of the premises subject to this consent.

Measures must be implemented to prevent tracking of sediment by vehicles onto roads.

Vehicle loads must be covered when entering and exiting the site with material.

Water Quality

- All topsoil, sand, aggregate, spoil or any other material shall be stored clear of any drainage line, easement, water body, stormwater drain, footpath, kerb or road surface and there shall be measures in place in accordance with the approved erosion and sediment control plan.

Air Quality

- Dust screens shall be erected and maintained in good repair around the perimeter of the subject land during land clearing, demolition, and construction works.

Where operations involve excavation, filling or grading of land, or removal of vegetation, including ground cover, dust is to be suppressed by regular watering until such time as the soil is stabilised to prevent airborne dust transport. Where wind velocity exceeds five knots the Principal Certifying Authority may direct that such work is not to proceed.

All vehicles involved in the delivery, demolition or construction process departing from the property shall have their loads fully covered before entering the public roadway.

Removal of Dangerous and/or Hazardous Waste

- All dangerous and/or hazardous material shall be removed by a suitably qualified and experienced contractor, licensed by WorkCover NSW. The removal of such material shall be carried out in accordance with the requirements of WorkCover NSW. The material shall

be transported and disposed of in accordance with DECCW (EPA) requirements.

- All demolition and construction waste must be separated as it is generated and kept in separate bays, builder’s site bins and/or skips.

- All lightweight or granular excavation or construction wastes such as wrapping, packaging materials, bags, insulation, sand, soil etc. must be kept fully enclosed to prevent them from becoming wind-blown litter in strong wind conditions or from washing into drains, sewers or waterways or onto neighbouring properties or public land in wet weather.

Waste Management Plan

- The Waste Management Plan submitted to and approved by Council must be adhered to at all times throughout all stages of the development. Supporting documentation (receipts/dockets) of waste/recycling/disposal methods carried out, is to be kept and must be produced upon the request of Council or any other authorised officer.

Note: 	Any non-compliance with this requirement will result in penalties being issued.

Contamination

- The development, including all civil works and demolition, must comply with the requirements of the Contaminated Land Management Act, 1997, State Environmental Planning Policy No. 55 – Remediation of Land, and Managing Land Contamination – Planning Guidelines (Planning NSW/EPA 1998).

Imported Fill Material

- Filling material must be limited to the following:

(a)	Virgin excavated natural material (VENM)

(b)	Excavated natural material (ENM) certified as such in accordance with Protection of the Environment Operations (Waste) Regulation 2014; and/or

(c)	Material subject to a Waste Exemption under Clause 91 and 92 Protection of the Environment Operations (Waste) Regulation 2014 and recognised by the NSW Environment Protection Authority as being “fit for purpose” with respect to the development subject of this application.

Certificates proving that the material imported is ENM or VENM must be provided to the Principal Certifying Authority prior to filling. Certificates are to be provided to Council officers if and when requested.

Fill imported on to the site must be compatible with the existing soil characteristic for site drainage purposes.

Record Keeping of Imported Fill

- Records of the following must be submitted to the principal certifying authority monthly and at the completion of earth works:

(a)	The course (including the address and owner of the source site), nature and quantity of all incoming loads including the date, the name of the carrier, and the vehicle registration;

(b)	The results of a preliminary contamination assessment carried out on any fill material used in the development.

(c)	The results of any chemical testing of fill material.

Site Remediation Works

- The site must be remediated in accordance with:

(a)	Remedial Action Plan: Proposed Warehouse Development 28 Yurrunga Street, Prestons, NSW (Ref. P1907209JR04V01, Revision 1) prepared by Martens Consulting PTY LTD dated 6 December 2019

(b)	State Environmental planning Policy No. 55 – Remediation of Land;

(c)	National Environment Protection (Assessment of Site Contamination) Measure (ASC NEPM, 1999 as amended 2013); and Traffic Management

(d)	The guidelines in force under the Contaminated Land Management Act 1997.

The applicant must engage an appropriately qualified and experienced environmental consultant to supervise all aspects of site remediation and validation works in accordance with the approved Remedial Action Plan.

Liverpool City Council must be informed in writing of any proposed variation to the remediation works. Liverpool City Council must approve these variations in writing prior to commencement/ recommencement of works.

Unidentified Contamination

- Any new information which comes to light during demolition or construction works which has the potential to alter previous conclusions about site contamination must be notified to Council and the accredited certifier immediately after discovery. A Section 4.55 Application under the EP&A Act shall be made for any proposed works outside the scope of the approved development consent.

Traffic Conditions

- All works within the road reserve are to be at the applicant cost and all signage is to be in accordance with the RMS’s Traffic Control at Worksites Manual and the RMS’s Interim Guide to Signs and Markings.

- If a works zone is required, an application must be made to Council’s Transport Planning section.  The application is to indicate the exact location required and the applicable fee is to be included.  If parking restrictions are in place, an application to have the restrictions moved, will need to be made.

Notice must be given to Council’s Transport Planning section of any interruption to pedestrian or vehicular traffic within the road reserve, caused by the construction of this development. A Traffic Control Plan, prepared by an accredited practitioner must be submitted for approval, 48 hours to prior to implementation. This includes temporary closures for delivery of materials, concrete pours etc.

- Applications must be made to Council’s Transport Planning section for any road closures. The applicant is to include a Traffic Control Plan, prepared by a suitably qualified person, which is to include the date and times of closures and any other relevant information.

- The endorsed Construction Traffic Management Plan (CTMP) is to be implemented during the construction.

Car Parking Areas

- Car parking spaces and driveways must be constructed of a minimum of two coat finish seal or better. The spaces must be clear of obstructions and columns, permanently line marked and provided with adequate manoeuvring facilities. The design of these spaces must comply with Council’s DCP 2008, and Australian Standard 2890.1 Parking Facilities – Off Street Car Parking.

All car parking areas to be appropriately line marked and sign posted in accordance with the approved plans.  All customer/visitor/staff parking areas are to be clearly signposted limiting car parking for customers/visitors/staff only. The applicant is to cover the costs of installation and maintenance of the signage.

The on-site parking spaces shown in the approved plans must be identified in accordance with A.S.2890.1 Parking Facilities – Off-Street Car Parking.

Street Lighting

- Street lighting is to be provided for all new and existing streets as part of the proposed development to Liverpool City Council’s standards.

The developer shall submit a Public Lighting Design Brief to Council for approval for the provision of street lighting on all new public roads dedicated to Council.  A street lighting design plan must be prepared by an accredited service provider for approval prior to construction.  All street lighting must comply with the electricity service provider Street Lighting Policy and illumination requirements and Council’s Street Lighting policy.

All cost associated with the installation of street lighting shall be borne by the developer.

Ventilation

- The premises shall be ventilated in accordance with the requirements of the NCC/BCA (if using deemed to satisfy provisions: AS 1668, Parts 1 & 2).

- The design, construction, installation and commissioning of the mechanical ventilation systems(s) serving the premises shall be carried out in accordance with Australian Standard 1668 Parts 1 & 2.

The mechanical exhaust discharge point shall be designed and installed by an appropriately qualified person, and shall be positioned to comply with Section 3.7 of Australian Standard 1668 Part 2 – 1991.

External

- Switchboards for utilities shall not be attached to the street and/or road elevations of the development.

- The mailboxes are to be consistent with the design and colours and materials for the development.

- Any external lighting is to incorporate full cut-off shielding and is to be mounted so as to not cause any glare or spill over light nuisance within the development, neighbouring properties or road users.

- The reflectivity index of glass used in the external facade of the building is not to exceed 20%.

Demolition Inspection

- The following inspections are required to be undertaken by Council in relation to approved demolition works:

- Immediately prior to the commencement of the demolition or handling of any building structure that contains asbestos. The applicant shall also notify the occupants of the adjoining premises and Workcover NSW prior to the commencement of any works.  Please note that demolition works are not permitted to commence on site until such time as a satisfactory inspection result is obtained from Council.

- Immediately following completion of the demolition. Please note that proof of appropriate disposal of demolition materials (including asbestos) may be required at this time in accordance with the approved Waste Management Plan.

To book an inspection with Council, please call 1300 362 170.

Craning and Hoarding

- If the work is likely to cause pedestrian or vehicular traffic in a public area to be obstructed or rendered inconvenient; or if craning of materials is to occur across a public area or road reserve area a construction hoarding must be erected to prevent any substance from, or in connection with the construction site, falling onto a public area.

Such hoarding or barrier must be designed and erected in accordance with Council’s guidelines on hoarding construction. Relevant application under the Roads Act approval must be completed and fees paid prior to the construction of a hoarding on Council road reserve area.

Construction Requirements

- Lifting or craning materials over a public footway or roadway is not permitted unless a “B” class construction hoarding has been installed in compliance with Work Cover authority requirements.

Security and Safety

- Adequate lighting is required at the entrances and main foyers or the building, basement carpark, and mailbox area.

- Surveillance cameras are required to be installed covering the entrance and exit and main areas of the car park.

- The underground car park is required to be locked with access to be provided to residents only.

Vegetation and Landscaping

- No known environmental or noxious weeds or known invasive plant species shall be included in the landscaping/revegetation.

- Mulch generated from exotic trees or other weed species cleared shall not be used on site. It shall be removed from the site and disposed of appropriately and in accordance with legislative requirements.

- Any imported soil and/or mulch shall be free of contaminants, seed and propagules of weeds and undesirable species.  Mulch shall not be used on flood liable land.

- Premium quality organic garden soil shall be incorporated into all planting areas in sufficient quantity to achieve optimum plant growing conditions.

Salinity

- All building materials utilised below dampcourse level or water proof membranes are to be resistant to damage from salt damp.  Dampcourses and water proof membranes are to have a high resistance to construction damage, are to be themselves resistant to damage from salt damp and are not to be bridged in any manner by porous materials.

- Any site drainage or landscaping works are not to result in the significant mobilisation of soil salts on or off the site.

Waste Storage Area

- Any bin bays must be:

(a)	Provided with mechanical ventilation;

(b)	Provided with a hose cock for hosing the garbage bin bay and a sewered drainage point in or adjacent to the bin storage area.  The drainage point should have a fine grade drain cover sufficient to prevent coarse pollutants from entering the sewer.  If the hose cock is located inside the bin storage bay, it is not to protrude into the space indicated for the placement of bins;

(c)	Provided with sufficient light to permit usage at night; and

(d)	Allocated with sufficient space within the bin bay to allow for access to all required bins by occupants and waste collectors, as well as manoeuvring of bins within the bay and for the removal and return of bins by the waste collector

E.	PRIOR TO ISSUE OF AN OCCUPATION CERTIFICATE

The following conditions shall be complied with prior to issue of either an Interim or Final Occupation Certificate by the Principal Certifying Authority:

Building/Compliance

- An Occupation Certificate must be obtained from the Principal Certifying Authority prior to any occupation of the building work encompassed in this development consent (including alterations and additions to existing buildings), in accordance with the relevant provisions of the Environmental Planning & Assessment Act 1979.

- Details of critical stage inspections carried out by the principal certifying authority together with any other certification relied upon must be provided to Council with the occupation certificate.

- The premises must not be utilised until an Occupation Certificate is issued by the PCA. Copies of all documents relied upon for the issue of the Occupation Certificate must be attached to the Occupation Certificate and registered with Council.

(Temporary Covid-19) Section 7.11 Payment (Liverpool Contributions Plan 2009)

- As a consequence of this development, Council has identified an increased demand for public amenities and public services.  The following payment is imposed in accordance with Liverpool Contributions Plan 2009 as amended.

The total contribution for each stage is provided below and will be adjusted at the time of payment in accordance with the contributions plan:

Stage 1 = $879,998.00

Stage 2 = $879,998.00

A breakdown of the contributions payable is provided in the attached payment form.

The Contributions Plan may be inspected online at www.liverpool.nsw.gov.au

Payment must be accompanied by the attached form.

Stage 1 Contribution

- Prior to the issue of any occupation certificate for Stage 1, all section 7.11 contributions must be paid as required by condition 125. Evidence is to be provided to the satisfaction of the relevant certifying authority that all outstanding contributions have been paid to Council. If no construction certificate in respect to Stage 1 has been issued on or before 25 September 2022, the monetary contribution for Stage 1 as required by condition 125 must be paid before the issue of any construction certificate after that date for Stage 1 works.

Stage 2 Contribution

- Prior to the issue of any occupation certificate for Stage 2, all section 7.11 contributions must be paid as required by condition 125. Evidence is to be provided to the satisfaction of the relevant certifying authority that all outstanding contributions have been paid to Council. If no construction certificate in respect to Stage 2 has been issued on or before 25 September 2022, the monetary contribution for Stage 1 as required by condition 125 must be paid before the issue of any construction certificate after that date for Stage 2 works.

Cladding

- Prior to issuing an occupation certificate the Principal Certifying Authority must be satisfied that suitable evidence has been provided to demonstrate that the external wall cladding material and system is consistent with the consent documentation, NCC and relevant Australian Standards.

Fire Safety Certificate

- A single and complete Fire Safety Certificate, certifying the installation and operation of all of the fire safety measures within the building must be submitted to Council with the Occupation Certificate.

Traffic

- The developer should provide localised road widening at Bernera Road/Yarrunga Street/Yato Road intersection to allow a 26m B-double to turn left at the same time when right turns occur from Bernera Road to Yarrunga Street and Yato Road. The design plan of the proposed widening should be submitted to Council Traffic and Transport Section for review and approval. A meeting is suggested between the developer and Council Traffic and Transport Section to discuss design requirements prior to the design plan being prepared.

S138 Roads Act – roadworks requiring approval of civil drawings.

- Prior to the issue of an Occupation Certificate for building works the Certifying Authority shall ensure that a S138 Roads Act application, including the payment of application and inspection fees, has been lodged with, and approved by Liverpool City Council (being the Roads Authority under the Roads Act), for road widening of Bernera Road/Yarrunga Street/Yato Road intersection to allow a 26m B-double to turn left at the same time when right turns occur from Bernera Road to Yarrunga Street and Yato Road.

Engineering drawings are to detail road widening at the Bernera Road/Yarrunga Street/Yato Road intersection to allow a 26m B-double to turn left at the same time when right turns occur from Bernera Road to Yarrunga Street and Yato Road.  The design plan of the proposed widening will be submitted to Council Traffic and Transport Section for review and approval.

The proposed intersection upgrade works will require approval by Roads and Maritime Services under Section 87 of the Roads Act, 1993.

Engineering plans are to be prepared in accordance with the development consent, Liverpool City Council’s Design Guidelines and Construction Specification for Civil Works, Austroad Guidelines and best engineering practice.

Note:

- Where Liverpool City Council is the Certifying Authority for the development the Roads Act approval for the above works may be issued concurrently with the Construction Certificate.

Landscaping

- Upon completion of the approved landscape works associated with the development and prior to the issue of any Occupation Certificate, an Implementation Report is to be submitted to the PCA attesting to the satisfactory completion of the landscape works in accordance with the approved landscape plan. The report is to be prepared by a suitably qualified person.

Liverpool City Council clearance – Roads Act/ Local Government Act

- Prior to the issue of an Occupation Certificate, the Principal Certifying Authority shall ensure that all works associated with a S138 Roads Act approval or S68 Local Government Act approval have been inspected and signed off by Liverpool City Council.

- All roadworks including the required localised road widening at Bernera Road/Yarrunga Street/Yato Road intersection, signposting and street lighting are to be completed to Liverpool Council requirements, at no expense to Liverpool Council or Roads and Maritime Services.

Works as Executed

- Prior to the issue of an Occupation Certificate, works-as-executed drawings and compliance documentation shall be submitted to the PCA in accordance with Council’s Design Guidelines and Construction Specification for Civil Works.

An original set of works-as-executed drawings and copies of compliance documentation shall also be submitted to Council with notification of the issue of the Occupation Certificate where Council is not the PCA.

Structural Engineer Certificate

- A Structural Engineer’s construction certification of all structures is to be issued to the PCA prior to the issue of the Occupation Certificate.

Stormwater Compliance

- Prior to the issue of an Occupation Certificate the PCA shall ensure that the:

  - On-site detention system/s;

  - Stormwater pre-treatment system/s; and

  - Basement carpark pump-out system.

  - Have been satisfactorily completed in accordance with the approved Construction Certificate and the requirements of this consent,

  - Have met the design intent with regard to any construction variations to the approved design, and

  - Any remedial works required to been undertaken have been satisfactorily completed.

Details of the approved and constructed system shall be provided as part of the Works-As-Executed drawings.

Restriction as to User and Positive Covenant

- Prior to the issue of an Occupation Certificate a restriction as to user and positive covenant relating to the on-site detention system/s, stormwater pre-treatment system/s and basement carpark pump-out system shall be registered on the title of the property.

The restriction as to user and positive covenant shall be in Liverpool City Council’s standard wording as detailed in Council’s Design and Construction Guidelines and Construction Specification for Civil Works.

Basement Pump-out System

- Stormwater runoff from the proposed driveway to the underground garage shall be via a pump-out system subject to the following conditions:

(a)	The pump-out system shall be independent of any gravity drainage lines except at the site property boundary inspection pit where a surface grated inlet pit shall be constructed, from which a connection may be permitted to the gravity stormwater system.

(b)	Engineering details and manufacturer’s specifications for pumps and switching system shall be submitted for approval prior to issue of construction certificate.

(c)	An 88B positive covenant shall be placed on the property title.  This requires the property owner to be responsible for the proper maintenance and repair of the abovementioned pumps, pipes and pit system.  Council is the Authority benefited and the property owner is burdened by this restriction.  Evidence of the creation of the positive covenant shall be forwarded to Council prior to the issue of an OC.

Roadworks

- All roadworks and signposting is to be completed to Liverpool Council requirements, at no expense to Liverpool Council or Roads and Maritime Services.

- All signs and linemarking as approved by the LTC has been installed.

Footpaths

- Construction of 1.5m wide by 100mm thick (with one layer of SL72 reinforcing mesh) concrete path paving on one side of all residential access roads and both sides of all collector and distributor roads. Path paving will not be required in minor cul-de-sac with less than fifteen lots.

Rectification of Damage

- Prior to the issue of an Occupation Certificate any damage to Council infrastructure not identified in the dilapidation report, as a result of the development shall be rectified at no cost to Council.

Any rectification works within Yarrunga Street will require a Roads Act application.  The application is to be submitted and approved by Liverpool City Council prior to such works commencing.

Service Providers

- A Section 73 Compliance Certificate under the Sydney Water Act 1994 must be submitted to the PCA prior to issue of Occupation Certificate.

- Notification of arrangement for the development from Endeavour Energy shall be submitted to Council.

- Prior to the issue of an occupation certificate, written certification from all relevant service providers that the telecommunications infrastructure is installed in accordance with:

- The requirements of the Telecommunications Act 1997;

- For a fibre ready facility, the NBN Co’s standard specifications current at the time of installation; and

- For a line that is to connect a lot to telecommunications infrastructure external to the premises, the line shall be located underground.

Unless otherwise stipulated by telecommunications legislation at the time of construction, the development must be provided with all necessary pits and pipes, and conduits to accommodate the future connection of optic fibre technology telecommunications.

Waste

- All waste products associated with the use of the residential flat building are to be placed in containers and stored within the building.

- A ‘restriction as to user’ is to be placed on the title of the property at the Applicant’s expense, which may not be altered or removed without Council’s consent, which states:

‘Liverpool City Council is not responsible for the provision of any green waste services or green waste bins to this property.’

- All waste management facilities, equipment (except waste bins), features and permanent signage will be installed and operational prior to the issue of an Occupation Certificate

Dilapidation Report

- The construction of concrete footpath paving and associated works along the frontage of the site shall be carried out by a licensed construction contractor at the applicant’s expense and shall in accordance with Council’s specifications.

- Any rectification works required by Council regarding the condition of Council infrastructure shall be undertaken, at full cost to the developer.

- Prior to the issue of an Occupation Certificate, the Principal Certifying Authority shall ensure that all works and/or requirements associated with the existing power pole adjacent to the driveway in the site frontage have been signed off by the relevant Authority.

Directional Signage

- Prior to the issue of an Occupation Certificate, directional signage and linemarking shall be installed indicating directional movements and the location of customer parking to the satisfaction of the Principal Certifying Authority.

Recommendations of Acoustic Report

- Upon completion of works and prior to the issue of an Interim or Final Occupation Certificate, written certification prepared by a suitably qualified acoustic consultant shall be submitted to and approved by the Principal Certifying Authority (PCA).

The written certification prepared by the suitably qualified acoustic consultant shall confirm that the development complies with all requirements and recommendations detailed within the approved acoustic report titled Noise Impact Assessment (Ref. SYD0923, Rev. 04) prepared by ADP Consulting Pty Ltd dated 17 June 2020 and any further requirements associated with mechcnaila plant and equipment selection. The acoustic consultant shall confirm that the development or use is capable of operating in accordance with the design criteria.

Note: ‘Suitably qualified acoustic consultant’ means a consultant who is a member of the Australian Acoustical Society or employed by an Association of Australasian Acoustical Consultants (AAAC) member firm.

Mechanical Ventilation Certification

- Upon completion of works and prior to the issue of an Interim or Final Occupation Certificate, a Mechanical Ventilation Certificate of Completion and Performance prepared by a professional engineer or other suitably qualified person shall be submitted to the Principal Certifying Authority (PCA) for their review and approval. The certification shall be accompanied by details of the tests carried out in relation to ventilation and acoustics and confirm that the systems comply with the approved plans, specifications, Building Code of Australia and Australian Standard AS 1668 Parts 1 and 2’.

Display of Street Numbers

- Street numbers must be prominently displayed at the front of the development in a contrasting colour to the building materials and at the front of each individual unit.  The number should be a minimum height of 120mm and be visible at night.

Noise Management Plan

- A Noise Management Plan shall be prepared under the supervision of a suitably qualified acoustic consultant. The Noise Management Plan must identify and implement strategies to minimise noise from the proposed development and incorporate: approaches for promoting noise awareness by patrons and staff; training procedures; a complaint lodgement procedure to ensure that members of the public and local residents are able to report noise issues; an ongoing review process and a plan for responding to noise complaints.

The Noise Management Plan shall clearly specify the responsibilities of site personnel in managing noise and include a detailed list of steps taken to manage potential noise impacts. This documentation shall be submitted to the PCA and Council for review and approval prior to issue of an Interim or Final Occupation Certificate.

Note: ‘Suitably qualified acoustic consultant’ means a consultant who possesses Australian Acoustical Society membership or are employed by an Association of Australasian Acoustical Consultants (AAAC) member firm.

Validation Report

- Prior to issue of the occupation certificate a detailed Validation report must be submitted to Council and the Principal Certifying Authority. The Report must be prepared in accordance with:

  - NSW Contaminated Land Planning Guidelines (1998);

(b) 	Relevant EPA guidelines. In particular the Contaminated Land Guidelines – Consultants Reporting on Contaminated Land (NSW EPA 2020); and

(c)	National Environmental Protection (Assessment of Site Contamination) Measure 1999 (as amended 2013).

The Validation Report must be prepared or reviewed and approved by a suitably qualified and experienced contaminated land consultant.

The report's cover or title page of the document shall include a personalised electronic seal for either the CEnvP(SC) or CPSS CSAM scheme.

The Validation report must verify that the land is suitable for the purposed use(s), and that the remediation and validation of the site has been undertaken in accordance with the approved Remedial Action Plan.

Note: ‘Suitably qualified and experienced contaminated land consultant’ means someone who is certified under either the Environment Institute of Australia and New Zealand’s Certified Environmental Practitioner (Site Contamination) Scheme (CEnvP(SC)) or the Soil Science Australia Certified Professional Soil Scientist Contaminated Site Assessment and Management (CPSS CSAM) Scheme.

Parking Management System

- The applicant is to prepare a parking management system that gives all tenants access to the parking spaces to be provided to assist in the full utilisation of the parking provided.

Bonds

- A maintenance bond in the form of a bank Guarantee or cash bond ($TBA at the time of payment), shall be lodged with Council prior to the issue of a Occupation Certificate. The bond shall cover maintenance and any damage to roads, drainage lines, public reserves or other council property or works required as a result of work not in accordance with Council’s standards, and /or development consent conditions. The bond will be held by Council for a minimum period of 12 months from the date of Council acceptance of final works.

F.	Conditions Relating to Use

The following general conditions shall be complied with at all times:

Car Parking

- A total of 438 off street (including 4 accessible car parking spaces) car spaces, 20 motorcycle spaces and 36 bicycle spaces must be provided on site. A total of 159 temporary car parking spaces must be provided for the duration of Stage 1 and whilst the existing warehouse remains operational during Stage 1 of the development.

There will be no on-street parking along the frontage of the development.

The applicant is to have a parking management system that gives all tenants access to the parking spaces to be provided to assist in the full utilisation of the parking provided.

- All parking areas shown on the approved plans must be used solely for this purpose.

- The operator of the development must not permit the reversing of vehicles onto or away from the road reserve, including any garbage and recycling collection vehicles.  All vehicles must be driven forward onto and away from the development and adequate space must be provided and maintained on the land to permit all vehicles to turn in accordance with AS 2890.1 Parking Facilities – Off Street Car Parking.

- All line marking and sign posting is to be maintained in good condition at all times, to the satisfaction of Council.

- Council’s on-street assets should be protected at all times.  Any damages should be rectified to Council satisfaction.

Landscaping

- Landscaping shall be maintained in accordance with the approved plan, in a healthy state and in perpetuity by the existing or future owners and occupiers of the development. If any of the vegetation comprising the landscaping dies or is removed, it is to be replaced with vegetation of the same species, and similar maturity as the vegetation which has died or was removed.

Mail-boxes

- The mailboxes must not be accessed by universal keys and must each have their own keys for private access.

- Vegetation must not cover or obstruct natural surveillance to the mailboxes.

Noise and Environmental Emissions

- The intruder alarm/s associated with the development shall only be permitted to operate in accordance with the requirements of Clause 53 of the Protection of the Environment Operations (Noise Control) Regulation 2000 under the POEO Act.

Waste Management

- Waste and recyclable material generated from the operations of the premises shall be managed in a satisfactory manner that does not give rise to offensive odour or encourage pest activity. All waste material shall be regularly removed from the premises. Waste shall not be permitted to accumulate near the waste storage bins.

- Waste must be adequately secured and contained within designated waste areas and must not leave the site onto neighbouring public or private properties.

- Waste bins must be stored in designated garbage areas, which must be kept tidy at all times. Bins must not be stored or allowed to overflow in parking or landscaping areas, must not obstruct the exit of the building, and must not leave the site onto neighbouring public or private properties.

- The development is required to be serviced by a waste collection contractor, once in any one week.

Waste Storage Area

- Any bin bays must be:

(a) 	Allocated with sufficient space within the bin bay to allow for access to all required bins by residents and waste collectors, as well as manoeuvring of bins within the bay and for the removal and return of bins by the waste collector.

Noise – General

- Noise associated with the use of the premises, including mechanical plant and equipment, shall not give rise to any one or more of the following:

(a) 	The use of the premises including the cumulative operation of any mechanical plant, equipment, public address system or other amplified sound equipment shall not give rise to the emission of ‘offensive noise’ as defined by the Protection of the Environment Operations Act 1997.

(b) 	The operation of any mechanical plant, equipment, public address system or other amplified sound equipment installed on the premises shall not cause:

i. The emission of noise as measured over a 15 minute period (LAeq (15 minute)) that exceeds the LA90 (15 minute) background noise level by more than 5 dB(A) when measured at the most affected residential boundary. Modifying factor corrections must be applied for tonal, impulsive, low frequency or intermittent noise in accordance with the NSW Environment Protection Authority’s ‘Noise Policy for Industry’ (2017);

ii. An internal noise level in any adjoining occupancy that exceeds the recommended design sound levels specified in Australian/New Zealand Standard AS/NZS 2107:2016 Acoustics – Recommended design sound levels and reverberation times for building interiors;

iii. ‘offensive noise’ as defined by the Protection of the Environment Operations Act 1997; and

iv. Transmission of vibration to any place of different occupancy greater than specified in AS 2670.

- Any alarm installed on the site is to be “silent back to base” type.

Unreasonable Noise and Vibration

- The proposed use of the premises and equipment installed must not create offensive noise so as to interfere with the amenity of the neighbouring properties. Should an offensive noise complaint be received and verified by Council an Acoustic assessment is to be undertaken (by an appropriately qualified consultant), and an acoustic report is to be submitted to Liverpool City Council for review. Any noise attenuation recommendations provided in the report and approved by Council must be implemented.

Note: ‘Suitably qualified acoustic consultant’ means a consultant who possesses Australian Acoustical Society membership or are employed by an Association of Australasian Acoustical Consultants (AAAC) member firm.

Storage of Flammable and Combustible Liquids

- Flammable and combustible liquids shall be stored in accordance with AS 1940:2017– The Storage and Handling of Flammable and Combustible Liquids.

Storage and Handling of Corrosive Substances

- Corrosive Substances must be stored and handled in accordance with AS 3780-2008 The Storage and Handling of Corrosive Substances.

Lighting

- Illumination of the site is to be arranged in accordance with the requirements and specifications of Australian Standard 4282:1997 – Control of the obtrusive effects of outdoor lighting so as not to impact upon the amenity of the occupants of adjoining and nearby premises.

Noise Management Plan

- The approved Noise Management Plan shall be complied with at all times.

Forklifts and Delivery Vehicles

- To minimise potential noise impacts, all forklifts and delivery vehicles associated with the use shall be equipped with smart (self-adjusting) reverse alarms, broadband reverse alarms, combination alarms or other suitable noise reduction technologies instead of tonal reversing alarms.

No diesel forklifts are to be utilised on the site. Forklifts are to be gas or electric as prescribed in the acoustic report titled Noise Impact Assessment (Ref. SYD0923, Rev. 04) prepared by ADP Consulting Pty Ltd dated 17 June 2020.

Environment

- The use of the premises shall not give rise to the emission into the surrounding environment of gases, vapours, dusts or other impurities that are a nuisance, injurious or prejudicial to health.

Compliance with Acoustic Assessment

- The acoustic report titled Noise Impact Assessment (Ref. SYD0923, Rev. 04) prepared by ADP Consulting Pty Ltd dated 17 June 2020 is to be complied with at all times. Operation requirements are as follows:

| Time of day | During Stage 1 | Upon completion of Stage 2 |
| --- | --- | --- |
| Day (7am to 6pm) | > Up to fourteen (14) trucks entering and leaving in any one 15min period > Up to fifty (50) forklifts total to be operating within warehouses | > Up to fourteen (14) trucks entering and leaving in any one 15min period > Up to fifty (50) forklifts total to be operating within warehouses |
| Evening (6pm to 10pm) | > Up to fourteen (14) trucks entering and leaving in any one 15min period > Up to fifty (50) forklifts total to be operating within warehouses | > Up to fourteen (14) trucks entering and leaving in any one 15min period > Up to fifty (50) forklifts total to be operating within warehouses |
| Night (10pm to 7am) | > Up to six (6) trucks entering and leaving in any one 15min period > Up to forty (40) forklifts total to be operating within warehouses > Forklifts and trucks to use broadband reverse alarms > Truck motors to be turned off when in warehouse areas | > Up to six (6) trucks entering and leaving in any one 15min period > Up to forty (40) forklifts total to be operating within warehouses > Forklifts and trucks to use broadband reverse alarms > Truck motors to be turned off when in warehouse areas |

Use of the Site Not Approved

- Each separate occupancy shall be subject to submission (and approval by Council), of a separate Development Application for its use.

G.	ADVISORY

- Sections 8.2, 8.3, 8.4 & 8.5 of the Environmental Planning and Assessment Act 1979 allow Council to reconsider your proposal. Should you wish to have the matter reconsidered you should make an application under that section with the appropriate fee.

- Under Sections 8.7 & 8.10 of the of the Environmental Planning and Assessment Act 1979 applicants who are dissatisfied with the outcome of a consent authority have a right of appeal to the Land and Environment Court. This right must be exercised within six (6) months from the date of this notice. The Court's Office is situated at Level 1, 225 Macquarie Street, Sydney (Telephone 9228 8388), and the appropriate form of appeal is available from the Clerk of your Local Court.

- In accordance with Section 4.53 of the Environmental Planning and Assessment Act 1979, unless otherwise stated by a condition of this consent, this consent will lapse unless the development is commenced within five years of the date of this notice.

- To confirm the date upon which this consent becomes effective, refer to Section 4.20 of the Environmental Planning and Assessment Act 1979. Generally the consent becomes effective from the determination date shown on the front of this notice. However if unsure applicants should rely on their own enquiries.

- To confirm the likelihood of consent lapsing, refer to Section 4.53 of the Environmental Planning and Assessment Act 1979. Generally consent lapses if the development is not commenced within five years of the date of approval. However if a lesser period is stated in the conditions of consent, the lesser period applies. If unsure applicants should rely on their own enquiries.

- In accordance with Sections 8.8 and 8.10 of the Environmental Planning and Assessment Act 1979, an objector who is dissatisfied with the determination of a consent authority to grant consent to a development application for designated development (including designated development that is integrated development), may, within 28 days after the date on which the application is taken to have been determined, appeal to the Land and Environment Court.

- The approval of this application does not imply or infer compliance with the Disability Discrimination Act and that the developer should investigate their liability under the Act.

- The requirements of all authorities including the Environmental Protection Authority and the Work Cover Authority shall be met in regards to the operation of the building.

- “DIAL BEFORE YOU DIG”

Underground assets may exist in the area that is subject to your application. In the interest of health and safety and in order to protect damage to third party assets please contact Dial before you dig at www.1100.com.au or telephone 1100 before excavating or erecting structures (This is the law in NSW). If alterations are required to the configuration, size, form or design of the development upon contact the Dial before You Dig service, an amendment to the development consent (or a new development application) may be necessary. Individuals owe asset owners a duty of care that must be observed when working in the vicinity of plant or assets. It is the individual’s responsibility to anticipate and request the nominal location of plant or assets on the relevant property via contacting the Dial before you dig service in advance of any construction or planning activities.

- TELECOMMUNICATIONS ACT 1997 (COMMONWEALTH)

Telstra (and its authorised contractors) are the only companies that are permitted to conduct works on Telstra’s network and assets. Any person interfering with a facility or installation owned by Telstra is committing an offence under the Criminal Code Act 1995 (Cth) and is liable for prosecution.

Furthermore, damage to Telstra’s infrastructure may result in interruption to the provision of essential services and significant costs. If you are aware of any works or proposed works which may affect or impact on Telstra’s assets in any way, you are required to contact: Telstra’s Network Integrity Team on Phone Number 1800 810 443.

- The Liverpool City Council Local Government area soils and ground water may be subject to varying levels of Salinity. Whilst Council may require applicants to obtain Salinity reports relating to some developments, no assessment may be made by Council in that regard. Soil and ground water salinity levels can change over time due to varying factors. It is recommended that all applicants make their own independent inquiries as to appropriate protection against the current and future potential effect of Salinity to ensure the ongoing structural integrity of any work undertaken.  Liverpool City Council will not accept any liability for damage occurring to any construction of any type affected by soil and or ground water Salinity.

- The cost of any necessary adjustments to utility mains and services shall be borne by the applicant.

- Care shall be taken by the applicant and the applicant’s agents to prevent any damage to adjoining properties.  The applicant or applicant’s agents may be liable to pay compensation to any adjoining owner if, due to construction works, damage is caused to such an adjoining property.

ATTACHMENT 2: 	SECTION 7.11 CONTRIBUTION

CONTRIBUTIONS PURSUANT TO SECTION 7.11 OF THE ENVIRONMENTAL

PLANNING & ASSESSMENT ACT, 1979

Liverpool Contribution Plan 2009 Prestons Industrial Area

When remitting payment as specified in the Conditions of Consent to the approval, this Form must be submitted with your payment.

These figures have been calculated to the CPI 2019 First Quarter and will be adjusted at the time of payment in accordance with the conditions of consent.

APPLICATION NO.:     DA-904/2019 Stage 1

- APPLICANT:		  Muhbbah Construction Pty Ltd

PROPERTY:		  28 Yarrunga Street, PRESTONS NSW 2170 (LOT 2 DP 536915)

PROPOSAL:      	Stage 1:  Demolition of the single storey brick administration building and associated structures within the southern portion of the site and construction of 4 large warehouse over two levels with ancillary office floor area. Retention of the existing warehouse within the northern portion of the site to operate concurrently with Stage 1 of the development with 159 temporary car parking spaces.

| Facilities | Amount ($) | Job No. |
| --- | --- | --- |
| Liverpool Contributions Plan 2009 |  |  |
| District Transport - Land | $54,976 | GL.10000001865.10055 |
| District Transport - Works | $91,466 | GL.10000001865.10055 |
| Local Transport |  |  |
| West of M7 - Works | $20,736 | GL.10000001865.10077 |
| West of M7 - Land | $52,971 | GL.10000001865.10077 |
| District Drainage - Land | $286,694 | GL.10000001866.10076 |
| District Drainage - Works | $50,278 | GL.10000001866.10076 |
| Local Drainage |  |  |
| West of M7 - Land | $119,494 | GL.10000001866.10075 |
| West of M7 - Works | $179,807 | GL.10000001866.10075 |
| Landscape Buffer - Land | $0 | GL.10000001869.10078 |
| Landscape Buffer - Works | $0 | GL.10000001869.10078 |
| Administration | $7,670 | GL.10000001872.10081 |
| Professional Fees | $15,907 | GL.10000001872.10080 |
| TOTAL | $879,998 |  |

---------------------------------------------- OFFICE USE ONLY -------------------------------------------

RECORD OF PAYMENT

# Total Amount paid: ______________________________

# Date:________________________

Receipt No.:_____________________________Cashier:_____________________

CONTRIBUTIONS PURSUANT TO SECTION 7.11 OF THE ENVIRONMENTAL

PLANNING & ASSESSMENT ACT, 1979

Liverpool Contribution Plan 2009 Prestons Industrial Area

When remitting payment as specified in the Conditions of Consent to the approval, this Form must be submitted with your payment.

These figures have been calculated to the CPI 2019 First Quarter and will be adjusted at the time of payment in accordance with the conditions of consent.

APPLICATION NO.:     DA-904/2019 Stage 2

- APPLICANT:		  Muhbbah Construction Pty Ltd

PROPERTY:		  28 Yarrunga Street, PRESTONS NSW 2170 (LOT 2 DP 536915)

PROPOSAL:      	Stage 2:  Demolition of the existing warehouse and associated structures within the northern portion of the site and Construction of 4 warehouse over two levels with  ancillary office floor area and basement carpark with a total of 438 vehicular parking spaces including 4 accessible car parking spaces, 20 motorbike parking spaces and 36 bicycle parking spaces.

| Facilities | Amount ($) | Job No. |
| --- | --- | --- |
| Liverpool Contributions Plan 2009 |  |  |
| District Transport - Land | $54,976 | GL.10000001865.10055 |
| District Transport - Works | $91,466 | GL.10000001865.10055 |
| Local Transport |  |  |
| West of M7 - Works | $20,736 | GL.10000001865.10077 |
| West of M7 - Land | $52,971 | GL.10000001865.10077 |
| District Drainage - Land | $286,694 | GL.10000001866.10076 |
| District Drainage - Works | $50,278 | GL.10000001866.10076 |
| Local Drainage |  |  |
| West of M7 - Land | $119,494 | GL.10000001866.10075 |
| West of M7 - Works | $179,807 | GL.10000001866.10075 |
| Landscape Buffer - Land | $0 | GL.10000001869.10078 |
| Landscape Buffer - Works | $0 | GL.10000001869.10078 |
| Administration | $7,670 | GL.10000001872.10081 |
| Professional Fees | $15,907 | GL.10000001872.10080 |
| TOTAL | $879,998 |  |

---------------------------------------------- OFFICE USE ONLY -------------------------------------------

RECORD OF PAYMENT

# Total Amount paid: ______________________________

# Date:________________________

Receipt No.:_____________________________Cashier:_________________________

ATTACHMENT 3: 	SYDNEY WATER

ATTACHMENT 4: 	TRANSPORT FOR NSW (TfNSW)